## Specifications

| Market Position: DeskBook - Leading Performance: Intel Desktop CPU Processor , Intel Chipset 845PE support FSB-533, DDR-333 SO-DIMM, 4x AGP, discrete VGA design with upgradeable VGA daughter board - Intel Chipset 845GV(FSB533, DDR-266) with UMA VGA support - DT 15” XGA or NB 15” SXGA+ or 16” UXGA/SXGA+ LCD Form Factor: 3-spindle design, brand new ID Wireless: 802.11b or 802.11a/b Mini PCI Solution, Storage: DT 3.5” 5400/7200 rpm HDD or NB 2.5” 9.5 mm HDD Combo (DVD+CD-RW) 24x10x8x24, 8x DVD or 24x CD-ROM. Cost-Effective: 6-layer mother board design with desktop components User-Friendly: Handler design No Battery Processor & Core logic - CPU: Intel Desktop P4/Celeron 1.7Ghz ~ 3.06GHz FCPGA TDP 82W support, Hyper-Thread ready - North Bridge: Dual layout reserved for Intel 845 PE and Intel 845GV - South Bridge: ICH4 with USB 2.0 support Weight and Dimension: - 353.6 x 300.5 x 51.9mm (15” DT LCD, 3.5” HDD), 4.0 kg - 353.6 x 300.5x 48.4mm (16” NB LCD, 3.5”HDD),4.2 kg ###### Display - DT 15”XGA and NB 15” SXGA & 16” UXGA/SXGA+ LCD - nVidia GeForce4 420/440 Go VGA Chip and embedded 32/64 MB DDR graphic memory on daughter card - Simultaneous LCD and CRT display at non-interlace external CRT - Dual View support - 3D video and 4x AGP support - Support TV-Out # System Memory - Two DDR SoDIMM socket up to 1024 MB - 512KB FLASH BIOS with boot block; shadow RAM support ##### Storage Subsystem - 40/80/60/120/200 GB DT 3.5” HDD up to 7200rpm ; 20G/40GB NB 9.5 mm 2.5” HDD - Module bay for Combo (24x10x8x24), DVD (8x) or CD-ROM(24x), 12.7 mm - Internal 1.44-MB, 3.5-inch FDD CardBus - One Type-III # Power Subsystem - ACPI Compliant - 150W AC adapter | # Keyboard and Pointing Device - 84/85/88-key, inverted "T" cursor layout - Built-in touchpad pointing device; located at the center of the palm rest with large two buttons below the Touchpad. - 12 function keys; 4 cursor keys; separate PgUp, PgDn, Home, End keys; two Windows keys; hotkey controls - Embedded numeric keypad - International language support - Scroll key for internet # Audio System - AC-97 audio system with two speakers - SPDIF support for option - SoundBlaster-Pro and MS-Sound System compatible - Super Audio DJ control with small LCD panel for option # V.92 Communications - 56Kbps ITU standard fax/modem in MDC module # Ethernet Connection: - IEEE 802.3 10/100 BaseT Ethernet LAN built-in Wireless LAN: - 802.11a or 802.11a/b build in mini-PCI Solution ##### I/O Ports - One PCMCIA type-III sockets - One RJ45 jack for Ethernet - One RJ11 56Kbps fax/modem - One DC-IN jack for AC adapter - One TV-out port - One external VGA port - One speaker/headphone-out jack support SPDIF - Line in - One Mic in - Four USB 2.0 port - One IEEE 1394 port - One PS/2 - One Serial Port - One Parallel Port - Two card reader port, MS and SD - Software/Firmware Support - Window XP Home and Windows XP Professional compliant, |
| --- | --- |