| Model Number | HD-620 | HD-620E |
| --- | --- | --- |
| Image System | NTSC/PAL |  |
| Resolution | Display 720x480/720x576 |  |
|  | Record 640x224/640x272 |  |
| Video input | BNCX4 |  |
| Video Looping output | BNCX4 |  |
| Display Frame | 4x30 fps/ 4x25 fps |  |
| Recording Frame Rate Quad | Max 30x4/25x4 |  |
| Recording Frame Rate MUX | Max 30/4 fps/Max 25/4 fps |  |
| Storage Media | Max 2 IDE Hard Disk (200gb max) |  |
| Image Format | M-JPEG |  |
| Compress Rate | Manual/Alarm/Schedule |  |
| Playback Speed | Frame by frame Forward Playback, Fast Forwardx2x4x6/Fast Reverse x6 |  |
| Menu Display | Time/Date/Setup/Menu |  |
| Alarm input | 4-input, Programmable |  |
| Relay Output | NO/NC Programmable Contact x1 |  |
| Password Control | One for HDD Format |  |
| Event List | 63 Events/Each HDD |  |
| Motion Detecion | Yes |  |
| Buzzer | Yes |  |
| Video Loss | Yes |  |
| Network Interface | TCP/IP Ethernet |  |
| Power Input | Ac 90-260 input (50/60hz) |  |
| Dimensions (W*H*D) | 420x44x300 |  |
| Rack Mountable | EIA 19” 1U |  |
| Operation Temperature | -10 – 40 degree C |  |

The unit is 4 cameras input appliance with multiple function which will bring you following features:

(1).    Device can be performed VHS system with live display, play back and video recording.

(2).    Support various type of camera with real, live mode display.

(3).    Multi speed recording election on normal recording mode or alarm recording mode, the highest speed of recording is 30/25(NTSC/PAL) fields in both mode.

(4).    Digitize data storage with M-Jpeg compression technology will give you organized video data management without using mess huge traditional video tape.

(5).    Device operates in hardware base with no OS (operating system) necessary for more reliability and stability.

(6).    Contrast, hue and brightness are adjustable for each camera individually.

(7).    Support up to 2 hard disk(HDD) from 40gb to 200gb compatible.

(8).    When external alarm is triggered, correspond cameras number will show up on the monitor with texts.

(9).   Motion detection function available, user can easily adjust sensitive of motion detection and setup buzzer alarm when motions are triggered.

(10).   Play back mode with pause(field by field) for forward.

(11).   Easy operation, setting can be easily, modified OSD (on  screen display) menu.

(12).   Device will overwrites data notice operator on monitor when the HDD is going full.

(13).   System will restore the previous setting and continue camera activity after restart.

(14)    Key lock function available to secure your DVR from button pressing accidentally.

(15).   Optional network function.