To: 	Dr. Ravi Pendse
Date: 	09/10/2001

Sir,

- We have completed Mobile IP Wireless setup scenario in Student’s Router’s Lab.

- We have also completed building the same scenario in the CARG Lab on ns-2 simulator.

- We are also attaching results from ns-2 and results that we got from Router’s Lab

Here is the scenario that we tested under varying network conditions to study the behavior of Wireless Network and Latency constraints.

MN

HA

FA1

FA2

TCP

Source

### Points to be noted

- Its not possible to have 100% one-to-one correspondence in between simulator setup and router’s lab setup

- Its very hard to find the parameters that match in both setups

- In Lab the setup being performed by changing the SSID of the bridges to simulate a situation where network boundaries are going to be changed as Mobile Node move from one point to another into different network territory.

#### Scenario #1

In this scenario we have tested our network with varying bandwidths and varying link delays and having maximum interface queue length to be 50.

| Bandwidth | Link Delay | MAX IF Queue | Average Delay Simulator (msec) | Average Delay LAB (msec) |
| --- | --- | --- | --- | --- |
| 1Mb | 2ms | 50 | 128.8176 | 94.0511 |
| 5.5MB | 2ms | 50 | 25.16622 | 30.4012 |
| 10MB | 2ms | 50 | 30.3097 | 26.5909 |
| 1Mb | 5ms | 50 | 98.5632 | 94.6467 |
| 5.5MB | 5ms | 50 | 18.533 | 30.4313 |
| 10MB | 5ms | 50 | 30.3652 | 26.8960 |
| 1Mb | 10ms | 50 | 121.9302 | 94.8725 |
| 5.5MB | 10ms | 50 | 23.0365 | 31.6187 |
| 10MB | 10ms | 50 | 30.3882 | 26.9467 |

#### Scenario #2

Again for this setup we have tested the network with varying bandwidths and varying link delays and having maximum interface queue length to be 100

| Bandwidth | Link Delay | MAX IF Queue | Average Delay Simulator (msec) | Average Delay LAB (msec) |
| --- | --- | --- | --- | --- |
| 1Mb | 2ms | 100 | 128.8176 | 94.8247 |
| 5.5MB | 2ms | 100 | 25.16622 | 30.7946 |
| 10MB | 2ms | 100 | 26.75297 | 25.3101 |
| 1Mb | 5ms | 100 | 98.5632 | 95.1362 |
| 5.5MB | 5ms | 100 | 18.6585 | 30.9367 |
| 10MB | 5ms | 100 | 30.3882 | 26.8788 |
| 1Mb | 10ms | 100 | 121.7713 | 95.7633 |
| 5.5MB | 10ms | 100 | 23.0365 | 30.5623 |
| 10MB | 10ms | 100 | 30.3631 | 25.9551 |

#### Scenario #3

Again for this setup we have tested the network with varying bandwidths and varying link delays and now the maximum interface queue length is changed to be 150

| Bandwidth | Link Delay | MAX IF Queue | Average Delay Simulator (msec) | Average Delay LAB (msec) |
| --- | --- | --- | --- | --- |
| 1Mb | 2ms | 150 | 131.4112 | 95.9831 |
| 5.5MB | 2ms | 150 | 25.16622 | 30.8187 |
| 10MB | 2ms | 150 | 26.1887 | 25.3142 |
| 1Mb | 5ms | 150 | 98.5632 | 95.3400 |
| 5.5MB | 5ms | 150 | 18.5531 | 31.27144 |
| 10MB | 5ms | 150 | 30.3631 | 27.5176 |
| 1Mb | 10ms | 150 | 121.0360 | 95.5662 |
| 5.5MB | 10ms | 150 | 23.0365 | 33.4698 |
| 10MB | 10ms | 150 | 30.4240 | 26.0430 |

Regards,

Muhammad Saqib Ilyas

Syed Shahzad Ali