Routing and Switching I

# Lab #10

R2

R3

In this lab, you will use RIP and IGRP together and get a chance to better understand the commands ‘distance’, ‘distribute-list’ and ‘offset-list’.

- Use a /24 mask on all the interfaces.

- Use the address range 130.10.0.0 for all the links.

Scenario

- Configure RIP and IGRP on all the routers.

- Create four loopbacks on R2.

## Constraints

- Loopback 0 of R2 should be seen by R3 as an IGRP route with an administrative distance of 100.

- Loopback 1 of R2 should be seen by R3 as a RIP route with an administrative distance of 120.

(Use distribute-list to accomplish this step.  Note.  There is more than one way to do this step.)

- Loopback 2 of R2 should be seen by R3 as a RIP route with an administrative distance of 120.

(Use distance command for this step.)

- Loopback 3 of R2 should not be reachable from R3.  (Use distance command or if possible offset-list command to do this.  Try all the various methods possible.  Do not use distribute-list command.)