Routing and Switching I

# Lab #3

R6

R1

e0    130.10.20.6/24

130.10.30.6/24

130.10.30.3/24

130.10.20.3/24

130.10.40.3/24

130.10.40.1/24

PC (H1)

130.10.10.1/24

Gateway 130.10.10.100

e1  130.10.10.100/24

R3

- Cable up the network as shown in the diagram.

- Assign the IP address as shown in the diagram to the appropriate interfaces.  For the serial links, a           has been used to indicate a DCE port.

- Issue these two commands on all the routers starting from the global config mode.

router rip

network 130.10.0.0

These two commands enable the routing protocol RIP.  For more information refer to Chapter 5 in Routing TCP/IP, Jeff Doyle or RFC 1058.

- Ping the host from R1.

R1#ping 130.10.10.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 130.10.10.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/3/4 ms

r2#

- There are two paths to reach H1 from R1.  R1-R3(serial)R6-H1 and R1-R3(ethernet)R6-H1.

How do we know which link was used by the ping packets?  This is where the extended features of ping come in handy.

R1#ping  <Hit an enter after typing ping>

R1#ping

Protocol [ip]:

Target IP address: 130.10.10.1

Repeat count [5]: 1000

Datagram size [100]:

Timeout in seconds [2]:

Extended commands [n]: y

Source address or interface:

Type of service [0]:

Set DF bit in IP header? [no]:

Validate reply data? [no]:

Data pattern [0xABCD]:

Loose, Strict, Record, Timestamp, Verbose[none]:

Sweep range of sizes [n]:

The router now allows you to set some extended options.

- No of packets sent .

This is a very useful feature often used in the lab to load a particular link.  In our research often times we have situations when we have to stress a link with constant load while checking out the behavior of the voice traffic.  If you end up working in this area, you will use this feature quite often.

If we want to send 1000 packets, then

R1#ping

Protocol [ip]:

Target IP address: 130.10.10.1

Repeat count [5]: 1000

Datagram size [100]:

Timeout in seconds [2]:

Extended commands [n]: n

.

.

.

- Suppose you are running an application that requires a fixed MTU of 437 bytes.  Additionally assume that in use of this application,  no IP packet fragmentation is allowed.  What this means is that from source to destination we need to determine if this MTU is supported by the network hardware.   Your job is  to find out if the network supports this packet size or not.  Wonderful Ping to the rescue.

r2#ping

Protocol [ip]:

Target IP address: 130.10.10.1

Repeat count [5]:

Datagram size [100]:437

Timeout in seconds [2]:

Extended commands [n]: n

.

.

.

.

- Since there are two paths from R1 to H1, how do we know which path was used? Use the Record option of the ping.

R1#p

Protocol [ip]:

Target IP address: 130.10.10.1

Repeat count [5]:

Datagram size [100]:

Timeout in seconds [2]:

Extended commands [n]: y

Source address or interface:

Type of service [0]:

Set DF bit in IP header? [no]:

Validate reply data? [no]:

Data pattern [0xABCD]:

Loose, Strict, Record, Timestamp, Verbose[none]: r

Number of hops [ 9 ]:

Loose, Strict, Record, Timestamp, Verbose[RV]:

Sweep range of sizes [n]:

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 130.10.10.1, timeout is 2 seconds:

Packet has IP options:  Total option bytes= 39, padded length=40

Record route: <*> 0.0.0.0 0.0.0.0 0.0.0.0 0.0.0.0

0.0.0.0 0.0.0.0 0.0.0.0 0.0.0.0 0.0.0.0

Reply to request 0 (12 ms).  Received packet has options

Total option bytes= 40, padded length=40

Record route: 130.10.40.2 130.10.30.3 130.10.10.100 130.10.10.1

130.10.30.4 130.10.40.3 130.10.40.2 <*> 0.0.0.0 0.0.0.0

End of list

Now we know the path taken by the packets.

- Create a loopback interface on R1.

r1#conf  t

r1(config)#int loopback 0

r1(config-if)#ip address 130.10.50.1 255.255.255.0

r1(config-if)#end

One of the troubleshooting tools is the debug command. Turn on IP packet debugging. This will show all the IP packets that are being processed by the router.

r1#debug ip packet

In order to stop the debug messages, use undebug ip packet or no debug ip packet

Open a DOS window on H1. Use the command

ping –n 50 130.10.50.1

“-n” option indicates the number of packets to be sent.

Now observe the debug messages on R1.

r1#

IP: s=130.10.10.1 (Ethernet0), d=130.10.50.1, len 60, rcvd 4

IP: s=130.10.50.1 (local), d=130.10.10.1 (Ethernet0), len 60, sending

IP: s=130.10.10.1 (Ethernet0), d=130.10.50.1, len 60, rcvd 4

IP: s=130.10.50.1 (local), d=130.10.10.1 (Ethernet0), len 60, sending

IP: s=130.10.10.1 (Ethernet0), d=130.10.50.1, len 60, rcvd 4

IP: s=130.10.50.1 (local), d=130.10.10.1 (Ethernet0), len 60, sending

IP: s=130.10.10.1 (Ethernet0), d=130.10.50.1, len 60, rcvd 4

IP: s=130.10.50.1 (local), d=130.10.10.1 (Ethernet0), len 60, sending

r1#

## Graded questions

- Which protocol does PING use?

- Which transport layer protocol does PING use?

- When you are pinging H1 from R1, what is the source IP address for the ping packets sent out from R1?

- While working on R1, how could you check if H1 can reach the loopback interface?  In other words, how can you verify if a ping from H1 to loopback of R1 is successful?

- Imagine a situation where in H1 is not able to respond to ping requests.  A hacker on the same LAN as of H1 has written an application that will respond to the ping requests on behalf of H1. The hackers machine is configured exactly the same way as H1 except that the IP address of the hacker machine is 130.10.10.4.  Will the command ping 130.10.10.1 on R1 indicate 100% success or will it fail?  Explain.

- What is “fast ping” and how do you use it on a Cisco router?