Lecture 13: October 03rd, 2000

Announcement: “ECE 764 – First test on October 19th, 2000”

- Routing Information Protocol (RIP)

- Configuring RIP

www.cisco.com/univercd/cc/td/doc/product/software/ios120/12cgcr/np1_c/1cprt1/1crip.htm

RIP commands: (for your information)

- Show ip route rip

- Auto-summary (press link to access web page)

- Default-information originate

- Default-metric

- Ip rip authentication key-chain

- Ip rip authentication mode

- Ip rip receive version

- Ip rip send version

- Ip split-horizon

- Neighbor (IGRP and RIP)

- Network (RIP)

- Offset-list

- Output-delay

- Router rip

- Timers basic

- Validate-update-source

- Version

Links for IP CLASSLESS:

http://www.cisco.com/warp/public/105/21.html#2

### Adjusting the Administrative Distance: The default administrative distance might not always be right for your network—you may want to adjust them so RIP routes are preferred over IGRP routes, for instance. Before explaining how to adjust the administrative distances, we need to look at the implications of changing the administrative distance. Changing the administrative distance on routing protocols can be very dangerous! Changing the default distances can actually lead to routing loops and other oddities in your network. We recommend you change administrative distance with caution, and only after you have thought through what you want to achieve, and all the consequences of your actions. For entire protocols, changing the distance is relatively easy—simply configure the distance using the distance command in the routing process sub configuration mode. You can also change the distance for routes learned from one source only in some protocols, and you can change the distance on just some routes. For static routes, you can change the distance of each route by entering a distance after the ip route command:

### ip route <network> <subnet mask> <next hop> <distance>

You can't change the administrative distance for all the static routes at once.

Default Administrative Distances for various protocols:

| ## Default Administrative Distances |  |
| --- | --- |
| Connected | 0 |
| Static | 1 |
| eBGP | 20 |
| EIGRP (internal) | 90 |
| IGRP | 100 |
| OSPF | 110 |
| IS-IS | 115 |
| RIP | 120 |
| EIGRP (external) | 170 |
| iBGP | 200 |
| EIGRP summary route | 5 |