## Routing and Switching I
Lab # 6
Due Date Oct. 26 2000

## Syed Shahzad Ali

## Owais Khan Humanyon
Zeeshan Gaya

## For Router R1

## en

## conf t

## hostname R1

## interface Serial0

## ip address 140.10.2.1 255.255.255.0

## no shut

## interface TokenRing0

## ip address 140.10.1.1 255.255.255.0

## ring-speed 4

## no shut

## exit

## router rip

## network 140.10.1.0

## network 140.10.2.0

## end

## wr m

## sh run

## sh ip route

## For R2

## en

## conf t

## hostname R2

## int s1

## ip address 140.10.2.2 255.255.255.0

## clock rate 4000000

## no shut

## int e0

## ip address 160.10.1.1 255.255.255.0

## no shut

## int s0

## ip add 160.10.2.1 255.255.255.0

## no shut

## exit

## router rip

## network 160.10.2.0

## network 160.10.1.0

## network 140.10.2.0

## end

## wr m

## sh run

## For R6

## en

## conf t

## hostname R6

## int s0

## ip address 160.10.2.2 255.255.255.0

## clock rate 4000000

## no shut

## int s1

## ip add 160.10.3.2 255.255.255.0

## clock rate 4000000

## no shut

## int e0

## ip add 160.10.1.2 255.255.255.0

## no shut

## int e1

## ip add 150.10.1.2 255.255.255.0

## no shut

## exit

## router rip

## network 160.10.1.0

## network 160.10.2.0

## network 160.10.3.0

## network 150.10.1.0

## end

## wr m

## For R3

## en

## conf t

## hostname R3

## int s0

## ip add 160.10.3.1 255.255.255.0

## no shut

## int s1

## ip add 160.10.4.1 255.255.255.0

## clock rate 4000000

## no shut

## exit

## router rip

## network 160.10.4.0

## network 160.10.3.0

## end

## wr m

## sh run

## Router R5

## en

## conf t

## hostname R5

## int s0

## ip add 160.10.4.2 255.255.255.0

## no shut

## int s1

## ip add 150.10.2.2 255.255.255.0

## clock rate 4000000

## no shut

## int e0

## ip add 130.10.10.100 255.255.255.0

## no shut

## exit

## router rip

## network 130.10.10.0

## network 150.10.2.0

## network 160.10.4.0

## end

## wr m

## sh run

## Router R4

## en

## conf t

## hostname R4

## int s0

## ip add 150.10.2.1 255.255.255.0

## no shut

## int e0

## ip address 150.10.1.1 255.255.255.0

## no shut

## exit

## router rip

## network 150.10.1.0

## network 150.10.2.0

## end

## wr m

## sh run

## Graded Questions

- Suppose R4’s ethernet had the ip address 160.10.10.191/26 and R6’s ethernet link to R4 had the ip address 160.10.10.193/26, would your routing table change?  Justify.

- If there is a problem in the previous question, how many different ways can you think of to solve it?  If there is no problem ignore this question.  (You will get full points).

- In step 11, we know that RIP does not support VLSM.  For a poorly designed network, how many ways can you think of to solve this problem?