THE GBCamera Printing Output format

Information gathered by Martin Eyre Email: MartinEyre@crispgroup.co.uk

Reading data from the GBCamera

The following document contains information on how to make your Pc emulate the Gameboy printer thus allowing you to download gameboy camera pictures.

The information has been gained by trial and error and even though I've succesfully managed to download data I don't garentee all the information is correct.

A Gameboy connector cable (Gameboy to Parrallel port)

I constructed a cable a bit like the one used with GBBasic execpt I joined the clk to pin 13 instead of pin 2, this allows the PC to read the CLK instead of sending it.

Note : When constructing a cable I noticed the colours of wires differed to what was outlined in a diagram describing the GBBasic cable.

Reading the Data

Data is sent from the GBCamera down the serial port one bit at a time, we know when a bit has arrived because the CLK goes low, we then read the data from pin 12.

Note: I always ignore the 1st bit of data sent I think this may be a start data transfer bit.

The GBCamera will send 10 bytes of data in the following format :-

Dec	Bin		Byte

136 	10001000 	001

051 	00110011 	002

001 	00000001 	003

000 	00000000 	004

000 	00000000 	005

000 	00000000 	006

001 	00000001 	007

000 	00000000 	008

000 	00000000 	009  <------------- GBCamera expects data of 129 here

000 	00000000 	010                   and zero here

The GBCamera will expect the GBPrinter to send back 129 on byte 9 and 0 on byte 10 this will then cause the GBCamera to send the rest of its data.

Do I need to send any other messages back to GBCamera ?

I've succsesfully read all the data by only sending the 2 mesaages as above, but I know that the GBPrinter sends back many other messages on headers as above and on the footers :-

Footer message example :-

Dec	Bin		Byte

139 	10001011 	001

053 	00110101	002

000 	00000000 	003 <------GBCamera expects data of 129 here

000 	00000000 	004 	   and the 8 followed by zero

How is the data constructed ?

What I do is look at the first 2 bytes sent and if they are 136 and 051 then I know this is header information.  The 3rd byte tells me what sort of header and how many bytes to read in this block :-

Header Type	Description		Total Bytes to Read

(3rd Byte)

001		unknown		10

002		unknown		10

015		unknown		10

004		data Length info		6

if the 3rd byte is none of the above then I presume its a footer which is always 4 bytes long.

As you can see I dont know many of the header types but I do know the important one of length information which is described as follows :-

Dec	Bin		Byte number

136 	10001000 	001

051	00110011 	002	first 2 bytes tell me its a header

004	00000100 	003	3rd tells me it contains length information about the data to follow

000 	00000000 	004

128 	10000000 	005	following 2bytes are the length of the data to follow

002 	00000010 	006	(2 * 256) + 128 = 640

After the length information always follows the data information normally always 640 bytes, this data is the photo data and is used to construct the photo.

Photo Data ?

The photo data block of 640 bytes makes up one row which is 8 bytes deep of the Photo.

(2 * 8) * 40 (per row) = 640

So when the data is read is constructed as follows on the paper/screen

00 01  16 17

02 03  18 19

04 05  20 21

06 07  22 23

08 09  24 25

10 11  26 26

12 13  .......

14 15

Afterr the data a footer always follow of 4 bytes.

When to end reading

At the moment I dont know when to end reading the GBCamera so I wait for it to stop sending data and I time out.

Software

I've written some software which I've succesfully got working on my PC but experienced problems on others.

How to use it :-

Copy the 2 files into any directory :

PIC.PCX

READP.EXE

Make sure mouse drive is load before starting.

Run readp.exe click on OK and the mouse will disappear and the PC will be waiting for data.

Put the GBCamera into printmode and press A

The GBCamera will down load to the PC and the %Bar will rise.

(if for any reason the %Bar doesn't rise and GBCamera Errors - pressing SPACEBAR will stop reading the parallel port and the mouse returns)

When the PC timesout the Photo will be displayed, and the mouse returns.

SAVE will save all the data read to a text file (data.out).

QUIT will quit to dos.

Its a DOS package written in C and in a dos shell from windows dosn't seam to work. (not sure why)

I needed to start my PC in DOS Mode.