| 4.19 Design Aspect of Vegetative Engineering Structures |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Table |  |  |  |  |  |  |  |  |  |  |
| System |  | Function |  | Method of operation |  | Application and Site Requirements |  |  | Maturity | Limitation |
| Horizontal line grass planting |  | Catches,reinforces, supports |  | Dense line retards suface water flow |  | Dry,slope <45° erodible, cut slopes |  |  | 2 Seasons | Thin line easily broken |
| Diagonal line grass plantation |  | Catches,reinforces, supports |  | Dense line guides suface water flow |  | Wet permeable, fine, cut slopes |  |  | 2 Seasons | Rills break through |
| Grass seeding |  | Catches,reinforces, supports |  | Dense grass, mat, rooting system |  | Consolidated debris slopes, <45° |  |  | 3 Seasons | Can cause liquefaction young plants get wahed away or dried |
| Palisades |  | Catches,reinforces, supports |  | Desnse line above and below the ground retards surface and shallow ground water flow |  | slope <35°, dry, erodible and consolidated debris |  |  | 2 Seasons | Causes small slumps, requires many cuttings, high mortality |
| Brush layering |  | catches,reinforces, drains |  | Dense line, strong buried branches retards surface and shallow ground water flow |  | slope <45°, dry, erodible and consolidated debris |  |  | One season if planted early and watered | Destructive to slopes during the excation, required many cuttings |
| Fascines |  | catches,reinforces, drains |  | Woody bundle, dense stems, porous, can drains soil if laid down slope |  | Consolidated debris slopes, <45° |  |  | 3 Seasons | Destructive to slopes, requires many cuttings, slow to develop,high mortality |
| Shrub Planting |  | Transpires, catches, armours, reinforces, anchors,supports |  | Bunchy leaves, multiple stems, lateral roots, root cylinder, tap roots |  | Any slopes <45° |  |  | At least 4 seasons |  |
| Tree Planting |  | Transpires, armours, reinforces, anchors,supports |  | Lateral and near vertical rootng systems, root cylinder |  | Any debris slopes <45° gully side slopes |  |  | At least 4 seasons | Top heavy on steep slopes, leaf drip, canopy shadows smaller plants |
| Bamboo Planting |  | Transpires, catches, armours, reinforces, supports |  | Dense poles, massive rooting systems, dense leaves,grows all year |  | Slopes <35°, base of slope, erodible slopes, preferably wet places |  |  | At least 5 seasons | Source plant damage, delicate, requires nursery space, heavy to transport |