CWTS TSM 03.20 V3.1.0 (2002-12)

China Wireless Telecommunication Standard;

3G digital cellular telecommunications system;

Security related network functions

(Release 3)

TSM

Contents

Foreword	4

0	Scope	5

0.1	Normative references	5

0.2	Definitions and abbreviations	5

1	General	6

2	Subscriber identity confidentiality	7

2.1	Generality	7

2.2	Identifying method	7

2.3	Procedures	8

2.3.1	Location updating in the same MSC area	8

2.3.2	Location updating in a new MSCs area, within the same VLR area	9

2.3.3	Location updating in a new VLR; old VLR reachable	10

2.3.4	Location Updating in a new VLR; old VLR not reachable	11

2.3.5	Reallocation of a new TMSI	12

2.3.6	Local TMSI unknown	13

2.3.7	Location updating in a new VLR in case of a loss of information	14

2.3.8	Unsuccessful TMSI allocation	14

2.3.9	Combined location area updating with the routing area updating	15

3	Subscriber identity authentication	17

3.1	Generality	17

3.2	The authentication procedure	17

3.3	Subscriber Authentication Key management	18

3.3.1	General authentication procedure	18

3.3.2	Authentication at location updating in a new VLR, using TMSI	19

3.3.3	Authentication at location updating in a new VLR, using IMSI	20

3.3.4	Authentication at location updating in a new VLR, using TMSI, TMSI unknown in "old" VLR	21

3.3.5	Authentication at location updating in a new VLR, using TMSI, old VLR not reachable	22

3.3.6	Authentication with IMSI if authentication with TMSI fails	22

3.3.7	Re-use of security related information in failure situations	23

4	Confidentiality of signalling information elements, connectionless data and user information elements on physical connections	24

4.1	Generality	24

4.2	The ciphering method	24

4.3	Key setting	25

4.4	Ciphering key sequence number	25

4.5	Starting of the ciphering and deciphering processes	26

4.6	Synchronisation	26

4.7	Handover	27

4.8	Negotiation of A5 algorithm	27

5	Synthetic summary	28

Annex A (normative): External specifications of security related algorithms	29

A.0	Scope	29

A.1	Specifications for Algorithm A5	29

A.1.1	Purpose	29

A.1.2	Implementation indications	29

A.1.3	External specifications of Algorithm A5	30

A.1.4	Internal specification of Algorithm A5	31

A.2	Algorithm A3	32

A.2.1	Purpose	32

A.2.2	Implementation and operational requirements	32

A.3	Algorithm A8	33

A.3.1	Purpose	33

A.3.2	Implementation and operational requirements	33

Annex B (normative): Security related network functions for TD-SCDMA Packet Radio Service	34

B.1	General	34

B.2	Subscriber identity confidentiality	34

B.2.1	Generality	34

B.2.2	Identifying method	35

B.2.3	Procedures	35

B.2.3.1	Routing area updating in the same SGSN area	35

B.2.3.2	Routing area updating in a new SGSN; old SGSN reachable	35

B.2.3.3	Routing area updating in a new SGSN; old SGSN not reachable	36

B.2.3.4	Reallocation of a TLLI	38

B.2.3.5	Local TLLI unknown	39

B.2.3.6	Routing area updating in a new SGSN in case of a loss of information	40

B.2.3.7	Unsuccessful TLLI allocation	40

B.3	Subscriber identity authentication	40

B.3.1	Generality	40

B.3.2	The authentication procedure	41

B.3.3	Subscriber Authentication Key management	41

B.3.3.1	General authentication procedure	41

B.3.3.2	Authentication at routing area updating in a new SGSN, using TLLI	42

B.3.3.3	Authentication at routing area updating in a new SGSN, using IMSI	43

B.3.3.4	Authentication at routing area updating in a new SGSN, using TLLI, TLLI unknown in 'old' SGSN	43

B.3.3.5	Authentication at routing area updating in a new SGSN, using TLLI, old SGSN not reachable	45

B.3.3.6	Authentication with IMSI if authentication with TLLI fails	45

B.3.3.7	Re-use of security related information in failure situations	46

B.4	Confidentiality of user information and signalling between UE and SGSN	47

B.4.1	Generality	47

B.4.2	The ciphering method	47

B.4.3	Key setting	47

B.4.4	Ciphering key sequence number	48

B.4.5	Starting of the ciphering and deciphering processes	48

B.4.6	Synchronisation	48

B.4.7	Inter SGSN routing area update	49

B.4.8	Negotiation of GPRS-A5 algorithm	49

B.5	Synthetic summary	50

B.6	Security of the GPRS backbone	50

Annex C (informative): Security issues related to signalling schemes and key management	51

C.1	Introduction	51

C.2	Short description of the schemes	51

C.3	List of abbreviations	52

Annex D (informative): Security information to be stored in the entities of the TD-SCDMA system	66

D.1	Introduction	66

D.2	Entities and security information	66

D.2.1	Home Location Register (HLR)	66

D.2.2	Visitor Location Register (VLR)	66

D.2.3	Mobile services Switching Centre (MSC)/Base Station System (BSS)	66

D.2.4	User Equipment (UE)	67

D.2.5	Authentication Centre (AuC)	67

History	68

# Foreword

This Technical Specification (TS) has been produced by the China Wireless Telecommunication Standard (CWTS).

The contents of the present document are subject to continuing work within CWTS and may change following formal CWTS approval. Should CWTS modify the contents of the present document, it will then be re-released by CWTS with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to CWTS for information;

2	presented to CWTS for approval;

3	or greater indicates CWTS approved release.

y	the second digit is incremented for changes of substance, i.e. technical enhancements, corrections, updates, etc. after approval by CWTS of new version within a given release.

z	the third digit is incremented when changes have been incorporated in the document prior approval by CWTS.

# Scope

This Technical Specification specifies the network functions needed to provide the security related service and functions specified in GSM 02.09 [3].

This specification does not address the cryptological algorithms that are needed to provide different security related features. This topic is addressed in Annex A. Wherever a cryptological algorithm or mechanism is needed, this is signalled with a reference to Annex A. The references describe only the functionality, but not how the cryptological streams are obtained.

NOTE:	some algorithms may be identical or may require the use of a common hardware.

## 0.1	Normative references

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific;

- For a specific reference, subsequent revisions do not apply;

- For a non-specific reference, the latest version applies;

- For this Release 3 document, references to TSM documents are for Release 3 versions (3.y.z).

- References to GSM documents are for Release 1999 versions unless otherwise specified.

- References to 3GPP documents are for Release 1999 versions.

[1]	TSM 01.04: "Abbreviations and acronyms"

[2]	GSM 01.61: "General Packet Radio Service (GPRS); GPRS ciphering algorithm requirements".

[3]	GSM 02.09: "Security aspects".

[4]	GSM 02.17: "Subscriber Identity Modules (SIM) Functional characteristics"

[5] 	3GPP TS 22.060: "General Packet Radio Service (GPRS); Service description; Stage 1"

[6]	3GPP TS 23.003: "Numbering, addressing and identification"

[7]	TSM 03.60: "TD-SCDMA Packet Radio Service (TD-PRS); Service description Stage 2"

[8]	3GPP TS 24.008: "Mobile radio interface layer 3 specification; Core Network Protocols - Stage 3"

[9]	TSM 04.08: "Mobile radio interface layer 3 specification; Part 1: Radio Resource Management"

[10] 	GSM 04.64: "General Packet Radio Service (GPRS); Logical Link Control (LLC)"

[11]	TSM 05.02: "Multiplexing and multiple access on the radio path"

[12]	TSM 05.03: "Channel coding"

[13]	3GPP TS 29.002: "Mobile Application Part (MAP) specification"

14	GSM 11.11: "Specification of the Subscriber Identity Module- Mobile Equipment (SIM-ME) interface"

## 0.2	Definitions and abbreviations

Abbreviations used in the present document are listed in TSM 01.04 [1].

Specific abbreviations used in Annex C are listed in sub-clause C.3.

# General

The different security related services and functions that are listed in GSM 02.09 [3] are grouped as follows:

-	Subscriber identity confidentiality;

-	Subscriber identity authentication;

-	Signalling information element and connectionless user data confidentiality and data confidentiality for physical connections (ciphering).

It shall be possible to introduce new authentication and ciphering algorithms during the systems lifetime. The fixed network may support more than one authentication and ciphering algorithm.

The security procedures include mechanisms to enable recovery in event of signalling failures. These recovery procedures are designed to minimise the risk of a breach in the security of the system.

General on figures in this specification:

-	In the figures below, signalling exchanges are referred to by functional names. The exact messages and message types are specified in TS 24.008 [8], TSM 04.08 [9] and TS 29.002 [13].

-	No assumptions are made for function splitting between MSC (Mobile Switching Centre), VLR (Visitor Location Register) and BSS (Base Station System). Signalling is described directly between UE and the local network (i.e. BSS, MSC and VLR denoted in the figures by BSS/MSC/VLR). The splitting in Annex C is given only for illustrative purposes.

-	Addressing fields are not given; all information relates to the signalling layer. The TMSI allows addressing schemes without IMSI, but the actual implementation is specified in the GSM 04-series.

-	The term HPLMN in the figures below is used as a general term which should be understood as HLR (Home Location Register) or AuC (Authentication Centre).

-	What is put in a box is not part of the described procedure but it is relevant to the understanding of the figure.

# 2	Subscriber identity confidentiality

## 2.1	Generality

The purpose of this function is to avoid the possibility for an intruder to identify which subscriber is using a given resource on the radio path (e.g. TCH (Traffic CHannel) or signalling channel) by listening to the signalling exchanges on the radio path. This allows both a high level of confidentiality for user data and signalling and protection against the tracing of a user's location.

The provision of this function implies that the IMSI (International Mobile Subscriber Identity), or any information allowing a listener to derive the IMSI easily, should not normally be transmitted in clear text in any signalling message on the radio path.

Consequently, to obtain the required level of protection, it is necessary that:

-	a protected identifying method is normally used instead of the IMSI on the radio path; and

-	the IMSI is not normally used as addressing means on the radio path (see GSM 02.09 [3]);

-	when the signalling procedures permit it, signalling information elements that convey information about the mobile subscriber identity must be ciphered for transmission on the radio path.

The identifying method is specified in the following sub-clause. The ciphering of communication over the radio path is specified in clause 4.

## 2.2	Identifying method

The means used to identify a mobile subscriber on the radio path consists of a TMSI (Temporary Mobile Subscriber Identity). This TMSI is a local number, having a meaning only in a given location area; the TMSI must be accompanied by the LAI (Location Area Identity) to avoid ambiguities. The maximum length and guidance for defining the format of a TMSI are specified in TS 23.003 [6].

The network (e.g. a VLR) manages suitable data bases to keep the relation between TMSIs and IMSIs. When a TMSI is received with an LAI that does not correspond to the current VLR, the IMSI of the UE must be requested from the VLR in charge of the indicated location area if its address is known; otherwise the IMSI is requested from the UE.

A new TMSI must be allocated at least in each location updating procedure. The allocation of a new TMSI corresponds implicitly for the UE to the de-allocation of the previous one. In the fixed part of the network, the cancellation of the record for a UE in a VLR implies the de-allocation of the corresponding TMSI.

To cope with some malfunctioning, e.g. arising from a software failure, the fixed part of the network can require the identification of the UE in clear. This procedure is a breach in the provision of the service, and should be used only when necessary.

When a new TMSI is allocated to a UE, it is transmitted to the UE in a ciphered mode. This ciphered mode is the same as defined in clause 4.

The UE must store its current TMSI in a non-volatile memory, together with the LAI, so that these data are not lost when the UE is switched off.

## 2.3	Procedures

This sub-clause presents the procedures, or elements of procedures, pertaining to the management of TMSIs.

### 2.3.1	Location updating in the same MSC area

This procedure is part of the location updating procedure, which takes place when the original location area and the new location area depend on the same MSC. The part of this procedure relative to TMSI management is reduced to a TMSI re-allocation (from TMSIo with "o" for "old" to TMSIn with "n" for "new").

The UE sends TMSIo as an identifying field at the beginning of the location updating procedure.

The procedure is schematised in figure 2.1.

UE

Figure 2.1: Location updating in the same MSC area

Signalling functionality:

Management of means for new ciphering:

The UE and BSS/MSC/VLR agree on means for ciphering signalling information elements, in particular to transmit TMSIn.

### 2.3.2	Location updating in a new MSCs area, within the same VLR area

This procedure is part of the location updating procedure, which takes place when the original location area and the new location area depend on different MSCs, but on the same VLR.

The procedure is schematised on figure 2.2.

UE

NOTE:	From a security point of view, the order of the procedures is irrelevant.

Figure 2.2: Location updating in a new MSCs area, within the same VLR area

Signalling functionality:

Loc.Updating:

stands for Location Updating

The BSS/MSC/VLR indicates that the location of the UE must be updated.

### 2.3.3	Location updating in a new VLR; old VLR reachable

This procedure is part of the normal location updating procedure, using TMSI and LAI, when the original location area and the new location area depend on different VLRs.

The UE is still registered in VLRo ("o" for old or original) and requests registration in VLRn ("n" for new). LAI and TMSIo are sent by UE as identifying fields during the location updating procedure.

The procedure is schematised in figure 2.3.

UE

NOTE:	From a security point of view, the order of the procedures is irrelevant.

Figure 2.3: Location updating in a new VLR; old VLR reachable

Signalling functionality:

Sec.Rel.Info.:

Stands for Security Related information

The MSC/VLRn needs some information for authentication and ciphering; this information is obtained from MSC/VLRo.

Cancellation:

The HLR indicates to VLRo that the UE is now under control of another VLR. The "old" TMSI is free for allocation.

### 2.3.4	Location Updating in a new VLR; old VLR not reachable

This variant of the procedure in sub-clause 2.3.3 arises when the VLR receiving the LAI and TMSIo cannot identify the VLRo. In that case the relation between TMSIo and IMSI is lost, and the identification of the UE in clear is necessary.

The procedure is schematised in figure 2.4

UE

NOTE:	From a security point of view, the order of the procedures is irrelevant.

Figure 2.4: Location Updating in a new VLR: old VLR not reachable

### 2.3.5	Reallocation of a new TMSI

This function can be initiated by the network whenever a radio connection exists. The procedure can be included in other procedures, e.g. through the means of optional parameters. The execution of this function is left to the network operator.

When a new TMSI is allocated to a UE, the network must prevent the old TMSI from being allocated again until the UE has acknowledged the allocation of the new TMSI.

If an IMSI record is deleted in the VLR by O&M action, the network must prevent any TMSI associated with the deleted IMSI record from being allocated again until a new TMSI is successfully allocated to that IMSI.

If an IMSI record is deleted in the HLR by O&M action, it is not possible to prevent any TMSI associated with the IMSI record from being allocated again. However, if the UE whose IMSI record was deleted should attempt to access the network using the TMSI after the TMSI has been allocated to a different IMSI, then authentication or ciphering of the UE whose IMSI was deleted will almost certainly fail, which will cause the TMSI to be deleted from the UE.

The case where allocation of a new TMSI is unsuccessful is described in sub-clause 2.3.8.

The procedure for the reallocation of a new TMSI is schematised in figure 2.5.

UE

Figure 2.5: Reallocation of a new TMSI

### 2.3.6	Local TMSI unknown

This procedure is a variant of the procedure described in sub-clauses 2.3.1 and 2.3.2, and happens when a data loss has occurred in a VLR and when a UE uses an unknown TMSI, e.g. for a communication request or for a location updating request in a location area managed by the same VLR.

This procedure is schematised in figure 2.6.

UE

NOTE:	Any message in which TMSIo is used as an identifying means in a location area managed by the same VLR.

Figure 2.6: Location updating in the same MSC area; local TMSI unknown

### 2.3.7	Location updating in a new VLR in case of a loss of information

This variant of the procedure described in 2.3.3 arises when the VLR in charge of the UE has suffered a loss of data. In that case the relation between TMSIo and IMSI is lost, and the identification of the UE in clear is necessary.

The procedure is schematised in figure 2.7.

UE

NOTE:	From a security point of view, the order of the procedures is irrelevant.

Figure 2.7: Location updating in a new VLR in case of a loss of information

### 2.3.8	Unsuccessful TMSI allocation

If the UE does not acknowledge the allocation of a new TMSI, the network shall maintain the association between the old TMSI and the IMSI and between the new TMSI and the IMSI.

For a UE-originated transaction, the network shall allow the UE to identify itself by either the old TMSI or the new TMSI. This will allow the network to determine the TMSI stored in the UE; the association between the other TMSI and the IMSI shall then be deleted, to allow the unused TMSI to be allocated to another UE.

For a network-originated transaction, the network shall identify the UE by its IMSI. When radio contact has been established, the network shall instruct the UE to delete any stored TMSI. When the UE has acknowledged this instruction, the network shall delete the association between the IMSI of the UE and any TMSI; this will allow the released TMSIs to be allocated to another UE.

In either of the cases above, the network may initiate the normal TMSI reallocation procedure.

Repeated failure of TMSI reallocation (passing a limit set by the operator) may be reported for O&M action.

### 2.3.9	Combined location area updating with the routing area updating

This sub-clause is only applicable if TD-PRS is supported.

This procedure is part of the location updating of a TD-SCDMA Packet Radio Service (TD-PRS) class A or B mobile when the Gs-interface (SGSN MSC/VLR signalling interface) is implemented. This procedure is not relevant if the Gs-interface is not implemented.

The location area updating procedure and the routing area updating procedure are combined to one UE Serving GPRS Support Node (SGSN) procedure. The UE includes a Location Area Update (LAU) indication in the Routing Area Update Request message. The SGSN performs the location updating towards the VLR on behalf of the UE.

The procedure described in figure 2.8 shows only the interaction between the SGSN and the VLR. The full procedure including the update to other network elements (e.g. HLR, old MSC/VLR) is described in TSM 03.60 [7].

┌─────┐ ┌─────────────┐ ┌────────────┐         ┌────────┐   
 │ UE  │ │     BSS     │ │    SGSN    │         │   VLR  │   
 └──┬──┘ └─────────────┘ └──────┬┬────┘         └────┬───┘á  
    │                           ││                   │    á  
    │ RAI, TLLI, LAU indication ││                   │       
    ├──────────────────────────>┤│                   │       
    │   (note 1)                ││                   │       
    │                      ┌────────────┐            │       
    │                      │  Security  │á           │       
    │                      │  functions │            │       
    │                      └────────────┘            │    á  
    │                           ││                   │       
    │                           ││ IMSI,LAI (note 2) │       
    │                           │├──────────────────>┤       
    │                           ││             ┌────────────┐
    │                           ││             │ Allocation │
    │                           ││             │  of TMSIn  │
    │                           ││             └────────────┘
    │                           ││                   │       
    │                           ││    TMSIn (note 3) │       
    │                           │├<──────────────────┤       
    │                           ││                   │   á   
    │                           ││                   │       
    │    Cipher(TMSIn)(note 4)  ││                   │       
    ├<──────────────────────────┤│                   │á      
    │                           ││                   │  á    
    │                           ││                   │  á    
    │    Acknowledge (note 5)   ││Acknowledge(note 6)│  á    
    ├──────────────────────────>┤├─────────────────>─┤  á    
    │                                         ┌────────────┐ 
    │                                         │Deallocation│á
    │                                         │of TMSIo    │ 
    │                                         └────────────┘

NOTE 1:	The Routing Area Update Request message including the old Routing Area Identifier (RAI), the Temporary Logical Link Identifier (TLLI), and an indication that a combined Location Area Update (LAU) is performed.

NOTE 2:	Location Updating message.

NOTE 3:	Location Updating Accept message including the new TMSI.

NOTE 4:	Routing Area Update Accept message including the new TMSI and the new TLLI (if any).

NOTE 5:	Routing Area Update Complete message including the TLLI and TMSI.

NOTE 6:	TMSI Reallocation Complete message including the TMSI.

Figure 2.8: Combined routing area and location updating in the same VLR

When the VLR does not change the TMSI, the old TMSI will stay in use and there is no need to send any TMSI to the UE.

In case of combined routing area update and inter-VLR location area updating procedure, the old TMSI will be cancelled and the HLR is updated as described in TSM 03.60 [7].

If the Location Updating message indicates a reject (if for example the UE try to enter a forbidden location area), then this should be indicated to the UE and the UE shall not access non-TD-PRS service until a successful Location Update is performed.

For the combined location and routing area update and the combined GPRS Attach and IMSI Attach for TD-PRS class A and B mobiles, the authentication is performed by the SGSN. The authentication procedure for TD-PRS is described in Annex B. The MSC/VLR relies on the SGSN authentication. This authentication procedure generates no ciphering key for circuit switched ciphering.

The ciphering key for circuit switched operation is allocated through an authentication by MSC/VLR when the circuit switched service is requested. Also, the MSC/VLR may use the old ciphering key if existing.

# 3	Subscriber identity authentication

## 3.1	Generality

The definition and operational requirements of subscriber identity authentication are given in GSM 02.09 [3].

The authentication procedure will also be used to set the ciphering key (see clause 4). Therefore, it is performed after the subscriber identity (TMSI/IMSI) is known by the network and before the channel is encrypted.

Two network functions are necessary: the authentication procedure itself, and the key management inside the fixed subsystem.

## 3.2	The authentication procedure

The authentication procedure consists of the following exchange between the fixed subsystem and the UE.

-	The fixed subsystem transmits a non-predictable number RAND to the UE.

-	The UE computes the signature of RAND, say SRES, using algorithm A3 and some secret information: the Individual Subscriber Authentication Key, denoted below by Ki.

-	The UE transmits the signature SRES to the fixed subsystem.

-	The fixed subsystem tests SRES for validity.

The general procedure is schematised in figure 3.1.

UE

NOTE:	IMSI is used to retrieve Ki in the network.

Figure 3.1: The authentication procedure

Authentication algorithm A3 is specified in Annex A.

## 3.3	Subscriber Authentication Key management

The Subscriber Authentication Key Ki is allocated, together with the IMSI, at subscription time.

Ki is stored on the network side in the Home Public Land Mobile Network (HPLMN), in an Authentication Centre (AuC). A PLMN may contain one or more AuC. An AuC can be physically integrated with other functions, e.g. in a Home Location Register (HLR).

### 3.3.1	General authentication procedure

When needed for each UE, the BSS/MSC/VLR requests security related information from the HLR/AuC corresponding to the UE. This includes an array of pairs of corresponding RAND and SRES. These pairs are obtained by applying Algorithm A3 to each RAND and the key Ki as shown in figure 3.1. The pairs are stored in the VLR as part of the security related information.

The procedure used for updating the vectors RAND/SRES is schematised in figure 3.2.

NOTE:	The Authentication Vector Response contains also Kc(1..n), which is not shown in this and the following figures. For discussion of Kc see clause 4.

Figure 3.2: Procedure for updating the vectors RAND/SRES

When an MSC/VLR performs an authentication, including the case of a location updating within the same VLR area, it chooses a RAND value in the array corresponding to the UE. It then tests the answer from the UE by comparing it with the corresponding SRES, as schematised in figure 3.3.

UE

Figure 3.3: General authentication procedure

### 3.3.2	Authentication at location updating in a new VLR, using TMSI

During location updating in a new VLR (VLRn), the procedure to get pairs for subsequent authentication may differ from that described in the previous sub-clause. In the case when identification is done using TMSI, pairs for authentication as part of security related information is given by the old VLR (VLRo). The old VLR shall send to the new VLR only those pairs, which have not been used.

The procedure is schematised in figure 3.4.

UE

Figure 3.4: Authentication at location updating in a new VLR, using TMSI

### 3.3.3	Authentication at location updating in a new VLR, using IMSI

When the IMSI is used for identification, or more generally when the old VLR is not reachable, the procedure described in sub-clause 3.3.2 cannot be used. Instead, pairs of RAND/SRES contained in the security related information is requested directly from the HPLMN.

The procedure is schematised in figure 3.5.

UE

Figure 3.5: Authentication at location updating in a new VLR, using IMSI

### 3.3.4	Authentication at location updating in a new VLR, using TMSI, TMSI unknown in "old" VLR

This case is an abnormal one, when a data loss has occurred in the "old" VLR.

The procedure is schematised in figure 3.6.

UE

Figure 3.6: Authentication at location updating in a new VLR, using TMSI,
TMSI unknown in "old" VLR

### 3.3.5	Authentication at location updating in a new VLR, using TMSI, old VLR not reachable

The case occurs when an old VLR cannot be reached by the new VLR.

The procedure is schematised in figure 3.7.

UE

Figure 3.7: Authentication at location updating in a new VLR, using TMSI, old VLR not reachable

### 3.3.6	Authentication with IMSI if authentication with TMSI fails

If authentication of a UE, which identifies itself with a TMSI is unsuccessful, the network requests the IMSI from the UE, and repeats the authentication using the IMSI. Optionally, if authentication using the TMSI fails, the network may reject the access request or location registration request, which triggered the authentication.

### 3.3.7	Re-use of security related information in failure situations

Security related information consisting of sets of RAND, SRES and Kc is stored in the VLR and in the HLR.

When a VLR has used a set of security related information to authenticate a UE, it shall delete the set of security related information or mark it as used. When a VLR needs to use security related information, it shall use a set which is not marked as used in preference to a set which is marked as used; if there are no sets which are not marked as used, then the VLR shall request fresh security related information from the HLR. If a set of fresh security related information cannot be obtained in this case because of a system failure, the VLR may re-use a set which is marked as used.

“System failure” in this context means that the VLR was unable to establish contact with the HLR, or the HLR returned a positive acknowledgement containing no sets of security related information, or the HLR returned an error indicating that there was a system failure or that the request was badly formatted.

If the HLR responds to a request for security related information with an indication that the subscriber is unknown or barred in the HLR, the VLR shall not re-use security information which has been marked as used.

It is an operator option to define how many times a set of security related information may be re-used in the VLR; when a set of security related information has been re-used as many times as is permitted by the operator, it shall be deleted.

If a VLR successfully requests security related information from the HLR, it shall discard any security related information which is marked as used in the VLR.

If a VLR receives from another VLR a request for security related information, it shall send only the sets which are not marked as used.

If an HLR receives a request for security related information, it shall send any sets which are not marked as used; those sets shall then be deleted or marked as used. If there are no sets, which are not marked as used, the HLR may as an operator option send sets which are marked as used. It is an operator option to define how many times a set of security related information may be re-sent by the HLR; when a set of security related information has been sent as many times as is permitted by the operator, it shall be deleted.

# 4	Confidentiality of signalling information elements, connectionless data and user information elements on physical connections

## 4.1	Generality

In GSM 02.09 [3], some signalling information elements are considered sensitive and must be protected.

To ensure identity confidentiality (see clause 2), the Temporary Subscriber Identity must be transferred in a protected mode at allocation time and at other times when the signalling procedures permit it.

The confidentiality of connection less user data requires at least the protection of the message part pertaining to OSI layers 4 and above.

The user information confidentiality of user information on physical connections concerns the information transmitted on a traffic channel at the UE-BSS interface (e.g. for speech). It is not an end-to-end confidentiality service.

These needs for a protected mode of transmission are fulfilled with the same mechanism where the confidentiality function is a OSI layer 2 function. The scheme described below assumes that the main part of the signalling information elements is transmitted on DCCH (Dedicated Control Channel), and that the CCCH (Common Control Channel) is only used for the allocation of a DCCH.

Four points have to be specified:

-	the ciphering method;

-	the key setting;

-	the starting of the enciphering and deciphering processes;

-	the synchronisation.

## 4.2	The ciphering method

Ciphering applies at Transport Channel layer. The Transport Channel layer data flow (carrying DCCH or TCH) is ciphered by a bit per bit or stream cipher, i.e. the ciphered data flow is obtained by the bit per bit binary addition of the user data flow and a ciphering bit stream, generated by algorithm A5 using a key determined as specified in sub-clause 4.3. The key is denoted by Kc, and is called "Ciphering Key".

Deciphering is performed by exactly the same method.

Algorithm A5 is specified in Annex A.

## 4.3	Key setting

Mutual key setting is the procedure that allows the user equipment and the network to agree on the key Kc to use in the ciphering and deciphering algorithms A5.

A key setting is triggered by the authentication procedure. Key setting may be initiated by the network as often as the network operator wishes.

Key setting must occur on a DCCH not yet encrypted and as soon as the identity of the mobile subscriber (i.e. TMSI or IMSI) is known by the network.

The transmission of Kc to the UE is indirect and uses the authentication RAND value; Kc is derived from RAND by using algorithm A8 and the Subscriber Authentication key Ki, as defined in Annex A.

As a consequence, the procedures for the management of Kc are the authentication procedures described in sub-clause 3.3.

The values Kc are computed together with the SRES values. The security related information (see sub-clause 3.3.1) consists of RAND, SRES and Kc.

The key Kc is stored by the user equipment until it is updated at the next authentication.

Key setting is schematised in figure 4.1.

UE

Figure 4.1: Key setting

## 4.4	Ciphering key sequence number

The ciphering key sequence number (CKSN) is a number, which is associated with the ciphering key Kc; CKSN and Kc are both stored together in the user equipment and in the network.

However since it is not directly involved in any security mechanism, it is not addressed in this specification but in TSM 04.08 [9] instead.

## 4.5	Starting of the ciphering and deciphering processes

The UE and the BSS must co-ordinate the instants at which the enciphering and deciphering processes start on DCCH and TCH.

On DCCH, this procedure takes place under the control of the network some time after the completion of the authentication procedure (if any), or after the key Kc has been made available at the BSS.

No information elements for which protection is needed must be sent before the ciphering and deciphering processes are operating.

The transition from clear text mode to ciphered mode proceeds as follows: deciphering starts in the BSS, which sends in clear text to the UE a specific message, here called "Start cipher". Both the enciphering and deciphering start on the UE side after the message "Start cipher" has been correctly received by the UE. Finally, enciphering on the BSS side starts as soon as a frame or a message from the UE has been correctly deciphered at the BSS.

The starting of enciphering and deciphering processes is schematised in figure 4.2.

UE

Figure 4.2: Starting of the enciphering and deciphering processes

When a TCH is allocated for user data transmission, the key used is the one set during the preceding DCCH session (Call Set-up). The enciphering and deciphering processes start immediately.

## 4.6	Synchronisation

The enciphering stream at one end and the deciphering stream at the other end must be synchronised, for the enciphering bit stream and the deciphering bit streams to coincide. The underlying Synchronisation scheme is described in Annex A.

## 4.7	Handover

When a handover occurs, the necessary information (e.g. key Kc, initialisation data) is transmitted within the system infrastructure to enable the communication to proceed from the old BSS to the new one, and the Synchronisation procedure is resumed. The key Kc remains unchanged at handover.

## 4.8	Negotiation of A5 algorithm

No more then seven versions of the A5 algorithm will be defined.

When a UE wishes to establish a connection with the network, the UE shall indicate to the network which of the seven versions of the A5 algorithm it supports. The network shall not provide service to a UE, which indicates that it does not support the ciphering algorithm(s).

The network shall compare its ciphering capabilities and preferences, and any special requirements of the subscription of the UE, with those indicated by the UE and act according to the following rules:

1)	If the UE and the network have no versions of the A5 algorithm in common and the network is not prepared to use a non-ciphered connection, then the connection shall be released.

2)	If the UE and the network have at least one version of the A5 algorithm in common, then the network shall select one of the mutually acceptable versions of the A5 algorithm for use on that connection.

3)	If the UE and the network have no versions of the A5 algorithm in common and the network is willing to use a non-ciphered connection, then a non-ciphered connection shall be used.

# 5	Synthetic summary

Figure 5.1 shows in a synopsis a normal location updating procedure with all elements pertaining to security functions, i.e. to TMSI management, authentication and Kc management.

UE

Figure 5.1: Normal location updating procedure

Annex A (normative):
External specifications of security related algorithms

## A.0	Scope

This Annex specifies the cryptological algorithms which are needed to provide the various security features and mechanisms defined respectively in GSM 02.09 [3] and in this specification.

The following three algorithms are considered:

-	Algorithm A3: Authentication algorithm;

-	Algorithm A5: Ciphering/deciphering algorithm;

-	Algorithm A8: Ciphering key generator.

Algorithm A5 must be common to all TD-SCDMA PLMNs and all user equipment (in particular, to allow roaming). The external specifications of Algorithm A5 are defined in sub-clause A.1.3.

The internal specifications of Algorithm A5 are managed under the responsibility of GSM/MoU; they will be made available in response to an appropriate request.

Algorithms A3 and A8 are at each PLMN operator discretion. Only the formats of their inputs and outputs must be specified. It is also desirable that the processing times of these algorithms remain below a maximum value. Proposals for Algorithm A3 and A8 are managed by GSM/MoU and available, for those PLMN operators who wish to use them, in response to an appropriate request.

## A.1	Specifications for Algorithm A5

### A.1.1	Purpose

As defined in the present specification, Algorithm A5 realises the protection of both user data and signalling information elements at the Transport Channel layer on the dedicated channels (TCH or DCCH).

Synchronisation of both the enciphering and deciphering (especially at handover) must be guaranteed.

### A.1.2	Implementation indications

Algorithm A5 is implemented both in the UE and in the BSS.

Ciphering (and therefore deciphering) shall not apply to Single Burst messages (i.e. to SB-T Transport Channels; see TSM 05.02 [11]); this implies that HANDOVER ACCESS and FRAME NUMBER INFORMATION messages (see TSM 04.08 [9]) shall always be sent in clear.

Ciphering shall take place before the channel coding process (see TSM 05.03 [12]), while deciphering shall take place after channel decoding process symmetrically. Both enciphering and deciphering need Algorithm A5 and start at different times (see clause 4).

The Algorithm A5 shall produce one sequence of Max_Kd encipher/decipher bits and one sequence of Max_Kd decipher/encipher bits (here called respectively “BLOCK1” and “BLOCK2”) for each transmit/receive Transport CHannel, each Transmission Time Interval (TTI).

At the network side, BLOCK1 shall be used to encipher and BLOCK2 to decipher while, at the UE side, BLOCK1 shall be used to decipher and BLOCK2 to encipher respectively. For ciphering, the sequence of Max_Kd encipher bits (BLOCK1 or BLOCK2, respectively at the BSS and at the UE side) shall be combined by a bit-wise modulo 2 addition with the Kd bits “d(i)” of the transport channel to transmit (also called “plain text” in the sequel) such that: the first encipher bit is added to d(0), the second encipher bit is added to d(1) and so on (see TSM 05.03 [12] for “d(i)” bits description and ordering principle). The resulting ciphered transport channel is then forwarded to the channel coding process.

Due to the TDMA technique, the plain text is organised into blocks of different size (i.e. of different Kd bit length) which are delivered, after ciphering and channel coding, over the air in “n” radio frames during a given TTI as reported in the following table A.1/TSM 03.20.

For deciphering, the sequence of Max_Kd decipher bits (BLOCK1 or BLOCK2, respectively at the UE and at the BSS side) shall be combined by a bit-wise modulo 2 addition with the Kd bits d(i) of the received (from the channel decoding process)  ciphered transport channel such that: the first decipher bit is added to d(0), the second decipher bit is added to d(1) and so on.

Max_Kd corresponds to the maximum transport channel block size (i.e. 244 bits). In case the actual transport channel block size for either uplink or downlink is shorter than Max_Kd bits, then only the first part (i.e. the most significant bits) of the corresponding output parameter BLOCK is used in the bit-wise addition and the rest of the bits are discarded.

Table A.1/TSM 03.20: Transport Channels data blocks

| Transport channel type | Transport channel block size in “Kd” bits | Transmission Time Interval (TTI) | “n” radio frames |
| --- | --- | --- | --- |
| TCH-T/HS | 112 bits | 40 ms | 8 |
| TCH-T/EFS | 244 bits | 40 ms | 8 |
| SACCH-T | 184 bits | 240 ms | 48 |
| C-SACCH-T/F | 185 bits | 40 ms | 8 |
| C-SACCH-T/H | 185 bits | 60 ms | 12 |
| SDCCH-T/3 | 184 bits | 120 ms | 4 |
| SDCCH-T/6 | 184 bits | 240 ms | 4 |
| FACCH-T/F | 185 bits | 40 ms | 8 |
| FACCH-T/H | 185 bits | 60 ms | 12 |

Synchronisation between ciphering and deciphering process is guaranteed by driving Algorithm A5 with an explicit time variable, COUNT, derived from the TDMA frame number, such that each Max_Kd block produced by A5 depends only on the TDMA frame numbering and the ciphering key Kc.

In particular COUNT, expressed in 22 bits, is coded as the first System Frame Number (SFN) at which the corresponding ciphered transport channel will start to be transmitted over the radio path. The parameter COUNT is coded as the field SFN according to TSM 04.08 [9]

Figure A.1 summarises the implementation indications listed above, with only one enciphering/deciphering procedure represented (the second one for deciphering/enciphering is symmetrical).

Figure A.1: Ciphering at the Network side and deciphering at the UE side

### A.1.3	External specifications of Algorithm A5

The two input parameters (COUNT and Kc) and the output parameters (BLOCK1 and BLOCK2) of Algorithm A5 shall use the following formats:

-	length of Kc:			64 bits;

-	length of COUNT: 	22 bits;

-	length of BLOCK1:	Max_Kd  bits;

-	length of BLOCK2:	Max_Kd bits.

Algorithm A5 shall produce BLOCK1 and BLOCK2 for each single transport channel in less than the minimum TTI duration (i.e.40 ms).

NOTE:	If the actual length of the ciphering key is less than 64 bits, then it is assumed that the actual ciphering key corresponds to the most significant bits of Kc, and that the remaining and less significant bits are set to zero. It must be clear that for signalling and testing purposes the ciphering key Kc is considered to be 64 unstructured bits.

### A.1.4	Internal specification of Algorithm A5

The internal specification of Algorithm A5 is managed under the responsibility of GSM/MoU; it will be made available to in response to an appropriate request.

## A.2	Algorithm A3

Algorithm A3 is considered as a matter for TD-SCDMA PLMN operators. Therefore, only external specifications are given. However a proposal for a possible Algorithm A3 is managed by GSM/MoU and available upon appropriate request.

### A.2.1	Purpose

As defined in this specification, the purpose of Algorithm A3 is to allow authentication of a mobile subscriber's identity.

To this end, Algorithm A3 must compute an expected response SRES from a random challenge RAND sent by the network. For this computation, Algorithm A3 makes use of the secret authentication key Ki.

### A.2.2	Implementation and operational requirements

At the UE side, Algorithm A3 is contained in a Subscriber Identity Module, as specified in GSM 02.17 [4].

At the network side, it is implemented in the HLR or the AuC. The two input parameters (RAND and Ki) and the output parameter (SRES) of Algorithm A3 shall use the following formats:

-	length of Ki:		128 bits;

-	length of RAND:	128 bits;

-	length of SRES:	  32 bits.

The run-time of Algorithm A3 shall be less than 500 ms.

## A.3	Algorithm A8

Algorithm A8 is considered as a matter for TD-SCDMA PLMN operators as is Algorithm A3.

A proposal for a possible Algorithm A8 is managed by GSM/MoU and available upon appropriate request.

### A.3.1	Purpose

As defined in this specification, Algorithm A8 must compute the ciphering key Kc from the random challenge RAND sent during the authentication procedure, using the authentication key Ki.

### A.3.2	Implementation and operational requirements

On the UE side, Algorithm A8 is contained in the SIM, as specified in GSM 02.17 [4].

On the network side, Algorithm A8 is co-located with Algorithm A3.

The two input parameters (RAND and Ki) and the output parameter (Kc) of Algorithm A8 shall follow the following formats:

-	length of Ki:		128 bits;

-	length of RAND:	128 bits;

-	length of Kc:		  64 bits.

Since the maximum length of the actual ciphering key is fixed by GSM/MoU, Algorithm A8 shall produce this actual ciphering key and extend it (if necessary) into a 64 bit word where the non-significant bits are forced to zero. It is assumed that any non-significant bits are the least significant bits and that, the actual ciphering key is contained in the most significant bits. For signalling and testing purposes the ciphering key Kc is considered to be 64 unstructured bits.

Annex B (normative):
Security related network functions for TD-SCDMA Packet Radio Service

This Annex is only applicable if TD-PRS is supported.

## B.1	General

This Annex gives an overview of the different security related services and functions for TD-SCDMA Packet Radio Service (TD-PRS) which is described in TS 22.060 [5] and TSM 03.60 [7].

They are grouped as follows:

-	Subscriber identity confidentiality;

-	Subscriber identity authentication;

-	Confidentiality of user information and signalling between UE and SGSN;

- 	Security of the GPRS backbone.

It shall be possible to introduce new authentication and ciphering algorithms during the systems lifetime. The fixed part of the network may support more than one authentication and ciphering algorithm.

The security procedures include mechanisms to enable recovery in the event of signalling failures. These recovery procedures are designed to minimise the risk of a breach in the security of the system.

In this Annex, the terms GPRS-Kc and GPRS-CKSN are introduced to provide a clear distinction from the ciphering parameters (Kc and CKSN) used for circuit switched. The GPRS-Kc is the ciphering key used for TD-PRS, and GPRS-CKSN is the corresponding Ciphering Key Sequence Number used for TD-PRS.

The use of these parameters is described in sub-clause B.4.

## B.2	Subscriber identity confidentiality

### B.2.1	Generality

The purpose of this function is to avoid the possibility for an intruder to identify which subscriber is using a given resource on the radio path by listening to the signalling exchanges or the user traffic on the radio path. This allows both a high level of confidentiality for user data and signalling and protection against the tracing of users location.

The provision of this function implies that the IMSI (International Mobile Subscriber Identity), or any information allowing a listener to derive the IMSI easily, should not normally be transmitted in clear text in any signalling message on the radio path.

Consequently, to obtain the required level of protection, it is necessary that:

-	a protected identifying method is normally used instead of the IMSI on the radio path;

-	the IMSI is not normally used as addressing means on the radio path (see GSM 02.09 [3]);

-	when the signalling procedures permit it, signalling information elements that convey information about the mobile subscriber identity must be ciphered for transmission on the radio path.

The identifying method is specified in the following sub-clause. The ciphering of communication over the radio path is specified in sub-clause B.4.

Furthermore, Anonymous Access allows a user to access the network without a subscriber identity (see TSM 03.60 [7]). Therefore, Anonymous Access always guarantees by its nature subscriber identity confidentiality. The following parts of the sub-clause B.2 are not applicable for Anonymous Access.

### B.2.2	Identifying method

The means used to identify a mobile subscriber on the radio path consists of a Temporary Logical Link Identity (TLLI). This TLLI is a local number, having a meaning only in a given Routing Area (RA); the TLLI must be accompanied by the Routing Area Identity (RAI) to avoid ambiguities. The maximum length and guidance for defining the format of a TLLI are specified in TS 23.003 [6].

The SGSN manages suitable data bases to keep the relation between TLLIs and IMSIs. When a TLLI is received with an RAI that does not correspond to the current SGSN, the IMSI of the UE must be requested from the SGSN in charge of the indicated routing area if its address is known; otherwise the IMSI is requested from the UE.

A new TLLI may be allocated in each routing area updating procedure. The allocation of a new TLLI corresponds implicitly for the UE to the de-allocation of the previous one. In the fixed part of the network, the cancellation of the record for a UE in a SGSN implies the de-allocation of the corresponding TLLI.

To cope with some malfunctioning, e.g. arising from a software failure, the fixed part of the network can require the identification of the UE in clear. This procedure is a breach in the provision of the service, and should be used only when necessary.

When a new TLLI is allocated to a UE, it is transmitted to the UE in a ciphered mode. This ciphered mode is the same as defined in sub-clause B.4.

The UE must store its current TLLI in a non-volatile memory, together with the RAI, so that these data are not lost when the UE is switched off.

### B.2.3	Procedures

This sub-clause presents the procedures, or elements of procedures, pertaining to the management of TLLIs.

These security procedures may also be applied between two PLMNs of different operators for seamless service when the PLMN is changed.

#### B.2.3.1	Routing area updating in the same SGSN area

This procedure is part of the routing area updating procedure, which takes place when the original routing area and the new routing area depend on the same SGSN. The part of this procedure relative to TLLI management is reduced to a TLLI re-allocation (from TLLIo with "o" for "old" to TLLIn with "n" for "new").

The UE sends TLLIo as an identifying field at the beginning of the routing area updating procedure.

The procedure is schematised in figure B.2.1.

┌─────┐                                  ┌────────────┐á
│ UE  │                                  │    SGSN    │á
└──┬──┘                                  └──────┬─────┘á
   │                                            │      á
   │            RAI, TLLIo                      │      á
   ├───────────────────────────────────────────>┤      á
   │                                            │      á
á  │                                            │      á
   │                                     ┌────────────┐á
   │                                     │ Allocation │á
   │                                     │  of TLLIn  │á
   │                                     └────────────┘á
   │          Ciphered(TLLIn)                   │      á
   ├<───────────────────────────────────────────┤      á
   │                                            │      á
   │            Acknowledge                     │      á
   ├───────────────────────────────────────────>┤      á
   │                                            │      á
                                         ┌─────────────┐
                                         │De-allocation│
                                         │  of TLLIo   │
                                         └─────────────┘

Figure B.2.1: Routing area updating in the same SGSN area

#### B.2.3.2	Routing area updating in a new SGSN; old SGSN reachable

This procedure is part of the routing area updating procedure, using TLLI and RAI, when the original routing area and the new routing area depend on different SGSNs.

The UE is still registered in SGSNo ("o" for old or original) and requests registration in SGSNn ("n" for new). RAI and TLLIo are sent by the UE as identifying fields during the routing area updating procedure. The Routing Area Update Request is not ciphered to allow the new SGSN to read RAI and TLLIo.

The procedure is schematised in figure B.2.2.

┌─────┐                 ┌────────────┐ ┌────────┐┌─────┐á
│ UE  │                 │   SGSNn    │ │ SGSNo  ││HPLMN│á
└──┬──┘                 └──────┬┬────┘ └────┬───┘└───┬─┘á
   │                           ││           │        │ áá
   │       RAI, TLLIo          ││ RAI,TLLIo │        │ áá
   ├──────────────────────────>┤├──────────>┤        │ áá
   │                           ││           │        │  á
   │                           ││    IMSI   │        │  á
   │                           │├<──────────┤        │  á
   │                           ││Sec.Rel.Inf│        │  á
   │                    ┌────────────┐      │        │  á
   │                    │Allocation  │      │        │  á
   │                    │of TLLIn    │      │        │  á
   │                    └────────────┘               │  á
   │                           ││                    │  á
   │                           ││                    │  á
   │   Ciphered(TLLIn) (note)  ││Update Loc.  (note) │  á
   ├<──────────────────────────┤├───────────────────>┤  á
   │                           ││                    │  á
   │                           ││                    │  á
   │     Acknowledge   (note)  ││ Acknowledge (note) │  á
   ├──────────────────────────>┤├<───────────────────┤  á
                                            │        │  á
                                            │Cancellation
                                            ├<───────┤  á
                                     ┌─────────────┐    á
                                     │De-allocation│    á
                                     │of TLLIo     │    á
                                     └─────────────┘    á

NOTE:	From a security point of view, the order of the procedures is irrelevant.

Figure B.2.2: Routing area updating in a new SGSN; old SGSN reachable

Signalling functionality:

Update Loc. stands for Update Location

The new SGSN informs the HLR that it is now handling the UE.

Sec.Rel.Info.:

Stands for Security Related information

The SGSNn needs some information for authentication and ciphering; this information is obtained from SGSNo.

Cancellation:

The HLR indicates to SGSNo that the UE is now under control of another SGSN. The "old" TLLI is free for allocation.

#### B.2.3.3	Routing area updating in a new SGSN; old SGSN not reachable

This variant of the procedure in sub-clause B.2.3.2 arises when the SGSN receiving the RAI and TLLIo cannot identify the SGSNo. In that case the relation between TLLIo and IMSI is lost, and the identification of the UE in clear is necessary.

The procedure is schematised in figure B.2.3.

┌────┐                  ┌────────────┐  ┌────────┐   ┌─────┐á
│ UE │                  │    SGSNn   │  │ SGSNo  │   │HPLMN│á
└─┬──┘                  └──────┬─────┘  └────────┘   └───┬─┘á
  │                            │                         │  á
  │       RAI, TLLIo           │                         │  á
  ├───────────────────────────>┤                         │  á
  │                     ┌────────────┐                   │  á
  │                     │old SGSN not│                   │  á
  │                     │  reachable │                   │  á
  │TLLI unknown (note 1)└────────────┘                   │  á
  ├<───────────────────────────┤                         │  á
  │                            │                         │  á
  │     IMSI     (note 1)      │                         │  á
  ├───────────────────────────>┤                         │  á
  │                            │                         │  á
  ┌──────────────────────────────────────────────────────┐  á
  │             Management of means for new              │  á
  │             ciphering (see clause D4)                │  á
  └──────────────────────────────────────────────────────┘  á
  │                     ┌────────────┐                   │  á
  │                     │ Allocation │                   │  á
  │                     │  of TLLIn  │                   │  á
  │                     └────────────┘                   │  á
  │                            ││                        │  á
  │                            ││                        │  á
  │    Ciphered(TLLIn) (note 2)││ Update Loc.  (note 2)  │  á
  ├<───────────────────────────┤├───────────────────────>┤  á
  │                            ││                        │  á
  │   Acknowledge    (note 2)  ││  Acknowledge  (note 2) │  á
  ├───────────────────────────>┤├<───────────────────────┤  á
                                            │            │  á
                                            │Cancellation│  á
                                            ├<───────────┤  á
                                     ┌─────────────┐        á
                                     │De-allocation│        á
                                     │   of TLLIo  │        á
                                     └─────────────┘        á

NOTE 1:	From a security point of view, the exact signalling messages (described in TSM 03.60 [7]) used to indicate that the TLLI is unknown, or to send the IMSI are irrelevant.

NOTE 2:	From a security point of view, the order of the procedures is irrelevant.

Figure B.2.3: Routing area updating in a new SGSN; old SGSN not reachable

#### B.2.3.4	Reallocation of a TLLI

This function may be initiated by the network at any time for a TD-PRS attached UE. The procedure can be included in other procedures, e.g. through the means of optional parameters. The execution of this function is left to the network operator.

When a new TLLI is allocated to a UE the network must prevent the old TLLI from being allocated again until the UE has acknowledged the allocation of the new TLLI.

If an MM context of a UE is deleted in the SGSN by O&M action, the network must prevent any TLLI associated with the deleted MM context from being allocated again until a new TLLI is successfully allocated to that IMSI.

If an IMSI record is deleted in the HLR by O&M action, it is not possible to prevent any TLLI associated with the IMSI record from being allocated again. However, if the UE whose IMSI record was deleted should attempt to access the network using the TLLI after the TLLI has been allocated to a different IMSI, then authentication or ciphering of the UE whose IMSI was deleted will fail, which will cause the TLLI to be deleted from the UE.

The case where allocation of a new TLLI is unsuccessful is described in sub-clause B.2.3.7.

This procedure is schematised in figure B.2.4.

┌─────┐                                 ┌────────────┐á
│ UE  │                                 │   SGSN     │á
└─────┘                                 └────────────┘á
   │                                    ┌────────────┐á
   │                                    │ Allocation │á
   │                                    │  of TLLIn  │á
   │                                    └────────────┘á
   │         Ciphered(TLLIn)                   │      á
   ├<──────────────────────────────────────────┤      á
   │                                           │      á
   │           Acknowledge                     │      á
   ├──────────────────────────────────────────>┤      á
   │                                           │      á
                                        ┌─────────────┐
                                        │De-allocation│
                                        │  of TLLIo   │
                                        └─────────────┘

Figure B.2.4: Reallocation of a new TLLI

#### B.2.3.5	Local TLLI unknown

This procedure is a variant of the procedure described in sub-clause B.2.3.1 and happens when a data loss has occurred in a SGSN and when a UE uses an unknown TLLI, e.g. for a communication request or for a routing area updating request in a routing area managed by the same SGSN. The SGSN indicates to the UE that the TLLI is unknown and the identification of the UE in clear is necessary.

This procedure is schematised in figure B.2.5.

┌─────┐                       ┌────────────┐  ┌───────┐á
│ UE  │                       │   SGSN     │  │ HPLMN │á
└──┬──┘                       └──────┬─────┘  └────┬──┘á
   │                                 │             │   á
   │     RAI, TLLIo  (note 1)        │             │   á
   ├────────────────────────────────>┤             │   á
   │                          ┌────────────┐       │   á
   │                          │  TLLIo is  │       │   á
   │                          │   unknown  │       │   á
   │    TLLI unknown  (note 2)└────────────┘       │   á
   ├<────────────────────────────────┤             │   á
   │                                 │             │   á
   │           IMSI    (note 2)      │             │   á
   ├────────────────────────────────>┤             │   á
   │                                 │             │   á
   ┌───────────────────────────────────────────────┐   á
   │         Management of means for new           │   á
   │          ciphering (see clause D4)            │   á
   └───────────────────────────────────────────────┘   á
   │                          ┌────────────┐           á
   │                          │ Allocation │           á
   │                          │  of TLLIn  │           á
   │       Ciphered(TLLIn)    └────────────┘           á
   ├<────────────────────────────────┤                 á
   │                                 │                 á
   │       Acknowledge               │                 á
   ├────────────────────────────────>┤                 á
   │                                 │                 á

NOTE 1:	Any message in which TLLIo is used as an identifying means in a routing area managed by the same SGSN.

NOTE 2:	From a security point of view, the exact signalling messages (described in TSM 03.60 [7]) used to indicate that the TLLI is unknown, or to send the IMSI are irrelevant.

Figure B.2.5: Routing area updating in the same SGSN area; local TLLI unknown

#### B.2.3.6	Routing area updating in a new SGSN in case of a loss of information

This variant of the procedure described in B.2.3.2 arises when the SGSN in charge of the UE has suffered a loss of data. In that case the relation between TLLIo and IMSI is lost, and the identification of the UE in clear is necessary.

The procedure is schematised in figure B.2.6.

┌────┐                  ┌────────────┐  ┌────────┐   ┌─────┐á
│ UE │                  │ SGSNn      │  │SGSNo   │   │HPLMN│á
└─┬──┘                  └──────┬┬────┘  └───┬────┘   └───┬─┘á
  │                            ││           │            │  á
  │       RAI, TLLIo           ││ RAI,TLLIo │            │  á
  ├───────────────────────────>┤├──────────>┤            │  á
  │                            ││           │            │  á
  │                            ││   Unknown │            │  á
  │     TLLI Unknown(note 1)   │├<──────────┤            │  á
  ├<───────────────────────────┤│           │            │  á
  │                            │                         │  á
  │      IMSI(note 1)          │                         │  á
  ├───────────────────────────>┤                         │  á
  │                            │                         │  á
  ┌──────────────────────────────────────────────────────┐  á
  │             Management of means for new              │  á
  │              ciphering (see clause D4)               │  á
  └──────────────────────────────────────────────────────┘  á
  │                     ┌────────────┐                   │  á
  │                     │Allocation  │                   │  á
  │                     │of TLLIn    │                   │  á
  │                     └────────────┘                   │  á
  │                            ││                        │  á
  │  Ciphered(TLLIn) (note 2)  ││Update location (note 2)│  á
  ├<───────────────────────────┤├───────────────────────>┤  á
  │                            ││                        │  á
  │     Acknowledge  (note 2)  ││   Acknowledge  (note 2)│  á
  ├───────────────────────────>┤├<───────────────────────┤  á
                                            │            │  á
                                            │Cancellation│ áá
                                            ├<───────────┤  á
                                     ┌─────────────┐        á
                                     │De-allocation│        á
                                     │  of TLLIo   │        á
                                     └─────────────┘        á

NOTE 1:	From a security point of view, the exact signalling messages (described in TSM 03.60 [7]) used to indicate that the TLLI is unknown, or to send the IMSI are irrelevant.

NOTE 2:	From a security point of view, the order of the procedures is irrelevant.

Figure B.2.6: Routing area updating in a new SGSN in case of a loss of information

#### B.2.3.7	Unsuccessful TLLI allocation

If the UE does not acknowledge the allocation of a new TLLI, the network shall maintain the association between the old TLLI and the IMSI and between the new TLLI and the IMSI.

For a UE-originated transaction, the network shall allow the UE to identify itself by either the old TLLI or the new TLLI. This will allow the network to determine the TLLI stored in the UE; the association between the other TLLI and the IMSI shall then be deleted.

For a network-originated transaction, the network shall identify the UE by its IMSI. When radio contact has been established, the network shall instruct the UE to delete any stored TLLI. When the UE has acknowledged this instruction, the network shall delete the association between the IMSI of the UE and any TLLI.

In either of the cases above, the network may initiate the normal TLLI reallocation procedure.

Repeated failure of TLLI reallocation (passing a limit set by the operator) may be reported for O&M action.

## B.3	Subscriber identity authentication

### B.3.1	Generality

The definition and operational requirements of subscriber identity authentication are given in GSM 02.09 [3].

The authentication procedure may be performed at any time by the network.

The authentication procedure will also be used to set the ciphering key (see sub-clause B.4). Therefore, it is performed after the subscriber identity (TLLI/IMSI) is known by the network for the management of new ciphering.

Two network functions are necessary: the authentication procedure itself, and the key management.

### B.3.2	The authentication procedure

The authentication procedure is described in sub-clause 3.2.

### B.3.3	Subscriber Authentication Key management

The management of Subscriber Authentication Key (Ki) is described in sub-clause 3.3.

#### B.3.3.1	General authentication procedure

When needed, the SGSN requests security related information for a UE from the HLR/AuC corresponding to the IMSI of the UE. This includes an array of pairs of corresponding RAND and SRES. These pairs are obtained applying Algorithm A3 to each RAND and the key Ki as shown in figure 3.1. The pairs are stored in the SGSN as part of the security related information.

The procedure used for updating the vectors RAND/SRES is schematised in figure D.3.2.

NOTE:	The Authentication Vector Response contains also GPRS-Kc(1..n), which is not shown in this and the following figures. For discussion of GPRS-Kc see sub-clause B.4.

┌──────────────┐                    ┌──────────────┐
│     SGSN     │                    │   HLR/AuC    │
└────┬─────────┘                    └───────┬──────┘
     │                                      │      á
     │Security Related Information Req(IMSI)│      á
     ├─────────────────────────────────────>┤      á
     │                                      │      á
     │                           ┌───────────┐     á
     │                           │ generate  │  Ki á
     │                           │ RAND(1..n)│  │  á
     │                           └─────┬─────┘  │  á
     │                                 │        │  á
     │                                 V        V  á
     │                              ┌─────────────┐á
     │                              │     A3      │á
     │                              └───────┬─────┘á
     │     Authentication Vector Response   │      á
     ├<─────────────────────────────────────┘      á
     │     (SRES(1..n), RAND(1..n))                á
     │                                             á
┌───────────────┐                                  á
│Store RAND/SRES│                                  á
│    vectors    │                                  á
└───────────────┘                                  á

Figure B.3.2: Procedure for updating the vectors RAND/SRES

When an SGSN performs an authentication, including the case of a routing area updating within the same SGSN area, it chooses a RAND value in the array corresponding to the UE. It then tests the answer from the UE by comparing it with the corresponding SRES, as schematised in figure B.3.3.

┌───────────┐                        ┌──────────────┐ á
│    UE     │                        │    SGSN      │ á
└───────────┘                        └──────┬───────┘ á
         │                                  │         á
         │           RAND(j)                │  SRES(j)á
         ├<─────────────────────────────────┤         á
                                            │      │  á
  │Ki    │RAND(j)                           │      │  á
  V      V                                  │      │  á
┌─────────────┐                             │      │  á
│     A3      │                             │      │  á
└────────┬────┘                             │      │  á
         │SRES(j)                           │      │  á
         │                                  │      │  á
         V                                  │      │  á
                                            │      │  á
         │           SRES(j)                │      │  á
         ├─────────────────────────────────>┤ ──┐  │  á
         │                                  │   │  │  á
         │                                  │   V  V  á
                                              ┌─────┐ á
                                              │  =  │ á
                                              └──┬──┘ á
                                                 V    á
                                              yes/no  á

Figure B.3.3: General authentication procedure

#### B.3.3.2	Authentication at routing area updating in a new SGSN, using TLLI

During routing area updating in a new SGSN (SGSNn), the procedure to get pairs for subsequent authentication may differ from that described in the previous sub-clause. In the case when identification is done using TLLI, pairs for authentication as part of security related information is given by the old SGSN (SGSNo). The old SGSN shall send to the new SGSN only those pairs, which have not been used. SGSNn may also request the triplets directly from HLR.

The procedure is schematised in figure B.3.4.

┌────┐                  ┌────────────┐ ┌────────┐ ┌─────┐á
 │ UE │                  │  SGSNn     │ │SGSNo   │ │HPLMN│á
 └─┬──┘                  └──────┬┬────┘ └────┬───┘ └───┬─┘á
   │                            ││           │         │  á
   │       RAI, TLLIo           ││ TLLIo, RAI│         │  á
   ├───────────────────────────>┤├──────────>┤         │  á
   │                            ││           │         │  á
   │                            ││ IMSI      │         │  á
 Ki│                            ││ RAND(1..n)│         │  á
 │ │                            ││ SRES(1..n)│         │  á
 │ │       RAND                 │├<──────────┤         │  á
 │ ├<───────────────────────────┤                      │  á
 │ │                            │                      │  á
 V V                            │                      │  á
┌─────┐                         │                      │  á
│ A3  │                         │                      │  á
└─────┘                         │                      │  á
   │       SRES                 │                      │  á
   ├───────────────────────────>┤                      │  á
   │                         ┌─────┐                   │  á
   │                         │  =  │                   │  á
   │                         └──┬──┘                   │  á
                                V                      │  á
                              yes/no                   │  á
                                ┌──────────────────────┐  á
                                │    Update Location   │  á
                                └──────────────────────┘  á

Figure B.3.4: Authentication at routing area updating in a new SGSN, using TLLI

#### B.3.3.3	Authentication at routing area updating in a new SGSN, using IMSI

When the IMSI is used for identification, or more generally when the old SGSN is not reachable, the procedure described in sub-clause B.3.3.2 cannot be used. Instead, pairs of RAND/SRES contained in the security related information is requested directly from the HPLMN.

The procedure is schematised in figure B.3.5.

┌────┐                  ┌────────────┐           ┌─────┐á
 │ UE │                  │    SGSNn   │           │HPLMN│á
 └─┬──┘                  └──────┬┬────┘           └───┬─┘á
   │       IMSI                 ││Sec. Rel. Info Req. │  á
   ├───────────────────────────>┤├───────────────────>┤  á
   │                            ││     (IMSI)         │  á
   │                            ││                    │  á
 Ki│                            ││       RAND(1,..n)  │  á
 │ │       RAND                 │├<───────────────────┤  á
 │ ├<───────────────────────────┤        SRES(1..n)   │  á
 V V                            │                     │  á
┌─────┐                         │                     │  á
│ A3  │                         │                     │  á
└─────┘                         │                     │  á
   │       SRES                 │                     │  á
   ├───────────────────────────>┤                     │  á
   │                         ┌─────┐                  │  á
   │                         │  =  │                  │  á
                             └──┬──┘                  │  á
                                V                     │  á
                              yes/no                  │  á
                                ┌─────────────────────┐  á
                                │   Update Location   │  á
                                └─────────────────────┘  á

Figure B.3.5: Authentication at routing area updating in a new SGSN, using IMSI

#### B.3.3.4	Authentication at routing area updating in a new SGSN, using TLLI, TLLI unknown in 'old' SGSN

This case is an abnormal one, when a data loss has occurred in the 'old' SGSN.

The procedure is schematised in figure B.3.6.

┌────┐                  ┌────────────┐ ┌────────┐ ┌─────┐á
 │ UE │                  │  SGSNn     │ │SGSNo   │ │HPLMN│á
 └─┬──┘                  └──────┬┬────┘ └────┬───┘ └───┬─┘á
   │                            ││           │         │  á
   │       RAI, TLLIo           ││RAI, TLLIo │         │  á
   ├───────────────────────────>┤├──────────>┤         │  á
   │                            ││           │         │  á
   │                            ││  Unknown  │         │  á
   │            TLLI Unknown    │├<──────────┤         │  á
   ├<───────────────────────────┤                      │  á
   │                            │                      │  á
   │           IMSI             │                      │  á
   ├───────────────────────────>┤│ Sec. Rel. Info Req. │  á
   │                            │├────────────────────>┤  á
   │                            ││       (IMSI)        │  á
   │                            ││                     │  á
   │                            ││RAND(1..n) SRES(1..n)│  á
   │                            │├<────────────────────┤  á
 Ki│       RAND                 │                      │  á
 │ ├<───────────────────────────┤                      │  á
 V V                            │                      │  á
┌─────┐                         │                      │  á
│ A3  │                         │                      │  á
└─────┘                         │                      │  á
   │       SRES                 │                      │  á
   ├───────────────────────────>┤                      │  á
   │                         ┌─────┐                   │  á
   │                         │  =  │                   │  á
                             └──┬──┘                   │  á
                                V                      │  á
                             yes/no                    │  á
                                ┌──────────────────────┐  á
                                │  Update Location     │  á
                                └──────────────────────┘  á

Figure B.3.6: Authentication at routing area updating in a new SGSN, using TLLI, TLLI unknown in 'old' SGSN

#### B.3.3.5	Authentication at routing area updating in a new SGSN, using TLLI, old SGSN not reachable

The case occurs when an old SGSN cannot be reached by the new SGSN.

The procedure is schematised in figure B.3.7

┌────┐                  ┌────────────┐ ┌────────┐ ┌─────┐á
 │ UE │                  │  SGSNn     │ │ SGSNo  │ │HPLMN│á
 └─┬──┘                  └──────┬─────┘ └────────┘ └───┬─┘á
   │                            │                      │  á
   │       RAI, TLLIo           │                      │  á
   ├───────────────────────────>┤                      │  á
   │                            │                      │  á
   │                     ┌────────────┐                │  á
   │                     │ SGSNo not  │                │  á
   │                     │ reachable  │                │  á
   │                     └────────────┘                │  á
   │          TLLI Unknown      │                      │  á
   ├<───────────────────────────┤                      │  á
   │                            │                      │  á
   │          IMSI              │                      │  á
   ├───────────────────────────>┤│ Sec. Rel. Info Req. │  á
   │                            │├────────────────────>┤  á
   │                            ││       (IMSI)        │  á
   │                            ││                     │  á
   │                            ││RAND(1..n) SRES(1..n)│  á
   │                            │├<────────────────────┤  á
 Ki│       RAND                 │                      │  á
 │ ├<───────────────────────────┤                      │  á
 V V                            │                      │  á
┌─────┐                         │                      │  á
│ A3  │                         │                      │  á
└─────┘                         │                      │  á
   │       SRES                 │                      │  á
   ├───────────────────────────>┤                      │  á
   │                         ┌─────┐                   │  á
   │                         │  =  │                   │  á
                             └──┬──┘                   │  á
                                V                      │  á
                              yes/no                   │  á
                                ┌──────────────────────┐  á
                                │      Update Location │  á
                                └──────────────────────┘  á

Figure B.3.7: Authentication at routing area updating in a new SGSN, using TLLI, old SGSN not reachable

#### B.3.3.6	Authentication with IMSI if authentication with TLLI fails

If authentication of a UE, which identifies itself with a TLLI is unsuccessful, the network requests the IMSI from the UE, and repeats the authentication using the IMSI. Optionally, if authentication using the TLLI fails the network may reject the access request or location registration request, which triggered the authentication.

### B.3.3.7	Re-use of security related information in failure situations

Security related information consisting of sets of RAND, SRES and a ciphering key (GPRS-Kc) is stored in the SGSN and in the HLR.

When a SGSN has used a set of security related information to authenticate a UE, it shall delete the set of security related information or mark it as used. When a SGSN needs to use security related information it shall use a set which is not marked as used in preference to a set which is marked as used; if there are no sets which are not marked as used then the SGSN shall request fresh security related information from the HLR. If a set of fresh security related information cannot be obtained in this case because of a system failure, the SGSN may re-use a set which is marked as used.

“System failure” in this context means that the SGSN was unable to establish contact with the HLR, or the HLR returned a positive acknowledgement containing no sets of security related information, or the HLR returned an error indicating that there was a system failure or that the request was badly formatted.

If the HLR responds to a request for security related information with an indication that the subscriber is unknown or barred in the HLR, the SGSN shall not re-use security information which has been marked as used.

It is an operator option to define how many times a set of security related information may be re-used in the SGSN; when a set of security related information has been re-used as many times as is permitted by the operator, it shall be deleted.

If a SGSN successfully requests security related information from the HLR, it shall discard any security related information which is marked as used in the SGSN.

If a SGSN receives from another SGSN a request for security related information, it shall send only the sets which are not marked as used.

If an HLR receives a request for security related information, it shall send any sets which are not marked as used; those sets shall then be deleted or marked as used. If there are no sets, which are not marked as used, the HLR may as an operator option send sets which are marked as used. It is an operator option to define how many times a set of security related information may be re-sent by the HLR; when a set of security related information has been sent as many times as is permitted by the operator, it shall be deleted.

## B.4	Confidentiality of user information and signalling between UE and SGSN

### B.4.1	Generality

In GSM 02.09 [3], some signalling information elements are considered sensitive and must be protected.

To ensure identity confidentiality (see clause 2), the new TLLI must be transferred in a protected mode at allocation time.

The confidentiality of user information concerns the information transmitted on the logical connection between UE and SGSN.

These needs for a protected mode of transmission are fulfilled by a ciphering function in the LLC layer. It is not an end-to-end confidentiality service.

Four points have to be specified:

-	the ciphering method;

-	the key setting;

-	the starting of the enciphering and deciphering processes;

-	the synchronisation.

### B.4.2	The ciphering method

The LLC layer information flow is ciphered with the algorithm GPRS-A5 as described in GSM 01.61 [2].

### B.4.3	Key setting

Mutual key setting is the procedure that allows the user equipment and the network to agree on the key GPRS-Kc to use in the ciphering and deciphering algorithms GPRS-A5. This procedure corresponds to the procedure described in sub-clause 4.3 besides the different confidential subscriber identity. The GPRS-Kc is handled by the SGSN independently from the MSC. If a UE is using both circuit switched and packet switched, two different ciphering keys will be used independently, one (Kc) in the MSC and one (GPRS-Kc) in the SGSN.

A key setting is triggered by the authentication procedure. Key setting may be initiated from the network as often as the network operator wishes. If an authentication procedure is performed during a data transfer, the new ciphering parameters shall be taken in use immediately at the end of the authentication procedure in both SGSN and UE.

Key setting may not be encrypted and shall be performed as soon as the identity of the mobile subscriber (i.e. TLLI or IMSI) is known from the network.

The transmission of GPRS-Kc to the UE is indirect and uses the authentication RAND value; GPRS-Kc is derived from RAND by using algorithm A8 and the Subscriber Authentication key Ki, in the same way as defined in Annex A for Kc.

As a consequence, the procedures for the management of GPRS-Kc are the authentication procedures described in sub-clause B.3.3.

The values GPRS-Kc are computed together with the SRES values. The security related information (see sub-clause B.3.3.1) consists of RAND, SRES and GPRS-Kc.

The key GPRS-Kc is stored by the user equipment until it is updated at the next authentication.

Key setting is schematised in figure B.4.1.

┌───────────┐                        ┌──────────────┐ á
│    UE     │                        │ Network side │ á
└────────┬──┘                        └──────┬───────┘ á
         │                                  │         á
         │       RAI and TLLI or IMSI       │         á
         ├─────────────────────────────────>┤         á
         │                                  │         á
         │                 RAND             │         á
         ├<─────────────────────────────────┤         á
         │                                  │         á
  │Ki    │RAND                              │RAND  │Kiá
  V      V                                  V      V  á
┌─────────────┐                        ┌─────────────┐á
│     A8      │                        │     A8      │á
└──────┬──────┘                        └──────┬──────┘á
       │ GPRS-Kc                              │ GPRS-Kc
       V                                      V       á
┌─────────────┐                        ┌─────────────┐á
│Store GPRS-Kc│                        │Store GPRS-Kc│á
└─────────────┘                        └─────────────┘á

Figure B.4.1: Key setting

### B.4.4	Ciphering key sequence number

The GPRS-CKSN (Ciphering Key Sequence Number) is a number, which is associated with each ciphering key GPRS-Kc. The GPRS-CKSN and GPRS-Kc are stored together in the user equipment and in the network. It permits the consistency check of the keys stored in the UE and in the network. Two independent pairs, Kc and CKSN (for circuit switched), and GPRS-Kc and GPRS-CKSN (for packet switched) may be stored in the UE simultaneously.

However since it is not directly involved in any security mechanism, it is not addressed in this specification but in [TS 24.008 [8] – section 4] instead. GPRS-CKSN coding is reported in TSM 04.08 [9].

### B.4.5	Starting of the ciphering and deciphering processes

The UE and the SGSN must co-ordinate the instants at which the ciphering and deciphering processes start. The authentication procedure governs the start of ciphering. The SGSN indicates if ciphering shall be used or not in the Authentication and Ciphering Request message. If ciphering is used, the UE starts ciphering after sending the Authentication and Ciphering Response message. The SGSN starts ciphering when a valid Authentication and Ciphering Response message is received from the UE.

Upon TD-PRS Attach, if ciphering is to be used, an Authentication and Ciphering Request message shall be sent to the UE to start ciphering.

If the GPRS-CKSN stored in the network does not match the GPRS-CKSN received from the UE in the Attach Request message, then the network should authenticate the UE.

As an option, the network may decide to continue ciphering without authentication after receiving a Routing Area Update Request message with a valid GPRS-CKSN. Both the UE and the network shall use the latest ciphering parameters. The UE starts ciphering after a receiving a valid ciphered Routing Area Update Accept message from the network. The SGSN starts ciphering when sending the ciphered Routing Area Update Accept message to the UE.

Upon delivery of the Authentication and Ciphering Response message or the Routing Area Update Accept message, the GPRS Mobility and Management entity in both SGSN and UE shall be aware if ciphering has started or not. LLC provides the capability to send both ciphered and non-ciphered PDUs. The synchronisation of ciphering at LLC frames level is done by a bit in the LLC header indicating if the frame is ciphered or not. Only a few identified signalling messages (e.g., Routing Area Update Request message) described in TS 24.008 [8] may be sent non-ciphered, any other frames sent non-ciphered shall be deleted. Once the encryption has been started, neither the UE nor the network shall go to a non-ciphered session.

### B.4.6	Synchronisation

The enciphering stream at one end and the deciphering stream at the other end must be synchronised, for the enciphering bit stream and the deciphering bit streams to coincide. Synchronisation is guaranteed by driving Algorithm GPRS-A5 with an explicit variable INPUT per established LLC and direction.

These initial INPUT values shall not be identical for the different LLC link. The initial INPUT value shall be determined by the network. It may be identical for uplink and downlink value because the direction is given to the ciphering algorithm as described in GSM 01.61 [2] and illustrated on the figure B.4.2. In a given direction, the INPUT value shall be unique for each frame.

The calculation of the INPUT value is described in GSM. The use of the INPUT value is described in GSM 01.61 [2] and illustrated on the figure B.4.2.

Figure B.4.2: Use of the INPUT parameter

### B.4.7	Inter SGSN routing area update

When an Inter SGSN routing area update occurs, the necessary information (e.g. key Kc, INPUT parameters) is transmitted within the system infrastructure to enable the communication to proceed from the old SGSN to the new one, and the Synchronisation procedure is resumed. The key Kc may remains unchanged at Inter SGSN routing area update.

### B.4.8	Negotiation of GPRS-A5 algorithm

Not more than seven versions of the A5 algorithm will be defined.

When a UE wishes to establish a connection with the network, the UE shall indicate to the network which version(s) of the GPRS-A5 algorithm it supports. The negotiation of GPRS-A5 algorithm happens during the authentication procedure.

The network may renegotiate the version of the GPRS-A5 algorithm in use, at inter SGSN routing area update, by performing an authentication procedure.

The network shall compare its ciphering capabilities and preferences, and any special requirements of the subscription of the UE, with those indicated by the UE and may take one of the following decisions:

- If the UE and the network have no versions of the GPRS A5 algorithm in common and the network is not prepared to use an unciphered connection, then the connection is released.

- If the UE and the network have at least one version of the GPRS A5 algorithm in common, then the network shall select one of the mutually acceptable versions of the GPRS A5 algorithms for use on that connection.

- If the UE and the network have no versions of the GPRS A5 algorithm in common and the network is willing to use an unciphered version, then an unciphered connection shall be used.

## B.5	Synthetic summary

Figure B.5.1 shows in a synopsis a routing area updating procedure with all elements pertaining to security functions, i.e. to TLLI management, authentication and GPRS-Kc management.

┌────┐                  ┌────────────┐ ┌──────────┐  ┌─────┐á
 │ UE │                  │     SGSNn  │ │   SGSNo  │  │HPLMN│á
 └─┬──┘                  └──────┬┬────┘ └────┬─────┘  └───┬─┘á
   │                            ││           │            │  á
   │       RAI, TLLIo           ││ RAI, TLLIo│            │  á
   ├───────────────────────────>┤├──────────>┤            │  á
   │  Routing area updating     ││           │            │  á
   │                            ││ IMSI      │            │  á
   │                            ││ RAND      │            │  á
   │                            ││ SRES      │            │  á
   │                            ││ GPRS-Kc   │            │  á
 Ki│         RAND               │├<──────────┤            │  á
 │ ├<───────────────────────────┤│           │            │  á
 V V                            ││           │            │  á
┌─────┐                         ││           │            │  á
│A3&A8│                         ││           │            │  á
└─────┘      SRES               ││           │            │  á
 │ ├───────────────────────────>┤│           │            │  á
 V │                          ┌─────┐        │            │  á
GPRS-Kc│                          │  =  │        │            │      á
   │                          └──┬──┘        │            │  á
   │                             V                        │  á
   │                            yes                       │  á
   │               ┌───────────────────┐                  │  á
   │               │start of ciphering │                  │  á
   │               └───────────────────┘                  │  á
   │                            ││     Update location    │  á
   │                            │├───────────────────────>┤  á
   │                            ││                        │  á
   │                            ││    Acknowledge         │  á
   │                            │├<───────────────────────┤  á
   │                      ┌─────────────┐    │            │  á
   │                      │Allocation   │    │            │  á
   │                      │of TLLIn     │    │            │  á
   │                      └─────────────┘    │            │  á
   │                            ││           │            │  á
   │  Routing area update Accept││           │            │  á
   ├<───────────────────────────┤│           │            │  á
   │                            ││           │            │  á
   │                            ││           │            │  á
   │    TLLI acknowledge        ││           │            │  á
   ├───────────────────────────>┤│           │            │  á
                                             │Cancellation│  á
                                             │<───────────┤  á
                                      ┌─────────────┐        á
                                      │De-allocation│        á
                                      │  of TLLIo   │        á
                                      └─────────────┘        á

Figure B.5.1: Routing area updating procedure

## B.6	Security of the GPRS backbone

The operator is responsible for the security of its own Intra-PLMN backbone, which includes all network elements and physical connections. The operator shall prevent unauthorised access to its Intra-PLMN backbone. A secure Intra-PLMN backbone guarantees that no intruder can eavesdrop or modify user information and signalling in the Intra-PLMN backbone.

The GPRS architecture utilises GPRS tunnelling and private IP addressing within the backbone to restrict unauthorised access to the backbone. User traffic addressed to a network element shall be discarded. Firewall functionality may provide these means at the access points (Gi reference point and Gp interface) of the Intra-PLMN backbone.

The Inter-PLMN links shall be negotiated between operators as part of the roaming agreement. They shall ensure that the Inter-PLMN links are secure providing integrity and confidentiality. For example, secure links can be achieved by point to point links, private Inter-PLMN backbones or encrypted tunnels over the public Internet.

Operators shall be able to determine the origin of packets coming from the inter-PLMN backbone. One example is to use a Frame Relay PVC between two operators.

Annex C (informative):
Security issues related to signalling schemes and key management

## C.1	Introduction

The diagrams in this Annex indicate the security items related to signalling functions and to some of the key management functions. The purpose of the diagrams is to give a general overview of signalling, both on the radio path and in the fixed network. The diagrams indicate how and where keys are generated, distributed, stored and used. The security functions are split between VLR and BSS/MSC.

## C.2	Short description of the schemes

Scheme 1:	Location registration

-	no TMSI available.

The situation occurs where a UE requests registration and for some reason e.g. TMSI is lost or this is the first registration, there is no TMSI available. In this case the IMSI is used for identification. The IMSI is sent in clear text via the radio path as part of the location updating.

Scheme 2:	Location updating

-	UE registered in VLR;

-	TMSI is still available.

The user equipment stays within the area controlled by the VLR. The user equipment is already registered in this VLR. All information belonging to the user equipment is stored in the VLR, so no connection with the HLR is necessary. Identification is done with the CKSN, LAI and TMSI. For authentication a new set of RAND, SRES and Kc is already available in the VLR.

Scheme 3:	Location updating

-	UE not yet registered in VLR;

-	TMSI is still available.

The UE has roamed to an area controlled by another VLR. The LAI is used to address the "old" VLR. The TMSI is used for identification. The "old" VLR informs the "new" VLR about this UE. The security related information is sent by the "old" VLR to the "new" VLR.

Scheme 4:	Location updating

-	UE not yet registered in VLR and no old LAI.

The VLR cannot identify the VLR where the UE was last registered. Identification is therefore done using the IMSI. The VLR cannot request authentication information from the previous VLR (LAI not available), so the HLR has to send the authentication information to the VLR.

Scheme 5:	Call set-up

-	mobile originated;

-	early assignment.

The users of the registered UE want to set-up a call. Identification is done using the TMSI. All signalling information elements in all messages on the radio path are encrypted with ciphering key Kc. The PLMN is setting up calls with "early assignment".

Scheme 6:	Call set-up

-	mobile originated;

-	off air call set-up.

As in scheme 5 the user of the registered UE wants to set-up a call. Identification is done using the TMSI. All signalling information elements in all messages on the radio path are encrypted with ciphering key Kc after the cipher mode command message. The PLMN is setting up calls with "off air call set-up".

Scheme 7:	Call set-up

-	mobile terminated;

-	early assignment.

A paging request is sent to the registered UE, addressed by the TMSI. All signalling information elements in all messages on the radio path are encrypted with ciphering key Kc after the cipher mode command message. The PLMN is setting up calls with "early assignment".

## C.3	List of abbreviations

In addition to the abbreviations listed in TSM 01.04 [1], the following abbreviations are used in the schemes:

A3	authentication algorithm

A5	signalling data and user data encryption algorithm

A8	ciphering key generating algorithm

BSS	Base Station System

CKSN	Ciphering Key Sequence Number

HLR	Home Location Register

IMSI	International Mobile Subscriber Identity

Kc	ciphering key

Kc[M]	message encrypted with ciphering key Kc

Kc[TMSI]	TMSI encrypted with ciphering key Kc

Ki	individual subscriber authentication key

LAI	Location Area Identity

MSC	Mobile services Switching Centre

R	Random number (RAND)

S	Signed response (SRES)

SFN	System Frame Number

TMSI o/n	Temporary Mobile Subscriber Identity old/new

TTI	Transmission Time Interval

UE	User Equipment

VLR o/n	Visitor Location Register old/new

Scheme 1	Location registration

- no TMSI available

UE

Scheme 1 (concluded)

UE

Scheme 2	Location updating

- UE registered in VLR

- TMSI is still available

UE

Scheme 2 (concluded)

UE

Scheme 3	Location updating

- UE not yet registered in VLR

- TMSI is still available

UE

Scheme 3 (concluded)

UE

Scheme 4	Location updating

- UE not yet registered in VLR; no old LAI

UE

Scheme 5	Call set-up

- Mobile originated

- early assignment

UE

Scheme 5 (concluded)

UE

Scheme 6	Call set-up

- Mobile originated

- Off air call set-up

UE

Scheme 6 (concluded)

UE

Scheme 7	Call set-up

- Mobile terminated

- Early assignment

UE

Scheme 7 (concluded)

UE

Annex D (informative):
Security information to be stored in the entities of the TD-SCDMA system

## D.1	Introduction

This Annex gives an overview of the security related information and the places where this information is stored in the TD-SCDMA network.

The entities of the TD-SCDMA network where security information is stored are:

-	Home location register;

-	Visitor location register;

-	Mobile services switching centre;

-	Base station system;

-	User equipment;

-	Authentication Centre.

## D.2	Entities and security information

### D.2.1	Home Location Register (HLR)

If required, sets of Kc, RAND and SRES coupled to each IMSI are stored in the HLR.

### D.2.2	Visitor Location Register (VLR)

Sets of Kc, RAND and SRES coupled to each IMSI are stored in the VLR. In addition the CKSN, LAI and TMSI are stored together with the presumed valid Kc.

After a new TMSI is generated, both the old and the new TMSI are stored. When the old TMSI is no longer valid, it is removed from the database.

### D.2.3	Mobile services Switching Centre (MSC)/Base Station System (BSS)

Encryption algorithm A5 is stored in the MSC/BSS.

Call related information stored in the MSC includes the ciphering key Kc and CKSN associated with the identity of the mobile engaged in this call.

After a new TMSI is generated, both the old and the new TMSI are stored. When the old TMSI is no longer valid, it is removed from the database.

### D.2.4	User Equipment (UE)

The user equipment stores permanently:

-	Authentication algorithm A3;

-	Encryption algorithm A5;

-	Ciphering key generating algorithm A8;

-	Individual subscriber authentication key Ki;

-	Ciphering key Kc;

-	Ciphering key sequence number;

-	TMSI.

The user equipment generates and stores:

-	Ciphering key Kc.

The user equipment receives and stores:

-	Ciphering key sequence number;

-	TMSI;

-	LAI.

### D.2.5	Authentication Centre (AuC)

In the authentication centre are implemented:

-	Authentication algorithm(s) A3;

-	Ciphering key generating algorithm(s) A8.

The secret individual authentication keys Ki of each subscriber are stored in an authentication centre.

# History

| Document history |  |  |  |
| --- | --- | --- | --- |
| DATE | OLD_VERS | NEW_VERS | SUBJECT |
| August 2002 | - | 3.0.0 | For First submission to CWTS #13 |
| August 2002 | 3.0.0 | 3.0.0 | No submitted to CWTS #13 |
| December 2002 | 3.0.0 | 3.0.1 | First submission to CWTS #14 |