China Wireless Telecommunication Standard;

3G digital cellular telecommunications system;

User Equipment – Base Station System (UE-BSS) interface;

Data Link (DL) Layer;

General aspects

(Release 3)

CWTS TSM 04.05 v3.1.0 (2002-12)

TSM

Contents

Foreword	3

1	Scope	4

1.1	References	4

1.2	Abbreviations	6

2	Concepts and Terminology	7

3.	Overview description of LAPDm functions and procedures	10

3.1.	General	10

3.2.	Unacknowledged operation	11

3.3.	Acknowledged operation	11

3.4.	Information transfer mode	11

3.4.1.	Information transfer on the Broadcast Channels	11

3.4.2.	Information transfer on the CCCHs	11

3.4.3.	Information transfer on the DCCHs	11

3.5.	Release of data links	11

4.	Service characteristics	12

4.1.	General	12

4.2.	Services provided to layer 3	12

4.2.1.	General	12

4.2.2.	Priority	12

4.2.3.	Segmentation	12

4.2.4.	Unacknowledged information transfer service	13

4.2.5.	Acknowledged information transfer services	13

4.3.	Services required from the physical layer	14

4.4.	Administrative services	15

4.4.1.	General description of administrative services	15

4.4.2.	Definition of primitives for administrative services	15

5.	Overview of data link layer structure	16

5.1.	Functional composition	16

5.2.	Identification of data link end points	16

5.3.	Data link procedure	16

5.4.	Data link distribution procedure	16

6.	Specific requirements	18

6.1.	Mode of operation and allowed SAPIs	18

6.2.	Acknowledged mode of operation	18

6.2.1.	Window size	18

6.2.2.	Processing capacity	18

History	19

# Foreword

This Technical Specification (TS) has been produced by the China Wireless Telecommunication Standard Group (CWTS).

The contents of the present document are subject to continuing work within CWTS and may change following formal CWTS approval. Should CWTS modify the contents of the present document. it will then be re-released by CWTS with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to CWTS for information;

2	presented to CWTS for approval;

3	or greater indicates CWTS approved release.

y	the second digit is incremented for changes of substance, i.e. technical enhancements, corrections, updates, etc. after approval by CWTS of new version within a given release.

z	the third digit is incremented when changes have been incorporated in the document prior approval by CWTS.

# 1	Scope

The present document describes in general terms the Link Access Procedures on the Dm channel, LAPDm. The application of this protocol to other channel types is for further study. Details are provided in TSM 04.06 [5].

The purpose of LAPDm is to convey information between layer 3 entities across the TD-SCDMA radio interface (UE to Network interface) using the Dm channel.

NOTE 1:	The term Dm channel is used for convenience to designate the collection of all the various signalling channels required in the TD-SCDMA system (see TSM 04.03 [3]).

The definition of LAPDm is based on the principles and terminology of:

-	CCITT Recommendations X.200 and X.210: the reference model for Open Systems Interconnection (OSI) [15 - 16];

-	CCITT Recommendations Q.920 and Q.921: the specification of LAPD for the user-network interface in ISDN [17 - 18];

-	CCITT Recommendation X.25 LAPB: user-network interface for packet mode terminals [15];

-	ISO 3309 and ISO 4335: High-level Data Link Control (HDLC) standards for frame structure and elements of procedures [19 - 20].

LAPDm is a protocol that operates at the data link layer of the OSI architecture. LAPDm exchanges information with layer 3 via Logical Channels and towards the physical layer via Transport Channels.

The relationship between the data link layer and other protocol layers is defined below..

NOTE 2:	The interface between the user equipment and external terminal equipment/terminal adapters is defined in the listed Technical Specifications: TS 27.001 [10], TS 27.002 [11], TS 27.003 [12] and TS 27.005 [13].

NOTE 3:	The physical layer on the radio interface is defined in TSM 04.04 [4] and layer 3 is defined in TS 24.007 [6], TS 24.010 [8], TSM 04.11 [9] and TSM 04.08 [7]. Reference should be made to these Technical Specifications for the complete definitions of the protocols and procedures across the TD-SCDMA radio interface (Uu).

NOTE 4:	The term "data link layer" is used in the main text of this Technical Specification. However, mainly in figures and tables, the terms "layer 2" and "L2" are used abbreviations. Furthermore, in accordance with TS 24.007 [6] and TSM 04.08 [7] the term "layer 3" is used to indicate the layer above the data link layer.

NOTE 5:	The relationship between Logical and Transport channels is defined in TSM 05.02 [21].

LAPDm is independent of the transmission bit rate. It requires physical channels with characteristics as defined in TSM 04.03 [3].

Section 2 below describes basic concepts used in this Technical Specification and TSM 04.06 [5].

Section 3 gives an overview description of LAPDm functions and procedures.

Section 4 summarizes the services that the data link layer provides to layer 3 and the services that the data link layer requirs from the physical layer.

Section 5 provides an overview of the data link layer structure.

## References

The following documents contain provisions, which through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.

- For this Release 3 document, references to TSM documents are for Release 3 versions (version 3.y.z).

- References to GSM documents are for Release 1999 versions unless otherwise specified.

- References to 3GPP documents are for Release 1999 versions.

[1]	TSM 01.04: "Abbreviations and acronyms".

[2]	GSM 04.01: "Mobile Station - Base Station System (MS - BSS) interface; General aspects and principles".

[3]	TSM 04.03: "User Equipment - Base Station System (UE - BSS) interface Channel structures and access capabilities".

[4]	TSM 04.04: "Layer 1 General requirements".

[5]	TSM 04.06: "User Equipment - Base Station System (UE - BSS) interface; Data Link (DL) layer specification".

[6]	3GPP TS 24.007: "Mobile radio interface signalling layer 3  General aspects".

[7]	TSM 04.08: "Mobile radio interface layer 3 specification; Part 1: Radio Resource Management".

[8]	3GPP TS 24.010: "Mobile radio interface layer 3  Supplementary services specification  General aspects".

[9]	TSM 04.11: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[10]	3GPP TS 27.001: "General on Terminal Adaptation Functions (TAF) for Mobile Stations (MS)".

[11]	3GPP TS 27.002: "Terminal Adaptation Functions (TAF) for services using asynchronous bearer capabilities".

[12]	3GPP TS 27.003: "Terminal Adaptation Functions (TAF) for services using synchronous bearer capabilities".

[13]	3GPP TS 27.005: "Use of Data Terminal Equipment - Data Circuit terminating Equipment (DTE - DCE) interface for Short Message Service (SMS) and Cell Broadcast Service (CBS)".

[14]	CCITT Recommendation X.25: "Interface between data terminal equipment (DTE) and data circuit - terminating equipment (DCE) for terminals operating in the packet mode and connected to public data networks by dedicated circuit".

[15]	CCITT Recommendation X.200: "Reference Model of Open Systems Interconnection for CCITT Applications".

[16]	CCITT Recommendation X.210: "Open Systems Interconnection layer service definition conventions".

[17]	CCITT Recommendation Q.920: "Integrated services digital network (ISDN) user-network interface - Data link layer  General aspects".

[18]	CCITT Recommendation Q.921: "Integrated services digital network (ISDN) user-network interface - Data link layer specification".

[19]	ISO 3309: "Information technology - Telecommunications and information exchange between systems - High level data link control (HDLC) procedures - Frame structure".

[20]	ISO/IEC 4335: "Information technology - Telecommunications and information exchange between systems - High level data link control (HDLC) procedures - Elements of procedures".

[21]	TSM 05.02 "Multiplexing and multiple access on the radio path".

## Abbreviations

Abbreviations used in the present document are listed in TSM 01.04 [1].

# Concepts and Terminology

The general layering principles used in the present document and other specifications in the 04 series are given in GSM 04.01 [2].

The data link layer is the next to lowest layer of the OSI reference model. The data link layer receives services from the physical layer and provides services to layer 3.

The services provided by the data link layer are the combination of the services and functions provided by both the data link layer and the physical layer.

A data link layer Service Access Point (SAP) is the point at which the data link layer provides services to layer 3. The Service Access Point is identified by a Service Access Point Identifier (SAPI). One or more data link connection endpoints can be associated with each data link layer SAP (See figure 1/TSM 04.05). A data link connection endpoint is identified by a data link connection endpoint identifier (as seen from layer 3) and by a Data Link Connection Identifier DLCI (as seen from the data link layer).

SAPIs and DLCIs used by LAPDm are defined in sub-clause 5.2.

Figure 1/TSM 04.05: Entities, service access points and endpoints

Co-operation between data link layer entities is governed by a peer-to-peer protocol specific to the layer. For information exchange between two or more layer 3 entities, an association must be established between the layer 3 entities in the data link layer using a data link layer protocol. This association is called a data link connection. Data link connections are provided by the data link layer between two or more SAPs (see figure 2/TSM 04.05).

Figure 2/TSM 04.05: Peer-to-peer relationship

Data link layer message units are conveyed between data link layer entities by means of physical connection.

Layer 3 requests services from the data link layer via service primitives. The same applies for the interaction between the data link layer and the physical layer. The primitives represent, in an abstract way, the logical exchange of information and control between the data link layer and its adjacent layers. They do not specify or constrain implementations.

The primitives that are exchanged between the data link layer and adjacent layers are of the following four types (see also figure 3/TSM 04.05).

Figure 3/TSM 04.05: Primitive action sequence

NOTE:	The same principle applies for data link layer - physical layer interactions.

The REQUEST primitive type is used when a higher layer is requesting a service from the next lower layer.

The INDICATION primitive type is used by a layer providing a service to notify the next higher layer of activities related to the primitive type REQUEST.

The RESPONSE primitive type is used by a layer to acknowledge receipt, from a lower layer, of the primitive type INDICATION.

The CONFIRM primitive type is used by the layer providing the requested service to confirm that the activity has been completed.

The precise specification of Layer-to-layer interactions is given in TSM 04.06 [5].

Information between peer entities and between entities in adjacent layers attached to the same SAP is transferred in two different types of message units:

- message units of a peer-to-peer protocol; and

- message units that contain layer-to-layer information concerning status and specialized service requests.

The message units of the layer 3 peer-to-peer protocol are carried by the data link connection. The message units containing layer-to-layer information concerning status and specialised service requests are never conveyed over a data link connection or a physical connection.

This Technical Specification introduces (see also figure 4/TSM 04.05):

a)	the peer-to-peer protocol for the transfer of information and control between any pair of data link layer service access points;

b)	the interactions between the data link layer and layer 3, and between the data link layer and the physical layer.

Figure 4/TSM 04.05: Data link layer reference model

# Overview description of LAPDm functions and procedures

## General

The purpose of LAPDm is to convey information between layer 3 entities across the TD-SCDMA radio interface using the Dm channel. Specifically LAPDm will support:

- Multiple layer 3 entities;

- Multiple physical layer entities;

- Broadcast  CHannel signalling, as here reported (see TSM 04.03 [3]):

i)	Broadcast Control CHannel (BCCH): it is downlink only;

ii)	Cell Broadcast Control CHannel (CBCH): it is downlink only and it is used to carry the Short Message Service Cell Broadcast (SMSCB).

- Common Control CHannels (CCCHs) signalling, as here reported (see TSM 04.03 [3]):

- Paging CHannel (PCH): it is downlink only, used to page mobiles;

- Access Grant CHannel (AGCH): downlink only, used to allocate a signalling only channel (FACCH) or directly a TCH;

- Notification CHannel (NCH): it is downlink only, used to notify mobile stations of Voice Group Call Service(VGCS) and Voice ìBroadcast Service (VBS) calls.

- Dedicated Control CHannels (DCCHs) signalling, as here reported (see TSM 04.03 [3]):

- Slow Associated Control CHannel (SACCH);

- Fast Associated Control CHannel (FACCH).

NOTE 1:	The Random Access CHannel (RACH), the Handover Access CHannel (HOACH), the Frame Number Information CHannel (FNICH) and the Forward Access Channel (FACH) do not utilise LAPDm; the data link layer does not act as a protocol interface and dedicated primitives are used between layer 3 and the physical layer.

The frame structure of data link layer messages is defined in TSM 04.06 [5].

The Dm channel between a network and a specific UE may be distorted on several control channels, e.g. PCH and FACCH during a connection (See also TSM 04.03 [3]). Selection and activation of these channels is performed by layer 3.

LAPDm includes functions for:

- the provision of one or more data link connections on a Dm channel. Discrimination between the data link connections is by means of a data link connection identifier (DLCI);

- allowing recognition of frame types;

- allowing layer 3 message units to be passed transparently between layer 3 entities;

- sequence control, to maintain the sequential order of frames across a data link connections;

- detection of format and operational errors on a data link;

- notification to the layer 3 entity of unrecoverable errors;

NOTE 2:	It is in the responsibility of layer 3 entity to recover from these errors;

- flow control; and

- contention resolution when establishing a data link after an access request has been made on the RACH.

Two types of operation of the data link layer are defined for layer 3 information transfer: unacknowledged operation and acknowledged (multiple frame) operation. They may co-exist on a Dm channel.

The Broadcast CHannels (BCCH and CBCH) and the CCCHs (PCH, AGCH and NCH) will only support unacknowledged operation. The DCCHs (SACCH and FACCH) will support both types of operation.

## Unacknowledged operation

In unacknowledged operation, layer 3 information is transmitted in Unnumbered Information (UI) frames.

At the data link layer, the UI frames are not acknowledged. Flow control mechanisms and error recovery mechanisms are not defined.

Applicability of unacknowledged operation to different types of control channels is specified in sub-clause 6.1.

## Acknowledged operation

In acknowledged operation, layer 3 information is transmitted in frames that are acknowledged by the receiving data link layer.

Error recovery procedures based on retransmission of unacknowledged frames are specified. In case of errors, which cannot be corrected by the data link layer, a report is issued to the layer 3 entity. Flow control procedures are also defined.

Applicability of acknowledged operation to different types of control channels is specified in sub-clause 6.1.

Only one form of acknowledged information transfer is defined, i.e. multiple frame operation.

For multiple frame operation, layer 3 information is sent in numbered Information (I) frames. In principle, a number of I frames may be outstanding at the same time. However, for many applications (e.g. signalling) a window size of 1 is required. Multiple frame operation is initiated by a multiple frame establishment procedure using a Set Asynchronous Balanced Mode (SABM) command.

If an access request has been made on the RACH, the establishment procedure also contains functions for resolving any ambiguity that may arise as a result of this access method.

## Information transfer mode

### Information transfer on the Broadcast Channels

The BCCH and the CBCH exist only in the network to UE direction and are used for broadcasting radio sub-system information to UEs. Only UI frames are sent on these channels.

### Information transfer on the CCCHs

The PCH, AGCH and NCH channels exist only in the network to UE direction. Only UI frames are sent on these channels.

### Information transfer on the DCCHs

The SACCH and FACCH channels exist in both directions (Network to UE and UE to Network). Both unacknowledged operation (UI frames) and multiple frame operation (I frames) are possible on these channels (see sub-clause 6.1). The type of operation required at any time is determined by layer 3.

## Release of data links

Multiple frame operation may be released in the following ways:

- normal release by exchange of commands/responses. This type of release is initiated by layer 3;

- local end release, i.e. without exchange of commands/responses, initiated and controlled by layer 3;

- abnormal local end release, i.e. without exchange of commands/responses, commanded by layer 3.

The release mode is indicated by layer 3.

No release mechanism using exchange of commands/responses is defined for unacknowledged operation.

# Service characteristics

## General

The data link layer provides services to layer 3 and utilizes the services provided by the physical layer.

In this Technical Specification and TSM 04.06 [5] the following general syntax is used for describing primitives.

XX - Generic Name - Type (Parameters).

where XX designates the layer providing the services. In this Technical Specification XX is DL and MDL for the data link layer and PH for the physical layer.

## Services provided to layer 3

### General

The specification of the interactions with layer 3 (primitives) provides a description of the services that the data link layer, plus the physical layer, offer to layer 3, as viewed from layer 3.

Two forms of information transfer services are associated with layer 3. The first is based on unacknowledged information transfer at the data link layer and the second service is based on acknowledged information transfer at the data link layer using multiple frame operation. Different information transfer services may co-exist on the same data link subject to restrictions imposed by the type of channel being used (see sub-clause 3.4).

### Priority

The priority between data links shall be as follows:

On FACCH:

Highest priority : SAPI = 0;

Lowest priority : SAPI = 3.

On SACCH:

The priority arrangement on the SACCH must ensure that if a SAPI = 3 frame is awaiting transmission, two SAPI = 0 frames are not sent in consecutive SACCH frames. In addition, for the mobile to network direction it must also be ensured that any SAPI = 3 frame is followed by at least one SAPI = 0 frame.

### Segmentation

For the acknowledged mode of information transfer the data link layer offers segmentation at the transmitter of layer 3 message units if the message unit is longer than the information field of the data layer frames. At the receiver the segmented layer 3 message units are concatenated such that the integrity of the layer 3 message unit is restored.

For unacknowledged operation the data link layer does not offer segmentation services.

### Unacknowledged information transfer service

NOTE:	In this case the information transfer is not acknowledged at the data link layer. Acknowledgement procedures may be provided at higher layers.

The characteristics of the unacknowledged information transfer service are summarized in the following:

- provision of a data link connection between layer 3 entities for unacknowledged information transfer of layer 3 message units;

- identification of data link connection endpoints to permit a layer 3 entity to identify another layer 3 entity;

- sending of frames in accordance with priority given to the message;

- no verification of message arrival within the data link layer.

The primitives associated with the unacknowledged information transfer service are:

DL-UNIT DATA-REQUEST/INDICATION

The DL-UNIT DATA-REQUEST primitive is used to request that a message unit be sent using the procedures for unacknowledged information transfer service; DL-UNIT DATA-INDICATION indicates the arrival of a message unit received by means of unacknowledged information transfer. Parameters associated with these primitives are the message unit, priority and the type of channel being used (BCCH, CBCH, PCH, AGCH, NCH or specific type of DCCHs).

### Acknowledged information transfer services

One mode of acknowledged operation is defined, i.e. multiple frame operation.

The characteristics of this service are summarized in the following:

- provision of a data link connection between layer 3 entities for acknowledged information transfer of layer 3 message units;

- identification of data link connection endpoints to permit a layer 3 entity to identify another layer 3 entity;

- sequence integrity of data link layer message units in the absence of machine malfunctions;

- notification to the peer entity in the case of errors, for example, loss of sequence;

- notification to the layer 3 entity of unrecoverable errors detected by the data link layer;

- flow control;

- sending of frames in accordance with the indicated SAPI value (see also sub-clause 4.2.2);

- segmentation and concatenation control functions;

- suspension of the service during change of dedicated channels and resumption of service on the new channel without message loss (SAPI = 0 only); duplication of messages which may occur are treated on layer 3 (SAPI = 0 only).

The following list gives an overview of the primitives associated with the multiple frame acknowledged information transfer services (for detailed specification, see TSM 04.06 [5]):

- Data transfer using I frames:

DL-DATA-REQUEST/INDICATION

The DL-DATA-REQUEST primitive is used to request that a message unit be sent using the procedures for multiframe acknowledged information transfer. The DL-DATA-INDICATION primitive indicates the arrival of a message unit received by means of acknowledged information transfer. The parameters associated with these primitives are the message unit and the type of channel being used.

- Establishment of multiple frame operation using the SABM command:

DL-ESTABLISH-REQUEST/INDICATION/CONFIRM

These primitives are used to request, indicate and confirm the establishment of multiple frame operation between two data link layer entities. Possible parameters are the message unit, the establish mode and the type of channel being used.

- Suspension of multiple frame operation:

DL-SUSPEND-REQUEST/CONFIRM

These primitives are used in a UE to request and confirm the suspension of multiple frame operation while changing a dedicated channel. A possible parameter is the type of channel to be affected.

- Resumption of multiple frame operation:

DL-RESUME-REQUEST/CONFIRM

These primitives are used in a UE to request and confirm the resumption of multiple frame operation after it has been suspended (see paragraph c) above). Possible parameters are the message unit and the type of channel to be affected.

- Restoration of multiple frame operation:

DL-RECONNECT-REQUEST/CONFIRM

These primitives are used in a UE to request and confirm the restoration of multiple frame operation on the old channel after failure of the channel change. Possible parameters are the message unit and the type of channel to be affected.

- Termination of multiple frame operation:

DL-RELEASE-REQUEST/INDICATION/CONFIRM

These primitives are used to request, indicate and confirm an attempt to terminate multiple frame operation between two data link layer entities or an attempt to perform local end release. The parameters associated with this primitive are the type of channel and the release mode.

## Services required from the physical layer

The services provided by the physical layer are described in detail in TSM 04.04 [4]. They are summarized in the following:

- physical layer connection for transparent transmission of frames. The bits of a frame are to be delivered to the peer data link entity in the same order in which they were submitted to the physical layer by the sender;

- indication of the physical status of the Dm channel;

- transmission of data link layer message units in the same order as they were issued by the data link layer;

- provision of frame synchronization;

- provision of error protection to ensure a low residual bit error rate at the data link layer.

The primitives between the data link layer and the physical layer are:

- Data transfer:

PH-DATA-REQUEST/INDICATION

These primitives are used to request that a message unit be sent and to indicate the arrival of message unit. Parameters associated with these primitives are the data link layer message unit, the priority and the type of channel being used.

- Connection establishment:

PH-CONNECT-INDICATION

This primitive is used to indicate that a specific physical resource has been established on the physical layer. The parameter associated with this primitive is the type of channel.

NOTE:	Activation of a physical resource is usually initiated by the layer 3 entity without involving data link layer entities.

- Transmission synchronization:

PH-READY-TO-SEND-INDICATION

This primitive is used to enable the data link layer to synchronize to the next instant of physical transmission. The parameter associated with this primitive is the type of channel.

PH-EMPTY-FRAME-REQUEST

This primitive is used by the data link layer instead of the PH-DATA-REQUEST primitive when no frame has to be sent after receiving the PH-READY-TO-SEND indication. The parameter associated with this primitive is the type of channel.

## Administrative services

### General description of administrative services

The data link layer entity supports several internal functions of the UE or the network not requiring layer 3 peer-to-peer information transfer. The functions provided by the data link layer are:

- error reporting between the data link layer and the layer 3 entity;

- abnormal release of the data link layer in case of protocol or other failures from which the data link layer cannot recover on its own.

The administrative functions and the interactions between the data link layer and the layer 3 entities are described in terms of service primitives.

### Definition of primitives for administrative services

The primitives between the layer 3 entity and the data link layer for supporting administrative services are:

- Error handling:

MDL-ERROR-INDICATION

This primitive is used by the data link layer to indicate that there is an error in the data link layer procedures that cannot be resolved by normal exception handling procedures. Parameters associated with this primitive are the reason for error reporting and the type of channel.

- Release:

MDL-RELEASE-REQUEST

This primitive is used by the layer 3 entity to initiate abnormal local end release of a data link. Parameters associated with this primitive are indications of which data links are to be released and the reason for abnormal release.

# Overview of data link layer structure

## Functional composition

Figure 5/TSM 04.05 is an example of a functional block diagram of the data link layer in the UE. In the example the data link connection for all physical channels terminates at the SAP identified by SAPI = 0 but only the data link connection for a SACCH and a FACCH terminates at the SAP identified by SAPI = 3. Other arrangements are possible depending on the capabilities of the UE.

The network shall contain a similar arrangement with one CCCH (PCH + AGCH), DCCH (SACCH + FACCH), as required, for each active UE.

Figure 5/TSM 04.05 illustrates two procedural types: the data link procedure and the data link distribution procedure.

## Identification of data link end points

The data link endpoints are identified by a Data Link Connection Identifier (DLCI).

The DLCI consists of two elements:

- the Service Access Point Identifier (SAPI) which is carried in the address field of each frame;

- the type of control channel on which the data link connection is or is to be established. This information is not carried in frames between data link layer peer entities but is managed locally in each end system and is carried in primitives between the layers.

When a layer 3 message unit is to be sent, layer 3 will select the appropriate SAP and data link connection end point. Layer 3 will indicate to the data link layer which data link connection end point has been chosen.

When receiving a frame containing a layer 3 message unit, the data link layer will receive from the physical layer an indication concerning the type of channel on which the frame was received. This information together with the SAPI contained in the frame enables the data link layer to deliver the layer 3 message unit to the required data link connection end point of the indicated SAP.

The SAP takes a specific value for each of the following functions carried on the Dm channel:

- call control signalling, mobility management signalling, supplementary services signalling and radio resource management signalling information as defined in TSM 04.08 [7] and TS 24.010 [8]: SAPI = 0;

- short message services as defined in TSM 04.11 [9]: SAPI = 3.

Other functions requiring specific SAPI values may be defined in the future.

## Data link procedure

There is at most one instance of the data link layer procedure for each SAPI on each type of channel supported on that SAPI.

For some combinations of SAPI and type of channel only a subset (e.g. unacknowledged operation) of the overall data link layer procedure is required.

The procedure analyses the control field and the length indicator field of the received frame (see TSM 04.06 [5]) and provides appropriate peer-to-peer responses and layer-to-layer indications. In addition, it analyses the data link layer service primitives and transmits the appropriate peer-to-peer commands and responses.

The procedure also performs segmentation and concatenation of layer 3 message units.

## Data link distribution procedure

This procedure is only required if there are more than one SAPI supported on a channel associated with a specific UE.

The procedure analyses the address field of a received frame and the type of physical channel contained in the primitive received from the physical layer. It then distributes the frames to the appropriate data link procedure block.

On frame transmission, the procedure delivers the frames to the required channel after layer 3 has established the respective association between the channel and its physical parameters. The procedure also provides for resolution of conflicts between the various data link procedure blocks on the same physical channel. The conflict resolution is based on the SAPI and the priority requested by layer 3 (see sub-clause 4.2.2).

Figure 5/TSM 04.05: Example of the data link layer configuration in the UE

# Specific requirements

## Mode of operation and allowed SAPIs

The various types of channels shall support SAPIs and modes of operation as follows:

| Logical Channels | Transport Channels | SAPI = 0 | SAPI = 3 |
| --- | --- | --- | --- |
| Broadcast Channel: |  |  |  |
| BCCH | BCCH-T | Unacknowledged | Not supported |
| CBCH | CCCH-T | Unacknowledged | Not supported |
| CCCHs: |  |  |  |
| PCH | CCCH-T | Unacknowledged | Not supported |
| AGCH |  |  |  |
| NCH |  |  |  |
| DCCHs: |  |  |  |
| FACCH | FACCH-T | Unacknowledged and acknowledged | Acknowledged |
|  | SDCCH-T | Unacknowledged and acknowledged | Acknowledged |
| SACCH | SACCH-T | Unacknowledged | Acknowledged |
|  | CSACCH-T | Unacknowledged | Not supported |
|  | SDCCH-T | Unacknowledged | Not supported |

## Acknowledged mode of operation

### Window size

The window size, k (see TSM 04.06 [5]), shall be:

-	for SAPI = 0, k = 1;

-	for SAPI = 3, k = 1.

Other SAPIs, for further study.

### Processing capacity

The processing capacity of the UE and the network shall be big enough to avoid that the data link layer entities enter the receiver busy state for SAPI = 0.

# History

| Document history |  |  |  |
| --- | --- | --- | --- |
| DATE | OLD_VERS | NEW_VERS | SUBJECT |
| May 2000 | 0.0.1 | 0.0.1 | First submission to CWTS #6 |
| December 2000 | 0.0.1 | 0.0.2 | Editorial Changes |
| December 2000 | 0.0.2 | 1.0.0 | Published by CWTS #8 |
| July 2001 | 1.0.0 | 2.0.0 | Approved by CWTS #10 |
| December 2001 | 2.0.0 | 2.0.1 | Second submission to CWTS #11 |
| December 2001 | 2.0.1 | 2.1.0 | Approved by CWTS #11 |
| August 2002 | 2.1.0 | 2.1.1 | Third submission to CWTS #13 |
| August 2002 | 2.1.1 | 3.0.0 | Approved by CWTS #13 |
| December 2002 | 3.0.0 | 3.0.1 | Fourth submission to CWTS #14 |