CWTS TSM 04.60 v 3.1.0 (2002-12)

China Wireless Telecommunication Standard;

3G digital cellular telecommunications system;

TD-SCDMA Packet Radio Service (TD-PRS);

User Equipment (UE) - Base Station System (BSS) interface; Radio Link Control/ Medium Access Control (RLC/MAC) protocol

(Release 3)

TSM

- Contents

Foreword	7

1	Scope	8

2	References	9

3	Definitions and abbreviations	10

3.1	Vocabulary	10

4	Layered overview of radio interface	11

4.1	Layer services	12

4.2	Layer functions	12

4.3	Service primitives	13

4.4	Services required from lower layers	13

4.5	Packet data Physical CHannels (PDPCH)	13

4.5.1	Pilot – Packet Data Physical Channel (P-PDPCH)	13

4.5.2	Shared – Packet Data Physical Channel (S-PDPCH)	13

5	Introduction to the Medium Access Control (MAC) procedures	14

5.1	General	14

5.2	Multiplexing principles	14

5.2.1	Temporary Block Flow	14

5.2.2	Temporary Flow Identity	14

5.2.3	User State Flag	14

5.2.4	Time Slot State Flag	14

5.2.5	Medium Access modes	15

5.3	Packet idle mode	15

5.4	Packet transfer mode	15

5.5	General procedures in packet idle and packet transfer modes	16

5.5.1	User equipment side	16

5.5.1.1	Cell reselection	16

5.5.1.2	Spare	16

5.5.1.3	System information on BCCH	16

5.5.1.3.1	Supervision of BCCH_CHANGE_MARK and update of BCCH information	17

5.5.1.3.2	SI13 reception failure	17

5.5.1.4	Acquisition of system information on the broadcast channel	17

5.5.1.4.1	Consistent sets of system information messages	18

5.5.1.4.2	Suspension of operation to receive system information	18

5.5.1.4.3	Request for acquisition of system information	18

5.5.1.5	Discontinuous reception (DRX)	18

5.5.1.6	Frequency Parameters	19

5.5.1.7	TLLI management	19

5.5.2	Network side	19

5.5.2.1	System Information broadcasting	19

5.5.2.1.1	Spare	19

5.5.2.1.2	System information on BCCH	19

5.5.2.1.3	System information on PACCH (and other logical channels)	20

5.5.2.1.4	Consistent sets of system information messages	20

5.6	Measurement reports	20

5.6.1	Network Control (NC) measurement reporting	20

5.6.2	Extended measurement (EM) reporting	22

5.6.3	Measurements on Demand	22

5.6.4	Measurement Reporting Priority	22

6	Paging procedures	23

6.1	Paging procedure for RR connection establishment	23

6.1.1	Paging initiation using paging sub-channel on CCCH	23

6.1.2	Paging initiation using PACCH	23

6.1.3	Paging response	23

6.2	Paging procedure for downlink packet transfer	24

6.2.1	Paging procedure using paging sub-channel on CCCH	24

6.2.2	Paging response	24

7	Medium Access Control (MAC) procedures when entering the packet transfer mode	24

7.1	TBF establishment initiated by the user equipment	25

7.1.1	Permission to access the network	25

7.1.2	Initiation of a TBF establishment	25

7.1.2.1	Initiation of the packet access procedure	25

7.1.2.2	Packet assignment procedure	26

7.1.2.2.1	On receipt of a CHANNEL REQUEST message	26

7.1.2.2.2	Packet access reject procedure	27

7.1.2.3	Pilot-PDPCH Packet access procedure	27

7.1.2.3.1	Maintenance of P-PDCHs and Radio Link Failure management	29

7.1.2.3.2	Timing Advance	29

7.1.2.3.3	Power control	29

7.1.2.3.4	Beam-Forming	30

7.1.2.4	Contention resolution for one phase access procedure	30

7.1.2.4.1	One phase packet access procedure completion	30

7.1.3	TBF establishment using two phase access	30

7.1.3.1	Initiation of the Packet resource request procedure	30

7.1.3.2	Packet resource assignment for uplink procedure	31

7.1.3.2.1	On receipt of a PACKET RESOURCE REQUEST message	31

7.1.3.3	Contention resolution at two phase access	31

7.1.3.4	Two phase packet access completion	32

7.1.4	Abnormal cases	32

7.1.5	Timing Advance on S-PDPCHs	32

7.1.6	Power control on S-PDPCHs	33

7.1.7	Beam-forming on S-PDPCHs	33

7.2	TBF establishment initiated by the network	33

7.2.1	Entering the packet transfer mode	33

7.2.1.1	Packet downlink assignment procedure	33

7.2.1.2	Packet polling procedure	34

7.2.2	Abnormal cases	35

7.3	Procedure for measurement report sending in packet idle mode	35

7.3.1	Measurement report sending procedure	35

7.4	Cell Change Order procedures in Packet Idle mode	36

7.4.1	Cell Change Order procedure initiated on CCCH	36

7.5	Measurement Order procedures in Packet Idle mode	36

7.5.1	Measurement Order procedures initiated on CCCH	36

8	Medium Access Control (MAC) Procedures in Packet Transfer Mode	37

8.1	RLC data blocks transfer	37

8.1.1	Uplink RLC data block transfer	38

8.1.1.1	Uplink RLC data block transfer on a P-PDCH	39

8.1.1.1.1	PACCH operation	40

8.1.1.1.2	Uplink Resource reallocation	40

8.1.1.2	Dynamic allocation uplink RLC data block transfer	41

8.1.1.2.1	PACCH operation	42

8.1.1.2.2	Neighbour cell measurements	42

8.1.1.2.3	Uplink Resource Reallocation	42

8.1.1.3	Fixed Allocation uplink RLC data block transfer	44

8.1.1.3.1	Transfer of RLC/MAC blocks	44

8.1.1.3.2	Uplink Resource Reallocation for open-ended TBF	45

8.1.1.3.3	Neighbour cell measurements	48

8.1.1.3.4	PACCH operation	48

8.1.1.4	Establishment of a Downlink TBF	49

8.1.1.4.1	Abnormal cases	49

8.1.1.5	Network initiated release of an uplink TBF	50

8.1.1.6	Abnormal cases	50

8.1.2	Downlink RLC data block transfer	51

8.1.2.1	Downlink RLC data block transfer on a P-PDPCH	51

8.1.2.1.1	PACCH operation	52

8.1.2.1.2	Downlink Resource reassignment	52

8.1.2.2	Downlink RLC data block transfer on a S-PDCH	53

8.1.2.2.1	PACCH operation	53

8.1.2.2.2	Downlink Resource Reassignment	53

8.1.2.3	Void	54

8.1.2.4	Establishment of uplink TBF	54

8.1.2.4.1	Abnormal cases	55

8.1.2.5	Network initiated abnormal release of a downlink TBF	55

8.1.2.6	Polling for Packet Downlink Ack/Nack	55

8.1.2.7 Abnormal cases	56

8.1.3	Concurrent TBF procedures for half duplex operation	56

8.1.3.1	Saving downlink TBF state and initiating uplink TBF	57

8.1.3.2	Saving downlink TBF state and restoring uplink TBF state	57

8.1.3.3	Ending downlink TBF and restoring uplink TBF state	57

8.1.3.4	Saving uplink TBF state and initiating downlink TBF	57

8.1.3.5	Saving uplink TBF state and restoring downlink TBF state	57

8.1.3.6	Ending uplink TBF and restoring downlink TBF state	57

8.2	Release of packet radio resources	57

8.3	Procedure for measurement report sending in Packet Transfer mode	58

8.4	Cell Change procedures in Packet Transfer mode	58

8.4.1	Network controlled cell reselection completion	59

8.4.2	Abnormal cases	59

8.5	Measurement Order procedures in Packet Transfer mode	59

8.6	PACKET CONTROL ACKNOWLEDGEMENT	60

8.7	Inherited PDPCHs management	60

8.8	Abnormal cases	60

8.8.1	Abnormal release with return to CCCH	61

8.8.2	Abnormal release with random access	61

8.8.3	Abnormal release with system information	61

9	Radio Link Control (RLC) procedures in packet transfer mode	61

9.1	Procedures and parameters for peer-to-peer operation	61

9.1.1	Send state variable V(S)	62

9.1.2	Control send state variable V(CS)	62

9.1.3	Acknowledge state variable V(A)	62

9.1.4	Acknowledge state array V(B)	62

9.1.5	Block sequence number BSN	63

9.1.6	Reduced Block Sequence Number RBSN	63

9.1.7	Receive state variable V(R)	63

9.1.8	Receive window state variable V(Q)	63

9.1.9	Receive state array V(N)	63

9.1.10	Reference sequence number (RSN) and received block bitmap (RBB)	63

9.1.11	Window Size	64

9.1.12	Segmentation of LLC PDUs into RLC data units	64

9.1.13	Re-assembly of LLC PDUs from RLC data units	65

9.1.14	Segmentation of RLC/MAC control messages into RLC/MAC control blocks	65

9.1.15	Re-assembly of RLC/MAC control messages from RLC/MAC control blocks	65

9.1.16	Priority of LLC PDUs	66

9.2	Operation during RLC/MAC control message transfer	66

9.3	Operation during RLC data block transfer	67

9.3.1	Countdown procedure	67

9.3.2	Acknowledged mode operation	67

9.3.2.1	Establishment of Temporary Block Flow	68

9.3.2.2	Operation of uplink Temporary Block Flow	68

9.3.2.3	Release of uplink Temporary Block Flow	68

9.3.2.4	Operation of downlink Temporary Block Flow	70

9.3.2.5	Release of downlink Temporary Block Flow	70

9.3.3	Unacknowledged mode operation	72

9.3.3.1	Establishment of Temporary Block Flow	72

9.3.3.2	Operation of uplink Temporary Block Flow	72

9.3.3.3	Release of uplink Temporary Block Flow	72

9.3.3.4	Operation of downlink Temporary Block Flow	75

9.3.3.5	Release of downlink Temporary Block Flow	75

9.4	Abnormal release cases	76

9.4.1	Abnormal release with random access	76

9.4.2	Abnormal release with cell reselection	76

10	RLC/MAC block structure	76

10.1	Spare bits	77

10.2	RLC data blocks	77

10.2.1	Downlink RLC data block	77

10.2.2	Uplink RLC data block	78

10.3	RLC/MAC control blocks	79

10.3.1	Downlink RLC/MAC control block	79

10.3.2	Uplink RLC/MAC control block	79

10.4	Header fields	79

10.4.1	User state flag (USF)/ Measurement on demand Command (MC) field	79

10.4.2	Time Slot State Flag (TS-SF) field	80

10.4.3	Transmit Power Command field	80

10.4.4	Retry (R) bit	81

10.4.5	Stall indicator (SI) bit	81

10.4.6	Supplementary/Polling (S/P) Bit	81

10.4.7	Relative Reserved Block Period (RRBP) field	81

10.4.8	Countdown Value (CV) field	82

10.4.9	Payload Type field	82

10.4.10	Final block indicator (FBI) bit	83

10.4.11	TS-SF Indicator (TS-SFI) bit	83

10.4.12	Transmit Power command Indicator (PCI) bit	83

10.4.13	TLLI Indicator (TI) bit	83

10.4.14	Address Control (AC) bit	83

10.4.15	Final Segment (FS) bit	84

10.4.16	Radio Transaction Identifier (RTI) field	84

10.4.17	Direction (D) bit	84

10.4.18	Temporary Flow Identity (TFI) field	84

10.4.19	Extension (E) Bit	84

10.4.20	Block Sequence Number (BSN) field	84

10.4.21	Reduced Block Sequence Number (RBSN) bit	85

10.4.22	More (M) bit	85

10.4.23	Length Indicator (LI) field	85

10.4.24	TLLI field	85

10.4.25	RLC data field	86

10.4.26	Control message contents field	86

11	Message functional definitions and contents	87

11.1	Handling of erroneous protocol data	87

11.1.1	Message classification	88

11.1.1.1	Distribution messages	88

11.1.1.2	Non-distribution messages	88

11.1.1.2.1	Format of the address information	88

11.1.2	Error detection mechanism	89

11.1.3	Error labels	89

11.1.3.1	Generic error labels	89

11.1.3.2	‘Ignore’ error label	89

11.1.3.3	‘Message escape’ error label	90

11.1.4	Error detection and order of precedence	90

11.1.4.1	Unknown message type	90

11.1.4.2	Message not compatible with current protocol state	90

11.1.4.3	Syntactically incorrect message	91

11.1.4.3.1	Messages with error label: ‘Distribution part error’	91

11.1.4.3.2	Messages with error label: ‘Address information part error’	91

11.1.4.3.3	Messages with error label: ‘Non-distribution part error’	91

11.1.4.3.4	Messages with error label: ‘Message escape’	91

11.1.4.3.5	Messages with error label: ‘Ignore’	91

11.1.4.4	Syntactic error in truncated concatenation	91

11.1.4.5	Exceptions	92

11.2	RLC/MAC control messages and RRM messages	92

11.2.1	Message format	93

11.2.1.1	Downlink RLC/MAC messages	93

11.2.1.2	Uplink RLC/MAC messages	94

11.3	Packet Messages UE originated	94

11.3.1	Packet Access	94

11.3.2	Packet Control Acknowledgement	95

11.3.3	Packet Cell Change Failure	97

11.3.4	Packet Downlink Ack/Nack	98

11.3.5	Packet Uplink Dummy Control Block	100

11.3.6	Packet Measurement Report	100

11.3.7	Packet Mobile TBF Status	103

11.3.8	Packet Resource Request	104

11.3.9	Packet PSI Status	105

11.4	Packet Messages Network originated	107

11.4.1	Packet Access Ack	107

11.4.2	Packet Access Reject	107

11.4.3	Packet Cell Change Order	108

11.4.4	Packet Downlink Assignment	112

11.4.5	Packet Downlink Dummy Control Block	115

11.4.6	Packet Measurement Order	116

11.4.7	Packet Resource Release	121

11.4.8	Packet Polling Request	122

11.4.9	Packet Power Control/Timing Control	123

11.4.10	Packet System Information Type 13	125

11.4.11	Packet TBF Release	129

11.4.12	Packet Uplink Ack/Nack	130

11.4.13	Packet Uplink Assignment	133

11.4.14	Packet Resource Reconfigure	137

11.4.15	Packet Paging Request	140

12	Information element coding	142

12.1	Overview	142

12.2	Extension Bits IE	142

12.3	Ack/Nack Description	142

12.4	ALLOCATION_BITMAP	144

12.5	Channel Request Description	144

12.6	Global TFI	145

12.7	Temporary Flow Identity (TFI)	145

12.8	Temporary Logical Link Identity (TLLI)	146

12.9	Time_Slot_State_Flag	146

12.10	PAGE_MODE	146

12.11 Starting Frame Number Description	146

12.11.1	Absolute Frame Number Encoding	146

12.11.2	Relative Frame Number Encoding	147

12.12	Network_Control_Order	147

12.13	NC_NON_DRX_PERIOD	147

12.14	NC_REPORTING_PERIOD_I / NC_REPORTING_PERIOD_T	148

13	Timers and counters	149

13.1	Timers on the UE side	149

13.2	Timers on the network side	154

13.3	Counters on the User Equipment side	156

13.4	Counters on the Network side	157

Annex A (Informative): Packet Resource Release	158

Annex B (informative): RLC data block encoding	160

B.1	Example 1	160

B.2	Example 2	160

B.3	Example 3	161

B.4	Example 4	162

B.5	Example 5	162

B.6	Example 6	163

B.7	Example 7	163

B.8	Example 8	164

B.9	Example 9	164

Annex C (informative): Message Sequence Diagrams	166

History	168

# Foreword

This Technical Specification (TS) has been produced by the China Wireless Telecommunication Standard group (CWTS).

The contents of the present document are subject to continuing work within CWTS and may change following formal CWTS approval. Should CWTS modify the contents of the present document, it will be re-released by CWTS with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to CWTS for information;

2	presented to CWTS for approval;

3	or greater indicates CWTS approved release.

y	the second digit is incremented for changes of substance, i.e. technical enhancements, corrections, updates, etc., after approval by CWTS of new version within a given release.

z	the third digit is incremented when changes have been incorporated in the document prior approval by CWTS.

# 1	Scope

The present document specifies the procedures used at the radio interface (Reference Point Uu, see TS 24.002 [6]) for the TD-SCDMA Packet Radio Service (TD-PRS) Medium Access Control /Radio Link Control (MAC/RLC) layer.

When the notation for "further study" or "FS" or "FFS" is reported, it means that the indicated text is not a normative portion of the present document.

The present document is applicable to the following TD-PRS Uu functional layers:

-	Radio Link Control functions,

-	Medium Access Control functions, and

-	Physical Link Control functions.

The procedures described in the present document are for the RLC/MAC functions of the TD-PRS radio interface (Uu) when operating on a Packet Data CHannel (PDCH).

TSM 03.64 [5] contains an overview of the TD-PRS radio interface (Uu).

TSM 04.03 [7] and TSM 04.04 [8] contains the definition of the control channels used in the present document.

TS 24.007 [10] contains a description in general terms of the structured functions and procedures of this protocol and the relationship of this protocol with other layers and entities.

TSM 04.08 [11] contains the definition of TD-PRS RLC/MAC procedures when operating on the Common Control Channel (CCCH).

GSM 04.64 [12] contains functional procedures for the Logical Link Control (LLC) layer.

Application to interface structure

The RLC/MAC procedures apply to the interface structures defined in TSM 04.03 [7]. They use the functions and services provided by layer 1 defined in TSM 04.04 [8]. TS 24.007 [10] gives the general description of layer 3 including procedures, messages format and error handling.

Test procedures

Test procedures of the radio interface signalling are described in TSM 11.10 [17] and TSM 11.2x series.

Use of logical control channels

The logical control channels are defined in TSM 05.02 [13]. Two similar sets of logical channels are defined. The first set consists of the logical channels:

-	Broadcast Control Channel (BCCH): downlink only, used to broadcast Cell specific information;

-	Paging Channel (PCH): downlink only, used to send page requests to User equipment (UEs);

-	Random Access Channel (RACH): uplink only, used to request packet switched or circuit switched resources;

-	Access Grant Channel (AGCH): downlink only, used to allocate packet switched or circuit switched resources;

The second set consists of the logical channels:

-	Packet Associated Control Channel (PACCH): bi-directional, associated with a Temporary Block Flow (TBF);

where the following ones are FFS:

-	Packet Broadcast Control Channel (PBCCH): downlink only, used to broadcast Cell specific information;

-	Packet Paging Channel (PPCH): downlink only, used to send page requests to User equipment (UEs) in packet switched mode;

-	Packet Random Access Channel (PRACH): uplink only, used to explicitly request packet switched resources;

- Packet Access Grant Channel (PAGCH): downlink only, used to allocate packet switched resources.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.

-	For this Release 3 document, references to TSM documents are for Release 3 versions (version 3.y.z).

-	References to GSM documents are for Release 1999 versions unless otherwise specified.

-	References to 3GPP documents are for Release 1999 versions.

- [1]	TSM 01.04: "Abbreviations and acronyms".

- [2]	3GPP TS 22.060: “General Packet Radio Service (GPRS); Service description, Stage 1”.

- [3]	3GPP TS 23.003: “Numbering, addressing and identification”.

- [4]	GSM 03.13: "Digital cellular telecommunications system (Phase 2+); Discontinuous Reception (DRX) in the GSM system".

- 5	TSM 03.64: "TD-SCDMA Packet Radio Service (TD-PRS); Overall description of TD-PRS radio Interface".

- [6]	3GPP TS 24.002: "GSM – UMTS Public Land Mobile Network (PLMN) access reference configuration".

- [7]	TSM 04.03: "User Equipment - Base Station System (UE - BSS) interface; Channel structures and access capabilities".

- [8]	TSM 04.04: "Layer 1 - General requirements".

- [9]	TSM 04.05: " Data Link (DL) layer General aspects".

- [10]	3GPP TS 24.007: “Mobile radio interface signalling layer 3; General Aspects”.

- [11]	TSM 04.08: "Mobile radio interface layer 3 specification; Part 1: Radio Resource Management".

- 12	GSM 04.64: "General Packet Radio Service (GPRS); Mobile Station - Serving GPRS Support Node (MS-SGSN) Logical Link Control (LLC) Layer Specification ".

- [13]	TSM 05.02: "Multiplexing and multiple access on the radio path".

- [14]	TSM 05.03: "Channel coding".

- [15]	TSM 05.08: "Radio subsystem link control".

- [16]	TSM 05.10: "Radio subsystem synchronisation".

- [17]	TSM 11.10: "User equipment (UE) conformance specification".

- [18]	TSM 11.21: "Base Station System (BSS) equipment specification; Radio aspects".

- [19]	TSM 03.60: “TD-SCDMA Packet Radio Service (TD-PRS); Service description; Stage 2”.

- [20]	3GPP TS 22.011: “Service accessibility”.

- [21]	3GPP TS 24.008: ”Mobile radio interface layer 3 specification; Core Network Protocols; Stage 3”.

- [22]	TSM 03.22: “Functions related to User Equipment (UE) in idle mode and group receive mode”.

- [23]	GSM 01.04: "Abbreviations and acronyms".

- [24]	GSM 05.08: ”Radio subsystem link control”.

- [25]	GSM 04.60: ”General Packet Radio Service (GPRS); Mobile Station (MS) – Base Station System (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol”.

- [26]	TSM 05.05: ”Radio transmission and reception”.

# 3	Definitions and abbreviations

Abbreviations used in the present document are listed in GSM 01.04 [23]; TS 22.060 [2] and TSM 01.04[1].

## 3.1	Vocabulary

The following terms are used in this Technical Specification:

- Block period: A block period is the sequence of 4 radio frames on a PDPCH used to convey one radio block.

- Packet idle mode: In packet idle mode, the user equipment is prepared to transfer LLC PDUs on packet data physical channels (see sub-clause 5.3). The user equipment is not allocated any radio resource on a packet data physical channel; it listens to the BCCH and the CCCH;

- Packet transfer mode: In packet transfer mode, the user equipment is prepared to transfer LLC PDUs on packet data physical channels (see sub-clause 5.4). The user equipment has allocated radio resources on one or more packet data physical channels for the transfer of LLC PDUs.

- PDCH: Packet Data CHannel (PDCH) identifies a stream of MAC/RLC Protocol Data Units (PDUs) that are delivered by/to MAC to/from Physical Layer as Service Data Units (SDUs). A SDU corresponds to the packet Transport Channel (TrCH) PKCH-T (se TSM 05.03 [14] for details).

- PDPCH: Packet Data Physical CHannel (PDPCH) identifies one physical resource (i.e. one spreading code; one midamble; one frequency and one time slot) for carrying PDCHs.

- Radio block: A radio block is the sequence of 4 normal bursts carrying one RLC/MAC protocol data unit (see TSM 04.04 [8]). (The one exception is a radio block occasionally used on PACCH consisting of a sequence of four access bursts, each carrying a repetition of one short RLC/MAC block.)

- Random values: In a number of places in this Technical Specification, it is mentioned that some value must take a "random" value, in a given range, or more generally with some statistical distribution. For such random values refer to TSM 04.08 [11].

- RLC/MAC block: An RLC/MAC block is the protocol data unit exchanged between RLC/MAC entities (see clause 10 and TSM 04.04 [8]).

- RLC/MAC control block: An RLC/MAC control block is the part of a RLC/MAC block carrying a control message between RLC/MAC entities (see sub-clause 10.3).

- RR connection: An RR connection is a physical connection established between a user equipment and the network to support the upper layers’ exchange of information flows. An RR connection is maintained and released by the two peer entities.

- RLC data block: An RLC data block is the part of a RLC/MAC block carrying user data or upper layers’ signalling data (see sub-clause 10.2).

- TBF: A Temporary Block Flow (TBF) identifies an RLC unidirectional, uplink or downlink, connection between the NetWorK and a User Equipment. The number of TBFs allocable per direction to one UE is limited to one; so that there can be either one uplink TBF or one downlink TBF or one uplink TBF and one downlink TBF at the same time dedicated to one user.

TBF abort: The term "abort" as applied to TBF is used when the TBF is abruptly stopped without using the Release of TBF procedures defined in clause 9.

TBF release: The term "release" as applied to TBF is used when the TBF is stopped using one of the Release of TBF procedures defined in clause 9.

- TD-PRS: TD-SCDMA Packet Radio Service (TD-PRS) identifies the packet service for a TD-SCDMA system.

- TD-PRS multislot class: The term TD-PRS multislot class refers to the different user equipment capabilities to transmit and receive on different combinations of multiple PDPCHs. The multislot classes are defined in TSM 05.02 [13]. Note that the user equipment may indicate different multislot classes for circuit mode services and for TD-PRS (see TSM 04.08 [11]). Different multislot class user equipment are capable of supporting different medium access modes (see sub-clause 5.2.5).

- Time Slot-State Flag (TS-SF): The Time Slot - State Flag (TS-SF) is used on downlink PDCH(s) to signal whether the indicated time slots will carry (in downlink or in uplink, depending on the allocation mode) radio blocks relevant for a specific packet user (see sub-clause 5.2.4; clause 10 and TSM 05.02 [13]).

Timer Expiry: A started timer has run the time specified.

Timer Restart: A timer that may already be running is stopped and then started again to run the time specified.

Timer Start: A timer is started to run the time specified.

- Timer Stop: A started timer is stopped and the elapsed time is then undefined.

- User State Flag (USF): The User State Flag (USF) is used on downlink S-PDCH(s) to allow multiplexing of uplink Radio blocks from different user equipment (see sub-clause 5.2.3, clause 10 and TSM 05.02 [13]) on the same uplink PDPCHs and to signal to the addressed user the resource status at the next radio block.

# 4	Layered overview of radio interface

The Radio Resource sub-layer provides the functions necessary for

-	Radio Resource (RR) management of packet data channels (PDCHs); and

-	Radio Link Control and Medium Access Control (RLC/MAC) on packet data channels.

As shown in Figure 1, the RR sub-layer provides services to the MM and LLC sub-layers. The RR sub-layer utilises the services of the Data Link layer (signalling layer 2) and the Physical Link layer. The packet logical channels PACCH and PDTCH are multiplexed onto the packet data channels on a per radio block basis and sent on PDPCH.

- Figure 2: Protocol architecture of Radio Resource (RR) sub-layer and RLC/MAC function

## 4.1	Layer services

The RR sub-layer provides services for the transfer of upper layer PDUs using a shared medium between multiple user equipment and the network. Direct communication is only possible between the network and one or more user equipment. The RLC/MAC function supports two modes of operation:

-	unacknowledged operation; and

-	acknowledged operation.

The RR sub-layer further provides services for the paging of user equipment.

## 4.2	Layer functions

The RLC function defines the procedures for segmentation and reassemble of LLC PDUs into RLC/MAC blocks and, in RLC acknowledged mode of operation, for the Backward Error Correction (BEC) procedures enabling the selective retransmission of unsuccessfully delivered RLC/MAC blocks. In RLC acknowledged mode of operation, the RLC function preserves the order of higher layer PDUs provided to it.

The MAC function defines the procedures that enable multiple user equipment to share a common transmission medium, which may consist of several physical channels. The function may allow a user equipment to use several physical channels in parallel, i.e. use several resources within the TDMA frame.

- For the user equipment originating access, the MAC function provides the procedures, including the contention resolution procedures, for the arbitration between multiple user equipment simultaneously attempting to access the shared transmission medium.

- For the user equipment terminating access, the MAC function provides the procedures for queuing and scheduling of access attempts.

## 4.3	Service primitives

- Information flow between layers is performed by the use of Service Primitives. Service Access Points (SAP) and their corresponding Service Primitives for the RR sub-layer are defined in TS 24.007 [10].

## 4.4	Services required from lower layers

The RLC/MAC function uses the services provided by the physical link layer as defined in  TSM 04.04 [8].

The RR sub-layer may use the services provided by the data link layer as defined in TSM 04.05 [9]. Moreover, the RR sub-layer directly uses services provided by the physical layer such as BCCH searching, as defined in TSM 04.04 [8].

## 4.5	Packet data Physical CHannels (PDPCH)

Packet data can be delivered at the air interface through two types of Packet Data Physical CHannels (PDPCH):

- Pilot – Packet Data Physical CHAnnels (P-PDPCH);

- Shared –Packet Data Physical CHAnnels (S-PDPCH).

### 4.5.1	Pilot – Packet Data Physical Channel (P-PDPCH)

A Pilot – Packet Data Physical CHannel (P-PDPCH) is characterised in that:

- It is univocally assigned to one user equipment only;

- It requires continuous burst transmission so that, when no valid MAC/RLC block has to be delivered, Packet Dummy control blocks (see sub-clause 7.1.2.3.1) shall be sent instead;

- The first UE assigned P-PDPCHs are established with the P-PDPCH packet access procedure (see sub-clause 7.2.3);

- It can be assigned both for downlink TBF and for uplink TBF;

- P-PDPCHs shall be assigned to a UE in duplex mode so that: when assigning a P-PDPCH for uplink (downlink) transmission, at least another P-PDPCH has to be set up in the opposite transmission direction.

Due to continuous transmission and the duplex assignment mode, P-PDPCHs can be used to perform Closed Loop Power control, Uplink Synchronisation Control (see TSM 05.08 [15]) as well as beam-forming coefficient estimation, for all (P/S-)PDPCHs assigned to a User Equipment.

In addition to the time slot number; spreading code; midamble sequence and carrier frequency, a P-PDPCH is also identified by the number of sub-channels and the UE specific assigned sub-channel  at which the UE can access (i.e. either receive or transmit) to the assigned resource; this information is provided by the Channel_Mode information element as defined in sub-clause 11.4.4.

### 4.5.2	Shared – Packet Data Physical Channel (S-PDPCH)

A Shared – Packet Data Physical CHannel (S-PDPCH) is characterised in that:

- It can be assigned to one or multiple user equipment sharing the same physical resources in the time domain;

- It can be assigned both with one phase or 2 phase packet access (see sub-clause 7.1);

- It does not require continuous transmission so that, when no valid MAC/RLC block has to be delivered, transmission can be suspended;

- It can be assigned both for downlink TBF and for uplink TBF.

# 5	Introduction to the Medium Access Control (MAC) procedures

## 5.1	General

The Medium Access Control procedures include the functions related to the management of the shared transmission resources, e.g. the packet data physical channels and the radio link connections on packet data physical channels.

The Medium Access Control procedures support the provision of Temporary Block Flows (TBFs) that allow the point-to-point transfer of signalling and user data within a cell between the network and a user equipment.

- Medium Access Control procedures for reception of PBCCH and PCCCH, which permits autonomous cell reselection performed by the user equipment are FFS.

## 5.2	Multiplexing principles

### 5.2.1	Temporary Block Flow

A Temporary Block Flow (TBF) is a physical connection used by the two RR entities to support the unidirectional transfer of LLC PDUs on packet data physical channels. The TBF has allocated radio resources on one or more PDPCHs and comprises a number of RLC/MAC blocks carrying one or more LLC PDUs. A TBF is temporary and is maintained only for the duration of the data transfer (i.e. until there are no more RLC/MAC blocks to be transmitted and, in RLC acknowledged mode, all of the transmitted RLC/MAC blocks have been successfully acknowledged by the receiving entity).

### 5.2.2	Temporary Flow Identity

- Each TBF has assigned a Temporary Flow Identity (TFI) by the network. The user equipment shall assume that the TFI value is unique among concurrent TBFs in each direction (uplink or downlink) on all PDPCHs used by the TBF. The same TFI value may be used concurrently for TBFs on other PDPCHs in the same direction and for TBFs in opposite directions.

- An RLC/MAC block associated with a certain TBF shall comprise a TFI. The TBF is identified by the TFI together with, in case of an RLC data block, the direction (uplink or downlink) in which the RLC data block is sent; and in case of an RLC/MAC control message, the direction in which the RLC/MAC control message is sent and the message type.

- Global_TFI is used to unambiguously identify the user equipment during packet transfer mode in an uplink or downlink RLC/MAC control message. If present, the Global TFI addresses the UE using either the uplink TFI or downlink TFI of the UE. Which TFI is used is at the discretion of the sender except where explicitly defined by procedure.

### 5.2.3	User State Flag

- The User State Flag (USF) allows to uniquely identify a packet user; therefore it is used by the network at the purpose to control the time multiplexing of different UEs on an uplink Shared – PDPCH.

- The USF field can be included in the header of an RLC/MAC block on a downlink S-PDPCH, as specified in clause 10. When P-PDPCHs have been assigned to a packet connection, there is no need for sending the USF field as the information delivered by the network on a downlink P-PDPCH  univocally addresses a well defined user equipment.

- The user equipment shall assume that the assigned USF value is unique among concurrent TBFs (uplink or downlink) which make use of the same uplink S-PDPCHs.

### 5.2.4	Time Slot State Flag

The Time  Slot – State Flag (TS-SF) indicates whether the physical resources on a given time slot (uplink or downlink) which have been assigned to a packet user are available for transmission or reception at the next, respectively uplink or downlink, radio block (see TSM 05.02 [13]).

The TS – SF field can be included in the header of an RLC/MAC block on a downlink P/S-PDPCH, as specified in clause 10. When transmitted on a downlink S-PDPCH, the relevant RLC/MAC block shall also include in the header the USF field.

### 5.2.5	Medium Access modes

Being a P-PDPCHs univocally assigned to a single user, the medium access mode for these channels looks like that of a Dedicated Physical Channel in circuit switched connections.

While two medium access modes are supported for the S-PDPCHs:

-	Dynamic Allocation characterised by the UE detecting a TS-SF, and eventually the assigned USF value (see sub-clause 5.2.3.), and the block or group of four blocks that it is allowed to transmit on the enabled PDCHs (see sub-clause 8.1.1.1);

-	Fixed Allocation characterised by fixed allocation of radio blocks (or time interval) and uplink PDCHs in the assignment message without an assigned USF (see sub-clause 8.1.1.3). Fixed Allocation may operate in half duplex mode, characterised by that downlink and uplink TBF are not active at the same time.

- The network shall ensure that the resource allocation used for a specific UE is compatible with its multislot class (the supported multislot class shall be indicated in the UE Radio Access Capability, see TSM 04.08 [11]).

- In the case of a downlink transfer, the term medium access mode refers to the measurement time scheduling, for the UE to perform neighbour cell power measurements (see sub-clause 8.1.1.3.3).

- For this release, all networks and user equipment shall support the following configurations:

- data transfer on downlink P-PDPCHs only;

- data transfer on downlink P-PDPCHs and downlink S-PDPCHs;

- data transfer on uplink P-PDPCHs only.

A UE which does not support either dynamic or fixed allocation for the uplink PDCHs, shall set the Establishment Cause field of the CHANNEL REQUEST message to “Packet Access, P-PDCH only supported in uplink direction” (see TSM 04.08 [11]). The network shall therefore assign only P-PDCHs in the uplink direction to that user equipment.

## 5.3	Packet idle mode

- In packet idle mode no temporary block flow exists.

- In packet idle mode, the user equipment monitors the relevant paging sub-channels on CCCH.

- In packet idle mode, upper layer may require the transfer of an LLC PDU, which implicitly triggers the establishment of a TBF and the transition to packet transfer mode.

## 5.4	Packet transfer mode

- In packet transfer mode, the user equipment has allocated radio resources providing a TBF for a physical point-to-point connection on one or more packet data physical channels for the unidirectional transfer of LLC PDUs between the network and the user equipment. Continuous transfer of one or more LLC PDUs is possible. Concurrent TBFs may be established in opposite directions. The RR sub-layer provides the following services:

- -	transfer of LLC PDUs in RLC acknowledged mode;

- -	transfer of LLC PDUs in RLC unacknowledged mode.

- When a transfer of LLC PDUs terminates, in either downlink or uplink direction, the corresponding TBF is released. When all TBFs have been released, in downlink and uplink direction, the user equipment returns to packet idle mode.

- Depending on the TD-PRS mode of operation (class A, B or C), the user equipment may leave both packet idle mode and packet transfer mode before entering dedicated mode, group receive mode or group transmit mode as defined in TSM  04.08 [11].

## 5.5	General procedures in packet idle and packet transfer modes

### 5.5.1	User equipment side

- The user equipment in packet idle and packet transfer modes shall monitor the system information broadcast in the cell.

- In packet idle mode, the user equipment shall monitor the radio blocks on CCCH, as defined in sub-clause 5.5.1.5. The determination of the paging group for the user equipment is defined in TSM  05.02 [13].

#### 5.5.1.1	Cell reselection

- Cell reselection in packet idle and packet transfer modes is specified in TSM 05.08 [15]. The RR entity on the user equipment side indicates to the upper layers the availability of a cell and a cell change when decided by the RR sub-layer. Upper layers are advised of system information broadcast in the cell when a new cell has been selected, or when a relevant part of this information changes.

- When the user equipment reselects a cell, the support of TD-PRS in the target cell is indicated in system information sent on BCCH, see TSM 04.08 [11].

- If a cell supports TD-PRS, the user equipment may perform packet access. If a cell does not support TD-PRS, the user equipment is not allowed to perform packet access.

- When a cell reselection is autonomously determined by the user equipment, the user equipment in packet idle or in packet transfer mode may continue its operation in the old serving cell, while acquiring certain system information for the target cell.

- When cell reselection is controlled by the network, the user equipment in packet transfer mode shall act upon the IMMEDIATE_REL value: it may continue its operation in the old serving cell, as in mobile steered cell reselection, or it shall immediately abort its TBF if it is indicated by the IMMEDIATE_REL value. The immediately time indication, here and in the remainder of this technical specification, shall be considered equal to the reaction time defined in TSM 05.10 [16].

- The operation in the old cell shall be aborted when one of the following conditions are met:

- -	the user equipment has received the SI13 message (see TSM 04.08 [11]) from the target cell; or

- -	the user equipment has received a Packet Cell Change order message with an IMMEDIATE_REL indicating an immediate TBF release from the serving cell; or

- -	the criterions for camping on the old cell are no longer fulfilled (see TSM 05.08 [15]).

- While the operation is maintained in the old cell, the user equipment may suspend its TBF(s) or suspend the monitoring of radio blocks on CCCH, in order to receive necessary information on BCCH in the target cell. Such suspension may be required in both packet idle and packet transfer modes. It is performed without notification to the network.

- Suspension of the operation in the old cell for this purpose is allowed during the time required, for each message and according to the user equipment’s multislot class, to receive the required messages on BCCH in the target cell. The allowable suspension of an uplink TBF may be extended with one block period, in case of dynamic allocation, if the user equipment is unable to receive the corresponding TS-SF and USF when needed due to the suspension of downlink operation.

- When the conditions are fulfilled to switch to the new cell, the UE shall abort any TBF in progress by immediately ceasing to decode the downlink, ceasing to transmit on the uplink, stopping all RLC/MAC timers except for timers related to measurement reporting. The UE shall then switch to the identified specified new cell and shall obey the relevant RLC/MAC procedures on this new cell.

- Under no circumstances, operations in the old cell shall be continued more than 5 seconds after a cell reselection has been determined.

#### Spare

#### 5.5.1.3	System information on BCCH

- The user equipment shall receive the SYSTEM INFORMATION (SI) messages broadcast on BCCH.

- When a new cell has been selected, the user equipment shall perform a complete acquisition of BCCH messages (see sub-clause 5.5.1.4). The user equipment shall not perform packet access in the selected cell, or enter the packet transfer mode, until it has:

- acquired the SYSTEM INFORMATION TYPE 1 (SI1), SYSTEM INFORMATION TYPE 3 (SI3), SYSTEM INFORMATION TYPE 4 (SI4) and SYSTEM INFORMATION TYPE 13 (SI13) messages;

- made at least one attempt to receive other SI messages that may be scheduled within one control multiframe cycle on BCCH (see TSM 05.02 [13]).

- When the SI13 message has been received, the user equipment shall supervise the BCCH_CHANGE_MARK and perform update of BCCH information.

##### 5.5.1.3.1	Supervision of BCCH_CHANGE_MARK and update of BCCH information

- The user equipment shall attempt to receive the SI13 or the PSI13 message at least every 30 seconds. The user equipment shall then take into account any occurrence of the PSI13 message that may be received on PACCH during packet transfer mode. If PSI13 is not received, the user equipment should attempt to receive the SI13 message on BCCH during periods in packet idle mode.

- If the user equipment has received neither the SI13 nor the PSI13 message within the last 30 seconds, it shall attempt to receive the SI13 message each time it is scheduled on BCCH. Such attempts shall be made during both packet idle and packet transfer modes. A user equipment in packet transfer mode may suspend its TBF for this purpose (see sub-clause 5.5.1.4.2).

- The SI13 and PSI13 messages contain the BCCH_CHANGE_MARK and SI_CHANGE_FIELD parameters. The user equipment shall store the value of the last received BCCH_CHANGE_MARK value.

- If the user equipment receives a BCCH_CHANGE_MARK and detects that the value has been incremented by one unit, compared to the previous value, the user equipment shall perform a partial acquisition of BCCH information. The information that shall be received is determined by the SI_CHANGE_FIELD parameter:

- If the SI_CHANGE_FIELD parameter indicates an update of a specific type or specific types of SI messages, the user equipment shall receive at least one instance of each of the indicated type(s) of SI messages.

- If the SI_CHANGE_FIELD parameter indicates an update of an unspecified type or types of SI messages, the user equipment shall receive at least one message instance within each consistent set of SI messages on BCCH. It shall also receive all SI messages on BCCH not belonging to a consistent set.

- If the SI_CHANGE_FIELD parameter indicates an update of an unknown type of SI message, the user equipment is not required to update any BCCH information.

When a SI message is received, the user equipment shall consider a SI change mark value, if such is received in the message and take appropriate action (see sub-clause 5.5.1.4.1).

If the user equipment receives a BCCH_CHANGE_MARK and detects that the value has been incremented by more than one unit, compared to the previous value, the user equipment shall perform a complete acquisition of BCCH messages (see sub-clause 5.5.1.4).

##### 5.5.1.3.2	SI13 reception failure

If the user equipment has not received the SI13 or the PSI13 message within the last 60 seconds, a SI13 reception failure has occurred. A SI13 reception failure shall result in a cell reselection.

#### 5.5.1.4	Acquisition of system information on the broadcast channel

This procedure shall be used by the TD-PRS user equipment to perform a complete or partial acquisition of BCCH information.

This procedure starts when the user equipment receives a BCCH_CHANGE_MARK, SI change mark value indicating that system information is changed. Moreover, the procedure shall start at any other indication, which may be received by the user equipment, that the stored system information for the serving cell is no longer valid.

At cell selection or cell reselection, the procedure starts when the user equipment has received the SI13 message.

In a complete acquisition of BCCH information, the user equipment shall receive all SI messages that are scheduled on the broadcast channel. The user equipment shall delete any SI change mark value that was stored before the acquisition of BCCH information started.

In a partial acquisition of BCCH information, only a certain subset of the SI messages that are scheduled on the broadcast channel shall be received. The user equipment may consider the state of the SI change mark values, without restriction, to reduce the total number of messages to receive.

When the user equipment acquires a set of SI messages on the broadcast channels, it may receive these messages during both packet idle and packet transfer modes. While the user equipment is in packet idle mode, an attempt to receive a required message shall be made each time the message is scheduled on the broadcast channel, until the message is received. While the user equipment is in packet transfer mode, it shall receive any PSI message that is sent by the network on PACCH.

If the user equipment has not received the required messages within 10 seconds after the start of this procedure, an attempt to receive a missing message shall be made each time the message is scheduled on the broadcast channel. These attempts shall then be performed during both packet idle and packet transfer modes. A user equipment in packet transfer mode may suspend its TBF(s) for this purpose, as specified in sub-clause 5.5.1.4.2.

A second acquisition of BCCH information may be initiated (e.g., when the user equipment receives a SI change mark value) before a previous acquisition is completed.  In this case, the user equipment shall discard and immediately begin re-acquiring all the system information messages of the particular type to which the change_mark value refers.

##### 5.5.1.4.1	Consistent sets of system information messages

A user equipment, receiving a SI message belonging to a consistent set of system information messages, shall store the last SI change mark value received for the set of messages (see sub-clause 5.5.2.1.4).

Whenever user equipment receives a SI change mark value, which is not equal to the previously stored value for the set of messages, the user equipment shall perform a partial acquisition of BCCH information. It shall then receive all instances of the SI messages belonging to the consistent set of system information messages.

If a user equipment detects an inconsistency amongst the SI count and index parameters within in a consistent set of system information messages or any other inconsistency making the information that is contained invalid, the user equipment shall discard the messages received so far and delete the stored SI change mark value. The user equipment may then restart the acquisition of the affected system information messages.

##### 5.5.1.4.2	Suspension of operation to receive system information

During certain conditions, the user equipment in packet transfer mode is allowed to suspend a TBF to receive certain information on BCCH. Such suspension is made without notification to the network.

Suspension of a TBF for this purpose is allowed during the time required, for each message and according to the user equipment’s multislot class, to receive the required messages on BCCH. The allowable suspension of an uplink TBF may be extended with one block period, in case of dynamic allocation, if the user equipment is unable to receive the corresponding TS-SF, and USF when needed, due to the suspension of downlink operation.

##### 5.5.1.4.3	Request for acquisition of system information

As an option, the UE may implement the request for acquisition of system information. If the network supports the PACKET PSI STATUS message, the UE may then send the PACKET PSI STATUS message to the network, each time an acquisition of PSI messages is initiated.

The PACKET PSI STATUS message shall indicate the present status of PSI messages stored in the user equipment. The PACKET PSI STATUS message is sent on PACCH when the UE is in packet transfer mode. The first sending of this message during the acquisition of PSI information shall take place at the first suitable opportunity after the acquisition is initiated.

During the acquisition of PSI message information, the PACKET PSI STATUS message may be sent up to four times to the network. The second sending of this message shall take place at the first suitable opportunity at least 1 second after that the message is sent the first time. Further sending shall take place at the first suitable opportunity at least 2 seconds after that the message was sent the previous time.

The PACKET PSI STATUS message shall not be sent when the UE has started to suspend its TBF(s) in order to receive the required PSI messages. The PACKET PSI STATUS message shall not be sent when the UE has acquired the complete set of PSI messages.

Note: PACKET PSI STATUS message is not used at the moment because only PSI13 is available. When more PSI messages will be included in TD-PRS system, the PSI_STATUS_IND bit, used by the network to indicate the support of PACKET PSI STATUS message, will be available in one of them.

#### 5.5.1.5	Discontinuous reception (DRX)

A user equipment in packet idle mode shall listen to the radio blocks on CCCH as defined in TSM 05.02 [13]. In the TD-PRS attach procedure, defined in TSM 04.08 [11], the user equipment requests values for the SPLIT_PG_CYCLE and NON_DRX_TIMER parameters to be applied on CCCH.

NOTE:	The support of the SPLIT_PG_CYCLE parameter is optional on CCCH, see TSM 05.02 [13].

The SPLIT_PG_CYCLE and NON_DRX_TIMER parameters control:

-	the occurrence of paging blocks on CCCH belonging to the user equipment (SPLIT_PG_CYCLE parameter, see TSM 05.02 [13]) when DRX is used (DRX mode, see TSM 03.64 [5]); and

-	the duration of the non-DRX mode period (see TSM 05.02 [13]) to be applied by the user equipment when it has left the packet transfer mode and enters the packet idle mode.

There are four cases when the UE shall enter the non DRX mode:

- Upon transition from the packet transfer mode to the packet idle mode, the user equipment shall enter the Transfer non DRX mode period. The duration of this period is determined by the minimum value of the NON_DRX_TIMER parameter, requested in the TD-PRS attach procedure, and the DRX_TIMER_MAX parameter, broadcast in the cell.

- When the user equipment receives a new value of the DRX_TIMER_MAX parameter, the user equipment is not required to consider the new value until the next time it enters packet idle mode.

- A user equipment operating in NC2 mode (see TSM 05.08 [15]) shall enter the NC2 non-DRX mode period after it has sent an NC measurement report. The duration of this period is defined by the NC_NON_DRX_PERIOD parameter.

- When initiating the MM procedures for TD-PRS attach and routing area update defined in TSM 04.08 [11], the user equipment shall enter the MM non DRX mode period. This period ends when either of the messages GPRS ATTACH ACCEPT, GPRS ATTACH REJECT, ROUTING AREA UPDATE ACCEPT or ROUTING AREA UPDATE REJECT is received by the user equipment. This period also ends after timeout when waiting for any of these messages, see TS 24.008 [21] for details.

The non DRX periods defined above run independent of each other and may overlap. The user equipment shall be in non DRX mode during any of the non DRX mode periods. Otherwise, the UE shall be in DRX mode.

If the user equipment establishes a dedicated connection during any of the non DRX mode periods, then that period shall continue to run.

#### 5.5.1.6	Frequency Parameters

Frequency parameters can be included in the assignment messages (i.e., IMMEDIATE ASSIGNMENT or PACKET RESOURCE RECONFIGURE) and define the radio frequency channel the user equipment has to use during the assigned TBF.

Frequency parameters for the assigned PDPCHs of a packet switched connection can therefore be changed during the call but they shall assume the same value for all the assigned physical resources at a time.

#### 5.5.1.7	TLLI management

In case the UE receives a message assigning a new P-TMSI from the network during the contention resolution procedure, the UE shall continue to use the old TLLI until the contention resolution is completed.

After contention resolution the UE shall apply new TLLI in RLC/MAC control block if the UE has received a new P-TMSI.

### 5.5.2	Network side

#### 5.5.2.1	System Information broadcasting

##### 5.5.2.1.1	Spare

##### 5.5.2.1.2	System information on BCCH

In addition to the requirements in TSM 04.08 [11], a SYSTEM INFORMATION TYPE 13 (SI13) message is regularly broadcast by the network on the BCCH to support TD-PRS. Optionally, additional types of SI messages may be broadcast on BCCH. Some of them may be broadcast in multiple number of instances. Based on this information, the user equipment is able to decide whether and how it gains access to the system via the current cell.

The SI13 message contains the BCCH_CHANGE_MARK and SI_CHANGE_FIELD parameters. The value of the BCCH_CHANGE_MARK may be incremented by one, modulo 8, each time the network makes a change in the BCCH information. Such change includes any addition, removal or replacement of SI messages, contents of SI messages, or change in the scheduling of SI messages on BCCH. Changes in the contents of the SI13 message shall not to be reflected in the BCCH_CHANGE_MARK. Changes of the contents of the RACH Control Parameters information element alone (see TSM 04.08 [11]) may optionally be reflected in the BCCH-CHANGE-MARK; if reflected, the SI-CHANGE-FIELD parameter may indicate only one of the SI message containing the RACH Control Parameters. When the BCCH_CHANGE_MARK is incremented, the SI_CHANGE_FIELD parameter shall be set to an appropriate value to indicate the nature of the latest change in the BCCH information.

The network may increment the BCCH_CHANGE_MARK value by more than one, modulo 8, in order to enforce a complete acquisition of BCCH information of all user equipment.

##### 5.5.2.1.3	System information on PACCH (and other logical channels)

The network may broadcast PSI messages on PACCH. In particular, if a user equipment is busy in packet transfer mode and thus unable to receive the relevant blocks on the broadcast channels (BCCH) for a period longer than 15 seconds, the network may broadcast the PSI13 message on PACCH such that the UE may receive the PSI13 messages at least every 15 seconds.

If the network supports the PACKET PSI STATUS message and this message is received from a user equipment, the network may schedule the missing PSI messages for that user equipment on PACCH.

##### 5.5.2.1.4	Consistent sets of system information messages

Certain types of SI messages are sent on the BCCH in a multiple number of instances. If such a SI message type is sent on BCCH, the user equipment shall receive a consistent set of that type of SI message. In some cases, more than one type of SI messages may be joined into one consistent set.

A consistent set of system information messages is identified by a SI change mark parameter included in each message in the set. All messages within a consistent set shall have the same value of this parameter.

The total number of system information messages of a certain type within a consistent set is indicated by a SI count parameter included in each message in the set. The position of a certain message instance within the consistent set of system information messages is indicated by a SI index parameter.

The SI count parameter shall have the value N–1, where N is the number of instances of the particular message type present in the consistent set. The SI index parameter shall have a range from zero to N–1. Different instances of a particular message type in a consistent set shall have different values of the SI index parameter.

For this release, (P)SI13 only is foreseen for the support of packet switched services. SI13 shall be broadcast on BCCH while PSI13 is sent on PACCH and both have the same content. There is only one instance of (P)SI13.

## 5.6	Measurement reports

The network may request measurement reports from the UE when in MM Ready state (see TSM 04.08 [11]). The measurement reporting principles are specified in TSM 05.08 [15].

The requested measurement reports can be of three types:

-	Network Control (NC) measurement reports, both in packet idle and in packet transfer mode ;

-	Extended measurement (EM) reports, both in packet idle and in packet ransfer mode;

-	Measurements on demand, in packet transfer mode only.

In addition to the requested measures, the UE can also spontaneously report, both in packet idle and in packet transfer mode, on measures which have been triggered by specific events.

### 5.6.1	Network Control (NC) measurement reporting

The behaviour of the user equipment is controlled by the parameter NETWORK_CONTROL_ORDER broadcast by the network in SI13 on the BCCH and in the PSI13 on the PACCH. The network can also send the NETWORK_CONTROL_ORDER parameter in a PACKET MEASUREMENT ORDER or in a PACKET CELL CHANGE ORDER message on the PACCH to a particular user equipment. The parameter NETWORK_CONTROL_ORDER may have one of the values NC0, NC1, NC2 or RESET, see TSM 05.08 [15]. When in mode NC1 or NC2, the user equipment shall perform the NC measurements as defined in TSM 05.08 [15]. The reporting periods are indicated in the NC_REPORTING_PERIOD_I and NC_REPORTING_PERIOD_T field of the PSI13 and SI13, the PACKET CELL CHANGE ORDER or the PACKET MEASUREMENT ORDER message.

If no individual order has been received from the network, the user equipment shall act upon the relevant parameter values which are broadcast in SI13 and PSI13 according to the following rules:

- If the NC_REPORTING_PERIOD_I field is present, the UE in packet idle mode shall autonomously report, i.e. without waiting for any explicit message from the network, the measurements according to the indicated reporting period, NETWORK_CONTROL_ORDER field value and its non DRX period. If the NC_REPORTING_PERIOD_I field is not present, the UE in packet idle mode shall not autonomously report any measurement.

- If the NC_REPORTING_PERIOD_T field is present, the UE in packet transfer mode shall autonomously report, i.e. without waiting for any explicit message from the network, the measurements according to the indicated reporting period and NETWORK_CONTROL_ORDER field value. If the NC_REPORTING_PERIOD_T field is not present, the UE in packet transfer mode shall not autonomously report any measurement.

Among the above mentioned parameters, additional information can be brought by the network via a PACKET CELL CHANGE ORDER or a PACKET MEASUREMENT ORDER message.

When the NC Measurement parameters are brought in multiple instances of a PACKET MEASUREMENT ORDER message, each instance can contain only part of the all information. The UE shall therefore collect all instances of a PACKET MEASUREMENT ORDER message and concatenate each partial information in ascending message instance order, before starting to interpret the command. If the value of the NETWORK_CONTROL_ORDER, or any of the NC parameters, differs between instances of a PACKET MEASUREMENT REPORT message, the UE shall assume as valid only the parameters listed in the instance having the highest number, ignoring all the others.

The user equipment shall apply to the timer T3158 either the NC_REPORTING_PERIOD_I when in packet idle mode or the NC_REPORTING_PERIOD_T when in packet transfer mode. The measurement results shall be sent to the network using the procedures specified in sub-clause 7.3 for packet idle mode, and in sub-clause 8.3 for packet transfer mode.

On expiry of timer T3158, the user equipment shall restart timer T3158 with the indicated reporting period, perform the measurements and send the PACKET MEASUREMENT REPORT message to the network.

A user equipment in mode NC1 or NC2 may receive a new indicated reporting period or change packet mode while timer T3158 is active. If the new indicated reporting period is less than the time to expiry of timer T3158, the user equipment shall immediately restart timer T3158 with the new indicated reporting period. Otherwise, the timer T3158 shall continue to run.

When the user equipment leaves the MM Ready state, the timer T3158 shall be stopped and no more measurement reports shall be sent to the network.

A user equipment may reselect a new cell or may be ordered to reselect a new cell with mode NC1 or NC2 while timer T3158 is active. If time to expiry of timer T3158 is greater than the indicated reporting period for the new cell, the user equipment shall immediately restart timer T3158 with the indicated reporting period for the new cell. Otherwise, the timer T3158 shall continue to run.

At cell reselection the NC measurement parameters valid for the user equipment in the new cell (NETWORK_CONTROL_ORDER, NC_NON_DRX_PERIOD, NC_REPORTING_PERIOD_I and NC_REPORTING_PERIOD_T) can be brought from the old cell (if received in a PACKET MEASUREMENT ORDER or PACKET CELL CHANGE ORDER message). If no parameters have been brought from the old cell, and until individual measurement parameters are received in the new cell, the user equipment shall use the measurement parameters which are broadcast by the target cell in SI13.

The default frequency list to be applied in the new cell shall be the frequency list of that cell until a new PACKET MEASUREMENT ORDER message is received. The frequency list could also have been modified by frequency parameters received in a PACKET_CELL_CHANGE_ORDER message in the old cell.

A user equipment involved in an RR connection (in class A mode of operation) or performing anonymous access, see TSM 04.08 [11], shall not send measurement reports to the network during that period. The user equipment shall return to the previous mode when the RR connection is released or when the TBF for the anonymous access is terminated.

### 5.6.2	Extended measurement (EM) reporting

The network may order a user equipment to send extended measurement reports. The behaviour of the user equipment is controlled by the parameters EXT_MEASUREMENT_ORDER and EXTENDED_MEASUREMENT_COMMAND_CHANGE_MARK in the PACKET MEASUREMENT ORDER message.

The network shall send the PACKET MEASUREMENT ORDER message on the PACCH to address a particular user equipment as defined in sub-clauses 7.5 and 8.5. The parameter EXT_MEASUREMENT_ORDER shall have one of the values EM1 or RESET, see TSM 05.08 [15].

When in mode EM1, the user equipment shall perform the measurements as defined in TSM 05.08 [15]. The EXT_REPORTING_PERIOD_I field of the PACKET MEASUREMENT ORDER message indicates, if present, the reporting period of the requested extended measurements when the UE is in packet idle mode. The EXT_REPORTING_PERIOD_T field of the PACKET MEASUREMENT ORDER message indicates, if present, the reporting period of the requested extended measurements when the UE is in packet transfer mode. When the EXT_REPORTING_PERIOD_I/T field is not included, the UE shall perform and report the requested extended measures in the relevant packet mode only once (see sub-clause 5.6.4 for details).

When the EXT Measurement parameters are brought in multiple instances of a PACKET MEASUREMENT ORDER message, each instance can contain only part of the all information. The UE shall therefore collect all instances of a PACKET MEASUREMENT ORDER message and concatenate each partial information in ascending message instance order, before starting to interpret the command. If the value of the EXTENDED_MEASUREMENT_COMMAND_CHANGE_MARK field differs between instances of a PACKET MEASUREMENT REPORT message, the UE shall assume as valid only the parameters listed in the instance having the highest number, ignoring all the others.

When instructed to perform EM measurement reporting, the user equipment shall start timer T3178 according to the indicated reporting period. The results shall be sent to the network using the procedure defined in sub-clause 7.3 or 8.3.

A user equipment may reselect to a new cell with mode EM1while timer T3178 is active. If the time to expiry of timer T3178 is greater than the indicated reporting period for the new cell, the user equipment shall immediately restart timer T3178 with the indicated reporting period for the new cell. Otherwise, the timer T3178 shall continue to run.

### 5.6.3	Measurements on Demand

The network can ask the UE to report on specific measurements by sending a Measurement on demand command in the MAC header of a downlink radio block (see sub-clause 10.4.1).

The necessary conditions required for asking such a kind of measure are the following:

-	A P-PDCH shall be established between the UE and the network;

- An NC_REPORTING_PERIOD_T or an EXT_REPORTING_PERIOD_T shall be assigned to the UE.

A Measurement on demand Command shall last for NC/EXT_REPORTING_PERIOD_T. The network can formulate a new Measurement on demand Command each integer multiple of NC/EXT_REPORTING_PERIOD_T.

In case both an NC_REPORTING_PERIOD_T and an EXT_REPORTING_PERIOD_T have been assigned to the user, the measurement on demand command shall last for the shortest period among the two.

The UE shall report on the requested measures at the next (NC or EXT) reporting period. The time relation between a Measurement on demand Command and the UE reporting period is described in TSM 05.08 [15].

### 5.6.4	Measurement Reporting Priority

The network can ask, with one PACKET MEASUREMENT ORDER message, both NC measurements and extended measurements; measurements on demand are also possible at the same time, carried piggy back on the MAC header of the same downlink radio block (see sub-clause 5.6.3).

The UE shall report, with one MEASUREMENT REPORT message, either NC measurement results or EM measurement results, but not the both at one time, as well as Measurements on demand results if requested by the network and event triggered measurements if the triggering events occurred.

Event triggered measurements and measurements on demand reports shall always have the highest priority with respect to the NC and the EM measurement reports.

When an extended measurement report is solicited by the network in a non periodic way, the UE shall report the requested extended measurements once available either at the next NC_REPORTING_PERIOD_I/T if defined, thus overruling the NC measurement report, otherwise as soon as possible.

When the extended measurement report is solicited by the network in a periodic way, the UE shall report the requested extended measurements once available at the next EXT_REPORTING_PERIOD_T/I, eventually overruling the NC measurement report.

In case one measurement report does not fit in a single MEASUREMENT REPORT message, the UE can send the measurement results in multiple messages, at the time instants and according to the rules which are relevant to its packet mode and to the kind of measurement results, NC or EM, to report. The detection of a new MEASUREMENT ORDER message changing some of the measurement parameters, shall reset the eventually pending measurement report and make the UE restart to collect the new measures.

# 6	Paging procedures

For a user equipment in packet idle mode, the network may initiate the establishment of an RR connection or a downlink packet transfer by the paging procedures. The paging procedure can only be initiated by the network on a paging sub-channel on CCCH. A number of user equipment can be paged for either downlink packet transfer or RR connection establishment in the same paging message.

For a user equipment in packet transfer mode, the network may initiate the establishment of an RR connection by the paging procedures. The paging procedure can only be initiated by the network on PACCH or on a paging sub-channel on CCCH. A number of user equipment can be paged for RR connection establishment in the same paging message.

Paging procedures for RR connection establishment are described in sub-clause 6.1. Paging procedures for downlink packet transfer are described in sub-clause 6.2.

## 6.1	Paging procedure for RR connection establishment

The network may initiate the establishment of an RR connection by the paging procedure for RR connection establishment.

The network initiates the paging procedure to trigger an RR connection establishment by broadcasting a paging request message on the appropriate paging sub-channel on CCCH. The paging sub-channels on CCCH are specified in TSM 05.02 [13] and GSM 03.13 [4]. The network may also send paging related information on PACCH to a user equipment in class A or class B mode of operation when such user equipment is in packet transfer mode.

If the user equipment in class B mode of operation is in packet transfer mode then the mobile is not required to decode the CS paging sub-channels on CCCH.

### 6.1.1	Paging initiation using paging sub-channel on CCCH

The paging initiation procedure and the paging request messages used on CCCH are specified in TSM 04.08 [11].

### 6.1.2	Paging initiation using PACCH

Paging initiation using PACCH applies to a user equipment in class A or class B mode of operation when such user equipment is in packet transfer mode and when the network operates according to mode I (see TSM 03.60 [19]). In this case, the network shall send the PACKET PAGING REQUEST message to the user equipment on the appropriate PACCH. The message includes the user equipment identification and the channel needed field which defines how user equipment of different capabilities shall code the establishment cause field in the CHANNEL REQUEST message, as specified in TSM 04.08 [11].

### 6.1.3	Paging response

Upon receipt of a Paging Request message, the purpose of which is to trigger the establishment of an RR connection, a user equipment operating in class B mode of operation and in packet transfer mode shall either ignore or answer the paging message according to TS 22.060 [2].

When answering to a paging message the purpose of which is to trigger the establishment of an RR connection, the user equipment, whatever its UE class mode of operation, shall follow the paging response procedures as specified in TSM 04.08 [11]. Additionally, a user equipment operating in class B mode of operation shall abort the current packet data transfer(s) if it was in packet transfer mode, and suspend any Packet service activity until return to idle mode (see TSM 04.08 [11]).

## 6.2	Paging procedure for downlink packet transfer

The network may initiate the packet paging procedure in order to obtain the user equipment cell location required for downlink packet transfer. The packet paging procedure can only be initiated by the network. The procedure is initiated by broadcasting PAGING REQUEST message on the appropriate paging sub-channel on CCCH. The paging sub-channels on CCCH are specified in TSM 05.02 [13] and GSM  03.13 [4].

Paging using the paging sub-channel on CCCH applies to a user equipment that is not in packet transfer mode.

### 6.2.1	Paging procedure using paging sub-channel on CCCH

The packet paging procedure and the paging request messages used on CCCH are specified in TSM 04.08 [11].

### 6.2.2	Paging response

Whenever the MM sub-layer in the user equipment indicates an LLC PDU in response to a PAGING REQUEST, the user equipment shall initiate the uplink TBF using a CHANNEL REQUEST with cause “one phase packet access”.

The procedure in response to a packet paging request on CCCH is specified in TSM 04.08 [11].

NOTE:	The user equipment initiates an implicit paging response by sending an LLC PDU to the network as defined in GSM 04.64 [12] and TSM 04.08 [11].

# 7	Medium Access Control (MAC) procedures when entering the packet transfer mode

The establishment of a Temporary Block Flow (TBF) can be initiated by either the user equipment or the network.

The request for establishment of a TBF occurs on CCCH as described in TSM 04.08 [11].

For the establishment of a (downlink/uplink) TBF, the network shall assign to the user packet resources via the IMMEDIATE ASSIGNMENT message.

The assigned resources can be Pilot –Packet Data Physical CHannels (P-PDPCHs) and/or Shared-Packet Data Physical CHannels (S-PDPCHs) in one of the following combinations.

- The network assigns P-PDPCHs only, such that: there shall be one Pilot- PDCH only in uplink and one or more Pilot-PDCHs in downlink

- The network assigns S-PDPCHs only, such that: there can be none or one or more S-PDCHs in downlink and/or none or one S-PDCH in uplink.

- The network assigns both P-PDPCHs and S-PDPCHs, such that: there shall be one PDCH only in uplink and this shall be of P-PDCH type and one or more P-PDCH and none, one or more S-PDCH respectively in downlink.

When assigning S-PDPCHs, the network shall specify the medium access mode either Dynamic or Fixed. If the Dynamic allocation medium access mode is used and no P-PDPCH resources have been assigned, the network shall include in the IMMEDIATE ASSIGNMENT message the USF value allocated for the uplink assigned S-PDPCHs; if the Fixed allocation medium access mode is used, the network shall include in the IMMEDIATE ASSIGNMENT message an ALLOCATION_BITMAP for the uplink assigned S-PDPCHs. The network may include gaps in the ALLOCATION_BITMAP where the user equipment shall monitor the PACCH and perform neighbour cell power measurements. In case of P-PDPCH assignment, the allocation medium access mode shall be that of a Dedicated Physical CHannel (DPCH) as described in sub-clause 4.5.1.

This clause describes additional procedures with respect to the ones reported in TSM 04.08 [11] for starting a packet call.

## 7.1	TBF establishment initiated by the user equipment

The purpose of the packet access procedure is to establish a TBF to support the transfer of LLC PDUs in the direction from the user equipment to the network. Packet access shall be done on CCCH, as defined in TSM 04.08 [11]. The packet access can assign either P-PDPCHs and/or S-PDPCHs; when assigning S-PDPCHs the packet access can be done either in one phase (see sub-clauses 7.1.2 and 7.1.2.4) or in two phases (sub-clauses 7.1.2 and 7.1.3).

TBF establishment can also be done on PACCH if a TBF for transfer of LLC PDUs in the direction from the network to the user equipment is already established (see sub-clause 8.1.2.4).

The packet access procedure is initiated by the user equipment. Initiation is triggered by a request from upper layers to transfer an LLC PDU. The request from upper layers specifies throughput, RLC mode and a Radio Priority to be associated with the packet transfer or indicates that the packet to be transferred contains signalling.

Upon such a request:

-	if access to the network is allowed (sub-clause 7.1.1), the user equipment shall initiate the packet access procedure as defined in sub-clause 7.1.2;

otherwise:

-	the RR sub-layer in the user equipment shall reject the request.

If the request from upper layers indicates signalling, the highest Radio Priority shall be used at determination if access to the network is allowed and the acknowledged RLC mode shall be used.

### 7.1.1	Permission to access the network

The network broadcasts on CCCH the list of authorised access classes and authorised special access classes in the RACH_CONTROL_PARAMETERS field.

Access to the network is allowed if the user equipment is a member of at least one authorised access class or special access class as defined in TS 22.011 [20].

### 7.1.2	Initiation of a TBF establishment

#### 7.1.2.1	Initiation of the packet access procedure

The user equipment shall initiate the packet access procedure by scheduling the sending of a signature on the UpPTS physical channel according to the rules detailed in TSM 04.08 [11] sub-clause 3.3.1.1 and simultaneously leaving the idle mode; the UE shall not send any CHANNEL REQUEST message before the network has acknowledged (by PHYSICAL INFORMATION message) the sent signature. The user equipment shall use for the purpose the last access parameters received on CCCH.

At sending of a CHANNEL REQUEST message, the user equipment shall monitor all CCCHs carried by the CCPCH associated to the used RACH physical channel to get the expected IMMEDIATE ASSIGNMENT message from the network and start timer T3146. A user equipment in class A or class B mode of operation (i.e. which is IMSI attached) shall respond to a PAGING REQUEST message indicating an RR connection establishment. A user equipment in class B mode of operation may abort the packet access procedure at the receipt of a PAGING REQUEST message indicating an establishment of an RR connection. PAGING REQUEST messages indicating a non-RR connection shall be ignored.

User equipment in class C mode of operation (i.e. which is not IMSI attached) shall not respond to any type of PAGING REQUEST messages during the packet access.

The CHANNEL REQUEST messages are sent on RACH and contain an indication of the type of access and parameters required to indicate the user equipment’s demand of radio resource.

If the user equipment intends to use the TBF to send user data or Layer 3 signalling only (e.g. Cell update; Page Response; GPRS Mobility Management or Session Management messages) and it supports P-PDCHs only in uplink direction, it shall inform the network by setting the Establishment Cause field in the CHANNEL REQUEST message to “Packet access; P-PDCHs only supported in uplink direction”.

If the user equipment intends to use the TBF to send user data and it supports uplink shared channels, it shall request two phase access if the requested RLC mode is unacknowledged mode. If the requested RLC mode is acknowledged mode and the amount of data can fit in 8 or less than 8 RLC/MAC blocks, the user equipment shall indicate Short Access as access type. If the requested RLC mode is acknowledged mode and the amount of data to send takes more than 8 RLC/MAC blocks, the user equipment shall request either one phase access or two phase access. The UE shall calculate the number of blocks to be transferred assuming channel coding scheme CS-1.

If the user equipment intends to use the TBF to send Layer 3 signalling only and it supports uplink shared channels, it shall request for a one phase packet access.

If the purpose of the packet access procedure is to send a MEASUREMENT REPORT or a PACKET CELL CHANGE FAILURE, the user equipment shall indicate 'Single block without TBF establishment' in the CHANNEL REQUEST message.

At the expiry of T3146, if the number of sent CHANNEL REQUEST messages has not yet reached its maximum value “N” (see TSM04.08 [11]) or if the UE did not request for a single block without TBF establishment, the user equipment can re-start the packet access procedure by sending a new signature, otherwise the packet access procedure is aborted, a packet access failure is indicated to upper layers and the UE shall return to packet idle mode.

If the user equipment receives an IMMEDIATE ASSIGNMENT message during the packet access procedure indicating a packet downlink assignment procedure, the user equipment shall abort the packet access procedure and respond to the IMMEDIATE ASSIGNMENT message as specified in sub-clause 7.2.1. The user equipment shall then attempt an establishment of uplink TBF, using the procedure specified in sub-clause 8.1.2.4 which is applicable in packet transfer mode.

#### 7.1.2.2	Packet assignment procedure

##### 7.1.2.2.1	On receipt of a CHANNEL REQUEST message

On receipt of a CHANNEL REQUEST message for packet call, the network may assign one or more PDPCHs to be used by the user equipment for the TBF.

The allocated PDPCHs are assigned to the user equipment in an IMMEDIATE ASSIGNMENT message, sent on any CCCH carried by the CCPCH associated to the RACH physical channel on which the network has received the CHANNEL REQUEST message (see TSM 04.08 [11] and TSM 05.02 [13]). The Request Reference information element shall be used to address the specific user equipment.

If the UE requested for a single block without TBF establishment, the network can only grant the single block period on the assigned packet uplink resources.

If the UE requested for a two phase packet access, the network can allocate either a single block or a pilot packet data channel.

If the UE indicated the support of uplink pilot channels only, the network can only allocate a pilot packet data channel.

If the UE requested for a one phase access or for a short packet access, the network can allocate a pilot packet data channel, or it can grant respectively a one phase or a two phase packet access. By granting a single block packet access, the network forces the UE to perform a two phase packet access.

At sending the IMMEDIATE ASSIGNMENT message granting for a one phase packet access, timer T3141 shall be started at the network side (for a detailed description of timer T3141 see TSM 04.08 [11]).

On receipt of an IMMEDIATE ASSIGNMENT message corresponding to one of its 3 last CHANNEL REQUEST messages the user equipment shall stop timer T3146 if running, send, if requested by the network via the IMMEDIATE ASSIGNMENT message (see TSM 04.08 [11]), a randomly selected signature on the UpPTS physical channel and wait for the network reply at the most for three consecutive attempts and then act, whether or not a valid network reply was detected, as follows:

-	If the IMMEDIATE ASSIGNMENT message does not include a TBF starting time information, or it contains a TBF starting time indication already elapsed, the UE shall immediately switch to the assigned packet resources, starting Timer T3164 if a dynamic allocation has been granted.

-	If the IMMEDIATE ASSIGNMENT message includes a TBF starting time not yet elapsed, the UE shall monitor the full CCPCH until the point in time indicated by the TBF starting time. Thereafter, it shall switch to the assigned resources, starting timer T3164 if a dynamic allocation has been granted. If while monitoring the CCCH the user equipment receives more than one IMMEDIATE ASSIGNMENT message, it shall act upon the most recently received message ignoring the previous one.

Once switched to the assigned packet resources, the UE shall proceed according to the kind of packet assignment received from the network.

If the user equipment has been allocated a P-PDPCH, it shall proceed with the P-PDPCH packet access procedure as reported in sub-clause 7.1.2.3.

If the user equipment has been allocated a single block and the user equipment has not indicated Single block without TBF establishment in the CHANNEL REQUEST message, the user equipment shall proceed with the two phase packet access procedure according to sub-clause 7.1.3.

If the user equipment has been allocated a single block and the user equipment has indicated Single block without TBF establishment in the CHANNEL REQUEST message, the user equipment shall proceed with the measurement report according to sub-clause 7.3.1.

If the user equipment has been allocated a single block and the user equipment has indicated Single block without TBF establishment in the CHANNEL REQUEST message for the purpose of sending a PACKET CELL CHANGE FAILURE message, it shall proceed according to sub-clause 8.4.1.

Otherwise the user equipment shall proceed with contention resolution for the one phase packet access procedure according to sub-clause 7.1.2.4.

If the user equipment detects in the IMMEDIATE ASSIGNMENT message an invalid Frequency Parameters information element, or the assigned PDCH has not been satisfactorily defined, or it has been assigned uplink shared channels while it supports P-PDCHs only in uplink direction, it shall abort the procedure and may then re-initiate this procedure if not yet repeated for the maximum allowed number of times.

##### 7.1.2.2.2	Packet access reject procedure

The network may send to the user equipment an IMMEDIATE ASSIGNMENT REJECT message on any CCCH carried by the relevant CCPCH associated to the RACH physical channel on which the channel request message was received. This message contains the request reference with time of reception of the CHANNEL REQUEST message, and optionally a WAIT_INDICATION field in the Reject structure of the IMMEDIATE ASSIGNMENT REJECT message.

On receipt of an IMMEDIATE ASSIGNMENT REJECT message containing a WAIT_INDICATION field in a Reject structure addressed to the user equipment, where the Request Reference in the Reject structure corresponds to one of its 3 last CHANNEL REQUEST messages,

-	the user equipment shall stop sending signatures and therefore CHANNEL REQUEST messages as well, start timer T3142 with the value indicated in the WAIT_INDICATION field and continue to listen to the CCCH until timer T3146 expires. During this time, the user equipment shall ignore additional IMMEDIATE ASSIGNMENT REJECT messages, but on reception of any IMMEDIATE ASSIGNMENT message corresponding to any other of its 3 last CHANNEL REQUEST messages the user equipment shall stop timers T3146 and T3142, and follow the same procedure as reported in sub-clause 7.1.2.2.1.

-	If no IMMEDIATE ASSIGNMENT message is received before expiration of timer T3146, the user equipment shall indicate a packet access failure to the upper layers and return to packet idle mode (listening to its paging channel). As an option, the user equipment may stop timer T3146 and return to packet idle mode as soon as it has received responses from the network on all, or in case more than 3 were sent, the last 3 of its CHANNEL REQUEST messages.

-	If an erroneous IMMEDIATE ASSIGNMENT message (e.g. the user equipment has been assigned more PDPCHs than it supports according to its multislot class) addressed to the user equipment is received before expiration of timer T3146, the UE shall act as stated in sub-clause 7.1.4.

-	If the user equipment receives an IMMEDIATE ASSIGNMENT message for a downlink TBF establishment, it shall stop timer T3146 if running and respond to the packet downlink assignment as reported in sub-clause 7.2.1.

-	The user equipment is not allowed to make a new attempt for an uplink packet access in the same cell until timer T3142 expires, but may attempt packet access in an other cell after successful cell reselection for radio conditions reasons (see TSM 05.08 [15]). A user equipment in class A or class B mode of operation may attempt to enter the dedicated mode in the same cell before timer T3142 has expired. During the time T3142 is running, the user equipment shall ignore all received PAGING REQUEST messages except paging request to trigger RR connection establishment.

-	The value of the WAIT_INDICATION field (i.e. timer T3142) relates to the cell from which it was received.

##### 7.1.2.3	Pilot-PDPCH Packet access procedure

At the assignment of Pilot – Packet Data Physical channels, the UE shall immediately, or at the indicated TBF starting time if present, switch to the assigned resources and act as follows:

Activate timer T3204, send a PACKET ACCESS message on the uplink assigned P-PDPCH(s) and then start monitoring the assigned downlink P-PDPCH(s) for the detection of the PACKET ACCESS ACK message.

When more than one P-PDCH has been assigned in downlink direction, the network shall send PACKET ACCESS ACK messages only on the first of the assigned downlink transport channels; the UE shall expect such signalling only from the first of the assigned downlink P-PDCHs.

NOTE:	“First” P-PDCH indicates that transport channel which, among those assigned in downlink direction, has the lowest number respect to the transport channel numbering rules defined in TSM 05.02 [13].

The PACKET ACCESS message is delivered as a sequence of N identical bursts each carrying the UE requested RLC mode indication; where N depends on the System Frame Number value modulo 4 (SFNmod4) when sending the first of the group of N PACKET ACCESS bursts as follows: N = (4 – SFNmod4).

The UE shall continue to transmit the PACKET ACCESS message till the reception of a PACKET ACCESS ACK message or the T3204 expiry.

If a PACKET ACCESS ACK message is received, the user equipment shall stop T3204, complete sending the remaining PACKET ACCESS bursts to reach the frame boundary, start  sending radio blocks and, after the first transmitted radio block, start the P-PDCH maintenance procedure (see sub-clause 7.1.2.3.1) and act as follows:

-	start sending RLC data blocks with the extended RLC Header in order to include its TLLI value. At sending of the first RLC data blocks, the UE shall start timer T3166. At the detection of the PACKET UPLINK ACK/NACK or even of a PACKET DOWNLINK ASSIGNMENT message carrying the same TLLI value as used by the UE for sending the RLC blocks, the UE shall stop timer T3166; from now onward, the UE shall send blocks without including the TLLI field. The retransmission of an RLC data block shall include the TLLI if that RLC data block was originally transmitted including this field, even in case the retransmission occurs after the completion of the contention resolution.

-	If the received PACKET UPLINK ACK/NACK message contains the correct TFI but a different TLLI with respect to the one indicated by the UE or timer T3166 expires, the user equipment shall stop all running timers and reinitiate the packet access procedure unless it has already been repeated 4 times. In that case, TBF failure has occurred and an RLC/MAC error should be reported to the higher layer. In such a case, the user equipment shall not transmit a PACKET CONTROL ACKNOWLEDGEMENT in the uplink radio block specified if a valid RRBP field is received as part of the PACKET UPLINK ACK/NACK message.

If timer T3204 expires, or the received message from the network is not the expected one, the user equipment shall reinitiate the packet access procedure unless it has already been repeated 4 times. In that case, the UE shall assume a TBF failure, stop all running timers and an RLC/MAC error should be reported to the higher layer.

The reception of a PACKET ACCESS ACK burst shall grant also the remaining  PACKET ACCESS bursts the UE has to transmit to reach the frame boundary.

When sending the IMMEDIATE ASSIGNMENT message or at the TBF starting time if signalled, the network shall switch to the assigned resources and act as follows:

-	Activate timer T3208 and start monitoring the assigned uplink P-PDPCH(s). On reception of the expected PACKET ACCESS burst, the network shall stop T3208, answer on the downlink P-PDPCH(s) with PACKET ACCESS ACK message delivered as sequence of N identical bursts, where N depends on the System Frame Number value modulo 4 (SFNmod4) when sending the first of the group of N PACKET ACCESS ACK bursts as follows: N = (4 – SFNmod4), start decoding for a radio block and, as soon as a radio block has been correctly decoded, start the P-PDCH maintenance procedure (see sub-clause 7.1.2.3.1).

The network shall respond with a PACKET UPLINK ACK/NACK message including the TLLI to any correctly received RLC data block containing the TLLI.

The network shall always acknowledge any correctly detected PACKET ACCESS message received after a PACKET ACCESS ACK message has been sent.

Before entering the P-PDCH maintenance procedure, the network shall continue to send PACKET ACCESS ACK bursts.

If timer T3208 expires, or the received message from the UE is not the expected one, the network shall stop all running timers and assume failed the P-PDPCH packet access procedure.

##### 7.1.2.3.1	Maintenance of P-PDCHs and Radio Link Failure management

Once entered the P-PDCH maintenance phase (see TSM 05.08 [15]), the UE and at the network shall start a continuous transmission of RLC/MAC data or control blocks on the assigned P-PDCHs in the respective transmission direction; if no valid data is available for transmission, the involved entity shall transmit RLC/MAC Dummy control blocks instead.

If an OUT_OF_SYNC state indication is received from the L1 from all assigned P-PDCH(s), the UE shall start timer T3224. At the detection of an IN_SYNC state indication for any of the assigned P-PDCH(s), the UE shall stop and reset timer T3224.

While timer T3224 is running, the UE shall act as specified for the ongoing procedures.

If timer T3224 expires, the UE shall consider it as a “Radio link failure”; it shall perform an abnormal release procedure (see sub-clauses 8.8.1 and 8.8.2).

If an OUT_OF_SYNC state indication is received from the L1 for the affected P-PDCH(s), the network shall start timer T3228. At the detection of an IN_SYNC state indication for that P-PDCH(s), the network shall stop and reset timer T3228.

While timer T3228 is running, the network can perform either of the following actions:

-	a selective packet resource release towards the affected P-PDPCH(s) (see sub-clause 8.2),

-	a packet resource reconfiguration (i.e. via a PACKET RESOURCE RECONFIGURE or a PACKET UPLINK ASSIGNMENT or a PACKET DOWNLINK ASSIGNMENT message).

If timer T3228 expires, the network shall declare a “Radio link failure” of the affected P-PDCH(s) and initiate an abnormal release of the involved TBF as described in sub-clause 8.1.1.5 for uplink TBF release or in sub-clause 8.1.2.5 for downlink TBF release.

Timers T3224 and T3228 shall be stop and reset at the UE and network side respectively, whenever the affected P-PDCH(s) is reconfigured by the network (i.e. via a PACKET RESOURCE RECONFIGURE or a PACKET UPLINK ASSIGNMENT or a PACKET DOWNLINK ASSIGNMENT or a selective PACKET RESOURCE RELEASE message).

For this release, parameter T3224 is assumed to be equal to zero.

Parameter T3228 is configurable by the operator.

The UE shall use the last value of timer T3224 received either from the BCCH or from PACCH.

#### Timing Advance

When accessing on an uplink P-PDPCH, the UE shall apply the Synchronisation Shift (SS) adjustment as received by the network with the PHYSICAL INFORMATION message (see sub-clauses 7.1.2.1 and 7.1.2.2.1), possibly corrected with the SS adjustment indication if carried in the IMMEDIATE ASSIGNMENT message (see TSM 04.08 [11]). The UE is not allowed to transmit on a P-PDPCH resource until it receives from the network the expected SS adjustment indication.

Thereafter, timing alignment shall be maintained by the uplink synchronisation control procedure as described in TSM 05.08 [15]. The uplink synchronisation control procedure makes use of the Layer 1 SS symbols contained in the bursts of the downlink P-PDPCH resources.

The network can also explicitly command a given timing adjustment to the UE with a PACKET POWER CONTROL/TIMING CONTROL message.

##### 7.1.2.3.3	Power control

When accessing on an uplink P-PDPCH resource, the UE shall apply the transmit power level, corrected by the estimated path loss, as received by the network with the IMMEDIATE ASSIGNMENT message (see TSM 04.08 [11]); while the network can set the initial transmit power level on a downlink P-PDPCH resource based on the UE reported (via the CHANNEL REQUEST message; see TSM 04.08 [11]) path loss and time slot interference.

Thereafter, the UE and the network transmit power levels shall be maintained by the closed loop power control procedure as described in TSM 05.08 [15]. The closed loop Power Control (PC) procedure makes use of the Layer 1 PC symbols contained in the bursts of the respective downlink or uplink P-PDPCH resources.

P-PDPCH resources which may be added during the course of a packet call (see clause 8), shall be included into the closed loop power control procedure (see TSM 05.08 [15]) at the indicated starting time, if present, otherwise respectively at the receive side as soon as the first data block is correctly decoded by the Layer 1 and, at the transmit side, as soon as the first data block has been delivered. P-PDPCH resources which may be released during the course of a packet call, shall be removed from the closed loop power control procedure as soon as the PACKET RESOURCE RELEASE message has been respectively delivered or received.

The network can also explicitly command a given transmit power level adjustment to the UE with the PACKET POWER CONTROL/TIMING CONTROL message.

##### 7.1.2.3.4	Beam-Forming

The network can exploit the continuous transmission and the duplex assignment mode of the P-PDPCH resources (see sub-clause 4.5.1) to estimate from the received signal the beam-forming coefficients to apply for the transmission.

#### 7.1.2.4	Contention resolution for one phase access procedure

The one phase access procedure applies in case the network has assigned to the user equipment S-PDPCHs.

As soon as it has switched to the assigned resources, in order to be uniquely identified when sending on uplink, the UE shall send RLC data blocks with the extended RLC Header to include its TLLI until the contention resolution is completed.

The retransmission of an RLC data block shall include the TTLI if that RLC data block was originally transmitted including this field, even in case the retransmission occurs after the completion of the contention resolution.

At sending of the first RLC data block, the user equipment shall stop timer T3164 if running (see TSM 04.08 [11] for a detailed description of timer T3164), set counter N3104 to 1 and start timer T3166.

The counter N3104 shall be stepped each time the user equipment sends an RLC data block.

The network shall respond by including the TLLI in the PACKET UPLINK ACK/NACK message after the first correctly received RLC data block that comprises the TLLI.

The contention resolution is completed on the network side when the network receives a TLLI value identifying the user equipment, as part of the contention resolution procedure on the TBF. The network shall then stop timer T3141.

The contention resolution is completed on the user equipment side when the UE receives a PACKET UPLINK ACK/NACK message with the same TLLI as the user equipment has included in the RLC header of the first RLC data blocks. The mobile shall then stop timer T3166 and counter N3104.

The contention resolution has failed on the user equipment side when the counter N3104 has reached its maximum value, or on expiry of timer T3166, or if the user equipment receives a PACKET UPLINK ACK/NACK message with the right TFI but with another TLLI than the user equipment has included in the RLC header of the first RLC data blocks; in such a case, the user equipment shall not transmit a PACKET CONTROL ACKNOWLEDGEMENT in the uplink radio block specified if a valid RRBP field is received as part of the PACKET UPLINK ACK/NACK message.

The user equipment shall then reset the counter N3104, stop timer T3166 if not expired, immediately stop transmitting on this TBF.

The UE shall reinitiate the packet access procedure unless it has already been repeated 4 times. In that case, TBF failure has occurred and an RLC/MAC error should be reported to the higher layer.

#### 7.1.2.4.1	One phase packet access procedure completion

The one phase packet access procedure is completed upon a successful contention resolution. The user equipment has entered the packet transfer mode.

### 7.1.3	TBF establishment using two phase access

The two phase access procedure applies in case the network has assigned to the user equipment S-PDPCHs.

#### 7.1.3.1	Initiation of the Packet resource request procedure

In the first phase of a two phase access, the same procedures as for one phase access or P-PDPCH assignment are used until the network sends an IMMEDIATE ASSIGNMENT message including a Single Block Allocation struct, denoting two phase access to the user equipment. In that message, the network reserves for the UE one radio block on uplink PDPCH(s) where the UE can transmit a PACKET RESOURCE REQUEST message.

The two phase access can be initiated:

- at the network side, by ordering the user equipment to send a PACKET RESOURCE REQUEST message. The order is sent implicitly to the user equipment in the IMMEDIATE ASSIGNMENT message by including the Single Block Allocation structure.

- at the user equipment side, by requiring a two phase access in the CHANNEL REQUEST message. In this case, if access is granted (by the IMMEDIATE ASSIGNMENT message), the network shall order the user equipment to send a PACKET RESOURCE REQUEST message. The order is sent implicitly to the user equipment in the IMMEDIATE ASSIGNMENT message by including the Single Block Allocation Structure.

When the user equipment has received an IMMEDIATE ASSIGNMENT message it shall respond with a PACKET RESOURCE REQUEST message in the allocated single radio block on the assigned uplink PDPCH(s). At sending of the PACKET RESOURCE REQUEST message, the user equipment shall start timer T3168.

The PACKET RESOURCE REQUEST message shall include the UE Radio Access Capability field (see sub-clause 11.3.8).

The user equipment may request an open-ended or a close-ended TBF. If a close-ended TBF is requested, the number of octets of user data that the UE has to transfer in the TBF shall be indicated in the PACKET RESOURCE REQUEST message.

The user equipment shall indicate in the RLC_OCTET_COUNT the maximum number of octets of user data that are to be transferred in the TBF in case of a close-ended TBF. The value '0' as a request for an open-ended TBF.

#### 7.1.3.2	Packet resource assignment for uplink procedure

##### 7.1.3.2.1	On receipt of a PACKET RESOURCE REQUEST message

On receipt of a PACKET RESOURCE REQUEST message, the network shall respond by sending a PACKET UPLINK ASSIGNMENT (radio resources assignment on one or more PDCHs to be used by the user equipment for the TBF) or a PACKET ACCESS REJECT message to the user equipment on the PACCH of a downlink PDCH.

On receipt of a PACKET UPLINK ASSIGNMENT message the user equipment shall switch to the assigned PDPCHs and stop T3168.

If the UE detects an invalid Frequency Parameters information element in the assignment message, it shall abort the procedure and may then re-initiate the access procedure.

On receipt of a PACKET ACCESS REJECT message that contains a WAIT_INDICATION field in a Reject structure addressed to the user equipment, the user equipment shall stop timer T3168 and start timer T3172 with the indicated value (Wait Indication). The user equipment is not allowed to make a new attempt for packet access in the same cell until timer T3172 expires, but may attempt a packet access in an other cell after successful cell reselection. When timer T3172 expires, the user equipment may initiate the establishment on an uplink TBF on CCCH.

On receipt of a PACKET ACCESS REJECT message that contains a Reject structure addressed to the user equipment without a WAIT_INDICATION field, the user equipment shall stop timer T3168 and shall decode the SYSTEM INFORMATION messages before re-attempting the uplink TBF establishment.

On expiry of timer  T3168, contention resolution has failed on the user equipment side. The UE shall then reinitiate the packet access procedure unless it has already been repeated 4 times. In that case, TBF failure has occurred and an RLC/MAC error should be reported to the higher layer.

#### 7.1.3.3	Contention resolution at two phase access

The contention resolution is completed on the network side when the network receives a TLLI value identifying the user equipment, as part of the contention resolution procedure on the TBF.

The contention resolution is completed on the user equipment side when the user equipment receives a PACKET UPLINK ASSIGNMENT message with the same TLLI as the user equipment has included in the PACKET RESOURCE REQUEST message. The user equipment shall then stop timer  T3168. It does not include its TLLI in any successive RLC data block.

The contention resolution has failed on the user equipment side when the user equipment does not receive a PACKET UPLINK ASSIGNMENT message with its TLLI before expiry of timer T3168. The user equipment shall then reinitiate the packet access procedure unless it has already been repeated 4 times. In that case, TBF failure has occurred.

#### 7.1.3.4	Two phase packet access completion

The two phase packet access procedure is completed upon a successful contention resolution. The user equipment has entered the packet transfer mode.

### 7.1.4	Abnormal cases

If a failure occurs on the user equipment side of the new TBF before user equipment has successfully entered the packet transfer mode, the newly reserved resources are released; the subsequent behaviour of the user equipment depends on the type of failure and previous actions.

-	If the failure is due to a TLLI mismatch or to the expiry of timers T3146 or T3204 or T3164 or T3166 or T3168 or to the fact that the counter N3104 reaches its maximum value in the contention resolution procedure, and the maximum number of repetitions as described respectively in sub-clauses 7.1.2.1, 7.1.2.3, 7.1.2.4, 7.1.3.2.1 or 7.1.3.3 has been performed, the user equipment shall remain in packet idle mode and notify to higher layer a TBF establishment failure; transactions in progress shall be aborted and cell reselection continued, unless the failure takes place during a RR-cell change order procedure, in which case the UE behaviour shall be as described in the Abnormal cases of the RR-Network Commanded Cell Change Order Procedure in TSM 04.08 [11].

-	If Timer T3146 expires and the UE requested for a single block without TBF establishment, the UE shall, unless otherwise specified, discard the message triggering the packet channel request and remain to idle mode.

-	If the user equipment has been assigned PDPCHs either in a non supported frequency/frequency band or which violates its multislot classor according to the uplink shared access mode while it supports P-PDCHs only in uplink direction, it shall reinitiate the packet access procedure unless it has already been repeated 4 times. In that case, the UE shall remain in packet idle mode and notify to higher layer that a TBF failure has occurred.

- If the assigned resources are not satisfactorily defined and the packet access procedure has been repeated the maximum allowed number of times, the UE shall notify a TBF failure to higher layers and return to packet idle mode.

-	On expiry of timer T3172, the user equipment shall reinitiate the packet access procedure unless it has already been reinitiated 4 times, in which case the user equipment shall return to packet idle mode and notify to higher layers a TBF establishment failure.

- If timer T3141 elapses before a successful contention resolution procedure is completed, the newly allocated TBF is released.

-	If the failure is due to any other reason, the user equipment shall return to packet idle mode, notify higher layer (TBF establishment failure), transactions in progress shall be aborted and cell reselection continues.

### Timing Advance on S-PDPCHs

When accessing on an uplink S-PDPCH, the UE shall apply the Synchronisation Shift (SS) adjustment as received by the network with the PHYSICAL INFORMATION message (see sub-clauses 7.1.2.1 and 7.1.2.2.1), eventually corrected with the SS adjustment indication carried in the IMMEDIATE ASSIGNMENT message (see TSM 04.08 [11]). The UE is not allowed to transmit on a S-PDPCH resource until it receives from the network the expected SS adjustment indication.

Thereafter, the following cases may apply:

- If a P-PDPCH is already active, the UE shall maintain the timing alignment on the uplink S-PDPCH resources exploiting the uplink synchronisation control procedure carried out on the established P-PDPCH resources (see sub-clause 7.1.2.3.2.). Therefore, the Layer 1 SS symbols of the S-PDPCHs shall remain unused.

- Otherwise, the UE shall adjust the timing according to the higher layer indication provided by the network.

The network can explicitly command a given timing adjustment to the UE with a PACKET POWER CONTROL/TIMING CONTROL message.

### 7.1.6	Power control on S-PDPCHs

When accessing on an uplink S-PDPCH resource, the UE shall apply the transmit power level, corrected by the estimated path loss, as received by the network with the IMMEDIATE ASSIGNMENT message (see TSM 04.08 [11]); while the network shall set the initial transmit power level on a downlink S-PDPCH resource based on the UE reported (via the CHANNEL REQUEST message; see TSM 04.08 [11]) path loss and time slot interference.

Thereafter, the following cases may apply:

- If a P-PDPCH is already active, the UE and the network shall set their respective transmit power level on the S-PDPCH resources to the same value (scaled by the respective spreading factors) in use for the time co-located P-PDPCH resources, thus exploiting the closed loop power control procedure carried out on the established P-PDPCH resources (see sub-clause 7.1.2.3.3). Therefore, the Layer 1 PC symbols of the S-PDPCHs shall remain unused.

- Otherwise, if no P-PDPCH resource is active or for those S-PDPCH resources which are not time co-located with any active P-PDPCH resource, open loop power control shall apply (see TSM 05.08 [15]).

The network can explicitly command a given transmit power level adjustment to the UE with the PACKET POWER CONTROL/TIMING CONTROL message.

### 7.1.7	Beam-forming on S-PDPCHs

Beam-forming requires continuous or frequent, with respect to the user equipment speed, transmission on the uplink direction. Therefore, S-PDPCH resources can exploit the continuous transmission on P-PDPCH resources for the beam coefficients estimation when present.

## 7.2	TBF establishment initiated by the network

The purpose of network initiated TBF establishment is to establish a TBF to support the transfer of LLC PDUs in the direction from the network to the user equipment. The procedure may be entered when the user equipment is in packet idle mode. Network initiated TBF establishment can also be done on PACCH if a TBF for transfer of LLC PDUs in the direction from the user equipment to the network is already established (sub-clause 8.1.1.4).

### 7.2.1	Entering the packet transfer mode

The procedure is triggered by a request from upper layers on the network side to transfer a LLC PDU to a user equipment in packet idle mode. The request from upper layers specifies an optional priority level, a QoS profile including the requested RLC mode, optional DRX parameters, an optional IMSI and an optional UE Radio Access Capability, multislot class and user equipment classmark to be associated with the packet transfer. The request is implicit when receiving an LLC PDU to a user equipment not already having any assigned radio resources. Upon such a request, the network shall initiate a packet downlink assignment procedure as defined in sub-clause 7.2.1.1.

#### 7.2.1.1	Packet downlink assignment procedure

The network may assign radio resources on one or more PDPCHs to be used for the TBF. The amount of radio resources to be reserved is a network dependent choice.

The allocated radio resources are assigned to the user equipment in an IMMEDIATE ASSIGNMENT message. The IMMEDIATE ASSIGNMENT message is transmitted on the CCPCH carrying the CCCH group of the addressed user equipment. The appropriate CCCH group is calculated from the IMSI, as described in TSM 05.02 [13].

If the user equipment does not apply DRX, or if the DRX parameters are not provided by the upper layers, the IMMEDIATE ASSIGNMENT message, or the first part of the IMMEDIATE ASSIGNMENT message (in case of a two-message assignment) can be sent on any PCH block carried by the CCPCH which the UE CCCH group belongs to, without any further restriction.

If the user equipment applies DRX, the IMMEDIATE ASSIGNMENT message, or the first part of the IMMEDIATE ASSIGNMENT message (in case of a two-message assignment), shall be sent on a PCH block corresponding to the paging group determined for the user equipment in packet idle mode, see TSM 05.02 [13].

The behaviour of the network, for the calculation of the CCCH group where the IMMEDIATE ASSIGNMENT message has to be sent, when the IMSI is not provided by the upper layers is implementation dependent. In this case, the IMMEDIATE ASSIGNMENT message, or the first part of the IMMEDIATE ASSIGNMENT message (in case of a two-message assignment), can be sent on any PCH block of any CCPCH without any further restriction.

In case of a two-message assignment, the network and the UE shall follow the procedures and the rules detailed in TSM 04.08 [11].

At the receipt of the IMMEDIATE ASSIGNMENT message or, in case of a two-message assignment, a matching pair of IMMEDIATE ASSIGNMENT messages, the UE shall stop monitoring the downlink CCCH and switch to the assigned resources.

In case the ACCESS_PRIORITIES information element in the IMMEDIATE ASSIGNMENT message indicates the request for sending a signature on the UpPTS physical channel, the user equipment shall send, before switching to the assigned PDPCHs, at the most 3 consecutive signatures till the detection of the PHYSICAL INFORMATION message (see TSM 04.08 [11]). In case no acknowledgement to any of the sent signatures is detected, the UE shall abort the procedure and return to the idle mode (see TSM 05.10 [16] for the timing constraints).

The IMMEDIATE ASSIGNMENT message may indicate an assignment starting time in the TBF Starting Time parameter. In this case, if the TBF starting time is not yet elapsed, the user equipment shall monitor the relevant CCPCH until the point in time denoted by the TBF Starting Time. Thereafter, or otherwise if the TBF starting time is already elapsed, it shall switch to the assigned PDPCHs. If while monitoring the CCPCH the user equipment receives more than one IMMEDIATE ASSIGNMENT message, it shall act upon the most recently received message and shall ignore the previous message.

In case the network has assigned P-PDPCH(s) only, the UE shall proceed with the P-PDPCH packet access procedure according to sub-clause 7.1.2.3 with the following constraints:

- At the detection of the expected PACKET ACCESS ACK message from the network, the UE shall immediately move to the P-PDCH maintenance procedure as reported in sub-clause 7.1.2.3.1, skipping the other steps of sub-clause 7.1.2.3, that is the TLLI contention resolution phase;

- Timer T3204 shall be started only once; if it expires, a TBF establishment failure occurs and the UE shall return to packet idle mode;

- The UE shall set the RLC mode in the PACKET ACCESS message according to the RLC mode value signalled by the network in the IMMEDIATE ASSIGNMENT message;

- The packet downlink assignment procedure is completed when the user equipment detects a valid PACKET ACCESS ACK message.

In case the network has assigned S-PDPCH(s) only, then:

- the user equipment shall, once switched to the assigned resources, start timer T3190; this timer is restarted when receiving the first valid RLC/MAC block.

- On expiry of timer T3190, the user equipment shall abort the procedure and return to packet idle mode.

- The Packet downlink assignment procedure is completed when the user equipment receives a valid    RLC/MAC block. The user equipment has entered the packet transfer mode.

In case the network has assigned both P-PDPCH(s) and S-PDPCH(s), the user equipment shall first proceed with the P-PDPCH packet access procedure as detailed above and then, if succeeded, access to the S-PDPCH(s) starting timer T3190. On expiry of timer T3190, the UE shall continue to operate on the assigned resources; as soon as a valid radio block is received from any of the assigned downlink shared resources, timer T3190 shall be started again.

Timing advance and power control on the assigned resources shall be carried out as described in sub-clauses 7.1.2.3.2 and 7.1.2.3.3 or in sub-clauses 7.1.5 and 7.1.6 respectively, according to the granted MAC access mode.

#### 7.2.1.2	Packet polling procedure

The network may send to the user equipment a PACKET POLLING REQUEST message. If the UE has received an IMMEDIATE ASSIGNMENT message with no starting time or with a starting time that has already elapsed, the PACKET POLLING REQUEST message shall be sent on PACCH, otherwise the network shall wait for the elapsing of the TBF starting time. The user equipment shall be addressed by its TLLI or TFI.

On receipt of a PACKET POLLING REQUEST message, the user equipment shall respond to the network with the PACKET CONTROL ACKNOWLEDGEMENT message in the reserved uplink radio block specified by the RRBP field on the indicated uplink timeslot. The reserved block is considered as a one block PACCH allocation.

The network can exploit this procedure for maintaining the UE timing alignment and for setting the transmit power level on the downlink S-PDPCHs based on the UE reported measurements via the PACKET CONTROL ACKNOWLEDGMENT message.

### 7.2.2	Abnormal cases

If a failure occurs at the UE side on the new TBF, before the user equipment has successfully entered the packet transfer mode, the newly reserved resources are released; the subsequent behaviour of the UE shall depend on the type of failure and previous actions.

-	If the user equipment has been assigned more PDCHs than it supports according to its UE multislot class, it shall return to packet idle mode.

-	If the user equipment has been assigned resources on a non supported frequency/frequency band, it shall return to packet idle mode.

-		On expiry of timer T3190, the user equipment shall return to packet idle mode if no P-PDPCH is active, otherwise it shall continue to operate on the assigned resources; as soon as a valid radio block is received from any of the assigned downlink shared resources, timer T3190 shall be started again.

-	On expiry of timer T3204, the user equipment shall return to packet idle mode.

If the failure is due to any other reason, the user equipment shall return to packet idle mode.

## 7.3	Procedure for measurement report sending in packet idle mode

The procedure for measurement report sending shall be initiated by the user equipment at expiry of either the NC measurement report interval timer T3158 or the EM measurement report interval timer T3178. At expiry of the timer T3158 or T3178, the user equipment shall restart the expired timer T3158 or T3178, perform the measurements and initiate the packet access.

The procedure for measurement report sending is initiated by the user equipment as reported in sub-clause 7.3.1.

If the user equipment initiates an RR connection establishment, the timers T3158 and T3178 shall be stopped and no measurement reports shall be sent. When the RR connection is released and if the user equipment has not changed cell, the measurement reporting procedure shall be restarted.

If the user equipment performs anonymous access, the timers T3158 and T3178 shall continue to run, but no measurement report shall be sent. When the anonymous access terminates, the measurement reporting shall be resumed.

If a cell change has occurred during the RR connection, the measurements shall be cancelled until new NC or EM-orders have been received (see sub-clause 5.6).

### 7.3.1	Measurement report sending procedure

The packet access procedure is initiated by the RR entity in the user equipment, with the user equipment sending a CHANNEL REQUEST message indicating ‘Single block without TBF establishment’ on the RACH.

The packet access procedure shall be as detailed in sub-clauses 7.1.2.1 and 7.1.2.2.

The network may then respond with an IMMEDIATE ASSIGNMENT message granting for a ‘single block access’ on a PDCH, or with an IMMEDIATE ASSIGNMENT REJECT message if uplink resources are not available.

At the reception of an IMMEDIATE ASSIGNMENT message, the UE shall send the PACKET MEASUREMENT REPORT message in the allocated radio block on the assigned PDCH and then immediately switch back to the CCCH associated to its paging group in non-DRX mode (see sub-clause 5.5.1.5).

The PACKET MEASUREMENT REPORT message shall contain either the NC Measurement Report struct or the EXT Measurement Report struct.

No TBF is established and the network shall not acknowledge the reception of the PACKET MEASUREMENT REPORT message.

If requested by the network in the IMMEDIATE ASSIGNMENT message, the UE shall try to perform timing and transmit power level adjustment by sending signatures on the UpPTS, before accessing on the assigned resources. The UE shall continue sending signatures till the detection of the PHYSICAL INFORMATION message (see TSM 04.08 [11]) or till the TBF starting time for the granted radio block. If, at the TBF starting time, the UE has not yet received the expected PHYSICAL INFORMATION message within the last 200 ms, it shall discard the Measurement Report initiating the packet access and return to packet idle mode.

If Timer T3146 expires before an IMMEDIATE ASSIGNMENT message is received, or if the received IMMEDIATE ASSIGNMENT message contains faulty parameters, the packet access procedure is aborted, the Measurement Report initiating the packet access shall be discarded and the UE shall return to packet idle mode.

On receipt of an IMMEDIATE ASSIGNMENT REJECT message the user equipment shall follow the procedure specified in sub-clause 7.1.2.2.2 with the exception of the actions taken when either of timers T3142 or T3146 expires. In this case, the measurement report initiating the packet access shall be discarded and the user equipment shall return to packet idle mode.

If any of the measurement report interval timers T3158 or T3178 expires before any of timers T3142 or T3146 expires, no new measurement shall be initiated but the timer T3158 or T3178 shall be restarted.

## 7.4	Cell Change Order procedures in Packet Idle mode

For an individual user equipment in packet idle mode, the network may initiate the cell change order procedure on CCPCH corresponding to the UE’s Paging Group (see TSM 05.02 [13]).

### 7.4.1	Cell Change Order procedure initiated on CCCH

The network may initiate the cell change order procedure by sending an IMMEDIATE ASSIGNMENT message for single block assignment in a CCCH block monitored by the user equipment. No TBF shall be established. The single block assignment procedure is specified in TSM 04.08 [11].

The network shall then send the PACKET CELL CHANGE ORDER message in the assigned downlink block to the user equipment. The PACKET CELL CHANGE ORDER message contains:

-	the characteristics of the new cell that are necessary to identify it (i.e. respectively (Code group + Basic midamble sequence + BCCH frequency) for a TD-SCDMA cell or (BSIC + BCCH frequency) for a GSM cell; see TSM 05.02 [13] and TSM 04.08 [11]);

-	the NC measurement parameters valid for the user equipment in the new cell (NETWORK_CONTROL_ORDER and optionally: NC_NON_DRX_PERIOD, NC_REPORTING_PERIOD_I and NC_REPORTING_PERIOD_T).

Upon receipt of the PACKET CELL CHANGE ORDER message, the user equipment shall stop all relevant RLC/MAC timers except for timers related to measurement reporting and start timer T3174. The user equipment shall then switch to the specified new cell and obey the relevant RLC/MAC procedures on this new cell. If a valid RRBP field was received in the PACKET CELL CHANGE ORDER message, then the UE shall send a PACKET CONTROL ACKNOWLEDGMENT message in the reserved uplink radio block specified by the RRBP field before switching to the new cell. If the timers related to measurement reporting expire while the cell reselection procedure has not yet been completed, these timers shall be restarted so that the UE resumes the measurement reporting procedures once camped on the new cell.

The user equipment shall obey the PACKET CELL CHANGE ORDER irrespective of whether or not the UE has any knowledge of the relative synchronisation of the target cell to the serving cell.

The procedure for completion of the cell change order is defined in sub-clause 8.4.1 and abnormal procedures are defined in sub-clause 8.4.2.

## 7.5	Measurement Order procedures in Packet Idle mode

To send either the NC Measurement order or the Extended Measurement order to an individual user equipment in packet idle mode, the network may establish a connection on CCCH.

### 7.5.1	Measurement Order procedures initiated on CCCH

The network may initiate the measurement order procedure by allocating a single block in an IMMEDIATE ASSIGNMENT message sent to the user equipment on a CCCH block in the same way as specified in sub-clause 7.4.1.

The network shall then send the PACKET MEASUREMENT ORDER message in the assigned downlink block to the user equipment. If the PACKET MEASUREMENT ORDER message contains multiple instances, the network has to repeat the complete procedure with new assignment for each instance of the message.

The PACKET MEASUREMENT ORDER message may contain the following optional Measurement order parameters:

-	TLLI (shall be included)

-	NC Measurement Parameters (NETWORK_CONTROL_ORDER; NC_NON_DRX_PERIOD; NC_REPORTING_PERIOD_I; NC_REPORTING_PERIOD_T; NC_FREQUENCY_LIST);

-	EXT Measurement Parameters (EXTENDED_MEASUREMENT_COMMAND_CHANGE_MARK; EXT_MEASUREMENT_ORDER; EXT_REPORTING_PERIOD_T; EXT_REPORTING_PERIOD_I; Intra-Cell Extended Measurements; Inter-Cell Extended Measurements).

Upon receipt of the PACKET MEASUREMENT ORDER message, the user equipment shall store the Measurement order parameters and immediately return to the relevant CCPCH without sending any acknowledgement to the network. The user equipment shall obey the NETWORK_CONTROL_ORDER and the EXT_MEASUREMENT_ORDER as specified in TSM 05.08 [15] and in sub-clause 5.6.

# 8	Medium Access Control (MAC) Procedures in Packet Transfer Mode

## 8.1	RLC data blocks transfer

The exchange of RLC data blocks between the network and a UE requires the establishment of a “Packet Connection” at the air interface (Uu).

A packet connection comprises a set of uplink and/or downlink PDPCHs. The allocated PDPCHs can be of the type Pilot – PDPCH and/or Shared –PDPCH, as defined in sub-clause 4.5. The packet physical resources can be assigned either through the RR IMMEDIATE ASSIGNMENT message (see TSM 04.08 [11]), or through the MAC PACKET UPLINK/DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE control messages.

When the assignment message contains the Priority Access Parameters field (see sub-clause 11.4.10), the UE is requested to be equalised before accessing on the new assigned resources. Depending on the assignment message, the UE shall behave as follows:

-	If the packet resources are assigned via an IMMEDIATE ASSIGNMENT message, the UE shall send signatures on the UpPTS following the rules described in sub-clause 7.1.2.2.1 or 7.2.1.1, respectively for a UE initiated and for a network initiated TBF establishment;

Otherwise:

-	The UE shall send signatures till the detection of the expected PHYSICAL INFORMATION message before accessing to the new assigned resources. The UE shall not access to the new assigned resources if, at the expected access time, or at the indicated RRBP radio block for acknowledging the assignment message if requested, a valid higher layer synchronisation shift (adjustment) command was not executed within the last 200 ms since the detection of the assignment message. In this case, the UE shall consider the assignment message as lost; it shall not acknowledge the assignment message at the indicated RRBP field when requested, and continue with the current operative mode.

The RLC data blocks transfer mode depends on the type of the assigned PDPCH resources, P-PDPCH or S-PDPCH respectively, on the respective transmission direction and, for shared resources, on the defined MAC access mode. As reported in sub-clause 4.5, P-PDPCH is a Physical Packet Data Channel uniquely assigned to a single user which requires continuous transmission whatever the TBF direction be; while on an S-PDPCH transmission depends on the assigned medium access mode, dynamic allocation or fixed allocation respectively, as well as on the TBF direction as described in the following sub-clauses.

Within the duration of a Packet Connection, uplink and/or downink TBFs are carried out in order to transmit packet data in the respective transmission direction.

When an assignment message assigns both pilot and shared resources, the UE shall first complete access to the pilot resources and then, once settled (i.e. the new pilot resources have entered the P-PDCH maintenance procedure; see sub-clause 7.1.2.3.1), to the shared ones, according to the rules and procedures detailed in the following sub-clauses. The access to pilot and shared resources shall happen at the same time if the new assigned pilot resources do not require the exchange of the PACKET ACCESS and PACKET ACCESS ACK messages.

### 8.1.1	Uplink RLC data block transfer

Prior to the initiation of an RLC data block transfer on the uplink, the network assigns the following parameters to characterise the uplink TBF in the IMMEDIATE ASSIGNMENT or PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message:

-	a unique Temporary Flow Identity (TFI). The user equipment shall set the TFI field of each uplink RLC data block to the TFI value assigned to the user equipment in the IMMEDIATE ASSIGNMENT or PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message.

-	a set of PDPCHs to be used for the uplink transfer;

-	a TBF Starting Time indication (optional in case of a dynamic allocation or for P-PDPCH assignment; mandatory for a fixed allocation).

The assigned PDPCHs can be either P-PDPCHs or S-PDPCHs or a mix of the two types; in case S-PDPCHs are assigned to one TBF, these shall all have the same allocation mode that is either dynamic or fixed.

All the RLC data blocks of a non-concurrent uplink TBF (i.e. of a TBF initiated via an IMMEDIATE ASSIGNMENT message) which are sent on a P-PDCH, or on a S-PDCH with one phase access, shall contain a TLLI field in the RLC data block header until the P-PDPCH is established or the one phase access contention resolution procedure is completed at the user equipment side (see sub-clauses 7.1.2.3 and 7.1.2.4). After the reaction time specified in TSM 05.10 [16], no other RLC data blocks shall contain a TLLI field, except for those retransmitted RLC data blocks that originally contained a TLLI, which shall be repeated including the same TLLI (see sub-clauses 7.1.2.3 and 7.1.2.4). The TLLI_BLOCK_CHANNEL_CODING parameter in the IMMEDIATE ASSIGNMENT message indicates whether an RLC data block containing a TLLI field in the RLC data block header shall be encoded using CS-1 or using the commanded channel coding scheme (see TSM 05.03 [14]). The user equipment shall send all other RLC data blocks using the commanded channel coding scheme.

Upon receipt of a command from the network to change channel coding scheme, the user equipment shall react in accordance with the time specified in TSM 05.10 [16].

Upon receipt of any message containing an uplink assignment (IMMEDIATE ASSIGNMENT message, PACKET UPLINK ASSIGNMENT message, PACKET RESOURCE RECONFIGURE message or PACKET UPLINK ACK/NACK message), the user equipment shall be ready to transmit in accordance with the requirements given in TSM 05.10 [16]. In particular, the UE shall start to transmit according to one of the following rules:

-	at the TBF starting time, if present,

-	after the acknowledgement of the uplink assignment message, if explicitly requested by the network,

-	immediately at the detection of the uplink assignment message.

The above mentioned rules are listed according to the priority order in which they shall be applied.

Note: In presence of both a TBF starting time indication and a valid RRBP field, the UE will act according to the message, executing it, at the indicated TBF starting time whereas the acknowledgement message shall be sent in the radio block indicated by the RRBP field. The TBF starting time takes higher priority than the RRBP field.

The user equipment shall transmit RLC/MAC blocks with the following priority:

-	RLC/MAC control blocks, except Packet Uplink Dummy Control Blocks

-	RLC data blocks (Note)

-	RLC/MAC control blocks containing Packet Uplink Dummy Control Blocks

During the TBF, if the countdown procedure has not started, the user equipment shall ask for new or different radio resources, by sending a PACKET RESOURCE REQUEST message (see sub-clauses 8.1.1.1.2 and 8.1.1.3.2), in the case when:

- the user equipment has more blocks to send than those allowed by the currently assigned resources.

Note: In presence of concurrent TBFs on pilot channels, priority of signalling over data is not applied if the UE has to send PACKET DOWNLINK ACK/NACK messages, except when explicitly requested by the network (see sub-clause 8.1.2.6).

#### 8.1.1.1	Uplink RLC data block transfer on a P-PDCH

If currently no P-PDCH is active between the network and the user equipment, the network shall assign to the UE a complete P-PDCH (i.e. having at least one physical resource per transmission direction; see sub-clause 4.5). The assignment shall be done by the IMMEDIATE ASSIGNMENT message in case no TBF is in progress, otherwise by either the PACKET UPLINK ASSIGNMENT or the PACKET RESOURCE RECONFIGURE message.

At the reception of an assignment for a P-PDCH, or at the TBF starting time if present, the UE shall switch to the assigned resources and start with the network the P-PDPCH packet access procedure as described in sub-clause 7.1.2.3, if no TBF is currently active, otherwise according to sub-clause 7.1.2.3 with the following distinctions:

- At the detection of the expected PACKET ACCESS ACK message from the network, the UE shall immediately move to the P-PDCH maintenance procedure as reported in sub-clause 7.1.2.3.1, skipping the other steps of sub-clause 7.1.2.3, that is the TLLI contention resolution phase;

-	If timer T3204 expires, the P-PDPCH packet access procedure failed; the UE shall notify a TBF failure to higher layers and perform abnormal release with random access (see sub-clause 8.8.2);

- The UE shall set the RLC mode value in the PACKET ACCESS message according to the requested uplink RLC mode.

During the P-PDPCH packet access procedure, the UE shall continue the transmission and/or the reception on the already allocated S-PDPCHs if any (see sub-clause 8.1.1.2 and 8.1.1.3).

When more than one P-PDCH has been assigned in uplink direction, the UE shall send PACKET ACCESS messages only on the first of the assigned uplink transport channels; the network shall expect such signalling only from the first of the assigned uplink P-PDCHs.

NOTE:	“First” P-PDCH indicates that transport channel which, among those assigned in uplink direction, has the lowest number respect to the transport channel numbering rules defined in TSM 05.02 [13].

As soon as the P-PDPCH packet access procedure has been completed, both the user equipment and the network shall enter the P-PDCH maintenance procedure as described in sub-clause 7.1.2.3.1.

If a P-PDCH is already active between the network and the user equipment, the network can assign to the UE additional uplink P-PDCH resources by means of the PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message. At the reception of the assignment message or at the TBF starting time if present, the UE shall switch to the newly assigned resources; then both the UE and the network shall immediately include the new assigned pilot resources in the P-PDCH maintenance and radio link failure management procedure (according to the rules described in sub-clause 7.1.2.3.1). No P-PDPCH packet access procedure shall therefore be initiated.

If an uplink TBF is already in progress, the UE shall continue to use the assigned parameters of the uplink TBF till the point in time it switches to the P-PDCH resources; at which time it shall use the newly assigned uplink TBF parameters. If while waiting for the frame number indicated by the TBF starting time the UE receives another assignment, it shall act upon the most recently received one ignoring the previous received assignment.

If the IMMEDIATE ASSIGNMENT or PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message contains the RLC_DATA_BLOCKS_GRANTED field, the TBF is a close-ended TBF; otherwise the TBF is open-ended.

During a close-ended TBF the UE shall transmit at the most the number of RLC data blocks indicated in the RLC_DATA_BLOCKS_GRANTED field. Transmission of RLC/MAC control blocks and retransmissions of RLC data blocks do not count towards the limit. When the user equipment nears the end of the close-ended TBF, it shall begin the count down procedure so that it sends the last RLC data block when CV = 0 (see sub-clause 9.3.1). The user equipment and network shall then follow the appropriate procedure for release of TBF defined in sub-clause 9.3.2.3 or sub-clause 9.3.3.3. Upon receipt of a PACKET TBF RELEASE message during a closed-end TBF, the user equipment shall follow the procedure in sub-clause 8.1.1.5.

If the number of RLC data blocks granted is not sufficient to empty the user equipment’s send buffer, the user equipment shall attempt to establish a new uplink TBF by sending a PACKET RESOURCE REQUEST, if the current uplink TBF has not yet been released, otherwise requiring the establishment of a new uplink TBF for the transmission of the outstanding LLC frames following the end of the close-ended TBF.

Whenever the network receives from the UE a valid RLC/MAC block different from an RLC/MAC dummy control block, it shall reset counter N3101. The network shall increment counter N3101 for each radio block allocated to that user equipment, for which no data or an RLC/MAC dummy control block is received. If N3101 = N3101max, the network shall stop sending any sort of block to the user equipment on the assigned downlink P-PDPCH resources and start timer T3169. When T3169 expires, the network may reuse the TFI.

#### 8.1.1.1.1	PACCH operation

The user equipment shall attempt to decode every downlink RLC/MAC block on all the assigned downlink P/S-PDCHs.

When a packet connection includes both Pilot and Shared PDCHs, uplink and downlink signalling bringing valid commands (i.e. except RLC/MAC Dummy Control blocks) shall be carried on P-PDCHs only; one RLC/MAC valid command only per radio block period is allowed on both transmit directions.

When more P-PDCHs have been allocated, the UE shall search for a downlink RLC/MAC valid command starting from the first assigned downlink P-PDCH (see Note); if not found, it shall proceed from the next one till the last assigned P-PDCH or the detection of the signalling message.

Whenever the user equipment detects an RLC/MAC control block, it shall act on the message.

The UE shall send MAC/RLC valid commands only on the first assigned uplink P-PDCH (see Note).

In case of concurrent TBFs, the transmission of RLC/MAC data or control blocks for the uplink TBF shall always take precedence over the transmission of the PACKET DOWNLINK ACK/NACK messages for the downlink TBF unless when:

-	a PACKET DOWNLINK ACK/NACK message is explicitly requested by the network via the polling mechanism (see sub-clause 8.1.2.6)

or

-	the UE has no valid data to transmit (i.e. it should send otherwise uplink DUMMY CONTROL blocks).

In case of resource reconfiguration (i.e. additional assignment or partial release of resources), the following rules shall apply:

a)	if, during the TBF, further uplink or downlink P-PDCH resources are assigned to the connection having lower number than the current lowest numbered one per corresponding direction (see Note), signalling shall continue to be transmitted on the same P-PDCH.

b)	if, during the TBF, uplink or downlink P-PDCH resources supporting signalling are released, signalling shall be transmitted on the remaining lowest numbered P-PDCH per corresponding direction (see Note).

The above implies that: in case of resource reconfiguration with global resources assignment, the P-PDCH supporting signalling must be recalculated as described in point b), while in case of resource reconfiguration with incremental resources assignment, signalling shall still be transmitted on the same P-PDCH, as described in point a).

NOTE:	”First” P-PDCH indicates that transport channel which, among those assigned respectively in uplink or in downlink direction, has the lowest number respect to the transport channel numbering rules defined in TSM 05.02 [13].

##### 8.1.1.1.2	Uplink Resource reallocation

The same procedures as reported in sub-clause 8.1.1.2.3 shall apply with the following additional constraints:

On receipt of a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with global assignment, the UE shall stop all running Pk_UE counters initialising as many Pk_UE counters as requested (according to TSM 05.08 [15]) as soon as it switches to the (re-)allocated P-PDPCH resources.

On receipt of a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with incremental assignment, the UE shall initialise as many Pk_UE counters as requested (according to TSM 05.08 [15]) as soon as it switches to the new allocated P-PDPCH resources.

The UE shall control and act upon counters Pk_UE as described in sub-clause 8.1.1.1.

At sending the PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message message with global assignment, the network shall stop all running Pk_NWK counters initialising as many Pk_NWK counters as requested (according to TSM 05.08 [15]) as soon as it switches to the (re-)allocated P-PDPCH resources.

At sending the PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with incremental assignment, the network shall initialise as many Pk_NWK counters as requested (according to TSM 05.08 [15]) as soon as it switches to the new allocated P-PDPCH resources.

The network shall control and act upon counter Pk_NWK as described in sub-clause 8.1.1.1.

#### 8.1.1.2	Dynamic allocation uplink RLC data block transfer

This sub-clause specifies user equipment and network behaviour for dynamic allocation uplink RLC data block transfer while in packet transfer mode. As reported in sub-clause 8.1, dynamic allocation applies to S-PDPCHs only.

When the user equipment receives an uplink assignment of S-PDPCHs that does not contain a TBF starting time, it shall begin monitoring the downlink assigned PDPCHs, for detection of the signalled TS-SF, and USF value if foreseen, for each assigned uplink S-PDPCH within the reaction time defined in TSM 05.10 [16]. If a TBF starting time information element is present and no uplink TBF is in progress, but a downlink TBF is in progress, the user equipment shall wait until the starting time before beginning to monitor the TS-SF and USF. While waiting for the starting time, the user equipment shall monitor the assigned downlink PDPCHs. If an uplink TBF is already in progress, the user equipment shall continue to use the assigned parameters of the uplink TBF until the TDMA frame number indicated by the TBF starting time occurs, at which time the user equipment shall begin to use the newly assigned uplink TBF parameters. If while waiting for the frame number indicated by the TBF starting time the user equipment receives another uplink assignment it shall act upon the most recently received uplink assignment and shall ignore the previous uplink assignment.

If the IMMEDIATE ASSIGNMENT or PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message contains the RLC_DATA_BLOCKS_GRANTED field, the TBF is a close-ended TBF. Otherwise the TBF is open-ended.

During a close-ended TBF the user equipment shall transmit at the most the number of RLC data blocks indicated in the RLC_DATA_BLOCKS_GRANTED field. In the case the access type in Channel Request was ‘Short Access’ (see 7.1.2), only the number of RLC data blocks requested in the Channel Request are allowed to be transmitted within the TBF, unless additional resources have been requested and assigned before the countdown procedure has started. Transmission of RLC/MAC control blocks and retransmissions of RLC data blocks do not count toward the limit. When the user equipment nears the end of the close-ended TBF, it shall begin the count down procedure so that it sends the last RLC data block when CV = 0 (see sub-clause 9.3.1). The user equipment and network shall then follow the appropriate procedure for release of TBF defined in sub-clause 9.3.2.3 or sub-clause 9.3.3.3. Upon receipt of a PACKET TBF RELEASE message during a close-ended TBF, the user equipment shall follow the procedure in sub-clause 8.1.1.5. If the number of RLC data blocks granted is not sufficient to empty the user equipment’s send buffer, the user equipment shall attempt to establish a new uplink TBF by sending a PACKET RESOURCE REQUEST, if the current uplink TBF has not yet been released, otherwise requiring the establishment of a new uplink TBF, for the transmission of the outstanding LLC frames following the end of the close-ended TBF.

Whenever the user equipment detects the TS-SF, and the eventually the assigned USF value, on an assigned downlink PDPCH, it shall transmit either a single RLC/MAC block or a sequence of four RLC/MAC blocks on the indicated uplink PDPCH(s). The time relation between an uplink block, which the user equipment shall use for transmission, and the occurrence of the TS-SF value is defined in TSM 05.02 [13]. The number of RLC/MAC blocks to transmit is controlled by the USF_GRANULARITY parameter characterising the uplink TBF.

When the user equipment transmits an RLC/MAC block to the network, it shall start timer T3180. When the user equipment detects a TS-SF value, and possibly the assigned USF value, enabling the UE for an uplink transmission on any of the assigned uplink shared resources, it shall restart timer T3180. If timer T3180 expires and no P-PDCH is active, the user equipment shall perform the abnormal release with random access procedure (see sub-clause 8.8.2). Otherwise, the UE shall continue to operate on the assigned resources; as soon as the transmission of an uplink radio block will be enabled again for any of the assigned uplink shared resources, the UE shall start timer T3180 again.

Whenever the network receives a valid RLC/MAC block from the user equipment, it shall reset counter N3101. The network shall increment counter N3101 for each radio block, allocated to that user equipment, for which no data is received. If N3101 = N3101max, the network shall stop the scheduling of RLC/MAC blocks from the user equipment and start timer T3169. When T3169 expires, the network may reuse the USF and TFI.

##### 8.1.1.2.1	PACCH operation

The user equipment shall attempt to decode every downlink RLC/MAC block on all assigned downlink PDCHs. Whenever the user equipment receives an RLC/MAC block containing an RLC/MAC control block, it shall attempt to interpret the message contained therein. If the message addresses the user equipment, the UE shall act on the message.

Whenever the user equipment detects the TS-SF and possibly the assigned USF value on any assigned downlink PDCH, then it may transmit a PACCH block in the next block period (see TSM 05.02 [13]). In case only S-PDCH(s) are assigned, the UE shall send MAC/RLC valid commands only on the first uplink S-PDCH (see Note) within those indicated by the network via the TS-SF field; otherwise, if P-PDCHs are also active, the UE shall act as indicated in sub-clause 8.1.1.1.1. The user equipment shall not transmit an RLC data block in any uplink radio block allocated via the polling mechanism (see sub-clause 10.4.4).

NOTE:	“First” S-PDCH indicates that transport channel which, among those assigned in uplink direction, has the lowest number respect to the transport channel numbering rules defined in TSM 05.02 [13].

##### 8.1.1.2.2	Neighbour cell measurements

The user equipment shall perform neighbour cell measurements during the unused PDPCHs according to its measurements capabilities.

The network shall ensure that there are sufficient gaps as to allow the necessary measurements based on the UE’s measurement capabilities.

##### 8.1.1.2.3	Uplink Resource Reallocation

The user equipment and the network are not allowed to change the RLC mode of an already established TBF during resource reallocation. Change of RLC mode shall be achieved through release of on-going TBF and establishment of a new TBF with the newly requested RLC mode.

During an uplink packet transfer, upper layers may request to transfer another LLC PDU with a different Radio Priority, a different peak throughput class or a different RLC mode than the one which is in transfer. An LLC PDU containing signalling shall be treated as having the highest Radio Priority, and the acknowledged RLC mode shall be used.

If the user equipment has not started the countdown procedure and the new LLC PDU has the same RLC mode as the current uplink TBF and either a higher radio priority or the same radio priority but a higher peak throughput class, the user equipment shall immediately request a resource reallocation for uplink according to the new Radio Priority and peak throughput class of the new LLC PDU by sending a PACKET RESOURCE REQUEST message on the PACCH and starting timer T3168. Then the user equipment shall complete the transmission of the current LLC PDU.

The PACKET RESOURCE REQUEST message shall include the UE Radio Access Capability field (see sub-clause 113.8) if sent for the first time on a non concurrent uplink TBF.

If the new LLC PDU has the same RLC mode as the current uplink TBF and either a lower Radio Priority or the same radio priority but a lower peak throughput class, the user equipment shall first complete the sending of the LLC PDU in transfer. When the sending of LLC PDUs at the higher Radio Priority or the same radio priority but higher peak throughput class stops, without waiting for the acknowledgement from the network if in RLC acknowledged mode, the user equipment shall then perform the request of a resource reallocation for uplink for any remaining LLC PDU(s) by sending a PACKET RESOURCE REQUEST message on the PACCH and start timer T3168.

If the new LLC PDU does not have the same RLC mode as the current uplink TBF but has a higher radio priority, the user equipment shall complete the transmission of the current LLC PDU using the countdown procedure including acknowledgement from the network, if in RLC acknowledged mode. The user equipment shall then release the TBF and establish a new uplink TBF for transmission of the new LLC PDU. When the sending of LLC PDUs with a higher radio priority is completed using the countdown procedure, including acknowledgement from the network if in RLC acknowledged mode, the user equipment shall try to establish an uplink TBF for the transmission of any remaining LLC PDU(s). The UE can request the establishment of a new uplink TBF, by sending a PACKET DOWNLINK ACK/NACK message with a Channel Request Description IE, if during a downlink TBF.

On receipt of the PACKET RESOURCE REQUEST or PACKET DOWNLINK ACK/NACK message with a Channel Request Description IE, the network shall respond by sending a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE or a PACKET ACCESS REJECT message to the user equipment on the PACCH from any of the assigned downlink PDPCH.

After the transmission of the PACKET RESOURCE REQUEST message with the reason for changing the priority or peak throughput class of an assigned uplink TBF the user equipment shall continue to use the currently assigned uplink TBF assuming that the requested priority or peak throughput class is already assigned to that TBF.

On receipt of a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message the user equipment shall stop timer T3168 and switch to the assigned PDPCHs.

The user equipment is then not allowed to send new PACKET RESOURCE REQUEST messages until either a new packet transfer request is received from the upper layers or when sending of LLC PDU(s) at a lower Radio Priority has to be continued.

On expiry of timer T3168 the user equipment shall retransmit the PACKET RESOURCE REQUEST message unless the PACKET RESOURCE REQUEST has already been transmitted four times in which case the user equipment shall report a TBF release to higher layers and perform an abnormal release with random access (see sub-clause 8.8.2).

If no PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message is received before the user equipment has completed its currently assigned TBFs the user equipment shall stop timer T3168.

The network may at any time during the uplink TBF initiate a change of resources by sending on the downlink PACCH monitored by the UE, an unsolicited PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message to the user equipment. During the reallocation TFI is allowed to be changed.

On receipt of a PACKET ACCESS REJECT message, the user equipment shall stop timer T3168 if running, abort the uplink TBF with an indication to the higher layers and continue to operate on the remaining TBF, if active. If no downlink TBF exists, the UE shall return to packet idle mode where the DRX mode procedures shall be applied (see sub-clause 5.5.1.5). Before initiating a new packet access procedure the user equipment shall decode the broadcast RACH Control Parameters.

If the PACKET ACCESS REJECT message contains a WAIT_INDICATION field in a Reject structure addressed to the user equipment, the UE shall

-	start timer T3172 and if the UE has additional RLC data blocks to transmit, it shall initiate a new TBF establishment procedure on the RACH, but the user equipment is not allowed to make a new attempt for packet access in the same cell until timer T3172 expires, it may, however, attempt packet access in an other cell after successful cell reselection. A user equipment in TD-PRS UE class A or B mode of operation may attempt to enter the dedicated mode in the same cell before timer T3172 has expired. During the time T3172 is running, the user equipment shall ignore all received PAGING REQUEST messages except paging request to trigger RR connection establishment.

-	  The value of the WAIT_INDICATION field (i.e. timer T3172) relates to the cell from which it was received.

###### 8.1.1.2.3.1	Abnormal cases

The following abnormal cases apply:

-	if the user equipment receives a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with global assignment which does not properly specify a downlink and/or uplink PDCH or which violates the UE multislot class, the user equipment shall perform an abnormal release with random access (see sub-clause 8.8.2).

-	if the user equipment receives a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with global assignment which allocates uplink resources for a non supported access mode, the user equipment shall perform an abnormal release with random access (see sub-clause 8.8.2).

-	if the user equipment receives a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with global assignment containing an invalid Frequency Parameters information element, the user equipment shall perform an abnormal release with system information (see sub-clause 8.8.3), performing a partial acquisition of system information messages containing frequency information.

-	if the user equipment receives a PACKET RESOURCE RECONFIGURE or a PACKET UPLINK ASSIGNMENT message with global assignment containing a Frequency Parameters information element specifying a frequency that is not supported by the user equipment then the user equipment shall perform an abnormal release with random access (see sub-clause 8.8.2).

-	If a failure in a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with global assignment is due to any reason, the UE shall perform an abnormal release with random access (see sub-clause 8.8.2).

-	If a failure in a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with incremental assignment occurs, the UE shall discard the message, considering the message as lost. The UE may then repeat its uplink resource request, according to sub-clause 8.1.1.2.3.

#### 8.1.1.3	Fixed Allocation uplink RLC data block transfer

This sub-clause specifies user equipment and network behaviour for fixed allocation uplink RLC data block transfer while in packet transfer mode. As reported in sub-clause 8.1, fixed allocation applies to S-PDPCHs only.

A fixed allocation TBF can be operated as a close-ended TBF or as an open-ended TBF. A close-ended TBF occurs when the UE sends a PACKET RESOURCE REQUEST or PACKET DOWNLINK ACK/NACK message containing an RLC_OCTET_COUNT field that contains a value less than or equal to the maximum, and different from ‘0’. An open-ended TBF occurs when the RLC_OCTET_COUNT field contains the value '0'.

A close-ended TBF transfers at the most the number of octets specified in the RLC_OCTET_COUNT field. The user equipment shall signal the number of RLC data octets plus the number of RLC data block length octets to be transferred. The network will automatically provide sufficient resources for the number of octets requested. The user equipment does not need to send further PACKET RESOURCE REQUEST messages to the network. If the user equipment sends a subsequent PACKET RESOURCE REQUEST message to the network, the TBF becomes an open-ended TBF. A closed-ended TBF may be ended by the network before the number of requested octets has been transferred. In this case the TBF ends when the network sends a FINAL_ALLOCATION indication in a fixed allocation assignment message or a PACKET ACCESS REJECT message to the user equipment.

An open-ended TBF transfers an arbitrary number of octets. The user equipment is required to send a PACKET RESOURCE REQUEST message for each fixed allocation. Each time the user equipment receives a fixed allocation, if it wishes to continue the transmission, it must then send another PACKET RESOURCE REQUEST to the network. The open-ended TBF ends when the network sends a FINAL_ALLOCATION indication in a fixed allocation assignment message or a PACKET ACCESS REJECT message to the user equipment, or when the mobile has exhausted its supply of data to be transmitted and has executed the countdown procedure.

In a one phase access, the TBF shall be operated as an open-ended TBF.

##### Transfer of RLC/MAC blocks

The IMMEDIATE ASSIGNMENT or PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message contains a Fixed Allocation struct when signalling a fixed allocation.

The Time_Slot_State_Flag shall assign from 1 to 6 time slots for packet data transmission to the fixed allocation, taking into account the multislot restrictions of the user equipment.

The bits in the ALLOCATION_BITMAP indicate which block periods are assigned to the allocation. The user equipment shall transmit an RLC/MAC block on each assigned PDCH on those timeslots indicated in the Time_Slot_State_Flag field in each block period assigned to the allocation.

The ALLOCATION_BITMAP_LENGTH field, if present, indicates the length of the ALLOCATION_BITMAP field. If not present, the ALLOCATION_BITMAP continues until the end of the message.

The network shall acknowledge packet transfers by sending PACKET UPLINK ACK/NACK messages on the monitored PACCH.

The network may allocate additional resources for the retransmissions with a PACKET UPLINK ACK/NACK or an unsolicited PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message.

When the network uses PACKET UPLINK ACK/NACK message for a new uplink resource assignment, the following constrains shall apply:

-	In case the network reconfigures the time slot occupancy, this shall be such that no new time slot will be assigned to the UE, while some of the currently allocated time slots can be released;

-	The release of a current time slot shall also mean the release of all the packet resources assigned on that time slot to the UE;

The user equipment shall not request resources or adjust its RLC_OCTET_COUNT for retransmissions requested in the PACKET UPLINK ACK/NACK (see sub-clause 8.1.1.3.2). The user equipment may retransmit erroneous blocks in any allocated uplink block.

During a close-ended TBF the network automatically sends sufficient fixed allocation resources for the user equipment to transmit the number of octets specified in the RLC_OCTET_COUNT of the initial PACKET RESOURCE REQUEST or PACKET DOWNLINK ACK/NACK message. The network signals the end of the close-ended TBF to the UE by setting the FINAL_ALLOCATION bit to '1' in the PACKET UPLINK ASSIGNMENT, the PACKET RESOURCE RECONFIGURE, or the PACKET UPLINK ACK/NACK, or by sending the PACKET ACCESS REJECT message or the PACKET TBF RELEASE message to the user equipment.

Upon receipt of an uplink assignment containing a fixed allocation and with the field FINAL_ALLOCATION set to 1, the user equipment shall execute the countdown procedure such that the countdown ends before the current allocation is exhausted.

Upon receipt of a PACKET ACCESS REJECT message, the user equipment shall release the TBF using the procedures in 9.3.2.3 or 9.3.3.3, such that the countdown ends within the current allocation. Then, if the user equipment has additional RLC data blocks to transfer, it shall initiate a new establishment procedure on the RACH, or via the PACKET DOWNLINK ACK/NACK if during a downlink TBF.

Upon receipt of a PACKET TBF RELEASE message, the user equipment shall follow the procedure in sub-clause 8.1.1.5.

During a close-ended TBF the user equipment may change the Radio Priority of the TBF or extend the TBF by sending a PACKET RESOURCE REQUEST message or a PACKET DOWNLINK ACK/NACK containing a Channel Request Description IE. The close-ended TBF then becomes an open-ended TBF and the procedures in sub-clause 8.1.1.3.2 apply.

##### 8.1.1.3.2	Uplink Resource Reallocation for open-ended TBF

The user equipment and the network are not allowed to change the RLC mode of an already established TBF during resource reallocation. Change of RLC mode shall be achieved through release of on-going TBF and establishment of a new TBF with the newly requested RLC mode.

During an uplink packet transfer, upper layers may request to transfer another LLC PDU with a different Radio Priority, a different peak throughput class or a different RLC mode than the one which is in transfer. An LLC PDU containing signalling shall be treated as having the highest Radio Priority, and the acknowledged RLC mode shall be used.

If the user equipment has not started the countdown procedure and the new LLC PDU has the same RLC mode as the current uplink TBF and either a higher radio priority or the same radio priority but a higher peak throughput class, the UE shall immediately request a resource reallocation for uplink according to the new Radio Priority and peak throughput class of the new LLC PDU by sending a PACKET RESOURCE REQUEST message on the PACCH and starting timer T3168; the PACKET RESOURCE REQUEST message shall include the UE Radio Access Capability field (see sub-clause 113.8) if sent for the first time on a non concurrent TBF. Then the UE shall complete the transmission of the current LLC PDU. If the new LLC PDU has the same RLC mode as the current uplink TBF and either a lower Radio Priority or the same radio priority but a lower peak throughput class, the user equipment shall first complete the sending of the LLC PDU in transfer. When the sending of LLC PDUs at the higher Radio Priority or the same radio priority but higher peak throughput class stops, without waiting for the acknowledgement from the network if in RLC acknowledged mode, the user equipment shall then perform the request of a resource reallocation for uplink for any remaining LLC PDU(s) by sending a PACKET RESOURCE REQUEST message on the PACCH and start timer T3168.

If the new LLC PDU does not have the same RLC mode as the current uplink TBF but has a higher radio priority, the user equipment shall complete the transmission of the current LLC PDU using the countdown procedure including acknowledgement from the network, if in RLC acknowledged mode. The user equipment shall then release the TBF and establish a new uplink TBF for transmission of the new LLC PDU. When the sending of LLC PDUs with a higher radio priority is completed using the countdown procedure, including acknowledgement from the network if in RLC acknowledged mode, the user equipment shall try to establish an uplink TBF for the transmission of any remaining LLC PDU(s). The UE can request the establishment of a new uplink TBF, by sending a PACKET DOWNLINK ACK/NACK message with a Channel Request Description IE, if during a downlink TBF.

On receipt of the PACKET RESOURCE REQUEST or PACKET DOWNLINK ACK/NACK message with a Channel Request Description IE, the network shall respond by sending a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE or a PACKET ACCESS REJECT message to the user equipment on the monitored downlink PACCH.

After the transmission of the PACKET RESOURCE REQUEST message with the reason for changing the priority or peak throughput class of an assigned uplink TBF the user equipment shall continue to use the currently assigned uplink TBF assuming that the requested priority or peak throughput class is already assigned to that TBF.

On receipt of a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message the user equipment shall stop timer T3168 and switch to the assigned PDPCHs.

The user equipment is then not allowed to send new PACKET RESOURCE REQUEST messages until either a new packet transfer request is received from the upper layers or when sending of LLC PDU(s) at a lower Radio Priority has to be continued.

On expiry of timer T3168, the user equipment shall retransmit the PACKET RESOURCE REQUEST message unless the PACKET RESOURCE REQUEST message has already been transmitted four times in which case the user equipment shall perform an abnormal release with random access.

If no PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message is received before the user equipment has completed its currently assigned TBFs the user equipment shall stop timer T3168.

The network may at any time during the uplink TBF initiate a change of resources by sending on the downlink PACCH monitored by the UE, an unsolicited PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE, or an uplink resource reassignment in a PACKET UPLINK ACK/NACK message to the user equipment. During the reallocation TFI is allowed to be changed.

When the network uses PACKET UPLINK ACK/NACK message for a new uplink resource assignment, the following constrains shall apply:

-	In case the network reconfigures the time slot occupancy, this shall be such that no new time slot will be assigned to the UE, while some of the currently allocated time slots can be released;

-	The release of a current time slot shall also mean the release of all the packet resources assigned on that time slot to the UE;

On receipt of a PACKET ACCESS REJECT message, the user equipment shall stop timer T3168 if running,  abort the uplink TBF with an indication to the higher layers and continue to operate on the downlink TBF, if active. If no downlink TBF exists, the UE shall return to packet idle mode where the DRX mode procedures shall be applied (see sub-clause 5.5.1.5). Before initiating a new packet access procedure the user equipment shall decode the broadcast RACH Control Parameters.

If the PACKET ACCESS REJECT message contains a WAIT_INDICATION field in a Reject structure addressed to the user equipment, the UE shall:

-	start timer T3172 and if the user equipment has additional RLC data blocks to transmit, it shall initiate a new TBF establishment procedure on the RACH, but it is not allowed to make a new attempt for packet access in the same cell until timer T3172 expires, it may, however, attempt packet access in an other cell after successful cell reselection. A user equipment in TD-PRS UE class A or B mode of operation may attempt to enter the dedicated mode in the same cell before timer T3172 has expired. During the time T3172 is running, the user equipment shall ignore all received PAGING REQUEST messages except paging request to trigger RR connection establishment.

-	The value of the WAIT_INDICATION field (i.e. timer T3172) relates to the cell from which it was received.

###### 8.1.1.3.2.1	At the beginning of each fixed allocation

At the beginning of each allocation of an open-ended TBF the user equipment shall either request to continue the TBF by transmitting a PACKET RESOURCE REQUEST or a PACKET DOWNLINK ACK/NACK containing a Channel Request Description IE message on the uplink PACCH, or it shall begin the countdown procedure so that it ends within the current allocation.

The user equipment shall signal the number of RLC data octets ready to transmit, plus the number of RLC data block length octets ready to transmit, in the RLC_OCTET_COUNT field of the PACKET RESOURCE REQUEST or PACKET DOWNLINK ACK/NACK message. The user equipment shall always indicate the current state of its transmit buffer at the time the message is sent. In RLC acknowledged mode, previously transmitted but currently unacknowledged octets shall not be included in the RLC_OCTET_COUNT.

###### 8.1.1.3.2.2	Upon receipt of the resource reallocation request

Upon receipt of the PACKET RESOURCE REQUEST or PACKET DOWNLINK ACK/NACK with a Channel Request Description IE, the network shall continue the TBF by sending a PACKET UPLINK ASSIGNMENT or PACKET UPLINK ACK/NACK containing a fixed allocation to the user equipment, or shall end the TBF by sending a PACKET ACCESS REJECT message. Alternatively, the network may end the TBF by sending an uplink assignment containing a fixed allocation with the FINAL_ALLOCATION bit set to 1.

Upon receipt of an uplink assignment containing an ALLOCATION_BITMAP, the user equipment shall begin transmitting on the new resources at the indicated TBF Starting Time. If there is a conflict between a previous allocation and the new allocation, the new allocation shall take precedence.

Upon receipt of a PACKET UPLINK ACK/NACK with a REPEAT_ALLOCATION, the user equipment shall start a new allocation when the current allocation ends. This new allocation shall begin immediately after the current allocation ends and shall use the most recently received ALLOCATION_BITMAP. If the user equipment receives multiple PACKET UPLINK ACK/NACK messages with REPEAT_ALLOCATION during an allocation, the UE shall repeat the ALLOCATION_BITMAP only once. If the user equipment receives a PACKET UPLINK ACK/NACK without the REPEAT_ALLOCATION indication, it shall transmit to the end of its current allocation without repeating the allocation, regardless of any previous REPEAT_ALLOCATION indications that may have been received.

The network may also specify a Time_Slot_State_Flag indication with REPEAT ALLOCATION in the PACKET UPLINK ACK/NACK. The Time_Slot_State_Flag applies to the next allocation after the current allocation expires. The Time_Slot_State_Flag field is a bitmap with a bit corresponding to each timeslot. For each bit set in the Time_Slot_State_Flag with REPEAT ALLOCATION, the UE shall disregard the ALLOCATION_BITMAP for that timeslot and shall transmit on all uplink radio blocks on each assigned reosurce for that timeslot for the duration of the next allocation. If a bit is not set in the Time_Slot_State_Flag field with REPEAT ALLOCATION, then the ALLOCATION_BITMAP shall apply to that timeslot.

###### 8.1.1.3.2.3	Upon exhaustion of the current resource allocation

If the user equipment exhausts its assigned fixed allocation and has more RLC data blocks to transmit, it shall start timer T3188 and monitor the downlink of all assigned PDPCHs. If the user equipment receives an assignment message (PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE) containing a new uplink allocation, it shall stop timer T3188 and use the new allocation at the assigned starting time.

If the user equipment receives a PACKET UPLINK ACK/NACK with a REPEAT ALLOCATION after its current allocation has been exhausted, it shall stop timer T3188, wait until the next repeated allocation boundary and then begin transmitting using the repeated ALLOCATION_BITMAP.

If timer T3188 expires and no P-PDCH is active, the user equipment shall perform an abnormal release with random access (see sub-clause 8.8.2); otherwise it shall continue to operate on the assigned resources.

###### 8.1.1.3.2.4	Ending the TBF

Upon receipt of a PACKET ACCESS REJECT message, the user equipment shall stop timer T3188, if running, release the TBF using the procedures in sub-clauses 9.3.2.3 or 9.3.3.3, such that the countdown ends within the current allocation. Then, if the user equipment has additional RLC data blocks to transfer, it shall initiate a new establishment procedure on the RACH, or using a PACKET DOWNLINK ACK/NACK if during a downlink TBF.

Upon receipt of an uplink assignment containing a fixed allocation and with the field FINAL_ALLOCATION set to 1, the UE shall execute the countdown procedure such that the countdown ends before the current allocation is exhausted.

###### 8.1.1.3.2.5	Abnormal Cases

The following abnormal cases apply:

-	if the user equipment receives a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with global assignment specifying a frequency that is not supported by the UE, it shall perform an abnormal release with random access (see sub-clause 8.8.2).

-	if the user equipment receives a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with global assignment containing invalid frequency parameters, it shall perform an abnormal release with system information (see sub-clause 8.8.3), performing a partial acquisition of system information messages containing frequency information.

-	If the information in the PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with global assignment does not properly specify a PDCH, or violates the UE's multislot capabilities, it shall perform an abnormal release with random access (see sub-clause 8.8.2).

-	if the user equipment receives a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with global assignment which allocates uplink resources for a non supported access mode, the user equipment shall perform an abnormal release with random access (see sub-clause 8.8.2).

-	If a failure in the PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with global assignment is due to any other reason, the user equipment shall perform an abnormal release with random access (see clause 8.8.2).

-	If a failure in a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with incremental assignment occurs, the UE shall discard the message, considering the message as lost. The UE may then repeat its uplink resource request, according to sub-clause 8.1.1.3.2.

##### 8.1.1.3.3	Neighbour cell measurements

The user equipment shall signal its measurement capabilities in the PACKET resource request message.

The network shall leave sufficient gaps in the uplink allocation bitmap to allow the UE to make the required neighbour cell power measurements with respect to its multi-slot capabilities and timeslot assignment. The user equipment shall make neighbour cell power measurements according to its Measurement Capabilities during the available gaps.

A UE operating in half duplex mode may be directed by the network to perform neighbour cell power measurements in predefined gaps via the Measurement Mapping parameters. The location in time and the size of the gaps are signalled by the following parameters:

-	the starting time of the first TDMA frame of the first gap;

-	a bitmap indicating the timeslots that are part of the gap; and

-	the number of RLC/MAC block periods between gaps.

If the user equipment has received the Measurement Mapping parameters, it need not decode the downlink radio blocks(s) comprising the gap during each occurrence of the gap.

If a UE not operating in the half duplex mode receives a PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message containing the measurement mapping parameters, it shall ignore the Measurement Mapping parameters.

If the assigned timeslot configuration for uplink TBF and downlink TBF simultaneously in progress prevent the UE from making these measurements, the downlink TBF assignment shall be considered invalid and the procedures of sub-clause 8.1.1.2.3.1 apply.

##### 8.1.1.3.4	PACCH operation

The user equipment shall attempt to decode every downlink RLC/MAC block on all assigned downlink PDPCHs. Whenever the user equipment receives an RLC/MAC block containing an RLC/MAC control block, it shall attempt to interpret the message contained therein. If the message addresses the user equipment, the UE shall act on the message.

After the fixed allocation is exhausted, the UE shall continue to monitor all assigned downlink PDPCH(s).

The user equipment may transmit a PACCH block on any uplink radio block allocated via the ALLOCATION_BITMAP.

In case only S-PDCH(s) are assigned, the UE shall send MAC/RLC valid commands,  within a block period and on the allowed uplink time slots, a PACCH only on the first assigned S-PDCH (see Note); otherwise, if P-PDCHs are also active, the UE shall act as indicated in sub-clause 8.1.1.1.1.

In the case of simultaneous uplink and downlink TBFs, the user equipment shall not transmit an RLC data block in any uplink radio block allocated via the polling mechanism (see sub-clause 10.4.4).

NOTE:  “First” S-PDCH indicates that transport channel which, among those assigned in uplink direction, has the lowest number respect to the transport channel numbering rules defined in TSM 05.02 [13].

##### 8.1.1.4	Establishment of a Downlink TBF

During an uplink (fixed or dynamic allocation, and/or on a P-PDPCH) TBF, the network may initiate a downlink TBF by sending the PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message on the monitored PACCH. If a PACKET RESOURCE RECONFIGURE message is sent, then the message shall contain the DOWNLINK_TFI_ASSIGNMENT field.

The downlink radio resources are assigned to the UE in a PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message. The assigned timeslot configuration of the simultaneous uplink and downlink TBF must be compliant with the UE’s multislot class, and must allow the performing of neighbour cell power measurements as described in sub-clause 8.1.2.7.

On receipt of an assignment message, or after the TBF starting time if present, the user equipment shall switch to the assigned resources and follow one of the two following procedures.

In case the assigned resources for the uplink TBF are S-PDPCHs accessed by dynamic allocation mode or if the user equipment is not assigned to operate in half duplex mode, then:

-	The UE shall start follow the procedures reported in sub-clause 8.1.2 with the following additional constraints:

the user equipment shall prioritise transmission of RLC/MAC control blocks associated with the downlink TBF over RLC/MAC control blocks associated with the uplink TBF;

if a timer or counter expiry causes the uplink TBF to be aborted in the user equipment, the UE shall also abort the downlink TBF and perform an abnormal release with random access (see sub-clause 8.8.2).

If uplink and downlink TBFs are already established, then the network may send a PACKET RESOURCE RECONFIGURE message without DOWNLINK_TFI_ASSIGNMENT. The user equipment shall interpret this as a reassignment of the resource allocations of the concurrent uplink and downlink TBFs and the downlink TFI is not changed.

Else, in case the assigned resources for the uplink TBF are P-PDPCHs and/or S-PDPCHs accessed by fixed allocation mode and the user equipment is assigned to operate in half duplex mode, then:

- the network shall wait for the user equipment to finish its current uplink resource allocation, and for the TBF starting time to elapse if present, before sending RLC data blocks on the downlink.

- If the user equipment is operating the uplink TBF in half duplex mode and receives a PACKET RESOURCE RECONFIGURE message, it shall exit the half duplex mode and act on the PACKET RESOURCE RECONFIGURE.

- Whenever a user equipment operating on an uplink TBF in half duplex mode receives an assignment on the PACCH, it shall complete the currently assigned fixed allocation. If the uplink TBF is not completed the user equipment shall, after expiry of the TBF starting time if present, or if the TBF starting time has already expired, save the RLC state variables associated with the uplink TBF and suspend and save the state of the following timers :

T3182 - Wait for Acknowledgement

T3184 - No Ack/Nack Received

T3188 - Allocation Exhausted

- Whenever a user equipment operating on an uplink TBF in half duplex mode receives a downlink assignment on the PACCH and has previously saved the state of the downlink TBF and has not since entered idle mode, the user equipment shall restore the saved downlink RLC state variables and timer values.

- The user equipment shall then act upon the PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message.

##### 8.1.1.4.1	Abnormal cases

If a failure occurs on the user equipment side before the new TBF has been successfully established, the newly reserved resources are released. The subsequent behaviour of the UE depends on the type of failure and previous actions:

-	If the information available in the user equipment, after the reception of a PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with incremental assignment does not satisfactorily define a PDCH or violates the UE multislot capability, the UE shall ignore the message.

-	If the user equipment is operating the uplink TBF in half duplex mode and receives a PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message that does not indicate half duplex mode, the UE shall ignore the message.

-	If a failure in the PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with incremental assignment is due to any other reason, then the user equipment shall ignore the message.

-	If the information in the PACKET RESOURCE RECONFIGURE with global assignment does not properly specify an uplink and/or downlink PDPCH or violates the UE’s multislot capabilities, the user equipment shall perform an abnormal release with random access (see sub-clause 8.8.2).

-	If the PACKET RESOURCE RECONFIGURE with global assignment does not include a Downlink TFI_Assignement field, then the UE shall perform an abnormal release with random access (see sub-clause 8.8.2).

-	If a failure in the PACKET RESOURCE RECONFIGURE with global assignment is due to any other reason, then the user equipment shall perform an abnormal release with random access (see sub-clause 8.8.2).

#### Network initiated release of an uplink TBF

The network may initiate release of an Uplink TBF by transmitting a PACKET TBF RELEASE message to the user equipment on the PACCH. A cause value indicates the reason for release.

If the cause value is "Normal release" the user equipment shall continue to the next LLC PDU boundary, starting the count down procedure (see sub-clause 9.3.1) at whatever value of CV is appropriate to count down to zero at the LLC PDU boundary, and then release the uplink TBF according to the procedures in sub-clause 9.3.2.3 or 9.3.3.3. If the user equipment has more LLC PDU(s) to send, the user equipment may initiate the establishment of a new uplink TBF as defined in sub-clause 7.1.

If the cause value is "Abnormal Release" the user equipment shall immediately stop transmitting and follow the abnormal release with random access procedure (see sub-clause 8.8.2).

However, if a valid RRBP field is received as part of the PACKET TBF RELEASE message, the user equipment shall transmit a PACKET CONTROL ACKNOWLEDGMENT message in the uplink radio block specified.

#### 8.1.1.6	Abnormal cases

The following abnormal cases apply:

-	if the user equipment receives a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with global assignment containing an invalid Frequency Parameters information element, it shall perform an abnormal release with system information (see sub-clause 8.8.3).

-	if the user equipment receives a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with global assignment specifying a frequency which is not supported by the UE, it shall perform an abnormal release with random access (see sub-clause 8.8.2).

-	if the user equipment receives a PACKET UPLINK ASSIGNMENT, PACKET RESOURCE RECONFIGURE, or a PACKET UPLINK ACK/NACK with an ALLOCATION_BITMAP whose TBF starting time has elapsed, it shall use whatever portion of the fixed allocation remains. If none of the fixed allocation remains, the UE shall ignore the message.

-	if the user equipment receives a PACKET UPLINK ACK/NACK with missing mandatory fields, it shall perform an abnormal release with random access.

-	if the user equipment has not started or has not completed the countdown procedure and it receives a Packet Uplink Ack/Nack with the Final Ack Indicator set, it shall perform an abnormal release with random access.

Note: A PACKET UPLINK ASSIGNMENT or a PACKET RESOURCE RECONFIGURE message sent to a multi-band UE shall not be considered invalid if it indicates a new frequency that is in a different frequency band to that of the serving cell.

### 8.1.2	Downlink RLC data block transfer

Prior to the initiation of RLC data block transfer on the downlink, the network assigns the following parameters to the downlink TBF in the IMMEDIATE ASSIGNMENT (see TSM 04.08 [11]) or PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message:

- a unique Temporary Flow Identity (TFI). The TFI applies to all radio blocks transferred in regards to the downlink Temporary Block Flow (TBF).

NOTE:	the uniqueness of the TFI can be limited to the set of used PDPCHs so that different users on different PDPCHs can be associated to the same TFI.

-	a set of PDPCHs to be used for the downlink transfer;

-	optionally, a TBF starting time indication.

For each TBF, the network shall prioritise RLC/MAC control blocks, not containing a PACKET DOWNLINK DUMMY CONTROL BLOCK message, to be transmitted ahead of RLC data blocks for that TBF. If the network has no other RLC/MAC block to transmit but wishes or need in case of P-PDPCH resources to transmit on the downlink, the network shall transmit an RLC/MAC control block containing a PACKET DOWNLINK DUMMY CONTROL BLOCK message.

Depending on the assigned resources, P-PDPCH or S-PDPCH respectively, the transfer of RLC data blocks shall be different as described in the following sub-clauses.

#### 8.1.2.1	Downlink RLC data block transfer on a P-PDPCH

If currently no P-PDCH is active between the network and the user equipment, the network shall assign to the UE a complete P-PDCH (i.e. having at least one physical resource per transmission direction; see sub-clause 4.5). The assignment shall be done by the IMMEDIATE ASSIGNMENT message in case no TBF is in progress, otherwise by either the PACKET DOWNLINK ASSIGNMENT or the PACKET RESOURCE RECONFIGURE message.

-	At the reception of an assignment for a P-PDCH, or at the TBF starting time if present, the UE shall immediately switch to the assigned resources and start with the network the P-PDPCH packet access procedure as described in sub-clause 7.1.2.1.1 with the following constraints:

-	If timer T3204 expires, the P-PDPCH packet access procedure failed; the UE shall perform abnormal release with return to CCCH (see sub-clause 8.8.1) if no uplink TBF exists, otherwise with random access (see sub-clause 8.8.2).

During the P-PDPCH packet access procedure, the UE shall continue the transmission and/or the reception on the already allocated S-PDPCHs if any (see sub-clause 8.1.2.2).

As soon as the P-PDPCH packet access procedure has been completed, the user equipment and the network shall enter the P-PDCH maintenance procedure as described in sub-clause 7.1.2.3.1.

If a P-PDCH is already active between the network and the user equipment, the network can assign to the UE additional downlink P-PDCH resources by means of the PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message. At the reception of the assignment message, or at the TBF starting time if present, the UE shall immediately switch to the newly assigned resources; and then both the UE and the network shall immediately include the new assigned resources in the P-PDCH maintenance and radio link failure management procedure (according to the rules described in sub-clause 7.1.2.3.1). No P-PDPCH packet access procedure shall therefore be initiated.

If a downlink TBF is already in progress, the UE shall continue to use the assigned parameters of the downlink TBF till the point in time it switches to the P-PDCH resources; at which time it shall immediately use the newly assigned downlink TBF parameters.

If while waiting for the frame number indicated by the TBF starting time the UE receives another assignment, it shall act upon the most recently received one ignoring the previous received assignment.

Procedures on receipt of an IMMEDIATE ASSIGNMENT message while no TBF is in progress are specified in sub-clause 7.2.1.1.

Upon receipt of a PACKET TBF RELEASE message referring to the downlink TBF, the UE shall follow the procedure in sub-clause 8.1.2.5.

##### 8.1.2.1.1	PACCH operation

The user equipment shall attempt to decode every downlink RLC/MAC block on all the assigned downlink (P/S-)PDCHs. Whenever the user equipment detects an RLC/MAC control block, it shall act on the message.

When a packet connection includes both Pilot and Shared resources, uplink and downlink signalling bringing valid commands (i.e. except RLC/MAC Dummy Control blocks) shall be carried on P-PDCHs only; one RLC/MAC valid command only per block period is allowed on both transmit directions.

When more downlink P-PDCHs have been allocated, the UE shall search for a downlink RLC/MAC valid commands starting from the first assigned downlink P-PDCH (see Note); if not found, it shall proceed from the next one till the last assigned P-PDCH or the detection of the signalling message.

Whenever the user equipment detects an RLC/MAC control block, it shall act on the message.

The UE shall send MAC/RLC valid commands only on the first assigned uplink P-PDCH (see Note).

The user equipment shall acknowledge the received downlink RLC data blocks by sending the PACKET DOWNLINK ACK/NACK message.

The PACKET DOWNLINK ACK/NACK message shall be sent, at the specific uplink radio block when explicitly requested by the network via the polling procedure (see sub-clause 8.1.2.6).

In case no other valid data is available for transmission (see sub-clause 8.1.1.1.1), the UE can also send PACKET DOWNLINK ACK/NACK messages at any uplink radio block  on the first uplink P-PDCH.

The downlink RLC data block can be received from any of the assigned downlink P-PDCHs resources or S-PDCHs.

In case of resource reconfiguration (i.e. additional assignment or partial release of resources), the following rules shall apply:

a)	if, during the TBF, further uplink or downlink P-PDCH resources are assigned to the connection having lower number than the current lowest numbered P-PDCH per corresponding direction (see Note), signalling shall continue to be transmitted on the same P-PDCH.

b)	if, during the TBF, uplink or downlink resources supporting signalling are released, signalling shall be transmitted on the remaining lowest numbered P-PDCH per corresponding direction (see Note).

The above implies that: in case of resource reconfiguration with global resources assignment, the P-PDCH supporting signalling must be recalculated as described in point b), while in case of resource reconfiguration with incremental resources assignment, signalling shall still be transmitted on the same P-PDCH.

NOTE: “First” P-PDCH indicates that transport channel which, among those assigned respectively in uplink or in downlink direction, has the lowest number respect to the transport channel numbering rules defined in TSM 05.02 [13].

##### 8.1.2.1.2	Downlink Resource reassignment

The network initiates resource reassignment by sending a PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message on the downlink PACCH. This message indicates a change in resources in the same TBF. The Control Ack bit in the message shall be set to ‘0’. During the reassignment TFI is allowed to be changed. The user equipment shall use the TFI indicated in the PACKET DOWNLINK ASSIGNMENT when using the resource indicated in the message.

The network is not allowed to change the RLC mode of an already established TBF during resource reallocation. Change of RLC mode shall be achieved through release of on-going TBF and establishment of a new TBF with the newly requested RLC mode using the procedures described in sub-clause 8.1.2.3.

On receipt of a PACKET RESOURCE RECONFIGURE message with global assignment, the UE shall stop all running Pk_UE counters initialising as many Pk_UE counters as requested (according to TSM 05.08 [15]) as soon as it switches to the (re-)allocated P-PDPCHs.

On receipt of a PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with incremental assignment, the UE shall initialise as many Pk_UE counters as requested (according to TSM 05.08 [15]) as soon as it switches to the new allocated P-PDPCH resources.

The UE shall control and act upon counter Pk_UE as described in sub-clause 8.1.2.1.

At sending the PACKET RESOURCE RECONFIGURE message with global assignment, the network shall stop all running Pk_NWK counters initialising as many Pk_NWK counters as requested (according to TSM 05.08 [15]) as soon as it switches to the (re-)allocated P-PDPCH resources.

At sending the PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with incremental assignment, the network shall initialise as many Pk_NWK counters as requested (according to TSM 05.08 [15]) as soon as it switches to the new allocated P-PDPCH resources.

The network shall control and act upon counter Pk_NWK as described in sub-clause 8.1.2.1.

#### 8.1.2.2	Downlink RLC data block transfer on a S-PDCH

This sub-clause specifies user equipment behaviour for downlink RLC data block transfer on a S-PDCH while in packet transfer mode.

Upon reception of a downlink assignment that does not contain a TBF starting time the user equipment shall switch to the assigned resources, start timer T3190 and then shall attempt to decode every downlink block on its assigned S-PDCHs. If the PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message contains a TBF starting time information element and there is no downlink TBF in progress, but an uplink TBF is in progress, the user equipment shall remain on the assigned PDCHs until the TDMA frame number indicated by the TBF starting time, at which time the user equipment shall start timer T3190 and immediately begin decoding the assigned downlink S-PDCH(s). If the PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message contains a TBF starting time and there is a downlink TBF already in progress, the user equipment shall continue to use the parameters of the downlink TBF in progress until the TDMA frame number indicated in the TBF starting time occurs, at which time the user equipment shall immediately begin to use the new assigned downlink TBF parameters. If while waiting for the frame number indicated by the TBF starting time the user equipment receives another downlink assignment, the user equipment shall act upon the most recently received downlink assignment and shall ignore the previous downlink assignment.

Procedures on receipt of an IMMEDIATE ASSIGNMENT message while no TBF is in progress are specified in sub-clause 7.2.1.1.

If the user equipment receives a valid RLC data block addressed to itself, the user equipment shall restart timer T3190. If timer T3190 expires, the user equipment shall perform an abnormal release with return to CCCH (see sub-clause 8.8.1) if no P-PDPCH is active, otherwise it shall continue to operate on the assigned resources; as soon as a valid radio block is received from any of the assigned downlink shared resources, timer T3190 shall be started again.

Upon receipt of a PACKET TBF RELEASE message referring to the downlink TBF, the user equipment shall follow the procedure in sub-clause 8.1.2.5.

##### 8.1.2.2.1	PACCH operation

The user equipment shall attempt to decode every dowlink RLC/MAC block on all the assigned downlink (P/S-)PDCHs. Whenever the user equipment detects an RLC/MAC control block addressed to itself, it shall act on the message.

When a packet connection includes also P-PDCHs, uplink and downlink signalling bringing valid commands shall only be carried on P-PDCHs as described in sub-clause 8.1.2.1.1, otherwise the UE shall send the acknowledgement to the received downlink RLC data blocks via the PACKET DOWNLINK ACK/NACK message, or any valid signalling, at the specific uplink radio block on the specified uplink resources as described in sub-clause 8.1.1.2.1, when explicitly requested by the network via the polling procedure (see sub-clause 8.1.2.6).

#### 8.1.2.2.2	Downlink Resource Reassignment

The network initiates resource reassignment by sending a PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message on the downlink PACCH. This message indicates a change in resources in the same TBF. The Control Ack bit in the message shall be set to ‘0’. During the reassignment TFI, is allowed to be changed. The user equipment shall use the TFI indicated in the PACKET DOWNLINK ASSIGNMENT when using the resources indicated in the message.

The network is not allowed to change the RLC mode of an already established TBF during resource reallocation. Change of RLC mode shall be achieved through release of on-going TBF and establishment of a new TBF with the newly requested RLC mode using the procedures described in sub-clause 8.1.2.3.

On receipt of a PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message, and after the TBF starting time if present, the UE shall switch to the assigned S-PDPCHs. Upon switching to the new S-PDPCHs the UE shall restart timer T3190.

When the user equipment receives an RLC/MAC block addressed to itself on any of the new downlink assigned resources it shall restart timer T3190. If timer T3190 expires, the UE shall perform an abnormal release with return to CCCH (see sub-clause 8.8.1) if no P-PDPCH is active, otherwise it shall continue to operate on the assigned resources; as soon as a valid radio block is received from any of the assigned downlink shared resources, timer T3190 shall be started again.

#### 8.1.2.3	Void

#### 8.1.2.4	Establishment of uplink TBF

The user equipment may request establishment of an uplink transfer during a downlink TBF by including a Channel Request Description information element in the PACKET DOWNLINK ACK/NACK message or by sending a PACKET RESOURCE REQUEST message if sent on P-PDCH. Initiation is triggered by a request from upper layers to transfer a LLC PDU. The request from upper layers specifies a Radio Priority to be associated with the packet transfer. Upon such a request,

-	if access to the network is allowed, according to the latest values for authorised special access classes that the UE has received (see sub-clause 7.1.1), the user equipment shall initiate the packet access procedure.

-	otherwise, the RR sub-layer in the UE shall reject the request.

The user equipment initiates the packet access procedure by sending the Channel Request Description information element in the PACKET DOWNLINK ACK/NACK message or in the PACKET RESOURCE REQUEST message on the PACCH and starting timer T3168.

On receipt of a Channel Request Description information element, the network may assign radio resources to the user equipment on one or more PDPCHs by transmitting a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message on the PACCH, or may reject the request by sending a PACKET ACCESS REJECT message on the PACCH. If the PACKET RESOURCE RECONFIGURE message is sent, then the message shall contain the UPLINK_TFI_ASSIGNMENT field.

On receipt of a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message, the UE shall follow the procedure below. On reception of an uplink assignment the user equipment shall stop timer T3168.

If the UE is not assigned to operate in half duplex mode, the UE shall, after expiry of the TBF starting time if present, act upon the uplink assignment.

If the UE is assigned to operate in half duplex mode, it shall, after expiry of the TBF starting time if present, stop the downlink TBF, save the RLC state variables associated with the downlink TBF and save the state of the following timers:

T3190 - Wait for Valid Downlink Data Received from the Network

T3192 - Wait for Release of the TBF after reception of the final block

Whenever a UE operating on a downlink TBF in half duplex mode receives an uplink assignment on the PACCH and has previously saved the state of the uplink TBF and not since entered idle mode, it shall restore the saved uplink RLC state variables and timer values.

If the UE is operating the downlink TBF in half duplex mode and receives a PACKET RESOURCE RECONFIGURE message, it shall exit half duplex mode and act on the PACKET RESOURCE RECONFIGURE.

The UE shall then switch to the assigned uplink PDPCHs and begin to send RLC data blocks on the assigned PDPCH(s). The TLLI shall not be included in any of the uplink RLC data blocks in that case.

On receipt of a PACKET ACCESS REJECT message that contains a Reject structure addressed to the mobile station, the mobile station shall stop timer T3168 and indicate a packet access failure to upper layer.

If the PACKET ACCESS REJECT message contains a WAIT_INDICATION field in a Reject structure addressed to the UE, the user equipment shall start timer T3172 with the indicated value (Wait Indication). The user equipment is not allowed to make a new attempt for packet access in the same cell until timer T3172 expires, but may attempt packet access in an other cell after successful cell reselection. When timer T3172 expires, if the downlink TBF is still active the user equipment shall initiate the establishment of an uplink TBF using the procedure in this sub-clause. If no TBF is active, the user equipment shall initiate the establishment of an uplink TBF on CCCH.

If timer T3168 expires, the user equipment shall retransmit the Channel Request Description information element in the next PACKET DOWNLINK ACK/NACK or PACKET RESOURCE REQUEST message, unless it has been transmitted four times in which case it shall perform an abnormal release with random access. If the downlink TBF is released, including expiry of timer T3192, before expiry of timer T3168 the user equipment shall stop timer T3168 and perform an abnormal release with random access (see sub-clause 8.8.1).

##### 8.1.2.4.1	Abnormal cases

If a failure occurs on the UE side before the new TBF has been successfully established, the newly reserved resources are released. The subsequent behaviour of the UE depends on the type of failure and previous actions.

-	If the information in the PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with global assignment does not properly specify a PDCH or violates the user equipment’s multislot capabilities, the user equipment shall perform an abnormal release with random access (see sub-clause 8.8.2).

-	If the UE is operating the downlink TBF in half duplex mode and receives a PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message that does not indicate half duplex mode, it shall ignore the message, considering it as lost. It may repeat its uplink resource request as described in sub-clause 8.1.2.4.

-	If the information in the PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with incremental assignment does not properly specify a PDCH or violates the user equipment’s multislot capabilities, the user equipment shall ignore the message, considering it as lost. It may repeat its uplink resource request as described in sub-clause 8.1.2.4.

-	If the PACKET UPLINK ASSIGNMENT does not include a correct UPLINK_TFI_ASSIGNMENT field, then the user equipment shall perform an abnormal release with random access (see sub-clause 8.8.2).

-	If a failure in the PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE with global assignment is due to any other reason, the user equipment shall perform an abnormal release with random access (see sub-clause 8.8.2).

-	If a failure in the PACKET UPLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE with incremental assignment is due to any other reason, the user equipment shall ignore the message, considering it as lost. It may repeat its uplink resource request as described in sub-clause 8.1.2.4.

#### Network initiated abnormal release of a downlink TBF

The network may initiate immediate abnormal release of a downlink TBF by transmitting a PACKET TBF RELEASE message to the UE on the PACCH.

The UE shall immediately stop monitoring its assigned downlink PDPCHs. If a valid RRBP field is received as part of the PACKET TBF RELEASE message, the UE shall transmit a PACKET CONTROL ACKNOWLEDGMENT message in the specified uplink radio block.

If there is no on-going uplink TBF, the UE then shall enter packet idle mode. Upon entering packet idle mode, the UE shall apply DRX mode procedures as specified in sub-clause 5.5.1.5.

#### 8.1.2.6	Polling for Packet Downlink Ack/Nack

Whenever the user equipment receives an RLC data block addressed to itself and with a valid RRBP field in the RLC data block header (i.e., it is polled), it shall transmit a Packet Downlink ACK/NACK message in the uplink radio block, specified by the RRBP field on an uplink PDPCH whatever the BSN value of the received RLC data block, unless another RLC/MAC control message is waiting to be transmitted, in which case the other RLC/MAC control message shall be sent at first. However, the user equipment shall transmit an RLC/MAC control message other than a PACKET DOWNLINK ACK/NACK message at most every second time it is polled. Furthermore the user equipment shall not transmit an RLC/MAC control message other than a PACKET DOWNLINK ACK/NACK message if the PACKET DOWNLINK ACK/NACK message would contain a Final Ack Indicator or Channel Request Description IE. The UE shall not send a PACKET CONTROL ACKNOWLEDGEMENT message unless otherwise specified.

Whenever the network receives a valid RLC/MAC control message from the UE, it shall reset counter N3105. The network shall increment counter N3105 for each radio block, allocated to that UE with the RRBP field, for which no RLC/MAC control message is received. If N3105 = N3105max, the network shall release the downlink TBF internally and start timer T3195. When T3195 expires, the network may reuse the TFI.

The user equipment shall make SIR_Level measurements on each assigned downlink time slot using the procedures in TSM 05.08 [15]. The user equipment shall include the SIR_Level measurement results in at least every other PACKET DOWNLINK ACK/NACK message.

The network shall poll the user equipment by respecting the resources allocated to the UE.

In the case of simultaneous uplink and downlink TBFs, the transmission of the polling response takes precedence over the transmission of allocated uplink radio blocks.

#### 8.1.2.7 Abnormal cases

If a failure occurs at the UE side before the new downlink TBF has been successfully established, the newly reserved resources are released. The subsequent behaviour of the UE depends on the type of failure and previous actions:

-	If the UE receives a PACKET RESOURCE RECONFIGURE with global assignment message and detects an invalid Frequency Parameters information element in the message, it shall perform an abnormal release with system information (see sub-clause 8.8.3), performing a partial acquisition of system information messages containing frequency information.

-	If the information in the PACKET RESOURCE RECONFIGURE with global assignment does not properly specify an uplink and/or downlink PDCH or violates the UE's multislot capabilities, the UE shall perform an abnormal release with random access (see sub-clause 8.8.2), if an uplink TBF is in progress, otherwise with return to CCCH (see sub-clause 8.8.1).

-	If the information in the PACKET RESOURCE RECONFIGURE with incremental assignment does not properly specify an uplink and/or downlink PDCH or violates the UE's multislot capabilities, the UE shall ignore the message.

-	If the PACKET DOWNLINK ASSIGNMENT does not include a DOWNLINK_TFI_ASSIGNMENT field, then the UE shall perform an abnormal release with random access (see sub-clause 8.8.2), if an uplink TBF is in progress, otherwise with return to CCCH (see sub-clause 8.8.1).

-	If a failure in the PACKET RESOURCE RECONFIGURE is due to any other reason, the UE shall abort the procedure and perform an abnormal release with random access (see sub-clause 8.8.2), if an uplink TBF is in progress, otherwise with return to CCCH (see sub-clause 8.8.1).

-	If the information available at the UE, after the reception of a PACKET DOWNLINK ASSIGNMENT message does not satisfactorily define a PDCH or violates the UE’ multislot capability, the UE shall ignore the PACKET DOWNLINK ASSIGNMENT message.

-	If the UE is operating an uplink TBF in half duplex mode and it receives a PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message that does not indicate half duplex mode, the UE shall ignore the message.

-	If a failure in the PACKET DOWNLINK ASSIGNMENT is due to any other reason, the UE shall abort the procedure. If an uplink TBF exists, the UE shall continue the normal operation of the uplink TBF. If an uplink TBF does not exist, the UE shall perform an abnormal release with return to CCCH (see sub-clause 8.8.1).

### 8.1.3	Concurrent TBF procedures for half duplex operation

A UE not operating in half duplex mode shall follow the procedures of sub-clauses 8.1.1.4 and 8.1.2.4. If uplink and downlink TBFs are already established, the network may send a PACKET RESOURCE RECONFIGURE message in order to change the uplink and downlink resource allocation of the on-going TBFs. In the message the network may assign a new downlink and/or uplink TFI to be used for the TBFs.

If the assignment message indicates half duplex mode operation, the procedures defined in this sub-clause shall be followed.

Procedures are defined to:

-	allow the network and the UE to save the state of one TBF to allow data transfer in the other TBF; and

-	allow a TBF whose state has been saved to be restored at release of the active TBF.

#### 8.1.3.1	Saving downlink TBF state and initiating uplink TBF

A UE operating using the fixed allocation medium access mode may initiate an uplink TBF during a downlink TBF using the procedure defined in sub-clause 8.1.2.4.

#### 8.1.3.2	Saving downlink TBF state and restoring uplink TBF state

During a downlink TBF the UE may indicate that it wishes to transfer RLC data on the uplink TBF by initiating the procedure defined in sub-clause 8.1.2.4.

#### 8.1.3.3	Ending downlink TBF and restoring uplink TBF state

If the network sends an RLC data block with the FBI field set to indicate the last RLC data block of the TBF and an associated uplink TBF for the UE exists, the network shall also transmit a PACKET UPLINK ASSIGNMENT message on the downlink PACCH to the UE.

If a UE receives an RLC data block with the FBI set to indicate the last RLC data block of the TBF and an associated uplink TBF for the UE exists, it shall follow the downlink TBF release procedures defined in sub-clause 9.3.2.5 or sub-clause 9.3.3.5. If the UE receives an uplink assignment during the release procedure and a conflict exists between the downlink and uplink allocations, it shall first complete the downlink TBF procedures and then act upon the uplink assignment.

#### 8.1.3.4	Saving uplink TBF state and initiating downlink TBF

The network may initiate a downlink TBF during an uplink TBF to a UE operating using the fixed allocation medium access mode by using the procedure defined in sub-clause 8.1.1.4.

#### 8.1.3.5	Saving uplink TBF state and restoring downlink TBF state

The UE sending RLC data on an uplink TBF and with an active but saved downlink TBF may be commanded by the network to save the state of the uplink TBF and restore the state of the downlink TBF and then operate the downlink TBF. Upon receipt of a PACKET DOWNLINK ASSIGNMENT message, the UE shall follow the procedure in sub-clause 8.1.1.4.

#### 8.1.3.6	Ending uplink TBF and restoring downlink TBF state

A UE operating in the fixed allocation medium access mode shall, if a downlink TBF exists, release its uplink TBF by following the procedures in sub-clause 9.3.2.3 or sub-clause 9.3.3.3 and immediately begin to switch to the PDPCH(s) allocated to the downlink TBF.

## 8.2	Release of packet radio resources

The network may release a sub-set of the assigned packet resources or all the assigned packet resources to a packet call by sending to the user equipment the PACKET RESOURCE RELEASE message.

At the detection of a PACKET RESOURCE RELEASE message containing the UNAVAILABLE_RU field, the UE shall reply with a PACKET CONTROL ACKNOWLEDGMENT message and then stop transmitting and/or receiving on/from the indicated resource units and remove such resources from the list of assigned PDPCHs.

At the detection of a PACKET RESOURCE RELEASE message not containing the UNAVAILABLE_RU field, the UE shall reply with a PACKET CONTROL ACKNOWLEDGMENT message and then stop transmitting and/or receiving on/from all the currently assigned packet resource units.

If all the UE’s assigned uplink PDPCHs are requested to be removed from its list of assigned packet resources and an uplink TBF was still in progress, the user equipment shall perform an abnormal release with random access (see sub-clause 8.8.2). If no uplink TBF was in progress, the user equipment shall perform an abnormal release with return to CCCH (see sub-clause 8.8.1).

If the request to remove packet resources leaves a P-PDPCH unpaired (i.e. no more operating in duplex mode, see sub-clause 4.5.1), the UE shall perform an abnormal release with random access (see sub-clause 8.8.2), if an uplink TBF was still in progress; otherwise it shall perform an abnormal release with return to CCCH (see sub-clause 8.8.1).

In case the network does not receive the PACKET CONTROL ACKNOWLEDGMENT message in the indicated RRBP uplink radio block, can repeat the PACKET RESOURCE RELEASE message up to 4 times after that the network shall release all TBFs towards that user equipment.

## 8.3	Procedure for measurement report sending in Packet Transfer mode

The procedure for NC or EM measurement report sending shall be initiated by the user equipment at expiry of the NC/EM measurement report interval timer, T3158 or T3178 respectively (see sub-clauses 5.6.1 and 5.6.2). At expiry of the timer T3158/T3178 the UE shall restart the timer T3158/T3178, perform the measurements and send the PACKET MEASUREMENT REPORT message containing the ‘NC/EM measurement report struct’ on PACCH.

Following a downlink TBF establishment, the PACKET MEASUREMENT REPORT message shall not be sent on the uplink PACCH associated with this TBF until two PACKET DOWNLINK ACK/NACK messages has been sent to the network.

## 8.4	Cell Change procedures in Packet Transfer mode

A cell reselection is made controlled either by the UE or by the network.

When a cell reselection is made controlled by the user equipment, the UE shall apply the cell reselection procedure defined in sub-clause 5.5.1.1.

When a cell reselection is initiated by the network for an individual UE, the cell change order procedure is started by sending a PACKET CELL CHANGE ORDER message to the user equipment on the PACCH.

The PACKET CELL CHANGE ORDER message contains:

-	The characteristics of the new cell that are necessary to identify it (respectively: (Code Group + Basic Midamble Code + BCCH frequency) or (BSIC + BCCH frequency) for a TD-SCDMA or GSM cell);

-	The NC measurement parameters valid for the UE in the new cell (NETWORK_CONTROL_ORDER and optionally: NC_NON_DRX_PERIOD, NC_REPORTING_PERIOD_I and NC_REPORTING_PERIOD_T).

-	The IMMEDIATE_REL parameter in order to indicate whether the user equipment shall immediately move to the target cell and interrupt its on-going TBF in the serving cell, or it shall move to the target cell by applying the same procedure as for an autonomous cell reselection.

Upon receipt of the PACKET CELL CHANGE ORDER message, the user equipment shall start timer T3174 and act upon the IMMEDIATE_REL value which has been received in the Packet Cell Change Order such that:

-	if required, it shall immediately abort any operation in the old cell, except for the acknowledgement (by a PACKET CONTROL ACKNOWELEDGEMENT message) in case of valid RRBP field possibly included in the PACKET CELL CHANGE ORDER message, stopping all RLC/MAC timers but the ones related to measurement reporting;

-	otherwise it may continue its operation in the old serving cell, as in UE steered cell reselection.

The user equipment shall then switch to the specified new cell and shall obey the relevant RLC/MAC procedures on this new cell.

If the timers related to measurement reporting expire while the cell reselection procedure has not yet been completed, these timers shall be restarted so that the UE resumes the measurement reporting procedures once camped on the new cell.

The user equipment shall obey the PACKET CELL CHANGE ORDER irrespective of whether or not the user equipment has any knowledge of the relative synchronisation of the target cell to the serving cell.

### 8.4.1	Network controlled cell reselection completion

The user equipment regards the procedure as completed when it has received a successful response to its CHANNEL REQUEST message on the new cell. It shall then stop timers T3174.

If timer T3174 expires before a response to the CHANNEL REQUEST message has been received on the new cell, or, if an IMMEDIATE ASSIGNMENT REJECT message is received from the new cell, or, if the contention resolution procedure fails on the new cell (see sub-clauses 7.1.2.4 and 7.1.3.4), then the user equipment shall start timer T3176 and return to the old cell. If the UE was in packet idle mode or in downlink packet transfer mode before the cell change, it shall initiate a random access on the old cell, with access type "single block without TBF establishment", and then transmit the PACKET CELL CHANGE FAILURE message on the single block. If the UE was in uplink packet transfer mode or in a simultaneous uplink and downlink packet transfer mode before the cell change, it shall establish with the old cell a new uplink TBF and send the PACKET CELL CHANGE FAILURE message on this TBF. The UE shall then resume its uplink transfer on this TBF. When the UE has sent a PACKET CELL CHANGE FAILURE message, timer T3176 shall be stopped. If T3176 expires and it was previous in an uplink packet transfer mode or in a simultaneous uplink and downlink packet transfer mode on the old cell, it shall perform the abnormal release with random access. If the UE was previous in a downlink packet transfer mode only on the old cell it shall perform an abnormal release with return to CCCH.

### 8.4.2	Abnormal cases

On the user equipment side, if the PACKET CELL CHANGE ORDER message instructs the UE to use a frequency that it is not capable of using, then the UE shall return a PACKET CELL CHANGE FAILURE message with cause "frequency not implemented".

If the PACKET CELL CHANGE ORDER message is received by the UE while a circuit switched connection is on-going, then the UE shall return a PACKET CELL CHANGE FAILURE message with the cause "on-going CS connection".

If the PACKET CELL CHANGE ORDER message is received by a user equipment performing an anonymous access, the UE shall return a PACKET CELL CHANGE FAILURE message with the cause "anonymous access".

If the PACKET CELL CHANGE ORDER message is received while the user equipment is in GMM Standby state, the UE shall return a PACKET CELL CHANGE FAILURE,

- if the GMM Ready timer has a negotiated value equal to zero, with the cause set to "Forced to the Standby state",

- if the GMM Ready timer has a negotiated value not equal to zero, with the cause set to "GMM Standby state".

The message PACKET CELL CHANGE FAILURE is sent on the PACCH if an uplink TBF exist.

If an uplink TBF does not exist, the user equipment shall initiate a random access, with access type "single block without TBF establishment", and then transmit the PACKET CELL CHANGE FAILURE message on the single block. If a TBF exist, the UE shall remain on the current PDPCH(s).

On the network side, lower layer failures occurring on the old channels after the sending of the PACKET CELL CHANGE ORDER message are ignored.

## 8.5	Measurement Order procedures in Packet Transfer mode

The network may initiate the measurement order procedure by sending a PACKET MEASUREMENT ORDER message on the PACCH to a UE in packet transfer mode.

The PACKET MEASUREMENT ORDER message may also contain the following optional parameters:

-	NC Measurement Parameters (NETWORK_CONTROL_ORDER; NC_NON_DRX_PERIOD; NC_REPORTING_PERIOD_I; NC_REPORTING_PERIOD_T; NC_FREQUENCY_LIST);

-	EXT Measurement Parameters (EXTENDED_MEASUREMENT_COMMAND_CHANGE_MARK; EXT_MEASUREMENT_ORDER; EXT_REPORTING_PERIOD_T; EXT_REPORTING_PERIOD_I; Intra-Cell Extended Measurements; Inter-Cell Extended Measurements).

Upon receipt of the PACKET MEASUREMENT ORDER message, the UE shall store the received parameters and obey the NETWORK_CONTROL_ORDER and the EXT_MEASUREMENT_ORDER as specified in TSM 05.08 [15] and in sub-clause 5.6.

## 8.6	PACKET CONTROL ACKNOWLEDGEMENT

A PACKET CONTROL ACKNOWLEDGEMENT message shall always be sent in the uplink block, specified by the corresponding valid RRBP field of a downlink RLC/MAC control block, and not in any other uplink block that may be allocated to the user equipment.

However the transmission of the PACKET CONTROL ACKNOWLEDGEMENT takes precedence over the transmission of allocated uplink radio blocks.

## 8.7	Inherited PDPCHs management

During an ongoing TBF, uplink or downlink or concurrent, the network can reconfigure the resources assigned to the UE via one of the following messages: PACKET UPLINK ASSIGNMENT, PACKET DOWNLINK ASSIGNMENT, PACKET RESOURCE RECONFIGURE, PACKET RESOURCE RELEASE.

-	At the reception of a PACKET UPLINK ASSIGNMENT message or a PACKET RESOURCE RECONFIGURE message with global resource assignment, the UE shall immediately, or at the TBF Starting Time indication if present, consider the previous assigned resources as released, stopping possibly related procedures and timers. If the assignment message includes resources already in use, the UE shall first release them, according with the above described rule, and then shall act on them following the signalled parameters.

-	At the reception of a PACKET UPLINK ASSIGNMENT message or a PACKET RESOURCE RECONFIGURE message with incremental resource assignment or a PACKET DOWNLINK ASSIGNMENT message, the UE shall immediately, or at the TBF Starting Time indication if present, consider the resources currently in use as reconfirmed. Possibly pending procedures and related timers shall continue to run (e.g. T3192 or TBF Ending Time occurrence).

In both cases, the network and the UE shall act upon the new assignment according to the signalled parameters, following the procedures described in sub-clauses 8.1.1 or 8.1.2, or those in sub-clauses 9.3.2.3 and 9.3.2.5 for acknowledge mode or in sub-clauses 9.3.3.3 and 9.3.3.5 for unacknowledge mode of operation.

At the reception of a PACKET RESOURCE RELEASE message the UE shall release, immediately or at the RR Ending Time indication if present, all indicated resources, following the procedure described in section 8.2. Possibly pending procedures and related timers shall continue to run (e.g. RR Ending Time occurrence or TBF Ending Time occurrence) for the resources not released. If the UE receives further PACKET RESOURCE RELEASE messages indicating packet resource units already signalled to be released but still active, it shall act upon them following the signalled parameters.

## 8.8	Abnormal cases

The following abnormal cases apply:

-	If a user equipment receives a PACKET DOWNLINK ASSIGNMENT message assigning to a S-PDCH a different MAC mode than the MAC mode of an already operating uplink TBF on S-PDCHs, the PACKET DOWNLINK ASSIGNMENT message shall be ignored.

-	If a user equipment receives a PACKET UPLINK ASSIGNMENT message assigning for a S-PDCH a different MAC mode than the MAC mode of an already operating downlink TBF on S-PDCHs, the PACKET UPLINK ASSIGNMENT message shall be ignored.

-	If the user equipment receives a PACKET RESOURCE RECONFIGURE or a PACKET UPLINK ASSIGNMENT message indicating an incremental resource description without any resource description, it shall interpret the message as a reconfirm of the currently assigned resources.

-	If the user equipment receives a PACKET RESOURCE RECONFIGURE or a PACKET UPLINK ASSIGNMENT message indicating an incremental resource description and describing resources already assigned, it shall interpret the message as a reconfirm for the already assigned resources and as an assignment for the new defined ones.

-	If the user equipment receives a PACKET RESOURCE RECONFIGURE or a PACKET UPLINK ASSIGNMENT message indicating a global resource description without any resource description, it shall perform an abnormal release with random access (see sub-clause 8.8.2), if an uplink TBF is running, otherwise it shall perform an abnormal release with return to CCCH (see sub-clause 8.8.1).

-	If the user equipment receives a PACKET RESOURCE RECONFIGURE or a PACKET UPLINK ASSIGNMENT message indicating an incremental resource description with a frequency assignment, it shall perform an abnormal release with random access, if an uplink TBF is running, otherwise it shall perform an abnormal release with return to CCCH.

-	If the user equipment receives a PACKET POWER CONTROL/TIMING CONTROL or a PACKET UPLINK ACK/NACK message containing a timing and/or a power control information for a not assigned time slot, it shall ignore this information and process the rest of the message.

-	If the user equipment receives a PACKET RESOURCE RELEASE, a PACKET RESOURCE RECONFIGURE or a PACKET UPLINK ASSIGNMENT message releasing P-PDPCH resources so that a unidirectional P-PDCH remains, the UE shall perform an abnormal release with random access, if an uplink TBF is running, otherwise it shall perform an abnormal release with return to CCCH.

-	If the user equipment receives a PACKET MEASUREMENT ORDER or a PACKET CELL CHANGE ORDER message with the parameter NC_MEASUREMENT_ORDER set to either NC1 or NC2 value not followed by at least one of the two fields (NC_NON_DRX_PERIOD; NC_REPORTING_PERIOD_I) and NC_REPORTING_PERIOD_T, it shall ignore the all message.

-	If the user equipment receives a PACKET SYSTEM INFORMATION TYPE 13 message with the parameter NC_MEASUREMENT_ORDER set to either NC1 or NC2 value not followed by at least one of the two fields (NC_NON_DRX_PERIOD; NC_REPORTING_PERIOD_I) and NC_REPORTING_PERIOD_T, it shall ignore the all message.

-	If the Measurement Parameters (NC and/or EXT) are sent in more than one instance of the PACKET MEASUREMENT ORDER message, the UE shall not obey the measurement order until all instances of the message has been correctly received.

### 8.8.1	Abnormal release with return to CCCH

The user equipment shall abort all TBFs in progress and return to packet idle mode. Upon entering packet idle mode, the UE shall apply DRX mode procedures as specified in sub-clause 5.5.1.5.

### 8.8.2	Abnormal release with random access

The user equipment shall abort all TBFs in progress and its associated resources, return to the CCCH and initiate establishment of a new uplink TBF as defined in sub-clause 7.1.

### 8.8.3	Abnormal release with system information

The user equipment shall abort the TBF and its associated resources, immediately return to the BCCH and reread all relevant BCCH information. If the user equipment was performing an uplink TBF when the abnormal release occurred, it shall then perform an abnormal release with random access (see sub-clause 8.8.2). Otherwise it shall perform an abnormal release with return to CCCH (see sub-clause 8.8.1).

# 9	Radio Link Control (RLC) procedures in packet transfer mode

The RLC function is responsible for:

-	Interface primitives allowing the transfer of Logical Link Control layer PDUs (LLC PDU) between the LLC layer and the MAC function.

-	Segmentation of LLC PDUs into RLC data blocks and re-assembly of RLC data blocks into LLC PDU.

-	Segmentation of RLC/MAC control messages into RLC/MAC control blocks and re-assembly of RLC/MAC control messages from RLC/MAC control blocks.

-	Backward Error Correction (BEC) procedures enabling the selective retransmission of RLC data blocks.

In this clause Packet Ack/Nack refers to any of the following messages:

-	PACKET DOWNLINK ACK/NACK

-	PACKET UPLINK ACK/NACK

## 9.1	Procedures and parameters for peer-to-peer operation

A TBF is comprised of two peer entities RLC endpoints. Each RLC endpoint has a receiver that receives RLC/MAC blocks. Each RLC endpoint has also a transmitter that transmits RLC/MAC blocks.

Each endpoint’s receiver has a receive window of size WS (see sub-clause 9.1.9). In RLC acknowledged mode, the receive window is defined by the receive state variable V(Q) in the following inequality [ V(Q)  BSN < V(Q)+WS ] modulo 2*WS. All BSNs which meet that criteria are valid within the receive window. In RLC unacknowledged mode, all values of BSN are within the receive window.

Each endpoint’s transmitter has a transmit window of size WS. In RLC acknowledged mode, the transmit window is defined by the send state variable V(S) in the following inequality: [ V(A)  BSN < V(S) ] modulo 2*WS, where [ V(S) - V(A) ] modulo 2*WS  WS. All BSNs which meet that criteria are valid within the transmit window. In RLC unacknowledged mode, all values of BSN are within the transmit window.

### 9.1.1	Send state variable V(S)

Each RLC endpoint transmitter shall have an associated send state variable V(S). V(S) denotes the sequence number of the next in-sequence RLC data block to be transmitted. V(S) can take on the value 0 through (2*WS)-1. V(S) shall be set to the value 0 at the beginning of each TBF in which the RLC endpoint is the transmitter. The value of V(S) shall be incremented by 1 after transmission of the RLC data block with BSN = V(S). In RLC acknowledged mode, V(S) shall not exceed V(A) modulo 2*WS by more than the maximum allowed number of outstanding RLC data blocks WS.

### 9.1.2	Control send state variable V(CS)

The network RLC endpoint transmitter shall have one instance of an associated control send state variable V(CS) for each parallel control transaction identified by the RTI field of the RLC/MAC control block header. V(CS) denotes the sequence number of the next in-sequence RLC/MAC control block to be transmitted for the control transaction. V(CS) can take on the values 0 or 1. V(CS) shall be set to the value 0 prior to the transmission of each RLC/MAC control block that contains the first octet of an RLC/MAC control message of the control transaction and the value of V(CS) shall be set to 1 after the transmission of the RLC/MAC control block with RBSN = 0.

### 9.1.3	Acknowledge state variable V(A)

In RLC acknowledged mode, each RLC endpoint transmitter shall have an associated acknowledge state variable V(A). V(A) contains the BSN value of the oldest RLC data block that has not been positively acknowledged by its peer. V(A) can take on the values 0 through (2*WS)-1. V(A) shall be set to the value 0 at the beginning of each TBF in which the RLC endpoint is the transmitter. The value of V(A) shall be updated from the values received from its peer in the received block bitmap (RBB) of the Packet Ack/Nack message (see sub-clause 9.1.10).

Furthermore, [ V(S) - V(A) ] modulo 2*WS  WS.

### 9.1.4	Acknowledge state array V(B)

In RLC acknowledged mode, each RLC endpoint transmitter shall have an associated acknowledge state array (V(B)). V(B) is an array of 2*WS elements indicating the acknowledgement status of WS previous RLC data blocks. The array is indexed relative to the acknowledge state variable V(A) modulo 2*WS or relative to the reference sequence number (RSN). The values of V(B) shall be updated from the values received from its peer in the received block bitmap (RBB) of the Packet Ack/Nack message (see sub-clause 9.1.10)

The transmitter shall transmit the oldest RLC data block whose corresponding element in V(B) indexed relative to V(A) has the value NACKED. As each RLC data block is transmitted, the corresponding element in V(B) is set to the value PENDING_ACK.

If [ V(S) < V(A) + WS ] modulo 2*WS and no RLC data blocks have a corresponding element in V(B) with the value NACKED, the RLC data block with BSN = V(S) shall be transmitted and the corresponding element in V(B) shall be set to the value PENDING_ACK. If there are no further RLC data blocks available for transmission (i.e. the RLC data block with BSN= V(S) does not exist), the sending side shall transmit the oldest RLC data block whose corresponding element in V(B) has the value PENDING_ACK, then the next oldest block whose corresponding element in V(B) has the value PENDING_ACK, etc. If all RLC data blocks whose corresponding element in V(B) has the value PENDING_ACK have been transmitted once, the process shall be repeated beginning with the oldest RLC data block.

If V(S) = V(A) + WS modulo 2*WS (i.e., the transmit window is stalled), the sending side shall transmit the oldest RLC data block whose corresponding element in V(B) has the value PENDING_ACK, then the next oldest RLC data block whose corresponding element in V(B) has the value PENDING_ACK, etc. If all RLC data blocks whose corresponding element in V(B) has the value PENDING_ACK has been transmitted once, the process shall be repeated beginning with the oldest RLC data block. This process of transmitting the oldest RLC data blocks whose value in V(B) has the value PENDING_ACK shall continue indefinitely.

When an element in V(B) falls outside of the active transmit window, i.e., [ V(A)  BSN < V(S) ] modulo 2*WS, the element shall be set to the value INVALID.

If the user equipment is the transmitter, it shall start an instance of timer T3198 for each RLC data block sent. The timer T3198 shall have the expiry value set to BS_CV_MAX block periods.

### 9.1.5	Block sequence number BSN

Each RLC data block contains a block sequence number (BSN) field that is 9 bits in length. At the time that an in-sequence RLC data block is designated for transmission, the value of BSN is set equal to the value of the send state variable V(S).

### 9.1.6	Reduced Block Sequence Number RBSN

Each downlink RLC/MAC control block contains a Reduced Block Sequence Number (RBSN) bit. At the time that an in-sequence RLC/MAC control block is designated for transmission, the value of RBSN is set equal to the value of the control send state variable V(CS).

### 9.1.7	Receive state variable V(R)

Each RLC endpoint receiver shall have an associated receive state variable V(R). The receive state variable denotes the BSN of the next in-sequence RLC data block expected to be received. V(R) shall be set to the value ‘0’ at the beginning of each TBF in which the RLC endpoint is the receiver. V(R) can take on the value 0 through (2*WS)-1.

In RLC acknowledged mode, V(R) shall be set to [ BSN' + 1 ] modulo 2*WS, where BSN' is the BSN of most recently received RLC data block, provided the RLC data block was error free and [ V(Q)  BSN' < V(Q) + WS ] modulo 2*WS.

In RLC unacknowledged mode, V(R) shall be set to [ BSN' + 1 ] modulo 2*WS, where BSN' is the BSN of most recently received RLC data block.

### 9.1.8	Receive window state variable V(Q)

Each RLC endpoint receiver shall have an associated receive window state variable V(Q). The receive window state variable denotes the BSN of the oldest RLC data block within the receive window that has not been received. V(Q) shall be set to the value 0 at the beginning of each TBF in which the RLC endpoint is the receiver. The receive window state variable can take on the value 0 through (2*WS)-1.

In RLC acknowledged mode, the value of V(Q) shall be updated when the RLC receiver receives the RLC data block whose BSN is equal to V(Q). The value of V(Q) shall then be set to the value of the oldest BSN in the receive window that has not been received, or it shall be set to V(R) if all RLC data blocks in the receive window have been received properly.

In RLC unacknowledged mode, if [V(R) - V(Q)] mod 2*WS > WS after updating V(R), then V(Q) is set to [V(R) - WS] mod 2*WS.

### 9.1.9	Receive state array V(N)

Each RLC endpoint receiver shall have an associated receive state array V(N). V(N) is an array of 2*WS elements indicating the receive status of WS previous RLC data blocks. The array is indexed relative to the receive state variable V(R) modulo 2*WS. When an RLC data block is received with BSN such that [ V(Q) BSN < V(R) ] modulo 2*WS, the corresponding element in V(N) is set to the value RECEIVED.

When an element in V(N) falls outside of the active window (i.e., [ BSN < V(R) - WS ] modulo 2*WS) the element is set to the value INVALID.

### 9.1.10	Reference sequence number (RSN) and received block bitmap (RBB)

The Packet Ack/Nack message contains a reference sequence number (RSN), a received block bitmap (RBB), a begin of window (BOW) bit, an end of window (EOW) bit and the length L of the RBB bitmap (see sub-clause 12.3). The Packet Ack/Nack message is sent by the RLC receiver and is received by the RLC transmitter. The RSN and RBB are determined as defined in this sub-clause and transmitted in both RLC acknowledged and RLC unacknowledged mode. The RSN and RBB may be ignored by the RLC transmitter in unacknowledged mode.

The RSN is a reference point and contains the BSN value following that of the last RLC block represented in the RBB bitmap.

The length L of the RBB bitmap shall be equal to or less than the number of available bits for RBB field coding, and equal to or less than [V(R) – V(Q)] modulo 2*WS.

The RBB is a bitmap representing Block Sequence Numbers, so that the last bit in the bitmap represents the (RSN –1) value and the first bit in the bitmap represents the BSN with value (RSN-L) modulo 2*WS.

The BSN values specified in the RBB are interpreted by subtracting the bit position in the bitmap from the RSN modulo 2*WS.

A valid BSN value in the RBB is one that is in the range [ V(A)  BSN < V(S) ] modulo 2*WS.

These inequalities shall be interpreted in the following way:

BSN is valid if, and only if, [ BSN - V(A) ] modulo 2*WS < [ V(S) - V(A) ] modulo 2*WS.

If [(RSN-L) modulo 2*WS]  = V(Q), then BOW shall be set. If  RSN = V(R), then EOW shall be set.

At the RLC transmitter:

If the BOW bit is set in the RBB bitmap, then this bitmap acknowledges all blocks between and including V(A) and (RSN – L -1) modulo 2*WS, and the corresponding elements in V(B) shall be set to the value ACKED. Also a bitmap value of ‘0’ is assumed at the bit position corresponding to (RSN - L) modulo 2*WS which corresponds to V(Q).

For each bit in the RBB whose corresponding BSN value is within the transmit window, if the bit contains the value ‘1’, the corresponding element in V(B) indexed relative to RSN shall be set to the value ACKED. If the bit contains the value ‘0’, the element in V(B) shall be set to the value NACKED. A bit within the RBB whose corresponding BSN is not within the transmit window, shall be ignored. If the RLC transmitter is on the user equipment side, the bit contains the value ‘0’ and the instance of timer T3198 corresponding to BSN is not expired (i.e. the RLC data block was recently (re)transmitted and thus can not be validly negatively acknowledged in this particular Packet Ack/Nack message), the element in V(B) shall not modified.

At the RLC receiver:

If the EOW bit is set in the RBB bitmap, then this bitmap acknowledges all blocks between and including [V(R) -L] and [V(R) –1] modulo 2*WS.

If the EOW bit is not set in the RBB bitmap, then this bitmap acknowledges all blocks between and including (RSN –1) and (RSN -L) modulo 2*WS, or (RSN –1) and V(Q), respectively for BOW not set or set.

For each bit in the bitmap, the bit is assigned the value ‘1’ if the corresponding element in V(N) indexed relative to RSN has the value RECEIVED. The bit is assigned the value ‘0’ if the element in V(N) has the value INVALID.

For this release it is assumed that the beginning of the acknowledge window is always V(Q).

When polled within a downlink RLC data block, the user equipment shall acknowledge all the RLC data blocks that have been correctly received up to and including the radio block where the UE is polled.

As an implementation option, the UE may also acknowledge as many as possible of the RLC data blocks that are correctly received after the radio block where the UE is polled.

### 9.1.11	Window Size

The window size (WS) shall be 256 (the value 512 shall be 2*WS).

### 9.1.12	Segmentation of LLC PDUs into RLC data units

Segmentation of LLC PDUs is supported to allow transport of LLC PDUs larger than the data field of a single RLC data block. If the contents of an LLC PDU do not fill an integer number of RLC data blocks, the beginning of the next LLC PDU shall be placed within the final RLC data block of the first LLC PDU, with no padding or spacing between the end of the first LLC PDU and the beginning of the next. If the final LLC PDU in the TBF does not fill an integer number of RLC data blocks, filler octets shall be used to fill the remainder of the RLC data block.

The received (and segmented) LLC PDUs shall be put into RLC data blocks in the same order as they are received from higher layers. A Block Sequence Number (BSN) is included in the header of each RLC data block to number the RLC data block. The RLC data blocks are to be numbered consecutively, modulo 2*WS, to allow re-assembly of the LLC PDUs on the receiving side.

Once an RLC data block has been transmitted over the physical link, should it be necessary to re-transmit the RLC data block, it shall be re-transmitted using the same channel coding scheme, BSN, and CV as it had in the previous transmission.

### 9.1.13	Re-assembly of LLC PDUs from RLC data units

RLC data blocks shall be collected at the receiver until all RLC data blocks comprising an LLC PDU have been received. The RLC headers shall be removed from each RLC data block at this time and the RLC data units re-assembled into an LLC PDU and passed to the next higher layer.

The size of the LLC PDU delivered to the higher layer shall not exceed 1560 octets. Any octet received beyond this maximum limit and until the next identified LLC PDU boundary shall be discarded.

During RLC acknowledged mode operation, received LLC PDUs shall be delivered to the higher layer in the order in which they were originally transmitted.

During RLC unacknowledged mode operation, received LLC PDUs shall be delivered to the higher layer in the order in which they are received. Fill bits having the value ‘0’ shall be substituted for RLC data units not received. The number of fill bits substituted shall be determined using Table 1. In the uplink direction the channel coding scheme shall be the commanded channel coding scheme. In the downlink direction the channel coding scheme shall be the channel coding scheme of the last correctly received RLC data block. If no RLC data blocks have been correctly received by the user equipment, the requested channel coding scheme shall be used. If no requested channel coding scheme has been sent to the network, the user equipment shall use the number of fill bits for CS-1.

Table 2: RLC unacknowledged mode fill bits

| Channel Coding Scheme | Number of fill bits |
| --- | --- |
| CS-1 | 160 |
| CS-2 | 240 |
| CS-3 | 424 |
| CS-4 | 608 |

### 9.1.14	Segmentation of RLC/MAC control messages into RLC/MAC control blocks

The network may segment RLC/MAC control messages into one or two RLC/MAC control blocks depending on the length of the RLC/MAC control message. If the contents of a control message do not fit an integer number of control blocks, filler octets shall be used to fill the remainder of the RLC/MAC control block. Only the last RLC/MAC control block containing elements of the control message shall contain filler octets. The Final Segment (FS) bit of the RLC/MAC control block header shall be set according to whether the RLC/MAC control block contains the final segment of an RLC/MAC control message.

The user equipment shall not segment RLC/MAC control messages.

### 9.1.15	Re-assembly of RLC/MAC control messages from RLC/MAC control blocks

RLC/MAC control blocks shall be collected at the receiver until all RLC/MAC control blocks comprising an RLC/MAC control message have been received.

If the contents of the RLC/MAC control blocks received by the user equipment with RTI value equal to x do not form a complete RLC/MAC control message when an RLC/MAC control block with RTI value described by the inequality  [x+12 < RTI  x+20] modulo 32 is received on the same PDCH as the RLC/MAC control blocks with RTI value equal to x, the user equipment shall discard and ignore the RLC/MAC control blocks with RTI value equal to x, irrespective of whether the RLC/MAC control block described by the inequality is addressed to the user equipment or not.

In packet idle mode, the user equipment shall be capable of receiving eight RLC/MAC control messages in parallel. If the user equipment receives RLC/MAC control blocks containing part of a ninth RLC/MAC control message while it still has RLC/MAC control blocks for eight partially received RLC/MAC control messages, the user equipment shall discard the RLC/MAC control blocks of the oldest partially received message.

In packet transfer mode, the user equipment shall be capable of receiving two RLC/MAC control messages in parallel on the same PDPCH. If the user equipment receives RLC/MAC control blocks containing part of a third RLC/MAC control message while it still has RLC/MAC control blocks for two partially received RLC/MAC control messages, the user equipment shall discard the RLC/MAC control blocks of the oldest partially received message.

The user equipment shall start an instance of timer T3200 following the receipt of an RLC/MAC control block whose RTI value does not correspond to the RTI value of a partially received RLC/MAC control message or if the RLC/MAC control blocks were received on different PDCHs. In non-DRX mode the duration of timer T3200 shall be four BS_CV_MAX block periods. In DRX mode the duration of timer T3200 shall be four times the DRX period (see TSM 03.64 [5]).

On receipt of an RLC/MAC control block containing a segment of an RLC/MAC control message such that the user equipment now has the complete RLC/MAC control message, the UE shall stop the corresponding instance of timer T3200.

If the user equipment discards a partially received RLC/MAC control message while the corresponding instance of timer T3200 is running, it shall stop the corresponding instance of timer T3200.

On expiry of an instance of timer T3200, the user equipment shall discard and ignore all segments of the corresponding partially received RLC/MAC control message.

Upon successful change of PDPCH allocation, the user equipment shall discard all partially received RLC/MAC control messages and stop the corresponding instances of timer T3200.

The user equipment shall discard any control message segment that contains an unknown TFI.

### 9.1.16	Priority of LLC PDUs

The user equipment shall not transmit LLC PDUs during a TBF that have a lower Radio Priority than the priority that was used at initial access or the priority sent in the last PACKET RESOURCE REQUEST message. The user equipment may change the Radio Priority of an uplink TBF by sending a PACKET RESOURCE REQUEST message to the network (see sub-clause 8.1.1.1.2 and sub-clause 8.1.1.3.2).

## 9.2	Operation during RLC/MAC control message transfer

RLC/MAC control blocks shall be used to transport RLC/MAC control messages. Segments of only one RLC/MAC control message shall be transported per RLC/MAC control block.

RLC/MAC control blocks shall be sent at a higher priority than RLC data blocks.

The receiving side shall determine the length of the RLC/MAC control message contents by interpreting the RLC/MAC control block contents.

No general acknowledgement shall be made as part of the transfer of RLC/MAC control blocks or RLC/MAC control messages. The receiver shall not acknowledge an RLC/MAC control block except when a valid RRBP field is present in the MAC header of the RLC/MAC control block. The receiver shall not acknowledge an RLC/MAC control message except when the RLC/MAC procedures explicitly specify an acknowledgement.

Each downlink RLC/MAC control block header, if present, contains a Radio Transaction Identifier (RTI) field that is 5 bits in length and performs in effect a modulo 32 count of the downlink RLC/MAC control messages sent on a PDCH. The RTI field shall be used to group the RLC/MAC control blocks that make up an RLC/MAC control message. The RTI field allows the transmitting and receiving entities to distinguish between up to 32 RLC/MAC control messages in a single transmit direction therefore allowing up to 32 parallel transactions per PDCH.

The network shall not use the same RTI value at the same time on the same PDCH for two separate RLC/MAC control messages. The network may use the same RTI value at the same time on separate PDCHs. The network shall transmit both segments of a segmented control message on the same PDCH.

## 9.3	Operation during RLC data block transfer

The RLC ARQ functions support two modes of operation: RLC acknowledged mode, and RLC unacknowledged mode. RLC acknowledged mode operation uses retransmission of RLC data blocks to achieve high reliability. RLC unacknowledged mode operation does not utilise retransmission of RLC data blocks. A TBF may operate in either RLC acknowledged mode or RLC unacknowledged mode.

The user equipment sets the RLC mode of the uplink TBF by setting the RLC_MODE bit to either RLC acknowledged mode or RLC unacknowledged mode in the PACKET RESOURCE REQUEST (two phase access or uplink TBF establishment when downlink TBF is active), PACKET ACCESS (P-PDPCH access) or PACKET DOWNLINK ACK/NACK (uplink TBF establishment when downlink TBF is active) message. In a one phase access, the RLC mode defaults to RLC acknowledged mode.

The network sets the RLC mode of the downlink TBF by setting the RLC_MODE bit in the IMMEDIATE ASSIGNMENT or PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message.

### 9.3.1	Countdown procedure

The user equipment shall send the Countdown Value (CV) in each uplink RLC data block to indicate to the network the absolute BSN (BSN’) of the last RLC data block that will be sent in the uplink TBF. The CV shall be calculated as follows.

where:

TBC = total number of RLC data blocks that will be transmitted in the TBF,

BSN’ = absolute block sequence number of the RLC data block, with range from 0 to (TBC - 1),

NRB = number of PDCHs assigned to the uplink TBF in the assignment message, with range 1 to 96,

the function round() rounds upwards to the nearest integer,

BS_CV_MAX is a parameter broadcast in the system information,

the division operation is non-integer and results is zero only for (TBC - BSN’ - 1) = 0.

The final RLC data block transmitted in the TBF (i.e., the RLC data block with BSN’ = TBC - 1) shall have CV set to the value ‘0’. No other RLC data blocks transmitted during the TBF shall have the value '0'.

Once the user equipment transmits a value of CV other than 15, the user equipment shall transmit exactly (TBC - BSN' - 1) untransmitted RLC data blocks. If the user equipment receives a change in the Channel Coding Command in a PACKET UPLINK ACK/NACK message during the countdown procedure, it shall act upon the new Channel Coding Command. The user equipment shall then recalculate the CV values for any untransmitted RLC data blocks using the new RLC data block size. If the user equipment has successfully completed the contention resolution procedure during one phase access and the countdown procedure is already running, it shall recalculate the CV values for any untransmitted RLC data blocks. Any data that arrives from the higher layer after the commencement of the countdown process shall be sent within a future TBF.

The user equipment may retransmit during the countdown in response to a Packet Ack/Nack or if stalled.

The user equipment may transmit an RLC/MAC control block during the countdown only in response to a poll request.

For fixed allocation, once the UE counts down to zero, at that point the UE forfeits its current uplink allocation and shall not transmit again using that allocation.

If the UE receives a new allocation during the countdown, the UE shall use this new allocation to the end of the countdown procedure. The network shall provide unsolicited uplink resources for any retransmissions that may be required.

### 9.3.2	Acknowledged mode operation

The transfer of RLC data blocks in the RLC acknowledged mode uses retransmissions of RLC data blocks. The transmitting side numbers the RLC data blocks via the block sequence number (BSN). The BSN is used for retransmission and for re-assembly. The receiving side sends PACKET Ack/Nack messages in order to request retransmission of RLC data blocks.

#### 9.3.2.1	Establishment of Temporary Block Flow

The establishment of a TBF occurs as described in clause 7. RLC functions related to the ARQ function shall not operate until RLC data block transfer has been initiated.

If the last uplink TBF ended with an incompletely transmitted LLC PDU or any unacknowledged LLC PDUs, the user equipment shall begin transmission on the new TBF with the oldest unacknowledged LLC PDU.

#### 9.3.2.2	Operation of uplink Temporary Block Flow

The user equipment shall transmit an RLC/MAC block in each assigned uplink data block. RLC/MAC control blocks have preference to RLC data blocks, i.e., temporarily replacing the PDTCH with PACCH.

The network shall send PACKET UPLINK ACK/NACK messages when needed.

The user equipment shall indicate a transmit window stall condition when V(S) = V(A) + WS. Upon detecting a transmit window stall condition, the user equipment shall set the Stall indicator (SI) bit in all subsequent uplink RLC data blocks until the stall condition ceases to exist.

Upon detecting the stall condition the user equipment shall also start timer T3182. Timer T3182 shall be stopped upon reception of a PACKET UPLINK ACK/NACK message that makes V(S) < V(A) + WS. If timer T3182 expires, the user equipment shall decrement counter N3102 by PAN_DEC, and perform an abnormal release with random access (see sub-clause 8.8.2).

Whenever the user equipment receives a PACKET UPLINK ACK/NACK message that allows the advancement of V(S) or V(A), the user equipment shall increment N3102 by PAN_INC, however N3102 shall never exceed the value PAN_MAX. Upon cell reselection, the user equipment shall set counter N3102 to the value PAN_MAX. When N3102  0 is reached, the user equipment shall perform an abnormal release with cell re-selection. If PAN_DEC, PAN_INC, or PAN_MAX are not received as part of the system information or are set to the value 0, counter N3102 shall be disabled.

A user equipment operating with a fixed allocation shall start or restart timer T3184 upon reception of a PACKET UPLINK ACK/NACK message. If timer T3184 expires, the user equipment shall perform an abnormal release with cell re-selection (see sub-clause 9.4.2).

#### 9.3.2.3	Release of uplink Temporary Block Flow

The user equipment initiates release of the uplink TBF by beginning the countdown process (see sub-clause 9.3.1). When the user equipment has sent the RLC data block with CV = 0 and there are no elements in the V(B) array set to the value Nacked, it shall start timer T3182. The user equipment shall continue to send RLC data blocks on each assigned uplink data block, according to the algorithm defined in sub-clause 9.1.4.

If the network has received all RLC data blocks when it detects the end of the TBF (i.e. when CV=0 and V(Q) = V(R)), it shall send the PACKET UPLINK ACK/NACK message with the Final Ack Indicator bit set to ‘1’, include a valid RRBP field in the RLC/MAC control block header and clear counter N3103. If the network has not received all of the RLC data blocks when it detects the end of the TBF, it shall send a PACKET UPLINK ACK/NACK message to the user equipment and if necessary allocate sufficient uplink resources for the user equipment to retransmit the required RLC data blocks.

Upon reception of a PACKET UPLINK ACK/NACK message the user equipment shall stop timer T3182. If the PACKET UPLINK ACK/NACK message has the Final Ack Indicator bit set to ‘1’ and includes the TBF Ending Time indication, the user equipment has new data to transmit but there is not an ongoing downlink TBF and the user equipment is not assigned to operate in half duplex mode, or it is assigned to operate in half duplex mode but it has not received a downlink assignment during the cowntdown or while timer T3182 is running, then the UE shall release the TBF and may, while waiting for the TBF Ending Time occurrence, request the establishment of a new uplink TBF. The UE shall forward the request by sending a PACKET RESOURCE REQUEST message on the resources of the released TBF, which continue to remain active.

In the case that S-PDPCHs only are active, the UE is allowed to send the PACKET RESOURCE REQUEST messages only at the network indicated uplink radio blocks, i.e. as an answer to the PACKET UPLINK ACK/NACK with the Final Ack Indicator bit set to “1”; the network shall consider this message as implicit acknowledgement to the PACKET UPLINK ACK/NACK message.

Otherwise, when there are P-PDPCHs, the UE can send the PACKET RESOURCE REQUEST message also at any time before the TBF Ending Time occurrence after or as the acknowledgement to the PACKET UPLINK ACK/NACK with the Final Ack Indicator bit set to “1”. At sending the PACKET RESOURCE REQUEST message the UE shall start timer T3168 and wait for the network reply. If timer T3168 expires the UE can send again the PACKET RESOURCE REQUEST message restarting timer T3168, unless it has already been repeated 4 times or the TBF Ending Time occurs.

While waiting for the TBF Ending Time occurrence, both the UE and the network shall continue to receive or transmit on the assigned resources according to their respective access mode.

While waiting for the TBF Ending Time occurrence, the network can send to the UE either of the following control messages: PACKET UPLINK ASSIGNMENT; PACKET DOWNLINK ASSIGNMENT; PACKET RESOURCE RECONFIGURE; PACKET ACCESS REJECT; PACKET RESOURCE RELEASE or PACKET UPLINK ACK/NACK with Final Ack Indicator bit set to ‘1’. The network can address the UE by re-using the TFI and possibly the USF of the released TBF or assigning new TFI and possibly USF values.

At the reception of a PACKET UPLINK ASSIGNMENT message or a PACKET RESOURCE RECONFIGURE message with global assignment or incremental assignment for an uplink TBF establishment, the UE shall stop waiting for the TBF Ending Time occurrence, stop timer T3168 if running and act upon the new assignment.

At the reception of a PACKET DOWNLINK ASSIGNMENT message or a PACKET RESOURCE RECONFIGURE message with incremental assignment for a downlink TBF establishment, the UE shall not stop waiting for the TBF Ending Time occurrence and act upon the new assignment.

The PACKET RESOURCE RECONFIGURE, the PACKET DOWNLINK ASSIGNMENT and PACKET UPLINK ASSIGNMENT messages can include incremental resource description as, during waiting for the TBF Ending Time occurrence, the resources of the released TBF are still active.

At the reception of a PACKET ACCESS REJECT message, the UE shall stop waiting for the TBF Ending Time occurrence, stop timer T3168 if running, release all active resources and can try to establish a new uplink TBF on the CCCH.

At the reception of a PACKET RESOURCE RELEASE message, the UE shall act as follows:

-	if the PACKET RESOURCE RELEASE message does not include RR (Resource Release) Ending Time indication, the UE shall stop waiting for the TBF Ending Time occurrence if running for the indicated resources, acknowledge the PACKET RESOURCE RELEASE message at the network indicated valid RRBP field and then release the indicated resources.

-	if the PACKET RESOURCE RELEASE message includes RR Ending Time indication, the UE shall start waiting for the RR Ending Time occurrence and stop waiting for the TBF Ending Time occurrence if running for the indicated resources, acknowledge the PACKET RESOURCE RELEASE message at the network indicated valid RRBP field, wait for the occurrence of the RR Ending Time and then release the indicated resources.

If the TBF Ending Time occurs and no new TBF has been established, or the PACKET UPLINK ACK/NACK message having Final Ack Indicator bit set to “1” contains a TBF Ending Time occurrence already elapsed, the UE shall release the TBF and all active resources, stop any establishment procedure possibly pending, and enter the packet idle mode. . Upon entering the packet idle mode, the UE shall apply DRX mode procedures as specified in sub-clause 5.5.1.5 and may try to establish a new uplink TBF via the common channels.

If the PACKET UPLINK ACK/NACK message has the Final Ack Indicator bit set to ‘1’ (with or without the TBF Ending Time indication), the user equipment has new data to transmit and there is an ongoing downlink TBF, the UE shall acknowledge the PACKET UPLINK ACK/NACK message with a PACKET CONTROL ACK message at the network indicated uplink radio block and may request the establishment of a new uplink TBF sending a PACKET DOWNLINK ACK/NACK message with the Channel Request Description IE at the signalled RRBPs or, in the case of active P-PDPCHs, a PACKET RESOURCE REQUEST message at any transmitting radio block. The UE and the network shall therefore follow the same procedures as described in sub-clause 8.1.2.4.

At the reception of a PACKET RESOURCE RELEASE message, the UE shall act as follows:

-	if the PACKET RESOURCE RELEASE message includes the RR Ending Time indication, the UE shall start waiting for the RR Ending Time occurrence and stop waiting for the TBF Ending Time occurrence if running for the indicated resources, acknowledge the PACKET RESOURCE RELEASE message at the network indicated valid RRBP field, wait for the occurrence of the RR Ending Time and then release the indicated resources.

-	 if the PACKET RESOURCE RELEASE message does not include the RR Ending Time indication, the UE shall stop waiting for the TBF Ending Time occurrence if running for the indicated resources, acknowledge the PACKET RESOURCE RELEASE message at the network indicated valid RRBP field and then release the indicated resources.

If the PACKET UPLINK ACK/NACK message has the Final Ack Indicator bit set to '1' and at least one of the conditions specified above for requesting the establishment of a new uplink TBF fails, the user equipment shall transmit the PACKET CONTROL ACKNOWLEDGEMENT message at the indicated RRBP and release the TBF. If the user equipment is operating in half duplex mode and receives a downlink assignment during the countdown or while timer T3182 was running, it shall then act on the downlink assignment. If there is no ongoing downlink TBF, the user equipment shall enter the packet idle mode, applying the DRX mode procedures as specified in sub-clause 5.5.1.5, where it may try to establish an uplink TBF via the common channels.

If, while waiting for the TBF ending time occurrence, the UE receives a successive PACKET UPLINK ACK/NACK message having the Final Ack Indicator bit set to ‘1’ and possibly including the TBF Ending Time indication, the UE shall acknowledge the received message at the indicated RRBP, (re-) start, if included, the TBF Ending Time occurrence according to the indicated value and possibly (re-) ask for the establishment of an uplink TBF going through the steps reported above.

If the PACKET UPLINK ACK/NACK message requests retransmission of RLC data blocks, the UE shall, if necessary, wait for allocation of uplink resources and then retransmit the RLC data blocks requested. The user equipment shall then start timer T3182 and wait for a PACKET UPLINK ACK/NACK message as described above.

If the user equipment is operating in half duplex mode and receives a downlink assignment during the countdown or while timer T3182 is running, and then T3182 expires, the UE shall then immediately act on the downlink assignment and request an uplink TBF via the Packet Downlink Ack/Nack. Otherwise, if timer T3182 expires, the user equipment shall perform an abnormal release with random access (see sub-clause 8.8.2).

At the occurrence of the TBF Ending Time or if the TBF Ending Time indication is not included in the (last) received PACKET UPLINK ACK/NACK message with the Final Ack Indicator bit set to ‘1’ and there is an ongoing downlink TBF, or at the occurrence of the TBF Ending Time after the reception of a PACKET DOWNLINK ASSIGNMENT message or a PACKET RESOURCE RECONFIGURE message with incremental assignment for a downlink TBF establishment with a TBF Starting Time still running, the UE shall release all active uplink resources but the P-PDPCHs, if present, carrying the preferred signalling channel (see sub-clauses 8.1.1.1.1. and 8.1.2.1.1) for the downlink TBF.

When the network receives the PACKET CONTROL ACKNOWLEDGEMENT message in the radio block indicated by the RRBP field, it may reuse the TFI, the USF and the released physical resources, possibly after the TBF Ending Time occurrence or the RR Ending Time occurrence depending on which of the two is running.

If the network does not receive the PACKET CONTROL ACKNOWLEDGEMENT message or the PACKET RESOURCE REQUEST message at the radio block indicated by the RRBP field, it shall increment counter N3103 and retransmit the PACKET UPLINK ACK/NACK message. If counter N3103 exceeds its limit, the network shall start timer T3169. When timer T3169 expires the network may reuse the TFI, the USF and all the physical resources.

#### 9.3.2.4	Operation of downlink Temporary Block Flow

The user equipment receives RLC/MAC blocks on the assigned downlink PDCHs. On each assigned PDCH, the user equipment shall in the RLC header identify the TFI and decode the RLC data blocks intended for the user equipment. The operation during the TBF shall be as defined in sub-clause 9.1.

#### 9.3.2.5	Release of downlink Temporary Block Flow

The network initiates the release of a downlink TBF by sending an RLC data block with the Final Block Indicator (FBI) set to the value ‘1’ and with a valid RRBP field. The sent RLC data block must have the highest BSN’ (see sub-clause 9.3.1) of the downlink TBF. The network shall start timer T3191. While timer T3191 is running, the network may retransmit the RLC data block with the FBI bit set to the value ‘1’.

If the user equipment receives an RLC data block with the FBI bit set to the value ‘1’ and with a valid RRBP field, it shall transmit a PACKET DOWNLINK ACK/NACK message in the specified uplink radio block.

Whenever the user equipment receives an RLC data block with a valid RRBP and it has received all RLC data blocks of the TBF, the UE shall send the PACKET DOWNLINK ACK/NACK message with the Final Ack Indicator bit set to ‘1’, stop timer T3190 if running and start or restart timer T3192.

If the user equipment receives more than one RLC data block with the FBI set to '1', it shall accept the data from only the first one of these blocks.

Anyway the UE shall send the PACKET DOWNLINK ACK/NACK message to acknowledge each received RLC data block with the FBI set to '1' and restart timer T3192.

If the network receives a PACKET DOWNLINK ACK/NACK message before timer T3191 expires and if retransmissions are required, then it shall stop timer T3191 and retransmit necessary RLC data blocks according to the ARQ protocol before re-initiating the release of the downlink TBF. The FBI is set to '1' only if the RLC data block with the highest BSN' of the TBF is retransmitted. If no retransmission is required, the network shall stop timer T3191.

While timer T3192 is running, the network and the UE shall continue to receive or transmit on the assigned resources according to their respective access mode.

If timer T3191 expires, then the network shall release the TBF.

If at the reception of an RLC data block with the FBI bit set to the value ‘1’ and with a valid RRBP field the user equipment has data to send, and there is not an ongoing uplink TBF, the UE can transmit a PACKET DOWNLINK ACK/NACK message in the specified uplink radio block, including a Channel Request Description IE starting or restarting timer T3192.

If the UE is operating on P-PDCH(s), it can also send, at any time after the acknowledgement of the last received RLC data block with the FBI bit set to the value ‘1’ while timer T3192 is running, a PACKET RESOURCE REQUEST message for the purpose to establish an uplink TBF. At sending the PACKET RESOURCE REQUEST message the UE shall start timer T3168 and wait for the network reply. If timer T3168 expires the UE can send again the PACKET RESOURCE REQUEST message restarting timer T3168, unless it has already been repeated 4 times or timer T3192 expires.

While timer T3192 is running, the network can respond to the UE with either of the following control messages: PACKET UPLINK ASSIGNMENT; PACKET DOWNLINK ASSIGNMENT; PACKET RESOURCE RECONFIGURE; PACKET ACCESS REJECT or PACKET RESOURCE RELEASE.

At the reception/transmission of a PACKET DOWNLINK ASSIGNMENT or a PACKET RESOURCE RECONFIGURE message with global assignment or incremental assignment for a downlink TBF establishment, the UE shall stop Timer T3192. The network and the UE shall then use the same procedures as for an downlink data block transfer defined in sub-clause 8.1.2.

At the reception/transmission of a PACKET UPLINK ASSIGNMENT message with global assignment, the UE shall stop Timer T3192 and timer T3168, if running. The network and the UE shall then use the same procedures as for an uplink data block transfer defined in sub-clause 8.1.1.

The PACKET RESOURCE RECONFIGURE; the PACKET DOWNLINK ASSIGNMENT and PACKET UPLINK ASSIGNMENT messages can include incremental resource description as, during timer T3192 running, the resources of the released TBF are still active.

At the reception/transmission of a PACKET UPLINK ASSIGNMENT message with incremental assignment or a PACKET RESOURCE RECONFIGURE message with incremental assignment for an uplink TBF establishment, timer T3192 shall continue to run and timer T3168 shall be stopped, if running. The network and the UE shall then use the same procedures as for an uplink or a downlink data block transfer, according to the respective assignment message content.

At the reception of a PACKET ACCESS REJECT message, the UE shall not stop timer T3192 but it shall stop timer T3168, if running.

At the reception of a PACKET RESOURCE RELEASE message, the UE shall act as follows:

-	if the PACKET RESOURCE RELEASE message includes the RR Ending Time indication, the UE shall start waiting for the RR Ending Time occurrence and stop timer T3192 if running for the indicated resources, acknowledge the PACKET RESOURCE RELEASE message at the network indicated valid RRBP field, wait for the occurrence of the RR Ending Time and then release the indicated resources.

-	if the PACKET RESOURCE RELEASE message does not include the RR Ending Time indication, the UE shall stop timer T3192 if running for the indicated resources, acknowledge the PACKET RESOURCE RELEASE message at the network indicated valid RRBP field and then release the indicated resources.

If the network has received the PACKET DOWNLINK ACK/NACK message with the Final Ack Indicator bit set to ‘1’ and it has new data to transmit for the user equipment, the network may establish a new downlink TBF for the UE by sending the PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with the Control Ack bit set to ‘1’ on PACCH.

If the user equipment, after sending the PACKET DOWNLINK ACK/NACK message with the Final Ack Indicator bit set to ‘1’, receives a PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with the Control Ack bit set to ‘1’ while timer T3192 is running, it shall stop timer T3192, consider the previous downlink TBF as released and act upon the new assignment.

At timer T3192 expiring, one of the following actions shall be taken:

- If the UE is operating in half duplex mode and receives an uplink assignment during the TBF release procedure, it shall immediately act upon the received assignment.

- If there is an ongoing uplink TBF, or after the reception of a PACKET UPLINK ASSIGNMENT message with incremental assignment or a PACKET RESOURCE RECONFIGURE message with incremental assignment for an uplink TBF establishment with TBF starting time still running, the UE shall   stop receiving from all downlink resources but from the P-PDPCHs, if present, carrying the preferred signalling channel (see sub-clauses 8.1.1.1.1 and 8.1.2.1.1) for the uplink TBF.

- Otherwise, the UE shall release all active packet resources and enter the packet idle mode. Upon entering the packet idle mode, the UE shall apply DRX mode procedures as specified in sub-clause 5.5.1.5.

While Timer T3192 is running, the network can address the UE on the PACCH using the TFI of the released downlink TBF.

### 9.3.3	Unacknowledged mode operation

The transfer of RLC data blocks in the RLC unacknowledged mode does not include any retransmissions, except during the release of an uplink TBF where the last transmitted uplink block may be retransmitted (see sub-clause 9.3.3.3). The block sequence number (BSN) in the RLC data block header is used to number the RLC data blocks for re-assembly. The receiving side sends Packet Ack/Nack messages in order to convey the necessary other control signalling (e.g. monitoring of channel quality for downlink transfer or timing advance or transmit power correction for uplink transfers).

#### 9.3.3.1	Establishment of Temporary Block Flow

If the last uplink TBF ended with an incompletely transmitted LLC PDU, the user equipment shall begin transmission on the new TBF with the last incompletely transmitted LLC PDU.

#### 9.3.3.2	Operation of uplink Temporary Block Flow

The network shall send PACKET UPLINK ACK/NACK messages when needed.

The user equipment shall set the Stall indicator (SI) bit to ‘0’ in all RLC data blocks. If the user equipment transmits k RLC data blocks without receiving a Packet Ack/Nack message, it shall start timer T3182. Timer T3182 shall be stopped upon reception of a PACKET UPLINK ACK/NACK message.

If timer T3182 expires, the user equipment shall decrement counter N3102 by PAN_DEC and perform an abnormal release with random access (see sub-clause 8.8.2).

Whenever the user equipment receives a PACKET UPLINK ACK/NACK message, it shall increment N3102 by PAN_INC, however N3102 shall never exceed the value PAN_MAX. Upon cell reselection, the user equipment shall set counter N3102 to the value PAN_MAX. When N3102  0 is reached, the user equipment shall perform an abnormal release with cell re-selection.

If PAN_DEC, PAN_INC or PAN_MAX are not received as part of the system information or are set to the value 0, counter N3102 shall be disabled. A user equipment operating with a fixed allocation shall start or restart timer T3184 upon reception of a PACKET UPLINK ACK/NACK message. If timer T3184 expires, the user equipment shall perform an abnormal release with cell re-selection (see sub-clause 9.4.2).

#### 9.3.3.3	Release of uplink Temporary Block Flow

The user equipment initiates release of the uplink TBF by beginning the countdown process (see sub-clause 9.3.1). It indicates the end of the TBF by setting the CV value to 0 and starts timer T3182.

When the network detects the end of the TBF (i.e. when CV=0), it shall send a PACKET UPLINK ACK/NACK message with the Final Ack Indicator bit set to ‘1’, include a valid RRBP field in the RLC/MAC control block header and clear counter N3103.

In case the network receives multiple blocks with CV=0, only the first needs to be acknowledged with PACKET UPLINK ACK/NACK message.

Upon reception of a PACKET UPLINK ACK/NACK message with the Final Ack Indicator bit set to “1”, the user equipment shall stop timer T3182 and transmit the PACKET CONTROL ACKNOWLEDGMENT message.

If the PACKET UPLINK ACK/NACK message has the Final Ack Indicator bit set to ‘1’ and includes the TBF Ending Time indication, the user equipment has new data to transmit but there is not an ongoing downlink TBF and, the user equipment equipment is not operating in half duplex mode, or it is assigned to operate in half duplex mode but it has not received downlink assignment during the countdown or while timer T3182 is running, the UE  shall release the TBF and may, while waiting for the TBF Ending Time occurrence, request the establishment of a new uplink TBF. The UE shall forward the request by sending a PACKET RESOURCE REQUEST message on the resources of the released TBF, which continue to remain active.ROL ACK message with the CTRL_ACK field set to “00”.

In the case that S-PDPCHs only are active, the UE is allowed to send the PACKET RESOURCE REQUEST messages only at the network indicated uplink radio blocks, i.e. as an answer to the PACKET UPLINK ACK/NACK with the Final Ack Indicator bit set to “1”; the network shall consider this message as implicit acknowledgement to the PACKET UPLINK ACK/NACK message.

Otherwise, when there are P-PDPCHs, the UE can send the PACKET RESOURCE REQUEST message also at any time before the TBF Ending Time occurrence after or as the acknowledgement to the PACKET UPLINK ACK/NACK with the Final Ack Indicator bit set to “1”. At sending the PACKET RESOURCE REQUEST message the UE shall start timer T3168 and wait for the network reply. If timer T3168 expires the UE can send again the PACKET RESOURCE REQUEST message restarting timer T3168, unless it has already been repeated 4 times or the TBF Ending Time occurs.

While waiting for the TBF Ending Time occurrence, both the UE and the network shall continue to receive or transmit on the assigned resources according to their respective access mode.

While waiting for the TBF Ending Time occurrence, the network can send to the UE either of the following control messages: PACKET UPLINK ASSIGNMENT; PACKET DOWNLINK ASSIGNMENT; PACKET RESOURCE RECONFIGURE; PACKET ACCESS REJECT; PACKET RESOURCE RELEASE or PACKET UPLINK ACK/NACK with Final Ack Indicator bit set to ‘1’. The network can address the UE by re-using the TFI and possibly the USF of the released TBF or assigning new TFI and possibly new USF values.

At the reception of a PACKET UPLINK ASSIGNMENT message or a PACKET RESOURCE RECONFIGURE message with global assignment or incremental assignment for an uplink TBF establishment, the UE shall stop waiting for the TBF Ending Time occurrence, stop timer T3168 if running and act upon the new assignment.

At the reception of a PACKET DOWNLINK ASSIGNMENT message or a PACKET RESOURCE RECONFIGURE message with incremental assignment for a downlink TBF establishment, the UE shall not stop waiting for the TBF Ending Time occurrence and act upon the new assignment.

The PACKET RESOURCE RECONFIGURE, the PACKET DOWNLINK ASSIGNMENT and PACKET UPLINK ASSIGNMENT messages can include incremental resource description as, during waiting for the TBF Ending Time occurrence, the resources of the released TBF are still active.

At the reception of a PACKET ACCESS REJECT message, the UE shall stop waiting for the TBF Ending Time occurrence, stop timer T3168 if running,, release all active resources and can try to establish a new uplink TBF on the CCCH.

At the reception of a PACKET RESOURCE RELEASE message, the UE shall act as follow:

-	if the PACKET RESOURCE RELEASE message includes the RR Ending Time indication, the UE shall start waiting for the RR Ending Time occurrence and stop waiting for the TBF Ending Time occurrence if running for the indicated resources, acknowledge the PACKET RESOURCE RELEASE message at the network indicated valid RRBP field, wait for the occurrence of the RR Ending Time and then release the indicated resources.

-	if the PACKET RESOURCE RELEASE message does not include the RR Ending Time indication, the UE shall stop waiting for the TBF Ending Time occurrence if running for the indicated resources, acknowledge the PACKET RESOURCE RELEASE message at the network indicated valid RRBP field and then release the indicated resources.

If the TBF Ending Time occurs and no new TBF has been established, or the PACKET UPLINK ACK/NACK message having Final Ack Indicator bit set to “1” contains a TBF Ending Time occurrence already elapsed, the UE shall release the TBF and all active resources, stop any establishment procedure possibly pending and enter the packet idle mode. Upon entering the packet idle mode, the UE shall apply DRX mode procedures as specified in sub-clause 5.5.1.5 and it may try to establish a new uplink TBF via the common channels.

If the PACKET UPLINK ACK/NACK message has the Final Ack Indicator bit set to ‘1’ (with or without the TBF Ending Time indication), the user equipment has new data to transmit and there is an ongoing downlink TBF, the UE shall acknowledge the PACKET UPLINK ACK/NACK message at the network indicated uplink radio block and may request the establishment of a new uplink TBF sending a PACKET DOWNLINK ACK/NACK message with the Channel Request Description IE at the signalled RRBPs or, in case of active P-PDPCHs, a PACKET RESOURCE REQUEST message at any transmitting radio block. The UE and the network shall therefore follow the same procedures as described in section 8.1.2.4.

At the reception of a PACKET RESOURCE RELEASE message, the UE shall act as follows:

-	if the PACKET RESOURCE RELEASE message includes the RR Ending Time indication, the UE shall start waiting for the RR Ending Time occurrence and stop waiting for the TBF Ending Time occurrence if running for the indicated resources, acknowledge the PACKET RESOURCE RELEASE message at the network indicated valid RRBP field, wait for the RR Ending Time occurrence and then release the indicated resources.

-	if the PACKET RESOURCE RELEASE message does not include the RR Ending Time indication, the UE shall stop waiting for the TBF Ending Time occurrence if running for the indicated resources, acknowledge the PACKET RESOURCE RELEASE message at the network indicated valid RRBP field and then release the indicated resources.

If the PACKET UPLINK ACK/NACK message has the Final Ack Indicator bit set to '1' and at least one of the conditions specified above for requesting the establishment of a new uplink TBF fails, the user equipment shall transmit the PACKET CONTROL ACKNOWLEDGEMENT message at the indicated RRBP and release the TBF. If the user equipment is operating in half duplex mode and received a downlink assignment during the countdown or while timer T3182 was running, it shall then act on the downlink assignment. If there is no ongoing downlink TBF, the user equipment shall enter packet idle mode, applying the DRX mode procedures as specified in sub-clause 5.5.1.5, where it may try to establish an uplink TBF via the common channels.

If, while waiting for the TBF ending time occurrence, the UE receives a successive PACKET UPLINK ACK/NACK message having the Final Ack Indicator bit set to ‘1’ and possibly including the TBF Ending Time indication, the UE shall acknowledge the received message at the indicated RRBP, (re-)start, if included, the TBF Ending Time occurrence according to the indicated value and possibly (re-)ask for the establishment of an uplink TBF going through the steps reported above.

If the PACKET UPLINK ACK/NACK message does not have the Final Ack Indicator bit set to ‘1’, the user equipment shall, when possible, repeat sending the last block with CV=0, until a PACKET UPLINK ACK/NACK message with Final Ack Indicator bit set to ‘1’ is received. Upon each retransmission of the last block with CV=0, the user equipment shall restart timer T3182. The block with CV=0 shall not be retransmitted more than four times. If the medium access mode is dynamic allocation, the repetitions are transmitted when the UE is scheduled via its USF. If fixed allocation is used, the UE shall transmit the repetitions within any remaining allocated uplink blocks. If timer T3182 expires, the user equipment shall release the TBF as if a PACKET UPLINK ACK/NACK message with Final Ack Indicator bit set to “1”was received.

At the occurrence of the TBF Ending Time, or if the TBF Ending Time indication is not included in the (last) received PACKET UPLINK ACK/NACK message with the Final Ack Indicator bit set to ‘1’ and there is an ongoing downlink TBF, or at the occurrence of the TBF Ending Time after the reception of a PACKET DOWNLINK ASSIGNMENT message or a PACKET RESOURCE RECONFIGURE message with incremental assignment for a downlink TBF establishment with a TBF Starting Time still running, the UE shall release all active uplink resources but the P-PDPCHs, if present, carrying the preferred signalling channel (see sub-clauses 8.1.1.1.1. and 8.1.2.1.1) for the downlink TBF.

When the network receives the PACKET CONTROL ACKNOWLEDGEMENT message in the radio block indicated by the RRBP field, it may reuse the TFI, the USF and the physical resources, possibly after either the TBF Ending Time occurrence or the RR Ending Time occurrence depending on which of the two is running.

If the network does not receive the PACKET CONTROL ACKNOWLEDGEMENT message or the PACKET RESOURCE REQUEST message at the radio block indicated by the RRBP field, it shall increment counter N3103 and retransmit the PACKET UPLINK ACK/NACK message. If counter N3103 exceeds its limit, the network shall start timer T3169. When timer T3169 expires the network may reuse the TFI, the USF and the physical resources.

#### 9.3.3.4	Operation of downlink Temporary Block Flow

The user equipment receives RLC/MAC blocks on the assigned downlink PDCHs. On each assigned PDCH, the user equipment shall in the RLC header identify the TFI and decode the RLC data blocks intended for the UE. The operation during the TBF shall be as defined in sub-clause 9.1.

#### 9.3.3.5	Release of downlink Temporary Block Flow

The network initiates release of a downlink TBF by sending an RLC data block with the Final Block Indicator (FBI) set to the value ‘1’ and with a valid RRBP field. The sent RLC data block must have the highest BSN' (see sub-clause 9.3.1) of the downlink TBF. The network shall start timer T3191. The network may retransmit the last block with FBI set to the value '1' and with a valid RRBP field. For each retransmission the timer T3191 is restarted.

For each RLC data block with the FBI bit set to ‘1’ and with a valid RRBP field, the user equipment shall transmit the PACKET CONTROL ACKNOWLEDGEMENT message in the uplink radio block specified by the RRBP field. If the user equipment receives more than one RLC data block with the FBI bit set to ‘1’ and with valid RRBP fields that point the same uplink block period, it shall transmit the PACKET CONTROL ACKNOWLEDGEMENT message only once. The user equipment shall then stop timer T3190 if running and start timer T3192. If the user equipment then receives a subsequent RLC data block with a valid RRBP and the FBI bit set to ‘1’, it shall retransmit the PACKET CONTROL ACKNOWLEDGEMENT message and restart timer T3192.

If the user equipment receives more than one RLC data block with the FBI set to '1', it shall accept the data from only the first one of these blocks.

If the network receives the PACKET CONTROL ACKNOWLEDGEMENT message before timer T3191 expires, the network shall stop timer T3191. While timer T3192 is running, the network and the UE shall continue to receive or transmit on the assigned resources according to their respective access mode.

If timer T3191 expires, the network shall release the TBF.

If at the reception of an RLC data block with the FBI bit set to the value ‘1’ and with a valid RRBP field the user equipment has data to send, and there is not an ongoing uplink TBF, the UE can transmit a PACKET CONTROL ACKNOWLEDGEMENT message in the specified uplink radio block including a Channel Request Description IE, starting or restarting timer T3192.

If the UE is operating on P-PDCH(s), it can also send, at any time after the acknowledgement of the last received RLC data block with the FBI bit set to the value ‘1’ while timer T3192 is running, a PACKET RESOURCE REQUEST message for the purpose to establish an uplink TBF. At sending the PACKET RESOURCE REQUEST message the UE shall start timer T3168 and wait for the network reply. If timer T3168 expires the UE can send again the PACKET RESOURCE REQUEST message restarting timer T3168, unless it has already been repeated 4 times or timer T3192 expires.

While timer T3192 is running, the network can respond to the UE with either of the following control messages:  PACKET UPLINK ASSIGNMENT; PACKET DOWNLINK ASSIGNMENT; PACKET RESOURCE RECONFIGURE; PACKET ACCESS REJECT or PACKET RESOURCE RELEASE message.

At the reception/transmission of a PACKET DOWNLINK ASSIGNMENT or a PACKET RESOURCE RECONFIGURE message with global assignment or incremental assignment for a downlink TBF establishment, the UE shall stop timer T3192. The network and the UE shall then use the same procedures as for a downlink data block transfer defined in sub-clause 8.1.2.

At the reception/transmission of a PACKET UPLINK ASSIGNMENT message with global assignment, the UE shall stop Timer T3192 and timer T3168, if running. The network and the UE shall then use the same procedures as for an uplink data block transfer defined in sub-clause 8.1.1.

The PACKET RESOURCE RECONFIGURE; the PACKET DOWNLINK ASSIGNMENT and PACKET UPLINK ASSIGNMENT messages can include incremental resource description as, during timer T3192 running, the resources of the released TBF are still active.

At the reception/transmission of a PACKET UPLINK ASSIGNMENT message with incremental assignment or a PACKET RESOURCE RECONFIGURE message with incremental assignment for an uplink TBF establishment, the UE shall not stop timer T3192 and timer T3168 shall be stopped, if running. The network and the UE shall then use the same procedures as for an uplink or a downlink data block transfer, according to the respective assignment message content.

At the reception of a PACKET ACCESS REJECT message, the UE shall not stop timer T3192 but it shall stop timer T3168, if running.

At the reception of a PACKET RESOURCE RELEASE message, the UE shall stop Timer T3192 and respond as follows:

-	if the PACKET RESOURCE RELEASE message includes the RR Ending Time indication, the UE shall start waiting for the RR Ending Time occurrence and stop timer T3192 if running for the indicated resources, acknowledge the PACKET RESOURCE RELEASE message at the network indicated valid RRBP field, wait for the RR Ending Time occurrence and then release the indicated resources.

-	if the PACKET RESOURCE RELEASE message does not include the RR Ending Time indication, the UE shall stop timer T3192 if running for the indicated resources, acknowledge the PACKET RESOURCE RELEASE message at the network indicated valid RRBP field and then release the indicated resources.

If the network has received the PACKET CONTROL ACKNOWLEDGEMENT message with the Final Ack Indicator bit set to “1” and it has new data to transmit for the user equipment, the network may establish a new downlink TBF for the user equipment by sending the PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with the Control Ack bit set to ‘1’ on PACCH.

If the user equipment, after sending the PACKET CONTROL ACKNOWLEDGEMENT message, receives a PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message with the Control Ack bit set to ‘1’ while timer T3192 is running, it shall stop timer T3192, consider the previous downlink TBF released and act upon the new assignment.

At timer T3192 expiring, one of the following actions shall be taken:

- If the UE is operating in half duplex mode and receives an uplink assignment during the TBF release procedure, it shall then immediately act upon the assignment;

- If there is an ongoing uplink TBF, or after the reception of a PACKET UPLINK ASSIGNMENT message with incremental assignment or a PACKET RESOURCE RECONFIGURE message with incremental assignment for an uplink TBF with TBF starting time still  running, the UE shall  stop receiving from all downlink resources but from the P-PDPCHs, if present, carrying the preferred signalling channel (see section 8.1.1.1.1 and 8.1.2.1.1) for the uplink TBF;

-	Otherwise, the UE shall release all resources and enter packet idle mode. Upon entering packet idle mode, the UE shall apply DRX mode procedures as specified in sub-clause 5.5.1.5.

While Timer T3192 is running, the network can address the UE on the PACCH using the TFI of the released downlink TBF.

## 9.4	Abnormal release cases

### 9.4.1	Abnormal release with random access

The user equipment shall abort all TBFs in progress and return to the CCCH and initiate establishment of an uplink TBF as defined in sub-clause 7.1.

### 9.4.2	Abnormal release with cell reselection

The user equipment shall abort all TBFs in progress and try to perform abnormal cell reselection (see TSM 05.08 [15]) and initiate establishment of an uplink TBF as defined in sub-clause 7.1 on the new cell.

# 10	RLC/MAC block structure

An RLC/MAC block consists of a MAC header and a RLC data block or RLC/MAC control block. The RLC/MAC block structure is shown in Figure 3.

Figure 4: RLC/MAC block structure

## 10.1	Spare bits

Where the description of RLC/MAC blocks in this Technical Specification contains bits defined to be 'spare bits', these bits shall set to the value '0' by the sending side, and their value shall be ignored by the receiving side.

## 10.2	RLC data blocks

The RLC data block consists of an RLC header, an RLC data unit, and spare bits. An RLC/MAC block containing an RLC data block may be encoded using any of the available channel coding schemes CS-1, CS-2, CS-3 or CS-4 (see TSM 05.03 [14]; other coding schemes are FFS). RLC/MAC blocks encoded using CS-1 or CS-2 do not contain spare bits. The size of the RLC data block for each of the channel coding schemes is shown in Table 3.

Table 4: RLC data block size

| Channel Coding Scheme | RLC data block size without spare bits | Number of spare bits | RLC data block size |
| --- | --- | --- | --- |
|  | (octets) |  | (octets) |
| CS-1 | 22 | 0 | 22 |
| CS-2 | 32 | 0 | 32 |
| CS-3 | 55 | 4 | 55 4/8 |
| CS-4 | 78 | 4 | 78 4/8 |

### 10.2.1	Downlink RLC data block

The Downlink RLC data block together with its MAC header is formatted as shown in Figure 5.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  | 6 | 5 |  | 4 |  | 3 |  |  | 2 |  |  | 1 |  |  |  |
| TS-SFI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Payload Type |  | RRBP |  |  | S/P |  | USF/MC |  |  |  |  |  |  |  |  | MAC header |  |
| TS-SF |  |  |  |  |  |  |  |  |  |  |  |  |  | PCI |  |  | Octet 1 (MAC optional) |
| Transmit Power command |  |  |  |  |  |  |  |  |  |  |  |  |  | PCI |  |  | Octet 2 (MAC optional) |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  | PCI |  |  | … (MAC optional) |
| Transmit Power command |  |  |  |  |  |  |  |  |  |  |  |  |  | PCI |  |  | Octet M (MAC optional) |
| BSN |  |  | TFI |  |  |  |  |  |  |  |  |  |  | FBI |  |  | Octet M+1 RLC Header |
|  |  |  |  | BSN |  |  |  |  |  |  |  |  |  | E |  |  | Octet M+2 RLC Header |
| Length indicator |  |  |  |  |  |  |  |  | M |  |  | E |  |  | Octet M+3 (RLC optional) |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  | … |  |  |
| Length indicator |  |  |  |  |  |  |  |  | M |  |  | E |  |  | Octet N (RLC optional) |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet N+1 |
| RLC data |  |  |  |  |  |  |  |  |  |  |  |  |  |  | … |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet O-1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet O |
|  |  |  | spare |  |  |  |  |  |  | spare |  |  |  |  |  | (if present) |  |

Figure 6: Downlink RLC data block with MAC header

### 10.2.2	Uplink RLC data block

The Uplink RLC data block together with its MAC header is formatted as shown in Figure 5.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  | 6 | 5 |  | 4 |  | 3 |  |  | 2 |  |  | 1 |  |  |  |  |  |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Payload Type |  | Countdown Value |  |  |  |  |  |  |  | SI |  |  | R |  |  | MAC header |  |  |  |  |
| BSN |  |  | TFI |  |  |  |  |  |  |  |  |  |  | TI |  |  | Octet 1 |  |  |  |
|  |  |  |  | BSN |  |  |  |  |  |  |  |  |  | E |  |  | Octet 2 |  |  |  |
| Length indicator |  |  |  |  |  |  |  |  | M |  |  | E |  |  | Octet 3 (optional) |  |  |  |  |  |
| . . . |  |  |  |  |  |  |  |  |  |  |  |  |  |  | . . . |  |  |  |  |  |
| Length indicator |  |  |  |  |  |  |  |  | M |  |  | E |  |  | Octet M (optional) |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet M+1 \ |  |  |  |
|  |  |  |  | TLLI |  |  |  |  |  |  |  |  |  |  |  | Octet M+2 } (optional) |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet M+3 / |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet M+4 / |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet M+5 (M+1 if no TLLI) |  |  |  |
| RLC data |  |  |  |  |  |  |  |  |  |  |  |  |  |  | . . . |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet N-1 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet N |  |  |  |
|  |  |  | spare |  |  |  |  |  |  | spare |  |  |  |  |  | (if present) |  |  |  |  |

Figure 7: Uplink RLC/MAC data block together with its MAC header

## 10.3	RLC/MAC control blocks

The RLC/MAC control block consists of a control message contents field and in the downlink direction an optional control header. RLC/MAC control messages shall be transported within RLC/MAC control blocks. An RLC/MAC control blocks shall always be encoded using the coding scheme CS-1 (see TSM 04.04 [8]).

### 10.3.1	Downlink RLC/MAC control block

The Downlink RLC/MAC control block together with its MAC header is formatted as shown in Figure 8.

| 8 | 7 |  | 6 |  |  | 5 |  | 4 |  |  | 3 |  |  | 2 |  | 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TS-SFI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Payload Type |  |  |  | RRBP |  |  |  |  | S/P |  |  | USF/MC |  |  |  |  |  |  |  |  | MAC header |  |  |  |  |
| TS-SF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCI |  |  |  |  | Octet 1 (MAC optional) |  |  |
| Transmit Power command |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCI |  |  |  |  | Octet 2 (MAC optional) |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCI |  |  |  |  | …(MAC optional) |  |  |
| Transmit Power command |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCI |  |  |  |  | Octet M (MAC optional) |  |  |
| RBSN |  | RTI |  |  |  |  |  |  |  |  |  |  |  |  | FS |  |  | AC |  |  |  |  | Octet M+1 (RLC optional) |  |  |
| Spare |  |  |  |  | TFI |  |  |  |  |  |  |  |  |  |  |  | D |  |  |  | Octet M+2 (RLC optional) |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet M+3 |  |  |  |
| Control Message Contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | . . . |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 21 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 22 |  |  |  |

Figure 9: Downlink RLC/MAC control block together with its MAC header

### 10.3.2	Uplink RLC/MAC control block

The Uplink RLC/MAC control block together with its MAC header is formatted as shown in Figure 10.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Payload Type |  |  |  |  |  | spare |  |  |  |  |  | R |  |  | MAC header |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 2 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 3 |
| Control Message Contents |  |  |  |  |  |  |  |  |  |  |  |  |  | . . . |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 21 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 22 |

Figure 11: Uplink RLC/MAC control block together with its MAC header

## 10.4	Header fields

### 10.4.1	User state flag (USF)/ Measurement on demand Command (MC) field

The USF field can be sent in a downlink RLC/MAC block to indicate which user equipment is allowed to transmit at the next Radio block (see TSM 05.02 [13]) on an uplink S-PDCH and to signal to the addressed user equipment whether there will not be valid information for it on a downlink S-PDCH at the next radio block; the available S-PDCHs for transmission and the unavailable S-PDCHs for reception at the next radio block are implicitly or explicitly indicated by the TS-SF field according to the following rules:

- If the TS-SF field is present, the UE is allowed to send radio blocks on the assigned S-PDCHs which are positioned on the enabled uplink time slots (see sub-clause 10.4.2) and is allowed not to detect radio blocks on the assigned S-PDCHs which are positioned on the enabled downlink time slots (see sub-clause 10.4.2);

- Otherwise, the UE is allowed to send radio blocks on all the assigned uplink S-PDCHs.

The USF field is three bits in length and therefore 8 different USF values can be assigned from the same downlink PDCH. Different user equipment can share the same USF value on a downlink S-PDCH if and only if they use different physical resources in the uplink.

On downlink P-PDCHs the use of the USF field is not required (see sub-clause 5.2.4) and therefore these bits can be used to carry the Measurement on demand Commands (MC) as defined below:

- A Measurement on demand Command shall last for NC/EXT_REPORTING_PERIOD_T (see sub-clause 5.6.4 for details), where NC/EXT_REPORTING_PERIOD_T corresponds to the value assigned to the user in packet transfer mode;

- A new Measurement on demand Command can be formulated by the network each NC/EXT_REPORTING_PERIOD_T;

- If there are more P-PDCHs assigned, only the one carried by the preferred signalling channel shall carry the Measurement on demand Commands; the other P-PDCHs may carry the same commands or nothing.

- The Measurement on demand Command shall follow the procedures described in TSM 04.08 [11];

- The Measurement on demand Command shall follow the coding rules described in TSM 04.04 [8] with the constraint that, when the available MC bits are more than requested by the Measurement on demand Command, the remaining bits (LSB) will be filled with < zero Padding >.

The time relation between a Measurement on demand Command and the UE reporting period is described in TSM 05.08 [15].

### 10.4.2	Time Slot State Flag (TS-SF) field

The TS-SF field can be sent in a downlink RLC/MAC data or control block to indicate to a user equipment the use of the assigned (both uplink and downlink) S-PDCHs at the next radio block.

The TS-SF field is coded as a bit map with respect to the 7 time slots of the frame such that:

bit 8 corresponds to time slot 0;

bit 7 corresponds to time slot 1;

bit 6 corresponds to time slot 2;

bit 5 corresponds to time slot 3;

bit 4 corresponds to time slot 4;

bit 3 corresponds to time slot 5;

and bit 2 corresponds to time slot 6 (see TSM 05.02 [13] for time slot numbering).

A bit in the TS-SF field set to value “1”, indicates to the UE that the assigned PDCHs on the corresponding time slot are available for transmission (if in uplink) or reception (if in downlink) at the next radio block. After receiving an RLC/MAC block disabling a given downlink time slot, the UE need not to monitor that time slot for the next radio block.

In case of detected errors in the TS-SF field, the UE shall act as if the downlink time slots were enabled and the uplink time slots were disabled.

The specific UE is addressed either by the USF field on a downlink S-PDPCH or implicitly by the carrying PDPCH, if on a downlink P-PDCH.

### 10.4.3	Transmit Power Command field

The Transmit Power Command field can be sent on a downlink RLC/MAC data or control block to indicate to the addressed user equipment the network expected power level on the uplink time slots which have been enabled via the TS-SF field (see sub-clause 10.4.2).

The inclusion of this field in the block header is optional and subjected to the following requirements:

The transmit power command field can be included only in conjunction with the TS-SF field;

Note:	the TS-SF can though be included unconditionally with respect to the Transmit power control field inclusion.

When included, there will be as many Transmit power control fields as the uplink time slots that have been enabled via the TS-SF field. If this field is not included but the UE has been enabled to transmit, the user equipment shall compute the power level to transmit based on open loop  power controls.

The transmit power command field is encoded with 7 bits as the BSS RSCP described in TSM 05.08 [15].

### 10.4.4	Retry (R) bit

The Retry (R) bit shall indicate whether the user equipment transmitted the CHANNEL REQUEST message (see TSM 04.08 [11]) one time or more than once during its most recent channel access. The user equipment shall send the same value for the R bit in each uplink RLC/MAC block of the TBF.

Table 5: Retry (R) bit

| Bit |  |
| --- | --- |
| 1 | Retry (R) bit |
| 0 | UE sent channel request message once |
| 1 | UE sent channel request message twice or more |

### 10.4.5	Stall indicator (SI) bit

The Stall indicator (SI) bit indicates whether the UE’s RLC transmit window can advance (i.e., is not stalled) or can not advance (i.e., is stalled). The user equipment shall set the SI bit in all uplink RLC data blocks.

Table 6: Stall indicator bit

| Bit |  |
| --- | --- |
| 2 | Stall indicator |
| 0 | UE RLC transmit window is not stalled |
| 1 | UE RLC transmit window is stalled |

### 10.4.6	Supplementary/Polling (S/P) Bit

The S/P bit is used to indicate whether the RRBP field is valid or not valid.

Table 7: Supplementary/Polling (S/P) bit

| Bit |  |
| --- | --- |
| 4 | S/P |
| 0 | RRBP field is not valid |
| 1 | RRBP field is valid |

### 10.4.7	Relative Reserved Block Period (RRBP) field

The RRBP value specifies a single uplink block in which (see sub-clause 8.6), the user equipment shall transmit either a PACKET CONTROL ACKNOWLEDGEMENT message or a PACCH block to the network. If the RRBP field is received as part of an RLC/MAC block containing an RLC/MAC control block, the user equipment shall transmit a PACKET CONTROL ACKNOWLEDGEMENT message in  the specified uplink radio block. The user equipment shall only react on RLC/MAC control blocks containing a valid RRBP field, if the user equipment is unambiguously addressed either in the downlink RLC/MAC control block header or by the carrying (P-)DPCH or in the control message itself. If the control message is segmented into more than one downlink RLC/MAC control blocks, the user equipment shall react only on RLC/MAC control blocks containing a valid RRBP field, if the user equipment is unambiguously addressed either in the downlink RLC/MAC control block header or by the carrying (P-)DPCH.

If the user equipment receives two or more RLC/MAC blocks containing an RLC/MAC control message with different RRBP values such that they specify the same uplink block, the user equipment shall transmit one PACKET CONTROL ACKNOWLEDGEMENT message in the specified uplink radio block.

If the RRBP field is received as part of an RLC/MAC block containing an RLC data block, the user equipment shall transmit a PACCH block in the specified uplink radio block, when answering on a P-PDCH. If the user equipment receives two or more RLC/MAC blocks containing an RLC data block with different RRBP values such they specify the same uplink radio block, the user equipment shall transmit one PACCH block in the specified uplink radio block.

If the user equipment receives an RLC data block and an RLC/MAC control block with different RRBP values such that they specify the same uplink radio block, the user equipment shall transmit an PACKET CONTROL ACKNOWLEDGEMENT message in the specified uplink radio block.

A polled control message shall always be sent in the uplink block specified by the corresponding valid RRBP field of a downlink RLC/MAC control block on the signalled PDCH, and not in any other uplink block that may be allocated to the user equipment.

Table 8 indicates the number of TDMA frames the user equipment shall wait before transmitting the uplink RLC/MAC block. The delay is relative to the first TDMA frame (N) of the downlink block containing the RRBP value. For definition of TDMA frame numbering, see TSM 05.02 [13].

Table 9: Relative Reserved Block Period (RRBP) field

| bit |  |
| --- | --- |
| 6 5 |  |
| 0 0 | Uplink block with TDMA frame number = (N+8) mod 2359296 |
| 0 1 | Uplink block with TDMA frame number = (N+12) mod 2359296 |
| 1 0 | Uplink block with TDMA frame number = (N+16) mod 2359296 |
| 1 1 | Uplink block with TDMA frame number = (N+20) mod 2359296 |

### 10.4.8	Countdown Value (CV) field

The Countdown Value (CV) field is sent by the user equipment to allow the network to calculate the number of RLC data blocks remaining for the current uplink TBF. The CV value shall be calculated according to the process described in sub-clause 9.3.1. The CV field is 4 bits in length and is encoded as a binary number with range 0 to 15.

### 10.4.9	Payload Type field

The Payload Type field shall indicate the type of data contained in remainder of the RLC/MAC block. The encoding of the Payload Type field is shown in Table 10.

Table 11: Payload Type field

| bit |  |
| --- | --- |
| 8 7 | Payload Type |
| 0 0 | RLC/MAC block contains an RLC data block |
| 0 1 | RLC/MAC block contains an RLC/MAC control block without optional RLC octets |
| 10 | In the downlink direction, the RLC/MAC block contains an RLC/MAC control block that includes optional RLC octets. In the uplink direction, this value is reserved. |
| 1 1 | Reserved |

### 10.4.10	Final block indicator (FBI) bit

The Final block indicator (FBI) bit indicates that the downlink RLC data block is the last RLC data block of the downlink TBF.

Table 12: Final block indicator bit

| bit |  |
| --- | --- |
| 1 | Final block indicator |
| 0 | Current block is not the last RLC data block in TBF |
| 1 | Current block is the last RLC data block in TBF |

### 10.4.11	TS-SF Indicator (TS-SFI) bit

The TS-SF Indicator (TS-SFI) bit indicates the presence of an optional TS-SF field within the RLC/MAC data or control block.

Table 13: TS-SF Indicator (TS-SFI) bit

| Bit |  |
| --- | --- |
| 8 | TS-SFI indicator (TS-SFI) bit |
| 0 | TS-SF field is not present |
| 1 | TS-SF field is present |

### 10.4.12	Transmit Power command Indicator (PCI) bit

The transmit Power Command Indicator (PCI) bit indicates the presence of an optional Transmit Power Command field within the RLC/MAC data or control block.

Table 11: Transmit Power Command Indicator (PCI) bit

| Bit |  |
| --- | --- |
| 1 | Transmit Power Command indicator (PCI) bit |
| 0 | Transmit Power Command field is not present |
| 1 | Transmit Power Command field is present |

### 10.4.13	TLLI Indicator (TI) bit

The TLLI Indicator (TI) bit indicates the presence of an optional TLLI field within the RLC data block.

Table 14: TLLI Indicator (TI) bit

| bit |  |
| --- | --- |
| 1 | TLLI indicator (TI) bit |
| 0 | TLLI field is not present |
| 1 | TLLI field is present |

### 10.4.14	Address Control (AC) bit

The Address Control (AC) bit is used to indicate the presence of the optional TFI/D octet in the header of downlink RLC/MAC control blocks.

Table 15: Address Control (AC) bit

| bit |  |
| --- | --- |
| 1 | Address Control (AC) bit |
| 0 | TFI/D octet is not present |
| 1 | TFI/D octet is present |

### 10.4.15	Final Segment (FS) bit

The Final Segment (FS) bit indicates that the downlink RLC/MAC control block contains the final segment of an RLC/MAC control message.

Table 16: Final Segment (FS) bit

| bit |  |
| --- | --- |
| 2 | Final Segment (FS) bit |
| 0 | Current block does not contain the final segment of an RLC/MAC control message |
| 1 | Current block contains the final segment of an RLC/MAC control message |

### 10.4.16	Radio Transaction Identifier (RTI) field

The Radio Transaction Identifier (RTI) field is used to group the downlink RLC/MAC control blocks that make up an RLC/MAC control message and identifies the segmented control message sequence with which the downlink RLC/MAC control block is associated. The RTI field is five bits in length with range 0 to 31.

### 10.4.17	Direction (D) bit

The Direction (D) bit indicates the direction of the TBF identified by the TFI field in the downlink RLC/MAC control block header.

Table 17: Direction (D) bit

| bit |  |
| --- | --- |
| 1 | Direction (D) bit |
| 0 | TFI field identifies an uplink TBF |
| 1 | TFI field identifies a downlink TBF |

### 10.4.18	Temporary Flow Identity (TFI) field

In RLC data blocks, the TFI identifies the Temporary Block Flow (TBF) to which the RLC data block belongs. For the downlink and the uplink, the TFI field is 5 bits in length encoded as a binary number with range 0 to 31.

In downlink RLC/MAC control blocks, the TFI identifies the Temporary Block Flow (TBF) to which the RLC/MAC control message contained in the downlink RLC/MAC control block relates. When present and if transmitted on a S-PDCH, this field indicates the user equipment to which the control message is addressed so that all other user equipment sharing the same PDCH shall ignore the control message. If this field is present and the contents of the control message also contains a TFI addressing the user equipment, the user equipment shall ignore the TFI in the control message contents. If this field is not present and the RLC/MAC control message is sent on a S-PDCH, all user equipment sharing the same PDCH shall interpret the contents of the control message.

### 10.4.19	Extension (E) Bit

The Extension (E) bit is used to indicate the presence of an optional octet in the RLC data block header.

Table 18: Extension (E) bit

| Bit |  |
| --- | --- |
| 1 | E bit |
| 0 | Extension octet follows immediately |
| 1 | No extension octet follows |

### 10.4.20	Block Sequence Number (BSN) field

The Block Sequence Number (BSN) field carries the absolute Block Sequence Number (BSN’) modulo 2*WS of each RLC data block within the TBF. The BSN is 9 bits in length and is encoded as a binary number with range 0 to (2*WS)-1. In the header: bit 8 in octet 1 is the MSB, bit 2 in octet 2 is the LSB.

### 10.4.21	Reduced Block Sequence Number (RBSN) bit

The Reduced Block Sequence Number (RBSN) bit carries the sequence number of the downlink RLC/MAC control blocks. The RBSN bit is encoded as a binary number with range 0 to 1.

### 10.4.22	More (M) bit

The M bit, along with the E bit and the Length Indicator (LI), is used to delimit LLC PDUs within a TBF. When the M bit is present, it indicates whether or not another LLC PDU follows the current one within the RLC data block. The function of the M and E bits when they occur in the same octet is defined in Table 19.

Table 20: M bit and E bit

| Bit |  |
| --- | --- |
| M E |  |
| 0 0 | The LI field of the current LLC PDU shall continue in the next octet |
| 0 1 | no LLC data after the current LLC PDU, no more extension octets |
| 1 0 | a new LLC PDU starts after the current LLC PDU and there is another extension octet, which delimits the new LLC PDU |
| 1 1 | a new LLC PDU starts after the current LLC PDU and continues until the end of the RLC information field, no more extension octets |

Configuration ‘00’ of the M and E bits can be used when RLC/MAC block is coded using CS-4 (see sub-clause 10.2).

### 10.4.23	Length Indicator (LI) field

The Length Indicator is used to delimit LLC PDUs within the RLC data block. The first Length Indicator shall indicate the number of octets of the RLC data field belonging to the first LLC PDU, the second Length Indicator shall indicate the number of octets of the RLC data field belonging to the second LLC PDU, etc. Only the last segment of any LLC PDU of a TBF (either this segment carries the entire LLC PDU or not) shall be identified with a Length Indicator within the corresponding RLC data block.

A singular case occurs when the end of the LLC PDU would fit within the RLC data block but the addition of the Length Indicator octet (to indicate the LLC PDU boundary) causes the LLC PDU to extend into the next RLC data block. In this case, this additional LI field shall take the value 0 whatever is the length of the last but one LLC PDU segment.

The final RLC data block of a TBF shall have a Length Indicator field corresponding to the final LLC PDU unless this PDU fills the RLC data block precisely without the LI field being added (i.e. the singular case mentioned above never applies in this situation).

The LI field is 6 bits in length and shall be encoded as a binary number with range 1 to 19, 1 to 29, 1 to 52 or 1 to 75, according to the coding scheme in use, i.e. CS-1, CS-2, CS-3 or CS-4 respectively. The value 0 shall indicate that no LLC PDU boundary exists. In this case the M bit shall be set to’ 0’ and the E bit shall be set to ‘1’ on the transmitting side, while on the receiving side the M bit shall be ignored and the E bit shall be interpreted as having the value ‘1’.

In case of CS-4, the extended LI notation shall be used, according to sub-clause 10.4.22, using configuration ‘00’ of the M and E bits.

All other values are reserved, and in this version of the protocol, the user equipment shall ignore all fields of the RLC data block except for the header fields.

### 10.4.24	TLLI field

The TLLI field contains a TLLI encoded as the contents of the TLLI information element defined in TSM 04.08 [11].

### 10.4.25	RLC data field

The RLC data field contains octets from one or more LLC PDUs. The RLC data field may contain parts of one or two LLC PDUs and all of an arbitrary number of LLC PDUs. The E bit, the M bit, and the Length Indicator delimit the RLC data field into LLC PDUs. If the last LLC PDU of the TBF does not fill the entire RLC data field, an extension octet shall be used to indicate the number of valid RLC data octets and the remainder of the RLC data field shall be filled with filler octets with the value ‘00101011’. Only the last RLC data block of the TBF may contain filler octets.

### 10.4.26	Control message contents field

The Control message contents field shall contain exactly one segment from one RLC/MAC control message field (i.e., RLC/MAC control block).

# 11	Message functional definitions and contents

This clause defines the structure of the RLC/MAC control messages. These are non-standard L3 messages as defined in TS 24.007 [10]. The formats for the messages are valid only for the PDCH. The format for RLC/MAC control messages for use on the CCCH are defined in TSM 04.08 [11].

Each definition given in the present clause includes:

-	a brief description of the message direction and use;

-	a CSN.1 description of the message information elements and fields (see TS 24.007 [10]). Definition of information elements may immediately follow the definition of the message. If the definition of an information element immediately follows the message definition, the information element name ends with 'struct'. Otherwise the information element name ends with 'IE' and the definition of the information element is defined in clause 12 or in TSM 04.08 [11]. The definition of a 'struct' is valid only within the table in which it is defined. No references shall be made to a 'struct' definition from outside of the table in which it is defined or from outside this document. The definition of an information element is valid throughout clause 11 and clause 12;

-	a note specifying, where appropriate, conditions for information elements or fields with presence requirement C or O in the relevant message which together with other conditions specified in this TS define when the information elements shall be included or not, what non-presence of such information elements or fields means, and - for IEs with presence requirement C - the static conditions for presence and/or non-presence of the information elements or fields (see TS 24.007 [10]);

-	a table follows which contains a definition for each field referenced in the message definition or in an information element struct immediately following the message definition.

Bit fields within RLC/MAC messages shall have the highest numbered bit of the bit field in the highest numbered bit of the lowest number octet. The mapping of an 11 bit field is illustrated in Figure 12.

| Bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  |  | Octet N |
|  |  |  |  | bit 11 | Bit 10 | bit 9 | bit 8 |  | Octet N+1 |
| bit 7 | bit 6 | bit 5 | bit 4 | bit 3 | Bit 2 | bit 1 |  |  | Octet N+2 |
|  |  |  |  |  |  |  |  |  | Octet N+3 |

Figure 13: Field mapping within RLC/MAC messages

The length of an RLC/MAC control message is an integer number of RLC/MAC control blocks. Padding bits are necessary to fill the message up to the desired length. The padding bits may be the ‘null’ string. Otherwise, the padding bits starts with bit ‘0’, followed by ‘spare padding’.

< padding bits > ::= { null | 0 < spare padding > ! < Ignore : 1 bit** = < no string > > } ;

The padding sequence used for 'spare padding' in this EN, see TS 24.007 [10], is a repetition of octet ‘00101011’, starting on an octet boundary.

## 11.1	Handling of erroneous protocol data

This section specifies procedures for the handling of unknown and erroneous protocol data by the receiving entity.

These error-handling procedures are mandatory for the user equipment.

A message is defined to be syntactically incorrect if it violates rules of clauses 11 and 12, or if it contains at least one value defined as "reserved" in clauses 11 and 12. However, if the rules of clause 11 and 12 define a specific interpretation for a "reserved" value, the specified interpretation takes precedence and the considered field remains syntactically correct.

Decoding a received message based on its CSN.1 description yields the complete acceptance or rejection of the message. Error handling allows a message to be partially accepted even when some parts are erroneous.

Error detection mechanisms are introduced to identify which parts of a message to be protected against which kinds of errors.

### 11.1.1	Message classification

The packet data channel (PDCH) can be a dedicated or a shared resource depending on whether the carrying PDPCH is a P-PDPCH or S-PDPCH respectively. Messages sent by the network on a S-PDCH, can be read by all user equipment sharing that resource.

The message type is identified by the MESSAGE_TYPE field contained in each message. The message type is used for classification and determining the message syntax.

Messages sent from the network to the user equipment are classified as either distribution messages or non-distribution messages.

#### 11.1.1.1	Distribution messages

A distribution message is recognised by the most significant bit of the message type being set to bit ‘1’. The general format of a distribution message sent from the network to the user equipment station is

< Distribution message > ::=

< MESSAGE_TYPE : 1 bit (5) >

< Distribution contents >

< padding bits > ;

A distribution message may be received by all user equipment. Depending on the protocol state of the UE, it shall be analysed as specified in clauses 5, 6, 7, 8 and 9 of this TS.

The specific syntax of the ‘Distribution contents’ depends on the message type. The ‘padding bits’ can be reduced to the null string.

#### 11.1.1.2	Non-distribution messages

A non-distribution message is recognised by the most significant bit of the message type being set to bit ‘0’. The general format of a message sent from the network to the user equipment is

< Non-distribution message > ::=

< MESSAGE_TYPE : 0 bit (5) >

< Distribution contents >

< Address information > < Non-distribution contents >

< padding bits > ;

A non-distribution message may be received by all user equipment.

The ‘Distribution contents’ of a non-distribution message contains Page Mode information. Depending on the protocol state of the user equipment, it shall be analysed by a UE receiving the message, as specified in clause 5 of this TS.

The ‘Address information’ contained in the message shall be analysed by a UE receiving the message. The ‘Non-distribution contents’ following the address information shall be ignored by any UE not identified by the address information. The allowed addressing options and the specific syntax of the ‘Non-distribution contents’ depend on the message type. The ‘padding bits’ can be reduced to the null string.

##### 11.1.1.2.1	Format of the address information

The general format of the ‘Address information’ in a non-distribution message is

< Address information > ::=

0	< Global TFI IE > |	-- see clause 12.6

1 0	< TLLI > |

1 1 1	< Request Reference IE > ;	-- see TSM 04.08 [11]

The description of a certain message type may specify a restricted set of addressing options being syntactically correct in the message. A message received with a disallowed addressing option shall be regarded as syntactically incorrect.

### 11.1.2	Error detection mechanism

The symbol ‘!’ indicates an error branch. It acts as a separator (similar to the ‘|’ choice symbol) where the choice on the right of the ‘!’ are to be considered as an ‘error’ branch. The symbol ‘!’ allows partial analysis of data in a received message, with some parts of the message to be ignored due to it being syntactically incorrect.

The description on the left of ‘!’ defines the set of syntactically correct data and shall be recognised correctly. Otherwise, the data associated shall be rejected and the description within the error branch shall be used.

The description within the error branch, on the right of ‘!’, shall accept any syntactically incorrect data. Therefore, according to the error label the relevant error handling procedure shall be implemented.

### 11.1.3	Error labels

There are different categories of error labels introduced in clauses 11 and 12 of this TS.

#### 11.1.3.1	Generic error labels

Generic error labels are defined for syntactical errors ‘Unknown message type’, ‘Distribution part error’, ‘Address information part error’ and ‘Non-distribution part error’.

The general format of a distribution message, including these error labels, is:

< Distribution message > ::=

< MESSAGE_TYPE : 1 bit (5) >

{	< Distribution contents >

< padding bits >

! < Distribution part error : bit (*) = < no string > > }

! < Unknown message type : bit (*) = < no string > > ;

The general format of a non-distribution message, including these error labels, is:

< Non-distribution message > ::=

< MESSAGE_TYPE : 0 bit (5) >

{	< Distribution contents >

{	< Address information >

{	< Non-distribution contents >

< padding bits >

! < Non-distribution part error : bit (*) = < no string > > }

! < Address information part error : bit (*) = < no string > > }

! < Distribution part error : bit (*) = < no string > > }

! < Unknown message type : bit (*) = < no string > > ;

These error labels allow ignoring a part of the message that is syntactically incorrect. Once an error is detected, the error branch is called, followed by a null string that expands to the end of the message. The corresponding data is ignored.

#### 11.1.3.2	‘Ignore’ error label

An ‘Ignore’ error label is used to ignore part of the message. The generic description is:

< content > ! < Ignore : bit (*) = < no string > >	-- Ignore by indefinite length

Or:

< content of fixed length n > ! < Ignore : bit (n) = < no string > >	-- Ignore by definite length

An ‘Ignore’ error label shall be applied by the receiver of a downlink RLC/MAC control message when specified in the message description in clauses 11 and 12 of this TS. This error label allows ignoring a part of the message that is syntactically incorrect. Once the error is detected, the error branch ‘Ignore’ is called followed by a null string.

When this error label is used with an indefinite length (bit (*) = < no string >), the null string expands to the end of the message and the corresponding data is ignored.

NOTE:	If this error label is used with the indefinite length within a structure or delimited description (i.e. within { } brackets), any description following the structure or delimited description must allow truncation, in order to be consistent with the CSN.1 description of the message.

When this error label is used with a definite length (bit (n) = < no string >), the null string is a defined number of bits and the corresponding data is ignored.

#### 11.1.3.3	‘Message escape’ error label

The ‘Message escape’ error label is used to provide an escape for, e.g., a future modification of the message syntax. The generic description is:

0 < Content > ! < Message escape : 1 bit (*) = < no string > >

A ‘Message escape’ error label shall be applied by the receiver of a downlink RLC/MAC control message when specified in the message description in clauses 11 and 12 of this TS. The description on the left of the error branch needs to be correctly recognised. Otherwise, the error branch ‘Message escape’ is called and the remaining part of the message is ignored.

NOTE:	Any description following a structure or delimited description (i.e. within { } brackets) including this error label must allow truncation. Otherwise, it is not consistent with the CSN.1 description of the message.

### 11.1.4	Error detection and order of precedence

A user equipment shall detect and process errors in the order in which they are defined in this sub-clause (e.g., a message, which is not compatible with the current protocol state AND is syntactically incorrect, shall be treated as if it is not compatible with the current protocol state).

At certain error events defined in this sub-clause, the PACKET MOBILE TBF STATUS message shall be sent by the user equipment. In case of multiple error events, and, due to restrictions defined in clauses 5, 6, 7, 8 and 9, the user equipment is not able to send a first status message until the occurrence of a subsequent event generating a second status message, the UE shall suppress the sending of the second and additional status messages until the first status message has been sent to the network.

For UEs implemented according to this version of the protocol, the sending of the PACKET MOBILE TBF STATUS message is optional. This option is implementation dependent. A UE implementing this option shall send the PACKET MOBILE TBF STATUS message as specified in this clause.

#### 11.1.4.1	Unknown message type

If a user equipment receives a message with message type either not defined or not implemented (generic error label: ‘Unknown message type’), the message shall be ignored.

#### 11.1.4.2	Message not compatible with current protocol state

When a non-distribution message is received, which is not expected by the addressed receiver in its current protocol state, the user equipment shall follow the procedures that are described in clauses 5, 6, 7, 8 and 9 of this TS.

If no such reaction is specified, the user equipment shall ignore the message. If in packet transfer mode, the UE, which is identified by the address information shall return a status message (PACKET MOBILE TBF STATUS message) with TBF_CAUSE #4, "Message not compatible with current protocol state".

Unexpected distribution messages are ignored.

#### 11.1.4.3	Syntactically incorrect message

When a message containing a syntactically incorrect data is received, depending on the error detection mechanisms that may be defined in the CSN.1 description of the message, the message can be rejected or partially accepted.

Exceptions to the rules in this sub-clause are given in sub-clause 11.1.4.5.

NOTE:	The order, in which the error labels mentioned in this clause are detected and processed, depends on the nesting of error labels defined by the description of each message type in sub-clauses 11.2 and 12. E.g., a message, which contains syntactically incorrect data in both the addressing information AND the non-distribution contents, is typically received with the error label ‘Address information part error’.

##### 11.1.4.3.1	Messages with error label: ‘Distribution part error’

For syntactically incorrect messages received with generic error label: ‘Distribution part error’, data corresponding to the description following the error label shall be recognised as erroneous data and be ignored.

##### 11.1.4.3.2	Messages with error label: ‘Address information part error’

For syntactically incorrect messages received with generic error label: ‘Address information part error’, data corresponding to the description following the error label shall be recognised as erroneous data and be ignored. The distribution contents preceding the error label may be analysed and treated as described in clause 5 of this TS.

##### 11.1.4.3.3	Messages with error label: ‘Non-distribution part error’

For syntactically incorrect messages received with generic error label: ‘Non-distribution part error’, data corresponding to the description following the error label shall be recognised as erroneous data and be ignored.

The distribution contents preceding the error label may be analysed and treated as described in clause 5 of this TS.

The address information preceding the error label shall be analysed. In packet transfer mode, the user equipment identified by the address information shall return a PACKET MOBILE TBF STATUS message with TBF_CAUSE #2 "Syntactically incorrect message, non-distribution part error".

##### 11.1.4.3.4	Messages with error label: ‘Message escape’

For syntactically incorrect messages with error label: ‘Message escape’, data corresponding to the description following the error label shall be recognised as erroneously received mandatory data and be rejected.

The distribution contents preceding the error label may be analysed and treated as described in clause 5 of this TS.

If the address information proceeds the error label and it is received correctly, it shall be analysed. In packet transfer mode, the user equipment identified by the address information shall return a PACKET MOBILE TBF STATUS message with TBF_CAUSE #3 "Syntactically incorrect message, message escape".

##### 11.1.4.3.5	Messages with error label: ‘Ignore’

For syntactically incorrect messages with error label: ‘Ignore’, data corresponding to the description following the error label shall be recognised as unnecessary data. If a syntactically incorrect message with the ‘Ignore’ error label is received, depending on the length of the null string associated with the error label (sub-clause 11.1.2.1), the corresponding data shall be ignored.

#### 11.1.4.4	Syntactic error in truncated concatenation

Truncated concatenation is sequences of components encapsulated by the { } brackets followed by the symbol ‘//’. The concatenation is any of the concatenations starting with null and up to any number of components.

{ < a >  < b >  < c > } //

The above set is equivalent to

{ < a >  < b >  < c > }	or

{ < a >  < b > }	or

{ < a > }	or

null

Any syntactically incorrect component shall truncate the sequence. The correctly received components are accepted and the truncated components are ignored.

NOTE:	If the ‘padding bits’ at the end of a message are included within the concatenation, truncation requires the resulting concatenation to fit exactly with the received message length. Otherwise, it is a syntactical error, which may cause rejection of the complete message or part thereof.

#### 11.1.4.5	Exceptions

Void.

## 11.2	RLC/MAC control messages and RRM messages

Table 21.a summarises the RLC/MAC control messages. Table 16.b summarises the messages for Radio Resource Management. For each control message, with the exception of the PACKET ACCESS/PACKET ACCESS ACK messages and the messages described in TSM 04.08 [11], the message type shall be a fixed number of bits from the beginning of the message.

PACKET ACCESS and PACKET ACCESS ACK messages have a different format with respect to the other packet control messages, as described in sub-clauses 11.3.1 and 11.4.1 respectively.

Table 22.a: RLC/MAC control messages

| Uplink TBF establishment messages: | Reference |
| --- | --- |
| Packet Access | 11.3.1 |
| Packet Access Ack | 11.4.1 |
| Packet Access Reject | 11.4.2 |
| Packet Resource Request | 11.3.8 |
| Packet Uplink Assignment | 11.4.13 |
| Downlink TBF establishment messages: | Reference |
| Packet Downlink Assignment | 11.4.4 |
| TBF release messages: | Reference |
| Packet TBF Release | 11.4.11 |
| Paging messages: | Reference |
| Packet Paging Request | 11.4.15 |
| RLC messages: | Reference |
| Packet Downlink Ack/Nack | 11.3.4 |
| Packet Uplink Ack/Nack | 11.4.12 |
| System information messages: | Reference |
| Packet System Information Type 13 | 11.4.10 |
| Packet PSI Status | 11.3.9 |
| Miscellaneous messages: | Reference |
| Packet Control Acknowledgement | 11.3.2 |
| Packet Cell Change Failure | 11.3.3 |
| Packet Cell Change Order | 11.4.3 |
| Packet Downlink Dummy Control Block | 11.4.5 |
| Packet Uplink Dummy Control Block | 11.3.5 |
| Packet Measurement Report | 11.3.6 |
| Packet Measurement Order | 11.4.6 |
| Packet Mobile TBF Status | 11.3.7 |
| Packet Resource Release | 11.4.7 |
| Packet Polling Request | 11.4.8 |
| Packet Power Control/Timing Control | 11.4.9 |
| Packet Resource Reconfigure | 11.4.14 |

Table 16.b: Messages for Radio Resource Management

| Uplink TBF establishment messages: | Reference |
| --- | --- |
| Immediate Assignment | TSM 04.08 [11] |
| Immediate Assignment Reject | TSM 04.08 [11] |
| Channel Request | TSM 04.08 [11] |
| Downlink TBF establishment messages: | Reference |
| Immediate Assignment | TSM 04.08 [11] |
| Immediate Assignment Reject | TSM 04.08 [11] |
| Paging messages: | Reference |
| Paging Request | TSM 04.08 [11] |

### 11.2.1	Message format

All RLC/MAC control messages, with the exception of those using the burst format as the PACKET CONTROL ACKNOWLEDGEMENT and the PACKET ACCESS/PACKET ACCESS ACK messages, follow the same non-standard format (see TS 24.007 [10]).

#### 11.2.1.1	Downlink RLC/MAC messages

Downlink RLC/MAC control messages, except the PACKET ACCESS ACK message, are received in RLC/MAC control block format. The different types of messages are distinguished by the MESSAGE_TYPE field.

< Downlink RLC/MAC control message > ::=

< MESSAGE_TYPE : bit (6) == 1 00001 >	< Packet Access Reject message content > |

< MESSAGE_TYPE : bit (6) == 0 00001 >	< Packet Cell Change Order message content > |

< MESSAGE_TYPE : bit (6) == 0 00010 >	< Packet Downlink Assignment message content > |

< MESSAGE_TYPE : bit (6) == 0 00011 >	< Packet Measurement Order message content > |

< MESSAGE_TYPE : bit (6) == 1 00010 >	< Packet Resource Release message content > |

< MESSAGE_TYPE : bit (6) == 1 00011 >	< Packet Downlink Dummy Control Block message content > |

< MESSAGE_TYPE : bit (6) == 0 00100 >	< Packet Polling Request message content > |

< MESSAGE_TYPE : bit (6) == 0 00101 >	< Packet Power Control/Timing Control message content > |

< MESSAGE_TYPE : bit (6) == 1 00100 >	< PSI13 message content >

< MESSAGE_TYPE : bit (6) == 0 00110 >	< Packet Resource Reconfigure message content > |

< MESSAGE_TYPE : bit (6) == 0 00111 >	< Packet TBF Release message content > |

< MESSAGE_TYPE : bit (6) == 0 01000 >	< Packet Uplink Ack/Nack message content > |

< MESSAGE_TYPE : bit (6) == 0 01001 >	< Packet Uplink Assignment message content > |

< MESSAGE_TYPE : bit (6) == 0 01010 >	< Packet Paging Request message content > |

< MESSAGE_TYPE : bit (6) == 1 00101 >	< Reserved > |

< MESSAGE_TYPE : bit (6) == 1 10001 >	< Reserved > |

< MESSAGE_TYPE : bit (6) == 1 10010 >	< Reserved > |

< MESSAGE_TYPE : bit (6) == 1 10011 >	< Reserved > |

< MESSAGE_TYPE : bit (6) == 1 10100 >	< Reserved > |

< MESSAGE_TYPE : bit (6) == 1 10101 >	< Reserved > |

< MESSAGE_TYPE : bit (6) == 1 10110 >	< Reserved > |

< MESSAGE_TYPE : bit (6) == 1 10111 >	< Reserved > |

! < Unknown message type : bit (*) = < no string > > ;

The PACKET ACCESS ACK message is defined in sub-clause 11.4.1.

#### 11.2.1.2	Uplink RLC/MAC messages

Uplink RLC/MAC control messages, except the PACKET ACCESS, are received in the RLC/MAC control block format. The different types of messages are distinguished by the MESSAGE_TYPE field.

< Uplink RLC/MAC control message > ::=

< MESSAGE_TYPE : bit (6) == 000000 >	< Packet Cell Change Failure message content > |

< MESSAGE_TYPE : bit (6) == 000001 >	< Packet Control Acknowledgement message content > |

< MESSAGE_TYPE : bit (6) == 000010 >	< Packet Downlink Ack/Nack message content > |

< MESSAGE_TYPE : bit (6) == 000011 >	< Packet Uplink Dummy Control Block message content > |

< MESSAGE_TYPE : bit (6) == 000100 >	< Packet Measurement Report message content > |

< MESSAGE_TYPE : bit (6) == 000101 >	< Packet Resource Request message content > |

< MESSAGE_TYPE : bit (6) == 000110 >	< Packet Mobile TBF Status message content > |

< MESSAGE_TYPE : bit (6) == 000111 >	< Packet PSI Status message content > ;

The PACKET ACCESS message is defined in sub-clause 11.3.1 while the PACKET CONTROLACKNOWLEGMENT is defined in sub-clause 11.3.2.

## 11.3	Packet Messages UE originated

### 11.3.1	Packet Access

This message is sent on the PACH by the user equipment to the network when accessing for the first time on an uplink P-PDCH.

The PACKET ACCESS message is formatted as N (see sub-clause 7.1.2.3) identical PACKET ACCESS bursts which are delivered on the assigned uplink P-PDPCHs following the Layer 1 coding rules for the single burst messages as described in TSM 05.03 [14].

The PACKET ACCESS burst corresponds to 32 bits having the following structure and content:

MAC Control Block

Signalling

Figure 8: PACKET ACESS burst structure

Table 23: Packet ACCESS burst information elements

< Packet Access burst content > ::=

<RLC_MODE: bit>

<Release_Version: bit (2)>

<Power_Level: bit (7)>

<Synchronisation_Shift : bit (11)>

<Spare bits (11)> ;

Table 18: Packet ACCESS burst information element details

RLC_MODE (bit)

The UE shall set this field according to the requested RLC mode for an uplink TBF, and shall copy the network requested RLC mode for a downlink TBF. The RLC_MODE coding is as follows:

- Acknowledged RLC mode

- Unacknowledged RLC mode

Release_Version (2 bits)

The UE shall set this field according to its release version. The Release_Version coding is as follows:

0 0		UE release version: this release

From 0 1 to 1 1	Reserved

Power_Level (7 bits)

This field contains the UE transmitted power level for sending the PACKET ACCESS message. This information can be used by the network to control its own power level when addressing the user equipment in the next frames. The Power_Level field is coded as the Power Level field defined in TSM 04.08 [11].

Synchronisation_Shift (11 bits)

This field contains the UE applied synchronisation shift for sending the PACKET ACCESS message. The Synchronisation_Shift field is coded as the Synchronisation Shift field defined in TSM 04.08 [11].

### 11.3.2	Packet Control Acknowledgement

This message is sent on the PACCH from the user equipment to the network. The message shall be sent in the indicated uplink radio block as specified by the corresponding valid RRBP field of a downlink RLC/MAC control block.

The RLC/MAC control block format is shown in table 19 and table 20.

Message type:	PACKET CONTROL ACKNOWLEDGEMENT

Direction:		user equipment to network

Table 19: PACKET CONTROL ACKNOWLEDGEMENT information elements

< Packet Control Acknowledgement message content > ::=

< TLLI : bit (32) >

< CTRL_ACK : bit (2) >

{ 0 | 1 < Requested_Measurement_report: < Requested_Measurement_report struct>> 

{ 0 | 1 < Channel Request Description : < Channel Request Description IE > > }

< padding bits > ;

< Requested_Measurement_report struct>::=

{ 0 | 1 < Synchronisation_Shift_report: Synchronisation_Shift_report struct > 

{ 0 | 1 < Power_Level_report: Power_Level_report struct > 

{ 0 | 1 < Interference_Level_report: Interference_Level_report struct > ;

< Synchronisation_Shift_report struct > :: =

< Time_Slot_State_Flag: bit (7) >

< Synchronisation_Shift: bit (11) > ;

< Power_Level_report struct > :: =

< Time_Slot_State_Flag: bit (7) >

< Power_Level: bit (7) > ;

< Interference_Level_report struct > :: =

< Time_Slot_State_Flag: bit (7) >

< ISCP: bit (7) >;

Table 20: PACKET CONTROL ACKNOWLEDGEMENT information element details

TLLI (32 bits)

This field contains the TLLI of the user equipment. This field is encoded as defined in clause 12.8.

CTRL_ACK (2 bits)

This field contains acknowledgement information for the group of RLC/MAC control blocks that make up an RLC/MAC control message. The UE shall set the CTRL_ACK field to indicate which segments of an RLC/MAC control message have been received by the time of transmission of the PACKET CONTROL ACKNOWLEDGEMENT message.

If the PACKET CONTROL ACKNOWLEDGEMENT message is being transmitted in response to a valid RRBP field received as part of an RLC/MAC block with Payload Type equal to '10', the CTRL_ACK field shall be set according to the following table:

bit

2 1

0 0	reserved - this value shall not be sent. If received it shall be interpreted as bit value '0 1'.

0 1	the UE received an RLC/MAC control block addressed to itself and with RBSN = 1, and did not receive an RLC/MAC control block with the same RTI value and RBSN = 0.

1 0	the UE received an RLC/MAC control block addressed to itself and with RBSN = 0, and did not receive an RLC/MAC control block with the same RTI value and RBSN = 1. This value is sent irrespective of the value of the FS bit.

1 1	the UE received two RLC/MAC blocks with the same RTI value, one with RBSN = 0 and the other with RBSN = 1.

If the PACKET CONTROL ACKNOWLEDGMENT message is being transmitted in response to a valid RRBP field received as part of an RLC/MAC block with Payload Type not equal to ‘10’, the CTRL_ACK field shall be set to the value ‘11’.

If the user equipment receives an RLC/MAC block with Payload Type equal to ‘10’ and an RLC/MAC block with Payload Type not equal to ‘10’ with different RRBP values such that they specify the same uplink block, the UE shall set the CTRL_ACK field according to the group of RLC/MAC control blocks that the RLC/MAC block with Payload Type ‘10’ belongs.

The UE shall report among the network requested measurements via the PACKET POLLING REQUEST message (see sub-clause 11.4.8) only those which have a valid value.

Channel Request Description

This field is present only when PACKET CONTROL ACKNOWLEDGEMENT message is sent to request an uplink TBF establishment after the release of a downlink TBF as reported in sub-clause 9.3.3.5. This information element is defined in sub-clause 12.5.

Time_Slot_State_Flag (7 bits)

This field contains the indication on which time slots the UE shall report the respective measure. Its coding is defined in sub-clasue 11.3.4.

Synchronisation_Shift (11 bits)

This field contains the UE applied Synchronisation Shift value on the indicated time slot in the Time_Slot_State_flag field such that: the first listed Synchronisation_Shift value shall refer to the highest flagged time slot number and the last listed Synchronsation_Shift value shall refer to the lowest flagged time slot number. This field is coded as the Synchronisation Shift field described in TSM 04.08 [11].

Power_Level (7 bits)

This field contains the UE transimtted Power Level on the indicated time slot in the Time_Slot_State_flag field such that: the first listed Power_Level value shall refer to the highest flagged time slot number and the last listed Power_Level value shall refer to the lowest flagged time slot number. This field is coded as the Power Level field described in TSM 04.08 [11].

ISCP (7 bits)

This field contains the UE measured interference level (see TSM 05.08 [15]) on the indicated time slot in the Time_Slot_State_flag field such that: the first listed ISCP value shall refer to the highest flagged time slot number and the last listed ISCP value shall refer to the lowest flagged time slot number. This field is coded as the UE measured ISCP field described in TSM 04.08 [11].

### 11.3.3	Packet Cell Change Failure

This message is sent on the PACCH from the user equipment to the network to indicate that a commanded cell change order has failed.

Message type:	PACKET CELL CHANGE FAILURE

Direction:		user equipment to network

Table 21: PACKET CELL CHANGE FAILURE information elements

< Packet Cell Change Failure message content > ::=

< TLLI : bit (32) >

< Target Cell description: <Cell Structure>>

< CAUSE : bit (4) >

< padding bits > ;

< Cell Structure>::=

< Mobile System: 000 >  -- If the target  mobile system is TD-SCDMA, otherwise

< CIC (7) > |

< Mobile System: 001 > -- If the target mobile system is GSM..

< BSIC (6) > 

< FBN: bit (4)>

< RFqN: bit (8)>;

Table 22: PACKET CELL CHANGE FAILURE information element details

TLLI (32 bits)

This field is defined in sub-clause 12.8.

Mobile System (3 bits)

This field specifies the mode (GSM or TD.SCDMA respectively) of the target cell on which the failure occurred. It is coded as the Mobile System value defined in TSM 04.08 [11].

FBN (4 bits)

This field identifies the Frequency Band of the target cell on which the failure occurred. It is coded as the FBN value defined in TSM 04.08 [11] for the respective Mobile System.

RFqN (8 bits)

This field identifies the carrier frequency in the specified Frequency Band (FBN) of the target cell on which failure occurred. It is coded as the RFqN value defined in TSM 04.08 [11] sub-clause 10.1.14, for the respective Mobile System.

BSIC (6 bits)

This field contains the BSIC of the BCCH frequency of the target GSM cell on which the failure occurred. It is coded as the BSIC value defined in TS 24.008 [21].

CIC (7 bits)

This field contains the Cell Identity Code (i.e. code group + basic midamble number) of the BCCH frequency of the target TD-SCDMA cell on which the failure occurred. It is coded as the CIC value defined in TSM 05.02 [13].

CAUSE (8 bits)

This field indicates the cause of the cell change order failure on the target cell.

Bit

4 3 2 1

0 0 0 0			Frequency not implemented

0 0 0 1			No response on target cell

0 0 1 0			Immediate Assign Reject or Packet Access Reject on target cell

0 0 1 1			On going CS connection

0 1 0 0			Anonymous Access

0 1 0 1			UE in GMM Standby state

0 1 1 0			Forced to the Standby state

All others		Reserved for future use

### 11.3.4	Packet Downlink Ack/Nack

This message is sent on the PACCH from the user equipment to the network to indicate the status of downlink RLC data blocks received and to report the channel quality of the downlink. The UE may optionally initiate an uplink TBF.

Message type:	PACKET DOWNLINK ACK/NACK

Direction:		user equipment to network

Table 23: Packet Downlink Ack/Nack information elements

< Packet Downlink Ack/Nack message content > ::=

< DOWNLINK_TFI : bit (5) >

< Ack/Nack Description : < Ack/Nack Description IE > >

{ 0 | 1 < Channel Request Description : < Channel Request Description IE > > }

< Channel Quality Report : < Channel Quality Report struct > >

< padding bits > ;

< Channel Quality Report struct > ::=

< C_VALUE : bit (7) >

{ 0 | 1 < P-PDCH_Average_TrCH_BLER: bit (6) > }

< SIGN_VAR : bit (6) >

< SIR_Level_Report: < SIR_Level Struct >> ;

< SIR_Level struct>::=

< Time_Slot_State_Flag: bit (7) >

{ < SIR_Level_TS: bit (6) > } * val (Time_Slot_State_Flag) ;-- This information is provided for each flagged time slot of the Time_Slot_State_Flag field

Table 24: Packet Downlink Ack/Nack information element details

DOWNLINK_TFI (5 bits)

This field contains the TFI of the UE downlink TBF. This field is defined in sub-clause 12.7.

Ack/Nack Description

This information element is defined in sub-clause 12.3.

Channel Request Description

This information element is defined in sub-clause 12.5.

C_VALUE (7 bits)

This field contains the value of the C parameter calculated by the user equipment (see TSM 05.08 [15]). It is encoded as the binary representation of the UE measured RSCP value as defined in TSM 05.08 [15].

P-PDCH_Average_TrCH_BLER (6 bits)

This field contains the average TrCH_BLER parameter calculated by the user equipment over the received blocks (see TSM 05.08 [15]) from all the assigned downlink P-PDCH. If no P-PDCH is assigned to the user equipment, this measure shall not be reported. It is encoded as the binary representation of the UE measured TrCH_BLER as defined in TSM 04.08 [11].

SIGN_VAR (6 bits)

This field contains the signal variance parameter SIGN_VAR calculated by the user equipment (see TSM 05.08 [15]).

Bit

6 5 4 3 2 1

0 0 0 0 0 0		0dB2 to 0.25 dB2

0 0 0 0 0 1		>0.25 dB2 to 0.50 dB2

0 0 0 0 1 0		>0.50 dB2 to 0.75 dB2

:   :   : 			      :                 :

1 1 1 1 1 0		>15.50 dB2 to 15.75 dB2

1 1 1 1 1 1		>15.75 dB2

Time_Slot_State_Flag (7 bits)

This field contains the indication on which time slots the UE shall report the measured SIR. It is encoded as a bit map with respect to the 7 time slots of the radio frame such that:

bit 7 corresponds to time slot 0;

bit 6 corresponds to time slot 1;

bit 5 corresponds to time slot 2;

bit 4 corresponds to time slot 3;

bit 3 corresponds to time slot 4;

bit 2 corresponds to time slot 5;

bit 1 corresponds to time slot 6 (see TSM 05.02 [13] for time slot numbering).

A bit in the Time_Slot_State_Flag field set to value “1” indicates that the UE shall report the measured SIR on the corresponding time slot.

SIR_Level_TS (6 bits)

This field contains the UE measured SIR parameter (see TSM 05.08 [15]) on the time slot which has been flagged in the correspondent Time_Slot_State_Flag field. The first reported SIR shall refer to the first flagged time slot, the last reported SIR shall refer to the last flagged time slot, where first time slot corresponds to the flagged MSB of the Time_Slot_State_Flag field.

SIR_Level_TS is encoded as the UE measured SIR, as defined in TSM 04.08 [11].

### 11.3.5	Packet Uplink Dummy Control Block

This message is sent on the PACCH from the user equipment to the network when the UE has no other block to transmit.

Message type:	PACKET UPLINK DUMMY CONTROL BLOCK

Direction:		user equipment to network

Table 25: Packet UPLINK DUMMY CONTROL BLOCK information elements

< Packet Uplink Dummy Control Block message content > ::=

< TLLI : bit (32) >

< padding bits > ;

Table 26: Packet UPLINK DUMMY CONTROL BLOCK information element details

TLLI (32 bits)

This field contains the TLLI of the user equipment. This field is encoded as defined in sub-clause 12.8.

### 11.3.6	Packet Measurement Report

This message is sent either on the PACCH if in packet transfer mode or on an assigned block on a PDCH, from the user equipment to the network to report measurement results. The message may contain measurement results from either the Network Control measurements or from the Extended measurements, but not both simultaneously, as well as measurement results from a measurement on demand, if requested by the network, and/or event triggered measures, if any.

More than one message may be required depending on the number of measurements to report.

Message type:	PACKET MEASUREMENT REPORT

Direction:		user equipment to network

Table 27: PACKET MEASUREMENT REPORT message content

< Packet Measurement Report message content > ::=

< TLLI : bit (32) >

{ 0 | 1 <Measurement on Demand: < Measurement on Demand Command struct > > }

{ 0 | 1 <Measurement Event Triggered: < Measurement Event Triggered struct > > }

{ 0 < NC Measurement Report : < NC Measurement Report struct > >

| 1 < EXT Measurement Report : < EXT Measurement Report struct > > }

< padding bits > ;

< NC Measurement Report struct > ::=

< NC_MODE : bit (1) >

< RSCP_SERVING_CELL : bit (7) >

{ 0 | 1 < TrCH_BLER: bit (6) > }

{ 0 | 1 < SIR_Level: bit (6) > }

{ 0 | 1 < NUMBER_OF_TD-SCDMA_NC_MEASUREMENTS: bit (3) > }

{	< FREQUENCY_N : bit (6) >

< Basic_Midamble_Number_N : bit (2) >

< RSCP_N : bit (7) > } * (val(NUMBER_OF_TD-SCDMA_NC_MEASUREMENTS))

{ 0 | 1 < NUMBER_OF_GSM_NC_MEASUREMENTS: bit (3) > }

{	< FREQUENCY_N :bit (6) >

< BSIC_N : bit (6) >

< RXLEV_N : bit (6) > } * (val(NUMBER_OF_GSM_NC_MEASUREMENTS))

{ 0 -- Escape message

! < message escape : 1 bit (*) = < no string > > } ;

< EXT Measurement Report struct > ::=

< Extended_Measurement_Change_Mark_Used: bit (2) >

{0|1 <Intra-Cell Extended Measurement report: <Intra-Cell Extended measurement report struct>>}

{0|1 <Inter-Cell Extended Measurement report: <Inter-Cell Extended measurement report struct >>}

{ 0 | 1 < Extension bits: Extension bits IE > } ; -- Possible future extensions; see sub-clause 12.2

Table 28: PACKET MEASUREMENT REPORT information element details

TLLI (32 bits)

This field contains the TLLI of the user equipment. This field is encoded as defined in sub-clause 12.8.

The Measurement on Demand Command struct reports the result of a measurement on demand which may have been requested by the network. The coding rules are defined in TSM 04.08 [11].

The Measurements Event Triggered struct reports measures which may have been triggered by the relevant events. The coding rules are defined in TSM 04.08 [11].

The NC Measurement report contains a list of measurements performed by the UE towards the serving cell and the neighbour cells, according to the indications received by the network via the NC Measurement parameters. The neighbour cell list referred to by the UE for this report is based on the list broadcast on the BCCH, eventually corrected by the additions/deletions defined by the latest PACKET CELL CHANGE ORDER or PACKET MEASUREMENT ORDER message received by the network.

As a PACKET MEASUREMENT ORDER message can be repeated in a number of instances, each message instance can contain a sub-list of frequency parameters/neighbour cells. The sub-lists shall then be concatenated in ascending message instance order and added to the end of the BCCH list, so providing the resulting neighbour cell list to be referred to. Each neighbour cell position in the resulting list shall then be assigned an ascending index used for measurement reports. If the same cell is defined more than once in the resulting list, each occurrence will get an index, but measurements shall only be performed and reported for the last added position.

FREQUENCY_N (6 bits)

This field indicates the cell upon which the reported measurement was made; it acts as an index into the resulting neighbour cell list such that:

FREQUENCY_N= 0 refers to the first cell and FREQUENCY_N = n refers to the last cell in the BCCH list. FREQUENCY_N = n+1 refers then to the first neighbour cell in the‘Add Frequency list struct’ in the first message instance and N = n+m to the last frequency in the ‘Add Frequency list struct’ in the last message instance of the PACKET MEASUREMENT ORDER message or of the PACKET CELL CHANGE ORDER message. Deletion of cells in a measurement order does not affect the index.

NC_MODE (1 bit)

This field indicates if the UE was in mode NC1 or NC2 when sending the measurement report.

- UE in mode NC1

- UE in mode NC2

RSCP_SERVING_CELL (7 bits)

This field contains the value of the RSCP parameter for the serving cell calculated by the user equipment (see TSM 05.08 [15]). This field is encoded as the binary representation of the UE measured RSCP parameter value defined in TSM 04.08 [11]. The reference channel shall be the P-PDPCH if present, or the S-PDPCHs if transmitted at constant power level, otherwise the BCCH.

TrCH_BLER (6 bits)

When in packet transfer mode, this field contains the average TrCH_BLER parameter calculated by the user equipment over the received blocks (see TSM 05.08 [15]) from all the assigned downlink P-PDCHs if any; otherwise this parameter shall not be reported.

When in packet idle mode, this field contains the average TrCH_BLER parameter calculated by the user equipment over the received blocks (see TSM 05.08 [15]) from the CCPCH carrying its paging group if a valid value is available; otherwise this parameter shall not be reported.

The TrCH_BLER field is encoded as the binary representation of the UE measured TrCH_BLER as defined in TSM 04.08 [11].

SIR_Level (6 bits)

When in packet transfer mode, this field contains the UE measured SIR parameter (see TSM 05.08 [15]) averaged over all active downlink time slots.

When in packet idle mode, this field contains the UE measured SIR parameter (see TSM 05.08 [15]) from the CCPCH carrying its paging group if a valid value is available, otherwise this parameter shall not be reported.

The SIR_Level field is encoded as the UE measured SIR, as defined in TSM 04.08 [11].

Basic_Midamble_Number_N (2 bits)

This field indicates the basic midamble number of the TD-SCDMA frequency upon which the measurement was made. Its coding is defined in TSM 04.08 [11].

BSIC_N (6 bits)

This field indicates the BSIC of the GSM frequency upon which the measurement was made. The field is encoded as the BSIC value defined in TS 24.008 [21].

RSCP_N (7 bits)

This field indicates the UE measured RSCP of the TD-SCDMA frequency upon which the measurement was made (see TSM 05.08 [15]). It is encoded as the UE RSCP value defined in TSM 04.08 [11].

RXLEV_N (6 bits)

This field indicates the UE measured RXLEV of the GSM frequency upon which the measurement was made (see TSM 05.08 [15]). It is encoded as the RXLEV value defined in TSM 04.08 [11].

The EXT Measurement report contains a list of extended measurements performed by the UE towards the serving and/or the neighbour cells according to the indications received by the network via the Extended Measurement parameters.

The Extended Measurement parameters can be repeated in a sequence of PACKET MEASUREMENT ORDER message instances. The extended measurements parameters of all message instances shall then be concatenated into a resulting Extended Measurement parameters list in order of appearance within a message instance, and then in order of ascending message instances.

Extended_Measurement_Change_Mark_Used (2 bits)

This field reports the same value assumed by the PACKET MEASUREMENT ORDER message (see sub-clause 11.4.6) which has been used by the UE to fill in the Extended Measurement report.

The Intra-Cell Extended Measurement report contains the results of the extended measurements performed by the UE towards the serving cell. The coding rules are defined in TSM 04.08 [11].

The Inter-Cell Extended Measurement report contains the results of the extended measurements performed by the UE towards the neighbour cells. The coding rules are defined in TSM 04.08 [11].

### 11.3.7	Packet Mobile TBF Status

This message is sent from the user equipment to the network on the uplink PACCH to indicate erroneous messages have been received relating to either a downlink or an uplink TBF.

Message type:	PACKET MOBILE TBF STATUS

Direction:		user equipment to network

Table 29: Packet MOBILE TBF STATUS information elements

< Packet Mobile TBF Status message content > ::=

< GLOBAL TFI : < Global TFI IE > >

< TBF_CAUSE : bit (3) >

{ 0 | 1	< STATUS_MESSAGE_TYPE : bit (6) > }

< padding bits > ;

Table 30: Packet MOBILE TBF STATUS information element details

Global TFI

This information element contains the TFI of the UE’s downlink TBF or uplink TBF. This field is defined in sub-clause 12.7.

TBF_CAUSE (3 bits)

The TBF_CAUSE field indicates the error cause value of the current TBF. This field is encoded according to the following table:

bit

2 1

0 0 0		Normal event;

0 0 1		Status, unspecified;

0 1 0		Syntactically incorrect message, non-distribution part error;

0 1 1		Syntactically incorrect message, message escape;

1 0 0		Message not compatible with current protocol state.

All other values are reserved and may be interpreted "Status, unspecified".

STATUS_MESSAGE_TYPE (6 bits)

The STATUS_MESSAGE_TYPE field, if present, is the binary representation of the message type of the downlink RLC/MAC control message that caused the status condition. Message type values are defined in sub-clause 11.2.1.1.

### Packet Resource Request

This message is sent on the PACCH by the user equipment to the network to request a change in the uplink assigned resources.

Message type:	PACKET RESOURCE REQUEST

Direction:		user equipment to network

Table 31: Packet RESOURCE REQUEST information elements

< Packet Resource Request message content > ::=

{ 0 | 1 < ACCESS_TYPE : bit (2) > }

{ 0 < Global TFI : < Global TFI IE > >

| 1 < TLLI : < TLLI IE > > }

{ 0 | 1 <  Radio Access Capability : <  Radio Access Capability IE > > }

< Channel Request Description : < Channel Request Description IE > >

{ 0 | 1 < CHANGE_MARK : bit (2) > }

< C_VALUE : bit (7) >

{ 0 | 1 < SIGN_VAR : bit (6) >}

< padding bits > ;

Table 32: Packet RESOURCE REQUEST information element details

Global TFI

This information element contains the TFI of the UE's uplink TBF, if available, or the TFI of the UE's downlink TBF. If no TFI is available, this field is omitted. This field is defined in sub-clause 12.7.

ACCESS_TYPE (2 bits)

This field indicates the reason for requesting the access. It shall be included only in response to a single block assignment.

Bit

2 1

0 0		Two Phase Access Request

0 1		Page Response

1 0		Cell Update

1 1		Mobility Management procedure

TLLI

This information element is defined in sub-clause 12.8.

Radio Access Capability

This information element is coded as defined by the UE RA capability value part defined in the  Radio Access Capability IE defined in TSM 04.08 [11]. This information element shall always be sent with the first PACKET RESOURCE REQUEST message for a non concurrent uplink TBF. When this information element is sent, all spare bits shall be suppressed by the transmitter.

Channel Request Description

This information element is defined in sub-clause 12.5.

CHANGE_MARK (2 bits)

This field contains the SI13_CHANGE_MARK value stored by the UE. If the UE does not have a valid SI13 change mark for the current cell, it shall omit this field. The coding of this field is network dependent.

C_VALUE (7 bits)

This field contains the value of the C parameter calculated by the user equipment (see TSM 05.08 [15]). It is encoded as the binary representation of the UE measured RSCP value as defined in TSM 05.08 [15].

SIGN_VAR (6 bits)

This field contains the signal variance parameter SIGN_VAR calculated by the user equipment (see TSM 05.08 [15]). ). This field is not present for TBF establishment using two phase access.

Bit

6 5 4 3 2 1

0 0 0 0 0 0	0dB2 to 0.25 dB2

0 0 0 0 0 1	>0.25 dB2 to 0.50 dB2

0 0 0 0 1 0	>0.50 dB2 to 0.75 dB2

:   :   : 			      :                 :

1 1 1 1 1 0	>15.50 dB2 to 15.75 dB2

1 1 1 1 1 1	>15.75 dB2

### Packet PSI Status

This message is sent on the PACCH from the user equipment to the network to indicate which PSI messages the UE has received.

Message type:	PACKET PSI STATUS

Direction:		user equipment to network

Table 33: Packet PSI STATUS information elements

< Packet PSI Status message content > ::=

< GLOBAL_TFI : < Global TFI IE > >

< Received PSI Message List : < PSI Message List struct > >

< padding bits > ;

< PSI Message List struct > ::=

{ 1	< MESSAGE_TYPE : bit (6) >

< PSIX_CHANGE_MARK : bit (2) >

{ 0 | 1 < PSIX_COUNT : bit (4) >

< Instance bitmap : bit (val(PSIX_COUNT) + 1) > } } ** 0

< ADDITIONAL_MSG_TYPE : bit > ;

Table 34: Packet PSI STATUS information element details

Global TFI

This information element identifies the TBF where this message is sent. The coding of this information element is defined in sub-clause 12.7.

Received PSI Message List (construction)

This construction contains a list of correctly received PSI messages. In this version of the protocol, no message type is foreseen in this list.

The sender of this message may indicate as many messages in this list as can be fit into the message. Messages are listed by message type in descending order of priority. If there are more PSI messages than can be indicated in this list, the presence of additional message type(s) shall be indicated at the end of the list.

If the sender of this message has received a PSI message which is part of a consistent set of PSI messages (see sub-clause 5.5.2.1.4), the Instance Bitmap may indicate which instances of this message type have been received.

MESSAGE_TYPE (6 bit field)

This field is the binary representation of the message type (see sub-clause 11.2).

PSIX_CHANGE_MARK (2 bits)

This field is the binary representation of the PSI change mark parameter received for a certain PSI message type.

Range: 0 to 3.

PSIX_COUNT (4 bits)

This field is the binary representation of the PSI count parameter received for a certain PSI message type. This field indicates the length of the corresponding Instance bitmap field and shall be provided only if the corresponding Instance bitmap field is provided in the message.

Range: 0 to 7 or 0 to 15, depending on message type.

Instance bitmap (1 – 16 bit field)

This field is a bitmap indicating which instances of a certain message type are received within a consistent set of PSI messages. This field shall be included when a sub-set of these messages has been received. This field shall not be included when the complete set of these messages has been received.

The most significant bit of this bitmap (bit N) refers to the message instance with the PSI index parameter = N–1, where N is the number of instances of the particular message type (PSI count + 1). The least significant bit of this bitmap (bit 1) refers to the message instance with the PSI index parameter = 0. Each bit position is coded:

- Message instance is not received;

1	Message instance is received.

ADDITIONAL_MSG_TYPE (1 bit)

- No additional message type is present.

1	Additional message type(s) are present.

## 11.4	Packet Messages Network originated

### 11.4.1	Packet Access Ack

This message is sent on the PAKCH by the network to the user equipment to acknowledge the detection of a PACKET ACCESS message.

The PACKET ACCESS ACK message is formatted as N (see sub-clause 7.1.2.3) identical PACKET ACCESS ACK bursts which are delivered on an assigned downlink P-PDPCH(s) following the Layer 1 coding rules for the single burst messages as described in TSM 05.03 [14].

The PACKET ACCESS ACK burst corresponds to 32 bits having the following structure and content:

MAC Control Block

Signalling

Figure 9: PACKET ACESS ACK burst structure

Table 35: Packet ACCESS ACK burst information elements

< Packet Access Ack message content > ::=

<Synchronisation_Shift_Adjustment: bit (7)>

< Transmit Power Level Adjustment : bit (4) >

<Spare bits (21)> ;

Table 36: Packet ACCESS ACK burst information element details

Synchronisation_Shift_Adjustment (7 bit field)

With this field the network can command a timing adjustment to the UE, based on the detected Packet Access burst. The Synchronisaiton_Shift_Adjustment coding is defined in TSM 04.08[11].

Transmit power level adjustment (4 bits)

This field contains a transmit power level adjustment indication to the UE. It is encoded as the RPL_P-CCPCH field described in TSM 04.08 [11].

### 11.4.2	Packet Access Reject

This message is sent on the PACCH by the network to the user equipment to indicate that the network has rejected the UE access request. This message may contain fields addressing more than one user equipment.

Message type:	PACKET ACCESS REJECT

Direction:		network to user equipment

Classification:	distribution message

Table 37: Packet ACCESS REJECT information elements

< Packet Access Reject message content > ::=

< PAGE_MODE : bit (2) >

< Reject : < Reject struct > >

{ { 1 < Additional Reject: < Reject struct > > } ** 0

< padding bits > } //		-- truncation at end of message allowed, bits ‘0’ assumed

! < Distribution part error: bit (*) = < no string > >;

< Reject struct > ::=

{ 0	< TLLI : bit (32) >

| 1	< Global TFI : < Global TFI IE > > }

{ 0 | 1	< WAIT_INDICATION : bit (8) >

< WAIT _INDICATION_SIZE : bit (1) > }

! < Ignore : bit (*) = <no string> > ;

Table 38: Packet ACCESS REJECT information element details

PAGE_MODE (2 bits)

This field is defined in clause 12.10.

Reject struct
The user equipment shall only accept the first Reject struct addressed to it and ignore all other Reject structs.

TLLI (32 bits)

This information field shall be included if the PACKET ACCESS REJECT message is sent in response to a Channel Request Description IE contained in a PACKET DOWNLINK ACK/NACK message. This information field is defined in sub-clause 12.8.

Global TFI

This information element contains the TFI of the UE's downlink TBF or uplink TBF. This field is defined in sub-clause 12.7.

WAIT_INDICATION (8 bits)

The Wait Indication field indicates the time the user equipment shall wait before attempting another channel request. If this field is present the cause value is 'Wait'. If this field is not present, the cause value is 'Read System Information'. This field is coded as the binary representation of the T3172 timeout value in units of 20 milliseconds or in units of seconds. The units are indicated in the WAIT_INDICATION_SIZE field. Range 0 to 255.

WAIT_INDICATION_SIZE (bit)

This field indicates the units of the WAIT_INDICATION field.

0	the WAIT_INDICATION field is coded in units of seconds

1	the WAIT_INDICATION field is coded in units of 20 milliseconds

### 11.4.3	Packet Cell Change Order

This message is sent on the PACCH by the network to the user equipment to command the UE to leave the current cell and change to a new cell.

Message type:	PACKET CELL CHANGE ORDER

Direction:		network to user equipment

Classification:	non-distribution message

Table 39: PACKET CELL CHANGE ORDER information elements

< Packet Cell Change Order message content > ::=

< PAGE_MODE : bit (2) >

{	{ 0	< Global TFI : < Global TFI IE > >

| 10	< TLLI : bit (32) > }

{ 0		-- Message escape

{	< IMMEDIATE_REL: bit >

< Target Cell Description: <Cell Structure>>

< NC Measurement Parameters: < NC Measurement Parameters struct>>

< padding bits >

! < Non-distribution part error : bit (*) = < no string > > }

! < Message escape : 1 bit (*) = <no string> > }

! < Address information part error : bit (*) = < no string > > }

! < Distribution part error : bit (*) = < no string > >;

< NC Measurement Parameters struct > ::=

< NETWORK_CONTROL_ORDER : bit (2) >

{ 0 | 1 < NC_NON_DRX_PERIOD : bit (3) >

< NC_REPORTING_PERIOD_I : bit (3) > }

{  0 | 1< NC_REPORTING_PERIOD_T : bit (3) > }

{ 0 | 1 < NC_FREQUENCY_LIST : NC Frequency list struct >} ;

< NC Frequency list struct > ::=

{ 0 | 1	 < NR_OF_REMOVED_FREQ : bit (5) >

{ < REMOVED_FREQ_INDEX : bit (6) > } * (1 + val(NR_OF_REMOVED_FREQ)) }

{ 1 < List of added Frequency : < Add Frequency list struct > >} ** 0;

< Add Frequency list struct > ::=

< NR_OF_FREQUENCIES: bit (5) >

{ < Added Cell Description: < Cell Structure >>

{ 0 | 1	< Cell selection params : < Cell Selection struct > > } } * (val (NR_OF_FREQUENCIES));

< Cell Selection struct > ::=

< EXC_ACC : bit >

< CELL_BAR_ACCESS_2 : bit (1) >

< SAME_RA_AS_SERVING_CELL : bit (1) >

{ 0 | 1	< (GPRS_)RXLEV_ACCESS_MIN : bit (6) >

< (GPRS_)UE_TXPWR_MAX_CCH : bit (7/5)>}..--7 or 5 bits for TD-SCDMA or GSM cell

respectively

{ 0 | 1 < (GPRS_)TEMPORARY_OFFSET : bit (3) >

< (GPRS_)PENALTY_TIME : bit (5) > }

{ 0 | 1	< (GPRS_)CELL_RESELECT_OFFSET : bit (5) > }

{ 0 | 1	< HCS params : < HCS struct > > }

{ 0 | 1 < SI13_LOCATION : < SI13_LOCATION struct > > } ;

< SI13_LOCATION struct > ::=

{ 0 	-- Message escape

< SI13_LOCATION : bit (1) >

! < Message escape : 1 bit (*) = <no string> > ;

< HCS struct > ::=

< (GPRS_)PRIORITY_CLASS : bit (3) >

< (GPRS_)HCS_THR : bit (5) > ;

Table 40: PACKET CELL CHANGE ORDER information element details

PAGE_MODE (2 bits)

This field is defined in sub-clause 12.10.

Global TFI (32 bits)

This information element contains the TFI of the user equipment’s downlink TBF or uplink TBF. This field is defined in sub-clause 12.7.

TLLI (32 bits)

This field is defined in sub-clause 12.8.

IMMEDIATE_REL (bit)

This field indicates whether the UE shall immediately move to the target cell and interrupt its on-going TBF in the serving cell, or it shall move to the target cell by applying the same procedure as for an autonomous cell reselection. This field is coded according to the following table:

0	Same procedure as for an autonomous cell reselection

1	Immediate release of the on-going TBF.

Cell Structure (18 or 19 bits)

This structure contains the BCCH frequency and the identity of the new GSM (18 bits) or TD-SCDMA (19 bits )cell. Its coding is defined in sub-clause 11.3.3.

NC_Measurement_Parameters struct

This structure contains the NETWORK_CONTROL_ORDER and the optional parameters NC_NON_DRX_PERIOD, NC_REPORTING_PERIOD_I, NC_REPORTING_PERIOD_T and the NC_FREQUENCY LIST. These parameters shall apply in the target cell (see sub-clause 5.6.1). When the NETWORK_CONTROL_ORDER field is set to either NC1 or NC2, at least one of the two immediately following optional parameters shall be reported; the missing one being interpreted as if no measurement report is requested for the relevant mode, respectively packet idle mode (I) or packet transfer mode (T).

NETWORK_CONTROL_ORDER (2 bit field)

NC_NON_DRX_PERIOD (3 bits)

NC_REPORTING_PERIOD_I (3 bits)

NC_REPORTING_PERIOD_T (3 bits)

For detailed element definitions see sub-clauses 12.12, 12.13, 12.14.

Parameters NR_OF_REMOVED_FREQ, REMOVED_FREQ_INDEX and NR_OF_FREQUENCIES are defined in sub-clause 11.4.6.

Cell_selection_ struct

This structure defines the mandatory and optional parameters which identify the new added cells. If optional fields are omitted, the values for these parameters are assumed the same as for the preceding cell unless otherwise specified for the parameter; the first neighbour cell defined in the list uses as its default parameter values those defined for the serving cell.

EXC_ACC (1 bit)

The EXC_ACC field is used by the network to prevent UEs without exclusive access rights from camping on the cell. The usage of EXC_ ACC is described in TSM 03.22 [22]. The coding of EXC_ ACC is described in TSM 04.08 [11].

CELL_BAR_ACCESS_2 (1 bit)

This field indicates the status for cell reselection, as described in the following:
0		Status for cell reselection is set to normal;
1		Status for cell reselection is set to barred.

SAME_RA_AS_SERVING_CELL (1 bit)

The same RA as serving cell field contains one bit, set to
0	if the cell is in a Routeing Area different from the serving cell, or
1	if the cell is in the same Routeing Area as the serving cell.

(GPRS_)RXLEV_ACCESS_MIN (6 bits)

The (GPRS_)RXLEV_ACCESS_MIN field is coded respectively as the RXLEV_ACCESS_MIN field described in TSM 04.08 [11] for a TD-SCDMA cell and as the GPRS_ RXLEV_ACCESS_MIN field described in TS 24.008 [21] for a GSM cell.

(GPRS_)UE_TXPWR_MAX_CCH (7/5 bits)

The (GPRS_)UE_TXPWR_MAX_CCH field is coded respectively as the UE_TXPWR_MAX_CCH field (7 bits) described in TSM 04.08 [11] for a TD-SCDMA cell and as the GPRS_UE_TXPWR_MAX_CCH field (5 bits) described in TS 24.008 [21] for a GSM  cell.

(GPRS_)TEMPORARY_OFFSET (3 bits)

This field is coded respectively as the binary representation of the TEMPORARY_OFFSET defined in TSM 04.08 [11] for a TD-SCDMA cell and as the GPRS_ TEMPORARY_OFFSET defined in TS 24.008 [21] for a GSM cell. It is used by the user equipment as part of its calculation of C2 for the cell reselection process as respectively described in TSM 05.08 [15] and GSM 05.08[24].

(GPRS_)PENALTY_TIME (5 bits)

The (GPRS_)PENALTY_TIME is coded respectively as the binary representation of the PENALTY_TIME described in TSM 04.08[11] for a TD-SCDMA cell and as GPRS_ PENALTY_TIME  described in GSM 04.60 [25] for a GSM cell.

(GPRS_)CELL_RESELECT_OFFSET (5 bits)

The (GPRS_)CELL_RESELECT_OFFSET is used by the user equipment to apply a positive or negative offset to the value of C2 as defined respectively in TSM 05.08 [15] for a TD-SCDMA cell and in GSM 04.60 [25] for a GSM cell.

(GPRS_)PRIORITY_CLASS (3 bits)

The (GPRS_)PRIORITY_CLASS field contains the binary representation of the HCS priority as defined in GSM 04.60 [25] for a GSM cell while PRIORITY_CLASS field for a TD-SCDMA cell is still TBD.

(GPRS_)HCS_THR (5 bits)

The (GPRS_)HCS_THR field is the HCS signal strength threshold as defined in GSM 04.60 [25] for a GSM cell while HCS_THR field for a TD-SCDMA cell is still TBD.NC_FREQUENCY_LIST

For detailed element definitions see the Packet Measurement Order message.

SI13_LOCATION construction

The optional SI13_LOCATION struct may indicate the block position of SI13 message.

The SI13_LOCATION field, if present, indicates the logical channel where SYSTEM INFORMATIION TYPE 13 is broadcast (see TSM 05.02 [13]) according to the following rules:

- SI13 message is broadcast on the first BCCH block of the P-CCPCH;

- SI13 message is broadcast on the second BCCH block of the P-CCPCH.

### 11.4.4	Packet Downlink Assignment

This message is sent on the PACCH by the network to the user equipment to assign downlink resources to the UE when a downlink TBF is not yet established.

In case downlink P-PDPCH resources are assigned, and no P-PDCH is already established between the user equipment and the network, at least one uplink P-PDPCH resource shall be assigned as well in order to fulfil the duplex operative mode of a P-PDCH (see sub-clause 4.5.1).

The PACKET DOWNLINK ASSIGNMENT message defines additional resources with respect to the ones already established. The new assigned resources shall therefore use the same frequency in the same frequency band as the already established ones. When no resources are assigned, this means that the current ones are reconfirmed.

Message type:	PACKET DOWNLINK ASSIGNMENT

Direction:		network to user equipment

Classification:	non-distribution message

Table 41: Packet Downlink ASSIGNMENT information elements

< Packet Downlink Assignment message content > ::=

< PAGE_MODE : bit (2) >

{	{ 0	< Global TFI : < Global TFI IE > >

| 10	< TLLI : bit (32) > }

{ 0	-- Message escape

{	< MAC_MODE: bit (2) >

{	< RLC_MODE : bit (1) >

< CONTROL_ACK : bit (1) >

{ 0 | 1 < RESOURCE_ALLOCATION: <Downlink Packet Channel struct >> }

{	{ 0 | 1 < DOWNLINK_TFI_ASSIGNMENT : bit (5) > }

{ 0 | 1 < TBF Starting Time: < Starting Frame Number Description IE>> }

{ 0 | 1 < Access Priority Parameters: bit (13) > }

{ 0 | 1 < Measurement Mapping : < Measurement Mapping struct > > }

< padding bits > } //		-- truncation at end of message allowed, bits ‘0’ assumed

! < Non-distribution part error : bit (*) = < no string > > }

! < Message escape : 1 bit (*) = <no string> > }

! < Address information part error : bit (*) = < no string > > }

! < Distribution part error : bit (*) = < no string > > };

< Downlink Packet Channel struct > ::=

{ 0 | 1 < Power_Reference : bit (4) > }

{ 0 | 1 < Pilot channel set description: < Pilot channel set struct > > }

{ 0 | 1 < Shared channel set description: < Shared channel set struct > > } ;

< Pilot channel set struct > :: =

< Channel Mode: bit (5) >

< Downlink: < Resource_Units struct > >

{ 0 | 1 < Uplink: < Resource_Units struct > > } ;

< Shared channel set struct > :: =

< Downlink: < Resource_Units struct > > ;

< Resource_Units struct > :: =

< Time Slot Number: bit (3) >

< CS_Mode_Type: bit (2) >

< Code_Values: < Code_Values struct > >

 0 | 1 < Resource_Units struct > ;

< Code_Values struct> :: =

< Midamble_Shift: bit (4) >

< SC: bit (5) >

 0 | 1 < Code_Values > ;

< Measurement Mapping struct > ::=

< Measurement Starting Time : < Starting Frame Number Description IE > >

< MEASUREMENT_INTERVAL : bit (5) >;

Table 42: PACKET Downlink ASSIGNMENT information element details

PAGE_MODE (2 bits)

This field is defined in sub-clause 12.10.

Global TFI

This information element contains the TFI of the UE downlink TBF or uplink TBF. This field is defined in sub-clause 12.7.

TLLI (32 bits)

This field is defined in sub-clause 12.8.

MAC_MODE (2 bits)

This information field indicates the medium access method to be used during the TBF.

Bit

2 1

0 0	Dynamic Allocation

0 1	Reserved

1 0	Fixed and/or Pilot resource allocation, no half duplex mode

1 1	Fixed and/or Pilot resource allocation, half duplex mode

RLC_MODE (1 bit)

This field indicates the RLC mode of the requested TBF.

- RLC acknowledged mode

- RLC unacknowledged mode

CONTROL_ACK (1 bit)

This field shall be set to '1' if the network establishes a new downlink TBF for the UE whose timer T3192 is running. Otherwise this field shall be set to '0'.

RESOURCE_ALLOCATION structure

This field defines the downlink assigned resources, discriminating between pilot resources and shared resources respectively.

Power_Reference (4 bits)

If transmitted, this field signals to the UE that it can use any of the assigned shared downlink packet resources as reference for power measurements, being transmitted at constant power level. In case this field is not present, the UE shall use for the purpose either the BCCH or the DwPTS.

The Power_Reference field is normalised to Spreading Factor 16 and encodes the power difference with respect to the DwPTS physical channel. The Power_Reference field is coded as the RPL_P-CCPCH field described in TSM 04.08 [11].

Channel_Mode (5 bits)

This field defines the pilot channel configuration according to the following rules:

bit

5 4 3 2 1

0 0 0 0 0		all pilot resources (uplink and downlink) are assigned to the UE at each radio frame;

0 0 0 1 T	all pilot resources (uplink and downlink) are assigned to the UE each 2 radio frames;

0 0 1 T T 	all pilot resources (uplink and downlink) are assigned to the UE each 3 radio frames;

0.1.0.T .T	all pilot resources (uplink and downlink) are assigned to the UE each 4 radio frames;

1 1 0 T T	Reserved;

1 1 1 T T	Reserved.

Where the parameter “T” indicates the sub-channel number binary coded (see also TSM 04.08 [11]).

For this release, only the first channel mode will be supported. The multi-frame distribution of the different pilot sub-channels is described in TSM 05.02 [13].

Time Slot Number (3 bits)

This field defines the time slot of the assigned resources. Its coding is defined in TSM 04.08 [11].

CS_MODE_Type (2 bits)

This field defines the Coding Scheme (CS) mode type according to the following rules:

bit

2 .1

0 0	The coded block will use the payload of one basic resource unit, that is one resource unit at Spreading Factor (SF) 16 (see TSM 05.03 [14]);

0 1	The coded block will use the payload of two basic resource units (see TSM 05.03 [13]);

1 0	Reserved;

1 1	Reserved.

The CS_MODE_Type shall be specified for each allocated resource.

For coding scheme CS-3 and CS-4 CS_MODE_Type coded as ‘0 1’ only, shall be used (see TSM 05.03 [13]).

Code_Values

This field defines the midamble and the Spreading Code (SC) of the assigned resources. Its coding is defined in TSM 04.08 [11].

DOWNLINK_TFI_ASSIGNMENT (5 bits)

This information element, if present, assigns the TFI to the UE to identify the downlink TBF described by this message. TFI is encoded as defined in sub-clause 12.7.

TBF Starting Time

The TBF Starting Time field contains a starting time that indicates the TDMA frame number during which the assigned TBF may start. If no downlink TBF is in progress, the UE need not monitor the TFI field of downlink RLC data blocks until the indicated TDMA frame number. After the indicated TDMA frame number, the UE shall operate as during a downlink TBF. If a downlink TBF is already in progress, the UE shall continue to use the parameters of the existing TBF until the TDMA frame number occurs. When the indicated TDMA frame number occurs, the UE shall immediately begin to use the new assigned parameters. This information element is defined in sub-clause 12.11.

Measurement Starting Time

The Measurement Starting Time field contains a starting time that indicates the frame number during which the first assigned measurement period shall occur. The UE must make one or more neighbour cell power measurements during the assigned frame number and during the following 3 TDMA frames. This information element is defined in sub-clause 12.11.

MEASUREMENT_INTERVAL (5 bits)

The Measurement Interval field indicates the number of block periods from the start of one assigned measurement period to the beginning of the next measurement period.

Bit

5 4 3 2 1

0 0 0 0 0		make measurements during every block period

0 0 0 0 1		make measurements during every other block period

0 0 0 1 0		make measurements during every 3rd block period

. . .

1 1 1 1 1		make measurements during every 32nd block period

Access Priority Parameters (13 bits)

If set, this field requests the UE to send SYNC1 bursts till the network acknowledgement before accessing on the uplink resources. Its coding is defined in sub-clause 11.4.10.

### 11.4.5	Packet Downlink Dummy Control Block

This message is sent on the PACCH by the network to the user equipment when the network has no other block to transmit.

Message type:	PACKET DOWNLINK DUMMY CONTROL BLOCK

Direction:		network to user equipment

Classification:	distribution message

Table 43: Packet DOWNLINK DUMMY CONTROL BLOCK information elements

< Packet Downlink Dummy Control Block message content > ::=

< PAGE_MODE : bit (2) >

< padding bits >

! < Distribution part error : bit (*) = < no string > > ;

Table 44: Packet DOWNLINK DUMMY CONTROL BLOCK information element details

PAGE_MODE (2 bits)

This field is defined in sub-clause 12.10.

### 11.4.6	Packet Measurement Order

This message is sent on the PACCH by the network to a user equipment giving information for NC and EXT measurement reporting, as well as network controlled cell reselection. If not all information fits into one message, the remaining information will be sent in other instances of the Packet Measurement Order message.

Message type:	PACKET MEASUREMENT ORDER

Direction:		network to user equipment

Classification:	non-distribution message

Table 45: Packet Measurement Order information elements

< Packet Measurement Order message content > ::=

< PAGE_MODE : bit (2) >

{	{ 0	< Global TFI : < Global TFI IE > >

| 10	< TLLI : bit (32) > }

{	< PMO_INDEX : bit (3) >

< PMO_COUNT : bit (3) >

{ 0 | 1 < NC Measurement Parameters : <NC Measurement Parameters struct > >}

{ 0 | 1 <EXT Measurement Parameters: <EXT Measurement Parameters struct > >}

< padding bits >

! < Non-distribution part error : bit (*) = < no string > > }

! < Address information part error : bit (*) = < no string > > }

! < Distribution part error : bit (*) = < no string > > ;

< NC Measurement Parameters struct > ::=

< NETWORK_CONTROL_ORDER : bit (2) >

{ 0 | 1	< NC_ NON_DRX_PERIOD : bit (3) >

< NC_REPORTING_PERIOD_I : bit (3) > }

{ 0 | 1	< NC_REPORTING_PERIOD_T : bit (3) > }

{ 0 | 1	< NC_FREQUENCY_LIST : < NC Frequency list struct > > } ;

< NC Frequency list struct > ::=

{ 0 | 1 { < NR_OF_REMOVED_FREQ : bit (5) >

{ < REMOVED_FREQ_INDEX : bit (6) > } * (1 + val(NR_OF_REMOVED_FREQ)) } }

{ 1 < List of added Frequency struct : < Add Frequency list struct > >} ** 0 ;

< Add Frequency list struct > ::=

< NR_OF_FREQUENCIES : bit (5) >

< Added_Cell_Description: < Cell Structure > >

{ 0 |1	< Cell selection params: < Cell Selection struct > >}* (val(NR_OF_FREQUENCIES)) ;

< Cell Selection struct > ::=

< EXC_ACC : bit >

< CELL_BAR_ACCESS_2 : bit (1) >

< SAME_RA_AS_SERVING_CELL : bit (1) >

{ 0 | 1	< (GPRS_)RXLEV_ACCESS_MIN : bit (6) >

< (GPRS_)UE_TXPWR_MAX_CCH : bit (7/5) > } --7 or 5 bits for TD-SCDMA or GSM cell respectively

{ 0 | 1	< (GPRS_)TEMPORARY_OFFSET : bit (3) >

< (GPRS_)PENALTY_TIME : bit (5) > }

{ 0 | 1	< (GPRS_)CELL_RESELECT_OFFSET : bit (5) > }

{ 0 | 1	< HCS params : < HCS struct > > }

{ 0 | 1	< SI13_LOCATION : < SI13_LOCATION struct > > } ;

< SI13_LOCATION struct > ::=

{ 0	-- Message escape

< SI13_LOCATION : bit (1) >

! < Message escape : 1 bit (*) = < no string > >} ;

< HCS struct > ::=

< TD-(G)PRS_PRIORITY_CLASS : bit (3) >

< TD-(G)PRS_HCS_THR : bit (5) > ;

< EXT Measurement Parameters struct > ::=

< Extended_Measurement_Command_Change_Mark: bit (2) >

< EXT_Measurement_Order : bit (2) == 00 >	< no string >

< EXT_Measurement_Order : bit (2) == 01 >	< EM1 struct >

< EXT_Measurement_Order : bit (2) == 10 to 11 >	Reserved

< EM1 struct > ::=

{ 0 | 1 < EXT_Reporting_Period_I: bit (3) > }

{ 0 | 1 < EXT_Reporting_Period_T: bit (3) > }

{ 0 | 1 < Intra-Cell Extended Measurements: < Intra-Cell Extended Measurements struct  >> }

{ 0 | 1 < Inter-Cell Extended Measurements: < Inter-Cell Extended Measurements struct  >> }

{ 0 | 1 < Extension bits IE: Extension bits IE> } ; -- Possible future extensions; see sub-clause 12.2

Table 46: Packet Measurement Order information element details

The Packet Measurement Order message contains measurement parameters either for Network Control (NC) measurements or for Extended (EXT) measurements or for both. If parameters for one of the measurement types are not included, a previous Packet Measurement Order message for that type shall still be valid.

The ‘NC measurement parameters struct’ gives parameters for the serving cell and may contain neighbour cell list deviations (i.e. add/delete) with respect to the list broadcast on the BCCH. This structure contains the Network Control Order, the NC parameters and an NC Frequency List struct. If the value of the Network Control Order, or any of the NC parameters, differs between instances of the PACKET MEASUREMENT ORDER message, the UE shall assume as valid the value of the parameter in the instance with the highest PMO_INDEX and shall ignore all the others.

If included, the NC Frequency List struct is a deviation list which contains removed or added  neighbour cells to the adjacent cell list sent out in SYSTEM INFORMATION TYPE 2 message instances on the BCCH (see TSM 04.08 [11]). The global number of  neighbour cells (i.e. the ones reported in SI2 plus those added with the NC Frequency List Struct) shall not exceed 128; while the total number of adjacent cells to measure shall not exceed 32. If the global list includes more than 32 neighbour cells, the UE shall measure only the first 32 with the lowest indexes.

The list of added frequencies may contain TD-PRS (GPRS) cell re-selection parameters. In case an added frequency is already available in the adjacent cell list, the cell re-reselection parameters for that cell may have been modified in the NC Frequency List struct; in this case, only the new parameters shall be used for cell reselection and only the new position shall be referred to in the measurement reports.

If the NC frequency list struct is not included in the message (no added or deleted frequencies) the UE shall return to the adjacent cell list on BCCH.

The 'EXT measurement parameters struct' gives the extended measurement parameters to be used in the serving and/or in the neighbour cells. This structure contains the Extended_Measurement_Command_Change_Mark, the EXT Measurement Order and, eventually, Intra-cell and/or inter-cell Extended Measurement commands.

PAGE_MODE (2 bits)

This field is defined in sub-clause 12.10.

PMO_INDEX (3 bits) and PMO_COUNT (3 bits)

The purpose of the PMO_INDEX field and the PMO_COUNT field is to indicate the number of individual messages within the sequence of Packet Measurement Order messages and to assign an index to identify each one of them. The PMO_INDEX field is binary coded, range: 0 to 7, and provides an index to identify the individual Packet Measurement Order message. The PMO_COUNT field is binary coded, range: 0 to 7, and provides the PMO_INDEX value for the last (highest indexed) message in the sequence of Packet Measurement Order messages. A measurement order shall not be effected by the user equipment until all instances of a Packet Measurement Order message are received.

Global TFI

If present, this information element indicates the user equipment to which this message is addressed. This field is defined in sub-clause 12.7.

TLLI (32 bits)

If present, this field indicates the user equipment to which this message is addressed. This field is defined in sub-clause 12.8.

The NC_Measurement_Parameters struct contains the NETWORK_CONTROL_ORDER and the optional parameters NC_NON_DRX_PERIOD, NC_REPORTING_PERIOD_I, NC_REPORTING_PERIOD_T and the NC_FREQUENCY LIST. When the NETWORK_CONTROL_ORDER field is set to either NC1 or NC2, at least one of the two immediately following optional parameters shall be reported; the missing one being interpreted as if no measurement report is requested for the relevant mode, respectively packet idle mode (I) or packet transfer mode (T).

NETWORK_CONTROL_ORDER (2 bits)

NC_NON_DRX_PERIOD (3 bits)

NC_REPORTING_PERIOD_I (3 bits)

NC_REPORTING_PERIOD_T (3 bits)

For detailed element definitions and coding see sub-clause 11.4.10 and sub-clauses 12.12, 12.13, 12.14 respectively.

NR_OF_REMOVED_FREQ (5 bits)

This field indicates the number of frequencies in the adjacent cell list which shall not be used for NC-measurements and gives the number of instances of the parameter REMOVED_FREQ_INDEX.

Bit

5 4 3 2 1

0 0 0 0 0		0 frequencies in the adjacent cell list shall be excluded from the NC measurements

0 0 0 0 1		1 frequency in the adjacent cell list shall be excluded from the NC measurements

0 0 0 1 0		2 frequencies in the adjacent cell list shall be excluded from the NC measurements

:    :

1 1 1 1 0		30 frequencies in the adjacent cell list shall be excluded from the NC measurements

1 1 1 1 1		All frequencies in the adjacent cell list shall be excluded from the NC measurements.

REMOVED_FREQ_INDEX (6 bits)

This field indicates the index to the  neighbour cell position (0 to n) in the adjacent cell list sent on BCCH. The neighbour cell pointed at shall not be used for NC-measurements. The index = 0 shall point to the first neighbour cell of the BCCH list and index = n to the last neighbour cell of the BCCH list, according to the neighbour cell numbering rules defined for the Measurement results IE in TSM 04.08 [11]. The removed frequencies shall keep their indexes, but no measurement shall be performed.

Range: 0 to 63.

Add Frequency list struct contains the frequency list for NC measurements.

NR_OF_FREQUENCIES (5 bits)

This field indicates, in the add frequency list, the number of frequencies of the new GSM or TD-SCDMA cell which shall be used for NC-measurements.

Cell Structure (18 or 19 bits)

This structure contains the BCCH frequency and the identity of the new GSM (18 bits) or TD-SCDMA (19 bits )cell respectively. Its coding is defined in sub-clause 11.3.3.

The Cell selection params in the Add Frequency list struct shall only be included when a frequency is added to the NC_FREQUENCY_LIST. For description of the cell selection parameters see sub-clause 11.4.3.

Extended_Measurement_Command_Change_Mark (2 bits)

This field changes each time a new Extended measurement command is delivered to the UE. If the Extended_Measurement_Command_Change_Mark value differs between instances of the PACKET MEASUREMENT ORDER message, the UE shall assume valid only the EXT measurement parameters brought by the instance having the highest number, ignoring all the others.

Range: “00” to “11”.

EXT_Measurement_Order (2 bits)

This field indicates to the UE how to interpret the following extended measurements parameters. It is coded as follows:

bit

2 1

0 0		RESET

0 1		EM1

1 0 to 1 1	Reserved for future use

If EXT_Measurement_Order is set to “EM1”, then at least one of the optional parameters in the EM1 struct will follow. If EXT_Measurement_Order is set to “RESET”, then the UE shall stop performing and sending any extended measurement.

EXT_Reporting_Period_I (3 bits)

If this field is present, the UE in packet idle mode shall periodically report to the network the requested extended measurements according to the indicated reporting period. If this field is not present, the UE shall send only once the requested extended measurement according to the rules defined in sub-clause 5.6.4. This field is coded according to the following table:

bit

3 2 1

0 0 0		1.92 sec

0 0 1		3.84 sec

0 1 0		7.68 sec

0 1 1		15.36 sec

1 0 0		30.72 sec

1 0 1		61.44 sec

1 1 0		122.88 sec

1 1 1		245.76 sec

EXT_Reporting_Period_T (3 bits)

If this field is present, the UE in packet transfer mode shall periodically report to the network the requested extended measurements according to the indicated reporting period. If this field is not present, the UE shall send only once the requested extended measurement according to the rules defined in sub-clause 5.6.4. This field has the same coding as the EXT_Reporting_Period_I.

Intra-Cell Extended Measurement struct

This structure defines the kind of extended measurements the UE is requested to perform towards the serving cell. The coding is defined in TSM 04.08 [11].

Inter-Cell Extended Measurement struct

This structure defines the kind of extended measurements the UE is requested to perform towards neighbour cells. The referred neighbour cells shall include the updating eventually provided by the NC Measurement Parameters struct of the PACKET MEASUREMENT ORDER message carrying this extended measurement request. The coding is defined in TSM 04.08 [11].

### 11.4.7	Packet Resource Release

This message is sent on PACCH by the network to notify the addressed user equipment that one or more physical resource will be immediately released and become unavailable for packet data traffic.

Message type:	PACKET RESOURCE RELEASE

Direction:		network to user equipment

Classification:	distribution message

Table 47: Packet RESOURCE RELEASE information elements

< Packet PDCH Release message content > ::=

< PAGE_MODE : bit (2) >

{	{ 0	< Global TFI : < Global TFI IE > >

| 10	< TLLI : bit (32) > }

{ 0 | 1 < UNAVAILABLE_RU: bit (112) }

{ 0 | 1 < RR Ending Time: < Starting Frame Number Description IE > >}

< padding bits >

! < Address information part error : bit (*) = < no string > > }

! < Distribution part error : bit (*) = < no string > > ;

Table 48: Packet RESOURCE RELEASE information element details

PAGE_MODE (2 bit field)

This field is defined in sub-clause 12.10.

Global TFI

If present, this information element indicates the user equipment to which this message is addressed. This field is defined in sub-clause 12.6.

TLLI (32 bits)

If present, this field indicates the user equipment to which this message is addressed. This field is defined in sub-clause 12.8.

UNAVAILABLE_RU (112 bits)

This field contains the indication of which resource units have to be released. It is encoded as a bit map with respect to the maximum number of resource units available per radio frame (given by the product of the time slot number (7) and the maximum code number (16) per time slot) as follows:

bit 112 corresponds to the resource unit having the highest absolute number

…

bit 1 corresponds to the resource unit having the lowest absolute number

A resource unit composed by N basic resource units shall use N bits of the UNAVAILABLE_RU field.

For the numbering principle of the resource units, refer to TSM 04.08 [11].

A bit in the UNAVAILABLE_RU field set to value “1” indicates that the corresponding resource unit has to be released. If the UNAVAILABLE_RU field is not present in the message, the addressed UE shall release all the assigned packet resource units.

RR (Resource Release) Ending Time

The RR Ending Time field contains an ending time that indicates at which TDMA frame number the indicated resources shall be released.

When the indicated TDMA frame number occurs, the UE shall release the indicated resources as specified in sub-clauses 9.3.2.3, 9.3.2.5, 9.3.3.3 and 9.3.3.5.

The UE shall continue to use the parameters of the existing resources, acting on further RLC/MAC control messages possibly received, until the indicated TDMA frame number occurs.

This information element is defined in sub-clause 12.11.

### 11.4.8	Packet Polling Request

This message is sent on the PACCH by the network to the user equipment to solicit a PACKET CONTROL ACKNOWLEDGEMENT message from the UE. The network can also request the report of specific measurements.

Message type:	PACKET POLLING REQUEST

Direction:		network to user equipment

Classification:	non-distribution message

Table 49: Packet POLLING REQUEST information elements

< Packet Polling Request message content > ::=

< PAGE_MODE : bit (2) >

{	{ 0	< Global TFI : < Global TFI IE > >

| 10	< TLLI : bit (32) >

{ 0 |1 < Requested_Measurements : < Requested_Measurements struct > > }

{	< padding bits >

! < Non-distribution part error : bit (*) = < no string > > }

! < Address information part error : bit (*) = < no string > > }

! < Distribution part error : bit (*) = < no string > > ;

< Requested_Measurements struct > :: =

{ 0 | 1 	<Synchronisation_Shift&Power_Level: < Synchronisation_Shift&Power_Level struct >>}

{ 0 | 1 	< Interference_Level: < Interference_Level struct > > }

{ 0 -- Escape message

! < message escape: 1 bit (*) = < no string > > } ;

< Synchronisation_Shift&Power_Level struct > :: =

< Time_Slot_State_Flag: bit (7) >

< Synchronisation_Shift_I: bit >

< Power_Level_I: bit > ;

< Interference_Level struct > :: =

< Time_Slot_State_Flag: bit (7) >

{ < Midamble_Shift_N: bit (4) > } * (val(Number of Flagged Time slots in Time_Slot_State_Flag field));

Table 50: Packet POLLING REQUEST information element details

PAGE_MODE (2 bits)

This field is defined in sub-clause 12.10.

TLLI (32 bits)

This field is defined in sub-clause 12.8.

Global TFI

This information element contains the TFI of the UE's downlink TBF or uplink TBF. This field is defined in sub-clause 12.7.

Time_Slot_State_Flag (7 bits)

Depending on whether it appears in the Synchronisation_Shift&Power_level structure or in the Interference_Level structure, this field contains the indication respectively of which time slots the UE is requested to report the applied synchronisation shift and/or transmitted power level or on which time slots the UE is requested to report the measured interference level. In both cases, the flagged time slots shall be the ones the requested measure refers to. The Time_Slot_State_Flag field is encoded as described in sub-clause 11.3.4.

Synchronisation_Shift_I (bit)

This field contains the indication whether the UE is requested to report the applied synchronisation shift value on the indicated time slots in the Time_Slot_State_Flag field. It is coded as follows:

bit

1

- The UE shall not report the applied synchronisation shift value;

- The UE should report the applied synchronisation shift value.

Power_Level_I (bit)

This field contains the indication whether the UE is requested to report the applied transmit power level on the indicated time slots in the Time_Slot_State_Flag field. It is coded as follows:

bit

1

0	The UE shall not report the applied transmit power level;

1	The UE should report the applied transmit power level.

Midamble_Shift_N (4 bits)

This field indicates which midamble code the UE has to use to perform the requested interference measurement on the indicated time slot. As many midamble code values will be listed in the Interference_Level structure as the number of flagged time slots in the Time_Slot_State_Flag field is; such that the first Midamble_Shift_N value is associated to the lowest number flagged time slot and the last Midamlbe_Shift value is associated to the highest number flagged time slot respectively. It is coded as the Midamble_Shift value defined in TSM 04.08 [11].

### 11.4.9	Packet Power Control/Timing Control

This message is sent on PACCH by the network to the user equipment in order to update the UE timing and/or power control parameters.

Message type:	PACKET POWER CONTROL/TIMING CONTROL

Direction:		network to user equipment

Classification:	non-distribution message

Table 51: Packet POwer CONTROL/TIMING CONTROL information elements

< Packet Power Control/Timing Control message content > ::=

< PAGE_MODE : bit (2) >

{	{ 0	< Global TFI : < Global TFI IE > >

{ 0		-- Message escape

{ 0| 1 < Power Control Parameters : < Power Control Parameters struct >> }

{0 | 1 < Timing Control Parameters : < Timing Control Parameters struct >> }

< padding bits >

! < Non-distribution part error : bit (*) = < no string > > }

! < Message escape : 1 bit (*) = <no string> > }

! < Address information part error : bit (*) = < no string > > }

! < Distribution part error : bit (*) = < no string > > ;

< Power Control Parameters struct > :: =

< Time_Slot_State_Flag: bit (7) >

{ < Power Control Command: bit (7) > } * (val(number of flagged time slots in Time_Slot_State_flag field));

< Timing Control Parameters struct > :: =

< Time_Slot_State_Flag: bit (7) >

{ < Timing Control Command: bit (11) > } * (val(number of flagged time slots in Time_Slot_State_flag field));

Table 52: Packet POwer CONTROL/TIMING CONTROL information element details

Time_Slot_State_Flag (7 bits)

This field contains the indication on which time slots the respective control command refers. Its coding is defined in sub-clasue 11.3.4.

Power Control Command (7 bits)

This field defines the transmit Power level the UE should apply on the indicated time slot such that the first listed Power Control Command corresponds to the highest flagged time slot number in the Time_Slot_State_Flag field and the last listed Power Control Command corresponds to the lowest flagged time slot number in the Time_Slot_State_Flag field. The Power Control Command field is coded as the Power Control Command field described in TSM 04.08 [11].

Timing control Command (11 bits)

This field defines the Synchronisation Shift value the UE should apply on the indicated time slot such that the first listed Timing Control Command corresponds to the highest flagged time slot number in the Time_Slot_State_Flag field and the last Timing Control Command corresponds to the lowest flagged time slot number in the Time_Slot_State_Flag field. The Timing Control Command field is coded as the Synchronisation Shift field described in TSM 04.08 [11].

Referenced Address struct

This information element indicates the identity of the user equipment to which this message is addressed. All other s shall ignore this message.

Global TFI

This information element contains the TFI of the UE's downlink TBF or uplink TBF. This field is defined in sub-clause 12.7.

### Packet System Information Type 13

This message may be broadcast by the network on the PACCH (see sub-clause 5.5.2.1). The message provides the user equipment with TD-PRS cell specific access-related information. The information in this message shall be the same as provided in the SI13 message on BCCH, see TSM 04.08 [11].

Message type:	PACKET SYSTEM INFORMATION TYPE 13

Direction:		network to user equipment

Classification:	distribution message

Table 53: Packet SYSTEM INFORMATION TYPE 13 information elements

< PSI13 message content > ::=

< PAGE_MODE : bit (2) >

< BCCH_CHANGE_MARK : bit (6) >

< SI_CHANGE_FIELD : bit (4) >

< Cell Option parameters : < Cell option struct > >

< Cell Selection parameters : < Cell Selection Parameters struct > >

< Power Control Parameters: < Power Control Parameters struct > >

< Measurement Parameters: < Measurement Parameters struct > >

 0 | 1 < Access Priority Parameters : bit (13) >

< padding bits >

! < Distribution part error : bit (*) = < no string > > ;

< Cell options struct >::=

< Timer T3146: bit (3) >

< Timer T3204: bit (3) >

< Counter Pk_UE : bit (3) >

< Timer T3168 : bit (3) >

< Timer T3192 : bit (3) >

< NMO : bit (2) >

< DRX_TIMER_MAX : bit (3) >

< BS_CV_MAX: bit (4) >

< SPGC_CCCH_SUP : bit >

{ 0 | 1 	< PAN_DEC : bit (3) >

< PAN_INC : bit (3) >

< PAN_MAX : bit (3) >}

-- Possible future extension:

{ 0 | 1	< Extension Bits IE > } ;				-- sub-clause 12.2

< Cell Selection Parameters struct >:: =

< RAC : bit (8) >

< PRIORITY_ACCESS_THR : bit (3) >;

< Power Control Parameters struct > :: =

< ALPHA : bit (4) >

< T_AVG_W : bit (5) >

< T_AVG_T : bit (5) >

< N_AVG_I : bit (4) > ;

< Measurement Parameters struct > :: =

< NETWORK_CONTROL_ORDER : bit (2) >

< PC_MEAS_CHAN : bit >

 0 | 1 < NC_REPORTING_PERIOD_T: bit (3) > 

 0 | 1 < NC_REPORTING_PERIOD_I: bit (3) >

NC_NON_DRX_PERIOD : bit (3)  ;

Table 54: Packet SYSTEM INFORMATION TYPE 13 information element details

PAGE_MODE (2 bits)

This field describes which type of page mode used, i.e. either normal paging, extended paging, paging reorganization or same as before from the previous page mode. The user equipment shall ignore this field if the message is received on the PACCH. Coding of this field is defined in TSM 04.08 [11].

BCCH_CHANGE_MARK (6 bits)

This field indicates the status of the information on BCCH. The value of this field shall be changed each time the information on BCCH, except for the contents of the SI-13 message, is changed.

SI_CHANGE_FIELD (4 bits)

This field is the binary representation of which information was changed at the last indication in BCCH_CHANGE_MARK. Range 0 to 15:

bit

2 1

0 0 0 0		Update of unspecified SI message or SI messages;

0 0 0 1		Update of SI1 message;

0 0 1 0		Update of SI2 message;

0 0 1 1		Update of SI3, SI4, SI7 or SI8 message;

0 1 0 0		Update of SI9 message;

All other values shall be interpreted as 'update of unknown SI message type'.

SPGC_CCCH_SUP (bit)

This field indicates the support of the parameter SPLIT_PG_CYCLE on CCCH from the network side:

0	SPLIT_PG_CYCLE is not supported on CCCH in this cell;

1	SPLIT_PG_CYCLE is supported on CCCH in this cell.

RAC (8 bits)

This field is the binary representation of the Routing Area Code, see TS 23.003 [3].

PRIORITY_ACCESS_THR (3 bits)

This field is the binary representation of the parameter PRIORITY_ACCESS_THR:

Bit

3 2 1

0 0 0		packet access is not allowed in the cell;

0 0 1		spare, shall be interpreted as '000' (packet access not allowed);

0 1 0		spare, shall be interpreted as '000' (packet access not allowed);

0 1 1		packet access is allowed for priority level 1;

1 0 0		packet access is allowed for priority level 1 to 2;

1 0 1		packet access is allowed for priority level 1 to 3;

1 1 0		packet access is allowed for priority level 1 to 4;

1 1 1		spare, shall be interpreted as '110' (packet access allowed).

NETWORK_CONTROL_ORDER (2 bits)

This field is the binary representation of the parameter NETWORK_CONTROL_ORDER, and it is coded as follows:

bit

2 1

0 0		NC0:	UE controlled cell re-selection, no measurement reporting.

0 1		NC1:	UE controlled cell re-selection, UE sends measurement reports.

1 0		NC2:	Network controlled cell re-selection, UE sends measurement reports.

1 1		Reserved for future use, interpreted as NC0 by user equipment.

When the NETWORK_CONTROL_ORDER field is set to either NC1 or NC2, at least one of the two immediately following optional parameters shall be reported; the missing one being interpreted as if no measurement report is requested for the relevant mode, respectively packet idle mode (I) or packet transfer mode (T).

NC_REPORTING_PERIOD_T (3 bits)

If this field is present, the UE in packet transfer mode shall report the measurements to the network according to the indicated period, without waiting for a MEASUREMENT REPORT ORDER message. If this field is not present, the UE shall wait for the detection of a MEASUREMENT REPORT ORDER message before starting to report any measure.

Its coding is defined in sub-clause 12.14.

NC_REPORTING_PERIOD_I (3 bits)

If this field is present, the UE in packet idle mode shall report the measurements to the network according to the indicated parameters (i.e. reporting period and non DRX period). If this field is not present, the UE shall not perform measurement report when in packet idle mode.

Its coding is defined in sub-clause 12.14.

NC_NON_DRX_PERIOD (3 bits)

Its coding is defined in sub-clause 12.13.

PC_MEAS_CHAN (1 bit)

This field indicates the type of channel which shall be used for downlink measurements for power control:

0	BCCH

1	PDCH.

ALPHA (4 bits)

This field is the binary representation of the parameter  for UE output power control in units of 0.1, see TSM 04.08 [11]. Range: 0 to 10. Values greater than 10 shall be interpreted as 10 by the user equipment.

T_AVG_W (5 bits)

This field is the binary representation of the parameter TAVG_W for UE output power control, see TSM 05.08 [15]. Range: 0 to 25. Values greater than 25 shall be interpreted as 25 by the user equipment.

T_AVG_T (5 bits)

This field is the binary representation of the parameter TAVG_T for UE output power control, see TSM 05.08 [15]. Range: 0 to 25. Values greater than 25 shall be interpreted as 25 by the user equipment.

N_AVG_I (4 bits)

This field is the binary representation of the parameter NAVG_I for UE output power control, see TSM 05.08 [15]. Range: 0 to 15.

Access Priority Parameters (13 bits)

This optional filed may be used by the network to control the use of the UpPTS physical channels by the packet users. It is encoded as the Access Priority Parameters field defined in TSM 04.08 [11].

Timer T3146 (3 bits)

This timer controls the UE waiting time to get an IMMEDIATE ASSIGNMENT ( / IMMEDIATE ASSIGNMENT REJECT) message from the network during a packet access procedure. It is coded as defined in TSM 04.08 sub-clause 9.1.43.1.

Timer T3204 (3 bits)

This timer controls the P-PDPCH packet access procedure, described in sub-clause 7.1.2.3, at the UE side. It is coded as defined in clause 13.

Counter Pk_UE (3 bits)

This counter controls the P-PDCH maintenance procedure, described in sub-clause 7.1.2.3.1. It is coded as defined in clause 13.

Timer T3168 (3 bits)

This timer controls the waiting time for a PACKET UPLINK ASSIGNMENT message.

Timer T3192 (3 bits)

This timer controls the waiting time before releasing the resources of a downlink TBF at the reception of the final block, and the time interval during which the establishment of an uplink or a new downlink TBF is allowed (see sub-clauses 9.3.2.5 and 9.3.3.5).

The timeout value of timer T3192 is given in terms of block periods according to the following table:

Bits

3 2 1

0 0 0		immediate release

0 0 1		wait for 35 block periods before release

0 1 0		wait for 70 block periods before release

0 1 1		wait for 105 block periods before release

1 0 0		wait for 140 block periods before release

1 0 1		wait for 175 block periods before release

1 1 0		wait for 210 block periods before release

1 1 1		wait for 250 block periods before release.

NMO (2 bit)

This field is the binary representation of the Network Mode of Operation, see TSM 03.60 [19]:

bit

1

0 0		Network Mode of Operation I

0 1		Network Mode of Operation II

1 0		Network Mode of Operation III

1 1		Reserved.

DRX_TIMER_MAX (3 bits)

This field is the binary representation of the parameter DRX_TIMER_MAX. Range: 0 to 7. The parameter value is given as two taken to the power of the binary value minus one (2 (bv - 1) ) in units of 1 second. The binary value zero indicates the parameter value zero (i.e, the parameter takes the values: 0, 1 s, 2 s, 4 s, .. , 64 s.)

BS_CV_MAX (4 bit field)

This field is the binary representation of the parameter BS_CV_MAX. Range: 0 to 15. The value BS_CV_MAX=0 shall be interpreted as value BS_CV_MAX=1 for calculation of T3198, T3200 and N3104max values.

PAN_DEC (3 bit field)

This field is the binary representation of the parameter PAN_DEC. Range: 0 to 7.

PAN_INC (3 bit field)

This field is the binary representation of the parameter PAN_INC. Range: 0 to 7.

PAN_MAX (3 bit field)

This field defines the maximum value allowed for counter N3102.

Bit

2 1

0 0 0		maximum value allowed for counter N3102 is 4

0 0 1		maximum value allowed for counter N3102 is 8

. . .

1 1 1		maximum value allowed for counter N3102 is 32

### 11.4.11	Packet TBF Release

This message is sent on the PACCH by the network to the user equipment to initiate release of an uplink or downlink TBF.

Message type:	PACKET TBF RELEASE

Direction:		network to user equipment

Classification:	non-distribution message

Table 55: Packet TBF RELEASE information elements

< Packet TBF Release message content > ::=

< PAGE_MODE : bit (2) >

{	00

< GLOBAL_TFI : Global TFI IE >

< UPLINK_RELEASE : bit (1) >

< DOWNLINK_RELEASE : bit (1) >

{	< TBF_RELEASE_CAUSE : bit (4)  = { 0000 | 0010 } >

< padding bits >

! < Non-distribution part error : bit (*) = < no string > > }

! < Address information part error : bit (*) = < no string > > }

! < Distribution part error : bit (*) = < no string > > ;

Table 56: Packet TBF RELEASE information element details

PAGE_MODE (2 bits)

This field is defined in clause 12.10.

Global TFI IE

This information element contains the TFI of the user equipment’s which uplink and/or downlink TBF to be released. This field is defined in clause 12.6.

Uplink_Release (1 bit field)

Downlink_Release (1 bit)

These fields indicate which TBF shall be release, uplink or downlink. Both directions can be released at the same time.

TBF_RELEASE_CAUSE (4 bit field)
This field indicates the reason for the release of the TBF. This field is encoded according to the following table:

bit
 4 3 2 1

0 0 0 0		Normal release

0 0 1 0		Abnormal release

All other values are reserved, the same behaviour in reception as if 'Abnormal release'.

### 11.4.12	Packet Uplink Ack/Nack

This message is sent on the PACCH by the network to the user equipment to indicate the status of the received RLC data blocks; this message may also update the timing and power control parameters of the UE.

A fixed allocation user equipment can receive a new uplink resource assignment.

Message type:	PACKET UPLINK ACK/NACK

Direction:		network to user equipment

Classification:	non-distribution message

Table 57: Packet UPLINK ACK/NACK information elements

< Packet Uplink Ack/Nack message content > ::=

< PAGE MODE : bit (2) >

{	00 < UPLINK_TFI : bit (5) >

{ 0		-- Message escape

{	< CHANNEL_CODING_COMMAND : bit (4) >

< Ack/Nack Description : < Ack/Nack Description IE > >

{ 0 | 1	< CONTENTION_RESOLUTION_TLLI : bit (32) > }

{0 | 1	< Power Control Parameters : < Power Control Parameters struct > > }

{ 0 | 1	< Timing Control Parameters : < Timing Control Parameters struct > > }

{ 0 | 1	< Extension Bits : Extension Bits IE > }	-- sub-clause 12.2

{ 0 | 1	< Fixed Allocation Parameters : < Fixed Allocation struct > > }

< padding bits >

! < Non-distribution part error : bit (*) = < no string > > }

! < Message escape : 1 bit (*) = <no string> > }

! < Address information part error : bit (*) = < no string > > }

! < Distribution part error : bit (*) = < no string > > ;

< Fixed Allocation struct > ::=

< FINAL_ALLOCATION : bit (1) >

{ 0		-- Repeat Allocation

{ 0 | 1 < Time_Slot_State_Flag : bit (7) > }

| 1		-- Allocation with Allocation bitmap

< TBF Starting Time : < Starting Frame Number Description IE > >

{ 0 | 1	<Time_Slot_State_Flag : bit (7) > }

{ 0	{ 0		-- with length of Allocation Bitmap

< ALLOCATION_BITMAP_LENGTH : bit (7) >

< ALLOCATION_BITMAP : bit (val(ALLOCATION_BITMAP_LENGTH)) >

| 1		-- without length of Allocation Bitmap (fills remainder of the massage)

< ALLOCATION_BITMAP : bit ** > }

! < Message escape : 1 bit (*) = < no string > > } };

Table 58: Packet UPLINK ACK/NACK information element details

PAGE_MODE (2 bits)

This field is defined in sub-clause 12.10.

UPLINK_TFI (5 bits)

This field identifies the uplink TBF to which this message applies. This field is coded the same as the TFI field defined in sub-clause 12.7.

CHANNEL_CODING_COMMAND (4 bits)

The Channel Coding Command field indicates the channel coding scheme that the user equipment shall use when transmitting on the uplink. It can specify a different Channel Coding Scheme but shall assume the same Coding Scheme Mode type (CS_MODE_Type) as currently in use.

Bits

4 3 2 1		value

0 0 0 0		CS-1

0 0 0 1		CS-2

0 0 1 0 		CS-3

0 0 1 1		CS-4

0 1 0 0 to 1 1 1 1	Reserved

Ack/Nack Description

This information element is defined in sub-clause 12.3.

CONTENTION_RESOLUTION_TLLI (32 bits)

The CONTENTION_RESOLUTION_TLLI field is present only if the network has decoded one of the uplink RLC data blocks containing the TLLI. The user equipment shall perform the contention resolution function if the TLLI information element is present. This field contains a TLLI, which is defined in sub-clause 12.8.

Power control parameters

This information element is described in sub-clause 11.4.9.

Timing control parameters

This information element is described in sub-clause 11.4.9.

Time_Slot_State_Flag (7 bits)

This field defines which time slots are maintained, by setting to “1” the corresponding bit, and which ones are released, by setting to “0” the corresponding bit. If not included, the current time slot configuration shall be implicitly granted. Its coding is defined in sub-clause 11.3.4.

FINAL_ALLOCATION (1 bit)

This field indicates whether this allocation is the last allocation of the TBF.

0	this allocation is not the last allocation of the TBF

1	this allocation is the last allocation of the TBF

Repeat Allocation

If present, the user equipment shall repeat the current allocation after the current allocation ends.

Allocation with Allocation bitmap

If present, the user equipment shall use the included Allocation bitmap and associated parameters. The UE shall not repeat the current allocation after the current allocation ends.

ALLOCATION_BITMAP_LENGTH (7 bits)

This field specifies the number of bits in the ALLOCATION_BITMAP.

Range 0 to 127.

TBF Starting Time

The TBF Starting Time field contains a starting time that indicates the earliest frame number during which the assigned TBF fixed configuration can start.

The UE shall continue to use the parameters of the existing TBF until the indicated TDMA frame number occurs. When the TDMA frame number occurs, the UE shall then use the assigned uplink resources from the indicated TDMA frame number forward, according to the parameters in the fixed allocation struct.

This information element is defined in sub-clause 12.11.

ALLOCATION_BITMAP (variable length field)

The ALLOCATION_BITMAP field has variable length. If the ALLOCATION_BITMAP_LENGTH field is not present, the ALLOCATION_BITMAP field fills the remainder of the message. The ALLOCATION_BITMAP field represents the uplink block periods during which transmission is allowed. This field is defined in sub-clause 12.4.

### 11.4.13	Packet Uplink Assignment

This message is sent on the PACCH by the network to the user equipment to assign uplink resources to the UE when an uplink TBF is not yet established.

In case uplink P-PDPCH resources are assigned, and no P-PDCH is already established between the user equipment and the network, at least one downlink P-PDPCH resource shall be assigned as well in order to fulfil the duplex operative mode of a P-PDCH (see sub-clause 4.5.1).

The PACKET UPLINK ASSIGNMENT message can both define additional resources only with respect to the ones already established and reconfigure the all packet resources, according to the Message_State_I field value.

When the Message_State_I field indicates “Global resource description”, it refers to the resources assigned to all active TBFs of the UE.

When the Message_State_I field indicates “Incremental resource description” and the message defines resources already established, the UE shall interpret this as a reconfirm. When the Message_State_I field indicates “Incremental resource description” and no resources are assigned by the message, this means that the current ones are reconfirmed.

When used to reconfigure the all resources, the message allows to change the assigned frequency.

The UE may be addressed by TFI, depending upon the used procedure.

Message type:	PACKET UPLINK ASSIGNMENT

Direction:		network to user equipment

Classification:	non-distribution message

Table 59: Packet UPlink ASSIGNMENT information elements

< Packet Uplink Assignment message content > ::=

< PAGE_MODE : bit (2) >

< Message_State_I : bit >

{	{ 0	< Global TFI : < Global TFI IE > >

| 10	< TLLI : bit (32) > }

{ 0		-- Message escape

{	< CHANNEL_CODING_COMMAND : bit (4) >

{ 0|1< RESOURCE_ALLOCATION: < Uplink Packet Channel struct>>}

{ 	{ 0 | 1 < UPLINK_TFI_ASSIGNMENT: bit (5) > }

{ 0 | 1 < FBN: bit (4) > }

{ 0 | 1 < Frequency_Parameters: < Frequency_Parameters struct > >  };

{ 0 | 1 < TBF Starting Time: <Starting Frame Number Description IE >>}

{ 0 | 1<Priority Access Parameters: bit (13)} }

{ 0 | 1<Power Control Parameters: <Power Control Parameters struct>>}

{0|1<Timing Control Parameters: <Timing Control Parameters struct>>}

< padding bits >

! < Non-distribution part error : bit (*) = < no string > > }

! < Message escape : 1 bit (*) = <no string> > }

! < Address information part error : bit (*) = < no string > > }

! < Distribution part error : bit (*) = < no string > > ;

< uplink Packet Channel struct > :: =

{ 0 | 1 < Pilot Channel set description: < Pilot channel set struct > > }

{ 0 | 1 < Shared Channel set description: < Shared channel set struct > > } ;

< Pilot channel set struct > :: =

< Channel Mode: bit (5) >

< HALF_DUPLEX_MODE: bit >

{ 0 | 1 < RLC_DATA_BLOCKS_GRANTED: bit (8) > }

< Uplink: <Resource_Units struct >>

{ 0 | 1 < Downlink: < Resource_Units struct > > } ;

< Shared channel set struct > :: =

{ 0 0	< Dynamic allocation: < Dynamic allocation struct > >

| 0 1	< Fixed allocation: < Fixed allocation struct > >

| 1 0 | 1 1 	<extension> ;

<Dynamic Allocation struct > ::=

< USF_GRANULARITY: bit >

{ 0 | 1 < USF: bit (3) > }

{ 0 | 1 < RLC_DATA_BLOCKS_GRANTED: bit (8) > }

< Uplink: < Resource_Units struct > > ;

<Fixed Allocation struct > ::=

< FINAL_ALLOCATION: bit >

< HALF_DUPLEX_MODE: bit >

< Uplink: < Resource_Units struct > >

{ 0 | 1	<Time_Slot_State_Flag : bit (7) > }

{0	{ 0	-- with length of Allocation Bitmap

< ALLOCATION_BITMAP_LENGTH : bit (7) >

< ALLOCATION_BITMAP : bit (val(ALLOCATION_BITMAP_LENGTH)) >

| 1	-- without length of Allocation Bitmap

< ALLOCATION_BITMAP: bit ** > }

! < Message escape : 1 bit (*) = <no string> > };

< extension > ::=	-- Future extension can be done by modifying this structure

null ;

< Frequency_Parameters struct > ::=

< RFqN: bit (8) >

{ 0 | 1 < Basic Midamble&Scrambling_Code: < Basic   Midamble&Scrambling_Code struct > > } ;

< Basic Midamble&Scrambling_Code struct > :: =

< Basic Midamble Number: bit (2) >

{ 0 | 1 < Code Group: bit (5) > } ;

Table 60: Packet UPLINK ASSIGNMENT information element details

PAGE_MODE (2 bits)

This field is defined in sub-clause 12.10.

Message_State_I (bit)

This field indicates whether this message describes incremental or global packet resources. It is coded as follows:

0	Incremental resource description

1	Global resource description

.Referenced Address struct

This information element contains the address of the UE addressed by the message.

Global TFI

This information element identifies the uplink TFI, if available, or the downlink TFI, to which this message applies. This field is defined in sub-clause 12.7.

CHANNEL_CODING_COMMAND (4 bits)

The Channel Coding Indicator field indicates the channel coding scheme that the UE shall use when transmitting data on the uplink. Its coding is defined in sub-clause 11.4.12.

UPLINK_TFI_ASSIGNMENT (5 bits)

This information element, if present, assigns the contained TFI to the UE to identify to uplink TBF described by this message. Its coding is in sub-clause 12.7.

Power control parameters

This information element is defined in sub-clause 11.4.9.

Timing control parameters

This information element is defined in sub-clause 11.4.9.

Resource_Units struct

This field defines the assigned resources. Its coding is defined in sub-clause 11.4.4.

Channel Mode (5 bits)

This field defines the  pilot channel mode; its coding is defined in sub-clause 11.4.4.

TBF Starting Time

The TBF Starting Time field contains a starting time that indicates the frame number during which the assigned TBF may start.

In case of dynamic allocation, if no uplink TBF is in progress, the UE need not to look for the Time_Slot_State_Flag and/or USF to know when it is allowed to transmit until the TDMA frame number occurs. When the indicated TDMA frame number occurs, the UE shall immediately begin to monitor for those fields and use the new assigned uplink TBF parameters when it is requested to transmit. If an uplink TBF is already in progress, the UE shall continue to use the parameters of the existing TBF until the TDMA frame number occurs. When the indicated TDMA frame number occurs, the UE shall immediately use the new assigned uplink TBF parameters when it is requested to transmit.

In case of fixed allocation, if no uplink TBF is in progress, the UE shall wait until the TDMA frame number occurs, and then it shall use the assigned uplink resources from the indicated TDMA frame number forward, according to the parameters in the fixed allocation struct. If an uplink TBF is in progress, the UE shall continue to use the parameters of the existing TBF until the TDMA frame number occurs. When the TDMA frame number occurs, the UE shall then use the assigned uplink resources from the indicated TDMA frame number forward, according to the parameters in the fixed allocation struct.

This information element is encoded as the Starting Frame Number Description IE. See sub-clause 12.11.

FBN (4 bits)

This filed defines the new assigned frequency band. It is encoded as the FBN field described in TSM04.08 [11].

Frequency_Parameters

RFqN (8 bits)

This field defines the new assigned frequency in the indicated FBN if assigned, or in the current frequency band. It is encoded as the RFqN field described in TSM 0408 [11].

Basic Midamble & Scrambling_Code

When present, this IE defines the pair “basic midamble” and “scrambling code sequence” to be used in association with the specified carrier frequency (RFqN). If this field is not present, it is intended that the same pair as for the carrier currently in use shall apply.

The Basic Midamble&Scrambling_Code IE is composed by the Basic Midamble Number and optionally by the Code Group information fields.

The Basic Midamble Number field points to the pair of basic midamble and scrambling code sequence within the defined Code Group.

The Code Group field points to a specific subset of 4 basic midamble - scrambling code sequence pairs. If this field is not present, it is intended that the same subset as for the carrier currently in use shall apply.

For the coding and a detailed description of the correspondence between Code group and pairs of Basic midamble-Scrambling code sequence, refer to TSM 05.02 [13].

RLC_DATA_BLOCKS_GRANTED (8 bits)

The RLC/MAC blocks Granted field assigns a fixed number of RLC data blocks that the UE shall transmit during the uplink TBF. If the RLC_DATA_BLOCKS_GRANTED field is present, the UE shall transmit only the assigned number of RLC data blocks. Otherwise the duration of the uplink TBF is undefined. Retransmissions of negatively acknowledged RLC data blocks do not apply toward the maximum number. This field is encoded as a binary number as shown:

bit

6 5 4 3 2 1

0 0 0 0 0 0 0 0		9 RLC data blocks

0 0 0 0 0 0 0 1		10 RLC data blocks

. . .

1 1 1 1 1 1 1 1		264 RLC data blocks

USF (3 bits)

This field indicates the USF value assigned to the UE for the allocated shared resources (range 0 to 7). This field is encoded as a binary representation of the USF value as defined in sub-clause 10.4.1.

USF_GRANULARITY (1 bit)

This information field indicates the USF granularity to be applied by the UE when it is assigned a TBF using Dynamic Allocation. It is coded as follows:

0	the UE shall transmit one RLC/MAC block

1	the UE shall transmit four consecutive RLC/MAC blocks

Time_Slot_State_Flag (7 bits)

This field defines which time slots are maintained, by setting to “1” the corresponding bit, and which ones are released, by setting to “0” the corresponding bit. If not included, the current time slot configuration shall be implicitly granted. Its coding is defined in sub-clause 11.3.4.

FINAL_ALLOCATION (1 bit)

This field indicates whether this allocation is the last allocation of the TBF. It is coded as follows:

0	this allocation is not the last allocation of the TBF

1	this allocation is the last allocation of the TBF

HALF_DUPLEX_MODE (1 bit)

This information field indicates whether the UE shall operate in half duplex mode. It is coded as follows:

0	the UE shall not operate in half duplex mode

1	the UE shall operate in half duplex mode

ALLOCATION_BITMAP_LENGTH (7 bits)

This specifies the number of bits in the ALLOCATION_BITMAP.

Range 0 to 127.

ALLOCATION_BITMAP (variable length field)

The ALLOCATION_BITMAP field has variable length. If the ALLOCATION_BITMAP_LENGTH field is not present, the ALLOCATION_BITMAP field fills the remainder of the message. The ALLOCATION_BITMAP field represents the uplink block periods during which transmission is allowed. This field is defined in sub-clause 12.4.

Access Priority Parameters (13 bits)

If set, this field requests the UE to send SYNC1 bursts till the network acknowledgement before accessing on the uplink resources. Its coding is defined in sub-clause 11.4.10.

### 11.4.14	Packet Resource Reconfigure

This message is sent on the PACCH by the network to the user equipment to assign or reconfigure uplink and/or downlink resources, independently of whether a TBF is already established in the respective resource assignment direction.

In case of P-PDPCH resource assignment, the network shall always assure the duplex operative mode of a pilot P-PDCH (see sub-clause 4.5.1).

Th PACKET RESOURCE RECONFIGURE message can both define additional resources only with respect to the already established ones and reconfigure the all packet resources, according to the Message_State_I field value.

When the Message_State_I field indicates “Global resource description”, it refers to the resources assigned to all active TBFs of the UE.

When the Message_State_I field indicates “Incremental resource description” and the message defines resources already established, the UE shall interpret this as a reconfirm. When the Message_State_I field indicates “Incremental resource description” and no resources are assigned by the message, this means that the current ones are reconfirmed.

When used to reconfigure the all resources, the message allows to change the assigned frequency.

Message type:	PACKET RESOURCE RECONFIGURE

Direction:		network to user equipment

Classification:	non-distribution message

Table 61: Packet RESOURCE RECONFIGURE information elements

< Packet Timeslot Reconfigure message content > ::=

< PAGE_MODE : bit (2) >

< Message_State_I: bit >

{	0 < GLOBAL_TFI : < Global TFI IE > >

{ 0		-- Message escape

{    {0|1	< CHANNEL_CODING_COMMAND : bit (4) > }

{0|1	< DOWNLINK_RLC_MODE : bit (1) >

< CONTROL_ACK : bit (1) > }

{ 0 | 1 < RESOURCE_ALLOCATION: < Packet Channel struct > > }

{	{ 0 | 1 < FBN: bit (4) > }

{ 0 | 1 < Frequency_Parameters: < Frequency_Parameters struct > > };

{ 0 | 1 < DOWNLINK_TFI_ASSIGNMENT : bit (5) > }

{ 0 | 1 < UPLINK_TFI_ASSIGNMENT : bit (5) > }

{ 0 | 1 < TBF Starting Time: Starting Frame Description IE > }

{ 0 | 1 < Priority Access Parameters: bit (13) > } }

{ 0 | 1 < Power Control Parameters: < Power Control Param. struct > >}

{ 0 | 1 < Timing Control Parameters: < timing Control Param. struct > >}

< padding bits >

! < Non-distribution part error : bit (*) = < no string > > }

! < Message escape : 1 bit (*) = <no string> > }

! < Address information part error : bit (*) = < no string > > }

! < Distribution part error : bit (*) = < no string > > ;

< Packet Channel struct > ::=

{ 0 | 1 < Power_Reference: bit (4) >}

{ 0 | 1 < Measurement Mapping: < Measurement Mapping struct > >}

{ 0 | 1 < Pilot Channel set description: < Pilot channel set struct > > }

{ 0 | 1 < Shared Channel set description: < Shared channel struct > > };

< Pilot channel set struct > :: =

< Channel Mode: bit (5) >

{ 0 | 1 < HALF_DUPLEX_MODE: bit >}

{ 0 | 1 < RLC_DATA_BLOCKS_GRANTED: bit (8) >}

{ 0 | 1 < Uplink: < Resource_Units_Struct > >}

{ 0 | 1 < Downlink: < Resource_Units_Struct > >} ;

< Shared Channel set struct > : : =

{ 0 | 1 < Downlink: < Resource_Units_Struct > >}

{ 0 | 1	{ 0 0	< Uplink Dynamic allocation: < Dynamic allocation struct > >

| 0 1 < Uplink Fixed allocation: < Fixed allocation struct > >

| 1 0 | 1 1 < extension > } } ;

< extension > : : =	 -- Future extension can be done by modifying this structure

null ;

< Frequency_Parameters struct > ::=

< RFqN: bit (8) >

{ 0 | 1 < Basic Midamble&Scrambling_Code: < Basic Midamble&Scrambling_Code struct > > } ;

< Basic Midamble&Scrambling_Code struct > :: =

< Basic Midamble Number: bit (2) >

{ 0 | 1 < Code Group: bit (5) > } ;

Table 62: Packet RESOURCE RECONFIGURE information element details

PAGE_MODE (2 bits)

This field is defined in sub-clause 12.10.

Message_State_I (bit)

This field indicates whether this message describes incremental or global packet resources. It is coded as follows:

0	Incremental resource description

1	Global resource description

Global TFI (6 bits)

This field identifies the uplink TFI, if available, or the downlink TFI, to which this message applies. This field is defined in sub-clause 12.7.

CHANNEL_CODING_COMMAND (4 bits)

The Channel Coding Indicator field indicates the channel coding scheme that the UE shall use when transmitting on the uplink. If this field is not present, the UE shall refer to the value previously assigned. Its coding is defined in sub-clause 11.14.12.

DOWNLINK_RLC_MODE (bit)

This field indicates the RLC mode of the requested TBF.

- RLC acknowledged mode

- RLC unacknowledged mode

If this field is not present, the UE shall refer to the value previously assigned.

CONTROL_ACK (bit)

This field shall be set to '1' if the network establishes a new downlink TBF for the UE whose timer T3192 is running. Otherwise this field shall be set to '0'. If this field is not present, the UE shall refer to the value previously assigned.

FBN (4 bits)

This filed defines the new assigned frequency band. It is encoded as the FBN field described in TSM04.08 [11].

Frequency_Parameters

This structure, if present, defines the new assigned frequency and optionally, the Basic Midamble Number and the Code Group information fields. It is encoded as reported in sub-clause 11.4.13.

DOWNLINK_TFI_ASSIGNMENT (5 bits)

This information element, if present, assigns the contained TFI to the UE to identify a downlink TBF described by this message. This field is coded the same as the TFI field defined in sub-clause 12.7.

UPLINK_TFI_ASSIGNMENT (5 bits)

This information element, if present, assigns the contained TFI to the UE to identify an uplink TBF described by this message. This field is coded the same as the TFI field defined in sub-clause 12.7.

Power control Parameters

This information element is defined in sub-clause 11.4.9.

Timing Control parameters

This information element is defined in sub-clause 11.4.9.

Power_Reference (4 bits)

This information element is defined in sub-clause 11.4.4.

Measurement Mapping construction

This information element is defined in sub-clause 11.4.4.

RLC_DATA_BLOCKS_GRANTED (8 bits)

The RLC/MAC blocks Granted field assigns a fixed number of RLC data blocks that the UE shall transmit during the uplink TBF. If the RLC_DATA_BLOCKS_GRANTED field is present, the UE shall transmit only the assigned number of RLC data blocks. Otherwise, the duration of the uplink TBF is undefined. Retransmissions of negatively acknowledged RLC data blocks do not apply toward the maximum number. Its coding is defined in sub-clause 11.4.13.

Channel Mode (5 bits)

This field defines the pilot channel configuration. Its coding is defined in sub-clause 11.4.4.

HALF_DUPLEX_MODE (bit)

This information field indicates whether the UE shall operate in half duplex mode. Its coding is defined in sub-clause 11.4.13.

Resource_Units struct

It is defined in sub-clause 11.4.4.

Dynamic Allocation struct

It is defined in sub-clause 11.4.13.

Fixed Allocation struct

It is defined in sub-clause 11.4.13.

TBF Starting Time

This field is defined in sub-clause 11.4.13.

Access Priority Parameters (13 bits)

If set, this field requests the UE to send SYNC1 bursts till the network acknowledgement before accessing on the uplink resources. Its coding is defined in sub-clause 11.4.10.

### 11.4.15	Packet Paging Request

This message is sent on PACCH to a user equipment in packet transfer mode to indicate page request for RR connection establishment. Depending on the method used to identify the user equipment, 1 - 4 user equipments can be addressed in the message. The user equipments are identified by either IMSI or TMSI.

Message type:	PACKET PAGING REQUEST MESSAGE

Direction:			network to user equipment

Classification:	distribution message

Table 63: PACKET PAGING REQUEST message content

< Packet Paging Request message content > ::=

< PAGE_MODE : bit (2) >

< NLN : bit (2) >

{	{ 1 < Repeated Page info : < Repeated Page info struct > > } ** 0

< padding bits > } //		-- truncation at end of message allowed, bits ‘0’ assumed

! < Distribution part error : bit (*) = < no string > > ;

< Repeated Page info struct > ::=

{							-- Page request for RR conn. establishment

{ 0	< TMSI : bit (32) >

| 1	< Length of Mobile Identity contents : bit (4) >

< Mobile Identity : octet (val (Length of Mobile Identity contents)) > }

< CHANNEL_NEEDED : bit (2) >

{ 0 | 1 < eMLPP_PRIORITY : bit (3) > } }

! < Ignore : bit (*) = <no string> > ;

Table 24: PACKET PAGING REQUEST information element details

PAGE_MODE (2 bit field)
This field is defined in clause 12.10.

NLN (2 bit field)
Notification List Number. The field is coded as defined in the P1 Rest Octets information element in TSM 04.08.

Repeated Page info struct
The Repeated Page info struct is repeated as many times as required to fulfil the number of wanted paged users. If the Paging Request Message is used with only TMSIs, the field can be repeated up to four times within one message. If the Paging Request Message is used with only IMSIs, the field can be repeated up to two times within one message.

The page request contains a Channel Needed and optionally a Priority parameter and can either be addressed with TMSI or IMSI.

Mobile Identity (variable length octet string)
This octet string is the representation of the Mobile Identity. It shall provide the international mobile subscriber identity, IMSI. The encoding of this octet string is the value part (starting with octet 3) of the type 4 information element Mobile Identity defined in TSM 04.08.

Any value other than IMSI for the type of identity in this octet string is spare. Such mobile identity shall be disregarded by the receiver but any further occurrence of the Repeated Page Info struct in the message shall be analysed.

Table 25 (continued): PACKET PAGING REQUEST information element details

TMSI (32 bit field)
TMSI is a unique Temporary Mobile Subscriber Identity. TMSI is associated with the mobile subscriber and defined in GSM 03.03. This field is coded as a binary number.
Range 0 to 4294967295

CHANNEL_NEEDED (2 bit field)
The channel needed field indicates which type of channel is needed for the user equipment for the transaction linked to the paging procedure. The field is coded according to following table:

bit
2 1
0 0		Any channel
0 1		Signalling only
1 0		TCH/F (Full rate)
1 1		TCH/H or TCH/F (Dual rate)

eMLPP_PRIORITY (3 bit field)
The optional eMLPP_PRIORITY field relates to User Equipment Identity i(i = 1,2,3,4). The eMLPP_PRIORITY field is coded as the Priority field defined in the P1 Rest Octets information element in TSM 04.08.

# 12	Information element coding

## 12.1	Overview

Information elements used within the context of only one RLC/MAC control message are defined in clause 11. All other information elements are defined within the present clause.

## 12.2	Extension Bits IE

The Extension Bits information element is used to provide a generalized means for possible future extension within a message. This information element is variable length and contains the length indicator and spare bits. It also contains the TBF Ending Time field used by the network to signal to the UE the waiting time before releasing an uplink TBF.

Table 64: EXTENSION BITS information element

< Extension Bits IE > ::=

< extension length : bit (7) >

{ 0 | 1 < TBF Ending Time : < Starting Frame Number Description IE >  > }

< spare bit (val(extension length)+1) > ;

Table 65: EXTENSION BITS information element details

TBF Ending Time This field controls the waiting time, at the UE side, before releasing the resources of the released uplink TBF. The TBF Ending Time field contains the indication of the frame number at which the still active resources of the released uplink TBF shall be released. The TBF Ending Time can be included in the PACKET UPLINK ACK/NACK message when the Final Ack Indicator bit is set to ‘1’ (see sub-clauses see sub-clauses 9.3.2.3 and 9.3.3.3).

This information element is defined in sub-clause 12.11.

## 12.3	Ack/Nack Description

The Ack/Nack Description information element contains the RLC parameters used to acknowledge or negatively acknowledge a group of RLC data blocks.

Table 66: ACK/NACK DESCRIPTION information elements

< Ack/Nack Description IE > ::=

< FINAL_ACK_INDICATION : bit (1) >

< MORE: bit(1)>

< REFERENCE_SEQUENCE_NUMBER : bit (12) >

< BEGINNING_OF_WINDOW : bit (1) >

< END_OF_WINDOW : bit (1) >

< LENGTH: bit (7)>

< RECEIVED_BLOCK_BITMAP : bit (val(LENGTH)) > ;

Table 67: ACK/NACK DESCRIPTION information element details

FINAL_ACK_INDICATION (1 bit )
This field indicates whether the entire TBF is being acknowledged. If the entire TBF is being acknowledged, the RSN and RBB fields contain no information and shall be ignored.

0	retransmission are requested and the TBF is incomplete
1	no retransmissions are requested and this message indicates acknowledgement of all RLC data in the TBF

MORE (1 bit)

This field indicates if other acknowledge information are available.

0	There are no other acknowledge information available.

1	There are other acknowledge information available

REFERENCE_SEQUENCE_NUMBER (RSN) (12 bit)
The RSN is a reference point and contains the BSN value following that of the last RLC block represented in the RBB bitmap. This field is encoded as the binary representation modulo 2*WS of the BSN.
Range from 0 to (2*WS)-1.

BEGINNING_OF_WINDOW (BOW, 1 bit )
This bit indicates if the Ack/Nack bitmap starts at the beginning of the window.
0	Beginning of the receiver window, V(Q), is not explicitly included in the bitmap.
1	Beginning of the receiver window, V(Q), is explicitly included in the bitmap.

END_OF_WINDOW (EOW, 1 bit )
This bit indicates if the Ack/Nack bitmap ends at the end of the window..
0	RSN not equal to V(R).
1	RSN equal to V(R).

LENGTH (L, 7 bit)

This field indicates the length L in bits of the bitmap (RBB). Range 0 to 127.

RECEIVE_BLOCK_BITMAP (RBB) (L bit)
The RBB is a bitmap representing Block Sequence Numbers. The number of bits (L bits) available for the bitmap depends on the inclusion or exclusion of other information elements in the messages using ACK/NACK Description IE. BOW, EOW, RSN, L and BSN are coded according to the following rules:

If (V(R) – V(Q)) modulo (2*WS) fits in the available number of bits for RBB coding and it is <= 127.

then BOW = 1 and EOW = 1

L = (V(R) – V(Q)) modulo (2*WS)

RSN = V(R)		BSN = (RSN - bit_number) modulo (2*WS)		for bit_number = 1to L.

If (V(R) – V(Q)) modulo (2*WS) does not fit in the available number of bits for RBB coding or it is > 127.

for BOW = 1 and EOW = 0 then

RSN = V(Q) + L	 	BSN = (RSN - bit_number) modulo (2*WS)	 	for bit_number = 1 to L,

The configuration BOW = 0 is reserved.

The value of each bit is encoded as:

0	Negative acknowledgement of the RLC data block with BSN = (RSN - bit_number) mod (2*WS)
1	Positive acknowledgement of the RLC data block with BSN = (RSN - bit_number) mod (2*WS)

## 12.4	ALLOCATION_BITMAP

The ALLOCATION_BITMAP represents uplink radio blocks, each bit representing an entire block period. Each bit indicates whether the user equipment is permitted to transmit during the corresponding uplink radio block period.

Table 68: ALLOCATION_BITMAP information element details

ALLOCATION_BITMAP (variable length field)

The ALLOCATION_BITMAP represents uplink radio block periods, each bit representing an entire radio block.

The bitmap describes a one dimensional array of block periods. For each block period indicated as part of the allocation in the bitmap, each of the timeslots indicated in the Time_Slot_State_Flag is assigned as part of the allocation. The array is indexed as follows:

block period[z]

z = n			for n = 0 to L,

where:

L = number of bits in the ALLOCATION_BITMAP - 1;

z = block period relative to TBF Starting Time;

n = bit number index into the ALLOCATION_BITMAP, range 0 to L;

TBF Starting Time indicates the first block period of the assigned allocation;

NTS = number of timeslots assigned in the Time_Slot_State_Flag, range 1 to 7.

The value of each bit is encoded as:

- block period[n] is not part of the assigned allocation

- block period[n] is part of the assigned allocation.

## 12.5	Channel Request Description

The Channel Request Description information element is sent by the user equipment to the network to request uplink resources.

Table 69: CHANNEL REQUEST DESCRIPTION information elements

< Channel Request Description IE > ::=

< PEAK_THROUGHPUT_CLASS : bit (4) >

< RADIO_PRIORITY : bit (2) >

< RLC_MODE : bit (1) >

< LLC_ PDU_TYPE : bit (1) >

< RLC_OCTET_COUNT : bit (16) > ;

Table 70: CHANNEL REQUEST DESCRIPTION information element details

PEAK_THROUGHPUT_CLASS (4 bit field)
This field indicates the peak throughput class for the PDP context of the LLC PDU that caused the Channel Request Description IE to be transmitted. The field is coded as the binary representation of the Peak Throughput Class specified in TSM 03.60.
Range: 1 to 9

RADIO_PRIORITY (2 bit field)
This field indicates the Radio Priority of the requested TBF. The field is encoded as the Radio Priority field of the Packet downlink Ack/Nack (see 11.3.4).

RLC_MODE (1 bit field)
This field indicates the RLC mode of the requested TBF.

0	RLC acknowledged mode
1	RLC unacknowledged mode

LLC_ PDU_TYPE (1 bit field)
This field indicates the type of the first LLC PDU to be transmitted over the requested uplink TBF.

0	LLC PDU is SACK or ACK
1	LLC PDU is not SACK or ACK

RLC_OCTET_COUNT (16 bit field)
The RLC_OCTET_COUNT field indicates the number of RLC data octets, plus the number of RLC data block length octets, that the user equipment wishes to transfer. The value '0' shall be interpreted as a request for an open ended TBF by the user equipment. All other values shall be interpreted as a request for a close-ended TBF. 
Range 0 to 65535

## 12.6	Global TFI

The Global TFI (Temporary Flow Identity) information element contains either an uplink TFI or a downlink TFI. The uplink or downlink TFI identifies a single Temporary Block Flow.

Table 71: GLOBAL TFI information elements

< Global TFI IE > ::=

{ 0 < UPLINK_TFI : bit (5) >

| 1 < DOWNLINK_TFI : bit (5) > } ;

Table 72: GLOBAL TFI information element details

UPLINK_TFI (5 bit field)
This field identifies an uplink TBF. This field is coded the same as the TFI field defined in subclause 12.7.

DOWNLINK_TFI (5 bit field)
This field identifies a downlink TBF. This field is coded the same as the TFI field defined in subclause 12.7.

## 12.7	Temporary Flow Identity (TFI)

The Temporary Flow Identity (TFI) uniquely identifies either a single uplink Temporary Block Flow (TBF) or a single downlink Temporary Block Flow (TBF).

Table 73: UPLINK_TFI information element details

UPLINK_TFI (5 bit field)
The Temporary Flow Identity field identifies an uplink Temporary Block Flow (TBF). This field is encoded as a binary number.
Range 0 to 31

Table 74: DOWNLINK_TFI information element details

DOWNLINK_TFI (5 bit field)
The Temporary Flow Identity field identifies a downlink Temporary Block Flow (TBF). This field is encoded as a binary number.
Range 0 to 31

## 12.8	Temporary Logical Link Identity (TLLI)

The Temporary Logical Link Identity (TLLI) is associated with the TD-PRS subscriber. TLLI is defined in TSM 03.03.

Table 75: TLLI information element details

TLLI (32 bit field)
The TLLI field is encoded as a binary number.
Range 0 to 4294967295

## 12.9	Time_Slot_State_Flag

The Time_Slot_State_Flag field indicates the timeslots for use during a TBF.

Table 76: Time_Slot_State_Flag information element details

Time_Slot_State_Flag (7 bit field)
This information field indicates the timeslots assigned for use during the TBF. Bit 7 indicates the status of timeslot 0, bit 6 indicates the status of timeslot 1, etc. At least one timeslot must be assigned.

0	Timeslot is not assigned
1	Timeslot is assigned

## 12.10	PAGE_MODE

The PAGE_MODE field controls the action of the user equipment belonging to the paging subgroup corresponding to the paging subchannel.

Table 77: PAGE_MODE information element details

PAGE_MODE (2 bit field)

bit
2 1		value
0 0		Normal Paging
0 1		Extended Paging
1 0		Paging Reorganization
1 1		Same as before

## 12.11 Starting Frame Number Description

There are two types of encoding for this IE : Relative Frame Number or Absolute Frame Number.

Table 78: STARTING FRAME NUMBER DESCRIPTION information element

< Starting Frame Number Description IE > ::=

{ 0 < Absolute Frame Number Encoding >

| 1 < Relative Frame Number Encoding > } ;

### 12.11.1	Absolute Frame Number Encoding

In this case, the field is encoded as the 16-bit Starting Time IE defined in TSM 04.08, and the value of the Starting FN is obtained directly.

### 12.11.2	Relative Frame Number Encoding

In this case, the field indicates the delay, relative to the first TDMA frame (N) of the RLC/MAC block containing the Starting Time field, before the assigned or requested resource becomes valid.

The value of this field is the 13-bit binary representation of the integer k, from which the offset to be applied to N can be derived.

The value of the Starting Frame Number is :

N + 4 + 4 k

Example :

Starting Frame Number Description (13-bit field)

k = 1			0000000000001		block with first TDMA frame number = N+8

k = 2			0000000000010		block with first TDMA frame number = N+12

k = 3			0000000000011		block with first TDMA frame number = N+16

Note 1: The value of (k+1) gives the number of relative blocks. The maximum number of relative blocks is therefore 8192.

Note 2: The value (k=0) should not be used, so as to leave time for the UE to analyse the message and get ready to receive or transmit.

## 12.12	Network_Control_Order

Table 79: NETWORK_CONTROL_ORDER information element

NETWORK_CONTROL_ORDER (2 bit field)
The NETWORK_CONTROL_ORDER field is coded according to the following table (for definition of NCx see TSM 05.08[15]):

bit
2 1
0 0		NC0 
0 1		NC1 
1 0		NC2 
1 1		Reserved

If the NETWORK_CONTROL_ORDER parameter = NC0, then the other parameters in the NC Measurement parameters struct may be omitted. If the NETWORK_CONTROL_ORDER parameter indicates either NC1 or NC2, at least one of the two immediately following optional parameters shall be reported; the missed one shall be interpreted as if no measurement report is requested for the relevant mode, respectively packet idle mode (I) or packet transfer mode (T).

## 12.13	NC_NON_DRX_PERIOD

Table 80: NC_NON_DRX_PERIOD information element

NC_NON_DRX_PERIOD (3 bit field)
This field indicates the minimum time the user equipment shall stay in non-DRX mode after an NC-measurement report has been sent. The field is coded according to the following table:

bit
3 2 1
0 0 0		No non-DRX mode after a measurement report has been sent (default value). 
0 0 1		0,24 sec 
0 1 0		0,48 sec 
0 1 1		0,72 sec 
1 0 0		0,96 sec 
1 0 1		1,20 sec
1 1 0		1,44 sec 
1 1 1		1,92 sec

## 12.14	NC_REPORTING_PERIOD_I / NC_REPORTING_PERIOD_T

Table 81: NC_REPORTING_PERIOD_I/NC_REPORTING_PERIOD_T information elements

NC_REPORTING_PERIOD_I (3 bit field)

NC_REPORTING_PERIOD_T (3 bit field)
These fields indicate the time period for cell reselection measurement reporting for packet idle mode (I) and packet transfer mode (T), respectively. The field is coded according to the following table:

bit
3 2 1
0 0 0		0.24 sec 
0 0 1		0.48 sec 
0 1 0		0.96 sec 
0 1 1		1.92 sec

1 0 0		3.84 sec 
1 0 1		7.68 sec 
1 1 0		15.36 sec 
1 1 1		30.72 sec

# 13	Timers and counters

The tables in clause 13.1 and 13.2 specifies the timers used in RLC/MAC protocol signalling. The denotation of columns is defined as follows:

timer ::=					name of the timer;

started ::=				under which conditions the timer is started;

stopped ::=				under which conditions the timer is stopped;

action at expiry ::=			which actions the TD-PRS entity shall perform at expiry;

value ::=					the duration between setting the timer and expiry of the timer ("s" denotes

"second(s)" "xx - yy" means that any value between xx and yy is permitted).

## 13.1	Timers on the UE side

Table 82: Specification of timers used in TD-PRS on the UE side

| Timer | Started | Stopped | Action at expiry | Value |
| --- | --- | --- | --- | --- |
| T3142 | On receipt of an IMMEDIATE ASSIGNMENT REJECT message containing a WAIT_INDICATION field. | On receipt of an IMMEDIATE ASSIGNMENT message. | UE shall reinitiate the PACKET ACCESS procedure unless it has already been reinitiated 4 times. Otherwise UE returns to packet idle mode. | Assigned in message |
| T3146 | Started when T3142 is started or when the UE sends a CHANNEL REQUEST message. | On the reception of an IMMEDIATE ASSIGNMENT (also assigning a DL TBF) or at the reception of an IMMEDIATE ASSIGNMENT REJECT without a WAIT_INDICATION field. | If the number of sent CHANNEL REQUEST messages has not yet reached its maximum value, the UE can re-start the packet access procedure, otherwise a packet access failure is declared and the UE shall return to packet idle mode. In case of measurement reporting: Measurement Report initiating the packet access is discarded and the UE returns to packet idle mode. | assigned in system information |
| T3158 | Started when ordered by a NETWORK_CONTROL_ORDER and then restarted each time a Network Controlled (NC) Measurement is performed in MM Ready state and in packet idle or packet transfer mode | See 05.08 | Restart the timer, perform the measurement and send a NC Measurement report. The timer shall be restarted with either of the parameters NC_REPORTING_PERIOD_I when in packet idle mode or with the parameter NC_REPORTING_PERIOD_T when in packet transfer mode | Defined by the parameter or by a random value (see TSM 05.08) |
| T3166 | At sending of the first RLC/MAC block at one phase access or pilot packet access. | On receipt of a PACKET UPLINK ACK/NACK message (even of PACKET DOWNLINK ASSIGNMENT for pilot access) with the same TLLI as the UE has included in the RLC header of the first RLC data block. | For pilot access: Immediately stop transmitting on the assigned TBF and reinitiate the packet access procedure unless it has already been repeated 4 times, otherwise a TBF failure has occurred occurred and the UE remains in packet idle mode. For one phase access: stop transmitting on the involved TBF. | 5 sec |
| T3168 | At sending the PACKET RESOURCE REQUEST message. | On receipt of a PACKET UPLINK ASSIGNMENT, a PACKET RESOURCE RECONFIGURE, PACKET ACCESS REJECT or a PACKET UPLINK ACK/NACK message. | Contention resolution has failed on the UE side. The UE shall then reinitiate the packet access procedure unless it has already been repeated 4 times. In that case, TBF failure has occurred and an RLC/MAC error should be reported to the higher layer layer in case of two phase packet access (sub-clause 8.1.3.2.1); UE performs abnormal release with random access in case of: Fixed (sub-clause 8.1.1.3.2), Dynamic (sub-clause 8.1.1.2.3) allocation, establishment of uplink TBF (sub-clause 8.1.2.4). | assigned in system information |
| Table 82 (concluded): Specification of timers used in TD-PRS on the UE side |  |  |  |  |
| Timer | Started | Stopped | Action at expiry | value |
| T3172 | On receipt of a PACKET ACCESS REJECT message | On receipt of PACKET UPLINK ASSIGNMENT message | UE reinitiate packet access procedure unless it has already been reinitiated 4 times; otherwise UE returns to packet idle mode and notify to higher layers a TBF establishment failure. If a downlink TBF is active the UE initiates the establishment of an uplink TBF as described in sub-clause 8.1.2.4. | Assigned in message |
| T3174 | On receipt of a PACKET CELL CHANGE ORDER message. | On receipt of a response to CHANNEL REQUEST in the new cell. | Return to old cell and start timer T3176. | 5 sec |
| T3176 | Expiry of T3174, IMMEDIATE ASSIGNMENT REJECT message is received from the new cell, or, if the contention resolution procedure fails on the new cell. | After sending of PACKET CELL CHANGE FAILURE message. | Performs the abnormal release with random access or with return to CCCH. | 5 sec |
| T3178 | Started when ordered by a EXT_MEASUREMENT_ORDER and then restarted each time an extended (EXT) Measurement is performed in MM Ready state and in packet idle mode or packet transfer mode. | See 05.08. | Restarts the timer, performs the measurements and sent an EXT Measurement report. The timer shall be restarted with either of the parameters EXT_REPORTING_PERIOD_I when in packet idle mode or with the parameter EXT_REPORTING_PERIOD_T when in packet transfer mode. | Defined by the parameter or by a Random value (see TSM 05.08) |
| T3180 | When transmitting an RLC/MAC block to the network in case of dynamic allocation. | When detecting an assigned USF value. | Perform Abnormal release with random access procedure if no P-PDCH is active, otherwise, the UE continues to operate on the assigned resources as described in sub-clause 8.1.1.2. | 5 sec |
| T3182 | After sending the last data block (with CV = 0), or Upon detecting a transmit window stall condition. | On receipt of the PACKET UPLINK ACK/NACK message. | Abnormal release with random access. | 5 sec |
| T3184 | On receipt of a PACKET UPLINK ACK/NACK message. | On receipt of PACKET UPLINK ACK/NACK message T3184 is also restarted. | Abnormal release with cell reselection. | 5 sec |
| T3188 | If a new fixed allocation has been requested, when all data has been sent on the assigned allocation. | On receipt of PACKET UPLINK ASSIGNMENT, PACKET RESOURCE RECONFIGURE, PACKET UPLINK ACK/NACK message containing a fixed allocation, or PACKET ACCESS REJECT. | If no P-PDCH is active, the UE shall perform abnormal release with random access, otherwise continue to operate on the assigned resources. | 5 sec |
| T3190 | At reception of a PACKET DOWNLINK ASSIGNMENT or a PACKET RESOURCE RECONFIGURE. | Restarted on receipt of a valid RLC/MAC data block addressed to itself. | UE performs abnormal release with return to CCCH, if no P-PDPCH is active, otherwise continues to operate on the assigned resources. | 5 sec |
| T3198 | When transmitting RLC data block. | None | Accept negative acknowledgement for RLC data block. | see clause 9.1.4 |
| T3200 | On receipt of an RLC/MAC control block containing a segment of an RLC/MAC control message. | On receipt of an RLC/MAC control block containing a segment of an RLC/MAC control message such that the user equipment now has the complete control message. | Discard and ignore all segments of the partially received RLC/MAC control message. | see clause 9.1.15 |
| T3204 | Timer T3204 shall be started (only once in case of TBF network initiated, otherwise up to 4 times) at the sending of PACKET ACCESS message. | On reception of a PACKET ACCESS ACK . | The UE returns to packet idle mode (sub-clause 7.2.2), or remains in packet idle mode and notify to higher layer a TBF establishment failure (sub-clause 7.1.4). The user equipment assumes failed the P-PDPCH packet access procedure, stop all running timers, notify a TBF failure to higher layers and perform an abnormal release with random access (sub-clauses 8.1.1.1). The UE reinitiates the packet access procedure unless it has already been repeated 4 times. In that case, TBF failure shall be assumed and an RLC/MAC error should be reported to the higher layer (sub-clause 7.1.2.3) | assigned in system information |
| T3224 | Started at the reception of an OUT_OF_SYNC state indication. | Stopped and reset at the detection of an IN_SYNC state indication. Stopped and reset whenever the affected P-PDPCH(s) is reconfigured by the network. | Radio link failure shall be declared and abnormal release procedures shall be performed. | assigned in system information |

T3142	Wait for Immediate Assignment after an Immediate Assignment Reject message has been received.

This timer is used on the user equipment side on receipt of an Immediate Assignment Reject message corresponding to one of the user equipment’s 3 last Channel Request messages. If T3142 expires before receiving an assignment message, the user equipment returns to packet idle mode.

T3146	Started when T3142 is started or when the UE sends a CHANNEL REQUEST message. This timer is stopped on the reception of an IMMEDIATE ASSIGNMENT or at the reception of an IMMEDIATE ASSIGNMENT REJECT without a WAIT_INDICATION field. The coding is as follows:

| Bits |  |  | Meaning |
| --- | --- | --- | --- |
| 3 | 2 | 1 |  |
| 0 | 0 | 0 | 40 ms (8 frames) |
| 0 | 0 | 1 | 80 ms (16 frames) |
| 0 | 1 | 0 | 160 ms (32 frames) |
| 0 | 1 | 1 | 320 ms (64 frames) |
| 1 | 0 | 0 | 640 ms (128 frames) |
| 1 | 0 | 1 | 1280 ms (256 frames) |
| 1 | 1 | 0 | 2560 ms (512 frames) |
| 1 | 1 | 1 | 5120 ms (1024 frames) |

T3158:	Wait for sending measurement reports for network controlled cell reselection.

This timer is used on the user equipment side to define the period for performing NC-measurements and send measurement reports in either packet idle or packet transfer mode (see TSM 05.08).

T3166:	Wait for Packet Uplink ACK/NACK or PACKET DOWNLINK ASSIGNMENT, in case of pilot access, after sending of first RLC/MAC data block

This timer is used on the user equipment side to define when to stop waiting for a Packet Uplink ACK/NACK message or PACKET DOWNLINK ASSIGNMENT, in case of pilot access , with the same TLLI as the UE has included in the RLC header of the first RLC data block .

T3168:	Wait for a packet uplink assignment message

This timer is used on the user equipment side to define when to stop waiting for a packet uplink assignment message after sending of a Packet Resource request message. The timeout value of timer T3168 is given as the binary value, ranging from 0 to 7, plus one in units of 500 millisecond.

T3172:	Wait for Packet Uplink Assignment after Packet Access Reject message has been received.

This timer is used on the user equipment side on receipt of a Packet Access Reject message corresponding to one of the user equipment’s 3 last Packet Channel Request messages. If T3172 expires before receiving an assignment message, the user equipment returns to packet idle mode.

After T3172 expiry the UE shall reinitiate packet access procedure unless it is already been reinitiated 4 times; otherwise the UE shall return to idle mode and notify to higher layers a TBF establishment failure. If a downlink TBF is active the UE initiates the establishment of an uplink TBF as described in sub-clause 8.1.2.4.

The UE shall not send Channel Request message as a response to a page until a Paging Request message for the user equipment is received.

T3174:	Wait for response on new cell after Packet Cell Change Order .

This timer is used on the user equipment side on receipt of a PACKET CELL CHANGE ORDER message. The timer is stopped upon successful access on the new cell. On expiry, the user equipment returns to the old cell and start timer T3176.

T3176:	Stop Cell Change failure procedure.

This timer started when T3174 expires. The timer is stopped upon transmission of the PACKET CELL CHANGE FAILURE message. On expiry, the user equipment performs the abnormal release with random access or with return to CCCH.

T3178:	Wait for sending extended measurement reports.

This timer is used on the user equipment side to define the period for performing extended measurements and send extended measurement reports either in packet idle mode or in packet transfer mode (see TSM 05.08).

T3180:	Wait for User State Flag after Data Block

This timer is used on the user equipment side to define when to stop waiting for the USFdetermining the assigned portion of the uplink shared channel after the previous RLC/MAC block is sent. If expired, the user equipment repeats the procedure for random access, if no P-PDCH is active; otherwise, the UE shall continue to operate on the assigned resources as described in sub-clause 8.1.1.2. This timer does not apply to P-PDCH(s).

T3182:	 Wait for Acknowledgement

This timer is used on the user equipment side to define when to stop waiting for temporary Packet Uplink Ack/Nack after the last RLC data block has been sent for the current send window or for the entire Temporary Block Flow.

T3184:	No Ack/Nack Received

This timer is used on the user equipment side to decide when to stop waiting for a Packet Uplink Ack/Nack. (This timer does not apply to mobiles performing a dynamic allocation transfer).

T3188:	Allocation Exhausted

This timer is used on the user equipment side to decide when to stop waiting to receive additional resources from the network. (This timer does not apply to a UE performing a dynamic allocation transfer).

T3190:	Wait for Valid Downlink Data Received from the Network

This timer is used on the user equipment side to stop waiting for the valid data from the network side either following the initial PACKET DOWNLINK ASSIGNMENT or PACKET RESOURCE RECONFIGURE message or after some previous downlink RLC/MAC block.

T3198:	RLC timer

T3198 is used by the user equipment to control when it will accept a negative acknowledgement for an RLC data block.

T3200	RLC/MAC control message reassembly guard.

T3200 is used by the user equipment to control when it will discard segments of a partially received RLC/MAC control message. The user equipment shall have one instance of timer T3200 for each segmented RLC/MAC control message that the user equipment is capable of receiving in parallel.

T3204	T3204 is activated by the UE when sending a PACKET ACCESS message. PACKET ACCESS messages are sent till the reception, from the downlink P-PDPCH, of a PACKET ACCESS ACK message or the T3204 expiry. If a PACKET ACCESS ACK message is received, the UE shall stop this timer. If this timer expires the UE  shall follow the behaviour described respectively either in sub-clause 7.1.2.3 or sub-clause 7.1.4 or sub-clause 7.2.2 or sub-clause 8.1.1.1. This timer is coded using 3 bits, according to the following table:

| Bits |  |  | Meaning |
| --- | --- | --- | --- |
| 3 | 2 | 1 |  |
| 0 | 0 | 0 | 40 ms |
| 0 | 0 | 1 | 80 ms |
| 0 | 1 | 0 | 160 ms |
| 0 | 1 | 1 | 320 ms |
| 1 | 0 | 0 | 640 ms |
| 1 | 0 | 1 | 1280 ms |
| 1 | 1 | 0 | 2560 ms |
| 1 | 1 | 1 | 5120 ms |

T3224 	T3224 is started, at the UE side, at the reception of an OUT_OF_SYNC state indication. Timer T3224 shall be stopped and reset both at the detection of an IN_SYNC state indication and whenever the affected P-PDPCH(s) resource is reconfigured by the network. If this timer expires the UE shall perform abnormal release procedures. This timer is coded using 3 bits, according to the following table:

| Bits |  |  | Meaning |
| --- | --- | --- | --- |
| 3 | 2 | 1 |  |
| 0 | 0 | 0 | 40 ms |
| 0 | 0 | 1 | 80 ms |
| 0 | 1 | 0 | 160 ms |
| 0 | 1 | 1 | 320 ms |
| 1 | 0 | 0 | 640 ms |
| 1 | 0 | 1 | 1280 ms |
| 1 | 1 | 0 | 2560 ms |
| 1 | 1 | 1 | 5120 ms |

## 13.2	Timers on the network side

Table 83: Specification of timers used in TD-PRS on the Network side

| Timer |  | Started |  | Stopped |  | action at expiry |  | typical value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T3141 |  | At the sending of the IMMEDIATE ASSIGNMENT message granting a one phase packet access. |  | Successful completion of one phase packet access procedure, UE in packet transfer mode. |  | newly allocated TBF is released. |  | Configurable by the network operator. |  |
| T3169 |  | If counter N3101 = N3101_MAX, or if counter N3103 = N3103_MAX. |  | None |  | The network releases USF, TFI and physical resources. |  | Configurable by the network operator. |  |
| T3191 |  | When the last RLC data block is sent with the FBI bit set to ‘1’. |  | When the final PACKET DOWNLINK ACK/NACK or PACKET CONTROL ACKNOWLEDGEMENT is received. Restarted at the transmission of an RLC/MAC data block with the FBI bit set to ‘1’. |  | The network releases TBF. |  | Configurable by the network operator. |  |
| T3195 |  | If counter N3105 = N3105_MAX. |  | None |  | The network releases TFI resources. |  | Configurable by the network operator. |  |
| T3208 |  | When sending IMMEDIATE ASSIGNMENT message (or at the TBF starting time) and switch monitoring assigned Uplink P-PDPCH. |  | At the reception of PACKET ACCESS. |  | Assumes P-PDPCH packet access procedure failed. |  | Configurable by the network operator. |  |
| T3228 | Started at the reception of an OUT_OF_SYNC indication. |  | Stop and reset at the detection of an IN_SYNC state indication. Stopped and reset whenever the affected P-PDPCH(s) is reconfigured. |  | Radio link failure of the affected P-PDCH(s) shall be declared, and initiated an abnormal release of the involved TBF. |  | Configurable by the network operator. |  |  |

T3141:	This timer is started when a temporary block flow is allocated with an IMMEDIATE ASSIGNMENT message during a packet access procedure granting a one phase packet access. It is stopped at the network side at the successful completion of the one phase contention resolution procedure.

Its value is network dependent.

T3169:	Wait for Reuse of USF and TFI after the user equipment uplink assignment is invalid

This timer is used on the network side to define when the current uplink assignment is surely invalid on the user equipment side so that the assigned USF and TFI can be reused on the uplink. During that period the corresponding USF is not broadcast. The value for T3169 is > T3180.

Its value is network dependent.

T3191:	Wait for reuse of TFI after sending of the last RLC Data Block

This timer is used on the network side to define when the current assignment is surely invalid on the user equipment side so that the TBF can be reused.

Its value is network dependent.

T3195:	Wait for reuse of TFI when there is no response from the UE (radio failure or cell change)

This timer is used on the network side to define when the current assignment is surely invalid on the user equipment side so that the TFI can be reused.

Its value is network dependent.

T3208	The network shall start timer T3208 when sending the IMMEDIATE ASSIGNMENT message or at the TBF starting time if signalled and switches to the assigned resources. On reception of the expected PACKET ACCESS block, the network shall stop T3208. If timer T3208 expires, or the received message from the UE is not the expected one, the network shall stop timer T3208 if still running and assume failed the P-PDPCH packet access procedure.

Its value is network dependent.

T3228 		T3228 is activated by the network, at the reception of an OUT_OF_SYNC state indication. Timer T3228 shall be stopped and reset both at the detection of an IN_SYNC_STATE indication and whenever the affected P-PDPCH(s) resource is reconfigured. If this timer expires a radio link failure of the affected P-PDPCH(s) shall be declared and an abnormal release of the involved TBF shall be initiated.

Its value is network dependent.

## 13.3	Counters on the User Equipment side

N3102:	At each cell reselection the user equipment shall set the counter N3102 to the value defined by the optional broadcast parameter PAN_MAX. Whenever the user equipment receives a Packet Ack/Nack that allows the advancement of V(S), the user equipment shall increment N3102 by the broadcast value PAN_INC, however N3102 shall never exceed the value PAN_MAX. Each time T3182 expires the user equipment shall decrement N3102 by the broadcast value PAN_DEC. When N3102  0 is reached, the user equipment shall perform an abnormal release with cell re-selection.

N3104:	When the user equipment sends the first RLC/MAC data block the counter N3104 shall be initialised to 1. For each new RLC/MAC data block the user equipment sends it shall increment N3104 by 1 until a PACKET UPLINK ACK/NACK message with the same TLLI as the user equipment has included in the RLC header of the first RLC data block is received. Then N3104 shall not be further incremented. If the N3104 counter is equal to N3104_MAX and no correct PACKET UPLINK ACK/NACK message has been received, the contention resolution fails and the user equipment behaves as specified in sub-clause 7.1.2.4.

N3104_MAX shall have the value:

N3104_MAX = 3 * BS_CV_MAX * number of uplink timeslots assigned.

N3106:	The value of this counter can be sent by the network to the UE on PACCH, during the P-PDCH maintenance procedure.

For this release, parameter N3106 is assumed to be equal to infinitive, this means that once entered the OUT_OF_SYNC state the UE shall never restart the maintenance procedure.

It represents the number of successive expected, correctly received, data blocks such that, after the expiry of the radio link counter, the P-PDCH maintenance procedure can be restarted.

Pk_UE:	Pk_UE is started, on the UE side, at the reception of a PACKET ACCESS ACK when T3204 is stopped. It is re-initialised each time a correct RLC/MAC data or control block is received, according to the rules specified in TSM 05.08 [15]. At the expiry of the radio link counter Pk_UE the UE shall act as described in TSM 05.08 [15]. When the UE receives, during an uplink packet transfer, a PACKET UPLINK ASSIGNMENT or PACKET DOWNLINK ASSIGNMENT (changing uplink pilot resources) or PACKET RESOURCE RECONFIGURE message, it shall behave as described in sub-clause 8.1.1.1.2. This counter is coded using 3 bits, according to the following table:

| Bits |  |  | Meaning (number of block periods) |
| --- | --- | --- | --- |
| 3 | 2 | 1 |  |
| 0 | 0 | 0 | 2 |
| 0 | 0 | 1 | 4 |
| 0 | 1 | 0 | 8 |
| 0 | 1 | 1 | 16 |
| 1 | 0 | 0 | 32 |
| 1 | 0 | 1 | 64 |
| 1 | 1 | 0 | 128 |
| 1 | 1 | 1 | 256 |

## 13.4	Counters on the Network side

N3101:	N3101max shall be greater than 8. Whenever the network receives from the UE a valid RLC/MAC block (and different from an RLC/MAC dummy control block for P-PDPCH channels), it shall reset counter N3101. The network shall increment counter N3101 for each radio block allocated to that user equipment, for which no data (or an RLC/MAC dummy control block for P-PDPCH channels) is received. If N3101 = N3101max, the network shall stop the scheduling of RLC/MAC blocks from the user equipment (or sending any sort of block to the user equipment on the assigned downlink P-PDPCH resources).

N3103:	N3103 is reset when transmitting the final PACKET UPLINK ACK/NACK message within a TBF (final ack indicator set to 1). If the network does not receive the PACKET CONTROL ACKNOWLEDGEMENT message in the scheduled block, it shall increment counter N3103 and retransmit the PACKET UPLINK ACK/NACK message. If counter N3103 exceeds its limit, the network shall start timer T3169.

N3105:	When the network receives a valid RLC/MAC control message from the user equipment, it will reset counter N3105. The network will increment counter N3105 for each radio block, allocated to that UE with the RRBP field, for which no RLC/MAC control message is received. If N3105=N3105max, the network shall release the downlink TBF internally. The value of N3105max is network dependent.

N3107:		The value of this counter is used on the network side, during the P-PDCH maintenance procedure. It represents the number of successive expected, correctly received, data blocks such that, after the expiry of the radio link counter, the L1 shall signal the re-establishment of the radio link synchronisation to the higher layers and restart the P-PDCH maintenance procedure.

Pk_NWK:	Pk_NWK is started, on the network side, at the reception of a PACKET ACCESS block when T3208 is stopped. It is re-initialised each time a correct RLC/MAC data or control block is received, according to the rules specified in TSM 05.08 [15]. At the expiry of the radio link counter Pk_NWK the network shall act as described in TSM 05.08 [15]. When the network performs a resource reassignment, by sending a PACKET DOWNLINK ASSIGNMENT or PACKET UPLINK ASSIGNMENT (changing downlink pilot resources) or PACKET RESOURCE RECONFIGURE message, it shall behave as described in sub-clause 8.1.2.1.2.

Its value is network dependent.

# Annex A (Informative):
Packet Resource Release

The network may release a sub-set of the assigned packet resources or all the assigned packet resources to a packet call by sending to the user equipment the PACKET RESOURCE RELEASE message.

PACKET RESOURCE RELEASE message contains the UNAVAILABLE_RU field defined in clause 11.4.7.

Here in the following are two examples of use of UNAVAILABLE_RU field.

Assuming the maximum number of codes per time slot (16), the UNAVAILABLE_RU field is represented as a bitmap, as shown in the Figure A1 below, where each bit corresponds to one specific basic Resource Unit (RU).

With reference to Figure A1, TSi represents the Time Slot number “i” and SCk represents the Spreading Code number “k” (see TSM 05.02 for details).

Figure A1: Association between UNAVAILABLE_RU field and allocated basic resource units

Example 1:

Assume a packet call having assigned, eventually in addition to others, the three following resource units:

RU a:	at SC 9 and on TS 3;

RU b:	at SC 8 and on TS 3;

RU c:	at SC 7 and on TS 3.

Note 1:	The above described RUs are basic resource units, having assigned Spreading Factor (SF) 16.

To release these 3 RUs, the correspondent bits in the UNAVAILABLE_RU field will be set to “1”, as reported in Figure A2.

Figure A2: resource units (SC=9, TS=3), (SC=8, TS=3) and (SC=7, TS=3) are released

Example 2:

Assume a packet call having assigned, eventually in addition to others, the two following resource units:

RU a:	at SC 16 and on TS 3;

RU b:	at SC 23 and on TS 3;

Note 2:	Each of the above described RUs, both at SF 8, corresponds to two basic resource units.

To release one of these 2 RUs, two bits in the UNAVAILABLE_RU field will be set to “1”; these 2 bits correspond to the two basic resource units whose RU to be released is the father leaf in the OVSF code tree (see TSM 05.02).

With reference to Figure A3 below, both RUs are released.

Figure A3: resource units (SC=23, TS=3) and (SC=16, TS=3) are released

# Annex B (informative):
RLC data block encoding

## B.1	Example 1

Figure B.1 provides an example of the use of the TS-SF indicator and PCI indicator when valid TS-SF and Transmit Power Command fields respectively, are available. TS-SFI field is set to 1 to indicate that a valid TS-SF field follows. For each assigned uplink Time Slot in the TS-SF field, Power Command Indicator field indicates the presence of the Transmit Power Command field associated. Transmit Power command field indicates to the addressed UE the network expected power level on the uplink Time Slots enabled via the TS-SF field. Transmit Power command field can be included only in conjunction with the TS-SF field. In this example a downlink RLC/MAC control block is used and 4 uplink Time Slots are assigned to the UE. Last PCI indicator field will be set to zero, no other Transmit Power Command fields are present.

| RLC/MAC control block N |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| TS-SFI |  |  |  |  |  |  |  |  |  |
| Payload Type |  | RRBP |  | S/P | USF/MC |  |  | MAC header |  |
| TS-SF |  |  |  |  |  |  | PCI | Octet 1 |  |
| Transmit Power command |  |  |  |  |  |  | PCI | Octet 2 |  |
| Transmit Power command |  |  |  |  |  |  | PCI | Octet 3 |  |
| Transmit Power command |  |  |  |  |  |  | PCI | Octet 4 |  |
| Transmit Power command |  |  |  |  |  |  | PCI | Octet 5 |  |
| RBSN | RTI |  |  |  |  | FS | AC | Octet 6 |  |
| Spare |  | TFI |  |  |  |  | D | Octet 7 |  |
| Control Message Content |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Octet 22 |  |
| Figure B.1: TS-SFI indicator example |  |  |  |  |  |  |  |  |  |

## B.2	Example 2

Figure B.2 provides an example of the use of the TS-SF indicator when no valid TS-SF and, consequently, Transmit Power Command fields are available. TS-SFI field is set to 0 to indicate that no valid TS-SF field follows.

| RLC/MAC control block N |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| TS-SFI |  |  |  |  |  |  |  |  |  |
| Payload Type |  | RRBP |  | S/P | USF/MC |  |  | MAC header |  |
| RBSN | RTI |  |  |  |  | FS | AC | Octet 1 |  |
| Spare |  | TFI |  |  |  |  | D | Octet 2 |  |
| Control Message Content |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Octet 22 |  |
| Figure B.2: TS-SFI indicator example |  |  |  |  |  |  |  |  |  |

## B.3	Example 3

Figure B.3 provides an example of the use of the Length indicator in conjunction with the M and E bits. In the example, LLC PDU 1 continues from a previous RLC data block and ends in the RLC data block shown. LLC PDU 2 follows LLC PDU 1 and is completely contained within the RLC data block. LLC PDU 3 follows LLC PDU 2, beginning in the RLC data block shown, and continues into the next RLC data block. Note that, in this example, it is assumed that 3 timeslots are allocated in uplink and the relative Transmit Power Command information is given.

| Downlink RLC data block N |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| TS-SFI |  |  |  |  |  |  |  |  |  |
| Payload Type |  | RRBP |  | S/P | USF/MC |  |  | MAC header |  |
| TS-SF |  |  |  |  |  |  | PCI | Octet 1 |  |
| Transmit Power command |  |  |  |  |  |  | PCI | Octet 2 |  |
| Transmit Power command |  |  |  |  |  |  | PCI | Octet 3 |  |
| Transmit Power command |  |  |  |  |  |  | PCI | Octet 4 |  |
| BSN |  | TFI |  |  |  |  | FBI | Octet 5 |  |
|  |  |  | BSN |  |  |  | E = 0 | Octet 6 |  |
| Length indicator = 4 |  |  |  |  |  | M = 1 | E = 0 | Octet 7 | LLC PDU 1 |
| Length indicator = 10 |  |  |  |  |  | M = 1 | E = 1 | Octet 8 |  |
|  |  |  |  |  |  |  |  | Octet 9 |  |
| LLC PDU 1 (cont) |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Octet 12 |  |
|  |  |  |  |  |  |  |  | Octet 13 |  |
|  |  |  |  |  |  |  |  | Octet 14 |  |
| LLC PDU 2 |  |  |  |  |  |  |  | . . . | LLC PDU 2 |
|  |  |  |  |  |  |  |  | Octet 22 |  |
|  |  |  |  |  |  |  |  | Octet 23 |  |
|  |  |  |  |  |  |  |  | Octet 24 |  |
| LLC PDU 3 |  |  |  |  |  |  |  | . . . | LLC PDU 3 |
|  |  |  |  |  |  |  |  | Octet N-1 |  |
|  |  |  |  |  |  |  |  | Octet N |  |

Figure B.3: Length indicator (LI) example

## B.4	Example 4

Figure B.4 provides an example of the use of the Length indicator when the end of an LLC PDU would fit within an RLC data block but the addition of the length indicator octet (to indicate the LLC PDU boundary) causes the LLC PDU to extend into another RLC data block. In the example, LLC PDU 1 continues from a previous RLC data block and has 17 remaining octets. The first 16 octets are placed into RLC data block N, the Length Indicator is set to 0 (to indicate that the LLC PDU does not end within the current RLC data block), and the 17th octet is placed in RLC data block N+1. Note that, in this example, it is assumed that 2 timeslots are allocated in uplink and the relative Transmit Power Command information is given.

| RLC data block N |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| TS-SFI |  |  |  |  |  |  |  |  |  |
| Payload Type |  | RRBP |  | S/P | USF/MC |  |  | MAC header |  |
| TS-SF |  |  |  |  |  |  | PCI | Octet 1 |  |
| Transmit Power command |  |  |  |  |  |  | PCI | Octet 2 |  |
| Transmit Power command |  |  |  |  |  |  | PCI | Octet 3 |  |
| BSN |  | TFI |  |  |  |  | FBI | Octet 4 |  |
|  |  |  | BSN |  |  |  | E = 0 | Octet 5 |  |
| Length indicator = 0 |  |  |  |  |  | M = 0 | E = 1 | Octet 6 |  |
|  |  |  |  |  |  |  |  | Octet 7 | LLC PDU 1 |
| LLC PDU 1 (cont) |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Octet 22 |  |
| RLC data block N + 1 |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| TS-SFI |  |  |  |  |  |  |  |  |  |
| Payload Type |  | RRBP |  | S/P | USF/MC |  |  | MAC header |  |
| TS-SF |  |  |  |  |  |  | PCI | Octet 1 |  |
| Transmit Power command |  |  |  |  |  |  | PCI | Octet 2 |  |
| . |  |  |  |  |  |  | . | . |  |
| Transmit Power command |  |  |  |  |  |  | PCI | Octet M |  |
| BSN |  |  | TFI |  |  |  | FBI | Octet M+1 |  |
|  |  |  | BSN |  |  |  | E = 0 | Octet M+2 |  |
| Length indicator = 1 |  |  |  |  |  | M = 1 | E = 1 | Octet M+3 (optional) |  |
| LLC PDU 1 (cont) |  |  |  |  |  |  |  |  |  |
| LLC PDU 2 |  |  |  |  |  |  |  | . . . | LLC PDU 2 |
|  |  |  |  |  |  |  |  | Octet 22 |  |

Figure B.4: Length indicator (LI) example

## B.5	Example 5

Figure B.5 provides an example of the use of the Length indicator when the end of an LLC PDU fits precisely into an RLC data block. In the example, LLC PDU 1 continues from a previous RLC data block and ends in the RLC data block shown. LLC PDU 2 follows LLC PDU 1 and fills precisely the RLC data block shown. TS-SFI field is set to zero.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| TS-SFI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Payload Type |  |  |  |  | RRBP |  |  |  | S/P |  | USF/MC |  |  |  |  | MAC header |  |
| BSN |  |  |  |  | TFI |  |  |  |  |  |  |  |  |  | FBI | Octet 1 |  |
|  |  |  |  |  |  |  | BSN |  |  |  |  |  |  |  | E = 0 | Octet 2 | LLC PDU 1 |
| Length indicator = 7 |  |  |  |  |  |  |  |  |  |  |  |  | M = 1 |  | E = 0 | Octet 3 |  |
| Length indicator = 11 |  |  |  |  |  |  |  |  |  |  |  |  | M = 0 |  | E = 1 | Octet 4 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 5 |  |
| LLC PDU 1 (cont) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | . . . |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 11 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 12 |  |
| LLC PDU 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | . . . | LLC PDU 2 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 22 |  |

Figure B.5: Length indicator (LI) example

## B.6	Example 6

Figure B.6 provides an example when the Length indicator and TS-SF field are not used, TS-SFI field is null. As the example does not contain any LLC frame boundaries, no Length Indicator octets are needed. 20 octets is used for LLC data in each RLC data block.

| RLC data block N |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| TS-SFI |  |  |  |  |  |  |  |  |  |
| Payload Type |  | RRBP |  | S/P | USF/MC |  |  | MAC header |  |
| BSN |  | TFI |  |  |  |  | FBI | Octet 1 |  |
|  |  |  | BSN |  |  |  | E = 1 | Octet 2 |  |
|  |  |  |  |  |  |  |  | Octet 3 |  |
|  |  |  |  |  |  |  |  | . |  |
| LLC PDU 1 (cont) |  |  |  |  |  |  |  | . . . | LLC PDU 1 |
|  |  |  |  |  |  |  |  | Octet 22 |  |
| RLC data block N + 1 |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| TS-SFI |  |  |  |  |  |  |  |  |  |
| Payload Type |  | RRBP |  | S/P | USF/MC |  |  | MAC header |  |
| BSN |  |  | TFI |  |  |  | FBI | Octet 1 |  |
|  |  |  | BSN |  |  |  | E = 1 | Octet 2 |  |
|  |  |  |  |  |  |  |  | Octet 3 | LLC PDU 1 |
| LLC PDU 1 (cont) |  |  |  |  |  |  |  | . . . |  |
|  |  |  |  |  |  |  |  | Octet 22 |  |

Figure B.6: Example when Length indicator (LI) can be omitted

## B.7	Example 7

Figure B.7 provides an example when the final LLC PDU (FBI=1) of a downlink TBF fills the RLC data block precisely in which case the Length indicator can be omitted. In the example, LLC PDU 1 continues from a previous RLC data block and ends in and fills precisely the RLC data block shown. TS-SFI field is set to zero.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| TS-SFI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Payload Type |  |  |  |  | RRBP |  |  |  | S/P |  | USF/MC |  |  |  |  | MAC header |  |
| BSN |  |  |  |  | TFI |  |  |  |  |  |  |  |  |  | FBI=1 | Octet 1 |  |
|  |  |  |  |  |  |  | BSN |  |  |  |  |  |  |  | E = 1 | Octet 2 | LLC PDU 1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 3 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 4 |  |
| LLC PDU 1 (cont) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | . . . |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | . . . |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 22 |  |

Figure B.7: Example when Length indicator (LI) can be omitted

## B.8	Example 8

Figure B.8 provides an example when the final LLC PDU (CV=0) of an uplink TBF fills the RLC data block precisely in which case the Length indicator can be omitted. In the example, LLC PDU 1 continues from a previous RLC data block and ends in and fills precisely the RLC data block shown.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  |  |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Payload Type |  |  |  | Countdown value = 0 |  |  |  |  |  |  |  |  |  | SI |  | R | MAC header |  |
| BSN |  |  |  | TFI |  |  |  |  |  |  |  |  |  |  |  | TI | Octet 1 |  |
|  |  |  |  |  |  |  |  | BSN |  |  |  |  |  |  |  | E = 1 | Octet 2 | LLC PDU 1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 3 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 4 |  |
| LLC PDU 1 (cont) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | . . . |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | . . . |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 22 |  |

Figure B.8: Example when Length indicator (LI) can be omitted

## B.9	Example 9

Figure B.9 provides an example when the Length indicator can be omitted. As the LLC PDU 1 begins in the RLC data block N and continues to the next one, no Length octet is needed.

| RLC data block N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 7 |  |  |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Payload Type |  |  |  | Countdown value |  |  |  |  |  |  |  |  |  | SI |  | R | MAC header |  |
| BSN |  |  |  |  |  |  |  | TFI |  |  |  |  |  |  |  | TI | Octet 1 |  |
|  |  |  |  |  |  |  |  | BSN |  |  |  |  |  |  |  | E = 1 | Octet 2 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 3 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 4 |  |
| LLC PDU 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | . . . | LLC PDU 1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | . . . |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 22 |  |
| RLC data block N+1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 7 |  |  |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Payload Type |  |  |  |  |  | Countdown value |  |  |  |  |  |  |  | SI |  | R | MAC header |  |
| BSN |  |  |  |  |  |  |  | TFI |  |  |  |  |  |  |  | TI | Octet 1 |  |
|  |  |  |  |  |  |  |  | BSN |  |  |  |  |  |  |  | E = 0 | Octet 2 |  |
|  |  |  |  |  |  | LI=10 |  |  |  |  |  |  |  | M=1 |  | E=1 | Octet 3 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 4 |  |
| LLC PDU 1 (cont) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | . Octet 13 |  |
| LLC PDU 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | . . . | LLC PDU 2 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 22 |  |

Figure B.9: Example when Length indicator (LI) can be omitted

# Annex C (informative):
Message Sequence Diagrams

The following figures illustrate message sequences for:

-	Pilot mobile originated access (figure C.1);

- one phase mobile originated access (figure C.2); and

-	two phase mobile originated access (figure C.3).

Figure C.1: Message Sequence Diagram for pilot packet access

Figure C.2: Message Sequence Diagram for one phase packet access

Figure C.3: Message Sequence Diagram for two phase packet access

# History

| Document history |  |  |  |
| --- | --- | --- | --- |
| DATE | OLD_VERS | NEW_VERS | SUBJECT |
| 04/2001 |  | 0.1.1 | First submission to CWTS |
| 04/2001 | 0.1.1 | 1.0.0 | Version after approval of CWTS |
| 07/2001 | 1.0.0 | 2.0.0 | Approved by CWTS meeting #10 |
| 12/2001 | 2.0.0 | 2.1.0 | Submitted to CWTS meeting #11 |
| 04/2002 | 2.1.0 | 2.2.0 | Submitted to CWTS meeting #12 |
| 08/2002 | 2.2.0 | 2.2.1 | Submitted to CWTS meeting #13 |
| 08/2002 | 2.2.1 | 3.0.0 | Approved by CWTS meeting #13 |
| 12/2002 | 3.0.0 | 3.0.1 | Submitted to CWTS meeting #14 |
| 12/2002 | 3.0.1 | 3.1.0 |  |