CWTS TSM 05.02 v 3.1.0 (2002-12)

China Wireless Telecommunication Standard;

3G digital cellular telecommunications system; Multiplexing and multiple access on the radio path

(Release 3)

TSM

- Contents

Foreword	5

1	Scope	6

1.1	Normative references	6

1.2	Abbreviations	6

2	General	8

3	Logical channels	9

3.1	General	9

3.2	Traffic channels	9

3.2.1	General	9

3.2.2	Speech traffic channels	9

3.2.3	Circuit switched data traffic channels	9

3.2.4	Packet switched data traffic channels	9

3.3	Control channels	10

3.3.1	General	10

3.3.2	Broadcast channels	10

3.3.2.1	Broadcast Control CHannels (BCCH)	10

3.3.2.2	Cell Broadcast CHannels (CBCH)	11

3.3.3	Common control type channels	11

3.3.3.1	Common Control CHannels (CCCH)	11

3.3.4	Dedicated Control Channels (DCCH)	11

3.3.4.1	Circuit switched dedicated control channels	11

3.3.4.2	Packet dedicated control channels	11

3.3.5	Control channel scheme (circuit switched mode)	11

3.3.6	Control channel scheme (packet switched mode)	12

3.3.7	Logical channels permitted combinations	12

4	The physical resource	13

4.1	General	13

4.1.1	Downlink and uplink	13

4.2	Radio frequency channels	13

4.3	Timeslots and TDMA frames	13

4.3.1	General	13

4.3.2	Timeslot number (TN)	13

4.3.3	Frame structure	14

4.3.3.1	General description	14

4.3.3.2	System Frame Number (SFN)	15

4.3.3.3	Hyper- / super- and multiframe structure	15

4.4	Codes	15

4.4.1	General	15

4.4.2	Codes grouping	16

4.4.3	Spreading Codes (SC)	17

5	Physical channels	18

5.1	General	18

5.2	Special physical channel for synchronisation	18

5.2.1	Downlink Pilot Time Slot physical channel (DwPTS)	18

5.2.2	Uplink Pilot Time Slot physical channel (UpPTS)	18

5.3	Dedicated Physical CHannel (DPCH)	18

5.3.1	General	18

5.3.2	Burst Structure	19

5.3.2.1	Burst format 1	19

5.3.2.1.1	L1 Control Symbols	20

5.3.2.1.2	SACCH-T symbols	22

5.3.2.2	Burst format 2	23

5.3.2.3	Midamble	23

5.4	Common Control Physical CHannel (CCPCH)	23

5.4.1	General	23

5.4.2	Primary Common Control Physical CHannel (P-CCPCH)	23

5.4.2.1	Time Slots and Spreading Codes	23

5.4.2.2	P-CCPCH Training sequence	23

5.4.3	Secondary Common Control Physical CHannel (S-CCPCH)	24

5.4.3.1	S-CCPCH Spreading Codes	24

5.4.4	Physical Forward Access CHannel (P-FACH)	24

5.4.5	Physical Random Access CHannel (P-RACH)	24

5.5	Packet Data Physical CHannel (PDPCH)	24

5.5.1	General	24

5.5.2	Pilot - Packet Data Physical CHannel (P-PDPCH)	24

5.5.3	Shared - Packet Data Physical CHannel (S-PDPCH)	25

5.6	Guard period	25

5.6.1	General	25

5.7	Parameters for channel definition and assignment	25

5.7.1	General	25

5.7.2	General parameters	25

5.7.3	Specific parameters	25

6	Logical channels operations	27

6.1	General	27

6.2	Mapping in frequency of logical channels onto physical channels	27

6.3	Mapping in time of logical channels onto physical channels	28

6.3.1	General	28

6.3.1.1	Key to the mapping table of clause 7	28

6.3.1.2	Mapping of BCCH data	28

6.3.1.2.1	System information message segmentation	29

6.3.1.2.2	System information message re-assembling	29

6.3.1.3	Mapping of SID frames	29

6.3.2	Mapping in time of packet logical channels onto physical channels	29

6.3.2.1	General	29

6.3.2.2	General	29

6.3.2.2.1	Mapping of Uplink Packet Traffic Channel (PDTCH/U) and Uplink Packet Access Control Channel (PACCH/U)	29

6.3.2.3	Mapping of the downlink channels	30

6.3.2.3.1	Mapping of the PDTCH/D and PACCH/D	30

6.4	Permitted channel combinations	30

6.4.1	Permitted channel combinations onto physical channels	30

6.4.2	Multi- Resource configurations	30

6.4.2.1	Multi resource configuration for circuit swtched connections	31

6.4.2.2	Multi resource configurations for packet swtched connections	31

6.5	Operation of channels and channel combinations	31

6.5.1	General	31

6.5.2	Determination of CCCH_GROUP and PAGING_GROUP for UE in idle mode	34

6.5.3	Determination of specific paging multiframe and paging block index	35

6.5.4	Determination of P-FACH_GROUP and P-RACH_GROUP for UE	35

6.5.5	Short Message Service Cell Broadcast (SMSCB)	35

6.5.6	Determination of PAGING_GROUP for UE in TD-PRS attached mode	35

6.5.7	Voice group and voice cell broadcast call notification	36

7	Tables of mapping from logical channel onto physical channel	37

Annex A (normative):  A.1	P-CCPCH filling	45

A.2	Half burst filling	45

A.3	S-CCPCH filling	45

Annex B (normative):  Multi-Resource capability	46

B.1 Constraints imposed by the service selected	46

B.2 Network requirements for supporting MS multi-resource classes	46

Annex C (normative):	47

C.1 Synchronisation sequences	47

C.1.2	The table of SYNC codes	47

C.1.3	The table of SYNC1 codes	48

C.1.4 	The table of Basic midamble codes	54

C.1.5 	The table of Scrambling codes	57

C.2	Complex form for the synchronisation sequences	60

Annex D (normative): Stand alone Dedicated Channel	61

D.1 Configuration: SDCCH-T/3(0..2)+SDCCH-T/C3(0..2)	61

D.2 Configuration: SDCCH-T/6(0..5)+SDCCH-T/C6(0..5)	61

Annex E (normative):  Spreading code definition	63

Annex F (normative):  Midamble allocation	65

F.1 Generation of the midamble sequence	65

F.2 Default midamble allocation	66

F.2.1	Allocation for K=2 ( or MO=64)	67

F.2.2	Allocation for K=4 (or MO=32)	67

F.2.3	Allocation for K=6 (or MO=21)	68

F.2.4	Allocation for K=8 (or MO=16)	68

F.2.5	Allocation for K=10 (or MO=12)	69

F.2.6	Allocation for K=12 (or MO=10)	69

F.2.7	Allocation for K=14 (or MO=9)	70

F.2.8	Allocation for K=16 (or MO=8)	70

F.3	DL common midamble case	71

F.3.1	Mapping scheme for K=16 Midambles.	71

F.3.2	Mapping scheme for K=14 Midambles.	71

F.3.3	Mapping scheme for K=12 Midambles.	72

F.3.4	Mapping scheme for K=10 Midambles.	72

F.3.5	Mapping scheme for K=8 Midambles.	72

F.3.6	Mapping scheme for K=6 Midambles.	73

F.3.7	Mapping scheme for K=4 Midambles.	73

F.3.8	Mapping scheme for K=2 Midambles.	73

Annex G (normative): Multi-slot capability	74

G.1 UE classes for multi-slot capability	74

History	75

# Foreword

This Technical Specification (TS) has been produced by the China Wireless Telecommunication Standard group (CWTS).

The contents of the present document are subject to continuing work within CWTS and may change following formal CWTS approval. Should CWTS modify the contents of the present document, it will be re-released by CWTS with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to CWTS for information;

2	presented to CWTS for approval;

3	or greater indicates CWTS approved release.

y	the second digit is incremented for changes of substance, i.e. technical enhancements, corrections, updates, etc., after approval by CWTS of new version within a given release.

z	the third digit is incremented when changes have been incorporated in the document prior approval by CWTS.

# 1	Scope

This Technical Standard defines the physical channels of the radio sub-system required to support the logical channels. It includes a description of the rules of multiplexing logical channels onto physical channels and a description of the radio frame structure, timeslots and bursts.

## 1.1	Normative references

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.

-	For this Release 3 document, references to TSM documents are for Release 3 versions (version 3.y.z).

-	References to GSM documents are for Release 1999 versions unless otherwise specified.

-	References to 3GPP documents are for Release 1999 versions.

[1]	TSM 01.04: " Abbreviations and acronyms".

[2]	3GPP TS 23.003: “Numbering, addressing and identification”.

[3]	TSM 04.03: " User Equipment - Base Station System (UE - BSS) interface Channel structures and access capabilities".

[4]	TSM 04.06: "User Equipment - Base Station System (UE - BSS) interface; Data Link (DL) layer specification".

[5]	TSM 04.08: " Mobile radio interface layer 3 specification; Part 1: Radio Resource Management".

[6]	TSM 05.03: "China Wireless Telecommunication Standard; 3G digital cellular telecommunications system; TD-SCDMA System for Mobile (TSM); Channel coding".

[7]	TSM 05.04: " Spreading and Modulation".

[8]	TSM 05.05: "China Wireless Telecommunication Standard; 3G digital cellular telecommunications system; TD-SCDMA System for Mobile (TSM); Radio transmission and reception".

[9]	TSM 05.08: " Radio subsystem link control".

[10]	TSM 05.10: " Radio subsystem synchronisation".

[11]	TSM 03.64: "TD-SCDMA Packet Radio Service (TD-PRS); Overall description of the TD-PRS Radio Interface".

[12]	TSM 04.60: "TD-SCDMA Packet Radio Service (TD-PRS); User Equipment (UE) - Base Station System (BSS) interface; Radio Link Control (RLC) and Medium Access Control (MAC) protocol".

[13] TSM 06.41: "Discontinuous Transmission (DTX) for Half Rate speech traffic channels".

[14] TSM 06.81: "Discontinuous Transmission (DTX) for Enhanced Full Rate (EFR) speech traffic channels".

## 1.2	Abbreviations

In addition to those below, abbreviations used in this TS are listed in TSM 01.04 [1].

| DLS | DownLink Sequence |
| --- | --- |
| M | Midamble Sequence |
| ULS | UpLink Sequence |

# 2	General

The radio subsystem is required to support a certain number of logical channels that can be separated into two categories, as defined in TSM 04.03:

i)	The traffic channels

ii)	The control channels

More information is given about these logical channels in clause 3 which also defines a number of special channels used by the radio sub-system.

Clause 4 of this document describes the physical resource available to the radio sub-system, clause 5 defines physical channels based on that resource and clause 6 specifies how the logical channels shall be mapped onto physical channels. Figure 8 depicts this process.

# 3	Logical channels

Abbreviations used in the present document are listed in GSM 01.04 [23]; TS 23.003 [2] and TSM 01.04[1].

## 3.1	General

This clause describes the logical channels that are supported by the TD-SCDMA radio subsystem. Logical channels are distinguished in traffic channels, when carrying user information, and control channel, when carrying signalling.

## 3.2	Traffic channels

### 3.2.1	General

Traffic CHannels (TCH's) are intended to carry either encoded speech or user data in circuit switched mode as defined in sub-clauses 3.2.2 and 3.2.3.

Packet Data Traffic CHannels (PDTCH's) are intended to carry user data in packet switched mode. Due to the dynamic multiplexing onto the same physical channel of different logical channels (see sub-clause 6), a PDTCH carries information which requires a different data rate depending on the used coding scheme as defined in sub-clause 3.2.4.

Traffic channels can be bi-directional or unidirectional; a bidirectional traffic channel can be used for uplink direction (i.e. UE transmitted and BTS received) and downlink direction (i.e. BTS transmitted and UE received), according to a time division duplexing scheme. A unidirectional traffic channel can be used for transmission in one direction only (uplink or downlink).

All packet data traffic channels are uni-directional, either uplink (PDTCH/U), for a mobile originated packet transfer, or downlink (PDTCH/D) for a mobile terminated packet transfer.

Multiple traffic channels can be allocated to the same UE. This is referred to as a multi resource unit configuration, which is defined in sub-clause 6.4.2.1.

Multiple packet data traffic channels can be allocated to the same UE. This is referred to as a multi resource unit packet configuration, which is defined in sub-clause 6.4.2.2.

The specific traffic channels available in the categories of speech and user data are defined in the following sub-clauses.

For the purpose of this TS, any reference to a traffic channel does not apply to PDTCH unless explicitly stated.

### Speech traffic channels

The traffic channels can carry encoded speech according to the following transport formats:

i)		Half rate traffic channel for 5.6 kbit/s speech (TCH-T/HS) using a basic resource unit at SF 16.

ii)	Enhanced full rate traffic channel for 12.2 kbit/s speech (TCH-T/EFS) using two basic resource units at SF 16 or one resource unit at SF 8.

### Circuit switched data traffic channels

The traffic channels can carry user data according to the following transport formats:

i)	Full rate traffic channel for 9.6 kbit/s user data (TCH-T/F9.6);

ii)	Full rate traffic channel for 14.4 kbit/s user data (TCH-T/F14.4);

Higher user data rates are obtained by combining the above circuit data traffic channels (e.g. 57.6 kbit/s can be obtained by combining 4 TCH/F14.4).

### Packet switched data traffic channels

The packet data traffic channels carry user data according to the following transport formats:

i) 	Mode 1 or Mode 2 packet data traffic channel for 8 kbit/s peak user data (PDTCH), using Coding Scheme 1 (CS1).

ii)	Mode 1 or Mode 2 packet data traffic channel for 12 kbit/s peak user data (PDTCH), using Coding Scheme 2 (CS2).

iii) 	Mode 2 packet data traffic channel for 21.4 kbit/s peak user data (PDTCH), using Coding Scheme 3 (CS3).

iv)	Mode 2 packet data traffic channel for 30.6 kbit/s peak user data (PDTCH), using Coding Scheme 4 (CS4).

Higher user data rates are obtained by combining the above packet data traffic channels (e.g. 100 kbit/s can be obtained by combining 13 PDTCHs carrying 8 kbit/s peak user data rate).

Note:	For a detailed description of Mode 1, Mode 2 and of the Coding Schemes see TSM 05.03 [6].

## 3.3	Control channels

### General

Control channels are intended to carry signalling or synchronisation data. Three categories of control channel are defined: broadcast, common, dedicated control channels. Specific channels within these categories are defined in the following sub-clauses.

### 3.3.2	Broadcast channels

#### 3.3.2.1	Broadcast Control CHannels (BCCH)

The Broadcast Control CHannel (BCCH), broadcasts general information on a base transceiver station per base transceiver station basis. Of the many parameters contained in the BCCH, the use of the following parameters, as defined in  TSM 04.08 are referred to in sub-clause 6.5:

P-CCPCH description, which indicates the organisation of the Common Control Channels (CCCHs) mapped onto Primary Common Control Physical CHannel (P-CCPCH): the P-CCPCH uses a 48-multiframe consisting of 12 blocks. The division of blocks to logical channels depends on the following parameters broadcast on the BCCH:

BLK_BCCH: bit(2), it indicates the number of blocks per multiframe on P-CCPCH reserved for BCCH.

BLK_AGCH: bit(3), it indicates the number of blocks per multiframe on P-CCPCH reserved for AGCH.

BLK_CBCH: bit(1), it indicates the number of blocks per multiframe on P-CCPCH reserved for CBCH.

BLK_RES: bit(3), it indicates the number of blocks per multiframe on P-CCPCH reserved for other use. These blocks shall not carry blocks from BCCH, AGCH, CBCH or PCH.

NOTE: The number of PCH can be obtained as the difference between the total number of blocks carried by the P-CCPCH (equals to 12) and the sum of parameters specified above.

S-CCPCH description, which indicates the organisation of the Common Control Channels (CCCH) mapped onto Secondary Common Control Physical CHannel (S-CCPCH); each S-CCPCH uses a 48-multiframe consisting of 12 blocks (as the P-CCPCH).

BLK_AGCH: bit(4), it indicates the number of blocks per multiframe on S-CCPCH reserved for AGCH.

BLK_CBCH: bit(1), it indicates the number of blocks per multiframe on S-CCPCH   reserved for CBCH.

BLK_RES: bit(3), it indicates the number of blocks per multiframe on S-CCPCH reserved for other use. These blocks shall not carry blocks from BCCH, AGCH, CBCH or PCH.

NOTE: The number of PCH can be obtained as the difference between the total number of blocks carried by the S-CCPCH (equals to 12) and the sum of parameters specified above.

PG MFRM: it indicates the number of multiframes between transmission of paging messages to mobiles of the same paging group: 3 bits range 2 to 9.

TD-PRS indicator: the BCCH shall indicate whether or not packet switched traffic is supported; if the TD-PRS Indicator is contained in the information element, it indicates that TD-PRS is supported in the cell.

The BCCH is used to broadcast information both for packet and circuit switched operation.

#### 3.3.2.2	Cell Broadcast CHannels (CBCH)

The CBCH, downlink only, is used to carry the Short Message Service Cell Broadcast (SMSCB).

### 3.3.3	Common control type channels

#### 3.3.3.1	Common Control CHannels (CCCH)

i)	Paging CHannel (PCH): downlink only, used to page mobiles.

ii)	Random Access CHannel (RACH): uplink only, used to request allocation of a TCH or a signalling channel.

iii)	Access Grant CHannel (AGCH): downlink only, used to allocate a signalling channel or directly a TCH.

iv)	Notification CHannel (NCH): downlink only, used to notify mobile stations of voice group and voice broadcast calls.

v)	Forward Access CHannel (FACH): downlink only, it is used to send the initial Power Control (PC) and Synchronisation Shift (SS) values to the UE.

### 3.3.4	Dedicated Control Channels (DCCH)

All associated control channels defined in the following sub-clauses are bi-directional, as well as the traffic

channels (TCHs) they are associated to. HOACH FNICH, PACH and PAKCH are unidirectional.

#### 3.3.4.1	Circuit switched dedicated control channels

i)	Slow, TCH or FACCH Associated, Control CHannel (SACCH).

ii)	Fast, TCH Associated, Control CHannel (FACCH).

iii)	Handover Access CHannel (HOACH), uplink only, that carries the single burst Handover Access Message.

iv)	Frame Number Information CHannel (FNICH), downlink only, that carries the single burst Frame Number Information Message.

#### 3.3.4.2	Packet dedicated control channels

i)	Packet Associated Control CHannel (PACCH), used to transmit L2 control blocks.

ii)	Packet Access CHannel (PACH), uplink only, that carries the single burst Packet Access Message.

iii)	Packet Access Ack CHannel (PAKCH), downlink only, that carries the single burst Packet Access Ack Message.

In the remainder of the text, PACCH, PACH, PAKCH and PDTCH logical channels are also referred to as PDCH (see TSM 04.60 [12] for PDCH definition).

### Control channel scheme (circuit switched mode)

The following Figure represents the circuit switched control channels scheme.

### 3.3.6	Control channel scheme (packet switched mode)

The following Figure represents the packet switched control channels scheme.

### 3.3.7	Logical channels permitted combinations

Only certain combinations of logical channels are allowed as defined in TSM 04.03 [3]. Sub-clause 6.4.1 lists the combinations in relation to physical channels.

# 4	The physical resource

## 4.1	General

The physical resource available to the radio sub-system is an allocation of part of the radio spectrum. This resource is partitioned in frequency, time and code domain. Frequency is partitioned by radio frequency channels, as defined in TSM 05.05 [8]. Time is partitioned by TDMA frames and a TDMA frame is partitioned in timeslots, as defined in sub-clause 4.3 of this TS; each timeslot is further partitioned by Walsh code (see TSM 05.05 [8]). Code partitioning allows to distinguish physical resources which use the same frequency channel and time slot.

In TD-SCDMA mobile system a physical resource, defined by the parameters radio frequency channel, timeslot  and spreading code (see sub-clause 4.2, 4.3 and 4.4 respectively), is called a Resource Unit (RU); a “basic resource unit” shall be additionally defined by spreading factor 16 (see sub-clause 6.4.2). A RU can be assigned for the downlink direction or for the uplink direction.

### 4.1.1	Downlink and uplink

The downlink comprises radio resource units (RUs) used from BTS to UE direction. The uplink is referred to radio resource units (RUs) used from UE to BTS direction.

In TD-SCDMA systems, duplexing is realised in the time domain (Time Division Duplex TDD): a duplex channel uses different timeslots for uplink transmission and for downlink transmission respectively.

## 4.2	Radio frequency channels

In TD-SCDMA system the radio frequency channel is designated by a Frequency Band Number (FBN) and by a Radio Frequency Number (RFqN) within the band. TSM 05.05 [8] defines sets of frequency bands, and allocates numbers to all the radio frequencies channels available to the system. The carrier spacing is 1.6 MHz, while the carrier raster is 200 KHz.

BCCH shall be carried by one carrier only within the cell. If more than one radio frequency channels per cell are used, the BCCH and all common control channels, enclosed DwPTS and UpPTS, shall always be located on the same carrier.

In multi resource configurations all RUs of one radio connection shall always be located on the same carrier.

## 4.3	Timeslots and TDMA frames

### 4.3.1	General

A TDMA frame (5 ms in duration) is composed by 7 timeslots used for traffic and signalling (normal timeslots) in addition to 3 special timeslots. The normal timeslots are 675 us in duration. The three special time slots are respectively the Downlink Pilot Time Slot (DwPTS), the Uplink Pilot Time Slot (UpPTS) and a Guard Period (GP1), as described in sub-clauses 5.2 and 5.6.

In Multi-carrier BTS all the carriers shall be TDMA frame synchronized.

### 4.3.2	Timeslot number (TN)

The timeslot number (TN) is the number of a timeslot within the frame structure. The timeslots within a TDMA frame shall be numbered independently from the transmission direction. The numbering method can be described as follows: between the UpPTS and the DwPTS  there are 7 traffic normal time slots, numbered from 0 to 6. Among these 7 time slots, Ts0 is always allocated as downlink, Ts1 is always allocated as uplink. The number of time slots is also assigned independently for uplink and downlink. A particular timeslot shall be referred to by its timeslot number (TN).

An example of this numbering method is given in Figure 1,where Ts1,Ts2 and Ts3 are uplink timeslots, while Ts4, Ts5 and Ts6, together with Ts0, are downlink ones.

### 4.3.3	Frame structure

#### 4.3.3.1	General description

A TDMA frame is subdivided into 7 normal time slots of 675 µs duration each and 3 special time slots: DwPTS

(Downlink Pilot), GP1 (Guard Period) and UpPTS (Uplink Pilot). The physical contents of the time slots are the bursts of corresponding length as described in section 5.5.2 .

Among the 7 normal time slots, the uplink and the downlink are separated by a switching point: all the normal time slots (at least one normal time slot) before this switching point are allocated as uplink, and all the normal time slots (at least one normal time slot) behind the switching point are allocated as downlink (Figure 1).

Examples for symmetric and asymmetric UL/DL allocations are given in Figure 2.

#### 4.3.3.2	System Frame Number (SFN)

TDMA frames shall be numbered by a frame number (System Frame Number SFN). The frame number shall be cyclic and shall have a range from 0 to FN_MAX where FN_MAX = (24*48*2048) -1 = 2359295 as defined in TSM 05.10 [10]. The frame number shall be incremented at the end of each TDMA frame. The complete cycle of TDMA frame numbers from 0 to FN_MAX is defined as a hyperframe.

#### 4.3.3.3	Hyper- / super- and multiframe structure

A diagram representation of all frame structures is presented in Figure 3. The longest recurrent time period of the structure is called hyperframe and has a duration of 3 h 16 min 36 s. One hyperframe is subdivided in 2048 superframes which have a duration of 5,76 seconds. The superframe is itself subdivided in multiframes; 2 types of multiframes exist in the system:

-	a Traffic Multiframe (48 per superframe) with a duration of 120 ms, comprising 24 TDMA frames. This multiframe is used to carry traffic data;

- a Control Multiframe (24 per superframe) with a duration of 240 ms comprising 48 TDMA frames divided in 12 blocks. This multiframe is used to carry BCCH, PCH, AGCH, CBCH and other channels each sent in 4 consecutive frames or equivalently in one block.

## 4.4	Codes

### 4.4.1	General

In a TD-SCDMA system code partitioning allows to multiply physical resources which use the same radio frequency channel and the same time slot. Four special code sets exist in the system. They are:

-	SYNC codes set: 32 different codes.

-	SYNC1 codes set: 256 different codes divided into 32 groups of 8 codes each.

-	Scrambling codes set: 128 different codes divided into 32 groups of 4 codes each.

-	Basic midamble codes set: 128 different codes divided into 32 groups of 4 codes.

### 4.4.2	Codes grouping

Codes from different code sets cannot be used indiscriminately within one cell. Each of the 32 different SYNC code sequences identifies one code group. One code group in turn univocally identifies:

-	1 SYNC code;

-	8 SYNC1 codes (signatures);

-	4 Scrambling codes;

-	4 basic Midamble codes.

The relationship among code groups and code sequences (i.e. SYNC; SYNC1; Scrambling codes and basic Midamble codes) is reported in Table 1 below.

There is a one to one correspondence between scrambling codes and basic midamble codes within a code group such that, a given scrambling code is univocally associated to a given basic midamble code.

32 code groups are available in a system. A cell is identified by one code group and, within that code group, by one pair scrambling code – basic midamble code out of the four possible ones. The SYNC code sequence transmitted by the cell implicitly provides, together with the 8 SYNC1 codes and the 4 possible basic midamble – scrambling code pairs, the cell code group; while the P-CCPCH midamble code identifies the pair scrambling code – basic midamble code of the cell. The Code group and the pair basic midamble – scrambling code of the cell define the Cell Identity Code (CIC). CIC corresponds to GSM BSIC, where the cell code group field corresponds to the GSM Base station Color Code (BCC); while the cell basic midamble number (or alternatively the cell scrambling code number) field corresponds to the GSM Network Colour Code (NCC).

The Cell code group field is coded with 5 bits while the cell basic midamble number field is coded with 2 bits, as shown in the figure below.

| Code Group (coding criteria) | Associated Codes |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | SYNC Sequence number | SYNC1 Sequence number | Scrambling Code number | Basic Midamble number (coding criteria) | Basic Midamble code ID |
| Group 1 (00000) | 0 | 0~7 | 0 | 0 (00) | 0 |
|  |  |  | 1 | 1 (01) | 1 |
|  |  |  | 2 | 2 (10) | 2 |
|  |  |  | 3 | 3 (11) | 3 |
| Group 2 (00001) | 1 | 8~15 | 4 | 0 (00) | 4 |
|  |  |  | 5 | 1 (01) | 5 |
|  |  |  | 6 | 2 (10) | 6 |
|  |  |  | 7 | 3 (11) | 7 |
| . . . . . |  |  |  |  |  |
| Group 32 (11111) | 31 | 248~255 | 124 | 0 (00) | 124 |
|  |  |  | 125 | 1 (01) | 125 |
|  |  |  | 126 | 2 (10) | 126 |
|  |  |  | 127 | 3 (11) | 127 |

Table 1: Relationship between the SYNC and SYNC1 sequences, scrambling and midamble codes.

In case of a multi-carrier cell, other basic midamble – scrambling code pairs can be used in association with physical resources activated at different frequencies respect to the one currently in use by the P-CCPCH and the SYNC code sequence. A multi-carrier cell shall broadcast this information via a SYSTEM INFORMATION message. The UE and the network shall apply the relevant basic midamble and scrambling code sequence values when transmitting or receiving on physical resources at a given carrier frequency (see TSM 04.08 [5]).

The values of SYNC codes, SYNC1 codes, Basic Midamble codes and Scrambling codes are, respectively, reported in sections: C.1.2, C.1.3, C.1.4 and C.1.5.

### 4.4.3	Spreading Codes (SC)

A Spreading Code (SC) defines a spreading factor (SF) and a Walsh code.

Within each time slot, an additional separation of user signals by spreading codes is used. This means that within one time slot more than one burst of corresponding length can be transmitted. These multiple bursts within the same time slot can be allocated to different users as well as partly or all to a single user.

For the multiple bursts within the same time slot, up to 16 different spreading codes can be used to allow the distinction of multiple bursts.

For this release, the UE shall use for all allocated codes on the same time slot, the same Spreading Factor value.

For this release, the UE shall use in downlink direction, only Spreading Factor 16, 8 and, for high data rate connections, Spreading Factor 1.

In uplink direction any spreading factor value from 16 down to 1 is allowed; the UE shall implement the SF assigned by the network.

# 5	Physical channels

## 5.1	General

- A physical channel is the combination of the burst structure and physical layer procedures related to it (e.g.: antenna diversity, Power Control, Synchronisation Shift procedures).

## 5.2	Special physical channel for synchronisation

### 5.2.1	Downlink Pilot Time Slot physical channel (DwPTS)

The DwPTS in each frame is designed for both initial cell search and synchronisation purposes. The BTS would transmit it omnidirectionally or sectorially at the full power level or adjustable power level. This physical channel is composed of 64 chips of SYNC and 32 chips of Guard Period (GP2), as shown in Figure 4. There are 32 different SYNC codes for the whole system; one unique SYNC is used to distinguish one cell and identify one code group. The code sequences are described in TSM 05.05 [8] and are reported as well in Annex C.

### 5.2.2	Uplink Pilot Time Slot physical channel (UpPTS)

- The UpPTS in each frame is designed for the initial uplink synchronisation between UE and BTS during a random access or a handover. The timeslot is composed of 128 chips of SYNC1 and 32 chips of Guard Period (GP2), as shown in Figure 5. The SYNC1 code sequences or signatures are defined in TSM 05.05 [8] and reported in Annex C as well. The code set is designed to distinguish different UEs in access procedures.

## 5.3	Dedicated Physical CHannel (DPCH)

### 5.3.1	General

- The DCHs are mapped onto the dedicated physical channel (DPCH). Burst structures valid for both, circuit switched and packet switched dedicated physical channels, are described in the following sub-clauses 5.3.2.1 and 5.3.2.2. The midamble code generation and allocation is described in sub-clause 5.3.2.3 and Annex F. Different Spreading Factor values and related channelisation codes can be used, as reported in Annex E.

### 5.3.2	Burst Structure

The burst structure consists of two data symbol fields, a midamble of 144 chips and a guard period (GP3) of 16 chips. Each data field is 352 chips long. The corresponding number of symbols depends on the spreading factor; therefore the gross data rates are different for different spreading factors as indicated in Table 2 below. Because of the QPSK modulation, one symbol shall carry two information bits.

The burst structure is shown in Figure 6, including the numbering convention for the symbols within the burst. Assuming Spreading Factor equal to 1, for convenience, each data field in the burst contains 352 symbols (one chip per symbol), numbered from S1 to S352 and from S353 to S704 respectively.

The transmission order of a burst within a timeslot is defined in terms of symbol number such that the symbol with the lowest number shall be transmitted first.

The content of the burst fields is described in Table 3.

| Spreading factor SF (Q) | Naming for RU | Number of symbols(N) per data field in Burst | Gross Data Rate(bit/s) per physical channel |
| --- | --- | --- | --- |
| 1 | RUSF1 | 352 | 281,600 |
| 2 | RUSF2 | 176 | 140,800 |
| 4 | RUSF4 | 88 | 70,400 |
| 8 | RUSF8 | 44 | 35,200 |
| 16 | RUSF16 (Basic RU) | 22 | 17,600 |

Table 2: Number of symbols per data field for different Spreading Factor values.

| Chip number (CN) | Length of field in Chips | Length of field in symbols | Length of field in µs | Contents of field |
| --- | --- | --- | --- | --- |
| 0-351 | 352 | cf Table2 | 275 | Data symbols |
| 352-495 | 144 | 9 | 112.5 | Midamble |
| 496-847 | 352 | cf Table2 | 275 | Data symbols |
| 848-863 | 16 | 1 | 12.5 | Guard Period (GP3) |

- Table 3: Contents of the different burst fields.

Two different burst formats are foreseen:

-	Format 1: this burst is used to carry circuit switched or packed switched traffic on an already established radio link; transport channels (see sub-clause 6.1) carried on bursts of format 1are interleaved for more than one radio frame (see TSM 05.03 [6]);

-	Format 2: this burst is used during the establishment phase of a radio link (e.g. at the Channel Request or during a handover) or for carrying Common Control CHannels (CCCHs); transport channels carried on bursts of format 2 can be interleaved for one radio frame only.

The content of the data symbol fields depends on the used burst format, as described in the following sub-clauses.

#### 5.3.2.1	Burst format 1

The burst format 1 provides the possibility to transmit L1 control symbols and SACCH-T symbols, both in uplink and downlink direction.

##### 5.3.2.1.1	L1 Control Symbols

Possible L1 control symbols are:

-	Power Control (PC) symbols, used for the inner loop power control procedure (see TSM 05.08 [9]);

-	Synchronisation Shift (SS) symbols, used for inner loop timing control procedure (see TSM 05.08 [9]);

-	Stealing Flag (StF) symbols, used to indicate the current transport channel configuration (see TSM 05.03 [6]).

The number of L1 control symbols available in a burst shall be inversely proportional to the used Spreading Factor (SF) value, according to the rule described in Table 4.

| L1 control symbol type | Number of L1 control symbols per burst |
| --- | --- |
| Power Control (PC) | 16/SF symbols |
| Timing Advance (SS) | 16/SF symbols |
| Stealing Flag (StF) | 16/SF symbols |

Table 4: Number of L1 control symbols in a burst in dependence of the used SF.

The L1 control symbols are positioned adjacent to the midamble field as reported in Figure 7, according to the following order:

-	StF symbols come immediately before the midamble field,

-	SS symbols come immediately after the midamble field,

-	PC symbols come immediately after the SS symbols.

No channel coding applies to the L1 control symbols.

###### 5.3.2.1.1.1	PC symbols

Each PC symbol of a burst shall be used to control, within one packet or circuit switched connection, the transmit power level of an uplink or a downlink time slot according to the rules defined in TSM 05.08 [9].

The PC symbol shall be coded as follows:

| bit values | Power Control command |
| --- | --- |
| 0 0 | Increase the transmit power by Stepsize_PC |
| 0 1 | Do nothing |
| 1 0 | Do nothing |
| 1 1 | Decrease the transmit power by Stepsize_PC |

The Stepsize_PC values and the supported commands (“alphabet”) are defined in TSM 04.08 [5].

In case more PC symbols are available per radio frame with respect to the time slots under power control, the first redundant PC symbol shall be coded as the first useful PC symbol of the radio frame, the second redundant PC symbol shall be coded as the second useful PC symbol of the radio frame and so on.

PC symbols equally apply both for an uplink and for a downlink burst.

###### 5.3.2.1.1.2	SS symbols

The SS symbols of a burst shall be used to control, within a packet or circuit switched connection, the UE transmit timing in all uplink resources according to the rules defined in TSM 05.08 [9].

The SS symbols shall be coded as follows:

| bit values | Synchronisation Shift command |
| --- | --- |
| 0 0 | Anticipate the transmit timing by Stepsize_SS |
| 0 1 | Do nothing |
| 1 0 | Do nothing |
| 1 1 | Delay the transmit timing by Stepsize_SS |

The Stepsize_SS values, frequency (“M”) and the supported commands (“alphabet”) are defined in TSM 04.08 [5].

In case more than 1 SS symbol is available per radio frame, the redundant SS symbols shall be coded as the first useful SS symbol of the radio frame.

Though foreseen for both transmission directions, the SS symbols in the uplink bursts stay unused (bit setting is fixed to “10” binary) being their use reserved for future applications.

###### 5.3.2.1.1.3	StF symbols

The StF symbols provide information on the transport channel they are referring to. The referred transport channel corresponds, in a circuit switched connection, to the FACCH-T while, in a packet switched connection, to the PKCH-T.

StF symbols in CS mode

In a CS connection, the Fast Associated Control CHannel (FACCH) is transmitted stealing the bits to the Traffic CHannel (TCH). In particular, as defined in TSM05.03 [6], a FACCH-T message is transmitted on N consecutive radio frames on the even and/or odd numbered bits of the N bursts, where “N” value corresponds to 8 or to 12, respectively for the FACCH-T/F and for the FACCH-T/H. The StF symbol shall signal to the receiving entity the composition of each single burst as follows:

| bit values | Stealing Flag indication (CS connection) |
| --- | --- |
| 0 0 | No FACCH-T symbol included |
| 0 1 | Even numbered bits are stolen by FACCH-T |
| 1 0 | Odd numbered bits are stolen by FACCH-T |
| 1 1 | All bits are stolen by FACCH-T |

In case FACCH-T is spread over more than one DPCH or in case more than 1 StF symbol is available in a burst, the redundant StF symbols shall be coded as the first useful StF symbol of the radio frame.

When DTX is used, in a CS speech connection, the rules described above for the FACCH-T/F and for the FACCH-T/H, can also be applied to CSACCH-T/F and CSACCH-T/H (see Table 5) respectively.

In CS mode, StF symbols equally apply both for an uplink and for a downlink burst.

StF symbols in PS mode

In a PS connection, the StF symbol defines the coding scheme of the packet transport channel, as detailed in TSM 05.03 [6] and TSM 03.64 [11]. In particular, for the N radio frames carrying the PKCH-T; a sequence of N StF symbol values shall define the specific coding scheme according to the following rules:

| bit values | Stealing Flag indication (PS connection) |
| --- | --- |
| 1 1 1 1 1 1 1 1 | The applied coding scheme is CS1 |
| 0 0 0 0 0 0 0 0 | The applied coding scheme is CS2 |
| 0 0 0 0 1 1 1 1 | The applied coding scheme is CS3 |
| 1 1 1 1 0 0 0 0 | The applied coding scheme is CS4 |
| All other sequences | Reserved |
| Note: | The value of N is 8. |

In case PKCH-T is spread over more than one PDPCH or in case more than 1 StF symbol is available in a burst, the redundant StF symbols shall be coded as the first useful StF symbol of the radio frame.

In PS mode, StF symbols equally apply both for an uplink and for a downlink burst.

##### 5.3.2.1.2	SACCH-T symbols

Besides the L1 control symbols, burst format 1 can optionally carry the SACCH-T symbols as well (see TSM 04.08 [5] and TSM 05.03 [6]).

SACCH-T transmission applies to circuit switched connections only, and to the DPCHs carrying the main signalling link (see TSM 04.08 [5]). Therefore, one or two DPCHs can be used for the purpose, depending on the network assigned resource configuration.

The SACCH-T symbols are transferred continuously at every burst; 4 or 2 symbols per burst are devoted to carry SACCH information (see Figure 10), respectively for the case of 1 or 2 DPCHs carrying the main signalling link, irrespective of the used SF.

The transmission of one SACCH-T requires 48 radio frames according to the coding rules defined in TSM 05.03 [6].

SACCH-T symbols are transmitted adjacent to the L1 symbols as reported in Figures 8.a and 8.b; the position of these ”x” symbols (with “x” equal to 4 or 2) varies with the spreading factor value and the specific “x” value, though it stay unchanged with respect to the L1 symbols.

For the distribution of the 4 SACCH-T symbols on two bursts of one radio frame, refer to TSM 05.03 [6].

SACCH-T equally applies both for an uplink and for a downlink burst.

#### 5.3.2.2	Burst format 2

Burst format 2 does not carry L1 control symbols nor SACCH symbols such that all the chips of the burst are used to carry higher layer signaling.

The burst format  2 is used for carrying the Common Control CHannels and the following single burst messages:

-	HO access(uplink; see TSM 04.08 [5]);

-	 Channel Request (uplink; see TSM 04.08 [5]);

-	Packet Access (uplink; see TSM 04.60 [12]);

-	 FNinfo message (downlink; see TSM 04.08 [5]);

-	Physical Information (downlink; see TSM 04.08 [5]);

-	Packet Access Ack (downlink; see TSM 04.60 [12]).

#### 5.3.2.3	Midamble

There are 128 different basic midamble codes, with length 128 chips, for the whole system. Each BTS has one basic midamble only, which belongs to the code group identified by the SYNC sequence broadcast onto the DwPTS. All the midambles used in that cell are generated by cyclically shifting the cell basic midamble sequence. A shifted version of the basic midamble represents a Training Sequence Code (TSC).

## 5.4	Common Control Physical CHannel (CCPCH)

### 5.4.1	General

BCCH, CCCH, RACH and FACH logical channels are mapped onto the common control physical channel (CCPCH). The burst structure is the burst format 2.

### 5.4.2	Primary Common Control Physical CHannel (P-CCPCH)

One Primary Common Control Physical CHannel (P-CCPCH) only can be configured per BTS.

The P-CCPCH has pre-fixed time position in the radio frame; as well as pre-fixed spreading codes and training sequence.

P-CCPCH posses a fixed radiation pattern which can be omnidirectional or beamformed to give the cell a certain shape. Unlike the other beamformed physical channels the P-CCPCH radiation pattern is fixed and not variable. The P-CCPCH is always transmitted with an antenna pattern configuration that offers full cell coverage.

#### 5.4.2.1	Time Slots and Spreading Codes

The P-CCPCH is located on the first Time Slot of the radio frame (Ts0; see sub-clause 4.3.2) where it has assigned two Resource Units at Spreading Factor (SF) 16 and at the following channelisation codes: cQ=16(k=1), cQ=16(k=2)(see Annex E for details).

#### 5.4.2.2	P-CCPCH Training sequence

The midamble code m(1) shall be used for P-CCPCH as training sequence (see sub-clause 5.3.2.3) also when Time Switched Transmission Diversity (TSTD) scheme is applied (see TSM 05.08 [9]); the midamble m(2)  is reserved for P-CCPCH if Space Code Transmission Diversity (SCTD) scheme is supported, in particular:

-	If no SCTD transmission diversity scheme is applied to P-CCPCH, m(1)  is used  m(2) is left unused

-	If SCTD transmission diversity scheme is applied to P-CCPCH, m(1) is used, together with spreading codes c16 (1) and c16 (2) (see TSM 05.08 [9]), for first antenna and m(2) is used, together with spreading codes c16 (3) and c16 (4) (see TSM 05.08 [9]), for diversity antenna.

For DL physical channels that are located in the same time slot as the P-CCPCH, midambles shall be allocated based on the default midamble allocation scheme, using the association for burst type 1 and K=8 midambles.

### 5.4.3	Secondary Common Control Physical CHannel (S-CCPCH)

None, one or more Secondary Common Control Physical CHannels (S-CCPCH) can be configured per BTS.

A S-CCPCH can be freely configured on any downlink time slot of the radio frame where it can have assigned one RUSF8 or two RUSF16 respectively according to the rules described in sub-clause 5.4.3.1. Which time slot, which spreading codes and which training sequence it uses, shall be indicated by the BCCH broadcast from the P-CCPCH.

S-CCPCHs can be omnidirectional or adaptively beamformed.

#### 5.4.3.1	S-CCPCH Spreading Codes

A S-CCPCH can have assigned either one RU at Spreading Factor 8 (RUSF8) or two RUs at Spreading Factor 16 (RUSF16) respectively.

The associated RUs can use any channelisation code with the following constraint: in case two RUSF16 are assigned, the spreading code of the second RU is implicitly given by the spreading code of the first RU, being the spreading code of the second RU the other leave of the same node in the code tree (see Annex E); therefore only one spreading code only will be described by the network when signalling a S-CCPCH configuration.

### 5.4.4	Physical Forward Access CHannel (P-FACH)

The Physical Forward Access CHannels (P-FACH) are used for power and timing adjustment of UE during the radio link establishment phase.

A P-FACH can be freely configured on any downlink time slot of the radio frame where it can have assigned either one RUSF8 or one RUSF16 respectively. Which time slot, which spreading codes and which training sequence it uses, shall be indicated by the BCCH broadcast from the P-CCPCH.

P-FACHs can be beamformed.

### 5.4.5	Physical Random Access CHannel (P-RACH)

The Physical Random Access CHannels (P-RACH) are used by UEs for sending channel request messages to the network.

A P-PRACH can be freely configured on any uplink time slot of the radio frame where it can have assigned either one RUSF8 or one RUSF16 respectively. Which time slot, which spreading codes and which training sequence it uses, shall be indicated by the BCCH broadcast from the P-CCPCH.

P-RACHs can be beamformed.

## 5.5	Packet Data Physical CHannel (PDPCH)

### 5.5.1	General

The logical channels “Packet Data Traffic Channel” (PDTCH) and “Packet Associated Control Channel” (PACCH) are mapped on the PDPCH. The packet data physical channel has exactly the same structure and fields as the DPCH, with the difference that for packet switched services the StFs indicate for the downlink bursts the channel coding scheme.

Packet data can be delivered at the air interface through two types of Packet Data Physical CHannels (PDPCH):

Pilot – Packet Data Physical CHannels (P-PDPCH);

Shared –Packet Data Physical CHannels (S-PDPCH).

### 5.5.2	Pilot - Packet Data Physical CHannel (P-PDPCH)

A Pilot – Packet Data Physical CHannel (P-PDPCH) is univocally assigned to one user equipment only.

It requires continuous burst transmission so that, when no valid MAC/RLC block has to be delivered, Packet Dummy control blocks (see TSM 04.60 [12]) shall be sent instead.

It can be assigned both for downlink TBF and for uplink TBF.

P-PDPCHs shall be assigned to a UE in duplex mode so that: when assigning a P-PDPCH for uplink (downlink) transmission, at least another P-PDPCH has to be set up in the opposite transmission direction.

### 5.5.3	Shared - Packet Data Physical CHannel (S-PDPCH)

A Shared – Packet Data Physical CHannel (S-PDPCH) can be assigned to one or multiple user equipment sharing the same physical resources in the time domain.

It can be assigned both with one phase or 2 phases packet access (see TSM 04.60 [12]).

It does not require continuous transmission so that, when no valid MAC/RLC block has to be delivered, transmission can be suspended.

It can be assigned both for downlink TBF and for uplink TBF.

## 5.6	Guard period

### 5.6.1	General

The Guard Period (16 chips long; GP with reference to Figure 6) used in the DPCH, CCPCH and PDPCH physical channels, is provided because it is required that both the UE and the BTS transmission be attenuated for the period between bursts with the necessary ramp up and ramp down occurring during the guard period, as defined in TSM 05.05 [8]. This time interval allows to minimise interference between adjacent bursts within the same or adjacent cells.

The Guard Period (32 chips long; GP with reference to Figure 4 and Figure 5) used in the UpPTS and DwPTS physical channels, is provided to minimise interference between the previous (to the DwPTS) and the following (to the UpPTS) physical channels within the same or adjacent cells.

The longer Guard Period (96 chips long; GP with reference to Figure 1) between the DwPTS and UpPTS physical channels, is provided for different purposes:

i)	it avoids overlapping of the UpPTS bursts to the following physical channels, taking into account the maximum range of the cell;

ii)	it can be exploited for BTS on air synchronisation, based on respective DwPTS transmission and reception;

iii)	it eases at the UE the DwPTS reception from different cells.

## 5.7	Parameters for channel definition and assignment

### 5.7.1	General

This sub-clause describes the set of parameters necessary to fully describe the mapping of any logical channel onto a physical channel. These parameters may be divided into general parameters, that are characteristic of a particular base transceiver station, and specific parameters, that are characteristic of a given physical channel.

### 5.7.2	General parameters

These are:

-	the set of radio radio frequency channels used in the cell (FBN and RFqNs), together with the identification of the BCCH carrier;

-	the code group, defining the UpPTS sequences and the DwPTS sequences;

-	the scrambling code number (SC) together with the basic midamble sequence within the code group, which allows the definition of the scrambling code and the basic midamble code;

-	the TDMA frame number (SFN);

These parameters are broadcast (or derived from parameters broadcast) in the SIs of BCCH.

### 5.7.3	Specific parameters

These parameters define a particular physical channel in a base transceiver station. They are:

-	the Spreading Code (SC), which defines Walsh code and spreading factor;

-	Training Sequenc Code (TSC), or midamble shift;

-	the Radio Frequency Number (RFqN);

-	the Timeslot Number (TN);

-	the type of logical channel;

-	the sub-channel number.

The last two parameters allow the determination of the frame sequence.

# 6	Logical channels operations

## 6.1	General

The MAC provides for the multiplexing/demultiplexing of higher layer information into/from transport blocks delivered to/from the physical layer on common or dedicated transport channels. Therefore, the MAC layer is responsible for mapping of logical channel(s) onto transport channel(s).

The Transport channel defines how and with what characteristics (Transport channel or transport block size; channel coding; rate matching and interleaving period (or equivalently Transmission Time Interval (TTI)) data are transferred over the radio interface.

The following table describes the relationship among logical, transport and physical channels.

| Physical Channel | Logical Channel | Transport Channel |
| --- | --- | --- |
| DPCH | TCH | TCH-T/EFS, TCH-T/HS, TCH-T/F9.6, TCH-T/F9.6M, TCH-T/F14.4, TCH-T/F14.4M |
|  | SACCH | SACCH-T, SDCCH-T/3 Mode1, SDCCH-T/6 Mode1, SDCCH-T/3 Mode2, SDCCH-T/6 Mode2, CSACCH-T/F, CSACCH-T/H (NOTE) |
|  | FACCH | SDCCH-T/3 Mode1, SDCCH-T/6 Mode1, SDCCH-T/3 Mode2, SDCCH-T/6 Mode2, FACCH-T/F, FACCH-T/H |
|  | HOACH | SB-T |
|  | FNICH |  |
| P-CCPCH | BCCH | BCCH-T |
| P-CCPCH | PCH, AGCH, CBCH, NCH | CCCH-T |
| S-CCPCH |  |  |
| P-RACH | RACH | SB-T |
| P-FACH | FACH |  |
| PDPCH | PDTCH | PKCH-T CS1 Mode1, PKCH-T CS1 Mode2, PKCH-T CS2 Mode1, PKCH-T CS2 Mode2, PKCH-T CS3 Mode2, PKCH-T CS4 Mode2 |
|  | PACCH |  |
|  | PACH | SB-T |
|  | PAKCH |  |
| DwPTS | --- | --- |
| UpPTS | --- | --- |

Table 5: Logical Channels to Physical Channels mapping through Transport Channels

NOTE: CSACCH-T/F and CSACCH-T/H transport channels are foreseen when DTX is applied as described in TSM 06.81 [14] and TSM 06.41 [13] respectively, or for sending EMO and EMR messages, according to network indications as reported in TSM 04.08 [5].

## 6.2	Mapping in frequency of logical channels onto physical channels

Within a cell the BCCH, together with all CCCHs, are mapped onto physical channels having assigned the same radio frequency channel.

The DCHs can be mapped onto physical channels having assigned any of the cell frequencies with the following constraint: for multi resource configurations all the resources shall use the same radio frequency channel.

## 6.3	Mapping in time of logical channels onto physical channels

## 6.3.1	General

The mapping in time of circuit switched logical channels is defined in the tables of clause 7, which also defines the position of the interleaved blocks within the multiframe.

#### 6.3.1.1	Key to the mapping table of clause 7

The following relates to the tables of clause 7. The columns headed:

i)	"Channel designation" gives the precise acronym for the channel to which the mapping applies.

ii)	"Allowable physical channel assignment" identifies the particular physical channel onto which the logical channel is mapped.

iii)	"Direction" defines whether the given mapping applies identically to downlink and uplink (D&U), or to downlink (D) or uplink (U) only.

iv)	"Allowable timeslots assignments" defines whether the channel can be supported on, or assigned to, any of the timeslots, or only on specific timeslots.

v)	"Allowable spreading factor" defines the Spreading Factor the channel can use in the cell.

vi)	"Allowable occupied RUs " defines the number of RU the channel uses in the cell.

vii)	"Repeat length in TDMA frames" defines how many TDMA frames occur before the mapping for the interleaved blocks repeats itself.

viii)	"Interleaved block TDMA frame mapping" defines, within the parentheses, the TDMA frames used by each interleaved block (e.g. 0..3). The given numbers equate to the system frame number (SFN) modulo the number of TDMA frames per repeat length; therefore, the frame is used when:

TDMA frame mapping number = (SFN) mod repeat given length

Where there is more than one block shown, each block is given a separate designation e.g. B0, B1. Where diagonal interleaving is employed then all of the TDMA frames included in the block are given, and hence the same TDMA frame number can appear more than once (see TSM 05.03 [6]).

#### 6.3.1.2	Mapping of BCCH data

In order to facilitate the UE operation, it is necessary to transmit all System Information messages in defined blocks within one 48-multiframe. In particular the first block of a 48-multiframe always carries the BCCH and eventually, if configured by the network operator, the second block as well. The BCCH shall only be sent on P-CCPCH.

Only one System Information message (i.e. System Information type 1) is sent on BCCH at a predefined,  fixed position with respect to the System Frame Number according to the following rule:

(SFN div 48) mod 4=0

In case two blocks have been configured per 48-multiframe, System Information type 1 shall be sent in the first block. The SI messages other than the SI1 message shall be sent in a sequence determined by the network, using the BCCH blocks within each multiframe which are not occupied by SI1. The same SI message shall not occur twice within the sequence. The sequence shall be repeated immediately.

Note 1: SI1 contains a change mark parameter. From this the UE can detect whether the information in other SIs has been changed.

A single System Information type may consist of more than one instance. This means that the information is spread to more than one message but is referred to as one System Information. All instances contain an instance counter; they are sent in ascending order in a single sequence. The UE should detect a consistent set of instances before making use of the contained data.

System Information type 13 is only related to TD-PRS services; it shall only be sent if TD-PRS support is indicated in System Information type 1 message.

System information type 7 message shall be sent by the network to all UEs within the cell in case additional measurements are demanded to the UEs for supporting the network in the channel allocation process.

It can be used by a multi-carrier cell to broadcast the supported frequencies, in addition to the one currently in use by the BCCH (see TSM 04.08 [5]).

##### 6.3.1.2.1	System information message segmentation

Some SIs (such as SI 2 and SI7) occupy more than 1 message instance. Before they are sent, they are segmented into different instances according to the following rules:

a)	instances are sent consecutively (i.e. in successive blocks) in ascending order. This rule shall be interrupted only by the case when SI1 must be sent (see sub-clause 6.3.1.2), then it shall be restarted;

b)	In addition to a), for SI type 2:

-	a Frequency List Structure always fits in one instance only.

-	Different systems (e.g. TD-SCDMA and GSM) shall always be described in different instances of a message set.

-	In case one block only has been reserved for BCCH, no more than 3 pages are allowed; in case two blocks have been reserved for BCCH , no more than 6 pages are allowed.

c)	In addition to a), for SI type 7:

-	all instances, but the last one, shall contain information till the last bit of the 23th octet.

-	The instances next to the first one shall contain all the information elements which shall be present in each instance and the remaining description continuing from the exact point left by the previous instance of the same set.

-	In case one block only has been reserved for BCCH, no more than 3 pages are allowed; in case two blocks have been reserved for BCCH, no more than 6 pages are allowed.

##### 6.3.1.2.2	System information message re-assembling

The UE should collect all the consistent set of instances of segmented SI to get the complete information.

#### 6.3.1.3	Mapping of SID frames

When the DTX mode of operation is active, it is required to transmit Silence Descriptor (SID) information, or equivalent dummy information, during the SACCH/T block period (48 TDMA frames). As the SID frames do not constitute a logical channel and their use is specific to DTX operation, the mapping of SID frames onto the TDMA frames is specified in TSM 05.08 [9].

### 6.3.2	Mapping in time of packet logical channels onto physical channels

#### 6.3.2.1	General

A physical channel allocated to carry packet logical channels is called a packet data physical channel (PDPCH). A PDPCH shall carry packet logical channels only.

Packet switched logical channels (PDCH, see sub-clause 3.3.4.2) are mapped dynamically onto a 48-multiframe.

#### 6.3.2.2	General

##### 6.3.2.2.1	Mapping of Uplink Packet Traffic Channel (PDTCH/U) and Uplink Packet Access Control Channel (PACCH/U)

The PDCH’s where the UE may expect occurrence of its PDTCH/U(s) or PACCH/U for a mobile originated transfer is indicated in resource allocation messages (see TSM 04.60 [12]). PACCH/U shall be allocated respecting the resources allocated to the UE and the UE multi resource class.

The occurrence of the PDTCH/U and/or the PACCH/U at given block(s) Bx (where Bx = B0…B11) in the 48-

multiframe structure for a given UE on a given PDCH shall be as follow:

-	if  the assigned PDCH is a P-PDCH its occurrence is well-known and defined by the signalled channel mode;

-	if the assigned PDCH is a S-PDCH with fixed allocation, its occurrence is well-known and defined by the signalled allocation bitmap parameters;

-	if the assigned PDCH is a S-PDCH with dynamic allocation, accessed by fixed allocation, its occurrence shall be indicated by the values reported in the ALLOCATION_BITMAP field (see TSM 04.60 [12]).

-	if the allocated PDCH is a S-PDCH accessed by dynamic allocation, its occurrence shall be indicated by the value of the User State Flag (USF) parameter or Time Slot State Flag (TS-SF) parameter or by the combination of the two parameters, contained in the header of a preceding block transmitted in a downlink PDCH, that is to say B(x-2) in the same multiframe if x2 or B(10) in the previous multi-frame if x=0 or B(11) in the previous multi-frame if x=1. If the USF in block B(x-2) indicates that block B(x) shall be used by a UE for which the USF_GRANULARITY is set to 1 (corresponding to 4 blocks) in the last assignment message, that UE shall also use the three following blocks for uplink transmission. The UE may transmit a PDTCH block or a PACCH block on any of the allowed uplink blocks. The occurrence of the PACCH/U associated to a PDTCH/D shall be indicated by the network by polling the UE (see TSM 04.60 [12]).

NOTE: this sub-clause specifies how the network shall signal to the UE is allowed to use the uplink resources. The operation of the UE is specified in TSM 04.60 [12].

#### 6.3.2.3	Mapping of the downlink channels

##### 6.3.2.3.1	Mapping of the PDTCH/D and PACCH/D

The PDCH where the UE may expect occurrence of its PDTCH/D(s) for a mobile terminated transfer or its PACCH/D, for both mobile originated and mobile terminated transfer are indicated in resource allocation messages (see TSM 04.60 [12]). The user equipment owner of the PDTCH/D or PACCH/D shall the indicated by the TFI (Temporary Flow Identity) (see TSM 04.60 [12]).

## 6.4	Permitted channel combinations

### 6.4.1	Permitted channel combinations onto physical channels

The following are the permitted ways, as defined by TSM 04.03 [3], in which channels can be combined onto physical channels (numbers appearing in parenthesis after channel designations indicate sub-channel numbers; channels and sub-channels need not necessarily to be assigned):

i)	TCH + FACCH + SACCH (NOTE3)

ii)	TCH

iii)	BCCH + CCCH (NOTE1)

iv)	PDTCH+PACCH

v)	CCCH (NOTE2)

vi)	FACCH + SACCH (NOTE4)

where CCCH = PCH + AGCH + NCH+CBCH.

NOTE1:	This configuration applies only to P-CCPCH.

NOTE2:	This configuration applies only to S-CCPCHs.

NOTE3: 	This configuration applies only to main signalling link.

NOTE4: 	This configuration applies to FACCH and SACCH when are delivered as SDCCH-T/x and SDCCH-T/Cx transport channel respectively. The parameter x can assume the value 3 or 6, the notation Cx (instead of x) is used to distinguish the SACCH channel.

Multiple FACCHs (i.e. 3 or 6 for configuration vi) above) shall be configured on the same DPCH(s).

### 6.4.2	Multi- Resource configurations

A multi resource configuration consists of multiple circuit or packet switched traffic channels together with associated control channels, allocated to the same UE.

Multi resource configurations can occupy up to 16 codes per time slot and up to 7 time slots per frame, but all the occupied resource units shall have assigned the same radio frequency channel.

#### 6.4.2.1	Multi resource configuration for circuit swtched connections

Two types of multi resource configurations exist, symmetric and asymmetric. The symmetric case consists of only bi-directional channels. The asymmetric case consists of both bi-directional and unidirectional channels.

The occupied physical channels shall consist of the following channel combinations as defined in sub-clause 6.4.1.

one main signalling link(type i) + x traffic channels (type ii)

The main signalling link is the bi-directional channel that carries the main signalling (FACCH and SACCH) for the multi resource configuration. The position of the main signalling link is assigned by the allocation message (TSM 04.08 [5]). Traffic channels may be added or removed without changing the main signalling link.

The allocation of channels to a Multi resource Configuration must always consider the multi slot capability of the UE, as defined by the multi-slot class described in annex G.

There is no limitation in this TS to the possible TCH types (see sub-clause 3.2) which may be used in a multi resource Configuration.

High Speed Circuit Switched Data (HSCSD) is one case of multi resource configuration. The full rate traffic channels of a HSCSD configuration shall convey the same user bit rate (see sub-clause 3.2.3).

#### 6.4.2.2	Multi resource configurations for packet swtched connections

A UE may be allocated several PDTCH/Us or PDTCH/Ds for one mobile originated or one mobile terminated communication respectively. In this context allocation refers to the list of PDCH that may dynamically carry the PDTCHs for that specific UE. The PACCH may be mapped onto any of the allocated PDCHs.

## 6.5	Operation of channels and channel combinations

### 6.5.1	General

a)	A base transceiver station shall always transmit a burst in the P-CCPCH at every TDMA frame to allow UEs to make measurements.

Note: the P-CCPCH is positioned on time slot TS0 and uses two Resource Units at Spreading Code 0 and 1 respectively.

b)	The parameter BLK_BCCH in the BCCH defines the number of blocks supporting BCCH channel in the P-CCPCH. The parameters BLK_AGCH, BLK_CBCH and BLK_RES define the number of blocks on P/S-CCPCHs reserved for other use but not paging blocks. The number of PCH blocks is obtained as the difference between the total number of blocks carried by the P/S-CCPCH (equals to 12) and the sum of the parameters specified above.

c)	The PCH, AGCH, NCH and BCCH are frame multiplexed (modulo 48) when mapped onto the same  physical channels. The physical channels are shared on a block by block basis. The number of blocks not used for paging (see b)) starting from, and including block number 0 is broadcast in the BCCH. As above the number of paging blocks per48-multiframe considered to be "available" shall be deduced by the number of blocks reserved for access grant messages.

d)	Configuration ii) of section 6.4.1 is allowed only on the P-CCPCH.

The BCCH defines the additional common control physical channels in respect to the P-CCPCH that are configured in the cell; this includes the S-CCPCH, the P-FACH and the P-RACH. These channels can use any of the available downlink/uplink time slots and spreading codes but all shall use the same radio frequency channel as for the P-CCPCH. The BCCH also defines what P-RACH  and  P-FACH belong to what CCPCH Within a cell, one and only one P-CCPCH is allowed, the configuration of which is fixed. At least one P-FACH and one  P-RACH are configured. Additional configuration information about P-FACH, P-RACH and S-CCPCH is broadcast on BCCH.

e)	P-CCPCH brings BCCH and can optionally carry other subchannels, like PCH, AGCH, CBCH and NCH.

The P-CCPCH uses a 48 multiframe consisting of 12 blocks with the following restrictions:

1)	The sequence of logical channels, starting from the lowest frame number, is fixed to the following order:

BCCH, PCH, AGCH, CBCH, Reserved Blocks (e.g. for NCH)

2)	The minimum and maximum number of blocks used by a logical channel within a 48-multiframe is:

|  | BCCH | PCH | AGCH | CBCH | Reserved Blocks |
| --- | --- | --- | --- | --- | --- |
| Min | 1 | 0 / 3 | 0 | 0 | 0 |
| Max | 2 | 11 | 7 | 1 | 7 |

3)	The number of blocks used for BCCH, AGCH, CBCH, Reserved Blocks is indicated in BCCH. The remaining blocks of the 12 blocks within the multiframe are used for PCH.

4)	The sum of blocks for BCCH, AGCH, CBCH, Reserved Blocks shall be less than or equal to 9, if PCH is configured. Therefore PCH, when configured, comprises at least 3 blocks.

f)	S-CCPCH can bring PCH, AGCH, CBCH and NCH

The S-CCPCH uses a 48 multiframe consisting of 12 blocks (as for the P-CCPCH), with the following restrictions:

1)	The sequence of logical channels, starting from the lowest frame number, is fixed to one of the following  sequences:

a)	idle (no transmission), PCH, CBCH, Reserved Blocks,

b)	idle (1 block),AGCH (1 block), PCH,CBCH, Reserved Blocks,

c)	AGCH (1  block), PCH, AGCH, CBCH, Reserved Blocks

d)	AGCH (2 blocks), PCH, AGCH, CBCH, Reserved Blocks.

2)	In case of sequence a), the number of idle blocks shall be the same as the number of configured BCCH blocks (1 or 2 respectively).

3)	The selected sequence depends on the BCCH blocks that have been configured on the P-CCPCH and on the AGCH blocks that have been configured on the S-CCPCH according to the following rules:

-	if one block of BCCH is configured on the P-CCPCH, combinations a) or c) shall apply; combination a) shall be used if no AGCH block has been configured while combination c) shall be used if BLK_AGCH is at least equal to 1;

-	if two blocks of BCCH are configured on the P-CCPCH, combinations a) or b) or d) shall apply; combination a) shall be used if no AGCH blocks have been configured. Combination b) shall be used if BLK_AGCH is equal to 1. Combination d) shall be used if BLK_AGCH is greater than 1.

4)	The minimum and maximum number of blocks used by a logical channel within a 48-multiframe is:

|  | idle | PCH | AGCH | CBCH | Reserved Blocks |
| --- | --- | --- | --- | --- | --- |
| min | 0 | 0 / 3 | 0 | 0 | 0 |
| max | 2 | 11 | 12 | 1 | 7 |

5)	The number of blocks used for AGCH, CBCH, Reserved Blocks is indicated in BCCH. The remaining blocks of the 12 blocks within the multiframe shall be used for PCH or are idle blocks.

6)	If PCH is configured it shall comprise at least 3 blocks.

7)	If no AGCH is configured then the sum of blocks for CBCH, Reserved Blocks must be less than or equal to 8 or 7 depending on the number of idle blocks. If AGCH is configured then the sum of blocks for AGCH, CBCH, Reserved Blocks must be less than or equal to 9 or must be 12.

8)	The P-FACH carries the FACH when acknowledging a correctly detected SYNC1 code sequence or it stays idle (no transmission) otherwise.

9)	The P-RACH carries the RACH transmitted by the Ues.

10)	The minimum and maximum number of Common Control Physical Channels within a cell is:

|  | P-FACH | P-RACH | P-CCPCH | S-CCPCH |
| --- | --- | --- | --- | --- |
| Min | 1 | 1 | 1 | 0 |
| Max | 3 | 3 | 1 | 3 |

The following general restrictions apply:

1)	only one CCPCH that carries both PCH and AGCH logical channels is allowed,

2)	only one CBCH is allowed. It shall be located on a CCPCH that carries also PCH,

3)	at least one and maximum three CCPCHs that carry the AGCH and PCH.

4)	only one NCH is allowed ; it shall be located in such a way that all UEs are able to receive the PCHs and the NCH.

5)	In order to ease the UEs detection, if one or more S-CCPCHs are configured in a cell, the parameters     BLK_BCCH, BLK_AGCH, BLK_CBCH, and BLK_RES shall be chosen such that neither the position of the CBCH block nor the position of the NCH channels on one P/S-CCPCH do overlap with the position of a PCH block on another P/S-CCPCH and in addition, CBCH block and NCH channels shall not overlap each other.

J)	The P-CCPCH and each S-CCPCH, if and only if carrying PCH blocks, define their own CCCH_GROUP of mobiles in idle mode. Mobiles in a specific CCCH_GROUP will listen for paging messages to which the CCCH_GROUP belongs. The method by which a mobile determines the CCCH_GROUP to which it belongs is defined in sub-clause 6.5.2.

k)	The CCCH optionally supports  the SPLIT_PG_CYCLE parameter which controls the occurrence of paging blocks on CCCH belonging to the user equipment when DRX period split is supported.

l)	The BCCH shall also broadcast the PG_MFRM parameter which indicates the number of 48-multiframe between transmission of paging messages to mobiles in idle mode  of the same paging group. The "available" paging blocks per P/S-CCPCH are then those "available" per 48-multiframe on that P/S-CCPCH (determined as described in point h)) multiplied by PG_MFRM. Mobiles are normally only required to monitor every Nth block of their paging channel, where N equals the number of "available" blocks in total (determined by the above BCCH parameters) on the paging channel of the specific CCCH which their CCCH_GROUP is required to monitor. Other paging modes (e.g. page reorganise or paging overload conditions described in TSM 04.08 [5]) may require the mobile to monitor paging blocks more frequently than this. All the mobiles listening to a particular paging block are defined as being in the same PAGING_GROUP. The method by which a particular mobile determines to which particular PAGING_GROUP it belongs and hence which particular block of the available blocks on the paging channel is to be monitored is defined in sub-clause 6.5.2.

m)	A UE which has its membership of at least one voice group or voice broadcast call group set to the active state shall, in addition to monitoring the paging blocks as described above, monitor the notification channel, NCH. The UE knows the location and the position of this logical channel from the NCH_LOCATION parameter broadcast from the BCCH. The channel may be present when a cell supports voice group or voice broadcast calls.

n)	For cell access, the UE selects a SYNC1 code sequence out of a set of 8 possible signatures that can be used within the cell (further restrictions on this choice may apply); this corresponds to implicitly select the P-FACH from which the UE shall expect the acknowledgement message (FACH), the P-RACH on which the UE shall send its Channel Request message and the S/P-CCPCH from  which the UE shall expect the Immediate Assignment (-/Reject) message on AGCH.  The correspondence between P-FACH/P-RACH and S/P-CCPCH is broadcast on the BCCH. The following restrictions apply:

i)	There is a one by one mapping between P-FACH and P-RACH; each configured P-FACH shall have associated one configured P-RACH.

ii)	The mapping from P-RACH to AGCH must map to an CCPCH that has been configured. The CCPCH must carry an AGCH. Each configured AGCH must be mapping destination of at least one P-RACH.

iii)	The parameter BS_CCCH_SDCCH_COMB in the BCCH defines whether the S-CCPCHs are combined with SDCCH-T/x + SDCCH-T/Cx onto the same physical channel.

### 6.5.2	Determination of CCCH_GROUP and PAGING_GROUP for UE in idle mode

Cell paging channels can be configured both on P-CCPCH and on S-CCPCHs.

Define:

-	M equal to the number of the configured P/S-CCPCH channels containing PCH blocks minus one;

-	Pi equal to the number of paging blocks within one 48 Control Multiframe on CCPCH number i where

(0 <= i <=M).

And, for a given IMSI or equivalently for a given UE, define the following value:

-	CCCH_Intermediate=((IMSI mod 1000) mod (j=0MPj))

Then the corresponding CCCH_Group assigned to a UE is given by:

-	CCCH_Group = 0,   if  0 <= CCCH_Intermediate <= P0 – 1;

-	CCCH_Group = i, if  j=0i−1Pj<= CCCH_Intermediate <=j=0iPj- 1   (i = 1…M).

For an assigned CCCH_Group i, the UE Paging_Group shall be given by:

-	PAGING_Group = (IMSI mod 1000) mod (Pi * PG_MFRM)

Where:

IMSI = International Mobile Subscriber Identity, as defined in GSM 03.03.

mod = Modulo.

The CCCH_Group value (0, …, M) shall identify a CCPCH according to the following algorithm:

- Create an ordered list of CCPCHs. The P-CCPCH shall be the first CCPCH in the list. If there are S-CCPCHs, they shall be listed after the P-CCPCH. The order of S-CCPCHs corresponds to their order of appearance in the message System Information Type 4 (see TSM 04.08).

- Delete all CCPCHs that do not carry PCH blocks from the list.

- In the remaining list, assign the CCCH_Group value 0 to the first CCPCH in the list, ..., and the CCCH_Group value M to the last CCPCH in the list.

The UE in idle mode shall monitor, on the CCPCH corresponding to its CCCH_Group, all PCH blocks of its PAGING_Group to detect its paging messages.

### 6.5.3	Determination of specific paging multiframe and paging block index

The required 48-multiframe occurs when:

PAGING_ Group div Pi = (FN div 48) mod (PG_MFRM)

The index to the required paging block of the "available" blocks in the 48-multiframe:

Paging block index = PAGING_ Group mod (Pi)

After adding the number of BCCH blocks to the Paging block index the result is then used to determine the actual paging channel interleaved block to be monitored.

### 6.5.4	Determination of P-FACH_GROUP and P-RACH_GROUP for UE

In TD-SCDMA system the P-FACH_GROUP (or P-RACH_GROUP) is not one to one with CCCH_GROUP. Before sending CHANNEL REQUEST message an UE must first send a SYNC1 code (signature), which is randomly chosen out of 8 possible signatures used within the cell, on UpPTS and then listen to the downlink P-FACH for power and time advance being adjusted.

In case the network explicitly indicates (Access priority parameter IE, see TSM 04.08 [5]) to the UE which SYNC1 sequences are allowed for first accesses to the cell,  the first SYNC1 code shall be randomly chosen into the sub-set indicated by the network.

One or more P-FACH (or P-RACH) can be configured in a cell. After sending the signature the UE should determine which P-FACH_GROUP it belongs to from signature number chosen randomly.

Define:

M = number of configured P-FACH (or P-RACH)

P-FACH_GROUP = signature number chosen mod M

From P-FACH_GROUP an UE can know which P-FACH it will listen to and on which P-RACH the CHANNEL REQUEST message will be sent (see TSM 04.08 [5]). The relationship between P-FACH_GROUP and AGCH_GROUP is broadcast on BCCH.

### 6.5.5	Short Message Service Cell Broadcast (SMSCB)

When a short message service cell broadcast (SMSCB) message is to be sent, the message shall be sent on the cell broadcast channel (CBCH) in the configured CCPCH, in four consecutive multiframes using the block defined in Table 3 of clause 7. The multiframes used shall be those in which TB = 0,1,2 and 3, where :

TB = (SFN DIV 48)mod(8)

Extension to the Multiframes corresponding to TB=4, 5, 6 and 7 is also allowed.

The SMSCB header shall be sent in the multiframe in which TB = 0, and TB = 4 in case of extension. When SMSCB is in use, this is indicated within the BCCH data (see TSM 04.08 [5]), and the parameter BLK_CBCH for the configured CCPCH, shall be set to one.

NOTE 1:	The UE reading of the extended CBCH is occasionally interrupted by UE idle mode procedures.

NOTE 2:	The UE reading of the CBCH is occasionally interrupted by UE idle mode procedures when the UE is TD-PRS attached and in packet idle mode.

### 6.5.6	Determination of PAGING_GROUP for UE in TD-PRS attached mode

CCCH shall be used in the TD-PRS attached mode for paging and access. If the determination of the specific paging multi-frame and paging block index as specified in this sub-clause is not supported on CCCH by both the UE and the BTS, the methods defined in sub-clauses 6.5.2 and 6.5.3 shall be used. This is negotiated at TD-PRS attach.

Given CCCH_Group as calculated in section  6.5.2 and related parameters: i and Pi.

The n blocks corresponding to the PAGING_Group the UE listen to are given as follow:

PAGING_Group n  = ( ( (IMSI mod 1000) mod Ni) +

Max( (n * Ni ) div SPLIT_PG_CYCLE, n) ) mod Ni

for n = 0, ... , Min (Ni, SPLIT_PG_CYCLE) -1

where

Pi * 64 = Ni

IMSI = International Mobile Subscriber Identity, as defined in GSM 03.03.

mod = Modulo.

div = Integer division

The number of available PAGING_Group, in presence of DRX, is Ni.

SPLIT_PG_CYCLE is a UE specific parameter negotiated at TD-PRS attach (see TSM 04.60). SPLIT_PG_CYCLE defines the DRX period where

DRX period = (64 * 48* 5) / SPLIT_PG_CYCLE msec = 15,360 / SPLIT_PG_CYCLE sec.

In non-DRX mode (SPLIT_PG_CYCLE = 0) the UE shall listen to all Pi blocks per 48-multiframe on the CCPCH carrying its paging group.

In DRX mode, the UE shall listen to all PCH blocks corresponding to its paging groups as defined by the different PAGING_GROUP values.

The required multi-frame occurs when:

PAGING_Group n div Pi  = (FN div 48) mod 64

The index to the required paging block of the "available" blocks in the multi-frame:

Paging block index = PAGING_Group n mod Pi

where the index is then used to determine the actual PCH block to be monitored on given CCPCH channel.

NC_NON_DRX_PERIOD or NON_DRX_TIMER period (see TSM 04.60) is limited to 4 seconds.

If SPLIT_PG_CYCLE>32 is negotiated, SPLIT_PG_CYCLE=32 shall be used.

These limitations allow DRX to always be used in order to provide the UE enough time for BSIC/CIC and System Information decoding.

### 6.5.7	Voice group and voice cell broadcast call notification

When mobile stations are to be alerted on a voice group or voice broadcast call, notification messages shall be sent on the notification channel (NCH) in the configured CCPCH, using the blocks defined in sub-clause 6.5.1.

When the NCH is in use, the parameter BLK_RES in the configured CCPCH, shall be set to a value to one or greater than one.

# 7	Tables of mapping from logical channel onto physical channel

Logical channel	Allowable	Direction	Allowable	Allowable	Allowable	Repeat	Interleaved block

(Transport channel)	physical		time slot 	spreading	Rus	length TDMA	TDMA frame

channel		assignment	factor	occupied	in frames	mapping

TCH	DPCH	D&U		Ts0 ...Ts6	8,16	2	8	B0(0...7),B1(4...7,0...3)

(TCH-T/EFS)

TCH	DPCH	D&U		Ts0…Ts6	16	1	8			B0(0...7),B1(4...7,0...3)

(TCH-T/HS)

FACCH

(FACCH-T/F)	DPCH	D&U		Ts0…Ts6	8,16	2	8			B0(0...7),B1(4...7,0...3)

FACCH

(FACCH-T/H)	DPCH	D&U		Ts0…Ts6	16	1	12			B0(0...11),B1(4...11,0...3)

SACCH

(NOTE 1)	DPCH	D&U		Ts0…Ts6	8,16	1,2	48			B(0…47)

SACCH

(NOTE 2)	DPCH	D&U		Ts0…Ts6	16	1	48			B(0…47)

TCH	DPCH	D&U		Ts0...Ts6	8,16	2	16			B0(0...15), B1(4...15,0...3)

(TCH-T/F9.6)										B2(8..15,0...7), B3(12...15,0...11)

TCH 	DPCH	D&U		Ts0...Ts6	8,16	2	16			B0(0...15), B1(4...15,0...3)

(TCH-T/F14.4)										B2(8..15,0...7), B3(12..15,0...11)

BCCH	P-CCPCH	D		Ts0	16	2	48			B0(0…3), B1(4…7)

NOTE 1:	SACCH associated to either a TCH/EFS or a TCH/F9.6 or a TCH/F14.4

NOTE 2:	SACCH associated to a TCH/HS

Table 6: Mapping of logical channels onto physical channels (see sub-clauses 6.3, 6.4, 6.5)

Logical channel	Allowable	Direction	Allowable	Allowable	Allowable		Repeat	Interleaved block

(Transport channel)	physical	time slot	spreading	RUs		length TDMA		TDMA frame

channel		assignments	factor	occupied	in frames	mapping

AGCH

NCH

CBCH	P-CCPCH	D	Ts0	16	2	48	B0(4…7)…Bn(44…47) (NOTE 3)

PCH	P-CCPCH	D	Ts0	16	2	48	B0(4…7)…Bn(44…47) (NOTE 4)

AGCH	S-CCPCH	D	Ts0..Ts6	16	2	48	B0(0…3)…Bm(44…47)

NCH

PCH 	S-CCPCH	D	Ts0..Ts6	16	2	48	B0(4…7)…Bm(44…47) (NOTE 4)

CBCH	S-CCPCH	D	Ts0..Ts6	16	2	48	B0(8…11)…Bn(44…47)

RACH	P-RACH	U	Ts1..Ts6	8,16	1,2	48	B0(0), B1(1)…B47(47)

FACH	P-FACH	D	Ts0..Ts6	8,16	1,2	48	B0(0), B1(1)…B47(47)

FNICH	DPCH	D	Ts0..Ts6	8,16	1,2	48	B0(0), B1(1)…B47(47)

HOACH	DPCH	U	Ts1..Ts6	8,16	1,2	48	B0(0), B1(1)…B47(47)

PACH	PDPCH	U	Ts0..Ts6	8,16	1,2	48	B0(0), B1(1)…B47(47)

PAKCH	PDPCH	D	Ts0..Ts6	8,16	1,2	48	B0(0), B1(1)…B47(47)

PDTCH,	PDPCH	D&U	Ts0..Ts6	16	1	4			B0(0,1,2,3)

PACCH

PDTCH,	PDPCH	D&U	Ts0..Ts6	8,16	1,2	4			B0(0,1,2,3)

PACCH

NOTE:	for the mapping of packet logical channels onto packet physical channels see TSM 04.60 [12].

NOTE 3:	the first block can be B0(8..11) if 2 BCCH  blocks are used and PCH is not configured, if PCH is configured and 2 BCCH  blocks are used the first block can be B0(20,..23), and finally, if 1 BCCH block and 3 PCH blocks are used, the first block can be B0(16..19).

NOTE 4:	the first block can be B0(8..11) if 2 BCCH  blocks are used.

Table 7: Mapping of logical channels onto physical channels (see sub-clauses 6.3, 6.4, 6.5)

Logical channel	                  Allowable	Direction	Allowable	Allowable		Allowable			Repeat			Interleaved block

(Transport channel)	physical		time slot	spreading		RUs				length TDMA		TDMA frame

channel 		assignments	factor		occupied			in frames			mapping

FACCH(NOTE 5)	DPCH	D	Ts0..Ts6	8,16	1, 2	24				B0(12,15,18,21)
(SDCCH-T/3 Mode1, SDCCH-T/3 Mode2)	U							    		B0(0,3,6,9)
			D									B1(13,16,19,22)
			U									B1(1,4,7,10)
			D									B2(14,17,20,23)
			U									B2(2,5,8,11)

SACCH		DPCH	D	Ts0..Ts6	8, 16	1, 2	24				B0(0,3,6,9)
(SDCCH-T/3 Mode1, SDCCH-T/3 Mode2)	U									B0(12,15,18,21)
(NOTE 5)(NOTE 6)			D									B1(1,4,7,10)
			U									B1(13,16,19,22)
			D								       	B2(2,5,8,11)
			U									B2(14,17,20,23)

NOTE 5:	this configuration (vi) of sub-clause 6.4.1) uses 1x 24-TDMA frames, and is described in sub-clause D1 of Annex D.

NOTE 6:	SACCH associated to a FACCH.

Table 8: Mapping of logical channels onto physical channels (see sub-clauses 6.3, 6.4, 6.5)

Logical channel	Allowable	Direction	Allowable	Allowable	Allowable		Repeat		Interleaved block

(Transport channel)	physical		time slot	spreading	RUs			length TDMA			TDMA frame

channel 		assignments	factor		occupied			in frames				mapping

FACCH (NOTE 7)	DPCH	D	Ts0 ..Ts6	8, 16	1,2	48	B0(24,30,36,42)
(SDCCH-T/6 Mode1, SDCCH-T/6 Mode2)U					B0(0,6,12,18)
		D					B1(25,31,37,43)
		U					B1(1,7,13,19)
		D					B2(26,32,38,44) 
		U					B2(2,8,14,20)  
		D					B3(27,33,39,45)
		U					B3(3,9,15,21)
		D					B4(28,34,40,46)
		U					B4(4,10,16,22)  
		D					B5(29,35,41,47)
		U					B5(5,11,17,23)

SACCH	DPCH	D	Ts0 ..Ts6	8, 16	1,2	48	B0(0,6,12,18)
(SDCCH-T/6 Mode1, SDCCH-T/6 Mode2)	U					B0(24,30,36,42)
(NOTE 7)(NOTE 6)		D					B1(1,7,13,19
		U					B1(25,31,37,43)
		D					B2(2,8,14,20)
		U					B2(26,32,38,44)
		D					B3(3,9,15,21)
		U					B3(27,33,39,45)
		D					B4(4,10,16,22)
		U					B4(28,34,40,46)
		D					B5(5,11,17,23)
		U					B5(29,35,41,47)

NOTE 7: 	this configuration (vi) of sub-clause 6.4.1) uses 1x 48-TDMA frames, and is described in sub-clause D2 of Annex D.

Table 9: Mapping of logical channels onto physical channels (see sub-clauses 6.3, 6.4, 6.5)

Figure 8: Mapping of logical channels onto physical channels through transport channels based on the physical resource

Framex

|  | Frame(x+1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Td0 |  | Tu0 | Tu1 | Tu2 | Td3 | Td2 | Td1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ……… |  |
| B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ……… |  |
| C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ……… |  |

.

one time slot

.

| D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ……… |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ……… |  |
| F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ………. |  |

One basic resource unit (RU)

Figure 9:  The structure imposed on the physical resource: Timeslots, TDD Frames and  code channels (16 code Channels)

| CHANNEL | FRAME NUMBER |
| --- | --- |
| BCCH | 1 |
| BCCH | 2 |
| BCCH | 3 |
| BCCH | 4 |
| PCH/BCCH/or other | 5 |
| PCH/BCCH/or other | 6 |
| PCH/BCCH/or other | 7 |
| PCH/BCCH/or other | 8 |
| PCH/or other | 9 |
| PCH/or other | 10 |
| PCH/or other | 11 |
| PCH/or other | 12 |
| PCH/or other | 13 |
| PCH/or other | 14 |
| PCH/or other | 15 |
| PCH/or other | 16 |
| PCH/or other | 17 |
| PCH/or other | 18 |
| PCH/or other | 19 |
| PCH/or other | 20 |
| PCH/or other | 21 |
| PCH/or other | 22 |
| PCH/or other | 23 |
| PCH/or other | 24 |
| PCH/or other | 25 |
| PCH/or other | 26 |
| PCH/or other | 27 |
| PCH/or other | 28 |
| PCH/or other | 29 |
| PCH/or other | 30 |
| PCH/or other | 31 |
| PCH/or other | 32 |
| PCH/or other | 33 |
| PCH/or other | 34 |
| PCH/or other | 35 |
| PCH/or other | 36 |
| PCH/or other | 37 |
| PCH/or other | 38 |
| PCH/or other | 39 |
| PCH/or other | 40 |
| PCH/or other | 41 |
| PCH/or other | 42 |
| PCH/or other | 43 |
| PCH/or other | 44 |
| PCH/or other | 45 |
| PCH/or other | 46 |
| PCH/or other | 47 |
| PCH/or other | 48 |

Figure 11: Example of TDD Frame Mapping for control channels

Note: “other” includes NCH, CBCH, AGCH.

# Annex A (normative): 
A.1	P-CCPCH filling

The P-CCPCH shall always be active in the cell. In case no valid information is available the following rule applies for filling the burst on the P-CCPCH:

a predefined sequence of 23 octets, before coding (see TSM 04.06 [4]).

## A.2	Half burst filling

In case of DTX, when a given traffic frame is scheduled for transmission and one of its adjacent traffic frames is not scheduled for transmission, half of the bits belonging to the bursts associated with the scheduled traffic frame need to be filled. The burst used to fill is referred to as "dummy burst".

The dummy burst shall not require channel coding procedures (bit scrambling, channel coding, interleaving) except for spreading operation.

The spreading code is that of the traffic burst that the dummy burst replaces.

A dummy burst is a sequence of “10”. The length of the sequence depends on the SF, mode and channel coding procedure (i.e. the interleaving depth; see TSM 05.03 [6]) used to code the traffic burst that the dummy burst replaces, following the same bit distribution rule on the physical channel (see TSM 05.03 [6]).

## A.3	S-CCPCH filling

The S-CCPCH(s) need not to be always active in the cell. In case no valid information is available the following rule applies on the S-CCPCH(s):

It shall be switched off.

# Annex B (normative): 
Multi-Resource capability

## B.1 Constraints imposed by the service selected

The service selected will impose certain restrictions on the allowed combinations of transmit and receive timeslots. Such restrictions are not imposed by this annex but should be derived from the description of the services.

The service selected will determine whether or not adjacent cell power measurements are required and therefore whether the time slot configuration is allowed for.

## B.2 Network requirements for supporting MS multi-resource classes

The multi-resource class of the UE will limit the combinations and configurations allowed when supporting multi-resource communication.

It is necessary for the network to decide whether the UE needs to perform adjacent cell power measurement for the type of multi-resource communication intended and whether the service imposes any other constraints before the full restrictions on TS assignments can be resolved.

Note The multi-slot capability classes are defined in Annex G.

# Annex C (normative):

# C.1 Synchronisation sequences

For the purposes of Annexes C and F, the SYNC, SYNC1 and midamble codes are defined as chip sequences ULS, DLS and M respectively, of the following size:

The SYNC DLS sequence is composed by 64 chips;

the SYNC1 ULS sequence is composed by 128 chips;

the midamble M sequence is composed by 144 chips.

Tables C.1.2 and C.1.3 report the all possible DLS and ULS sequences respectively available for a TD-SCDMA system; while table C.1.4 reports the all possible basic midamble sequences (used to generate the all possible midamble codes, see sub-clause F.1) available for a TD-SCDMA system.

With reference to the above mentioned tables, Table C.1.1 defines the mapping rules between 4 binary elements and the corresponding hexadecimal digit.

| 4 binary elements | Mapped on hexadecimal digit |
| --- | --- |
| -1-1-1-1 | 0 |
| -1-1-1 1 | 1 |
| -1-1 1-1 | 2 |
| -1-1 1 1 | 3 |
| -1 1-1-1 | 4 |
| -1 1-1 1 | 5 |
| -1 1 1-1 | 6 |
| -1 1 1 1 | 7 |
| 1-1-1-1 | 8 |
| 1-1-1 1 | 9 |
| 1-1 1-1 | A |
| 1-1 1 1 | B |
| 1 1-1-1 | C |
| 1 1-1 1 | D |
| 1 1 1-1 | E |
| 1 1 1 1 | F |

Table C.1.1: Mapping of 4 binary elements on a single hexadecimal digit

## C.1.2	The table of SYNC codes

| SYNC sequence number | SYNC codes of length 64 |
| --- | --- |
| 0 | B3A7CC05A98688E4 |
| 1 | 9D559BD290606791 |
| 2 | 2CE7BA12A017C3A2 |
| 3 | 34511D20672F4712 |
| 4 | 9A772841474603F2 |
| 5 | 9109B1A5CE01F228 |
| 6 | 8FD429B3594501C0 |
| 7 | 25251354AA3F8C19 |
| 8 | C9A3B8E0C043EA56 |
| 9 | BA04B888E5BC1802 |
| 10 | A735354299370207 |
| 11 | 74C3C8DA4415AE51 |
| 12 | F4FD0458A0124663 |
| 13 | A011D4E16C3D6064 |
| 14 | BDA0661B0CAA8C68 |
| 15 | 8E31123F28928698 |
| 16 | F095C1632E2906AB |
| 17 | B60B4A8A664071CF |
| 18 | AA094DCCE91E041A |
| 19 | C0C31CDA8A256807 |
| 20 | D516964FB18C1890 |
| 21 | 30DE01834F4AACCE |
| 22 | 8F700323BA5CAD34 |
| 23 | 1B50F4DEE0C1380C |
| 24 | 443382164F56F2D1 |
| 25 | E1E4005D49B846B4 |
| 26 | 040A97165330BFAA |
| 27 | C48E26881693AD78 |
| 28 | D4354B2FE02361CC |
| 29 | 5383AB6C8A10CE84 |
| 30 | D417A730F2F12244 |
| 31 | ABF0A0D905A939C4 |

## C.1.3	The table of SYNC1 codes

| SYNC1 sequence number | SYNC1 codes of length 128 |
| --- | --- |
| 0 | C11C20F0D1807DB8859175B798EC094A |
| 1 | 91278068081EC8E74543DBC1C9AD4235 |
| 2 | 38F5AEE2E513DB12A663BA04160103E5 |
| 3 | 7AA8A0A210F12A1E4332F2EDD33011FC |
| 4 | C180EA3B9BA1774EB9611BD249C4A508 |
| 5 | B072A2C839489D496B98CE9D0132FBC9 |
| 6 | B2723EAC6EB01667F2B33961C8074234 |
| 7 | C4144AD060F0EC095E227B92CF7C8280 |
| 8 | 653036A10D3054146FCF815986C63A14 |
| 9 | F899CA61435D64DC07FDF04C4A0C053A |
| 10 | B56F2D6893A8051407F4C341D88DC7DC |
| 11 | DC0BE838242142EDE6413A72C88D74AA |
| 12 | 22A2FD86E4086C70A4860B13C76E579F |
| 13 | A3CBC21322C97D2A02728E7875F39588 |
| 14 | D4EC4F694A082CB38E3B1558A0FCC89F |
| 15 | CC891141C4E216D235C15CF5D3F9B002 |
| 16 | A1993114C50B77CB0C0725D1E22FD016 |
| 17 | 24F73A979DE52F82E8800CCB93842A59 |
| 18 | 8F878FA04659842E294D8DEAB20BA2FD |
| 19 | AC90B0442D70662B028CF76A6BECDF09 |
| 20 | D94A284DF64D7B0102F0E084C29C88C8 |
| 21 | 8603200C7596F24E865FD3815693358D |
| 22 | B466B12CF433642BD8B08F1F452E0550 |
| 23 | 86A3A1772C1C99FCA7DBBA0C312E34A0 |
| 24 | 622A1889F72A9A2C042D46F08EFEE1AC |
| 25 | BF220A362BC0D3B0D7CE400954C6CFAE |
| 26 | D28D73C52E89CF57905C502244F63616 |
| 27 | AD4E1C2103697D64D8B9D4C035D90548 |
| 28 | 8F081A9BA12B6C6BD024531AA984D21C |
| 29 | E4092429BE82988E1E3585BF6A6AE550 |
| 30 | 08BD36E0A9C061782CB38B35B335CA56 |
| 31 | 1CDFF3CC2685D1C44F4A1059AB03F40A |
| 32 | 506ED4E88FB1CECE3243F2A27A0221A4 |
| 33 | 846CF58A7AB613C83A24130B5778C0E2 |
| 34 | A2711A99E26A0C75AC026F4CFAECE893 |
| 35 | D846EEEBA2432AC05A01043C62579DCF |
| 36 | 6B16B4E851CAF2121FC4CF88820C89E7 |
| 37 | AA4889A78207674A74E10C6F2BE11D48 |
| 38 | 8534CF8145BC991052814ED5C72709EE |
| 39 | 01AEF15D2290A84A607425746D9963C7 |
| 40 | 999188F758245D5164FE16D852942C71 |
| 41 | CF71C008599287E446E30745BD56E2D2 |
| 42 | 248414BA0DF8CDC4711FE7C8707ED0AD |
| 43 | EB2E263EC016191C81AB714BFE4D2B30 |
| 44 | 862082A7482FAC1C499793A0D8CED670 |
| 45 | DE2C22B2783AB75A7342608DE413840A |
| 46 | E31AA60B727F2CA2A78DAAC10665011D |
| 47 | CEF6CD06509870AC9E0177ACD550921D |
| 48 | E52C84D499FFCDC287581691471540F2 |
| 49 | B33BF6551A4322504BEE0930BCA1EC68 |
| 50 | 555BE6886D0FC43D72315E6C6D384148 |
| 51 | 8444F67451EE23CE1240C90F0B52A492 |
| 52 | 5C290D28E84060E69D09788A261B10FF |
| 53 | 337E0C35E83CD38CCC5D45804241F952 |
| 54 | A7879F0D31A8982A01EE6AC4952984DC |
| 55 | A37F506508928C70A83D69A2373781B9 |
| 56 | 42F55208EE12909803A7CBEB19B5419E |
| 57 | 57E5E268A328FCC9ED04B9E5420AC702 |
| 58 | EB033AD1222F84D8642C4E3FAAD28206 |
| 59 | 98EE1415F026AC0E862C520451697DD0 |
| 60 | 6A0528AEA4B7CD6702660D81F8821E19 |
| 61 | 763D626A87C603BCB09E1A4C800A378F |
| 62 | EEA61897879289340C23F669D6A03762 |
| 63 | A6571B3CC2D0E04F017ACC808B92DCE7 |
| 64 | DDF88B52EA1831D293A803CF23C8C471 |
| 65 | 6CA4D333A2684140475DAB491F61C17A |
| 66 | A7D2AD23043989A13289F7C3E135580A |
| 67 | B1C752FA66B41C81904EDE27EA000E2E |
| 68 | 8694BE3CC1CB36BE2A095F89CC619080 |
| 69 | 9C20334E1BBC596B25E151180BF99940 |
| 70 | 484256214F81070DD9C49A2B05A43DCE |
| 71 | 401A20BCBE29B7438A7AEE44635A9E23 |
| 72 | 8858585C3239CBF628033FA0DF189378 |
| 73 | EFA36404C1BA5118CC5F9052FD28D9C3 |
| 74 | 155609873D8A042D496E6477B747C4F8 |
| 75 | 8446077883A6D7D2549CC9742E3FD023 |
| 76 | E630142B189AA209371A6F0FFDBC30A7 |
| 77 | C46060535AC6DBB2095F1D7826D0CD5C |
| 78 | E00D19E48797148B28DEDA9D429362E2 |
| 79 | 645DE447E938485489416CAFCC1C571F |
| 80 | DA10AFBF2AE61C593A1D88584DE30598 |
| 81 | BB248AEA5FD3FE210CD48FC401E1A686 |
| 82 | A89F146BD9191F445301C081CB6F5625 |
| 83 | 15BBF04F247C59150208949EB6B9CC58 |
| 84 | 08F48BFA7804B5B2CC2E96510232E062 |
| 85 | 9AA2BE74005A3679C626B209580B8D03 |
| 86 | 9D40664A2C808F2F293E255398B37E6A |
| 87 | 6869C98A8AAD81CAE41A23C83FF9EEA0 |
| 88 | 576E8948E61BD0927C4140C3C04C4CF3 |
| 89 | 0F942C67A1137B6EAA058C2A74872C73 |
| 90 | 9D058E27ED546C10632684BBC84E5BC1 |
| 91 | 79D4B840E20148B134F90B51164BCBD0 |
| 92 | 0E35E1D8D1214C05FAC790B69B239150 |
| 93 | FFA1BB0232CD71480BE5CA1C2A269F89 |
| 94 | B2956F5F4E270446F9211584792628DB |
| 95 | F56CCA23421C8EC8F8A41F7DA4A41EA2 |
| 96 | 0B5ECA04F1789A7148C80C39D57D05F6 |
| 97 | A10B538E8A8CFC8F8925C485F2A88660 |
| 98 | 9925C2C715001D9FC78ACCC51DA1AF34 |
| 99 | 0DAC9CFDEA40429A8B12C7D320D60F70 |
| 100 | 377FC9A097017958440914E83118E39D |
| 101 | 8421096FA8B47E4E943B6473671955CC |
| 102 | 574086183477C4F68540CB7E858263B1 |
| 103 | 895B6A8980C6703C779F49F40C5CFC19 |
| 104 | D0D253E157BC19262150CEA668679E71 |
| 105 | B8889C60EBA812BD7F0B6498823296D2 |
| 106 | A13FB9F3A08528E44B13C12CF0D461AA |
| 107 | 8D4DCFBE43D6E2024B1F8470224AA330 |
| 108 | 536D159E119E0893838657B12A074E64 |
| 109 | DCFD49C504AD3A2F049A0CB70238EC8A |
| 110 | D363DB4C46C11757FA8FB18139789102 |
| 111 | 424A1E8A1D4DA256E4CA3BC8C2201BE3 |
| 112 | 417B619ED30FEB0A847CC3A191A20398 |
| 113 | 843FBBC95453C61786D1332612B45B4D |
| 114 | F26CACC0732CF8ED0C5BC1462B1620B4 |
| 115 | 88E0FE440C70E9249A92A7AF94638880 |
| 116 | 99A52B7D8C950308057E0661D7459960 |
| 117 | A5C28218BF5D16E63E42698A0A6B0896 |
| 118 | B2763BEEC784A12E8C50778536921806 |
| 119 | 987B2B6A3A77A059B30A082457AB84E0 |
| 120 | 820DB500F1B206358D7A7F210AB85AA8 |
| 121 | 97760A5CFC5E03EB439C914590045938 |
| 122 | 896A720E8857C8708A59F8C94DE0841E |
| 123 | 2D101F0CF95263843412577340DEBB11 |
| 124 | E8E5214B4DCF5D11A245B0149D49C87C |
| 125 | 51224EAA10099ACDE384834A5ADF03D8 |
| 126 | 64E51253554A230C186FDE4E8781BC09 |
| 127 | A499E391E69ED08890AC1A82A6115BEC |
| 128 | EE54C6E1834210D3EC1B07A456B92AA8 |
| 129 | 949DB5CA82420B54C1E0BCC111E704D9 |
| 130 | 9439EE9A9E4C447D1AA350926495047F |
| 131 | AD095CC0E7438AECE38D60980B3F2D00 |
| 132 | 83089C254C5EE9788072BC3D9282F798 |
| 133 | A27DC1A457BC5A56563D8A9B11203615 |
| 134 | 713053A9C0B1B08B14705FF5A7244DB4 |
| 135 | D36D4B9F4007354E0EC1B0CA8C8C7124 |
| 136 | 82E7C990612114F1CCE1BD9509FD4386 |
| 137 | C8D83FF0B48B14830D2015D53F8C0672 |
| 138 | 08AF223C869A36B169148FDDABB7D120 |
| 139 | B6C284C600AD0A99F86C449F8F4C53A6 |
| 140 | DC741B320C07682AF92AC4DBDE0C28C2 |
| 141 | 89B8D84FA902265850C0FA6FF0EB2C4F |
| 142 | A69445B3A52201DB984BC03D1956D7F3 |
| 143 | 0FE0F7224B7AD72E4D4530D0223F590C |
| 144 | 1B8C06F051434048EB925133AD3BD3F9 |
| 145 | E133D4C3C942726A351300C37E55D0DF |
| 146 | 9E09481D1881A66F562D8B453BC83AB2 |
| 147 | 2397B04B60A3C5700907BDBBA4E818C8 |
| 148 | 8F81F7A08CC6C8DA3D692AD34F50C012 |
| 149 | 9AB325352981BCCFA072F8FDE3009221 |
| 150 | 4FA88B7F1F8A620C31B0D486C52AC2F6 |
| 151 | 097AF0ADD16D7D39851049F0130EE444 |
| 152 | A5027732DACFF11C388D5820A4A9BA49 |
| 153 | 1CD981EA2EDB46218A407C7E20D4BE84 |
| 154 | D0FD94279FA67EC61A3904C0AD8ACA04 |
| 155 | EA73A9415EC2004D49E9D0F645961C75 |
| 156 | 005AF0614A7552041194DEECBF8DD016 |
| 157 | B514481533DA0A731705B93CF634E40D |
| 158 | 983054521841A6E4FF34B2C07B5684FE |
| 159 | C46D927D0FD2B2F509550025677C6871 |
| 160 | 2AD85C08127487C87ECE014D65169102 |
| 161 | 0F617852FA3930AA7EE74B400B2CC831 |
| 162 | AE9D395004C6E27540C378625D36E0D6 |
| 163 | DC4FA55750F10B0636248F12C212FFE4 |
| 164 | D3602B8D6CBF1809C88B827185631ECF |
| 165 | A94825850708E7723EA8F22C44BF78B2 |
| 166 | A62D231C16AEEFE0B0026B306662945A |
| 167 | 9C7BE810A86465A50551F89125D93B12 |
| 168 | 9712D9338B9CC60485C10172F50F121F |
| 169 | A3902CE0E0B9912591FF28C695728257 |
| 170 | 4167057891AB29473A9E0F67F3658921 |
| 171 | B3368B91EC12A284BC414C8F0D7F8D20 |
| 172 | EE21888101ABF06C1175828CB58B598D |
| 173 | E43923A00ECC32CCC2D162A4A44BD7F4 |
| 174 | CC9E30B8538AD51703EEB6F70801AB22 |
| 175 | B908AD2F1501DA1C156811736CD798CD |
| 176 | 2B46302ACCC2F808797FC648A614326D |
| 177 | 8A54494F1BE27235B8764023AA0FBCFA |
| 178 | BC1041E6F636421E89277DC154439103 |
| 179 | 275B39A63029B974E3561AE0A8FC8032 |
| 180 | 9283F6FE819B80492A22B85CE5CE5DC4 |
| 181 | 4CCB52C0CE058A78022C22DF5788CBCC |
| 182 | B0DF9608DE549A6F6C581516919A81E6 |
| 183 | 2CA185163CC36060D1E85BB0A7FBB988 |
| 184 | 66101D2846155CAC986FC790D2124EFC |
| 185 | 8016E3904644D2093579B83BD7AB5071 |
| 186 | 531CAB7085BEC14257439658023647CF |
| 187 | DF2910165AA5051E41F6EB198E4D491C |
| 188 | BA32052042B0FB2188DE7857DA1B6788 |
| 189 | 9E6D075AFF0EA4153615E140BF380666 |
| 190 | 9ACC5A037902534642A3BE391AA40F9B |
| 191 | 4D741A3B4499843010D7E5FA8988DC80 |
| 192 | FA1421C96EDC6092726154560B1C2FC8 |
| 193 | 882946076223CAE0B0BFE3EDA59826D5 |
| 194 | CEBB288C28B7472A0D3917012276C034 |
| 195 | BD35A6E00C9528DB38289CF823C34F30 |
| 196 | E2C93618B6B2800D51171A5F85746A55 |
| 197 | B43EF39A1A64F0E220AF740F9494291B |
| 198 | AC537817C2612744A58132A8AFBC44A3 |
| 199 | 98A321249A821DDBF81C38235A371A14 |
| 200 | AE1D46069090D81BB6B08FED9E687285 |
| 201 | 7EAE2415DC2CD60AE083249A33B56E05 |
| 202 | 3D942AAA9BC9F27289421CE0B301FB98 |
| 203 | 1548BA6D08530727AC6D059C005C6C42 |
| 204 | FF47C21142C65B502DA70647BAE831D1 |
| 205 | C83AA7FEAC5E51A08091E10DB0C233D9 |
| 206 | E86EDD2EC2DAA3104229EDC43471A16A |
| 207 | 22FAFB9C184B78B56EE91B6602C03244 |
| 208 | E45631DC509B1290C08D2C1A1F15DBFE |
| 209 | D203C51207092B56568FDAD9E2D44473 |
| 210 | 2AA87F31A7D1AB1C90024F936006C4A5 |
| 211 | 913136153593DEABC7305BF0C5A62180 |
| 212 | D8DA5FE401F2758642A082C53A6A5CB8 |
| 213 | 23C2295213147F324DE8EC1C103BAE88 |
| 214 | 883AF097FCDE82B366A1844245E0D727 |
| 215 | 79E5E9F8C933159ACADC22A06F900A70 |
| 216 | FE40502B44A9E44B2C336250D47538CC |
| 217 | 670452E19172C843176F1278FE41D584 |
| 218 | B7EAA436078E6886A3024F593AD57580 |
| 219 | 1044D4CDD7230E7B1953AD1232DF07E2 |
| 220 | 4D821ECAC3D845A2E1011695624576FF |
| 221 | 96622ED2FBD44D1B859D70601999F438 |
| 222 | CCC31C3D6D5B41B8D82FF4522A4C0146 |
| 223 | 4A84F7CD62E0C712980E6A0C89BF394F |
| 224 | 10E56751F000927284DBE174E68ECC4C |
| 225 | A3DE70921356F026E084CFE302A210A9 |
| 226 | B12DA0621B343A8C3FE941A32EA5D571 |
| 227 | D653135DE825A74B743E275C19020C71 |
| 228 | 5CAD301BF846B2EE921D33A3D4BB1220 |
| 229 | 1292445ACBB548C668FC3853578474E6 |
| 230 | B94B4B89C0654688C9E007D9061DF5FE |
| 231 | 75A2C91E76061A8680884E8BFD14A64A |
| 232 | 83726F3070B47ECE21504A5065D74A36 |
| 233 | 964A471444A270840919F7FE07382D14 |
| 234 | A582701EBFCA899B8497088C3560F300 |
| 235 | 64FCB63E21CAC63002D1E09FD1543274 |
| 236 | B1E1C83F689ADF422C865F98D288838A |
| 237 | A06A0D822165D3F3416B47419ECCB547 |
| 238 | 1D2068039A32B7EF728914ECE07CB416 |
| 239 | 64C0CF81F78E8823ECC8661A5295422A |
| 240 | 902A7243F593F2180E5A306A8438E6A9 |
| 241 | A4CCED356D56BF1B41C28E1504301FE8 |
| 242 | 82AE90E2F76B3055A2E3A966025CC01A |
| 243 | 8B90D5A62364E18574145C5895CEFF60 |
| 244 | 43F7EA1AB0D19032551AD9DE21307353 |
| 245 | DD5D8424AC60360B1C14E65815C9B15E |
| 246 | C632A67382ECB2681DFB8525140E2878 |
| 247 | 3A6ACF212B6F8B9C53FF224C2E00C16C |
| 248 | 86A90C267B1171093F362FE5CB14E3A0 |
| 249 | EA262EC36E6589C3BB005426AF2590F4 |
| 250 | 200F03126C5B0D7B901128E7757C5F70 |
| 251 | 68FC090C2221AA98BF0D24E85066EFC2 |
| 252 | 9E26CEC67832FC42A87E92FA1015212E |
| 253 | ACD889634F79506F2582EA03240F2A07 |
| 254 | AA65407E1F4A33BF9A62860A3D6A4CC0 |
| 255 | B1B950AC76A608AA32D04B03C7FF24D3 |

## C.1.4 	The table of Basic midamble codes

| Basic Midamble code ID | Basic Midamble codes of length 128 |
| --- | --- |
| 0 | B2AC420F7C8DEBFA69505981BCD028C3 |
| 1 | 0C2E988E0DBA046643F57B0EA6A435E2 |
| 2 | D5CEC680C36A4454135F86DD37043962 |
| 3 | E150D08CAC2A00FF9B32592A631CF85B |
| 4 | E0A9C3A8F6E40329B2F2943246003D44 |
| 5 | FE22658100A3A683EA759018739BD690 |
| 6 | B46062F89BB2A1139D76A1EF32450DA0 |
| 7 | EE63D75CC099092579400D956A90C3E0 |
| 8 | D9C0E040756D427A2611DAA35E6CD614 |
| 9 | EB56D03A498EC4FEC98AE220BC390450 |
| 10 | F598703DB0838112ED0BABB98642B665 |
| 11 | A0BC26A992D4558B9918986C14861EFF |
| 12 | 541350D109F1DD68099796637B824F88 |
| 13 | 892D344A962314662F01F9455F7BC302 |
| 14 | 49F270E29CCD742A40480DD4215E1632 |
| 15 | 6A5C0410C6C39AA04E77423C355926DE |
| 16 | 7976615538203103D4DBCC219B16A9E1 |
| 17 | A6C3C3175845400BD2B738C43EE2645F |
| 18 | A0FD56258D228642C6F641851C3751ED |
| 19 | EFA48C3FC84AC625783C6C9510A2269A |
| 20 | 62A8EB1A420334B23396E8D76BC19740 |
| 21 | 9E96235699D5D41C9816C921023BC741 |
| 22 | 4362AE4CAE0DCC32D60A3FED1341A848 |
| 23 | 454C068E6C4F190942E0904B95D61DFB |
| 24 | 607FEEA6E2E99206718A49C0D6A25034 |
| 25 | E1D1BCDA39A09095B5C81645103A077C |
| 26 | 994B445E558344DE211C8286DDD3D1A3 |
| 27 | C15233273581417638906ADB61FDCA3C |
| 28 | 8B79A274D542F096FB1388098230F8A1 |
| 29 | DF58AC1C5F44B2A40266385CE1DA5640 |
| 30 | B5949A1CC69962C464401D05FF5C1A7A |
| 31 | 85AC489841ED3EAA2D83BBB0039CC707 |
| 32 | AE371CC144BC95923CA8108D8B49FE82 |
| 33 | 7F188484A649D1C22BDA1F09D49B5117 |
| 34 | ADAA3C657089DEF7C0284903A491C9B0 |
| 35 | C3F96893C7504DC3B51488604AF64F4C |
| 36 | B4002F5AE0CE8623AC979D368E9148C1 |
| 37 | 0EEBCC0C795C02A106C24ABB36D08C6E |
| 38 | 4B0F537E384A893F58971580D9894433 |
| 39 | 08E0035AB29B7ECC53C15DAA0687CC8F |
| 40 | 8611ACBC4C82781D77654EE862506D60 |
| 41 | 63315261A8F1CB02549802DBFD197C07 |
| 42 | 9A2609A434F43E7DCADC0E22B2EF4012 |
| 43 | F4C9F0A127A88461209ABF8C69CE4D00 |
| 44 | C79124EE3FFC28C5C4524D2B01670D42 |
| 45 | C91985C4FED53D09361914354BA80E79 |
| 46 | 82AA517260779ECFF26212C1A10BDC29 |
| 47 | 561DE2040ACB458E0DBD354E43E111D9 |
| 48 | 2E58C7202D17392BC1235782CEFABB09 |
| 49 | C4FAA121C698047650F6503126A577C1 |
| 50 | E7B75206A9B410E44346E0DAE842A23C |
| 51 | 3F8B1C32682B28D098D3805ED130EA7F |
| 52 | 8D5FC2C1C6715F824B401434C8D4BB82 |
| 53 | 0B2A43453ACC028FE6EB6E1CB0740B59 |
| 54 | BC56948FC700BA4883262EE73E12D82A |
| 55 | 558D136710272912FA4F183D1189A7FD |
| 56 | 5709E7F82DC6500B7B12A3072D182645 |
| 57 | 86D4F161C844AE5E20EE39FD5493B044 |
| 58 | 8729B6EDC382B152185885F013DAE222 |
| 59 | 154C45B50720F4C362C14C77FE8335A1 |
| 60 | C6A0962890351F4EB802DE43A7662C9E |
| 61 | D19D69D6B380B4B22457CB80033519F0 |
| 62 | C7D89509FB0DAE9255998E0A00C2B262 |
| 63 | DFD481C652C0C905D61D66F1732C4AA2 |
| 64 | 06C848619AF1D6C910A8EAC4B622FC06 |
| 65 | 0635E29D4E7AC8ABC189890241F45ECA |
| 66 | B272B020586AAD7B093AC2F459076638 |
| 67 | B608ACE46E1A6BC96181EEDD88B54140 |
| 68 | 0A516092B3ED7849B168AFE223B8670E |
| 69 | D1A658C5009E04D0D7D5E9205EE663E8 |
| 70 | AC316DC39B91EB60B1AABD8280740432 |
| 71 | E3F06825476A026CD287625E514519FC |
| 72 | A56D092080DDE8994F387C175CC56833 |
| 73 | 15EA799DE587C506D0CD99A408217B05 |
| 74 | A59C020BAB9AF6D3F813C391CA244CD2 |
| 75 | 74B0101EB9F3167434B94BABC8378882 |
| 76 | CE752975C8DA9B0100386DB82A8C3D20 |
| 77 | BBB38DCDB1E9118570AC147DC05241A4 |
| 78 | 944ABBF0866098101F6971731AB2E986 |
| 79 | 2BB147B2A30C68B4853F90481A166EB6 |
| 80 | 444840ACCF3F23C45B56D7704BF18283 |
| 81 | 87604F7450D1AD188C452981A5C7FC9B |
| 82 | 8C3842EBC948A65BC4C8B387F11B7090 |
| 83 | 10B4767D071CF5DB2288E4029576135A |
| 84 | 6F07AAB697CD0089572C6B062E2018E4 |
| 85 | D3D65B442057E613A8655060C8D29E27 |
| 86 | 5EDA330514C604BF4E0894E09EC57A74 |
| 87 | B0899CD094060724DED82AE85F18A43A |
| 88 | B2D999B86DF902BC25015CAE3A0823C4 |
| 89 | C23CD40F04242B92D46EED82CD9A9A18 |
| 90 | D22DDCC5CB82960125DD24655F3C8788 |
| 91 | 54987218FBD99AE4340FD4C9458E9850 |
| 92 | BE4341822997A7B11EA1E8A1A2767005 |
| 93 | 255200FBA6EE48E6DE0A82B0461B8D0F |
| 94 | 6FBD58A663932423503690CF9C171701 |
| 95 | D215033A4AA87EC1C232BAC7EDA09370 |
| 96 | CA0959B01AE48E80204F1E4A3F29CE55 |
| 97 | 582043413B9B825903E3A3545ED59463 |
| 98 | 5016541922971C703D16E284CBDF633B |
| 99 | 7347EF160A1733CA98D43608A83A920B |
| 100 | 908B22AD433CCA00B3FD47C691F1A290 |
| 101 | BB22A272FC6923DF1B43BA4118806570 |
| 102 | 0FA75C87474836B47DC7624D61193802 |
| 103 | A22EBA0658A4D0FF1E9CA5030A65CC06 |
| 104 | 6C9C51CA15F1F4981F4C46180A6A6697 |
| 105 | 4C847ACF8BC15359C405322851C9BDE2 |
| 106 | C1D29499C0082C9DE473ED15B14D63E0 |
| 107 | 7E85ECC98AC761005076C5572869A431 |
| 108 | D8F11121595B8F49F78A7039E44126A0 |
| 109 | 1A0BC814445FD71C8E5B1A9163ED2059 |
| 110 | A7591F27F8B0C00C68CC41697954FA04 |
| 111 | 6CA2CE595E7406D79C4840183D41B9D0 |
| 112 | C093D3CC701FC20E66F5AB22516C5460 |
| 113 | D0E0CDE9B595546B96C4F8066B469020 |
| 114 | E99F743A451431C8B427054A4E6F2007 |
| 115 | C0D21A344A2C07DF2A6EBE6250C7B91E |
| 116 | F031223E282CF7A4D8EF174A908668AE |
| 117 | E4BD244AC16C55C7137FB068FD44280C |
| 118 | C44920DE2028F19FC2AAB36A0DCFDAD0 |
| 119 | 3FA7054E77135250699E6C8A11600742 |
| 120 | D5740B4D8870C1C5B5A214C4266FC537 |
| 121 | F0B7942D43BB6F38446442EB8126AB80 |
| 122 | 83DB9534EAD6238FA8968798CDF04848 |
| 123 | EB9663CDDC2B291690703125BABCB800 |
| 124 | 84D547225D4BBD20DEF1A583240C6E0F |
| 125 | B51F6A771838BE934724AEA6A2669802 |
| 126 | D92AC05E10496794BBDC115233B1C068 |
| 127 | D3ACF0078EDA9856BBB0AF8651132103 |

Table C.1.4:	Basic Midamble codes

## C.1.5 	The table of Scrambling codes

| Scrambling code number | Scrambling codes of length 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 | Bit 8 | Bit 10 | Bit 11 | Bit 12 | Bit 13 | Bit 14 | Bit 15 | Bit 16 |
| 0 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 |
| 1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 |
| 2 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 |
| 3 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 |
| 4 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 |
| 5 | -1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 |
| 6 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 |
| 7 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 |
| 8 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 |
| 9 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 |
| 10 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 |
| 11 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 |
| 12 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 |
| 13 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 |
| 14 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 |
| 15 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 |
| 16 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 |
| 17 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 |
| 18 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 |
| 19 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 |
| 20 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 |
| 21 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 |
| 22 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 |
| 23 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 |
| 24 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 |
| 25 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 |
| 26 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 |
| 27 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 |
| 28 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 |
| 29 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 |
| 30 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 |
| 31 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 |
| 32 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 |
| 33 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 |
| 34 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 |
| 35 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 |
| 36 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 |
| 37 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 |
| 38 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 |
| 39 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 |
| 40 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 |
| 41 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 |
| 42 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 |
| 43 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 |
| 44 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 |
| 45 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 |
| 46 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 |
| 47 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 |
| 48 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 |
| 49 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 |
| 50 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 |
| 51 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 |
| 52 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 |
| 53 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 |
| 54 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 |
| 55 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 |
| 56 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 |
| 57 | -1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 |
| 58 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 |
| 59 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 |
| 60 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 |
| 61 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 |
| 62 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 |
| 63 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 |
| 64 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 |
| 65 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 |
| 66 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 |
| 67 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 |
| 68 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 |
| 69 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 |
| 70 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 |
| 71 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 |
| 72 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 |
| 73 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 |
| 74 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 |
| 75 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 |
| 76 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 |
| 77 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 |
| 78 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 |
| 79 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 |
| 80 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 |
| 81 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 |
| 82 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 |
| 83 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 |
| 84 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 |
| 85 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 |
| 86 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 |
| 87 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 |
| 88 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 |
| 89 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 |
| 90 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 |
| 91 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 |
| 92 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 |
| 93 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 |
| 94 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 |
| 95 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 |
| 96 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 |
| 97 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 |
| 98 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 |
| 99 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 |
| 100 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 |
| 101 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 |
| 102 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 |
| 103 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 |
| 104 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 |
| 105 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 |
| 106 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 |
| 107 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 |
| 108 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 |
| 109 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 |
| 110 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 |
| 111 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 |
| 112 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 |
| 113 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 |
| 114 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 |
| 115 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 |
| 116 | -1 | 1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 |
| 117 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 |
| 118 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 |
| 119 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 |
| 120 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 |
| 121 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 |
| 122 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 |
| 123 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 |
| 124 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 |
| 125 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 |
| 126 | 1 | 1 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 |
| 127 | 1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 |

## C.2	Complex form for the synchronisation sequences

As QPSK modulation is used, the ULS, DLS, M sequences are transformed  into a complex form, denoted as the complex vectors respectively: ULS, DLS and M. The elements ULSi, DLSi and Mi are derived using the equations:

Mi = (j)i Mi  	for i=1,….,144

DLSi = (j)i DLSi  	for i=1,….,64

ULSi = (j)i ULSi  	for i=1,….,128

Hence, the elements: DLSi, ULSi, Mi of the complex codes are alternating real and imaginary.

These complex vectors are not subject to channelisation or scrambling process, and are directly or not directly (for midamble sequences values refers to section F.1) used in the pulse shaping process at modulation.

# Annex D (normative): Stand alone Dedicated Channel

This annex specifies the possible configurations for the FACCH and SACCH channels when delivered as SDCCH-T/x (Mode 1 or Mode 2) transport channel. The parameter x can assume the value 3 or 6.

The permitted channel combinations which involve the FACCH and the SACCH are specified in point vi) of sub-clause 6.4.1 and can be mapped onto physical channels as explained in the following clause.

## D.1 Configuration: SDCCH-T/3(0..2)+SDCCH-T/C3(0..2)

This configuration shall be mapped onto 1x24 TDMA frames, with an FACCH and a SACCH period for each user of 120ms. This channel can use one resource unit at spreading factor 16 and a convolutional code at rate ½ (indicated as Mode 1 in [6]) or alternatively two resource units at spreading factor 16 each (or alternatively one resource unit at spreading factor 8) and a convolutional code at rate 1/3 (indicated as Mode 2 in [6]). Rule of multiplexing for this configuration (example for Uplink).

| 1x24 TDMA frames |  | System Frame Number mod 24 |
| --- | --- | --- |
| Transport Channel | Carried Logical Channel |  |
| SDCCH-T/3(0) | FACCH 1st user | 0 |
| SDCCH-T /3(1) | FACCH 2nd user | 1 |
| SDCCH-T /3(2) | FACCH 3rd user | 2 |
| SDCCH-T /3(0) | FACCH 1st user | 3 |
| SDCCH-T /3(1) | FACCH 2nd user | 4 |
| SDCCH-T /3(2) | FACCH 3rd user | 5 |
| SDCCH-T /3(0) | FACCH 1st user | 6 |
| SDCCH-T /3(1) | FACCH 2nd user | 7 |
| SDCCH-T /3(2) | FACCH 3rd user | 8 |
| SDCCH-T /3(0) | FACCH 1st user | 9 |
| SDCCH-T /3(1) | FACCH 2nd user | 10 |
| SDCCH-T /3(2) | FACCH 3rd user | 11 |
| SDCCH-T /3(0) | SACCH 1st user | 12 |
| SDCCH-T /3(1) | SACCH 2nd user | 13 |
| SDCCH-T /3(2) | SACCH 3rd user | 14 |
| SDCCH-T /3(0) | SACCH 1st user | 15 |
| SDCCH-T /3(1) | SACCH 2nd user | 16 |
| SDCCH-T /3(2) | SACCH 3rd user | 17 |
| SDCCH-T /3(0) | SACCH 1st user | 18 |
| SDCCH-T /3(1) | SACCH 2nd user | 19 |
| SDCCH-T /3(2) | SACCH 3rd user | 20 |
| SDCCH-T /3(0) | SACCH 1st user | 21 |
| SDCCH-T /3(1) | SACCH 2nd user | 22 |
| SDCCH-T /3(2) | SACCH 3rd user | 23 |

## D.2 Configuration: SDCCH-T/6(0..5)+SDCCH-T/C6(0..5)

This configuration shall be mapped onto 1x48 TDMA frames, with an FACCH and SACCH period for each user of 240ms. These logical channels can use one resource unit at spreading factor 16 and a convolutional code at rate ½ (indicated as Mode 1 in [6]) or alternatively two resource units at spreading factor 16 each (or alternatively one resource unit at spreading factor 8) and a convolutional code at rate 1/3 (indicated as Mode 2 in [6]). The same coding scheme as for the configuration described in sub-clause D.1 applies.

| 1x48 TDMA frames |  | System Frame Number mod 48 |
| --- | --- | --- |
| Transport Channel | Carried Logical Channel |  |
| SDCCH-T /6(0) | FACCH 1st user | 0 |
| SDCCH-T /6(1) | FACCH 2nd user | 1 |
| SDCCH-T /6(2) | FACCH 3rd user | 2 |
| SDCCH-T /6(3) | FACCH 4th user | 3 |
| SDCCH-T /6(4) | FACCH 5th user | 4 |
| SDCCH-T /6(5) | FACCH 6th user | 5 |
| SDCCH-T /6(0) | FACCH 1st user | 6 |
| SDCCH-T /6(1) | FACCH 2nd user | 7 |
| SDCCH-T /6(2) | FACCH 3rd user | 8 |
| SDCCH-T /6(3) | FACCH 4th user | 9 |
| SDCCH-T /6(4) | FACCH 5th user | 10 |
| SDCCH-T /6(5) | FACCH 6th user | 11 |
| SDCCH-T /6(0) | FACCH 1st user | 12 |
| SDCCH-T /6(1) | FACCH 2nd user | 13 |
| SDCCH-T /6(2) | FACCH 3rd user | 14 |
| SDCCH-T /6(3) | FACCH 4th user | 15 |
| SDCCH-T /6(4) | FACCH 5th user | 16 |
| SDCCH-T /6(5) | FACCH 6th user | 17 |
| SDCCH-T /6(0) | FACCH 1st user | 18 |
| SDCCH-T /6(1) | FACCH 2nd user | 19 |
| SDCCH-T /6(2) | FACCH 3rd user | 20 |
| SDCCH-T /6(3) | FACCH 4th user | 21 |
| SDCCH-T /6(4) | FACCH 5th user | 22 |
| SDCCH-T /6(5) | FACCH 6th user | 23 |
| SDCCH-T /6(0) | SACCH 1st user | 24 |
| SDCCH-T /6(1) | SACCH 2nd user | 25 |
| SDCCH-T /6(2) | SACCH 3rd user | 26 |
| SDCCH-T /6(3) | SACCH 4th user | 27 |
| SDCCH-T /6(4) | SACCH 5th user | 28 |
| SDCCH-T /6(5) | SACCH 6th user | 29 |
| SDCCH-T /6(0) | SACCH 1st user | 30 |
| SDCCH-T /6(1) | SACCH 2nd user | 31 |
| SDCCH-T /6(2) | SACCH 3rd user | 32 |
| SDCCH-T /6(3) | SACCH 4th user | 33 |
| SDCCH-T /6(4) | SACCH 5th user | 34 |
| SDCCH-T /6(5) | SACCH 6th user | 35 |
| SDCCH-T /6(0) | SACCH 1st user | 36 |
| SDCCH-T /6(1) | SACCH 2nd user | 37 |
| SDCCH-T /6(2) | SACCH 3rd user | 38 |
| SDCCH-T /6(3) | SACCH 4th user | 39 |
| SDCCH-T /6(4) | SACCH 5th user | 40 |
| SDCCH-T /6(5) | SACCH 6th user | 41 |
| SDCCH-T /6(0) | SACCH 1st user | 42 |
| SDCCH-T /6(1) | SACCH 2nd user | 43 |
| SDCCH-T /6(2) | SACCH 3rd user | 44 |
| SDCCH-T /6(3) | SACCH 4th user | 45 |
| SDCCH-T /6(4) | SACCH 5th user | 46 |
| SDCCH-T/6(5) | SACCH 6th user | 47 |

# Annex E (normative): 
Spreading code definition

The Spreading Code (SC) determines both the channelisation code (or Walsh code) and the spreading factor of a resource unit. SC field is used by higher layer messages to assign resources (see [5]). At spreading factor 16, up to 16 different channelisation codes can be assigned; at spreading factor 8, 8 channelisation codes are possible; …;at spreading factor 1, 1 channelisation code only can be assigned. This means that in total there are 31 possible spreading code values as described in Table 10. With reference Table 10, the Walsh code sequence is identified by the parameter k which points to the corresponding leaf in the OVSF (Orthogonal Variable Spreading Factor) code tree; the spreading factor is identified by the parameter Q which defines the node of the OVSF code tree, the OVSF code tree is reported below. The spreading code can be also described as notation cQ(k)  where the parameters Q and k  are the ones defined above.

| Spreading code number | Spreading Factor (Q) | Walsh code number (k) | Spreading Code field coding |
| --- | --- | --- | --- |
| 0 | 16 | 1 | 00000 |
| 1 | 16 | 2 | 00001 |
| 2 | 16 | 3 | 00010 |
|  | … |  |  |
| 14 | 16 | 15 | 01110 |
| 15 | 16 | 16 | 01111 |
| 16 | 8 | 1 | 10000 |
| 17 | 8 | 2 | 10001 |
|  | … |  |  |
| 22 | 8 | 7 | 10110 |
| 23 | 8 | 8 | 10111 |
| 24 | 4 | 1 | 11000 |
| 25 | 4 | 2 | 11001 |
| 26 | 4 | 3 | 11010 |
| 27 | 4 | 4 | 11011 |
| 28 | 2 | 1 | 11100 |
| 29 | 2 | 2 | 11101 |
| 30 | 1 | 1 | 11110 |

Table 10:Coding of the SC field

# Annex F (normative): 
Midamble allocation

Midambles are part of the resource unit configuration, which is performed by higher layers.

For the downlink three different midamble allocation schemes are possible:

a)	UE specific midamble allocation: the association between midamble(s) and (sub-set of) code(s) assigned to a UE can be any.

b)	Default midamble allocation: The midamble is allocated depending on the associated spreading code(s), as detailed in sub-clause F.2.

c)	Common midamble allocation: The midamble is allocated per timeslot base such that all resources in one timeslot shall use the same training sequence.

The cell shall broadcast via System Information Type 1 (see TSM 04.08 [5]) whether the default midamble allocation applies.

In uplink direction, any spreading code-midamble mapping rule can be applied in the cell.

In the following sections case b) and c) are addressed.

## F.1 Generation of the midamble sequence

To derive the required training sequence codes (i.e. the shifted versions of the basic midamble sequence), the vector M as defined in Annex C.2, is periodically extended to the size:

imax = L + (K-1) * MO

Where:

L:	Midamble length (144 chips),

K:	Maximum number of supported training sequences in a timeslot,

MO:	Shift step to generate training sequence codes.

The allowed values for MO and K are defined in section F.2.

From this, we obtain a new vector m containing the periodic basic midamble sequence:

m = (m1, m2, …., mimax) = (m1, m2, …., mL+ (K - 1) * MO)

The first P (P is equal to 128 chips) elements of this vector m are the same ones as in vector M, the following elements repeat the beginning:

mi = mi-P for the subset i = (P+1), …, imax

Using this periodic basic midamble sequence m, for each user k a midamble m(k) of length L is derived, which can be written as a user specific vector:

m(k) = (m1(k) m2(k) , …, mL(k))

The L midamble elements mi(k) are generated for each midamble of the k users (k = 1,...,K) based on:

mi(k)  = mi + (K - k) * MO with i = 1, …, L and k = 1, …, K

The midamble sequences derived according to equations above have complex values and are not subject to channelisation or scrambling process, i.e. the elements mi(k)  represent complex chips for usage in the pulse shaping process at modulation.

## F.2 Default midamble allocation

The different training sequences, i.e. midambles, which can be activated in one time slot are shifted versions of a single basic midamble code assigned to the cell. The basic midamble codes available in a TD-SCDMA system are shown in Annex C. Different cells use different periodic basic midamble codes, i.e. different training sequences sets.

The network shall broadcast whether a default midamble allocation applies for the downlink as well as the maximum number  (K; see TSM04.08) of supported training sequences in a timeslot.

The default midamble allocation shall not apply for those downlink channels that are intended for a UE which will be the only UE assigned to a given time slot or slots for the duration of the assigned channel's existence.

The parameter K implicitly defines the size of the shift, i.e. the Midamble Offset (MO), which is used to derive the training sequences from the basic midamble code, being MO equal to 128/K. This parameter also defines the minimum estimation Window (W) for channel estimation, being W equal to min(16,MO). The different training sequences are obtained shifting the basic midamble code by multiples of MO chip length.

If the default midamble allocation is used, the rule to define the association between the training sequences and the spreading codes shall be based on the order of the spreading codes cQ(k) given by k and the order of the midambles m(y) given by y with the constraint that the midamble for a leaf at spreading factor Q is the same as for the upper leaf at spreading factor 2Q.

With reference to the spreading code cQ(k), the parameter Q defines the spreading factor while the parameter k defines the Walsh code number as specified in Annex E.

With reference to the training sequence m(y), the parameter y defines the shift in multiples of the midamble offset, with respect to the basic midamble code such that a given y corresponds to (K-y)*MO chips of shift. Y covers the range [1, K], see section F.1 for a detailed description.

Higher layers shall allocate the channelisation codes in a particular priority order. For each value of K, channelisation codes marked with * shall be allocated prior to the other associated channelisation codes (i.e. this applies in particular for K varying from 14 down to 2). In the case the latter channelisation codes are used, their allocation priority shall match the code number ascending order (in consequence, for such channelisation codes, cQ(k+1) can be allocated to an UE if and only if  cQ(k) too is allocated to the same UE).

The default midamble allocation can also be applied to downlink Common Control Physical Channels (e.g. P-FACH, S-CCPCH allocation).

When Main Channel Sets (see TSM 04.08) are supported by two SF16 spreading codes, one midamble only shall be allocated to them according to the default midamble allocation rules related to K value parameter valid in the time slot.

The default midamble allocation may not apply for those downlink timeslots whose resources are entirely assigned to one user only.

In the case that for a given spreading factor Q the number of supported midambles results to be less than the available channelisation codes, more than one channelisation code will be associated to one training sequence. For example with reference to Figure12 at spreading factor equal to 16 the training sequence m(1) is associated to channelisation codes c16(1) and c16(2). Channelisation codes associated to the same training sequence shall always be assigned to the same UE.

The network can assign more than one channelisation code to a single training sequence even in case different training sequences for each channelisation code are available, with the constraint that the selected training sequence be the one associated to the father leaf of the two or N assigned channelisation codes as reported in the tree structures defined below and that these channelisation codes belong to the same user; father leaf is the leaf of the previous level or x previous levels respectively, in the tree structures defined below, starting from which the assigned channelisation codes branch.

For example with reference to Figure 14 at spreading factor equal to 16 the training sequence m(1) can be associated to channelisation codes c16(1), c16(2) and c16(3) (in this case x corresponds to 2).

The following sub-clauses show the association between training sequences and spreading codes for different values of the midamble offset or, equivalently, of K. Only the values 2, 4, 6, 8, 10, 12, 14 and 16 are considered for K.

### F.2.1	Allocation for K=2 ( or MO=64)

Figure 12: Association of Midambles to Spreading Codes for K=2

For this configuration the minimum estimation window W corresponds to 16 chips.

### F.2.2	Allocation for K=4 (or MO=32)

Figure 13: Association of Midambles to Spreading Codes for K=4

For this configuration the minimum estimation window W corresponds to 16 chips.

### F.2.3	Allocation for K=6 (or MO=21)

Figure 14: Association of Midambles to Spreading Codes for K=6

For this configuration the minimum estimation window W corresponds to 16 chips.

### F.2.4	Allocation for K=8 (or MO=16)

Figure 15: Association of Midambles to Spreading Codes for K=8

For this configuration the minimum estimation window W corresponds to 16 chips.

### F.2.5	Allocation for K=10 (or MO=12)

Figure 16: Association of Midambles to Spreading Codes for K=10

For this configuration the minimum estimation window W corresponds to 12 chips.

### F.2.6	Allocation for K=12 (or MO=10)

Figure 17: Association of Midambles to Spreading Codes for K=12

For this configuration the minimum estimation window W corresponds to 10 chips.

### F.2.7	Allocation for K=14 (or MO=9)

Figure 18: Association of Midambles to Spreading Codes for K=14

For this configuration the minimum estimation window W corresponds to 9 chips.

### F.2.8	Allocation for K=16 (or MO=8)

Figure 19: Association of Midambles to Spreading Codes for K=16

For this configuration the minimum estimation window W corresponds to 8 chips.

## F.3	DL common midamble case

The network shall signal to the UE, when assigning new resources, whether a common midamble shall be used in a specific downlink timeslot.

In case a common midamble is used, the network shall implicitly tell to the UE the maximum number of channelisation codes which can be used in a timeslot by means of the specific training sequence assigned for that timeslot.

The following sub-clauses reports the association between the specific training sequence and the maximum number of supported channelisation codes.

In the following tables the use of a particular midamble shift is indicated by ‘1’.

### F.3.1	Mapping scheme for K=16 Midambles.

| m(1) | m(2) | m(3) | m(4) | m(5) | m(6) | m(7) | m(8) | m(9) | m(10) | m(11) | m(12) | m(13) | m(14) | m(15) | m(16) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 code |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 codes |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3 codes |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 codes |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 5 codes |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 7 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 8 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 9 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 10 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 11 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 12 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 13 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 14 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 15 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 16 codes |

Table 11: Association of Midambles to Spreading Codes for K=16

### F.3.2	Mapping scheme for K=14 Midambles.

| m(1) | m(2) | m(3) | m(4) | m(5) | m(6) | m(7) | m(8) | m(9) | m(10) | m(11) | m(12) | m(13) | m(14) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 code or 15 codes |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 code or 16 codes |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3 codes |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 codes |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 5 codes |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 7 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 8 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 9 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 10 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 11 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 12 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 13 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 14 codes |

Table 12: Association of Midambles to Spreading Codes for K=14

### F.3.3	Mapping scheme for K=12 Midambles.

| m(1) | m(2) | m(3) | m(4) | m(5) | m(6) | m(7) | m(8) | m(9) | m(10) | m(11) | m(12) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 code or 13 codes |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 code or 14 codes |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3 codes or 15 codes |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 codes or 16 codes |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 5 codes |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 6 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 7 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 8 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 9 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 10 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 11 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 12 codes |

Table 13: Association of Midambles to Spreading Codes for K=12

### F.3.4	Mapping scheme for K=10 Midambles.

| m(1) | m(2) | m(3) | m(4) | m(5) | m(6) | m(7) | m(8) | M(9) | m(10) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 code or 11 codes |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 code or 12 codes |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3 codes or 13 codes |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 4 codes or 14 codes |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 5 codes or 15 codes |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 6 codes or 16 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 7 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 8 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 9 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 10 codes |

Table 14: Association of Midambles to Spreading Codes for K=10

### F.3.5	Mapping scheme for K=8 Midambles.

| m(1) | m(2) | m(3) | m(4) | m(5) | m(6) | m(7) | m(8) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 code or 9 codes |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 2 codes or 10 codes |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 3 codes or 11 codes |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 4 codes or 12 codes |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 5 codes or 13 codes |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 6 codes or 14 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 7 codes or 15 codes |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 8 codes or 16 codes |

Table 15: Association of Midambles to Spreading Codes for K=8

### F.3.6	Mapping scheme for K=6 Midambles.

| m(1) | m(2) | m(3) | m(4) | m(5) | m(6) |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 0 | 1 code, 7 codes or 13 codes |
| 0 | 1 | 0 | 0 | 0 | 0 | 2 codes, 8 codes or 14 codes |
| 0 | 0 | 1 | 0 | 0 | 0 | 3 codes, 9 codes or 15 codes |
| 0 | 0 | 0 | 1 | 0 | 0 | 4 codes, 10 codes or 16 codes |
| 0 | 0 | 0 | 0 | 1 | 0 | 5 codes or 11 codes |
| 0 | 0 | 0 | 0 | 0 | 1 | 6 codes or 12 codes |

Table 16: Association of Midambles to Spreading Codes for K=6

### F.3.7	Mapping scheme for K=4 Midambles.

| m(1) | m(2) | m(3) | m(4) |  |
| --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 1 code, 5 codes, 9 codes or 13 codes |
| 0 | 1 | 0 | 0 | 2 codes, 6 codes, 11 codes or 14 codes |
| 0 | 0 | 1 | 0 | 3 codes, 7 codes, 11 codes or 15 codes |
| 0 | 0 | 0 | 1 | 4 codes, 8 codes, 12 codes or 16 codes |

Table 17: Association of Midambles to Spreading Codes for K=4

### F.3.8	Mapping scheme for K=2 Midambles.

| m(1) | m(2) |  |
| --- | --- | --- |
| 1 | 0 | 1 code, 3 codes, 5 codes, 7 codes, 9 codes, 11 codes, 13 codes or 15 codes |
| 0 | 1 | 2 codes, 4 codes, 6 codes, 8 codes, 10 codes, 12 codes, 14 codes or 16 codes |

Table 18: Association of Midambles to Spreading Codes for K=2

# Annex G (normative): Multi-slot capability

## G.1 UE classes for multi-slot capability

When an UE supports the use of multiple timeslots it shall belong to a multi-slot class as defined below:

Table G.1

| Multislot class | Maximum number of slots |  |  | Maximum number of channels per frame (Note) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Rx | Tx |  | Rxc | Txc |
| 1 |  | 1 | 1 |  | 1,2,..,16 | 1,2,..,16 |
| 2 |  | 2 | 2 |  | 1,2,..,32 | 1,2,..,32 |
| 3 |  | 3 | 3 |  | 1,2,..,48 | 1,2,..,48 |
| 4 |  | 1 | 1 |  | 1,2,..,16 | 1,2,..,16 |
| 5 |  | 1 | 2 |  | 1,2,..,16 | 1,2,..,32 |
| 6 |  | 2 | 1 |  | 1,2,..,32 | 1,2,..,16 |
| 7 |  | 1 | 3 |  | 1,2,..,16 | 1,2,..,48 |
| 8 |  | 2 | 2 |  | 1,2,..,32 | 1,2,..,32 |
| 9 |  | 3 | 1 |  | 1,2,..,48 | 1,2,..,16 |
| 10 |  | 1 | 4 |  | 1,2,..,16 | 1,2,..,64 |
| 11 |  | 2 | 3 |  | 1,2,..,32 | 1,2,..,48 |
| 12 |  | 3 | 2 |  | 1,2,..,48 | 1,2,..,32 |
| 13 |  | 4 | 1 |  | 1,2,..,64 | 1,2,..,16 |
| 14 |  | 2 | 4 |  | 1,2,..,32 | 1,2,..,64 |
| 15 |  | 3 | 3 |  | 1,2,..,48 | 1,2,..,48 |
| 16 |  | 4 | 2 |  | 1,2,..,64 | 1,2,..,32 |
| 17 |  | 3 | 4 |  | 1,2,..,48 | 1,2,..,64 |
| 18 |  | 4 | 3 |  | 1,2,..,64 | 1,2,..,48 |
| 19 |  | 1 | 5 |  | 1,2,..,16 | 1,2,..,80 |
| 20 |  | 1 | 6 |  | 1,2,..,16 | 1,2,..,96 |
| 21 |  | 2 | 5 |  | 1,2,..,32 | 1,2,..,80 |
| 22 |  | 5 | 1 |  | 1,2,..,80 | 1,2,..,16 |
| 23 |  | 5 | 2 |  | 1,2,..,80 | 1,2,..,32 |
| 24 |  | 6 | 1 |  | 1,2,..,96 | 1,2,..,16 |

Note: a channel is assumed as a basic resource unit at SF=16.

For UE operating in full duplex mode, indicating multi-slot class 1, 2 or 3, the network shall assign the same number of timeslots per direction; e.g. if the UE indicates multi-slot class 3, the network can assign 3 or 2 or 1 TS per	direction.

Rx:

Rx describes the maximum number of receive timeslots that the UE can use per TDMA frame. The UE must be able to support all integer values of receive TS from 0 to Rx (depending on the services supported by the UE). The receive TS need not be contiguous. The receive TS shall be allocated within window of size Rx.

Tx:

Tx describes the maximum number of transmit timeslots that the UE can use per TDMA frame. The UE must be able to support all integer values of transmit TS from 0 to Tx (depending on the services supported by the UE). The transmit TS need not be contiguous. The transmit TS shall be allocated within window of size Tx.

Rxc/Txc:

Rxc and Txc describe the maximum number of DCH (using RU at SF=16) that can be assigned to the UE in a TDMA frame, in reception and transmission direction respectively.

For UEs indicating multi-slot class from 5 to 24, the network should assign resources in the minimum number of timeslots, according to radio resources already assigned.

# History

| Document history |  |  |  |
| --- | --- | --- | --- |
| DATE | OLD_VERS | NEW_VERS | SUBJECT |
| 25/04/00 | 0.0.1 | 0.0.1 | First submission to CWTS |
| November 2000 | 0.0.1 | 0.1.0 | Approved CRs |
| December 2000 | 0.1.0 | 0.1.1 | Approved CRs |
| December 2000 | 0.1.1 | 0.2.0 | Further editorial changes and Second submission to CWTS |
| December 2000 | 0.2.0 | 1.2.0 | Version after approval of CWTS |
| April 2001 | 1.2.0 | 1.3.0 | Third submission to CWTS |
| July 2001 | 1.3.0 | 2.0.0 | Approved by CWTS meeting #10 |
| December 2001 | 2.0.0 | 2.1.0 | Submitted to CWTS meeting #11 |
| April 2002 | 2.1.0 | 2.2.0 | Submitted to CWTS meeting #12 |
| August 2002 | 2.2.0 | 2.2.1 | Submitted to CWTS meeting #13 |
| August 2002 | 2.2.1 | 3.0.0 | Approved by CWTS meeting #13 |
| December 2002 | 3.0.0 | 3.0.1 | Submitted to CWTS meeting #14 |
| December 2002 | 3.0.1 | 3.1.0 |  |