DHCP handler 1.0

----------------

DHCP.EXE is a utility that uses the DHCP protocol to find out a

client's IP address and some more network parameters. DHCP.EXE can be

used together with an Ethernet card and a packet driver for that card.

It supports the WWW/LX software suite, but can be used with different

software packages too.

Start DHCP.EXE from the command line using

DHCP

or

DHCP 1

if you want debugging output.

If the program succeeds, it terminates with an error code 0 and creates

a batch file named IP-UP.BAT in the current directory. If it does not

succeed, it continues trying until the user hits a key. It will then

terminate with an error code higher than 0.

The created batch file IP-UP.BAT looks as follows:

SET MYIP=15.85.45.140

SET REMIP=15.85.40.1

SET NETMASK=255.255.248.0

SET BROADCAST=255.255.255.255

SET DNS1=15.68.9.10

SET DNS2=15.68.2.1

Some of the lines may be missing in case the DHCP server did not

provide them. If you run that batch file, it will set environment

variables that are used by WWW/LX. These environment variables override

the values specified in WWW.CFG. So what you need to do to get it

running:

- Insert the Ethernet card into your machine

- Launch the packet driver

- Run DHCP.EXE

- Run IP-UP.BAT

- Launch WWW/LX

Of course, you can also put all that into a batch file (well, the first

step is difficult to achieve with a batch file :-)

Important note: DHCP does not need to assign an IP address for an

unlimited time. In fact, it is normal that the IP address is only

provided for a certain, limited time. After that time, a new DHCP

negotiation is expected. Of course, this cannot work because DHCP.EXE

only runs once at the beginning of the session. This is a shortcoming

of the approach.

DHCP.EXE is copyrighted freeware by A. Garzotto

30-Nov-98