PIC16F84 INSTRUCTION SET SUMMARY (Dec 1999)

Literal and control instructions:

| Mnemonic | Description | Function |
| --- | --- | --- |
| addlw k | Add literal to W | k + W ® W |
| andlw k | AND literal and W | k .AND. W ® W |
| call k | Call subroutine | PC + 1 ® TOS, k ® PC |
| clrwdt | Clear watchdog timer | 0 ® WDT (and prescaler if assigned) |
| goto k | Goto address (k is nine bits) | k ® PC (9 bits) |
| iorlw k | Incl. OR literal and W | k .OR. W ® W |
| movlw k | Move Literal to W | k ® W |
| option | Load OPTION register | W ® OPTION Register |
| retfie | Return from Interrupt | TOS ® PC, 1 ® GIE |
| retlw k | Return with literal in W | k ® W, TOS ® PC |
| return | Return from subroutine | TOS ® PC |
| sleep | Go into Standby Mode | 0 ® WDT, stop oscillator |
| sublw k | Subtract W from literal | K - W ® W |
| tris f | Configure port f | W ® I/0 control reg f |
| xorlw k | Exclusive OR literal and W | k .XOR. W ® W |

Byte-oriented instructions:

| Mnemonic | Description | Function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| addwf f,d | Add W and f | W + f ® d |  |  |  |  |  |  |  |  |  |
| andwf f,d | AND W and f | W .AND. f ® d |  |  |  |  |  |  |  |  |  |
| clrf f | Clear f | 0 ® f |  |  |  |  |  |  |  |  |  |
| clrw | Clear W | 0 ® W |  |  |  |  |  |  |  |  |  |
| comf f,d | Complement f | .NOT. f ® d |  |  |  |  |  |  |  |  |  |
| decf f,d | Decrement f | f - ® d |  |  |  |  |  |  |  |  |  |
| decfsz f,d | Decrement f, skip if zero | f - 1 ® d, skip if 0 |  |  |  |  |  |  |  |  |  |
| incf f,d | Increment f | f + 1 ® d |  |  |  |  |  |  |  |  |  |
| incfsz f,d | Increment f, skip if zero | f + 1 ® d, skip if 0 |  |  |  |  |  |  |  |  |  |
| iorwf f,d | Inclusive OR W and f | W .OR. f ® d |  |  |  |  |  |  |  |  |  |
| movf f,d | Move f | f ® d |  |  |  |  |  |  |  |  |  |
| movwf f | Move W to f | W ® f |  |  |  |  |  |  |  |  |  |
| nop | No operation |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | register f |  |  |  |  |  |  |
| rlf f,d | Rotate left f | ¯ | C | ¬ | 7 |  |  |  |  | 0 | ¬ |
|  |  | ® | ® |  |  |  |  |  |  |  |  |
|  |  |  |  |  | register f |  |  |  |  |  |  |
| rrf f,d | Rotate right f | ® | C | ® | 7 |  |  |  |  | 0 | ¯ |
|  |  |  |  |  |  |  |  |  |  | ¬ | ¬ |
| subwf f,d | Subtract W from f | f - W ® d |  |  |  |  |  |  |  |  |  |
| swapf f,d | Swap halves f | f(0:3) « f(4:7) ® d |  |  |  |  |  |  |  |  |  |
| xorwf f,d | Exclusive OR W and f | W .XOR. f ® d |  |  |  |  |  |  |  |  |  |

Bit-oriented instructions:

| Mnemonic | Description | Function |
| --- | --- | --- |
| bcf f,b | Bit clear f | 0 ® f(b) |
| bsf f,b | Bit set f | 1 ® f(b) |
| btfsc f,b | Bit test, skip next instruction if clear | skip if f(b) = 0 |
| btfss f,b | Bit test, skip next instruction if set | skip if f(b) = 1 |

Key:

| Field | Description |  |  |
| --- | --- | --- | --- |
| b | Bit address within an 8-bit file register |  |  |
| d | Destination select; | d = 0 | Store result in W |
|  |  | d = 1 | Store result in file register f. |
|  |  |  | Default is d = 1. |
| f | Register file address (0x00 to 0xFF) |  |  |
| k | Literal field, constant data or label |  |  |
| W | Working register (accumulator) |  |  |