---------------------------------------------------------

|| .PCV (PCVIC SYSTEM SNAPSHOT) FORMAT SPECIFICATIONS  ||

||						       ||

||		       Version 1.0		       ||

||						       ||

||	  as generated & used by PCVIC 1.0 - 1.14      ||

---------------------------------------------------------

The PCVIC system snapshot (.PCV)

A system snapshot stores the complete state of the emulated Vic-20, and can be

used to save and restore any program at any point within its execution.

Offset

$ 0 --- 'PCVIC system snapshot',0

$16 --- Version number (minor, major)

$18 --- Size of register state block

$1a --- Register state block: System register values and memory expansion

setting.

mmm --- Byterun compressed image of the Vic's memory.

The areas where the system ROMs are, $8000..$8FFF and $C000..$FFFF,

are excluded.

nnn --- Two-byte checksum of the data starting from offset $16 but excluding

the checksum itself

-----------------------------------------

$16 --- Version number (minor, major)

-----------------------------------------

0 (BYTE)     MINORVERSION

Value between 0 .. 99.

Examples: V1.12 -> minor version=12,  V1.01 -> minor version=1

1 (BYTE)     MAJORVERSION

Value between 0 .. 255.

----------------------------------------

$18 --- Size of register state block

----------------------------------------

0 (WORD)     RSBSIZE

Size of the register state block, not including the size field itself.

-----------------------------------------------------------------------------

$1a --- Register state block: System register values and memory expansion

setting.

-----------------------------------------------------------------------------

0 (WORD)     X

Low byte = 6502 X register

High byte must be set to 0

2 (WORD)     Y

Low byte = 6502 Y register

High byte must be set to 0

4 (WORD)     S

Low byte = 6502 S register

High byte must be set to 1

6 (BYTE)     OLDSCANCOUNT

Scrap register; don't care

7 (BYTE)     AUXFLAGS

Little-used 6502 flags:

Bit2..Bit5 are Bit2..Bit5 (1,B,D,I) of P register.

The other bits should be set to 0.

8 (WORD)     SCANLINE

ScanLine nr. 0..263(NTSC) or 0..311(PAL)

Values up to 32767 are possible.

10 (4 BYTES) IFRIER

Exact copies of the IFR and IER registers.

(Order is same as in the vic's memory)

10 (BYTE)    IFR1	VIA1: IFR register

11 (BYTE)    IER1	VIA1: IER register

12 (BYTE)    IFR2	VIA2: IFR register

13 (BYTE)    IER2	VIA2: IER register

14 (8 BYTES) PORTDATA

Values as found in the IRA/ORA & IRB/ORB latches.

Order is the same as in memory.

var.name	chip reg.name

14 (BYTE)    PERIP1B	VIA1   IRB

15 (BYTE)    LATCH1B	VIA1   ORB

16 (BYTE)    PERIP1A	VIA1   IRA

17 (BYTE)    LATCH1A	VIA1   ORA

18 (BYTE)    PERIP2B	VIA2   IRB

19 (BYTE)    LATCH2B	VIA2   ORB

20 (BYTE)    PERIP2A	VIA2   IRA

21 (BYTE)    LATCH2A	VIA2   ORA

22 (BYTE)    TIMERSTATUS

Lower 4 bits correspond to status of VIA timers:

bit0  1 = VIA1 Timer1 running

bit1  1 = VIA1 Timer2 running

bit2  1 = VIA2 Timer1 running

bit3  1 = VIA2 Timer2 running

High 4 bits are future expansion: set these to 0.

23 (4 BYTES) TIMERDATA

A copy of the low bytes of the VIAs' latches and timers.

23 (BYTE)    V1T2LATCHL

24 (BYTE)    V2T2LATCHL

25 (BYTE)    V1T2TIMERL

26 (BYTE)    V2T2TIMERL

27 (BYTE)    NMIFLANK

This variable is used to detect NMI flanks.

0     = NMI was low since last time the NMI pin was sampled.

Other = NMI high since last time sampled.

28 (BYTE)    MEMCONFIG

Each bit corresponds to the presence of an 8k block.

Bit=1 => area is RAM

Bit=0 => area is write-protected

(for ROM areas or unused addressing space)

Bit:		  7    6    5	 4    3    2	1    0

8K-Block start: xxxx xxxx A000 xxxx 6000 4000 2000 0000

xxxx means: Unavailable. (the system ROMs and I/O space are situated

here) Always set these bits to 0.

Note that the areas 0000..03ff and 1000..2000 are considered to be

always occupied by RAM. For an unexpanded Vic, all blocks are set to

0. Setting only the low block (bit0) it to 1 results in 3K RAM

expansion.

29 (WORD)    PC

6502 PC register

31 (WORD)    MAINFLAGS

Much-used 6502 flags.

Note that: '.' means: 'don't care'.

'0' means: always set to 0.

Bit: 7654 3210

Low byte  = SZ.. ....

High byte = 0V00 000C

33 (BYTE)    A

6502 A register

34 (BYTE)    SCANCOUNT

CPU cycle count within scan line.

0 = End of scan line

129(NTSC) or 131(PAL) = Start of scan line

35 END

-------------------------------------------------------------------------------

mmm --- Byterun compressed image of the Vic's memory.

The areas where the system ROMs are, $8000..$8FFF and $C000..$FFFF,

are excluded.

-------------------------------------------------------------------------------

Byterun (as is also used in ILBM images) achieves compression by replacing a

row of bytes with equal values by a count and a value byte.

See the C source 'PCVDUMP.C' for the detailed description of how to decompress

byterun-encoded data.

There are two noncontiguous memory blocks in the image, $0000..$7FFF and

$9000..$BFFF, which are compressed separately.

$0000..$7FFF Memory data

This complete area is always included, regardless of the value of

MEMCONFIG; some of it could be occupied by ROMs.

$9000..$93FF I/O Space

Only the areas $9000..$900F and $9110..$912F are defined. These

should contain the values as found when reading from the corresponding

VIC and VIA registers.

The port addresses on a real Vic are shadowed multiple times across

this area, though current versions of PCVIC do not support this. This

means PCVIC will fill the rest of the area with '$FF' bytes. The values

found in these shadow areas are 'don't care'. The remaining values are

undefined.

$9400..$97FF Colour memory

Only the low nybbles are defined.

$9800..$9FFF Free I/O Space

In PCV version 1.0, there is no room in the register state block for

optional special hardware which could have their I/O space here.

However, it is possible to map ROM to this area, and probably RAM as

well.

$A000..$CFFF Memory data

This complete area is always included, regardless of the value of

MEMCONFIG; some of it could be occupied by ROMs.

------------------------------------------------------------------------------

nnn --- Two-byte checksum of the data starting from offset $16 but

(of course) excluding the checksum itself

------------------------------------------------------------------------------

See the C source 'PCVDUMP.C' for the checksum calculation algorithm.