# Chapter 1: Introduction

## 1.3 Design Process

- design process: design specification, library development, design synthesis, design analysis, documentation, manufacturing

- design synthesis – converts a specification into a structure of processor-level components, such as processors, memories, and ASICs

- architecture synthesis – converts algorithms, flowcharts, or instruction sets into register-level components, such as counters, registers, stacks, queues, adders, and multipliers

- sequential synthesis – transforms a finite-state-machine description into gates and flip-flops – minimize the number of memory elements to be used in the circuit

- logic synthesis – transforms Boolean expressions into gate-level components

- equivalence verifiers – prove equivalence of two designs for every set of input values

- timing verifiers – prove constraint satisfaction for every input/output path in the design

- simulation – design is verified only for a given set of input values by generating the set of output values from a structural description and comparing it to the given set of expected output values obtained from the behavioral description

# Chapter 2: Data Types and Representations

## * 2.1 Positional Number System

- positional number system – any number is represented by a string of digits in which the position of each digit has an associated weight

## * 2.2 Octal and Hexadecimal Numbers

- octal number system – use radix 8

- hexadecimal number system – use radix 16

- (2 to 8) : separate into groups of 3 and replace each group with the corresponding octal digit

- (2 to 16) : separate into groups of 4

## * 2.3 Number System Conversions

- (10 to 2) : get remainders from dividing by 2 repeatedly

## * 2.4 Addition and Subtraction of Binary Numbers

## * 2.5 Representation of Negative Numbers

- sign-magnitude system – consists of the positive or negative sign and the value of the magnitude

- 0 is positive and 1 is negative in binary

- …

## * 2.6 Two’s-Complement Addition and Subtraction

- overflow – an operation produces a result that exceeds the range of the number system

## 2.7 Binary Multiplication

## 2.8 Binary Division

## 2.9 Floating-Point Numbers

## * 2.10 Binary Codes for Decimal Numbers

- binary-coded decimal (BCD) – assigns a 4 bit unsigned binary representation to each of the digits 0 through 9, while the code words 1010 through 1111 are not used

- weighted code – the value of each decimal digit is equal to the sum of all the products of the binary values in each bit position and their associated weights (8, 4, 2, 1 for BCD code)

- excess-3 code – self-complementing, generated from BCD code by adding 0011 to each code word

- biquinary code – uses 7 bits

## * 2.11 Character Codes

- ASCII (American Standard Code for Information Interchange) – each character is represented by a 7-bit string

## 2.12 Codes for Error Detection and Correction

## 2.13 Hamming Codes

# Chapter 3: Boolean Algebra and Logic Properties

## * 3.1 Algebraic Properties

- Binary operator – a rule that assigns to each pair of elements in S another unique element that is also included in S

- Closure – a set S is closed with respect to a binary operator iff for every x, y  S, x  y  S

- Associativity – a binary operator  defined on a set S iff for all x, y, z  S, (x  y)  z = x  (y  z)

- Identity Element – a set S with respect to a particular binary operator  whenever there exists an element e  S such that for every x  S, e  x = x  e = x

- Commutativity – a binary operator  defined on a set S iff for all x, y  S, x  y = y  x

- Inverse Element – A set S that has the identity element e with respect to a binary operator  is said to have an inverse iff for every x  S, there exists an element y  S such that  x  y = e

- Distributivity – If  and  are two binary operators on a set S,  is said to be distributive over if for all x, y, z  S,  x  (y  z) = (x  y)(x  z)

## * 3.2 Axiomatic Definition of Boolean Algebra (Switching Algebra)

Axiom 1: (Closure Property)

- B is closed with respect to the operator +

- B is also closed with respect to the operator 

Axiom 2: (Identity Element)

- B has an identity element with respect to , designated by 1

- B also has an identity element with respect to +, designated by 0

Axiom 3: (Commutativity Property)

- B is commutative with respect to +

- B is also commutative with respect to 

Axiom 4: (Distributivity Property)

- The operator  is distributive over +

- The operator + is distributive over 

Axiom 5: (Complement Element)

For every x  B, there exists an element x’  B such that

a) x + x’ = 1 and   b) x  x’ = 0

This second element, x’, is called the complement of x

Axiom 6: (Cardinality Bound)

There are at least two elements x, y  B such that x  y

## * 3.3 Basic Theorems of Boolean Algebra

Theorem 1 (Idempotency)

- x + x = x

- xx = x

#### Theorem 2

- x + 1 = 1

- x  0 = 0

Theorem 3 (absorption)

- yx + x = x

- (y + x)x = x

Theorem 4 (Involution)

- (x’)’ = x

Theorem 5 (Associativity)

- (x + y) + z = x + (y + z)

- x(yz) = (xy)z

Theorem 6 (DeMorgan’s Law)

- (x + y)’ = x’y’

- (xy)’ = x’ + y’

- Duality principle – one of each pair of axioms can be obtained from the other by using an important property of Boolean algebra – any algebraic equality derived from these axioms will still be valid whenever the OR and AND operators, and identity elements 0 and 1, have been interchanged

## * 3.4 Boolean Functions

## * 3.5 Canonical Forms

- minterm – 1 in only one row of its truth table and 0 in all other rows

- any Boolean function can be expressed as a sum of its 1-minterms

- maxterm – a Boolean function that is equal to 0 in only one row of its truth table and 1 in all other rows

- any Boolean function can be expressed as a product of its 0-maxterms

- canonical form – any Boolean function that is expressed as a sum of minterms or as a product of maxterms

- to convert from one canonical form to another, interchange the symbols  and , and list the numbers that were excluded from the original form

## * 3.6 Standard Forms

- standard form – each term may require fewer than n-1 operations

- sum of products – a Boolean expression that contains AND terms (product terms / implicants)

- product-of-sums – an AND expression that contains OR terms (sum terms / implicants)

- essential prime implicant – a 1-minterm is included in only one prime implicant – cannot be omitted from any standard form without changing the given function

## * 3.7 Other Logic Operations

AND – x  y – xy

XOR – x  y – xy’ + x’y

OR – x + y – x + y

NOR – x  y – (x + y)’

NAND – x  y – (xy)’

## * 3.8 Digital Logic Gates

- logic networks (logic circuits) – with variables in the expression as inputs to this logic network, which contains one or more logic gates

- logic gate – performs one or more Boolean operations

- gate library – collection of standard logic gates that we use in constructing logic networks

## * 3.9 Extension to Multiple Inputs and Multiple Operators

- to lower cost and delay of an implementation

- expand the library by constructing gates that have more than two inputs

- extend the library to include gates that can each perform more than one Boolean operations (gates combinations, e.g. AND-OR-INVERTER) – complex gates with NANDs or NORs

- any one of the standard gates, except for inverter and driver, could be extended to have more than two inputs due to commutative and associative

## * 3.10 Gate Implementations

- gate implementations – electronic circuits whose behavior can be explained very well in terms of voltage levels measured in volts (High and Low) but not necessarily in terms of Boolean values 0 and 1

- positive logic system – 0 is assigned to L and 1 is assigned to H

- negative logic system – L represents 1 and H represents 0, use small triangles as a polarity indicator for negative logic

- gate circuits are constructed to sustain variations in input and output voltage levels

- noise – undesirable voltage variations that are superimposed on the normal operating voltage levels

- noise margin – maximum noise voltage level that is tolerated by a gate

- standard load (fan-out) – the number of gates that each gate can drive, while providing voltage levels in the guaranteed range – depends on the amount of electric current a gate can source or sink while driving other gates

- power dissipation – a product of average current and power supple voltage

Pavg = VCC * (ICCH + ICCL)/2

- power dissipation increases with the frequency with which the gate output is changing – defines the battery life (the larger the power dissipation, the shorter the battery life); proportional to heat generated by the microchip or product

- propagation delay = (tPHL + tPLH)/2

- bipolar transistor – can be modeled as a voltage-controlled switch with three terminals: emitter, collector and base – each gate is constructed from resistors, diodes, and transistors

- resistor-transistor logic (RTL) – one resistor and one transistor were used for every gate input

- diode-transistor logic (DTL) – RTL with input resistors in gate circuits replaced by diodes – shorter rise and fall times but required more components

- transistor-transistor logic (TTL) – DTL with input diodes replaced with multiemitter transistors

- pull-up/pull-down transistors – alternate states in a lock-step manner in TTL

- open collector (OC) – omitting the pull-up transistor and its associated resistance

- wired-AND gate – no physical gate is used, but obtaining an AND function by tieing together OC gates.  F= (wx)’(yz)’ = (wx+yz)’

- metal-oxide-semiconductor (MOS) logic – reduced power consumption – built from a layer of metal above a layer of semiconductor with a layer of silicon dioxide serving as an insulation between them

- p-transistor – closed when the gate voltage is high and open when the gate voltage is low

- n-transistor – closed when the gate voltage is low and open when the gate voltage is high

- complementary MOS (CMOS) logic – low power dissipation (no current in a steady state), high fan-out value (only a small amount of current is needed during the transition), and simplicity in fabrication – requires fewest number of transistors per gate, brings highest density

- gate terminal is well insulated from the source and drain – source current only during transition from H to L and sink it during the transition from L to H

## * 3.11 VLSI Technology

- small-scale-integration (SSI) – contain up to 10 independent gates in a single packages – inputs and outputs of gates connected directly to pins

- medium-scale integration (MSI) – 10-100 gates in a single package – contain several bit slices of standard digital storage or arithmetic components

- large-scale integration (LSI) – 100 to several thousand gates in a single package – controllers, data paths, memory chips, and programmable logic components

- very-large-scale integration (VLSI) – hundreds of thousands of gates within a single package – microprocessors, microcomputer chips, system subfunctions

- the hardware cost of a VLSI chip is measured by its silicon area instead of the total number of ICs on a PC board

- custom approach – designed by hand, so that each transistor and each wire is individually defined by a set of polygons which are then used to generate the photographic masks needed in the fabrication process

- semicustom approach – use gate array (incorporates a large number of identical gates that are laid out in a regular two-dimensional array)

- field-programmable approach (FPGA) – arrays of logic components whose connectivity can be established simply by loading the appropriate configuration data into the device’s internal memory

- used for prototyping – offer a cost-effective solution for small-volume production

- low density and speed

- switching matrix (SM) – interconnecting a set of horizontal and vertical wires

# Chapter 4: Simplification of Boolean Functions

## * 4.1 Map Representation

- prime implicant (PI) – a subcube that is not contained within any other subcube

- essential prime implicant (EPI) – subcube that includes a 1-minterm that is not included in any other subcube

## * 4.2 Map Method of Simplication

- Map Generation from truth table

- Prime Implicant Generation

- Essential Implicant Selection

- Create Minimal Cover

- Standard Form

## * 4.3 Don’t Care Conditions

- completely specified – a function has a value of 0 or 1 for every combination of variable values

- don’t care minterms (d-minterms) or don’t care conditions - incompletely specified

## * 4.4 Tabulation Method

- tabulation method – does not rely on pattern recognition

- find all the prime implicants of a given Boolean function - compare each minterm in the canonical form with all its other minterms that differ in the value of only one variable

- identify the essential implicants and then use them to find all minimal covers

## * 4.5 Technology Mapping For Gate Arrays

Rule 1: xy = ((xy)’)’

Rule 2: x + y = ((x + y)’)’ = (x’y’)’

Rule 3: xy = ((xy)’)’ = (x’ + y’)’

Rule 4: x + y = ((x + y)’)’

- term decomposition – each n-input AND(OR) gate is decomposed into a tree of m-input AND(OR) gates

- retiming – conducted after the decomposition, conversion, and inverter elimination tasks, when the various delays through the schematic can be calculated accurately – assign paths that have the shortest delay through the tree networks to the signals with the longest delay through the entire logic network

## * 4.6 Technology Mapping for Custom Libraries

- group the gates in the original schematic in such a way that each group will represent one of the library gates, while at the same time, the mapped schematic will have the minimal delay or implementation cost

- to minimize delays, input signals should pass through as few gates as possible - only two levels of gates

- to minimize cost, as the implementation schematic should have as few gates as possible, and each gate should have as few inputs as possible

- critical path – reduce the delay on the longest input/output path

## 4.7 Hazard-Free Design

- hazard – one condition that may make a circuit malfunction - generate short unwanted glitches that appear at the output because of different propagation delays on different but converging paths through the circuit

- glitch – short-duration change in the output value when no output change is expected

- static 1-hazard – there are two 1-minterms that differ in only one variable but are not covered by a common product term in a sum-of-products implementation - two different product terms are used to cover two adjacent 1-minterms.

- static 0-hazard – there are two 0-minterms not covered by a common sum term in a product-of-sums implementation

- include an additional prime implicant covering both adjacent 1-minterms in the sum-of-products form to eliminate a hazard - prevent the occurrence of a glitch

- dynamic hazard - two signals that always have the same value become different for a short period of time - occurs when the same variable value propagates through the design on two different paths with different delays - causes a glitch on the output after transition from 0 to 1 or 1 to 0

- eliminate the static hazard by introducing a redundant prime implicant

- delay the change on d until t2 by inserting another gate with a delay of 2.4 ns or more on d between the OR gate and the output AND gate - the glitch has already disappeared when the delay occurs

# Chapter 5: Combinatorial Components

## * 5.1 Ripple-Carry Adders

- ripple-carry adder - any change in c0, x0, or y0 has to propagate or ripple through all the FAs - delay in the carry chain

## * 5.2 Carry-Look-Ahead Adders

- major part of the expressions for each carry in the chain can be precomputed

- carry-look-ahead (CLA) generator - can be used to replace the 4-bit carry chain that computes ci+1, ci+2, ci+3, and ci+4, thereby making the adder faster

…

## * 5.3 Adder/Subtractors

- Addition: A + B

- Subtraction: A + B’ + 1

## * 5.4 Logic Unit

- logic unit - enables perform one of several functions on any two operands

## * 5.5 Arithmetic-Logic Unit

- arithmetic-logic unit (ALU) - performs the basic arithmetic and logic operations in a microprocessor

- arithmetic extender (AE) - modifying logic used for arithmetic operations

- logic extender (LE) - modifying logic used for logic operations

- control variable determines whether the ALU to perform arithmetic operations or logic operations

- functional table – describe operations

## * 5.6 Decoders

- decoders (demultiplexers) - activate or enable only one of n subcomponents

- to activate a particular subcomponent at any given time, a binary address A is decoded into n enable lines, out of which only one line is equal to 1

- each output of each decoder on a particular level will enable one of the decoders on the next level, so that the number of decoders on each successive level will double with its distance from the top decoder

## * 5.7 Selectors

- selector (multiplexer/MUX) – combinatorial component that can select one of several data sources to be used as operands for an ALU – to be stored in a memory or to be sent over a bus

- on every level beyond the first, each selector is used to select one of the two outputs produced by selectors at the preceding level

- the number and size of the gates in the decoder, as well as the size of the output OR gate, would have to increase with the number of inputs, so that the larger gates have to be implemented as trees of gates, which significantly increases the cost and delay of the selector

## 5.8 Buses

- bus - can be easily routed around the other objects on a microchip or a PCB - equivalent in function as a selector

- tristate driver - output provides three different values, 0, 1, and Z - has a data line (D), an enable line (E), and an output line (Y)

- E = 1, then Y = D; E = 0, then Y = Z

## * 5.9 Priority Encoders

- priority encoders – complement of a decoder – to encode the position of the most significant input with a value of 1 and to disregard the values of all the inputs that are in less significant positions

## * 5.10 Magnitude Comparators

- comparators starts its comparison with the least significant bits x0 and y0, computing the Gi and Li for each suffix of X and Y, where the ith suffixes of X and Y are defined as the integers xixi-1…x0 and yiyi-1…y0 such that i <= n-1

- can be used in serial and parallel implementations of n-bit comparators that can compare any two unsigned integers X and Y

## * 5.11 Shifters and Rotators

- implemented with selectors

- used for packing and unpacking digits and characters, for field extraction and insertion, and for floating-point arithmetic operations

- shifter is capable of shifting or rotating one bit position to the left of right.  Thus, requiring a shift of more than one position, we have to pass the data through this shifter several times

- barrel shifter – designed to shift any number of positions at one time

## * 5.12 Read-Only Memories

- read-only memory (ROM) – universal logic element that is able to implement concurrently several different Boolean functions that are defined on the same set of variables - use FAs

- programmable ROMs (PROMs) – manufactured in large quantities and then personalized later in the field (fusing input line and gate) – easy to upgrade or replace – special case of the FPGAs

- high bit density in comparison to writable memories; nonvolatile – fixed data storage – very commonly used in the control units of standard processors and application-specific coprocessors to generate values of control signals

## * 5.13 Programmable Logic Arrays

- programmable logic arrays (PLAs) – use AND array (programmable decoder) as oppose to ROM – driven by arrangements of gates

- cost = (# of inputs + # of outputs) * (# of product terms)

- ROM – When AND plane is fixed, and OR plane is programmable

- PAL (Programmable Array of Logic) – When AND plane is programmable, and OR plane is fixed

- PLA – When AND & OR planes are programmable – best at source level

# Chapter 6: Sequential Logic

- asynchronous sequential circuits – change their state and output values whenever a change in input values occurs – any change in input signals a new input

- synchronous sequential circuits – change their state and output values at fixed points of time, which are specified by the rising or falling edge of a free-running clock signal – input levels are interpreted at specific clock events

- clock period – time between successive transitions in the same direction (re clock frequency)

- clock width – time during which the value of the clock signal is equal to 1

- duty cycle – ratio of clock width and clock period

- active high – state changes occur at the clock’s rising edge or during the clock width

## 6.1 SR Latch

- two input signals (S, R Q, Q’)

## 6.2 Gated SR Latch

## 6.3 Gated D Latch

## 6.4 Flip-Flops

- flip-flop – a device which is in one of two states

- level-sensitive latches – latches that are enabled whenever the control signal is at level 1

- Master-slave flip-flops

- Edge-triggered flip-flops – implemented with three interconnected SR latches

## * 6.5 Flip-Flop Types

- flip-flops : 1) change state only on the positive or negative edge of the clock signal  2)its data inputs must not change after time tsetup before and until time thold after the triggering edge of the clock signal

- asynchronous inputs – used to preset and clear the flip-flops independently of other flip-flop inputs (do not depend on the clock signal and therefore have precedence over all other synchronous operations

## * 6.6 Analysis of Sequential Logic

- analysis – requires the designer to generate one or more functional descriptions, using state diagrams, state and output tables, and input and output Boolean equations

- excitation equations – represent the inputs to the flip-flops – determine the state of the sequential circuit in the next clock cycle

- next-state equations – convert excitation equations into flip-flop characteristic equations

- state-based (Moore-type sequential circuit) – sequential circuit, in which the output values depend solely on its present state – truth table with constant output

- input-based (Mealy-type sequential circuit) – output values are dependent on the input values as well as the state of the circuit – full truth table specified

## * 6.7 Finite-State-Machine Model

…

## * 6.8 Synthesis of Sequential Logic

- Design description or timing diagram

- Develop state diagram

- Generate next-state and output tables

- Minimize states

- Encode inputs, states, outputs

- Derive next-state and output equations

- Choose memory elements

- Derive excitation equations

- Optimize logic implementation

- Derive logic schematic and timing diagrams

- Simulate logic schematic

- Verify functionality and timing

## 6.9 FSM Model Capture

## 6.10 State Minimization

## * 6.11 State Encoding

- there are generally more than n! encodings for n different states, since we could use more than log2n bits for the encoding of n states

- minimum-bit-change strategy – assign Boolean values to the states in such a way that the total number of bit changes for all state transitions is minimized

- prioritized-adjacency strategy – assigning adjacent encodings, which differ in one bit only, to all states that have a common source, destination, or output

- hot-one encoding – uses redundant encoding in which one flip-flop is assigned to each state – each state is distinguishable by its own flip-flop having a value of 1 while all the others have a value of 0 – works only for an FSM with a small number of empty states

## * 6.12 Choice of Memory Elements

- T flip-flops: counter-type sequential circuits (flip from 0 to 1 and back from 1 to 0 with great frequency

- D flip-flops: in applications where input information must be stored for some time and then used later (temporary storage or information scratch pads)

- SR flip-flops: different signals set and reset the flip-flops

- JK flip-flops – combine the behaviour of a T and an SR flip-flop

- T & D flip-flops are better suited for VLSI implementations because of fewer connections

## * 6.13 Optimization and Timing

- double inverters are used at the input to deliver more current, which decreases the delay caused by the charging and discharging of wire capacitances

# Chapter 7: Storage Components

## * 7.1 Registers

- registers – bitwise extension of a flip-flop (consists of n flip-flops driven by a common clock signal)

- parallel-load register – using a control signal (Load/Enable), which allows loading of data into a register

- Load signal controls the selector

## * 7.2 Shift Registers

- shift register (serial-in/parallel-out shift register) – use a selector to enable us to shift data stored in the register – shifts its contents one bit in the specified direction when the control signal Shift is equal to 1

- used to convert computer date for serial communications and assemble the serially transmitted data for processing inside a processor

## * 7.3 Counters

- counter – special type of register that incorporates an incrementer, which allows it to count upward or downward

- presetable counter – when a different value is need as the starting count other than 0 – use a NOR gate to detect when the count of 0 has been reached

- E signal enable counting in the direction specified by the D signal, and Load signal load a new input and disable counting whenever its value is equal to 1.  Behave like up/down counter otherwise

## * 7.4 BCD Counter

- BCD counter - use presetable counter

- Detect when the counter reaches a count of 9 and load 0 instead of 10 in the next clock cycle (by using AND gate)

## 7.5 Asynchronous Counter

## * 7.6 Register Files

- register files – 2n rows of m flip-flops (registers) – consists of a two-dimensional array of register-file cells (RFCs), read and write decoders, and an output-driving logic

- regularity – require less wiring in the final design

- write decoder would select one of the rows for storing the input values at the rising edge of the clock signal

- inputs to write the decoder consist of n address lines

- write enable signal WE (=0, not stored)

- allow only limited access to registers in file

- register files are very fast because every cell includes one complete latch or flip-flop; expensive, since these latches or flip-flops require half a dozen transistors in their design

- used to provide limited amounts of temporary storage in high-speed applications and microprocessors

## * 7.7 Random-Access Memories

- random-access memories – larger and slower, but well suited to serve as long-term storage for the program and the data used during computation

- read and write using the same IO ports depending on 0 or 1 (reduce the number of pins on the memory package)

- number of pins on the memory package determines the size of the package and therefore the amount of area on the printed-circuit board occupied by memory chips

- Row select =1, bit stored in the latch will be the output

- Write enable =1, value of Input will be stored in the latch – signal serves as a clock signal for the latch

- Static RAM (SRAM) – construct memory cell with four to six transistors, using cross-coupled inverters to serve as a latch, and implementing the input AND gate and output drivers with one transistor each – retain its content indefinitely as long as it is not rewritten and power is not disconnected

- Dynamic RAM (DRAM) – memory cells are implemented with only one transistor – content of cell is lost during each read operation, disappears permanently after a time – must be rewritten or refreshed periodically

- Volatile memories – content is lost if the power source is removed (SRAM/DRAM)

- Nonvolatile memories – content is preserved even if power supply is cut off (ROM/PROM)

- Memory access time – delay time to obtain data from memory (t2 – t0)

- Output-enable time – delay in enabling the output drivers (t2 – t1)

- Output-hold time – valid data will be available (t5 – t3)

- Output-disable time – delay in the disabling of output data (t5 – t4)

- Address setup time – t1 – t0 (since delay from address to output is longer than the delay from Chip-Select or RWS, the address lines are set earlier)

- …

## 7.8 Push-Down Stacks

## 7.9 First-in-First-out Queue

## *7.10 Simple Datapaths

- used in all standard processor and ASIC implementations to perform complex numerical computation or data manipulations & consists of some temporary storage

- variable values and constant are stored in registers or memories, they are fetched from storage components after the rising edge of the clock signal, they are transformed in combinatorial components during the time between two rising edges of the clock, and the results are stored back into the storage components at the next rising edge of the clock signal

## *7.11 General Datapaths

## *7.12 Control Unit Design

- Application-specific integrated circuit (ASIC) can perform a single task very well, but it lacks the generality and programmability that characterizes a processor

## 9.1 Instruction Sets

- operation code - specifies the operation to be performed

- address field - specifies where we can find the operands needed for that operation

Add A, B, C

Mem[A]  Mem[B] + Mem[C]

RF[A]  RF[B] + RF[C]

- register instructions - operate on values stored in registers

- move instructions - move data between the memory and the registers

- branch instructions - select one of the two possible next instructions to be executed on the basis of the datapath status

- load instructions - load data stored in the memory at location A, into the register RF[2]

Load R2, A 	(RF[2]  RF[2])

- store instructions – store the value in register RF[2] into the memory at location A

Store A, R2 	(Mem[A]  RF[2])

- branch instruction – compares the values in two registers and decides what to do next on the basis of this comparison

Beq R2, R3, A

- mode field – specify how we are to derive the data’s effective address from the information given in the address field  (e.g. contain indirect address)

- constant field – contains the value of a constant that is to be used as an operand

Add R2, R3, 1	(RF[2]  RF[3] + 1)

- the more address fields an instruction has, the longer the instruction width but the shorter the program will be for computing a given task, since it will contain fewer instructions – processor makes more memory accesses per instruction for fetching instructions and operands, but no difference by the end from shorter instructions

- three-address instruction – contain the location of both the operands and the result in addition to specifying the binary operation to be performed – very convenient but long

- two-address instruction – assume that the first operand and the result share the same memory location

- accumulator (ACC) – store the result of each operation as well as containing one of the operands

- strategy : increase the number of registers in the datapath by introducing a register file

- register file comprises several registers  need additional address fields for registers

- only the load and store instructions need to access the memory, while the arithmetic instructions will only access the registers , which have a shorter address field  can use three-address instructions for those operations whose operands and result are in the registers, and use two-address instructions for loading the data from the memory to register file and storing it back to the memory

- yields shorter instructions and a lower frequency of memory accesses

## 9.2 Addressing Modes

- implied addressing mode – an instruction does not need an explicit addressing field because the location of the operand or the result is specified within the opcode – does not need an address field because registers are unique (always on the top of the stack)

- immediate addressing mode – operand that is specified in the address field – has an operand field instead of an address field – used to specify constants (incrementing or decrementing loop)

- direct addressing mode – address field specifies the location of an operand or the result, which may be located in the memory or in the register file

- indirect addressing mode – address field specifies the location of the address of an operand or the result  need to access the memory twice in memory-indirect mode

- to use a register-indirect mode instruction, the programmer needs to ensure that the memory address is placed in the proper register before it is accessed  requires fewer bits (and faster) than is requested by memory

- relative addressing mode – the content of the address field (the offset) is added to the content of a specified register (i.e. branch)  shorter address field in the instruction format, since the offset usually requires fewer bits than would the entire memory address

- indexed addressing mode – used when need to access data that is stored in arrays, stacks, and queues.

- Address field of the instruction specifies a starting address (base), while the index of a particular data is specified in a dedicated index register or in one of the registers in the register file

- Reduces the number of bits needed in the address field, since the base addresses are usually multiples of 2n which contain (n-1) 0’s.

- Compare to relative addressing : two values contained in the address field and specific register are to be added to obtain the effective address

- Base is in a dedicated register and the offset is located in the address field for relative addressing

- Base is in the address field and the index is located in a dedicated register for indexed addressing

## 9.3 Processor Design

- instruction-execution cycle:

- processor fetches an instruction from the memory and loads it into an instruction register (IR), which stores the instruction while it is being executed in the processor’s datapath

- increments the content of the program counter (PC), which stores the address of the next instruction to be executed

- processor consider mode and address fields in the instruction

- determine the effective address of all the operands and result involved in the computation

- processor fetch operands from the memory and store them temporarily in a register file or in some other register in the datapath

- processor execute the operation that is specified in the opcode

- processor stores the result of this operation in the memory, or stores it temporarily in the register file for use by subsequent instructions

- Design process of processor:

- Design instruction set – by specifying the operations taken by each instruction

- Develop instruction-set (IS) flowchart – describes precisely all the operations that are performed in each instruction

- Allocate datapath components	may need to repeat 1-3

- Derive ASM chart – divides each instruction into clock cycles and specifies all the register transfers that take place in each clock cycle

- Design datapath

- Design control unit

## 9.4 Instruction Set Design

…

## 9.5 CISC Design

…

## 9.6 Reduced Instruction Set

- to increase performance : 1) Faster technology to shorten clock cycle  2) redesigned instruction set to pipeline instruction cycle

- when instructions are executed concurrently in different stages of the instruction-execution cycle pipeline, increase in performance from this pipeline, assuming that one instruction can be started in every clock cycle  when one instruction needs data generated by the previous instruction, we must delay the start of this instruction by three clock cycles until the required data are stored in the memory (pipeline stall)  need reordering

- …

## 9.7 RISC Design

- datapath and control are piprlined:

- Separation of instruction and data memories – processor must be able to fetch one instruction each clock cycle – allow concurrent instruction fetch and data fetch or store

- Addition of a control register in each state, instead of a single control unit – control unit is reduced to a series of control registers and small decode logic in the second stage

- Pipeline flushing for the control instructions (pipeline stalling)

…

## 9.8 Data Forwarding

- data dependence – occurs whenever one instruction generates a value that is used by one or more successive instructions – prevents processor from producing the correct results, since the proper data would not be available by the time it is needed

- no-op instructions inserted by the compiler during compile time or the control unit can initialize necessary stalls during instruction execution after detecting a data dependence between two instructions

- data forwarding – allowing data to skip one or more pipeline stages and “jump” ahead into a future stage – ensure that the results generated by one instruction can be used by the next instruction – keeps the pipeline working at full speed, by eliminating the need for the insertion of No-op instructions that delay data-dependent instructions

## 9.9 Branch Prediction

- branch prediction – allows the processor to execute instructions that follow the branch instruction as it would if the branch – if and when the processor finds that the branch should have been taken, the instructions that were started in this speculative manner will simply be abandoned and the pipeline is flushed