MANUAL FOR “FUNPROM” V2.27 PROGRAMMER/EDITOR/INTERNET UPDATE.

GENERAL INSTALLATION.

Connect the programmer to your printer port LPT1. (Note: - this maybe unstable on some Laptop’s as some do not supply enough power to the port, if you have problems with this, use your main PC)
Use a normal printer cable.
Insert the card into the programmer with the chips on the FunCard pointing down. If you are going to use the Seca (Mediaguard) digital files, put some insulation tape on the 6 pad side of the funcard, this is to stop a short circuit, no harm will be done it just wont work.
You might need to remove the corners on the FunCard that were made during the manufacturing, so that the card goes all the way into the programmer and the receiver.

Down load Funprom  “funprom 2.27b” from

http://www.funcard.net/whocares/index2.html

http://zap.to/irh

http://www.tsb.cx/

http://saqqz.webprovider.com/engindex1.htm

The latest version is funprom 2.27b that goes with this manual.

Next, if required, download the funsoft “fun100” for D2mac from

http://www.multimania.com/poirier/

http://www.funcard.net/whocares/index2.html

http://www.tsb.cx/

http://saqqz.webprovider.com/engindex1.htm

Next, if required, download “funseca_b1” for Seca (Mediguard) digital, from

http://www.funcard.net/whocares/index2.html

Both files (funprom and d2mac) can be down loaded at any other good sat sites as well, please remember that the twinpics site will not have the current keys in the ee_ext.hex file. But other sites above should have up to date keys in the ee_ext.hex; these do NOT have management keys in. For Seca (Mediaguard) at the moment only http://www.funcard.net/whocares/index2.html has the files you require, this of course will change as the files get spread across the net.

Ok, now you have downloaded the files, you now have to unzip them into your chosen directory or directories, the funprom-unzipped file should look similar to this: -

The Funsoft “fun100” for D2mac unzipped file should look similar to this: -

The soft “Funseca_b1” for Seca (Mediaguard) unzipped file should look similar to this: -

READ ALL DOCUMENTATION ON THE ABOVE FILES PLEASE, this will save a lot of questions later.

Now run the file for the funprom (fp2.27.exe), this will unpack the files into a directory of c:\windows\temp, change to this directory and run set-up. This will make a directory and install the files in

C:\Program Files\WhoCares\FunProm.

We would suggested you use the file manager to create a sub directory called Seca if your going to use d2mac and seca files, this is so you can not get confused on which file to use, like so: -

C:\Program Files\WhoCares\FunProm\Seca keys.

Copy the d2mac files ee_ext.hex, ee_int.hex and flash.hex from the funsoft directory into C:\Program Files\WhoCares\FunProm. And the Seca files into C:\Program Files\WhoCares\FunProm\Seca keys.

Ok, now run funprom by clicking on start>programes>funprom>funprom or drag the funprom icon to your desktop and double click on the icon and you should see: -

Pressing the Seca tab you should see something like this.

Press the programmer tab and you should see something like this: -

Press the download tab and you should see something like this: - NOTE, THE LIVE KEYS UPDATE IS NOT AVAILBLE FOR THE PRESENT TIME, WHEN IT COMES BACK THE SCREEN SHOULD LOOK SOMETHING LIKE BELOW AND ONLY FOR D2MAC.

PROGRAMING. D2mac.

Click on Flash file, a pop up window appears select flash.hex

Click on the Int eeprom, a pop up window appears, select ee_int.hex

Click on Ext eeprom, a pop up window appears, select ee_ext.hex

Note: - a pop up window may appear, and asks you if you want to upgrade your old ee_ext.hex file, click yes, this upgrades the internal ext_ee.hex structure to be compatible with the latest release of the funsoft. It also saves you time in redoing all of your keys again;), now, this is important. Save the file! It’s only in the editor’s memory, so save first then program.

Now, make sure your card is in the programmer with the chip facing downwards, press the programme button and away you go, wait until a message appears at the bottom left hand side of the funprom say’s “Device programmed”, remove card and place into the decoder, enjoy.

Note the “verify” function is disabled, because the verify function is done on the fly as bytes are written to the card.

Some additional information after you have programmed the card, if your decoder doesn’t auto switch or you have difficulty in knowing which mode you are in, the following may be useful, to be done through your remote control handset.

7799 switch between ec/s2 and ec/m then reset.

6699 switch between ec/s2 and ec/m then manually reset the card (pull out and re insert the card).

5599 activate the on channel switch between ec/s2 and ec/m

4499 de-activate the on channel switch between ec/s2 and ec/m

Red LED, wrong key

Orange LED on ec/m mode, off ec/s2 mode

Blinking Green LED (next to the Orange LED) command being processed.

For a more detailed explanation read Twinpics Documents that comes with Funsoft this would answer 90% of your questions.

Programming SECA

This is exactly the same procedure as the D2MAC section, except you use only the Seca flash.hex and Seca ee-ext.hex files that are in C:\Program Files\WhoCares\FunProm\Seca keys.  NO ee_int.hex is used at the moment.

TO EDIT THE KEYS D2MAC

Run Funprom, click on start>programes>funprom>funprom or double click on the icon on your desktop.

You will see this.

Move the cursor over File, left click and select Load hex file, left click and a window pops up then select the ee_ext.hex file.

After loading you should see something like below.

Note: - the TV3 keys and SA were valid at the time of doing this, later on these keys will be out of date but the locations remain the same.

We will now edit say the C+ 00 2B 10 channel, move your cursor to C+ 00 2B 10 and left single click, this is now highlighted, to the right hand side you will see all key positions and the SA position, if you don’t have the SA for this you will have to populate keys 00 to 03 only, if you have the MK/SA you can populate the required fields in C+ update 00 2B B0 remember to have the same SA in both indents, you should see something like this (No MK/SA shown)

To edit say key 00 move your cursor to the right hand side pane and left double click the white portion next to key 00 this what you should see, this overlaps the main funprom window but can be moved around the screen.

Now just type over the keys displayed with your own hex key, it automatically moves to the next cell, at the end press ok, this window is closed and your new key is now shown next to key 00, this has to be repeated for each key you want to amend. A quick way to get a working set of d2mac keys into the funcard is to rip them from a mm3 eeprom file. To do this move your cursor to, file>create hex from MM3, follow the prompts (make sure you have a valid mm3 eeprom file for this to work), you can also use the Internet download function if you are connected to the Internet.

When your satisfied that everything is ok SAVE THE FILE AS ee_ext.hex as the funprom will look for the ee_ext.hex file on your disc to load before programming, it doesn’t check for anything loaded into the PC’s memory, (this may be rectified in a later release).

To edit the channel name or SA, left double click on the channel you want to edit, a window pops up as shown.

All white boxes can be edited, notice there is space for CUSTP, there is no need to put in a CUSTP as the funcard doesn’t use it, the rest is self explanatory, here is an explanation of the “On channel” switching I hope this explains this function. It’s taken from one of our member’s replies to a question.

(Start)

The other option that works on most receivers is the "On channel switch". Enter code 5599 and reset the card.

I'll try to explain this in detail as not all seams to understand how to make use of this.

Let’s say your watching C+ and you want to go to TV1000. As decoding C+ your card must be in ECS mode, it can’t be done without switching to ECM.
SO! instead of just switching to TV1000 directly you switch to any CTV channel. (Eurosport, for instance) Doing that the card change to ECM mode and decodes Eurosport. Now you can switch to TV1000 and watch. If you want to go back to C+ you simply switch to Eurosport again and the card switches to ECS mode.

There are a couple of things to take notice about.
The card must be set in the "Switch on channel mode". Can be done either before programming (in editor) or bye entering 5599.
YOU MUST!! have correct codes for BOTH ecm AND ecs mode for the CTV channels. The card won't switch if it cannot decode the channel.
You CAN'T go from C+ >> Eurosport and then change your mind and go back to C+. The card switched to ECM mode when you just selected Eurosport. If you want to go back to C+ you must decode any ECM (NOT CTV) channel then go back to Eurosport again and make the card sw. to ecs. THEN you can go to C+.

I hope this makes some sense. Feel free to ask of any detail. This on channel thing work great ones you learned the "setbacks" it gives. You'll find the different codes for switching in the "Codes.html" help file.

(End)

(Ta Mac ;-) )

Move cursor to “edit EC”click on “config byte” you will see this, if your receiver has difficulty in switching this may come in handy. You can do this for each channel or on individual channels the choice is yours. Remember to save the file after editing before programming

# Seca keys Editing.

After loading the Seca file from C:\Program Files\WhoCares\FunProm\Seca you should see something like below.

The editing procedure is the same as with D2MAC, the only thing you might like to know is outlined below,

Right hand side of the window.

Key #00-P means Primary key

Key #00-S means Secondary key

Key #01-P means Primary key

Key #01-S means Secondary key and so on….

The management keys are usually #01-P or S to #03-P or S (but could be both) and the operational keys (the ones you find on the net) are on #0A to #0F.

The Left hand side window has an extra row boxes if you double click on the provider name, called the UA (Unique Address) if you have the UA you can enter this. You don’t require a CUSTWP the soft automatically does this for you.

At the moment the Aston Seca cam v1.03 can only hold 8 or 9 providers, this may change on a future Aston update.

REMEMBER TO SAVE THE FILE AFTER YOU HAVE EDITED IT BEFORE PROGRAMMING.

# Internet download Tab

IMPORTANT: LIVE CODES HAS SHUT DOWN, BELOW IS WHAT TO EXPECT IF LIVE CODES EVER COMES BACK AND THIS IS ONLY FOR D2MAC

Press the Download Tab and you should see something like this.

I’m using Machievelli’s site for this example http://www.macsat.com/

IMPORTANT: Make sure you have set up your normal internet USER NAME and PASSWORD by pressing the Dialup props button on the right hand side BEFORE USING THIS FUNCTION

Starting top left.

Hexfile site.

This should be self explanatory, but if you know a different site address for live hex files the URL has to go here, press the button “Save site” this address will be loaded automatically every time you start the FunProm

File types/New type.

Again should be self-explanatory. This shows the actual file name used on Machiavelli’s site. The file must be inputted in the “New Type” press the add button and it appears in the File types window, to remove, highlight the file name and press the remove button, if your happy with the site file press the “Save site button to save your site.

The down loaded file will be saved in C:\Program Files\WhoCares\FunProm\Download, you can move this file to the fumprom directory or select it from the program tab (Ext eeprom) select C:\Program Files\WhoCares\FunProm\Download\funextee.hex

Connect Button.

Connects you to the Internet site, when pressed this connects you to the desired site and downloads the file funextee.hex (in this example).

Live Code Site.

The site address for the live codes

Updated keys,

This shows you the idents/keys that are being updated

If your happy with the site press Save Site, this will be loaded automatically every time you start the FunProm

Connect button.

Connects you to the Internet site and starts to download the live files as a text file like below.

NOTE: - This function inserts the keys into the editors memory IT IS NOT SAVED, when finished press the keys tab and you will see the keys inserted, if your happy that everything is ok SAVE the file, (file>save hex> [file name], use this file you have created for programming.

Enjoy your card, there’s more to come 

TTfn

Written and produced by Whocares

With many thanks to TSB & Eklid  (ta m8’s) for there kind permission to use the ideas and extracts from http://tsb.eurocrypt.net/support/maneng/fcmanual.htm