Chemistry  Structural Questions for the HKCEE

Part VII		Important Industrial Products

- The following table gives some information about four nitrogen containing compounds which may be used as fertilizers.

| Compound | Solubility in water at room temperature (mol dm-3) | Cost per kg of compound ($) | Cost per kg of nitrogen in the compound ($) |
| --- | --- | --- | --- |
| (NH4)2HPO4 | 4.5 | 142.0 | 665.0 |
| (NH4)2SO4 | 6.0 | 15.1 | 71.0 |
| NH4NO3 | 26.8 | 20.0 | 57.1 |
| NH3 | 31.2 | 12.2 | 14.8 |

- Write an equation for the reaction of heat on ammonium nitrate. (1 mark)

- The growth of some plants requires a large amount of nitrogen. Name the compound that is the most suitable for use as a fertilizer for these plants. Explain your answer with 2 REASONS. (3 marks)

- What is the “real price” of fertilizer? Using this “real price”, explain which of the above fertilizers supplies the cheapest source of nitrogen. (2 marks)

- Suppose the yield of crops increases with the amount of fertilizer used. Suggest why a farmer does not use as much fertilizers as he can afford in order to increase the yield. (2 marks)

TOTAL: 8 marks

- Urea is a compound containing nitrogen and it is manufactured for use as fertilizers. Urea is made by heating a dry mixture of ammonia and carbon dioxide to 200oC in a pressurized, silver lined reactor. The mixture of urea and water formed is cooled to 15oC and crystals of urea are separated.

- Suggest two reasons why the reactor is lined with silver. (2 marks)

- Suggest why the mixture is cooled to 15oC. (2 marks)

- Suggest how crystals of urea can be obtained from the reaction mixture. (1 mark)

TOTAL: 5 marks

End

*** Please write your answers on single-line papers ***