| Expt. No. | Baud Rate Generator |
| --- | --- |
| Date : |  |

Aim:

To design and simulate the Baud Rate Generator VHDL.

Requirements:

WARP

Active HDL

Procedure:

- The Specifications of the Baud Rate are chosen.

- The input and the output ports of the above specification are defined to a Standard language (std_logic). The temporary variables are selected if necessary.

- Entity and Architecture is created for the above specification.

- The Result is verified by simulation and the waveforms are seen.

Program for Baud Rate Generator

Library IEEE;

Use IEEE.std_logic_1164.all;

Use IEEE.std_logic_unsigned.all;

Use IEEE.std_logic_arith.all;

Entity baud_gen_latest is

Port (

Sysclk: in STD_LOGIC;

rst_b: in STD_LOGIC;

sel: in STD_LOGIC_VECTOR (2 downto 0);

baclkx8: buffer STD_LOGIC;

bcclk: out STD_LOGIC

);

end baud_gen_latest;

architecture baud_gen_latest of baud_gen_latest is

Signal ctr1:std_logic_vector (3 downto 0):= "0000";

Signal ctr2:std_logic_vector (7 downto 0):= "00000000";

Signal ctr3:std_logic_vector (2 downto 0):= "000";

signal clkdiv:std_logic_vector(3 downto 0) := "0000";

begin

process(sysclk)

begin

if(sysclk'event and sysclk = '1') then

if(ctr1="1100") then ctr1 <= "0000";

else ctr1 <= ctr1 +1;

end if;

end if;

end process;

clkdiv <= ctr1 + 1;

process(clkdiv(3))

begin

if(rising_edge(clkdiv(3))) then

ctr2 <= ctr2 +1;

end if;

end process;

process(clkdiv(3))

begin

if (sel = "000") then

baclkx8 <= ctr2(0);

elsif sel = "001" then

baclkx8 <= ctr2(1);

end if;

end process;

process(baclkx8)

begin

if(rising_edge(baclkx8)) then

ctr3 <= ctr3 +1;

end if;

end process;

bcclk <= ctr3(2);

end baud_gen_latest;

Symbol:

Waveform:

Result:

Thus the Baud Rate Generator is designed and simulated using WARP and Active HDL.

| Expt. No. | Common Emitter (RF)Amplifier |
| --- | --- |
| Date : |  |

Aim:

To design an analog circuit namely RF amplifier and simulate that circuit through cadence design tools orcad and PSPICE simulator and to chart the electrical characteristics of RF amplifier.

Requirements:

Orcad 9.2

Cadence tool.

PSPICE Stimulus editor

Procedure:

- The Specifications of the RF Amplifier are chosen..

- The Various resistors and capacitors values are designed and selected after a design of the Amplifier.

- Draw the schematic in the Design editor of Orcad.

- The Result is verified by simulation and characteristics are verified by seeing the waveform.

Schematic:

Components:

| Name of the Component | Library |
| --- | --- |
| Resistor (R) | Analog_p.olb |
| Capacitor (C) | Analog_p.olb |
| Transistor (Q2n2222) | Transistor.olb |
| VSRC | Source.olb |
| VSIN | Source.olb |

PSPICE simulated code (generated by the simulator):

****     CIRCUIT DESCRIPTION

** Creating circuit file "rf-SCHEMATIC1-rf.sim.cir"

** WARNING: THIS AUTOMATICALLY GENERATED FILE MAY BE OVERWRITTEN BY SUBSEQUENT SIMULATIONS

*Libraries:

* Local Libraries :

* From [PSPICE NETLIST] section of E:\Program Files\OrcadLite\PSpice\PSpice.ini file:

.lib "nom.lib"

*Analysis directives:

.TRAN  0 10ns 0

.PROBE V(*) I(*) W(*) D(*) NOISE(*)

.INC ".\rf-SCHEMATIC1.net"

**** INCLUDING rf-SCHEMATIC1.net ****

* source RF

Q_Q1         N00117 N00175 N00146 Q2N2222

R_R1         N00117 N00262  2k

C_C1         0 N00146  10uf

R_R2         N00175 N00262  5.6k

C_C2         N00175 N00401  20uf

R_R3         0 N00175  2.2k

V_V1         N00262 0

+SIN 0 1mV 10Mhz 0 0 0

R_R4         0 N00146  8.6k

V_V2         N00401 0 DC 22v

**** RESUMING rf-SCHEMATIC1-rf.sim.cir ****

.END

**** 05/14/03 12:30:32 ************** PSpice Lite (Mar 2000) *****************

** Profile: "SCHEMATIC1-rf"  [ g:\Pradeep\orcad\rf-SCHEMATIC1-rf.sim ]

****     BJT MODEL PARAMETERS

Q2N2222

NPN

IS   14.340000E-15

BF  255.9

NF    1

VAF   74.03

IKF     .2847

ISE   14.340000E-15

NE    1.307

BR    6.092

NR    1

RB   10

RC    1

CJE   22.010000E-12

MJE     .377

CJC    7.306000E-12

MJC     .3416

TF  411.100000E-12

XTF    3

VTF    1.7

ITF     .6

TR   46.910000E-09

XTB    1.5

CN    2.42

D     .87

****     INITIAL TRANSIENT SOLUTION       TEMPERATURE =   27.000 DEG C

NODE   VOLTAGE     NODE   VOLTAGE     NODE   VOLTAGE     NODE   VOLTAGE

(N00117)-2.809E-27 (N00146) 18.33E-27 (N00175)-1.148E-27 (N00262)    0.0000

(N00401)   22.0000

VOLTAGE SOURCE CURRENTS

NAME         CURRENT

V_V1        -1.610E-30

V_V2         0.000E+00

TOTAL POWER DISSIPATION   0.00E+00  WATTS

JOB CONCLUDED

TOTAL JOB TIME             .07

After Simulation

Result:

Thus the RF amplifier is simulated through cadence design tools orcad and PSPICE simulator and the electrical characteristics of RF amplifier are charted.

| Expt. No. | Study of Monitor Routines in 8051 |
| --- | --- |
| Date : |  |

Aim:

To study about monitor routines for a reset in 8051.

Requirements:

Ride Development tool.

Procedure:

- The Specifications of the monitor routine for Reset are chosen.

- Write the reset signals for all the CPUs, memory registers,etc.

- Compile and Run the program to see the output.

Program for Monitor Routine for Reset:

ORG 0000H

ACALL ROUTINE

HERE:		SJMP HERE

ROUTINE: 	MOV DPTR, #00H

CLR A

MOV B, #00H

MOV SP, #07H

MOV PSW, #00H

MOV P0, 0FFH

MOV P1, 0FFH

MOV P2, 0FFH

MOV P3, 0FFH

MOV IP, #00H

MOV IE, #00H

MOV TCON, #00H

MOV TMOD, #00H

MOV TL0, #00H

MOV TH0, #00H

MOV TL1, #00H

MOV TH1, #00H

MOV SCON, #00H

MOV SBUF, #00H

MOV PCON, #00H

RET

END

Result:

Thus the study of monitor routine for reset in 8051 is studied.