| Expt. No. 1 | Octal Transceiver |
| --- | --- |
| Date : |  |

Aim:

To design, simulate and synthesize the Octal Transceiver using VHDL.

Requirements:

AT 40K FPGA Kit

Active HDL

Procedure:

- The Specifications of the Octal Transceiver is taken.

- The input and the output ports of the above specification are defined to a Standard language (std_logic). The temporary variables are selected if necessary.

- Entity and Architecture is created for the above specification.

- The Result is verified by simulation and the waveforms are seen.

Program for Octal Transceiver:

library ieee;

use ieee.std_logic_1164.all;

entity octal_trans is

port(a,b : inout std_logic_vector(7 downto 0);

control: in std_logic;

dir: in std_logic);

end octal_trans;

architecture oct of octal_trans is

begin

process(control,a,b,dir)

begin

if control = '1' then

if dir = '1' then

b <=a;

elsif dir = '0' then

a <= b;

else

a <= "ZZZZZZZZ";

b <= "ZZZZZZZZ";

end if;

end if;

end process;

end oct;

Symbol:

Waveform for Octal Transceiver:

Result:

Thus the Octal Transceiver is designed and simulated using VHDL.

| Expt. No. 2 | Simulation of RAM |
| --- | --- |
| Date : |  |

Aim:

To design, simulate and synthesize the Random Access Memory using VHDL.

Requirements:

AT 40K FPGA Kit

Active HDL

Procedure:

- The Specification of the Random Access Memory is taken.

- The input and the output ports of the above specification are defined to a Standard language (std_logic). The temporary variables are selected if necessary.

- Entity and Architecture is created for the above specification.

- The Result is verified by simulation and the waveforms are seen.

Program for RAM:

library IEEE;

use IEEE.std_logic_1164.all;

entity ram is

port (

datain: in STD_LOGIC_VECTOR (7 downto 0);

dataout: out STD_LOGIC_VECTOR (7 downto 0);

mem1: buffer STD_LOGIC_VECTOR (7 downto 0);

mem2: buffer STD_LOGIC_VECTOR (7 downto 0);

mem3: buffer STD_LOGIC_VECTOR (7 downto 0);

mem4: buffer STD_LOGIC_VECTOR (7 downto 0);

enable: in STD_LOGIC;

r_wb: in STD_LOGIC;

sel: in STD_LOGIC_VECTOR (1 downto 0)

);

end ram;

architecture ram of ram is

begin

process(enable,r_wb,sel)

begin

if(enable='1' and r_wb = '1') then

case sel is

when "00" =>dataout<=mem1;

when "01" =>dataout<=mem2;

when "10" =>dataout<=mem3;

when "11" =>dataout<=mem4;

when others => null;

end case ;

elsif(enable='1' and r_wb= '0') then

case sel is

when "00" => mem1 <=datain;

when "01" => mem1 <=datain;

when "10" => mem1 <=datain;

when "11" => mem1 <=datain;

when others => null;

end case;

end if;

end process;

end ram;

Symbol:

Waveform for RAM

Result:

Thus the RAM is simulated using VHDL in AT40K FPGA Kit.

| Expt. No.3 | Simulation of Read Only Memory (ROM) |
| --- | --- |
| Date : |  |

Aim:

To design, simulate and Synthesize the Read only Memory using VHDL.

Requirements:

AT40K FPGA Kit

Active HDL

Procedure:

- The Specification of ROM is taken.

- The input and the output ports of the above specification are defined to a Standard language (std_logic). The temporary variables are selected if necessary.

- Entity and Architecture is created for the above specification.

- The Result is verified by simulation and the waveforms are seen.

Design:

Program for ROM

library IEEE;

use IEEE.std_logic_1164.all;

entity rom is

port (

clk: in STD_LOGIC;

oe: in STD_LOGIC;

a: in INTEGER range 0 to 31;

dataout: out INTEGER range 0 to 15

);

end rom;

architecture rom of rom is

type rom_type is array(31 downto 0) of integer range 0 to 15;

signal ROM:rom_type;

begin

process(clk)

begin

if(clk'event and clk = '1') then

if(oe ='1') then

dataout <= ROM(a);

end if;

end if;

end process;

end rom;

Waveform for ROM:

Result:

Thus the ROM is designed and simulated using Active HDL

| Expt. No.4 | Simulation of Arithmetic and Logic Unit |
| --- | --- |
| Date : |  |

Aim:

To design, simulate and synthesize a Arithmetic and Logic Unit using VHDL.

Requirements:

AT40K FPGA Kit

Active HDL

Procedure:

- The specifications are taken.

- The input and the output ports of the above specification are defined to a Standard language (std_logic). The temporary variables are selected if necessary.

- Entity and Architecture is created for the above specification.

- The Result is verified by simulation and the waveforms are seen.

- Also the Code is downloaded to CPLD and the output is verified.

Symbol

Program for ALU

library IEEE;

use IEEE.std_logic_1164.all;

use ieee.std_logic_unsigned.all;

entity alu is

port (

a: in STD_LOGIC_VECTOR (3 downto 0);

b: in STD_LOGIC_VECTOR (3 downto 0);

instruction: in STD_LOGIC_VECTOR (3 downto 0);

c: out STD_LOGIC_VECTOR (3 downto 0)

);

constant reada : std_logic_vector(3 downto 0):="0000";

constant aandb : std_logic_vector(3 downto 0):="0001";

constant aorb : std_logic_vector(3 downto 0):="0010";

constant axorb : std_logic_vector(3 downto 0):="0011";

constant nota : std_logic_vector(3 downto 0):="0100";

constant anandb : std_logic_vector(3 downto 0):="0101";

constant anorb : std_logic_vector(3 downto 0):="0110";

constant axnorb : std_logic_vector(3 downto 0):="0111";

constant aaddb : std_logic_vector(3 downto 0):="1000";

constant asubb : std_logic_vector(3 downto 0):="1001";

constant aadd1 : std_logic_vector(3 downto 0):="1010";

constant asub1 : std_logic_vector(3 downto 0):="1011";

constant read1 : std_logic_vector(3 downto 0):="1100";

constant read0 : std_logic_vector(3 downto 0):="1101";

end alu;

architecture alu of alu is

begin

process(instruction)

begin

case instruction is

when reada => c <=a after 1ns;

when aandb => c <=a and b;

when aorb => c <=a or b;

when axorb => c <=a xor b;

when nota => c <=not a;

when anandb => c <=a nand b;

when anorb => c <=a nor b;

when axnorb	=> c <=a xnor b;

when aaddb => c <= a + b;

when asubb => c <= a - b;

when aadd1 => c <= a + 1;

when asub1 => c <= a - 1;

when read1 => c <="1111";

when read0 => c <="0000";

when others => c <="ZZZZ";

end case;

end process;

end alu;

Waveform for ALU:

Result:

Thus the Arithmetic and Logic Unit is simulated and the results are verified.

| Expt. No.5 | Traffic Light Control System |
| --- | --- |
| Date : |  |

Aim:

To design, simulate and synthesize the Traffic Light control system for the given specifications using VHDL.

Requirements:

Xilinx EDA Design Tools

Active HDL

Procedure:

- The Specification like input and the output ports are defined for the Traffic control System.

- The input and the output ports of the above specification are defined to a Standard language (std_logic). The temporary variables are selected if necessary.

- Entity and Architecture is created for the above specification.

- The Result is verified by simulation and the waveforms are seen.

- Also the Code is downloaded to CPLD and the output is verified.

Program for Traffic Light Control System:

library ieee;

use ieee.std_logic_1164.all;

ENTITY traffic_light IS

PORT(sensor : IN std_logic;

clock : IN std_logic;

red_light : OUT std_logic;

green_light : OUT std_logic;

yellow_light : OUT std_logic);

END traffic_light;

ARCHITECTURE simple OF traffic_light IS

TYPE t_state is (red, green, yellow);

SIGNAL present_state, next_state : t_state;

BEGIN

PROCESS(present_state, sensor)

BEGIN

CASE present_state IS

WHEN green =>

next_state <= yellow;

red_light <= '0';

green_light <= '1';

yellow_light <= '0';

WHEN red =>

red_light <= '1';

green_light <= '0';

yellow_light <= '0';

IF (sensor = '1') THEN

next_state <= green;

ELSE

next_state <= red;

END IF;

WHEN yellow =>

red_light <= '0';

green_light <= '0';

yellow_light <= '1';

next_state <= red;

END CASE;

END PROCESS;

PROCESS

BEGIN

WAIT UNTIL clock'EVENT and clock = '1';

present_state <= next_state;

END PROCESS;

END simple;

Symbol:

Waveform:

Result:

Thus the Traffic Light Control System has been designed and simulated using VHDL and output verified.

| Expt. No. 6 | Iterative Binary Comparator |
| --- | --- |
| Date : |  |

Aim:

To design, simulate and synthesize a Iterative Binary Comparator using VHDL for the given specification.

Requirements:

WARP

Active HDL

Design:

Two binary numbers X=Xn……….X3, X, X1, X0 and Y=Yn……….Y3, Y2, Y1, Y0 are to be Compared by an iterative network. The comparator should compare the least significant bits first, So that the inter cell signals will propagate from right to left.

Yn        Xn                      Y2        X2               Y1        X1                   Y0    X0

--

an+1			an------	  a3		  a2    			a1		  a0

Procedure:

- The Specifications are taken.

- The input and the output ports of the above specification are defined to a Standard language (std_logic). The temporary variables are selected if necessary.

- Entity and Architecture is created for the above specification.

- The Result is verified by simulation and the waveforms are seen.

- Also the Code is downloaded to FPGA and the output is verified.

Program for Comparator:

library IEEE;

use IEEE.std_logic_1164.all;

entity comparator is

port (

x: in STD_LOGIC_VECTOR (3 downto 0);

y: in STD_LOGIC_VECTOR (3 downto 0);

clock: in STD_LOGIC;

z: out STD_LOGIC

);

end comparator;

architecture comparator of comparator is

signal d,e,f,g:std_logic;

signal u:std_logic :='0';

begin

process(clock)

begin

if(clock'event and clock = '1') then

for i in 0 to 3 loop

d <= not(y(i));

e <= x(i) and d;

f <= x(i) and u;

g <= d and u;

u <= e or f or g;

end loop;

z <= u;

end if;

end process;

end comparator;

Symbol:

Waveform:

Result:

Thus the Iterative Binary Comparator is designed, simulated and verified using Xilinx EDA Tools.

| Expt. No. 7 | Barrel Shift Register |
| --- | --- |
| Date : |  |

Aim:

To design, simulate and synthesize the Barrel Shifter using VHDL.

Requirements:

Xilinx EDA Tools

Active HDL

Procedure:

- The Specifications are taken

- The input and the output ports of the above specification are defined to a Standard language (std_logic). The temporary variables are selected if necessary.

- Entity and Architecture is created for the above specification.

- The Result is verified by simulation and the waveforms are seen.

- Also the Code is downloaded to CPLD and the output is verified.

Design:

A network that adds one to the contents of a shift register has the following form.

X      			Z

Control

One Adder

Ps   Pi

Program for Barrel Shifter:

library IEEE;

use IEEE.std_logic_1164.all;

entity barrel_shift is

port (

i: in STD_LOGIC_VECTOR (0 to 7);

s: in STD_LOGIC_VECTOR (2 downto 0);

dataout: out STD_LOGIC_VECTOR (0 to 7)

);

end barrel_shift;

architecture barrel_shift of barrel_shift is

begin

process(s,i)

begin

case s is

when "000" =>

dataout <= i;

when "001" =>

dataout <= i(1) & i(2) & i(3) & i(4) & i(5) & i(6) & i(7) & i(0);

when "010" =>

dataout <= i(2) & i(3) & i(4) & i(5) & i(6) & i(7) & i(0) & i(1);

when "011" =>

dataout <= i(3) & i(4) & i(5) & i(6) & i(7) & i(0) & i(1) & i(2);

when "100" =>

dataout <= i(4) & i(5) & i(6) & i(7) & i(0) & i(1) & i(2) & i(3);

when "101" =>

dataout <= i(5) & i(6) & i(7) & i(0) & i(1) & i(2) & i(3) & i(4);

when "110" =>

dataout <= i(6) & i(7) & i(0) & i(1) & i(2) & i(3) & i(4) & i(5);

when "111" =>

dataout <= i(7) & i(0) & i(1) & i(2) & i(3) & i(4) & i(5) & i(6);

when others => null;

end case;

end process;

end barrel_shift;

Symbol:

Waveform:

Result:

Thus the Barrel Shift Register is simulated and synthesized.

| Expt. No. 8 | Universal Asynchronous Receiver and Transmitter |
| --- | --- |
| Date : |  |

Aim:

To design and simulate a Universal Asynchronous Receiver and Transmitter (UART) using EDA Tool.

Requirements:

Xilinx EDA Tool

Active HDL

Procedure:

- The Specifications are taken

- The input and the output ports of the above specification are defined to a Standard language (std_logic). The temporary variables are selected if necessary.

- Entity and Architecture is created for the above specification.

- The Result is verified by simulation and the waveforms are seen.

- Also the Code is downloaded to CPLD and the output is verified.

Program for UART:

library ieee;

use ieee.std_logic_1164.all ;

use ieee.std_logic_arith.all ;

use ieee.std_logic_unsigned.all ;

entity uart is

PORT (rst,clk16x,rxd,rdn,wrn : in std_logic;

din : in std_logic_vector(7 downto 0);

dout : out std_logic_vector(7 downto 0);

data_ready : out std_logic;

parity_error : out std_logic;

framing_error : out std_logic;

tbre : out std_logic;

tsre : out std_logic;

sdo : out std_logic);

end uart;

architecture v1 of uart is

component txmit

port (rst,clk16x,wrn : in std_logic;

din : in std_logic_vector(7 downto 0);

tbre,tsre,sdo: out std_logic);

end component ;

component rcvr

port (rst,clk16x,rxd,rdn : in std_logic;

data_ready, parity_error, framing_error : out std_logic;

dout : out std_logic_vector(7 downto 0));

end component ;

begin

u1 : txmit PORT MAP (rst => rst,clk16x => clk16x,wrn => wrn,din => din,tbre => tbre,tsre => tsre,sdo => sdo);

u2 : rcvr PORT MAP  (rst => rst,clk16x => clk16x,rxd => rxd,rdn => rdn,data_ready => data_ready,framing_error => framing_error,parity_error => parity_error,dout => dout) ;

end v1 ;

Schematic:

Waveform:

Result:

Thus the Universal Asynchronous Receiver Transmitter is designed and simulated.

| Expt. No. 9 | Digital Clock |
| --- | --- |
| Date : |  |

Aim:

To design a Digital Clock using Xilinx EDA Tool.

Requirements:

Xilinx EDA Tool

Active HDL

Procedure:

- The Specifications are taken.

- The input and the output ports of the above specification are defined to a Standard language (std_logic). The temporary variables are selected if necessary.

- Entity and Architecture is created for the above specification.

- The Result is verified by simulation and the waveforms are seen.

Schematic:

Program:

library IEEE;

use IEEE.std_logic_1164.all;

entity digitalclock is

port (

mins: inout integer range 0 to 59;

secs: inout integer range 0 to 59;

hrs: inout integer range 0 to 23;

clk: in STD_LOGIC

);

end digitalclock;

architecture behav of digitalclock is

begin

process(clk)

begin

if (clk'event and clk = '1') then

if secs<59 then

secs <= secs+1;

elsif

mins<59 then

mins <= mins+1;

secs<=00;

elsif hrs<23 then

hrs <= hrs+1;

mins<=00;

secs<=00;

elsif( hrs=23 and mins=59 and secs=59 )then

secs<=00;

mins<=00;

end if;

end if;

end process;

end behav;

Waveform:

Result:

Thus the Digital Clock is designed which shows Hours, minutes and seconds.

| Expt. No.10 | 8 bit Full Adder |
| --- | --- |
| Date : |  |

Aim:

To design a 8 bit Full Adder using VHDL language and EDA Tool.

Requirements:

Xilinx EDA Tool

Active HDL.

Procedure:

- The Specifications are taken.

- The input and the output ports of the above specification are defined to a Standard language (std_logic). The temporary variables are selected if necessary.

- Entity and Architecture is created for the above specification.

- The Result is verified by simulation and the waveforms are seen.

Program:

library IEEE;

use IEEE.std_logic_1164.all;

entity full_adder_8 is

port (

a: in STD_LOGIC_VECTOR (7 downto 0);

b: in STD_LOGIC_VECTOR (7 downto 0);

s: out STD_LOGIC_VECTOR (7 downto 0);

cout: out STD_LOGIC

);

end full_adder_8;

architecture full_adder_8 of full_adder_8 is

component full is

port(a,b,c:in std_logic;

sum,carry : out std_logic);

end component;

signal c:std_logic_vector(7 downto 0);

begin

c0:full port map(a=> a(0), b=> b(0), c=>'0', sum=> s(0), carry =>c(0));

c1:full port map(a(1),b(1),c(0),s(1),c(1));

c2:full port map(a(2),b(2),c(1),s(2),c(2));

c3:full port map(a(3),b(3),c(2),s(3),c(3));

c4:full port map(a(4),b(4),c(3),s(4),c(4));

c5:full port map(a(5),b(5),c(4),s(5),c(5));

c6:full port map(a(6),b(6),c(5),s(6),c(6));

c7:full port map(a(7),b(7),c(6),s(7),c(7));

cout <= c(7);

end full_adder_8;

Symbol:

Waveform:

Result:

Thus a 8 bit full adder is designed and simulated.

| Expt. No.11 | Binary Multiplier |
| --- | --- |
| Date : |  |

Aim:

To design Binary Multiplier using VHDL language and EDA Tool.

Requirements:

Xilinx EDA Tool

Active HDL.

Procedure:

- The Specifications are taken.

- The input and the output ports of the above specification are defined to a Standard language (std_logic). The temporary variables are selected if necessary.

- Entity and Architecture is created for the above specification.

- The Result is verified by simulation and the waveforms are seen.

Program:

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

entity multiply is

port(multiplicand:in std_logic_vector(3 downto 0);

multiplier:in std_logic_vector(3 downto 0);

output:out std_logic_vector(7 downto 0));

end multiply;

architecture multiply of multiply is

begin

process (multiplicand,multiplier)

variable q:std_logic_vector(3 downto 0);

variable b:std_logic_vector(3 downto 0);

variable a:std_logic_vector(3 downto 0);

variable c:std_logic_vector(3 downto 0);

variable z:std_logic;

variable p:std_logic_vector(3 downto 0);

variable g:std_logic_vector(3 downto 0);

begin

q:=multiplier;

b:=multiplicand;

a:="0000";

z:='0';

c(3):='0';

for count in 1 to 4 loop

if q(0) = '1' then

p:=a xor b;

g:=a and b;

c(0):=g(0) or (p(0) and z);

c(1):=g(1) or (p(1) and c(0));

c(2):=g(2) or (p(2) and c(1));

c(3):=g(3) or (p(3) and c(2));

a(0):=p(0) xor z;

a(1):=p(1) xor c(0);

a(2):=p(2) xor c(1);

a(3):=p(3) xor c(2);

q:=a(0) & q(3 downto 1);

a:=c(3) & a(3 downto 1);

else

q:=a(0) & q(3 downto 1);

a:=c(3) & a(3 downto 1);

end if;

end loop;

output<=a(3 downto 0) & q(3 downto 0);

end process;

end multiply;

Symbol:

Waveform:

Result:

Thus the multiplier is designed and simulated.

| Expt. No.12 | Divider |
| --- | --- |
| Date : |  |

Aim:

To design Binary Divider using VHDL language and EDA Tool.

Requirements:

Xilinx EDA Tool

Active HDL.

Procedure:

- The Specifications are taken.

- The input and the output ports of the above specification are defined to a Standard language (std_logic). The temporary variables are selected if necessary.

- Entity and Architecture is created for the above specification.

- The Result is verified by simulation and the waveforms are seen.

Program:

library IEEE;

use IEEE.std_logic_1164.all;

use ieee.std_logic_unsigned.all;

use ieee.std_logic_arith.all;

entity div is

port (

dividend: in STD_LOGIC_VECTOR (7 downto 0);

divisor: in STD_LOGIC_VECTOR (3 downto 0);

quotient: out STD_LOGIC_VECTOR (7 downto 0);

remainder: out STD_LOGIC_VECTOR (4 downto 0)

);

end div;

architecture arcdiv of div is

begin

process(dividend,divisor)

variable a,b,bcomp:std_logic_vector(4 downto 0);

variable q:std_logic_vector(7 downto 0);

begin

q:=dividend;

a:= "00000";

b:='0' & divisor;

bcomp:=not b;

bcomp:=bcomp +1;

for count in 1 to 8 loop

a:=a(3 downto 0) & q(7);

if a(4)='1' then

a:= a+b;

if a(4)='1' then

q:=q(7 downto 0) & '0';

else

q:=q(7 downto 0) & '1';

end if;

else

a:= a+ bcomp;

if a(4)='1' then

q:=q(7 downto 0) & '0';

else

q:=q(7 downto 0) & '1';

end if;

end if;

end loop;

if a(4)='1' then

a:=a+b;

end if;

quotient<= q;

remainder <=a;

end process;

end arcdiv;

Symbol:

Waveform:

Result:

Thus the Binary Divider is designed and simulated using EDA Tools.

| Expt. No.13 | Separation of ODD and EVEN Numbers |
| --- | --- |
| Date : |  |

Aim:

To separate Odd and Even numbers in the given sequence using DSP Processor TMS320C50

Requirements:

DSP Processor Kit TMS320C50

CRO

Assembly Program:

; SEPARATION OF ODD & EVEN COMPONENTS

.MMREGS                 ;initialisation

.TEXT

LDP #100H

LAR AR0,#8000H          ;auxilliary registers initialisation

LAR AR1,#8050H

LAR AR2,#8060H

LAR AR3,#5H

REPEAT:  MAR *,AR0               ;inputs are given to the auxilliary

LACC *+                 ;register AR0

SACB

RPT #15                 ;divide the input by 2

SUBC 000AH              ;value in 000A must be 2

AND #1H,10H             ;check the remainder,if it is 0,considerd

BCND EVEN,EQ            ;as a even component,if it is 1,considerd

;as a odd component

LACB

MAR *,AR1               ;odd components are stored in auxilliary

SACL *+                 ;register AR1

COUNT:   MAR *,AR3

BANZ REPEAT,*-

B H

EVEN:   LACB

MAR *,AR2               ;even components are stored in AR2

SACL *+

B COUNT

H:    B H

INPUT:

8000-0001

8001-0002

8002-0003

8003-0004

8004-0005

8005-0006

OUTPUT:

8050-0001

8051-0003

8052-0005

8060-0002

8061-0004

8062-0006

Result:

Thus the Even and Odd Numbers are separated from the sequence and stored in the corresponding location.

| Expt. No.14 | Band Pass Filter |
| --- | --- |
| Date : |  |

Aim:

To design a Band Pass Filter with Cut off Frequency range 1200Hz – 2000Hz (sampling rate 8 KHz ) using DSP Processor TMS320C50

Requirements:

DSP Processor Kit TMS320C50

CRO

Assembly Program:

*

*       M50EB CODEC BP FILTER CUT-OFF 1200Hz - 2000Hz

*       WINDOW FIR 40 TAP (samp rate = 8khz)

*

.mmregs

.text

BIAS  	  .set    97

X       	.set    98

SUM     .set    99

SIGN    .set    100

NEG7    .set    101

Q       .set    102

ONE     .set    103

TMP     .set    104

TMP0    .set    105

S       .set    106

*       Filter coefficients in Q15 format

B       START

CTABLE:

.WORD	0FFFEH

.WORD	0FFEFH

.WORD	0FFFAH

.WORD	0FFF0H

.WORD	0FF6EH

.WORD	0FF52H

.WORD	07EH

.WORD	012CH

.WORD	013H

.WORD	07EH

.WORD	0403H

.WORD	0472H

.WORD	0FEC3H

.WORD	0FC34H

.WORD	0123H

.WORD	0FED6H

.WORD	0ED8FH

.WORD	0E663H

.WORD	018BH

.WORD	0268AH

.WORD	0268AH

.WORD	018BH

.WORD	0E663H

.WORD	0ED8FH

.WORD	0FED6H

.WORD	0123H

.WORD	0FC34H

.WORD	0FEC3H

.WORD	0472H

.WORD	0403H

.WORD	07EH

.WORD	013H

.WORD	012CH

.WORD	07EH

.WORD	0FF52H

.WORD	0FF6EH

.WORD	0FFF0H

.WORD	0FFFAH

.WORD	0FFEFH

.WORD	0FFFEH

*        Move the Filter coefficients

*        from program memory to data memory

*

*

START:

LACC    #00E00H

SACL    TMP0

LST     #0, TMP0

LACC    #00FFCH

SACL    TMP0

LST     #1, TMP0

;

OPL	#0F839H, PMST

LACC	#0

SAMM	CWSR

LACC	#0555AH

SAMM	PDWSR

;

SETC    INTM

LDP     #0

CLRC    CNF

LAR     AR3, #0300H

MAR     *, AR0

LAR     AR0, #0200h

RPT     #027h

BLPD    #CTABLE,*+

SETC    CNF

;

;----F->8254 CONTROL, C,D,E ->COUNTER

;-----------TIMER INITIALISATION -------

LACC	#040H		;ENABLE GATE 1 OF 8254

SACL	TMP

OUT	TMP,00

;

LAC	#0056H

SACL	TMP

OUT	TMP,0FH

LAC	#0003

SACL	TMP

OUT	TMP,0DH

;

LAR     AR5,#03000H

LACC    #07980h

SACL    060H

LACC    #0F80EH

TBLW    060H

;

LACC    #RECEIVE

SACL    060H

LACC    #0F80FH

TBLW    060H

;

LACC    #07980h

SACL    060H

LACC    #0F810H

TBLW    060H

;

LACC    #TRANSMIT

SACL    060H

LACC    #0F811H

TBLW    060H

;

LACC    #00Ch

SACL    TSPC

LACC    #0CCh

SACL    TSPC

;

LACC    #01

SACL    ONE

LACC    #033

SACL    BIAS

LACC    #-07

SACL    NEG7

SPLK    #040h,IMR

CLRC    INTM             ; enable

WAIT:   NOP

NOP

NOP

B       WAIT

;

RECEIVE:

LDP     #0

MAR     *,AR0

BIT     TRCV,8           ; for sign test

LACC    TRCV,12          ; input data

AND     #07F00H,4        ; mask sign

XOR     #05500H,4        ; invert all bits

SACH    S               ;segment is moved to S

SUB     S,16               ; clear segment part

;

LAR     AR0,S

BANZ    SEGNZ,*-

ADD     ONE,11

SFL

BCND    PSVAL1,NTC

NEG

PSVAL1: SACH    X,4

B       JSTIFY

;

SEGNZ:  SAR     AR0,S

ADD     BIAS,11         ; add bias

SFL                     ; shift left one bit

SACH    SUM,4

LT      S               ; SUM value shifted as specified

LACT    SUM             ; by the segment value S

BCND    POSVAL,NTC

NEG

POSVAL: SACL    X

JSTIFY: LACC    X,3             ; make Q13 number to Q15

SACL    X

;

MAR     *,AR3

SACL    *

*-----------------------------------------------------

LDP     #6

LAR     AR1,#0327H

MAR     *,AR1

ZAP

RPT     #027H

MACD    0FF00H,*-

APAC

SACH    0,1

*-------------------------------------------------------

LDP     #0

MAR     *,AR3

LACC    *

SACL    X

;

MAR     *,AR0

LACC    X,16

BCND    NEGCMP,LT          ; for -ve

;

BCND    CMPSK1,EQ

LAR     AR0,NEG7

RPT     #06

NORM    *+              ; normalize the result

BANZ    SEGNZ1,*

SACH    Q

LACC     Q,1

CMPSK1: XOR     #05500H,4

B       SATCH

;

SEGNZ1: AND     #0F000H,14

SACH    Q

SAR     AR0,S

LACC    S,16

ABS

ADD     Q,2

XOR     #05500H,4        ; to invert

B       SATCH

;

NEGCMP: ADD     1

BCND    CMPSK2,EQ

SUB     1

ABS

LAR     AR0,NEG7

;

RPT     #06

NORM    *+

BANZ    SEGNZ2,*

SACH    Q

LACC     Q,1

CMPSK2: XOR     #0D500H,4

B       SATCH

;

SEGNZ2: AND     #0F000H,14

SACH    Q

SAR     AR0,S

LACC    S,16

ABS

ADD     Q,2

XOR     #0D500H,4

SATCH:  SACH    TDXR,4

RETE

TRANSMIT:

RETE

;

.END

Result:

Thus a Band Pass Filter with Cut off Frequency 1200Hz- 2000Hz is generated.

| Expt. No.15 | Waveform Generation |
| --- | --- |
| Date : |  |

Aim:

To generate a Square Wave, Sinusoidal wave and a triangular Wave using DSP Processor TMS320C50 kit.

Requirements:

DSP Processor Kit TMS320C50

CRO

Assembly Program for Square Wave:

FREQ      .SET  350

TEMP    .SET    0        ;temp = 7 bits

;

.mmregs

.text

START:

LDP     #100H      ;dp=1 0000 0000

splk    #0,TEMP    ;1000 0000 0000 0000=0

;concatenate dp and dma(temp).

CONT1:

lar     AR2,#FREQ    ;ar2=350

CONT:

out     TEMP,4     ; SEND TEMP TO DAC.

MAR     *,AR2        ;mar is a nop inst

;arp=2

BANZ    CONT,*-       ;loop if curr ar !=0

SPLK    #0FFFFH,TEMP

lar     AR2,#FREQ    ;ar2=350

CONT2:

out     TEMP,4     ; SEND TEMP TO DAC.

MAR     *,AR2        ;mar is a nop inst

;arp=2

BANZ    CONT2,*-       ;loop if curr ar !=0

B       START

.end

Assembly Program for Sine Wave (Sine wave for 100Hz)

;Sine Wave Generation. With these default values sine generated is 100 Hz.

INCFREQ    .set    3    ;minimum value 1 - change for increasing frequency

;every count will increase frequency in steps of 100hz

DECFREQ    .set    0    ;minimu value 0 - change for decreasing frequency

;every count will decrease frequency by half

LENGTH     .set    360

AMPLITUDE  .set    5

delay      .set    11    ;change to increase/decrease frequency in

;different steps;

TEMP    .set    0

TEMP1   .set    1

.mmregs

.text

START:

LDP     #100H

SPLK    #TABLE,TEMP     ;load start address of table

lar     AR2,#( (LENGTH/INCFREQ)+(LENGTH*DECFREQ) )-1

lar     ar3,#DECFREQ   ;repeat counter for reducing frequency

CONT:

CALL    DELAY

LACC    TEMP            ;load address of sine value

TBLR    TEMP1           ;read sine data to TEMP1

LT      TEMP1

MPY     #AMPLITUDE

PAC

SACL    TEMP1

mar     *,ar3

banz    repeat,*-

lar     ar3,#DECFREQ   ;repeat counter for reducing frequency

LACC    TEMP

ADD     #INCFREQ        ;increase table value

repeat:

SACL    TEMP            ;store it

OUT     TEMP1,4         ;send sine data to DAC

MAR     *,AR2

BANZ    CONT,*-

b       START

DELAY:

lar     ar7,#delay

mar     *,ar7

back:   banz    back,*-

ret

;

Assembly Program for Triangular Wave:

AMPLITUDE .SET  4

FREQ      .SET  350

TEMP    .SET    0

;

.mmregs

.text

START:

LDP     #100H

splk    #0,TEMP

CONT1:

lar     AR2,#FREQ

CONT:

out     TEMP,4

LACC    TEMP

ADD     #AMPLITUDE

SACL    TEMP

MAR     *,AR2

BANZ    CONT,*-

LAR     AR2,#FREQ

CONTx:

OUT     TEMP,4

LACC    TEMP

SUB     #AMPLITUDE

SACL    TEMP

MAR     *,AR2

BANZ    CONTx

B       CONT1

.end

Result:

Thus the Square Wave, Sine wave and Triangular Waveforms are generated using DSP Processor Kit.

| Expt. No.16 | Voice Recognition |
| --- | --- |
| Date : |  |

Aim:

To receive and transmit voice Signal using DSP Processor TMS320C50 kit.

Requirements:

DSP Processor Kit TMS320C50

CRO

Assembly Program for voice Recognition:

.mmregs

.text

BIAS    .set    97

X       .set    98

SUM     .set    99

SIGN    .set    100

NEG7    .set    101

Q       .set    102

ONE     .set    103

TMP     .set    104

TMP0    .set    105

S       .set    106

*       Filter coefficients in Q15 format

B       START

CTABLE:

.WORD	0FDEDH

.WORD	0FDF0H

.WORD	0FE11H

.WORD	0FE53H

.WORD	0FEB8H

.WORD	0FF3DH

.WORD	0FFE2H

.WORD	0A2H

.WORD	017BH

.WORD	0266H

.WORD	035EH

.WORD	045AH

.WORD	0553H

.WORD	0643H

.WORD	0721H

.WORD	07E6H

.WORD	088CH

.WORD	090DH

.WORD	0966H

.WORD	0993H

.WORD	0993H

.WORD	0966H

.WORD	090DH

.WORD	088CH

.WORD	07E6H

.WORD	0721H

.WORD	0643H

.WORD	0553H

.WORD	045AH

.WORD	035EH

.WORD	0266H

.WORD	017BH

.WORD	0A2H

.WORD	0FFE2H

.WORD	0FF3DH

.WORD	0FEB8H

.WORD	0FE53H

.WORD	0FE11H

.WORD	0FDF0H

.WORD	0FDEDH

*        Move the Filter coefficients

*        from program memory to data memory

START:

LACC    #00E00H

SACL    TMP0

LST     #0, TMP0

LACC    #00FFCH

SACL    TMP0

LST     #1, TMP0

;

OPL	#0F839H, PMST

LACC	#0

SAMM	CWSR

LACC	#0555AH

SAMM	PDWSR

;

SETC    INTM

LDP     #0

CLRC    CNF

LAR     AR3, #0300H

MAR     *, AR0

LAR     AR0, #0200h

RPT     #027h

BLPD    #CTABLE,*+

SETC    CNF

;

;----F->8254 CONTROL, C,D,E ->COUNTER

;-----------TIMER INITIALISATION -------

LACC	#040H		;ENABLE GATE 1 OF 8254

SACL	TMP

OUT	TMP,00

;

LAC	#0056H

SACL	TMP

OUT	TMP,0FH

LAC	#0003

SACL	TMP

OUT	TMP,0DH

;

LAR     AR5,#03000H

LACC    #07980h

SACL    060H

LACC    #0F80EH

TBLW    060H

;

LACC    #RECEIVE

SACL    060H

LACC    #0F80FH

TBLW    060H

;

LACC    #07980h

SACL    060H

LACC    #0F810H

TBLW    060H

;

LACC    #TRANSMIT

SACL    060H

LACC    #0F811H

TBLW    060H

;

LACC    #00Ch

SACL    TSPC

LACC    #0CCh

SACL    TSPC

;

LACC    #01

SACL    ONE

LACC    #033

SACL    BIAS

LACC    #-07

SACL    NEG7

LAR     AR2,#8000H

LAR     AR4,#07FEFH

SPLK    #040h,IMR

CLRC    INTM             ; enable

WAIT:   NOP

NOP

NOP

B       WAIT

;

RECEIVE:

LDP     #0

MAR     *,AR0

BIT     TRCV,8           ; for sign test

LACC    TRCV,12

AND     #07F00H,4        ; mask sign

XOR     #05500H,4        ; invert all bits

SACH    S               ;segment is moved to S

SUB     S,16               ; clear segment part

MAR     *,AR0

LAR     AR0,S

BANZ    SEGNZ,*-

ADD     ONE,11

SFL

BCND    PSVAL1,NTC

NEG

PSVAL1: SACH    X,4

B       JSTIFY

SEGNZ:  MAR     *,AR0

SAR     AR0,S

ADD     BIAS,11         ; add bias

SFL                     ; shift left one bit

SACH    SUM,4

LT      S               ; SUM value shifted as specified

LACT    SUM             ; by the segment value S

BCND    POSVAL,NTC

NEG

POSVAL: SACL    X

JSTIFY: LACC    X,3             ; make Q13 number to Q15

SACL    X

MAR     *,AR2

LACC    X

SACL    *+,0,4

BANZ    F,*-

SPLK    #0,IMR

SETC    INTM

F:      LACC    X,16

BCND    NEGCMP,LT

LACC    X,16          ; for -ve

;

BCND    CMPSK1,EQ

MAR     *,AR0

LAR     AR0,NEG7

RPT     #06

NORM    *+              ; normalize the result

BANZ    SEGNZ1,*

SACH    Q

LACC     Q,1

CMPSK1: XOR     #05500H,4

B       SATCH

;

SEGNZ1: AND     #0F000H,14

SACH    Q

MAR     *,AR0

SAR     AR0,S

LACC    S,16

ABS

ADD     Q,2

XOR     #05500H,4        ; to invert

B       SATCH

;

NEGCMP: ADD     1

BCND    CMPSK2,EQ

SUB     1

ABS

MAR     *,AR0

LAR     AR0,NEG7

;

RPT     #06

NORM    *+

BANZ    SEGNZ2,*

SACH    Q

LACC     Q,1

CMPSK2: XOR     #0D500H,4

B       SATCH

;

SEGNZ2: AND     #0F000H,14

SACH    Q

SAR     AR0,S

LACC    S,16

ABS

ADD     Q,2

XOR     #0D500H,4

SATCH:  SACH    TDXR,4

RETE

TRANSMIT:

RETE

;

.END

Result:

Thus the Voice Signal is Received and Transmitted using DSP Processor Kit.

| Expt. No.17 | Design of ADC and PWM in pSOC Tool |
| --- | --- |
| Date : |  |

Aim:

To design a 12 bit ADC and 16 bit Pulse Width Modulation using System On Chip Controller Tool (SOC).

Requirements:

Cypress Microsystems pSOC Designer

Cypress Emulator Kit (Debugger).

Procedure:

- The ADC and PWM are designed.

- Using Device Editor, the ADC and PWM are implemented

- Using Placement Editor, the components are placed according to the design.

- The code is generated in C and Assembly Language

- The code is downloaded into the Emulator tool kit and the debugging is done.

Result:

Thus the 12 bit ADC and PWM are designed using SOC2 Tools.

| Expt. No.18 | Design of 8 bit Counter using SOC Tool |
| --- | --- |
| Date : |  |

Aim:

To design 8 bit Counter using pSOC Device Editor tool.

Requirements:

Cypress Microsystems pSOC Designer

Cypress Emulator Kit (Debugger).

Procedure:

- The 8 bit counter is designed.

- Using Device Editor, the counter is implemented

- Using Placement Editor, the components are placed according to the design.

- The code is generated in C and Assembly Language

- The code is downloaded into the Emulator tool kit and the debugging is done.

Program Generated for Counter:

/* include the global header file */

#include <m8c.h>

/* Create Pragmas to support proper argument and return while passing */

#pragma fastcall Counter8_1_EnableInt

#pragma fastcall Counter8_1_DisableInt

#pragma fastcall Counter8_1_Start

#pragma fastcall Counter8_1_Stop

#pragma fastcall Counter8_1_WritePeriod

#pragma fastcall Counter8_1_WriteCompareValue

#pragma fastcall bCounter8_1_ReadCompareValue

#pragma fastcall bCounter8_1_ReadCounter

/* Prototypes of counter8 API */

extern void Counter8_1_EnableInt(void);

extern void Counter8_1_DisableInt(void);

extern void Counter8_1_Start(void);

extern void Counter8_1_Stop(void);

extern void Counter8_1_WritePeriod(BYTE bPeriod);

extern void Counter8_1_WriteCompareValue(BYTE bCompareValue);

extern BYTE bCounter8_1_ReadCompareValue(void);

extern BYTE bCounter8_1_ReadCounter(void);

/* Hardware Register Definitions */

#pragma ioport Counter8_1_CONTROL_REG; 0x023

BYTE 		Counter8_1_CONTROL_REG;

#pragma ioport Counter8_1_COUNTER_REG; 0x020

BYTE 		Counter8_1_COUNTER_REG;

#pragma ioport Counter8_1_PERIOD_REG; 0x021

BYTE 		Counter8_1_PERIOD_REG;

#pragma ioport Counter8_1_COMPARE_REG; 0x022

BYTE 		Counter8_1_COMPARE_REG;

#pragma ioport Counter8_1_FUNC_REG; 0x120;

BYTE 		Counter8_1_FUNC_REG;

#pragma ioport Counter8_1_INPUT_REG; 0x121

BYTE 		Counter8_1_INPUT_REG;

#pragma ioport Counter8_1_OUTPUT_REG; 0x122

BYTE 		Counter8_1_OUTPUT_REG;

//end of file

Result:

Thus the counter is designed using SOC2 Tool.

| Expt. No.19 | UART and Pseudo Random Generator |
| --- | --- |
| Date : |  |

Aim:

To design a UART Module and 32 bit Pseudo Random Generator (PRG) using System On Chip Controller Tool (SOC).

Requirements:

Cypress Microsystems pSOC Designer

Cypress Emulator Kit (Debugger).

Procedure:

- The UART Module and PRG are designed.

- Using Device Editor, the UART and PRG are implemented.

- Using Placement Editor, the components are placed according to the design.

- The code is generated in C and Assembly Language

- The code is downloaded into the Emulator tool kit and the debugging is done.

Result:

Thus the UART Module and 32 bit Pseudo Random Generator are designed using SOC2 Tool.