IF 363 EMBEDDED ARCHITECTURE

Part A (10 * 2 = 20 marks)

1. Define Propagation delay.

The amount of time elapses between when the input changes and when the output changes is called propagation delay.

2. Name the different types of clock generators

- Oscillators (usually come with 4 pins)

- Crystal (come with two terminals)

3. What is Harvard Architecture?

The Program Memory and the Data memory are separated to achieve an fast execution speed for a given clock rate. Instructions are fetched from program memory and data variables are fetched from other memory.

4. What is a Watch Dog timer.

The Watch Dog Timer is device which is used to reset the PIC if the chip ever malfunctions and deviates from its normal operation.

5. Define Serial Programming.

A programming technique in which the application program and the configuration word are programmed into a PIC chip using a serial Protocol.

6. What are the different Software Architectures for Embedded Architecture?

  - Round Robin

  - Round Robin with Interrupts

  - Function Queue Scheduling

  - Real Time Operating Systems

7. What are Hard Real Time Systems and Soft Real Time Systems?

A Hard Real Time System is a System in which the Deadlines has to be met, if the deadline are not met, then the system will crash.

A Soft Real Time System is a system in which the deadlines even missed only the performance will degrade.

8. What is a linker?

A linker combines separately compiled and assembled modules onto an executable image.  It also places code, data startup code, constant strings and so on at suitable address in ROM and RAM.

9. What are the various methods of intertask communication?

- Semaphore

- Mailboxes

- Message queue

- Pipes

- Events

Part B (5 * 16 = 80 marks)

11. 	Describe the function Queue scheduling Architecture and Compare it with the Real Time Operating System Architecture.

Page 127,133, David E Simon, “An embedded software primer”

12 	Describe the Capture mode and Compare mode of  PIC Controllers.

Page 103, John B Peatman, “Design with PIC Micro controllers”

(Or)

Explain how the Pulse Width Modulation is generated and write about PWM output for various applications.

Page 112, John B Peatman, “Design with PIC Micro controllers”

13. 	Write short notes on three different modes in 68HC11 Microcontroller.

Page 27, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

(Or)

Describe in detail about the expanded mode operation of 68HC11.

Page 28, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

14. 	Describe Logic Analyzer.

Page 313, David E Simon, “An embedded software primer”

(Or)

(a) Write short notes on Intel Hex File format

Page 268, David E Simon, “An embedded software primer”

(b) Describe linkers for an Embedded Software development

Page 263,264, David E Simon, “An embedded software primer”

15. 	Write about the memory management subsystem as applicable to any RTOS.

Page 196, David E Simon, “An embedded software primer”,

(Or)

Write short notes on the factors that are useful to decide an RTOS for an Embedded System.

Page 237, David E Simon, “An embedded software primer”