IF 363 Embedded Architecture

Unit I

Part A

What is the purpose of Decoupling capacitor?

Some part of the circuit needs a lot of power and voltage begins to fall because of that, the capacitor will give up its stored electricity to maintain the voltage level.

What are the ROM Variants used in Embedded Systems?

- Programmable Read Only Memory PROM

- Erasable Programmable Read Only Memory EPROM

- FLASH

- Electrically Erasable Programmable Read Only Memory EEPROM

What are the different memories used in Embedded Systems?

- Read only Memory (ROM)

- Random Access Memory (RAM)

- Cache memory

Define Setup time and Hold Time

The time before the rising edge of the clock during which the input remains constant is called setup time.

The time after the rising edge of the clock during which the input remains constant is called hold time.

Define Propagation delay.

The amount of time elapses between when the input changes and when the output changes is called propagation delay.

Name the different types of clock generators

- Oscillators (usually come with 4 pins)

- Crystal (come with two terminals)

What is bus cycle?

The microprocessor is in control of all signals, and it decides when to look for the data on the bus is called bus cycle.

What is Direct Memory Access (DMA)?

DMA is a circuitry that can read data from an I/O Device such as a serial port of a network and then write it into memory or read from memory and write to an I/O Device all without the software assistance and the associated overhead.

Write any four devices that can be built on the microprocessor.

- DMA

- I/O Pins

- Timers

- Memory caches and instruction pipelines

Define Interrupt Latency.

Interrupt Latency refers to the amount of time it takes a system to respond to an interrupt.

Define Critical Section and Atomic action.

A Set of instructions that must not be interrupted if the system is to work properly is called Critical section.

A set of instructions that will not be interrupted is said to be atomic.

UNIT II

What is Harvard Architecture?

The Program Memory and the Data memory are separated to achieve an fast execution speed for a given clock rate. Instructions are fetched from program memory and data variables are fetched from other memory.

Define Pipelining.

The CPU executes each instruction during the cycle following the Fetch. Pipelining instruction fetches and instruction executions to achieve the execution of one instruction every cycle.

What are the different addressing modes in PIC Controllers?

- Direct Addressing mode

- Indirect addressing mode

Write short notes on the timers employed in PIC Controllers.

There are 3 timers, timer0, timer1 and timer2

Timer0 used to count external events.

Timer1 useful for signal monitoring.

Timer2 used as a looptime counter and hence used to control the period of a PWM.

What is critical region?

A critical region is sequences of instructions in a mainline code that must be protected from an intervening interrupt or suffer a possible erroneous result.

What are the different applications of PWM output?

- Used to drive a D/A Circuit.

- Used to drive a Switched load.

- Used to control the DC motor with an H Bridge driver circuits.

Write the different modes of synchronous Serial Port (SSP)?

- Serial Peripheral Interface

- Inter Integrated Circuit

What is a Watch Dog timer.

The Watch Dog Timer is device which is used to reset the PIC if the chip ever malfunctions and deviates from its normal operation.

Define Serial Programming.

A programming technique in which the application program and the configuration word are programmed into a PIC chip using a serial Protocol.

What is a Parallel Slave Port?

The Parallel Slave port facilitates the design of PC Interface circuitry by presenting a 8 bit bus interface to the PC.

What are the differences between RISC and CISC?

RISC is faster, controlled by Hardwired control, complicated circuit.

CISC is slower, controlled by Microprogrammed control, easy to develop.

Unit III

What are the different addressing modes in 68HC11?

- Register Indirect mode

- Direct Mode

- Indexed Addressing mode

- Indexed addressing mode with auto increment

What are the different modes of operation of 68HC11 microcontroller?

- Bootstrap mode

- Expanded mode

- Single chip mode

What s busy waiting?

What is DMA and Polling as applicable to 68HC11?

What are interrupts?

What is Interrupt Service Routine?

Write the principles of the Input Capture.

Write the concept of Output Compare.

Write short notes of frequency measurement.

Write short notes on RS 232.

Write short notes on RS485.

Unit IV and Unit V

Define Assembler and a Compiler

A System software that converts an Assembly Language Program into object code is called an Assembler.

A system software that converts a High level language Program in to object code is called a Compiler.

What is Cross Assembler?

An assembler that runs on one computer but creates object code for a different computer.

What is Cross compiler?

A compiler that runs on one computer but creates object code for a different computer.

What is functional Debugging?

The process of detection, locating or correcting functional and logical errors in a program and the process of instrumenting a program for such purposes is called Functional debugging.

Define Instruction Set Simulator.

A software tool that runs on host and simulates the behavior of the microprocessor and memory in the target system.

What are the task states in Real Time Operating Systems?

The tasks can be either in any of the three states

- Running

- Ready

- Blocked

Define Semaphores and write the types of semaphores.

A System function with two operations (wait and signal) that provide for task synchronization and resource sharing.

The different types are

- Counting Semaphores

- Binary Semaphore

- Resource Semaphore

- Mutex

What are the different Software Architectures for Embedded Architecture?

  - Round Robin

  - Round Robin with Interrupts

  - Function Queue Scheduling

  - Real Time Operating Systems

What are Hard Real Time Systems and Soft Real Time Systems?

A Hard Real Time System is a System in which the Deadlines has to be met, if the deadline are not met, then the system will crash.

A Soft Real Time System is a system in which the deadlines even missed only the performance will degrade.

What is a linker?

A linker combines separately compiled and assembled modules onto an executable image.  It also places code, data startup code, constant strings and so on at suitable address in ROM and RAM.

What are the different ways in which the software is loaded in to the target system for Testing?

- by PROM Programmer

- ROM Emulator

- In Circuit Emulator

- Flash Memory

- Software Monitors

What the issues in Object Oriented Software Programming?

- Encapsulation

- Inheritance

- Polymorphism

For Embedded System Encapsulation is much more important than Inheritance and polymorphism.

What is Performance debugging or profiling?

The process of acquiring or modifying timing characteristics and execution patters of a program and the process of instrumenting a program for such purpose is called performance debugging.

Define Embedded Computer System.

A System that performs a specific dedicated operation where the computer is hidden or embedded inside the machine.

What are the various methods of intertask communication?

- Semaphore

- Mailboxes

- Message queue

- Pipes

- Events

What is Recursion?

A Recursive program is one that call itself and the process of doing this is called recursion.

What is profiling?

The process of acquiring or modifying timing characteristics and execution patters of a program and the process of instrumenting a program for such purpose is called profiling.

What is a locator?

A locator combines separately compiled and assembled modules onto an executable image.  It also places code, data startup code, constant strings and so on at suitable address in ROM and RAM.

Part B

Unit I

Describe the Shared Data Problem and how to solve the shared data problem. Write pseudo codes wherever necessary.

Page 92, David E Simon, “An embedded software primer”

Describe the different ways for implementing Direct Memory access as related to any microprocessor. Explain with the help of Timing Diagram.

Page 57, David E Simon, “An embedded software primer”,

(a)Write short notes on Interrupt Latency.

Page 103 David E Simon, “An embedded software primer”,

(b) Describe the various memory used in Embedded systems. List their Advantages and disadvantages.

Page 33, David E Simon, “An embedded software primer”,

Unit IV

Describe the function Queue scheduling Architecture and Compare it with the Real Time Operating System Architecture.

Page 127,133, David E Simon, “An embedded software primer”,

Describe the Round Robin Architecture and Round Robin with Interrupt Architecture. List their advantages and Disadvantages.

Page 123, David E Simon, “An embedded software primer”

Write elaborate notes on Cross Compilers and Assemblers.

Page 262, David E Simon, “An embedded software primer”

Write Notes on Embedded Software Development tools. Draw the block diagram for a typical software developments tool for an Embedded System.

Page 263, 264, David E Simon, “An embedded software primer”

(a) Write short notes on Intel Hex File format

Page 268, David E Simon, “An embedded software primer”

(b) Describe linkers for an Embedded Software development

Page 263,264, David E Simon, “An embedded software primer”

(a) Describe ROM Emulator and In-circuit Emulator.

Page 319, David E Simon, “An embedded software primer”

(b) What is Instruction Set Simulator and what are its abilities?

Page 302, 303, David E Simon, “An embedded software primer”

Describe Logic Analyzer.

Page 313, David E Simon, “An embedded software primer”

Unit V

Describe about Semaphores and how it is helpful in solving the shared data problem.

Page 153, David E Simon, “An embedded software primer”.

What are the various methods for intertask communication and describe about the message Queue.

Page 173, David E Simon, “An embedded software primer”.

Describe the rules that the interrupt routines in Real Time Operating System must follow?

Page 199, David E Simon, “An embedded software primer”.

Define the encapsulation Mechanism applicable to Semaphore and Queues.

Page 244, 247, David E Simon, “An embedded software primer”.

How the tasks getting schedules in an RTOS Environment and how a scheduler is helpful in scheduling tasks.

Page 140, David E Simon, “An embedded software primer”.

Describe the timer functions and what are the services provided by the timer in Embedded system.

Page 184, David E Simon, “An embedded software primer”

Write about the memory management subsystem as applicable to any RTOS.

Page 196, David E Simon, “An embedded software primer”,

Write short notes on the factors that are useful to decide an RTOS for an Embedded System.

Page 237, David E Simon, “An embedded software primer”

Write the scheduling considerations for a Hard Real Time System.

Page 253, David E Simon, “An embedded software primer”

What are the design considerations for an Embedded System.

P217, 224, David E Simon, “An embedded software primer”

Unit II

Describe in detail the Inter Integrated Circuit Bus (I2C) Operation.

Page 163, John B Peatman, “Design with PIC Micro controllers”

Draw the Block diagram of PIC16C74A controller and explain each part in detail.

Page 5, John B Peatman, “Design with PIC Micro controllers”

Write elaborate notes on PIC Microcontrollers and What factors accounts for the wide popularity of these controllers.

Page 2, John B Peatman, “Design with PIC Micro controllers”

Describe how the Baud Rate is selected and how Baud Rate accuracy is maintained.

Page 194, John B Peatman, “Design with PIC Micro controllers”

Describe the use of Analog to Digital converters in PIC Controllers.

Page 185, John B Peatman, “Design with PIC Micro controllers”

Describe the Capture mode and Compare mode of  PIC Controllers.

Page 103, John B Peatman, “Design with PIC Micro controllers”

Explain how the Pulse Width Modulation is generated and write about PWM output for various applications.

Page 112, John B Peatman, “Design with PIC Micro controllers”

Describe the Interrupt Constraints of a PIC Controller and Derive the Interrupt inequality.

Page 75, John B Peatman, “Design with PIC Micro controllers”

Unit III

Draw the Block diagram of Motorola 68HC11 Micro controller and explain each part in detail.

Page 26, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

Describe the Input Capture Mechanism and write its Details of 68HC11 Microcontroller.

Page 285, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

Describe the Output Compare mechanism of 68HC11 microcontroller.

Page 315, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

Describe the Frequency Measurement and how input capture and output capture helpful in determining the Frequency.

Page 333, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

Write short notes on Interrupts in 68HC11 Microcontrollers.

Page 219, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

Write about the Interrupt Vector and Priority of  68HC11 Microcontroller.

Page 225, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

Describe the Blind Cycle counting Synchronisation and Busy waiting Synchronisation methods of 68HC11 Microcontrollers.

Page 143, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

Write short notes on three different modes in 68HC11 Microcontroller.

Page 27, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

Describe in detail about the expanded mode operation of 68HC11.

Page 28, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”