IF 363 EMBEDDED ARCHITECTURE

Part A (10 * 2 = 20 marks)

1. Write any four devices that can be built on the microprocessor.

- DMA

- I/O Pins

- Timers

- Memory caches and instruction pipelines

2. Define Interrupt Latency.

Interrupt Latency refers to the amount of time it takes a system to respond to an interrupt.

3. What is Harvard Architecture?

The Program Memory and the Data memory are separated to achieve an fast execution speed for a given clock rate. Instructions are fetched from program memory and data variables are fetched from other memory.

4. What is critical region?

A critical region is sequences of instructions in a mainline code that must be protected from an intervening interrupt or suffer a possible erroneous result.

5. What is a Watch Dog timer.

The Watch Dog Timer is device which is used to reset the PIC if the chip ever malfunctions and deviates from its normal operation.

6. Define Instruction Set Simulator.

A software tool that runs on host and simulates the behavior of the microprocessor and memory in the target system.

7. What are the task states in Real Time Operating Systems?

The tasks can be either in any of the three states

- Running

- Ready

- Blocked

8. Define Semaphores and write the types of semaphores.

A System function with two operations (wait and signal) that provide for task synchronization and resource sharing.

The different types are

- Counting Semaphores

- Binary Semaphore

- Resource Semaphore

- Mutex

9. What are the different addressing modes in 68HC11?

- Register Indirect mode

- Direct Mode

- Indexed Addressing mode

- Indexed addressing mode with auto increment

10. What are the different modes of operation of 68HC11 microcontroller?

- Bootstrap mode

- Expanded mode

- Single chip mode

Part B (5 * 16 = 80 marks)

11. 	(a)Write short notes on Interrupt Latency.

Page 103 David E Simon, “An embedded software primer”,

(b) Describe the various memory used in Embedded systems. List their Advantages and disadvantages.

Page 33, David E Simon, “An embedded software primer”,

12 	Describe how the Baud Rate is selected and how Baud Rate accuracy is maintained.

Page 194, John B Peatman, “Design with PIC Micro controllers”

(Or)

Describe the use of Analog to Digital converters in PIC Controllers.

Page 185, John B Peatman, “Design with PIC Micro controllers”

13. 	Write short notes on Interrupts in 68HC11 Microcontrollers.

Page 219, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

Write about the Interrupt Vector and Priority of  68HC11 Microcontroller.

Page 225, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

(Or)

Describe the Blind Cycle counting Synchronisation and Busy waiting Synchronisation methods of 68HC11 Microcontrollers.

Page 143, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

14.	 (a) Write short notes on Intel Hex File format

Page 268, David E Simon, “An embedded software primer”

(b) Describe linkers for an Embedded Software development

Page 263,264, David E Simon, “An embedded software primer”

(Or)

(a) Describe ROM Emulator and In-circuit Emulator.

Page 319, David E Simon, “An embedded software primer”

(b) What is Instruction Set Simulator and what are its abilities?

Page 302, 303, David E Simon, “An embedded software primer”

15. 	How the tasks getting schedules in an RTOS Environment and how a scheduler is helpful in scheduling tasks.

Page 140, David E Simon, “An embedded software primer”.

(Or)

Describe the timer functions and what are the services provided by the timer in Embedded system.

Page 184, David E Simon, “An embedded software primer”