IF 363 EMBEDDED ARCHITECTURE

Part A (10 * 2 = 20 marks)

-

- What is the purpose of Decoupling capacitor?

Some part of the circuit needs a lot of power and voltage begins to fall because of

that, the capacitor will give up its stored electricity to maintain the voltage level.

- What are the ROM Variants used in Embedded Systems?

- Programmable Read Only Memory PROM

- Erasable Programmable Read Only Memory EPROM

- FLASH

- Electrically Erasable Programmable Read Only Memory EEPROM

3. Define Critical Section and Atomic action.

A Set of instructions that must not be interrupted if the system is to work properly is called Critical section.

A set of instructions that will not be interrupted is said to be atomic.

4. What is Harvard Architecture?

The Program Memory and the Data memory are separated to achieve an fast execution speed for a given clock rate. Instructions are fetched from program memory and data variables are fetched from other memory.

5. Define Pipelining.

The CPU executes each instruction during the cycle following the Fetch. Pipelining instruction fetches and instruction executions to achieve the execution of one instruction every cycle.

6. Define Assembler and a Compiler

A System software that converts an Assembly Language Program into object code is called an Assembler.

A system software that converts a High level language Program in to object code is called a Compiler.

7. What is Cross Assembler?

An assembler that runs on one computer but creates object code for a different computer.

8. Define Embedded Computer System.

A System that performs a specific dedicated operation where the computer is hidden or embedded inside the machine.

9. What are the various methods of intertask communication?

- Semaphore

- Mailboxes

- Message queue

- Events

10. What is Recursion?

A Recursive program is one that call itself and the process of doing this is called recursion.

Part B (5 * 16 = 80 marks)

11. 	Describe the different ways for implementing Direct Memory access as related to any microprocessor. Explain with the help of Timing Diagram.

Page 57, David E Simon, “An embedded software primer”

12. 	Describe in detail the Inter Integrated Circuit Bus (I2C) Operation.

Page 163, John B Peatman, “Design with PIC Micro controllers”

(Or)

Draw the Block diagram of PIC16C74A controller and explain each part in detail.

Page 5, John B Peatman, “Design with PIC Micro controllers”

13.	Draw the Block diagram of Motorola 68HC11 Micro controller and explain each part in detail.

Page 26, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

(Or)

Describe the Input Capture Mechanism and write its Details of 68HC11 Microcontroller.

Page 285, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

14. 	Describe the function Queue scheduling Architecture and Compare it with the Real Time Operating System Architecture.

Page 127,133, David E Simon, “An embedded software primer”,

(Or)

Describe the Round Robin Architecture and Round Robin with Interrupt Architecture. List their advantages and Disadvantages.

Page 123, David E Simon, “An embedded software primer”

15.	Describe about Semaphores and how it is helpful in solving the shared data problem.

Page 153, David E Simon, “An embedded software primer”.

(Or)

What are the various methods for intertask communication and describe about the message Queue.

Page 173, David E Simon, “An embedded software primer”.