IF 363 EMBEDDED ARCHITECTURE

Part A (10 * 2 = 20 marks)

1. What is bus cycle?

The microprocessor is in control of all signals, and it decides when to look for the data on the bus is called bus cycle.

2. What is Direct Memory Access (DMA)?

DMA is a circuitry that can read data from an I/O Device such as a serial port of a network and then write it into memory or read from memory and write to an I/O Device all without the software assistance and the associated overhead.

3. What are the different addressing modes in PIC Controllers?

- Direct Addressing mode

- Indirect addressing mode

4. Write short notes on the timers employed in PIC Controllers.

There are 3 timers, timer0, timer1 and timer2

Timer0 used to count external events.

Timer1 useful for signal monitoring.

Timer2 used as a loop time counter and hence used to control the period of a PWM.

5. What is a Parallel Slave Port?

The Parallel Slave port facilitates the design of PC Interface circuitry by presenting a 8 bit bus interface to the PC.

6. What are the differences between RISC and CISC?

RISC is faster, controlled by Hardwired control, complicated circuit.

CISC is slower, controlled by Micro programmed control, easy to develop.

7. What are the different ways in which the software is loaded in to the target

system for Testing?

- by PROM Programmer

- ROM Emulator

- In Circuit Emulator

- Flash Memory

- Software Monitors

8. What the issues in Object Oriented Software Programming?

- Encapsulation

- Inheritance

- Polymorphism

For Embedded System Encapsulation is much more important than Inheritance and polymorphism.

9. What is Performance debugging or profiling?

The process of acquiring or modifying timing characteristics and execution patters of a program and the process of instrumenting a program for such purpose is called performance debugging.

10. What are the different modes of operation of 68HC11 microcontroller?

- Bootstrap mode

- Expanded mode

- Single chip mode

Part B (5 * 16 = 80 marks)

11. 	Describe about Semaphores and how it is helpful in solving the shared data

problem.

Page 153, David E Simon, “An embedded software primer”.

12.	 Describe the Interrupt Constraints of a PIC Controller and Derive the Interrupt inequality.

Page 75, John B Peatman, “Design with PIC Micro controllers”

(Or)

Describe the Capture mode and Compare mode of  PIC Controllers.

Page 103, John B Peatman, “Design with PIC Micro controllers”

13. 	Draw the Block diagram of Motorola 68HC11 Micro controller and explain each part in detail.

Page 26, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing

(Or)

Describe the Blind Cycle counting Synchronisation and Busy waiting Synchronisation methods of 68HC11 Microcontrollers.

Page 143, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

14. 	Write elaborate notes on Cross Compilers and Assemblers.

Page 262, David E Simon, “An embedded software primer”

(Or)

Write Notes on Embedded Software Development tools. Draw the block diagram for a typical software developments tool for an Embedded System.

Page 263, 264, David E Simon, “An embedded software primer”

15. 	Write the scheduling considerations for a Hard Real Time System.

Page 253, David E Simon, “An embedded software primer”

(Or)

Describe the design considerations for an Embedded System.

P217, 224, David E Simon, “An embedded software primer”