IF 363 EMBEDDED ARCHITECTURE

Part A (10 * 2 = 20 marks)

1. What are the different memories used in Embedded Systems?

- Read only Memory (ROM)

- Random Access Memory (RAM)

- Cache memory

2. Define Setup time and Hold Time

The time before the rising edge of the clock during which the input remains constant is called setup time.

The time after the rising edge of the clock during which the input remains constant is called hold time.

3. Define Critical Section and Atomic action.

A Set of instructions that must not be interrupted if the system is to work properly is called Critical section.

A set of instructions that will not be interrupted is said to be atomic.

4. What are the different addressing modes in PIC Controllers?

- Direct Addressing mode

- Indirect addressing mode

5. Write short notes on the timers employed in PIC Controllers.

There are 3 timers, timer0, timer1 and timer2

Timer0 used to count external events.

Timer1 useful for signal monitoring.

Timer2 used as a looptime counter and hence used to control the period of a PWM.

6. What is Cross compiler?

A compiler that runs on one computer but creates object code for a different computer.

7. What is functional Debugging?

The process of detection, locating or correcting functional and logical errors in a program and the process of instrumenting a program for such purposes is called Functional debugging.

8. What is Recursion?

A Recursive program is one that call itself and the process of doing this is called recursion.

9. What is profiling?

The process of acquiring or modifying timing characteristics and execution patters of a program and the process of instrumenting a program for such purpose is called profiling.

Part B (5 * 16 = 80 marks)

11. 	Describe the Shared Data Problem and how to solve the shared data problem. Write pseudo codes wherever necessary.

Page 92, David E Simon, “An embedded software primer”

12. 	Draw the Block diagram of PIC16C74A controller and explain each part in detail.

Page 5, John B Peatman, “Design with PIC Micro controllers”

(Or)

Write elaborate notes on PIC Microcontrollers and What factors accounts for the wide popularity of these controllers.

Page 2, John B Peatman, “Design with PIC Micro controllers”

13. 	Describe the Output Compare mechanism of 68HC11 microcontroller.

Page 315, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

(Or)

Describe the Frequency Measurement and how input capture and output capture helpful in determining the Frequency.

Page 333, Jonarthan W. Valvano, “Embedded Micro computer Systems, Real time Interfacing”

14. 	Write elaborate notes on Cross Compilers and Assemblers.

Page 262, David E Simon, “An embedded software primer”

(Or)

Write Notes on Embedded Software Development tools. Draw the block diagram for a typical software developments tool for an Embedded System.

Page 263, 264, David E Simon, “An embedded software primer”

15. 	Describe the rules that the interrupt routines in Real Time Operating System must follow?

Page 199, David E Simon, “An embedded software primer”.

(Or)

Define the encapsulation Mechanism applicable to Semaphore and Queues.

Page 244, 247, David E Simon, “An embedded software primer”.