# First generation technology

First generation systems are analog and were designed for voice transfer. AMPS, NMT, TACS, etc are included among first generation systems. With the recent proliferation of PDC/PHS in Japan, post-analog technology has advanced rapidly and consequently only a very few analog systems remain in existence.

# Second generation technology

Second generation systems are digital and are capable of providing voice/data/fax transfer as well as a range of other value-added services. At present, second generation systems are still evolving with ever-increasing data rates via new technologies such as HSCSD (High Speed Circuit Switched Data) and GPRS (General Packet Radio Service). Second generation systems include GSM, US-TDMA (IS_136), cdmaOne _IS-95) and PDC. US-TDMA/PDC has been structured atop existing first generation analog technology and is premised on compatibility and parallel operation with analog networks. GSM/IS-95, on the other hand, is based on an entirely new concept, and has been subject to increasing adoption worldwide.

# Third generation technology

Third generation mobile communication systems are scheduled for operational startup in Japan and Europe in 2001-2002. Applying high-speed data transfer and state-of-the-art radio terminal technology, third generations systems enable multimedia and are currently in the process of being standardized under 3GPP. Among the three types of system to be standardized (i.e. WCDMA-DS, MC-CDMA, UTRA TDD), Japan and Europe will adopt WCDMA-DS in a strategy to take the lead through superior service.

# Harmonization of 3g

Harmonizing of UTRA and CDMA2000 (third generation system in the US). At present, there are three coinciding technologies under third generation systems: DS-CDMA (UTRA FDD), MC-CDMA and UTRA TDD. Current chip rate for both UTRA FDD and TDD is 3.84Mchips. MC-CDMA adopts the previous 3.68Mchips as is. As a result, transition from the existing 2G to 3G will be facilitated on a worldwide level. Despite the fact that it will not comprise one uniform system, convergence of 3G into three systems with maximum compatibility will still enable achieving the primary 3G goal of service to users over a seamless worldwide network

# What is 3-g?

- 3G is watching clips from your favorite soap in the train.

- 3G is sending images straight from the field to headquarters for analysis.

- 3G is sharing your Moroccan vacation with your friends--from Morocco.

- 3G is videoconferencing in a taxi.

3G is the Future of Everyday Communications.

3G is a global development of communication standards and technologies. This development will lead to your being able to access multimedia services with your mobile phone.

You may have seen terms in the news like GPRS, EDGE, WCDMA, and UMTS. All of these are concepts related to development of mobile communication networks to 3G.

Although the technology behind 3G may seem complicated, the ways in which 3G will affect all of our lives are easy to imagine. Just imagine having a combined camera, video camera, computer, stereo, and radio included in your mobile phone. Rich-media information and entertainment will be at your fingertips whenever you want anywhere there is a wireless network.

Mobile communication is moving from simple voice to rich media, where we use more of our senses to intensify our experiences.

But not all of this will happen at once. 3G is an evolution to a communications ideal that we cannot see the end of yet. We do know that mobile multimedia will hit the Japanese markets in 2001, and Europe and North America will follow soon after.

We also know that, regardless of whether you find new technologies intimidating or fascinating, 3G will bring a host of mobile services that will bring more information, more efficiency, and more joy to your life.

# Blue tooth and nokia

Bluetooth is a global de facto standard for wireless connectivity. Based on a low-cost, short-range radio link, Bluetooth cuts the cords that used to tie up digital devices.

When two Bluetooth equipped devices come within 10 meters range of each other, they can establish a connection together. And because Bluetooth utilizes a radio-based link, it doesn't require a line-of-sight connection in order to communicate. Your laptop could send information to a printer in the next room, or your microwave could send a message to your mobile phone telling you that your meal is ready.

In the future, Bluetooth is likely to be standard in tens of millions of mobile phones, PCs, laptops and a whole range of other electronic devices. As a result, the market is going to demand new innovative applications, value-added services, end-to-end solutions and much more. The possibilities opened up really are limitless, and because the radio frequency used is globally available, Bluetooth can offer fast and secure access to wireless connectivity all over the world. With potential like that, it's no wonder that Bluetooth is set to become the fastest adopted technology in history.

Figure 1. Roadmap to Third Generation

A new air interface is needed for 3G. This will require new Base Station Systems (BSSs). Specifically, the BSS changes needed are: The 3G radio access network will comprise a RNC (Radio Network Controller) and Node B.

RADIO NETWORK CONTROLLER

A Radio Network Controller (RNC) will replace the Base Station Controller. The RNC will include support for connection to legacy systems and provide efficient packet connection with the core network packet devices (SSGN or equivalent). The RNC performs radio network control functions that include call establishment and release, handover, radio resource management, power control, diversity combining and soft handover.

NODE B

A Node B is equivalent to a Base Station in the 2G network but also incorporates support for the 3G air interfaces.

CELL PLANNING

New cell planning methods will be needed to support the new frequency allocations for 3G and the radio interface changes- more 3G base stations will be needed compared to the comparable 2G coverage area. This gives an advantage to GSM 1800 and 1900 network operators whose cells already cover a smaller coverage area than those for GSM 900 networks. GSM 900 network operators will need to "fill in" coverage in between existing cell sites.

CORE NETWORK

The 3G core network will be an evolution from GPRS or equivalent 2.5G core network systems. GPRS nodes such as the Serving GPRS Support Node (SGSN) and Gateway GPRS Support Node (GGSN) are described in detail in "Data on GPRS" from Mobile Lifestreams. Upgrades to the mobile and transit switching systems to deliver packets will also be needed.

A new piece of network infrastructure for 3G is Media Gateways (MGW) that resides at the boundary between different networks to process end user data such as voice coding and decoding, convert protocols and map quality of service. The connectivity layer also provides access to backbone switches and non-mobile networks such as Cable Television. In some vendor solutions, MGWs are controlled remotely by the Mobile Switching Centre (MSC) and GSN servers by means of the Gateway Control Protocol. The ITU Study Group 16 and the IETF Megaco H.248 are working to ensure the GCP is an open standard protocol.

Existing network operators can then upgrade their Mobile Switching Centre (MSC) and GSNs to implement 3G OR ALTERNATIVELY to implement a new standalone MGW that is controlled from the server part of an upgraded 2G node.

BACKBONE NETWORK

The radio network will be connected to the core network by a backbone network allowing wideband access and interconnection of subscribers. The 3G backbone network can use any transport technology but is certain to be based on packet technologies such as Asynchronous Transfer Mode (ATM) and Internet Protocol (IP). The backbone network is built as a mesh of IP routing or ATM switching nodes interconnected by point to point links. Technologies such as IP over ATM may be used that uses ATM switching to multiplex IP traffic. This IP over ATM architecture supports voice traffic alongside IP. Many vendors prefer a "pure" end to end IP approach whereas others (such as Fujitsu profiled below) prefer an ATM/ IP hybrid to guarantee quality of service.

Alternatively, IP over SONET/ SDH is a different backbone network solution that eliminates the ATM layer by establishing point to point links between IP routers directly over SONET/ SDH rings which run over a Dense Wavelength Division Multiplexing (DWDM) layer that enables Terabits per second (Tbits/s) of aggregate network bandwidth

Serving GPRS Support Node (SGSN)

An SGSN delivers packets to mobile stations within its service area. SGSNs send queries to Home Location Registers (HLRs) to obtain profile data of GPRS subscribers. SGSNs detect new GPRS mobile stations in a given service area; and, finally, SGSNs process registration of new mobile subscribers and keep a record of their location inside a given service area.

Gateway GPRS Support Node (GGSN)

GGSNs are used as interfaces to external PDNs. GGSNs maintain routing information that is necessary to tunnel the Protocol Data Units (PDUs) to the SGSNs that service particular mobile stations. Other functions include network and subscriber screening and address mapping. One or more GGSNs may support multiple SGSNs.

Technique:-

Once a mobile station begins data transmission, routing is performed by the GSNs on a hop-by-hop basis through the mobile network using the destination address in the message header. Routing tables are maintained by the GSNs utilizing the GTP layer which may carry out Address Translation and Mapping functions to convert the external PDN addresses to an address that is usable for routing within PLMNs. The data itself will go through several transformations as it travels through the network. Depending on the destination PDN, the data can be:

Forwarded, using the relay function, to go from one node to the other in the route,

Tunneled to transfer data from one PLMN to another,

Compressed to use the radio path in an efficient manner (Compression algorithms may be used for manufacturers to differentiate themselves, however, they may face interoperability issues in heterogeneous networks), and/or

Encrypted to protect the mobile station from eavesdropping (Encryption algorithms can also be used as a differentiating factor).

3.3 Mobility Management

As a mobile station moves from one area to another, mobility management functions are used to track its location within each PLMN. Then, SGSNs communicate with each other and update the user location. The mobile station's profiles are preserved in the VLRs that are accessible to SGSNs via the local MSC. A logical link is established and maintained between the mobile station and the SGSN at each PLMN. At the end of transmission or when a mobile station moves out of the area of a specific SGSN, the logical link is released and the resources associated with it can be reallocated.