Introduction: -

The first generation of mobile phones consisted of the analog models that emerged in the early 1980s. The second generation of digital mobile phones appeared about ten years later along with the first digital mobile networks. During the second generation, the mobile telecommunications industry experienced exponential growth both in terms of subscribers as well as new types of value-added services. The current second-generation (2G) system provides circuit-switched voice and data services. The 2.5G system will provide low-rate packet-switched data service of up to 64 Kbps. However, voice and low-rate data services are insufficient in a world where high-speed Internet access is taken for granted. The trend is toward global information networks that offer flexible multimedia information services to users on demand-anywhere, and at any time. In order to support the new requirements, the mobile communication system must evolve to 3G. 3G will provide a higher data rate of up to 2 Mbps, while making more efficient bandwidth use.

Mobile phones are rapidly becoming the preferred means of personal communication, creating the world's largest consumer electronics industry. The rapid and efficient deployment of new wireless data and Internet services has emerged as a critical priority for communications equipment manufacturers. Network components that enable wireless data services are fundamental to the next-generation network infrastructure.

This paper presents an overview of the evolving THIRD GENERATION SYSTEM OF MOBILE COMMUNICATIONS (3G). This paper examines the industry plans to implement 3G technology to address the growing demand for wireless multimedia services. This paper also discusses how people will prefer 3G services for web browsing, e-commerce and other high-speed data applications. Alternately, this paper also discusses how third generation of mobile communications will greatly enhance the implementation of sophisticated wireless applications. Users will be able to utilize personal, location-based wireless information and interactive services. Also, many companies and corporations are restructuring their business processes to be able to fully exploit the opportunities provided by the emerging new wireless data services. Many advanced wireless services are already available today, and the introduction of 3G wireless technologies will add to their ubiquity.

The technological EVOLUTION: -

# First generation technology

First generation systems are analog and were designed for voice transfer. AMPS, NMT, TACS, etc are included among first generation systems. With the recent proliferation of PDC/PHS in Japan, post-analog technology has advanced rapidly and consequently only a very few analog systems remain in existence.

# Second generation technology

Second generation systems are digital and are capable of providing voice/data/fax transfer as well as a range of other value-added services. At present, second generation systems are still evolving with ever-increasing data rates via new technologies such as HSCSD (High Speed Circuit Switched Data), GPRS (General Packet Radio Service) and EDGE (Enhanced Data Rates for Global Evolution). Second generation systems include GSM, US-TDMA (IS_136), cdma One _IS-95) and PDC (Personal Digital Communication).

# Third generation technology

3G wireless technology represents the convergence of various 2G wireless telecommunications systems into a single global system that includes both terrestrial and satellite components. One of the most important aspects of 3G wireless technologies is its ability to unify existing cellular standards, such as CDMA, GSM, and TDMA, under one umbrella. The following three air interface modes accomplish this result: wideband CDMA, CDMA2000 and the Universal Wireless Communication (UWC-136) interfaces.

In general, a 3G phone should permit global roaming and have packet-based rather than circuit-switched data-transmission capability. The data rate should be at least 144 kbits/s from a moving vehicle, 384 kbits/s at pedestrian speed, and 2 Mbits/s from a fixed location. This makes such applications as e-mail, Internet access, and even video transmissions possible.

The 3G network architecture:-

The third generation (3G) mobile communication system can be divided into three segments:-

- A 3G Mobile Handset.

- A Radio Access Network that performs air-interface related functions

- A Core Network that performs switching functions and interfaces to external networks such as the Internet or a public-switched telephone network (PSTN).

The evolution to third generation (3G) technology is taking place in both the radio access network and the core network. For 3G, the new air interface standards include wideband-code division multiple access (w-cdma) and cdma2000-1X. The corresponding wireless networks are universal mobile telephone systems     (UMTS) and cdma2000.

(A) 3G Mobile Handset:-

For 3G phones, the most important components are the processors. All 3G cell phones will contain at least two processors, the embedded controller and a DSP chip. The controller takes care of the display, keyboard, and other usual housekeeping duties. Additionally, it deals with some data-handling functions, such as the protocol stack in certain phones. The GPRS, EDGE, 1XRTT, and other data protocols are managed by the controller too. As more data functions are added, the controller is expected to contribute more to the processing load.

The most widely used embedded controller in cell phones is ARM's popular 32-bit RISC ARM7 and ARM9 cores. Virtually every cell-phone manufacturer has adopted it as the controller standard. Even Motorola, which has used its own proprietary MCore/Dragonball processors, has now licensed ARM for future designs.

As for DSP chips, they're primarily used for baseband functions related to the air interface. A DSP handles baseband encoding and decoding, compression, decompression, filtering, and a variety of other processing chores that are unique to the particular standard. In some software-defined radiophones, the DSP may also handle IF filtering, downconversion, demodulation, and modulation functions.

Most major handset manufacturers, like Nokia and Ericsson, have adopted TI's DSP chips. Dennis Barrett, TI's C5000 platform manager, says that the company's C5000 chips are in over 66% of all digital cell handsets shipped today.

RAN

RAN

THE 3G NETWORK ARCHITECTURE

(B) Radio Access Network :-

The radio access network is present at the center of each cell of a cellular mobile system and it connects the mobile handsets with the core network, which is a mobile switching centre(MSC).

The Radio Access Network consists of two network elements, known as Node B and Radio Network Controllers (RNC).Node B is a base transceiver station which interfaces with the mobile handsets through an air-interface (w-cdma and cdma 2000-1x) and provides wireless voice and data connectivity to mobile handsets.

Radio Network Controllers (RNC) act as Base Station Controllers (BSC) which route the data calls to the packet-switched public land mobile network (PLMN) and the voice calls to the Mobile Switching Center (MSC) over an ATM.

The radio network is connected to the core network by a backbone network allowing wideband access and interconnection of subscribers. The 3G-backbone network uses packet technologies such as Asynchronous Transfer Mode (ATM) and Internet Protocol (IP). Data services run from the terminal device (RNC) over Internet protocol (IP) which in turn uses ATM as a reliable transport with quality of service (QoS). The backbone network is built as a mesh of IP routing or ATM switching nodes interconnected by point-to-point links.

(C) Core Network :-

The core network consists of a circuit-switched domain which includes 3G MSC for switching of voice calls and packet-switched domain, i.e. public land mobile network(PLMN), which includes 3G SGSNs and GGSNs. Charging for services and access is done through the Charging Gateway Function (CGF), which is also a part of the core network.

(i) Mobile switching center (MSC):-

The mobile switching center is responsible for setting up, managing, and                             clearing connections as well as routing calls to proper cell. It is a complete telephone exchange with capabilities of relaying calls between the fixed public switched telephone network (PSTN) and cellular network. MSC also houses databases like home location register (HLR) and visitor location register (VLR). These databases store permanent and temporary information about each 3G subscriber that belongs to an area controlled by the MSC.

HLR provides identifying information on user its international mobile subscriber identity, home subscription base and any services provided to the user. HLR also keeps information regarding the VLR in which the subscriber is registered.

VLR stores information about users in its particular area. It contains information on whether mobile handsets are switched on or off and if any supplementary services have been activated or deactivated.

HLR is the primary entity used when a call originates from a public telephone network. The VLR is primary entity used when a call originates from a mobile handset.

The MSC also consists of AuC (authentication center), which protects each user from unauthorized access or from use of subscription from unauthorized personnel, and EIR(equipment identity register), for registration of type of equipment that exists in mobile handset. EIR also provides security features such as blocking calls that have been determined to emanate from stolen mobile stations, as well as preventing certain station s form using the network that have not been approved by network vendor.

(ii) Public land mobile network(PLMN) :-

The PLMN consists of packet-switched technologies such as 3G SGSNs (serving GPRS support nodes) and 3G GGSNs (gateway GPRS support nodes).

Serving GPRS Support Node (SGSN):-

An SGSN delivers packets to mobile stations within its service area. SGSNs send queries to Home Location Registers (HLRs) to obtain profile data of 3G subscribers. SGSNs detect new 3G mobile stations in a given service area; and, finally, SGSNs process registration of new mobile subscribers and keep a record of their location inside a given service area.

Gateway GPRS Support Node (GGSN):-

GGSNs are used as interfaces to external PDNs (packet data networks) such as an internet backbone or X.25 network. GGSNs maintain routing information that is necessary to tunnel the Protocol Data Units (PDUs) to the SGSNs that service particular mobile stations. Other functions include network and subscriber screening and address mapping. One or more GGSNs may support multiple SGSNs.

A new piece of network infrastructure for 3G is Media Gateways (MGW) that resides at the boundary between different networks to process end user data such as voice coding and decoding, convert protocols and map quality of service. The connectivity layer also provides access to backbone switches and non-mobile networks such as Cable Television. In some vendor solutions, MGWs are controlled remotely by the Mobile Switching Centre (MSC) and GSN servers by means of the Gateway Control Protocol.

Mobility Management

As a mobile station moves from one area to another, mobility management functions are used to track its location within each PLMN. Then, SGSNs communicate with each other and update the user location. The mobile station's profiles are preserved in the VLRs that are accessible to SGSNs via the local MSC. A logical link is established and maintained between the mobile station and the SGSN at each PLMN. At the end of transmission or when a mobile station moves out of the area of a specific SGSN, the logical link is released and the resources associated with it can be reallocated.

3G protocol layers :-

The following is a brief description of each protocol layer in a 3G wireless network infrastructure:-

§ Global Mobility Management (GMM): protocol that includes attach, detach, security, and routing area update functionality.

§ Node B Application Part (NBAP): provides procedures for paging distribution, broadcast system information and management of dedicated and logical resources.

§ Packet Data Convergence Protocol (PDCP): maps higher level characteristics onto the characteristics of the underlying radio-interface protocols. PDCP also provides protocol transparency for higher layer protocols.

§ Radio Link Control (RLC): provides a logical link control over the radio interface.

§ Medium Access Control (MAC): controls the access signaling (request and grant) procedures for the radio channel.

§ Radio resource Control (RRC): manages the allocation and maintenance of radio communication paths.

§ Radio Access Network Application Protocol (RANAP): encapsulates higher layer signaling. Manages the signaling and GTP connections between RNC and 3G-SGSN, and signaling and circuit-switched connections between RNC and 3G MSC.

§ Radio Network Service Application Part (RNSAP): provides the communication between RNCs.

§ GPRS Tunnel Protocol (GTP): protocol that tunnels the protocol data units through the IP backbone by adding routing information. GTP operates on top of TCP/UDP over IP.

§ Mobile Application Part (MAP): supports signaling between SGSN/GGSN and HLR/AuC/EIR.

§ AAL2 Signaling (Q.2630.1, Q.2150.1, Q.2150.2, AAL2 SSSAR, and AAL2 CPS): protocols suite used to transfer voice over ATM backbone using ATM adaptation layer 2.

§ Sigtran (SCTP, M3UA): protocols suite used to transfer SCN signaling protocols over IP network.

Systems based on 3G:-

Various 3G based systems have been started evolving which are compatible with the current standards of GSM.some of these are Wideband CDMA (W-CDMA), CDMA2000, universal mobile telephone systems (UMTS), etc.

CDMA2000:

cdma2000 is a third-generation (3G) wireless system, which is based on the cdmaOne system. The cdma2000 system delivers high-bandwidth data and voice services to users of mobile equipment. CDMA2000 is backward compatible with the second generation CDMA IS-95 standard predominantly used in US. Figure below shows the infrastructure of a cdma2000 wireless network.

cdma 2000 infrastructure nodes:-

BTS: base transceiver station                           PDSN: packet data serving node

BSC: base station controller                              HA: home agent

MSC: mobile switching center                          IWF: interworking function

Universal mobile telephone systems (UMTS):

The universal mobile telecommunication system (UMTS) is a third-generation (3G) wireless system that delivers high-bandwidth data and voice services to mobile users. UMTS is the European third generation mobile system developed by ETSI. It will have at least two IMT2000 family members, based on Broadband ISDN and Global System for Mobile Communications (GSM) evolution.

UMTS has a new air interface based on W-CDMA and a core network based on general-packet radio service (GPRS). Figure below shows the infrastructure of a UMTS wireless network.

| Voice and data transport is performed by the transport layer nodes, colored blue: - Node B: Base transceiver station (BTS) - RNC: Radio network controller or base station controller (BSC) - SGSN: Serving GPRS support node - GGSN: Gateway GPRS support node | The call control function is mainly performed by the call control layer nodes, colored yellow: - CSCF: Call state control function - MGCF: Media gateway controller function - HSS: Home subscriber server |
| --- | --- |

3G  standardization and spectrum :-

International Mobile Telecommunications-2000 (IMT-2000) is the basic global 3G standard being developed by the International Telecommunications Union (ITU). IMT-2000, an evolution of 2G systems, proposes a family of 3G systems interoperating through standard interfaces. The European Union’s IMT-2000 family member is known as Universal Mobile Telecommunications System (UMTS). It is being developed under the Third Generations Partnership Project (3GPP) whose membership includes the standards organizations of Europe (ETSI), the United States (T1), Japan (TTC and ARIB), and Korea (TTA).

IMT-2000 family members will have unique radio interfaces. Though many radio interface technologies have been submitted to the ITU for IMT-2000 membership, three radio technologies have emerged as most prominent. These are:

1) The Wideband Code Division Multiple Access (W-CDMA) in Frequency

Division Duplex (FDD) and Time Division Duplex (TDD) modes, the radio

technologies used by the UMTS family

2) CDMA-2000, used by the CDMA and IS-41-based 3G family; and

3) 136-HS, used by the TDMA and IS-41-based 3G family.

W-CDMA uses channels of 5, 10, 15 or 20 MHz bandwidth located around 2

GHz, which allows for faster data rates and more users.

BLUETOOTH as a 3G enabler :-.

Bluetooth is a de-facto open standard for short range digital radio. It is designed to operate in the unlicensed ISM (Industrial, Scientific, Medical applications) band, which is generally available in most parts of the world. Bluetooth has the potential for impacting many areas, including applications that would have been inconceivable a few years ago e.g. a fridge-freezer telling a microwave oven what ingredients are available, allowing the microwave to suggest menu options! However, one particular area where Bluetooth will have a significant impact is in the support of other wireless delivery mechanisms such as cellular telephony. While national networks are suited to delivering communication on the move or wireless to any location, purely local interconnection is better handled by a local communication system such as bluetooth. A Bluetooth enabled Home Base Station would provide the interconnection for the local Bluetooth equipped terminals and a 3G terminal connection.

When an external connection is required only two local handsets can participate in voice connections. The Home Base Station forms a gateway between the home environment and national networks and services. Any two handsets can connect to each other of course without the Home Base Station unit, making the whole arrangement very flexible.

This example highlights the complementary functionality of Bluetooth and 3G cellular systems. The 3G system is used to deliver a “trunk” connection to a specific location and Bluetooth is used as final delivery and local network connection. This will considerably reduce the “unnecessary” traffic on the 3G network, creating a cost-effective solution for the convergence of fixed and mobile services and as a by-product, keep RF interference, to a minimum.

The following examples demonstrate how 3G and Bluetooth could work together, providing local intercommunication as well as wide area connectivity in a wide range of applications.

(i) Vending machines in shopping mall

All the automatic vending machines within a confined area can, through a                     Bluetooth access system, be connected to a central vending machine administration unit, that in turn uses a 3G access system to call for maintenance or supplies.

(ii) Email delivery to PC:-

A Bluetooth/3G terminal can receive e-mail as a data transmission and forward it, via Bluetooth to the PC (assuming it is within close proximity). When the reception is complete, the PC can notify the user via Bluetooth and a short message to his mobile terminal that he has e-mail, and if an item is urgent, this fact can be forwarded too. This concept allows the 3G terminal to be the local “headend” for a variety of applications that are locally interconnected via Bluetooth.

(iii) The Underground Train

Underground facilities suffer from “poor” coverage on cellular systems. Many underground rail operators are overcoming this by installing systems designed to provide driver and station staff with a reliable communication network. Systems such as TETRA (TErrestrial Trunked RAdio) provide sufficient capacity that there is spare to carry some passenger traffic too. Carriages equipped with Bluetooth transceivers would provide a gateway between the train TETRA system and the user’s 3G/Bluetooth terminal, and the TETRA system would provide the gateway to the surface public networks.