CDMA – THE FUTURISTIC MULTIPLE ACCESS TECHNIQUE

CDMA

Prepared by:

Ajay Desai

Preeti Badiyani

K.J. Somaiya College of Engineering

Multiple access :

Multiple access is a type of multiuser communication system in which a large number of users share a common channel to transmit information to receiver. In general, there are several different ways in which multiple users can send information through the communication channel to the receiver.

One simple method is to subdivide the available channel bandwidth into a number , say k , of frequency non overlapping sub-channels, as shown in the figure 1 , and to assign a sub-channel to each user upon request by the users . This method is called frequency division multiple access (FDMA).

Another method for creating multiple sub-channels for multiple access is to subdivide the frame duration Tf, into, say, k non-overlapping subintervals, each of duration Tf / k.

Then each user who wishes to transmit information is assigned to a particular time slot within each frame. This multiple access method is called time division multiple access (TDMA).

Why CDMA ?

We observe that in FDMA & TDMA , the channel is basically partitioned into independent single-user sub-channels. Here, problems arise when the data from users accessing the network is bursty in nature.

In other words, the information transmission from a single user is separated by periods of no transmission, where these periods of silence may be greater than the periods of transmission.

This is often the case in mobile cellular communication systems carrying digitized voice, since speech signals typically contain long pauses.

In such an environment where the transmission from the various users is bursty in nature and low-duty-cycle, FDMA and TDMA tend to be inefficient because a certain percentage of the available frequency slots or time slots assigned to users do not carry information. Ultimately, an inefficiently designed multiple access system limits the no. of simultaneous users of the channel.

An alternative to FDMA & TDMA is to allow more users to share a channel or sub-channel i.e.  the CDMA technique.

CODE DIVISION MULTIPLE ACCESS (CDMA) is a spread spectrum technology, which means that it spreads the information contained in a particular signal of interest over a much greater bandwidth than the original signal; the advantage is that the system becomes very bandwidth efficient.

SPREAD SPECTRUM:

Spread spectrum is a communication technique wherein the transmitted modulated signal is spread (increased) in bandwidth prior to transmission over the channel and then despread (decreased) in bandwidth by the same amount at the receiver. The transmitted sequence occupies a much more bandwidth than the coherent channel bandwidth i.e.minimum required bandwidth. The spectrum spreading at the transmitter and the receiver is obtained by special code which is independent of the data sequence (message signal). While this system is very bandwidth inefficient for a single user, the advantage of spread spectrum is that many users can simultaneously use the same bandwidth without significantly interfering with one other.

There are two main types of SPREAD SPECTRUM MULTIPLE ACCESS (SSMA):

(1) FREQUENCY   HOPPED MULTIPLE ACCESS (FHMA)

(2) DIRECT SEQUENCE MULTIPLE ACCESS (DSMA).

In both cases synchronization of transmitter and receiver is required. Both forms can be regarded as using a pseudo-random carrier, but they create that carrier in different ways.

(1) FREQUENCY HOPPING involves a rapid change in transmission frequency. A frequency hopping signal may be regarded as a sequence of modulated data bursts with time varying pseudo random carrier frequencies. This is typically accomplished by rapid switching of fast-settling frequency synthesizers in a pseudo-random pattern. Because of this , it is difficult to predict the system frequency at any given instant of time. Hence it is commercially not feasible.

(2) DIRECT SEQUENCE MULTIPLE ACCESS is also called CODE DIVISION MULTIPLE ACCESS (CDMA) which is the focus of this paper.

GENERATION OF PSEUDO-NOISE (PN) SEQUENCE:

A pseudo-noise (PN) or pseudorandom sequence is a binary sequence, high frequency signal. Although it is deterministic, a pseudonoise sequence has many characteristics that are similar to those of random binary sequences like:

(a) Having equal number of 1’s and 0’s,

(b) Very low correlation between shifted versions of the sequences, etc.

The PN sequence is usually generated using sequential logic circuits. A feedback shift register, as shown in the figure below, consists of consecutive stages of two state memory devices and feedback logic. Binary sequences are shifted through the shift registers in response to clock pulses, and the output of the various stages are logically combined and fed back as input to the first stage. The initial contents of the memory stages and the feedback logic circuit determine the successive contents of memory.

FEEDBACK LOGIC

STAGE

2

STAGE

M

STAGE

3

STAGE

1

Also the initial content of the memory devices cannot be 0, because, then the linear shift register will always be in zero state.

A typical PN code generator employing an EX-OR gate in the feedback logic is called as a linear PN sequence generator. It consists of three flip-flops as memory devices.

Let the initial states of the flip-flops be 100 i. e. s1s2s3 = 100.  A sequence of 2m – 1 bits is generated by the above circuit, where m –represents the number of flip-flops or memory devices used.

The code generation operation can be represented in the table below:

| STATE OF THE FLIP-FLOP |  |  | FEEDBACK LOGIC | OUTPUT PN SEQUENCE EQUAL TO S3 |
| --- | --- | --- | --- | --- |
| S1 | S2 | S3 | S3 + S1 |  |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 | 1 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 |
|  |  |  |  | And the sequence repeats |

A single pulse or symbol of the PN waveform is called a chip. The time duration of each chip (Tc) is an integral multiple of the bit interval (Tb) of the data sequence.

i. e. Tb = n Tc.

CDMA TECHNICAL OPERATION:

CDMA TRNSMITTER:

The basic block diagram of CDMA transmitter is as follows:

I/P

b(t)

s1(t)

c(t)

A CDMA transmitter involves the following basic operation:

1) Channel Encoding.

2) First stage of modulation.

3) Second stage of modulation.

- CHANNEL ENCODING:

First the binary information sequence is input to the channel encoder. The channel encoder encodes this input sequence according to some error control coding technique. Practically bipolar-NRZ (non-return-to-zero) is used. Let the data sequence be represented by bk and bipolar-NRZ waveform by b(t).

When                                  bk = 1,            b(t) = +1,

And                                     bk = 0,            b(t) = -1.

(2) FIRST STAGE OF MODULATION:

Here the baseband data is spread by multiplying the baseband data pulses with   a pseudo-noise sequence which is produced by a pseudo-noise (PN) code generator. Let ck denote the PN sequence. Here c(t) represents the bipolar-NRZ signal generated by ck .

When                                 ck = 1,            c(t) = +1,

And                                   ck = 0,            c(t) = -1.

WAVEFORMS:

Consider two bits of data sequence (1, 0) in bipolar-NRZ form. Let the PN sequence code generated be 0011101. The spread spectrum CDMA signal is obtained by multiplying the two waveforms as shown below:

DATA SIGNAL

PN SEQUENCE

SPREAD SPECTRUM CDMA SIGNAL

(3) Second stage of modulation:

The signal m(t) at the output of the modulator is given as modulating signal to BPSK modulator.

The transmitted signal is represented as:

s1(t) = 2Ps b(t) c(t) cos (2 ii fc t + 0)

Where,          Ps = power of the signal

fc = carrier frequency

0 = carrier phase angle at t = 0.

CDMA RECEIVER:

The block diagram of CDMA receiver is as follows:

The following are the stages that the received signal undergoes at the receiver :-

(1) The received signal is first passed through a intermediate frequency (IF) wideband (WB) filter. This               rejects all frequencies other than those lying within the spectrum of the transmitted data.

(2) Then this bandlimited signal is multiplied with the local PN code sequence c (t). If     c (t) = +1 then c2 (t) = 1, and this multiplication yields the despread signal s1 (t) given by,

s1(t) = 2Ps b(t) cos (2 ii fc t + 0)

This signal s1 (t) is input to the demodulator.

(3) Since s1 (t) has the form of a BPSK signal the BPSK demodulator extracts the original signal b (t).

The received spectrum of the desired signal and the interference at the output of the receiver wideband filter is shown below: (for diagram refer the hard copy)

Multiplication by the spreading waveform produces the spectra as shown: (for diagram refer the hard copy)

The despreading operation at the receiver shrinks the desired signal back to its original bandwidth B at the same time it spreads the undesired signal (interference) bandwidth by the same amount , thus reducing its power spectral density . The filtering action of the demodulator removes most of the interference spectrum that does not overlap with the signal spectrum. Thus, most of the original interference energy is eliminated and does not affect the receiver performance.

IS – 95:

A U. S. digital cellular system based on CDMA which promises enhanced capacity has been standardized as Interim Standard -95. The IS -95 system is designed to be compatible with the existing cellular system (AMPS) frequency band; hence mobiles and base stations can be economically produced for dual mode operation.

IS-95 allows each user within a cell to use the same radio channel, since this is a direct sequence spread spectrum CDMA system.

IS-95 is specified for a reverse link (mobile to base station) operation in the 829-849 MHz band and 869-894 MHz for the forward link (base station to mobile). It uses a different modulation and spreading technique for the forward and reverse links.

On the forward link the base station simultaneously transmits the user data for all mobiles in the cell using a different spreading sequence for each mobile. A pilot code is also transmitted simultaneously and at a higher power level, thereby allowing all mobiles to use coherent carrier detection while estimating channel conditions.

On the reverse link, all mobiles respond in an asynchronous fashion and have ideally a constant signal level due to power control applied by the base station.

The maximum user data rate is 9.6 kb/s (Tb). The spreading rate is 1.2288 Mchip/s (Tc)

i.e. Tb = 128 Tc

ADVANTAGES:

1. CAPACITY:

There is no absolute limit on the no. of users in CDMA. Rather, increasing the no. of users in a CDMA system raises the noise floor in a linear manner.

The primary parameters that determine CDMA cellular system capacity are processing gain, Eb/No, voice duty cycle, frequency reuse efficiency, and the no. of sectors in the cell site antenna.

In the present U.S. cellular environment, there are a maximum of 57 analog FM channels in a three sector cell site. When the demand for service is at a peak, the 58th caller is given a system busy signal. There is no way even to add one more signal to a fully occupied system. This call blocking behavior results in about 35% loss of capacity.

With the CDMA system, however, there is a much softer relationship between the no. of users and the grade of service. For e.g. the system operator could decide to allow a small degradation in the bit error rate and increase the no. of available channels during the peak hours. This capability is important for avoiding dropped calls at handoff because of a lack of no. of channels.

2. FREQUENCY RE-USE:

North American cellular system i.e. AMPS allocates 416 channels to each operator (30 kHz spacing, with a total allocation of 12.5 MHz in each direction). The same frequency obviously cannot be reused in any adjacent pair of cells because a user on the boundary between those cells would receive both signals with equal amplitude, leading to an unacceptably high interference level.

A plane can be tiled with hexagonal cells, labeled in accordance with the seven-way pattern shown in the figure. Thus, if a unique set of channels is assigned to each of the seven cells, then the pattern can be repeated without violating the adjacency requirement.

Traditional time or frequency slotted systems must be designed with a reuse ratio that satisfies the worst-case interference scenario, but only a small fraction of the users actually experience that worst-case. Use of noise-like carriers, with all users occupying the same spectrum, makes the effective noise the sum of all other-user signals. The receiver correlates its input with the desired noise carrier, enhancing the signal to noise ratio at the detector. The enhancement overcomes the summed noise enough to provide an adequate SNR at the detector. Because the interference is summed, the system is no longer sensitive to worst-case interference, but rather to average interference. Frequency reuse is universal, that is, multiple users utilize each CDMA carrier frequency... The reuse pattern is now

The rainbow cells indicate that the entire 1.25 MHz passband is used by each user, and that same passband is reused in each cell.

3. SOFT HANDOFF:

Since CDMA uses co-channels, it can use macroscopic spatial diversity to provide soft hand off. Soft handoff is performed by the MSC, which can simultaneously monitor a particular user from two or more base stations.

Soft handoff allows both the original cell and a new cell to temporarily serve the call during the handoff transition. This greatly minimizes the probability of a dropped call; it also makes the hand off undetectable by the user.

This is because the CDMA – based soft hand off system provides a make– before– break switching function.

4. LOW PROBABILITY OF INTERCEPT (LPI):

LPI is the desire to hide your signal from detection by an intelligent adversary so that your transmissions will remain unnoticed and, thus, neither jammed nor exploited in any manner. For this the signal spectral density is intentionally kept small with respect to the channel noise and receiver noise so that the presence of the signal is not detected easily.

The transmitted signal has low power levels, due to spreading of the signal.

i.e.                Pav   << 1

Nav

Where Pav = average received signal power,

Nav = noise power.

The intended receiver receives the information with the help of processing gain and coding gain. Other receivers do not know the pseudo-noise sequence hence they cannot receive the information signal with the help of processing gain and coding gain.

DISADVANTAGES:

- NEAR-FAR PROBLEM:

In CDMA the power of multiple users at the receiver determines the noise floor after decorrelation. In the mobile environment some users may be located near the base station, others may be located far away. If all of the mobile phones were to use the same transmitted power, then the close-by mobile stations would apparently jam the far-away mobile stations. The propagation path loss difference between those extreme users can be many tens of dB.

In general the strongest received mobile signal willl capture the demodulator at the base station. In CDMA, stronger received signal levels raise the noise floor at the base station demodulators for the weaker signals, thereby decreasing the probability that weaker signals will be received.

Way to overcome:

To combat the near-far problem power control is used in most CDMA implementations. Power control is provided by each base station in a cellular system and assures that each mobile within the base station coverage area provides the same signal level to the base station receiver.

- SELF – JAMMING :

Self- jamming is a problem in CDMA system. Self-jamming arises from the fact that spreading sequences of different users are not exactly orthogonal; hence in the despreading of particular PN code, non-zero contributions to the receiver decision statistic, for a desired user arise from the transmissions of other users in the system.

3.   REDUCTION IN MULTIPATH FADING:

Channel data rates are very high in CDMA systems. Consequently, the symbol (chip) duration is very short and usually much less than the channel delay spread. Since PN sequences have low autocorrelation, multipath which is delayed by ore than a chip will appear as noise.

Way to overcome:

All multipath returns contain information regarding the data that is being sent, that information can be extracted by an appropriately designed receiver. Such a receiver is called a RAKE receiver. The RAKE receiver attempts to resolve many individual multipath returns a possible and then sum them coherently

Future:

The existing platforms for mobile communications are highly reliable, high quality platforms but are not suitable for high speed data. Telecom recognized that in the future our customers will demand more complex wireless services - orientated around high speed data. The introduction of CDMA will provide Telecom with a leading technology, and network capable of supporting these demands. CDMA will ensure Telecom is well positioned to quickly and cost effectively upgrade to 3G and will provide high speed data applications and multimedia solutions in the near future. CDMA has a number of evolution advantages over other wireless technologies. The ability to evolve to either 3G standard provides Carrier flexibility to evolve to best technology provides Customer with certainty for the future. It also provides for full forward & backward compatibility for handsets.

CDMA is fully equipped network providing enhanced data functionality with high data rates of 14.4 kbps. Integration with existing Telecom IP VPN services is also possible. Soon after launch Telecom will deploy the CDMA1Xstandard providing packet data up to 144 kbps. CDMA uses conventional IP standards provides seamless connection from laptop computers & PDA devices access to the Internet, Intranets, e-mail, RAS servers ... via your CDMA terminal. Thus the CDMA network will provide WAP (Wireless Application Protocol) functionality technology standards.

As of today communication services based on CDMA are on a small scale use but in lieu of the tremendous advantage it has over the current system, it can be stated that CDMA is definitely the technology of the future.

Bibliography:

- Rappoport, T. S., “Wireless Communicatioins”, Prentice Hall, PTR, 1996.

- www.cdg.org

- www.cdmaonline.com