CS401 
[vu_pk]

www.groups.yahoo.com/group/vu_pk

IMPORTANT QUESTIONS
LINKS

IMPORTANT QUESTIONS

| Q: | What are Registers and why these are needed? |
| --- | --- |

| A: | Registers are like a scratch pad ram inside the processor and their operation is very much like normal memory cells. They have precise locations and remember what is placed inside them. They are used when we need more than one data element inside the processor at one time. |
| --- | --- |

| Q: | How we calculate Physical Address? |
| --- | --- |

| A: | for the whole megabyte we need 20 bits while CS and IP are both 16bit registers. We need a mechanism to make a 20bit number out of the two 16bit numbers. Consider that the segment value is stored as a 20 bit number with the lower four bits zero and the offset value is stored as another 20 bit number with the upper four bits zeroed. The two are added to produce a 20bit absolute address. A carry if generated is dropped without being stored anywhere and the phenomenon is called address wraparound. The process is explained with the help of the following diagram. 19-----------------------------------0 16bit Segment Register 0000 16bit Logical Address 0000 20bit Physical Address 15----------------------------0 15----------------------------0 Segment Address --------------0 Offset Address --------------0 Therefore memory is determined by a segment-offset pair and not alone by any one register which will be an ambiguous reference. Every offset register is assigned a default segment register to resolve such ambiguity. For example the program we wrote when loaded into memory had a value of 0100 in IP register and some value say 1DDD in the CS register. Making both 20 bit numbers, the segment base is 1DDD0 and the offset is 00100 and adding them we get the physical memory address of 1DED0 where the opcode B80500 is placed. |
| --- | --- |

| Q: | What is a Stack? |
| --- | --- |

| A: | Stack is a data structure that behaves in a first in last out manner. It can contain many elements and there is only one way in and out of the container. When an element is inserted it sits on top of all other elements and when an element is removed the one sitting at top of all others is removed first. To visualize the structure consider a test tube and put some balls in it. The second ball will come above the first and the third will come above the second. When a ball is taken out only the one at the top can be removed. The operation of placing an element on top of the stack is called pushing the element and the operation of removing an element from the top of the stack is called popping the element. The last thing pushed is popped out first; the last in first out behavior. |
| --- | --- |

| Q: | What is Big-Endian and Little-Endian? |
| --- | --- |

| A: | Big-endian and little-endian are terms that describe the order in which a sequence of bytes are stored in computer memory. Big-endian is an order in which the "big end" (most significant value in the sequence) is stored first (at the lowest storage address). Little-endian is an order in which the "little end" (least significant value in the sequence) is stored first. For example, in a big-endian computer, the two bytes required for the hexadecimal number 4F52 would be stored as 4F52 in storage (if 4F is stored at storage address 1000, for example, 52 will be at address 1001). In a little-endian system, it would be stored as 524F (52 at address 1000, 4F at 1001). IBM's 370 computers, most RISC-based computers, and Motorola microprocessors use the big-endian approach.On the other hand, Intel processors (CPUs) and DEC Alphas and at least some programs that run on them are little-endian. |
| --- | --- |

RELATED LINKS

http://en.tldp.org/HOWTO/Unix-and-Internet-Fundamentals-HOWTO/core-formats.html:

|  | A link for explanation about "How does my computer store things in memory". |
| --- | --- |

http://www.asciitable.com:

|  | You can see Standard ASCII table and Extended ASCII table on this link with ASCII codes expressed in HEX,OCT and Decimal notations. |
| --- | --- |

http://www.osdata.com/topic/language/asm/asmintro.htm:

|  | This is a tutorial about Assembly Language . Read it. |
| --- | --- |