1.	A student is provided with two filament lamps A and B. Each is rated 12 V 60 W, but lamp A has a carbon filament and lamp B a tungsten filament.

Calculate the resistance of each filament lamp under normal operating conditions.

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

Resistance = ..............................................

(3)

The graph shows the relationship between the current I and the potential difference V for the two filament lamps.

Describe how the resistances of lamp A and lamp B vary with current.

Lamp A: ..........................................................................................................................

.........................................................................................................................................

Lamp B: ..........................................................................................................................

.........................................................................................................................................

(2)

The diagram shows the two filament lamps connected in series across a battery of e.m.f 12 V and negligible internal resistance. Both filaments glow dimly, but the hlank filament of lamp A is the brighter.

Why does each filament glow only dimly?

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(1)

Suggest why the filament of lamp A is the brighter.

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(2)

(Total 8 marks)

2.	Complete the table by selecting a typical value for each physical quantity from the list below. Each value may be used once, more than once or not at all.

0.05 	0.3 mm s–1	2.0 × 10–8m	300 K
	6.0 	30 cm s–1	2.0 × 1015 m	3000 K
	2 k
	10 M

| Physical quantity | Typical value |
| --- | --- |
| Resistance of a voltmeter |  |
| Internal resistance of a car battery |  |
| Internal resistance of an e.h.t. supply |  |
| Resistivity of an insulator |  |
| Drift velocity of electrons in a metallic conductor |  |
| Temperature of a working filament bulb |  |

(Total 6 marks)

3.	A 500 W electric toaster operates from the 230 V mains. Calculate the current in the heating element of the toaster.

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

Current = ........................................................

(3)

The heating element of the toaster is made of much thinner wire than the wire in its supply cable. In which of these two wires do the electrons have the greater drift velocity?

..........................................................................................................................................

(1)

Explain your answer. You may be awarded a mark for the clarity of your answer.

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

(4)

(Total 8 marks)

4.	The diagram shows a type of resistor commonly used in electronic circuits.

It consists of a thin film of carbon wrapped around a cylindrical insulator. The diagram below (not to scale) shows a typical film of carbon, resistivity , before it is wrapped round the insulator.

Show that the resistance R of the carbon film is given by

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

(2)

This film has length l = 8.0 mm, width w = 3.0 mm and thickness t = 0.0010 mm
(i.e. t = 1.0 × 10–6 m). If the resistivity of carbon is 6.0 × 10–5m, calculate the resistance of the carbon film.

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

Resistance = ..................................................

(3)

Show that the resistance of a square piece of carbon film of uniform thickness is independent of the length of the sides of the square.

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

(2)

(Total 7 marks)

5.	Define the term specific heat capacity.

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

(2)

A student decides to measure the specific heat capacity of aluminium by an electrical method. He selects his apparatus and then assembles the aluminium block, the thermometer and the heating element as shown.

The student intends to substitute his results into the relationship

mcT = VIt

Draw a diagram of the electrical circuit he would need to set up in order to be able to carry out the experiment.

(3)

What other pieces of apparatus would he need?

..........................................................................................................................................

..........................................................................................................................................

(2)

He carries out the experiment and then calculates his value for the specific heat capacity of aluminium. He discovers that his value is higher than the accepted value of 900 J kg–1 K–1.

Suggest why his result is higher than 900 J kg–1 K–1.

..........................................................................................................................................

..........................................................................................................................................

(1)

With reference to the apparatus shown in the diagram, state two modifications that he should make in order to minimise the discrepancy.

1. ...........................................................................................................................................

..........................................................................................................................................

2. ...........................................................................................................................................

..........................................................................................................................................

(2)

(Total 10 marks)

6.	A sauna is a room in which there is a stove containing very hot bricks of basalt over which water is poured.

Before use the basalt is tested for resistance to cracking by first heating the bricks to 750°C and then dropping them into cold water.

Each brick has a mass of 1.4 kg and an initial temperature of 22 °C. Each receives 860 kJ of energy from the burning wood. Show that the specific heat capacity of basalt is approximately 850 J kg–1 K–1.

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

(3)

When several hot bricks are dropped into a trough of cold water, steam is immediately produced. After a short time the temperature of the water rises to a steady value. Some of the energy is used to heat the surroundings, as shown in the energy-flow diagram.

(2)

(Total 5 marks)

7.	Write down the ideal gas equation.

..........................................................................................................................................

..........................................................................................................................................

(1)

Use your equation to give the unit for the molar gas constant in SI base units.

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

(2)

The pressure p of a gas is related to its density p and the mean square molecular speed c2 by the formula

p =

Use this relationship and the ideal gas equation to show that the average kinetic energy of a molecule is proportional to the kelvin temperature of the gas.

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

(3)

Sketch a graph to show how the product pV (pressure × volume) varies with temperature in °C for one mole of ideal gas.

Comment on any significant features of your graph.

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

(4)

(Total 10 marks)

8.	Define the term e.m.f. of a cell.

..........................................................................................................................................

..........................................................................................................................................

..........................................................................................................................................

(2)

A student wants to use a graphical method to determine the internal resistance r of a cell of known e.m.f. .

Complete the diagram below showing how the cell should be connected in a circuit to allow the student to do this.

(2)

Sketch the graph the student should plot and state how she could determine r from the graph.

..........................................................................................................................................

..........................................................................................................................................

(2)

(Total 6 marks)