| Table 2-2: Wireless Standards |  |  |  |
| --- | --- | --- | --- |
|  | 802.11a | 802.11b | 802.11g |
| Data Rate | 54Mbps | 11Mbps | 54Mbps |
| Frequency | 5 GHz | 2.4 GHz | 2.4 GHz |
| Compatibility | None | With 802.11g | With 802.11b |
| Range | 25–75 feet | 100–150 feet | 100–150 feet |

| Table 2-9: 10Mb Ethernet Properties |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ethernet Type | Distance Limitation | Cable Type | Interface Type | Physical Topology | Logical Topology |
| 10Base5 | 500 meters | Thick coaxial cable—50 ohm (thicknet) | AUI | Bus | Bus |
| 10Base2 | 185 meters | Thin coaxial cable (thinnet) | BNC | Bus | Bus |
| 10BaseT | 100 meters | Unshielded twisted pair (UTP) cabling (CAT-3, -4, -5) | RJ-45 | Star (Hub) | Bus |

| Table 2-10: 100Mb Ethernet Properties |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ethernet Type | Distance Limitation | Cable Type | Cabling | Physical Topology | Logical Topology |
| 100BaseTX | 100 meters | UTP CAT-5 | RJ-45 | Star (Hub) | Bus |
| 100BaseFX | 400 meters half-duplex, 2,000 meters full-duplex | MMF 62.5/125 micron with SC and ST connectors | RJ-45 | Star (Hub) | Bus |
| 100BaseT4 | 100 meters | UTP CAT-3,4,5 | RJ-45 | Star (Hub) | Bus |

| Table 2-11: 1Gbps Ethernet Properties |  |  |
| --- | --- | --- |
| Ethernet Type | Distance Limitation | Cable Type |
| 1000BaseCX | 25 meters | Shielded twisted pair (STP) copper |
| 1000BaseLX | 3–10 kilometers | SMF |
| 1000BaseSX | 220 meters | MMF |
| 1000BaseT | 100 meters | CAT-5E and CAT-6 UTP |
| 1000BaseZX | 100 meters | SMF |

| Table 2-12: Cable Type Comparisons |  |  |  |
| --- | --- | --- | --- |
| Cable | Distance | Data Rates | Comparison |
| UTP | 100 meters | 10–100Mbps | Is easy to install but is susceptible to interference |
| STP (Shielded Twisted Pair) | 100 meters | 10–100Mbps | Is difficult to install |
| Coaxial | 500 meters | 10–100Mbps | Is easy to install but is difficult to troubleshoot |
| Fiber | 10 kilometers | 10Mbps–100Gbps | Is difficult and expensive to install, difficult to troubleshoot, but can span very long distances and is not susceptible to interference |