Specifications

- Continuous Power (20Hz-20kHz, 0.08% THD)

  - 4 ch: (4 Ohm) 50Wx4

  - 4 ch: (2 Ohm) 60Wx4

  - 2 ch: (4 Ohm) 120Wx2

- Maximum Power (EIAJ)

  - 4 ch: (4 Ohm) 100Wx4

  - 2 ch: (4 Ohm) 240Wx2

- Frequency Response: 10Hz - 50kHz (0,-1dB)

- Total Harmonic Distortion: 0.004% (1kHz, 4 Ohm)

- Signal-to-Noise Ratio: 105dB (IHF-A Weighted, at 1kHz)

- Dimensions: 10" x 2-3/8" x 12-3/8"

Features

- Bridgeable 2/3/4 Channel Capability

- 1-Ohm Stable Operation

- PWM Regulated MOSFET Power Supply

- Balanced Isolator Input Circuit

- Gold-Plated Screw-Type Power/Ground Terminals

- Gold-Plated Screw-Type Speaker Terminals

- Gold-Plated RCA Inputs (x2)

- Gold-Plated RCA Outputs

- Hi-Volt Input Level Control (400mV-4V)

- Speaker Level Input Adapter (included)

- Variable Crossover: 50-120Hz, -12dB/oct. (LPF/HPF) (Front & Rear Channels)

- Bass Boost (60Hz, 0 to +12dB)